3GPP TSG RAN WG1 #109-e			R1-2205008

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.16.12

Source: 	Qualcomm Incorporated

Title: 	UE features for MBS

Document for:	Discussion and Decision

- 1. Introduction

This contribution includes our views on each FG and the tracking changes are used with respect the version of the agreed FGs for Rel-17 NR MBS in [1].

Given the potential UE testing differentiation among licensed, unlicensed, and NTN band, the type for a UE feature should be ‘per Band’ for broadcast and ‘per FSPC’ for multicast in Rel-17 NR MBS.

- FG33-1

Since broadcast and multicast are separate features, FG33-1a for DCI-indicated slot-level repetition for MTCH should be added, separate from FG 33-3-1.

We suggest the changes on FG 33-1 as:

| 33. NR_MBS | 33-1 | Broadcast | - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI(s) for MTCH. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 4_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH for broadcast in different slots. - Support MCCH change notification indication via DCI. - support of higher layer configured slot-level repetition up to 8 for MTCH - Support of FDMed MCCH and PBCH |  | Up to RAN2 |  |  | Up to RAN2 | Up to RAN2 | Up to RAN2 |  |  | Up to RAN2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-1a | Dynamic slot-level repetition for Broadcast | Support of DCI-indincated slot-level repetition up to 16 for MTCH | 33-1 | Yes |  |  | Up to RAN2 | Up to RAN2 | Up to RAN2 |  |  | Up to RAN2 |

- FG33-2 and FG33-2x

We suggest the following changes for the remaining FFSs on FG 33-2 and FG33-2x as:

| 33. NR_MBS | 33-2 | Dynamic scheduling for multicast for PCell | - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI for PCell. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 4_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH for multicast in different slots. - Support {2, 4, 8} times semi-static slot-level repetition for group-common PDSCH for multicast | FFS | Yes |  |  | per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-2a | Support of ACK/NACK based HARQ-ACK feedback andRRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | 1) Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling 2) Support of PTM retransmission for multicast 3) support of Type-1 and Type-2 HARQ-ACK CB for multicast feedback only 4) Support of shared PUCCH resource configurations with unicast define separate PUCCH resource configurations from unicast as separate FG (as FG33-8-1) from FG 33-2a | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2b | DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling via DCI format 4_2 | 33-2a, 33-2f | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2d | PTP retransmission for multicast | Support of PTP retransmission for multicast | 33-2a | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2e | Multiple G-RNTIs for group-common PDSCHs | Capability on number of G-RNTI for groupcast | [33-2] | Yes |  |  | Per FSPC | N/A | N/A |  | Candidate values: {2, 3, ,…15} | Optional with capability signalling |
| 33. NR_MBS | 33-2f | Dynamic multicast with DCI format 4_2 | Support of DCI format 4_2 with CRC scrambled with G-RNTI for multicast | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2g | MIMO layers for multicast PDSCH | Supported maximal number of MIMO layers for multicast PDSCH | 33-2 | Yes |  | UE supports 1 MIMO layer only for multicast PDSCH | Per FSPC | N/A | N/A |  | Candidate values: {2,4,8} Note: If UE supports up to 8 layers, the UE supports TB2 | Optional with capability signalling |
| 33. NR_MBS | 33-2h | Dynamic scheduling for multicast for SCell | Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI for SCell. | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2l | Modulation order for multicast PDSCH | Supported maximal modulation order for multicast PDSCH | 33-2 | Yes |  | - for FR1, the network uses the modulation order signalled in pdsch-256QAM-FR1. - for FR2, the network uses the modulation order signalled per band i.e. pdsch-256QAM-FR2 if signalled. | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |

- FG33-3-3-x

We suggest the following changes for the remaining FFSs of FG 33-3-x and add FG33-3-2a for additional scaling factor for max data rate to support FDMed unicast PDSCH and multicast PDSCH, where details of the additional scaling factor are discussed in [2].

| 33. NR_MBS | 33-3-1 | Dynamic Slot-level repetition for group-common PDSCH | - Support up to X times dynamic slot-level repetition for group-common PDSCH for multicast. | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  | Candidate values for X is: {8, 16} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH | - Support FDM between one unicast PDSCH and one group-common PDSCH in a slot. | [33-1, 33-2] | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-2a | Calculating maximum data rate of FDMed unicast PDSCH and group-common PDSCH | Scaling factor for max data rate to support FDMed unicast PDSCH and group-common PDSCH in a slot per CC. | 33-3-2 | Yes |  | If not reported, same as the scaling factor for max data rate of unciast PDSCH | Per FSPC | N/A | N/A |  | Value of scaling factor: {1.6, 1.2, 1, and 0.75} | Optional with capability signalling |
| 33. NR_MBS | 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). | [33-1, 33-2] | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-3a | FDM-ed Type-1 and Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast | - Support of FDM-ed Type-1 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast - Support of Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast with max number X of G-RNTIs | 33-2a, 33-3-2 | Yes |  |  | Per FSPC | N/A | N/A |  | Note1: FDM-ed Type-1 HARQ-ACK codebook is generated by concatenating the Type-1 sub-codebook for unicast and the Type-1 sub-codebook for multicast. Note2: The Type-2 HARQ-ACK codebook is generated by concatenating the Type-2 sub-codebook for unicast and the Type-2 sub-codebook for multicast. Candidate values of X is {2, 3, 4} with X no lareger than max number of G-RNTIs of FG33-2e | Optional with capability signalling |
| 33. NR_MBS | 33-3-3b | Mode 2 TDM-ed Type-1 and Type-2 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast | - Support of Mode 2 TDM-ed Type-1 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast - Support of Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast with max number X of G-RNTIs | 33-2a | Yes |  |  | Per FSPC | N/A | N/A |  | Note1: Mode 2 TDM-ed Type-1 HARQ-ACK codebook is generated based on the union TDRA tables from unicast and multicast and the union of k1 sets from unicast and multicast. Note2: The Type-2 HARQ-ACK codebook is generated by concatenating the Type-2 sub-codebook for unicast and the Type-2 sub-codebook for multicast. Candidate values of X is {2, 3, 4} with X no lareger than max number of G-RNTIs of FG33-2e | Optional with capability signalling |
| 33. NR_MBS | 33-3-4 | Mode 1 for type1 codebook generation | Supports type1-Codebook-Generation-Mode configured as mode 1 | 33-3-3b | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-5 | Feedback multiplexing for unicast PDSCH and group-common PDSCH for multicast with same priority and different codebook type | Support of multiplexing HARQ-ACK for unicast and multicast with the same priority and different HARQ-ACK codebook types in the same PUCCH slot | 33-2a | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |

- FG33-4 and FG33-4-1

We suggest the following changes for the remaining FFSs of FG33-4 and FG33-4-1. In addition, we suggest adding FG33-4a for supporting more than one NACK-only in a PUCCH resource based on the RAN1 agreement.

Agreement

For supporting more than one NACK-only feedback in the same PUCCH transmission, define RRC configuration to configure between Alt1 and Alt4 (from previous agreements):

- Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits.

  - FFS: how to determine PUCCH resource

- Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission.

  - define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback,

    - FFS: The PUCCH slot for the transmission is based on the K1 in the “last DCI” scheduling multicast.

    - FFS: The PUCCH resource for the transmission is from PUCCH-config configured for NACK-only based feedback according to the mapping between number of TBs with PUCCH resource ID.

      - FFS mapping details.

      - How to determine the number of TBs is discussed separately, e.g., Type-1-like and/or Type-2-like codebook.

    - FFS: whether this applies to a single G-RNTI or multiple G-RNTIs

  - Alt4 is not supported for more than 4 TBs

- FFS: whether RRC configuration between Alt1 and Alt4 is per G-RNTI or per CFR

- FFS: UE capability

| 33. NR_MBS | 33-4 | NACK-only based HARQ-ACK feedback for multicast | - Support NACK-only based HARQ-ACK feedback, and support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling - Support of PTM retransmission for multicast - Support of Type-1 and Type-2 HARQ-ACK CB for multicast feedback only - Support of shared PUCCH resource configurations with unicast - Support one NACK-only based HARQ-ACK feedback in a PUCCH resource define separate PUCCH resource configurations from unicast as separate FG (FG33-8-3) from FG 33-4 | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-4a | More than one NACK-only based HARQ-ACK feedback for multicast in a PUCCH resource | Support of more than one NACK-only based HARQ-ACK feedback in a PUCCH resource | 33-4 | Yes |  |  | Per FSPC | N/A | N/A |  | Note: up to 4 TBs with NACK-only feedback in a PUCCH resource | Optional with capability signalling |
| 33. NR_MBS | 33-4-1 | DCI-based enabling/disabling NACK-only based feedback for dynamic scheduling for multicast | Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signaling via DCI format 4_2 | 33-4, 33-2b | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |

- FG33-5-1 and FG33-5-1x

We suggest the following changes for the remaining FFSs of FG33-5-1 and FG33-5-1x as

| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast | - Support one SPS group-common PDSCH configuration for multicast - Support {2, 4, 8} times semi-static slot-level repetition for SPS group-common PDSCH - Support of group-common PDCCH/PDSCH with CRC scrambled by G-CS-RNTI(s) for multicast - Support of DCI format 4_1 with CRC scrambled with G-CS-RNTI for multicast - ACK/NABK-based HARQ-ACK feedback for SPS group-common PDCCH activation and SPS release associated with G-CS-RNTI | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-5-1a | Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast | 1) Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling, SPS group-common PDSCH activation, and SPS release PDCCH 2) Support of PTM retransmission for SPS multicast associated with G-CS-RNTI 3) Support of Type-1 and Type-2 HARQ-ACK CB for SPS multicast feedback only 4) Support of shared SPS-PUCCH-AN-List with unicast | 33-5-1 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1b | Dynamic multicast with DCI format 4_2 | Support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1c | DCI-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast | Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-CS-RNTI for multicast by RRC signaling via DCI format 4_2 | 33-5-1a, 33-5-1b | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1d | PTP retransmission for SPS group-common PDSCH for multicast | Support of PTP retransmission associated with CS-RNTI for SPS multicast | 33-5-1a | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1e | Dynamic Slot-level repetition for SPS group-common PDSCH for multicast | Support up to X times dynamic slot-level repetition for SPS group-common PDSCH for multicast. | 33-5-1 | Yes |  |  | Per FSPC | N/A | N/A |  | Candidate values for X is: {8, 16} | Optional with capability signalling |
| 33. NR_MBS | 33-5-1f | NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast | 1) Support NACK-only based HARQ-ACK feedback, and support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling 2) Support of PTM retransmission associated with G-CS-RNTI for SPS multicast 3) Support of Type-1 and Type-2 HARQ-ACK CB for SPS multicast feedback only 4) Support of shared SPS-PUCCH-AN-List with unicast | 33-5-1 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1g | DCI-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast | Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-CS-RNTI for multicast by RRC signaling via DCI format 4_2 | 33-5-1f, 33-5-1b | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1h | Multiple G-CS-RNTIs for SPS group-common PDSCHs | Max number of G-CS-RNTIs for SPS multicast | 33-5-1 | Yes |  |  | Per FSPC | N/A | N/A |  | Candidate values: {2, 3,…8} | Optional with capability signalling |
| 33. NR_MBS | 33-5-2 | Multiple SPS group-common PDSCH configuration | - Support [N>1] SPS group-common PDSCH configuration per CFR for multicast, and up to K SPS group-common PDSCH configuration in a cell group - Support M>=1 activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  | Candidate values of K and M: {2, 3, .. 8} | Optional with capability signalling |

- FG33-6-x

We suggest the following changes for the remaining FFSs of FG33-6-x as

| 33. NR_MBS | 33-6-1 | DL priority indication for multicast in DCI | 1. Support of priority indicator field configured in DCI formats 4_2 with CRC scrambled with G-RNTI for multicast. 2. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast and multicast at a UE | 33-2a, 33-2f | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-6-1a | DL priority configuration for SPS multicast | Support of priority configured for multicast HARQ-ACK feedback of SPS multicast | 33-5-1a | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-2 | Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for unicast and multicast at a UE | 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for unicast and multicast at a UE. | 33-6-1 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-3 | More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot | 1. Supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot. | 33-6-1, 33-6-2 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |

- New FG33-8 for multicast PUCCH resource configration

| 33. NR_MBS | 33-8-1 | PUCCH resource configuration for multicast feedback for dynamically scheduled multicast | Support of a PUCCH-Config for multicast HARQ-ACK feedback, separate from that of unicast configurations | 33-2a or 33-4 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-8-2 | Up to 2 PUCCH resources configuration for multicast feedback for dynamically scheduled multicast | Support of a PUCCH-ConfigurationList for multicast HARQ-ACK feedback, separate from that of unicast configurations | 33-8-1 or 33-8-3, 33-6-1 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-8-3 | PUCCH resource configuration for multicast feedback for SPS GC-PDSCH | Support of a SPS-PUCCH-AN-List for multicast HARQ-ACK feedback of all multicast SPS configuration(s), separate from that of SPS unicast configurations | 33-5-1a or 33-5-1f | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |

- FG33-9

We suggest the changes for the remaining FFSs of FG 33-9 as:

| 33. NR_MBS | 33-9 | Supporting unicast PDCCH to release SPS group-common PDSCH | Supports unicast PDCCH scrambled with CS-RNTI to release SPS group-common PDSCH | 33-5-1 | Yes |  |  | Per FSPCPer UE | N/ANo | N/ANo |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# Reference:

[1] R1-2202928, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e”, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#108-e, February 21st – March 3rd, 2022.

[2] R1-220xxxx, “Other remaining issues for Rel-17 MBS”, Qualcomm, RAN1#109-e, May 9th – 20th, 2022.