3GPP TSG RAN WG1 #109-e			R1-2205009

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.16.13

Source: 	Qualcomm

Title: 	UE features for DSS and LTE_NR_DC_enh2

Document for:	Discussion and Decision

- Introduction

In this contribution, we propose details of UE features for DSS and TRS for fast SCell activation.

- Discussion

The proposed changes to the feature group list are provided in the tables with blue in Section 3. Rationales of the proposed changed are provided below.

FG34-1: Cross-carrier scheduling from SCell to PCell/PSCell with search space restrictions (Type A)

- Candidate value set: One or more of supported SCS combinations ({P(S)Cell SCS in kHz, sSCell SCS in kHz}) from following set are indicated by the UE: {15,15}, {15,30}, (15, 60), [{30,30}, {30,60},{60,60}])

  - [Qualcomm] So far, we do not see a use-case of cross-carrier scheduling from sSCell to P(S)Cell when P(S)Cell SCS is not 15kHz.

- [Candidate value set 2: frequency band pair(s) for {PCell/PSCell, sSCell}]

  - [Qualcomm] We propose to confirm this by removing the square blacket because of the following reason:

    - The feature is supposed to be used in a DSS carrier. For a DSS carrier, the UE is typically configured with LTE-CRS rate-matching for PDSCH. The relevant FGs for LTE-CRS rate-matching, FG5-28, FG14-1, and FG14-1a, are per-band capability. Therefore, for a given CA band combination, the UE may not support DSS features in all the bands. If the UE is not able to report frequency band pair(s) of {PCell/PSCell, sSCell} in a CA band combination, the UE has to support Rel-17 cross-carrier scheduling for any pair bands of {PCell/PSCell, sSCell}, including the pair(s) where there is no DSS operation on PCell/PSCell. This must cause interoperability issue.

- Component 4): FFS: N is based on pair of (PCell/PSCell SCS, sSCell SCS): N=1 for(15,15), (30,30), (60,60) and N=2 for (15,30), (30,60) and N=4 for (15, 60)

  - [Qualcomm] So far, we do not see a use-case of cross-carrier scheduling from sSCell to P(S)Cell when P(S)Cell SCS is not 15kHz.

- Component 4) candidate values of K1, K2: [(K1, K2) = (2,2) for FDD P(S)Cell; (K1, K2) = (2,4) for TDD P(S)Cell]

  - [Qualcomm] We are OK to confirm this by removing the square blacket

- Component 9) candidate values: [Value 1: PDCCH monitoring occasion(s) on PCell/PSCell and on sSCell for cross-carrier scheduling to PCell/PSCell is within the first 3 OFDM symbols of a PCell/PSCell slot. Value 2: PDCCH monitoring occasion(s) on PCell/PSCell and on sSCell for cross-carrier scheduling to PCell/PSCell is not restricted to the first 3 OFDM symbols of a PCell/PSCell slot]

  - [Qualcomm] We support having these candidate values. Value 2 essentially requires UE to implement “PDCCH case 2” equivalent and hence should not be mandated. We propose to confirm both values by removing the square blacket.

- Component 10) FFS: frame boundary alignment between PCell/PSCell and sSCell

  - [Qualcomm] During WI discussion, RAN1 agreed to support unaligned boundary between two cells other than PCell/PSCell and sSCell. Therefore, regardless of wether or not the component 10) is present, unaligned CA between PCell/PSCell and sSCell is not supported. We are OK to enable unaligned CA between PCell/PSCell and sCell if an optional UE capability is defined. We propose to delete the FFS to confirm the component 10), and discuss a separate FG for unaligned CA between PCell/PSCell and sSCell. Example is provided in the table below as FG34-3.

FG34-2: Cross-carrier scheduling from SCell to PCell/PSCell (Type B)

- Candidate value set: One or more of supported SCS combinations ({P(S)Cell SCS in kHz, sSCell SCS in kHz}) from following set are indicated by the UE: {15,15}, {15,30}, (15, 60), [{30,30}, {30,60},{60,60}])

  - [Qualcomm] So far, we do not see a use-case of cross-carrier scheduling from sSCell to P(S)Cell when P(S)Cell SCS is not 15kHz.

- [Candidate value set 2: frequency band pair(s) for {PCell/PSCell, sSCell}]

  - [Qualcomm] We propose to confirm this by removing the square blacket because of the following reason:

    - The feature is supposed to be used in a DSS carrier. For a DSS carrier, the UE is typically configured with LTE-CRS rate-matching for PDSCH. The relevant FGs for LTE-CRS rate-matching, FG5-28, FG14-1, and FG14-1a, are per-band capability. Therefore, for a given CA band combination, the UE may not support DSS features in all the bands. If the UE is not able to report frequency band pair(s) of {PCell/PSCell, sSCell} in a CA band combination, the UE has to support Rel-17 cross-carrier scheduling for any pair bands of {PCell/PSCell, sSCell}, including the pair(s) where there is no DSS operation on PCell/PSCell. This must cause interoperability issue.

- Component 4): FFS: N is based on pair of (PCell/PSCell SCS, sSCell SCS): N=1 for(15,15), (30,30), (60,60) and N=2 for (15,30), (30,60) and N=4 for (15, 60)

  - [Qualcomm] So far, we do not see a use-case of cross-carrier scheduling from sSCell to P(S)Cell when P(S)Cell SCS is not 15kHz.

- Component 7) candidate values: [Value 1: PDCCH monitoring occasion(s) on PCell/PSCell and on sSCell for cross-carrier scheduling to PCell/PSCell is within the first 3 OFDM symbols of a PCell/PSCell slot. Value 2: PDCCH monitoring occasion(s) on PCell/PSCell and on sSCell for cross-carrier scheduling to PCell/PSCell is not restricted to the first 3 OFDM symbols of a PCell/PSCell slot]

  - [Qualcomm] We support having these candidate values. Value 2 essentially requires UE to implement “PDCCH case 2” equivalent and hence should not be mandated. We propose to confirm both values by removing the square blacket.

- Component 8) FFS: frame boundary alignment between PCell/PSCell and sSCell

  - [Qualcomm] During WI discussion, RAN1 agreed to support unaligned boundary between two cells other than PCell/PSCell and sSCell. Therefore, regardless of wether or not the component 10) is present, unaligned CA between PCell/PSCell and sSCell is not supported. We are OK to enable unaligned CA between PCell/PSCell and sCell if an optional UE capability is defined. We propose to delete the FFS to confirm the component 10), and discuss a separate FG for unaligned CA between PCell/PSCell and sSCell. Example is provided in the table below as FG34-3.

FG34-3 (new): Non-aligned frame boundaries between PCell/PSCell and sSCell

As discussed in the above FG34-1 and FG34-2, we propose to add a new FG to accommodate the possible operation of unaligned CA between PCell/PSCell and sSCell. Since non-aligned frame boundaries is enabled only if/where a specific need is identifed, we propose to enable that a UE can report this UE capability per candidate value set of SCS combination(s) and candidate value set of frequency band pair(s) as in FG34-1 or FG34-2. Detailed proposed FG is given as FG34-3 below.

FG34-4 (new): disabling scaling factor a when sSCell is deactivated

At the RAN1#108-e meeting, introducing a new FG for the support of disabling scaling factor a when sSCell is deactivated was agreed. We propose to introduce a new FG to capture this agreement as FG34-4.

FG34-5 (new): disabling scaling factor a when sSCell is switched to dormant BWP

At the RAN1#108-e meeting, introducing a new FG for the support of disabling scaling factor a when sSCell is switched to dormant BWP was agreed. We propose to introduce a new FG to capture this agreement as FG34-5.

FG35-1: Aperiodic CSI-RS for tracking for fast SCell activation

- For the candidate values of components 3) and 4):

  - [Qualcomm] For legacy TRS (FG2-51), a UE is able to report maximum number of TRS resource sets from the value range of {1, … 64} per CC and {1, … 256} per UE. Compared to FG2-51, the current FG35-1 has too coarse granularity for value ranges of component 3) and component 4). We have a concern on this and think the value ranges of component 3) and component 4) should be finer. Instead of coming up with a very limited set of possible values in RAN1, we propose to leave them up to RAN2. RAN2 should be able to come up with a balanced conclusion on the granularity and overhead. Once RAN2 reaches the value ranges, RAN1 can further discuss, if necessary, whether one or some values are not applicable to FR2.

FG35-2 (new): supported aperiodic CSI-RS bandwidth for tracking for fast SCell activation for 10MHz UE channel bandwidth

At the RAN1#108-e meeting, support of additional TRS bandwidth for fast SCell activation for 10MHz UE channel bandwidth was agreed. RAN1 spec already captures this as FG35-2. We propose to define it as below. In general, it is a copy of trs-AdditionalBandwidth-r16.

- Proposed Updates on UE features for NR_DSS and LTE_NR_DC_enh2

| 34. NR_DSS | 34-1 | Cross-carrier scheduling from SCell to PCell/PSCell with search space restrictions (Type A) | Support of Cross-carrier scheduling from sSCell to PCell/PSCell with search space restrictions (Type A) - Cross-carrier scheduling from sSCell to PCell/PSCell with CIF - Search space restrictions: sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and at least following search space sets on PCell/PSCell can only be configured such that UE does not monitor them in overlapping [slot/symbol] of PCell/PSCell and sSCell - USS sets for DCI formats 0_1,1_1,0_2,1_2 - USS sets for DCI formats 0_0,1_0 - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI - Configuration of scaling factor α for BD and CCE limit handling and PDCCH overbooking handling on P(S)Cell - The number of unicast DCI limits for PCell/PSCell scheduling - Processing K1 unicast DCI scheduling DL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - Processing K2 unicast DCI scheduling UL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - FFS: N is based on pair of (PCell/PSCell SCS, sSCell SCS): N=1 for(15,15), (30,30), (60,60) and N=2 for (15,30), (30,60) and N=4 for (15, 60) - Same numerology between sSCell and P(S)Cell or sSCell SCS is larger than P(S)Cell SCS - USS set(s) for DCI format 0_1,1_1 configured on sSCell for CCS from sSCell to Pcell/PSCell and USS set(s) for DCI format 0_2,1_2 configured on sSCell for CCS from sSCell to PCell/PSCell if UE supports FG 11-1 (dci-Format1-2And0-2-r16) - sSCell USS set(s) (for CCS from sSCell to Pcell/PSCell) and Type0/0A/1/2 CSS sets on Pcell/PSCell can be configured so that the UE monitors them in overlapping [slot/symbol] of Pcell/PSCell and sSCell - no simultaneous monitoring between ‘USS sets (for P(S)Cell scheduling) on sSCell’ and ‘Type 0/0A/1/2/CSS sets on P(S)Cell for DCI formats with CRC scrambled by C-RNTI/MCS-C-RNTI/CS-RNTI’ - simultaneous monitoring of ‘USS sets (for P(S)Cell scheduling) on sSCell’ and ‘Type 0/0A/1/2/CSS sets on P(S)Cell for DCI formats with CRC not scrambled by C-RNTI/MCS-C-RNTI/CS-RNTI’ - FFS: Support of monitoring DCI formats 0_1,1_1,0_2 (if supported),1_2 (if supported) on PCell/PSCell USS set(s) - PDCCH monitoring occasion(s) - FFS: frame boundary alignment between PCell/PSCell and sSCell | 6-5 | Yes | N/A | Cross-carrier scheduling from SCell to PCell/PSCell with search space restrictions (Type A) is not supported | Per BC | No | Applicable to FR1 only | No | Candidate value set: One or more of supported SCS combinations ({P(S)Cell SCS in kHz, sSCell SCS in kHz}) from following set are indicated by the UE: {15,15}, {15,30}, (15, 60), [{30,30}, {30,60},{60,60}]) [Candidate value set 2: frequency band pair(s) for {PCell/PSCell, sSCell}] Component 4 candidate values: (K1, K2) = {(1,1) for FDD P(S)Cell; (K1, K2) = (1,2) for TDD P(S)Cell, [(K1, K2) = (2,2) for FDD P(S)Cell; (K1, K2) = (2,4) for TDD P(S)Cell]} Component 9 candidate values: [Value 1: PDCCH monitoring occasion(s) on PCell/PSCell and on sSCell for cross-carrier scheduling to PCell/PSCell is within the first 3 OFDM symbols of a PCell/PSCell slot. Value 2: PDCCH monitoring occasion(s) on PCell/PSCell and on sSCell for cross-carrier scheduling to PCell/PSCell is not restricted to the first 3 OFDM symbols of a PCell/PSCell slot] Note: The CCS from sSCell to PCell is applicable to FR1 only but there can be other SCells in FR2 configured for the UE Note: The SCell configured with Cross-carrier scheduling to PCell/PSCell is referred to as ‘sSCell’ | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 34. NR_DSS | 34-2 | Cross-carrier scheduling from SCell to PCell/PSCell (Type B) | Support of Cross-carrier scheduling (CCS) from sSCell to PCell/PSCell (Type B) - Cross-carrier scheduling from sSCell to PCell/PSCell with CIF - sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and search space sets on PCell/PSCell can be configured so that the UE monitors them in overlapping [slot/symbol] of PCell/PSCell and sSCell - Configuration of scaling factor α for BD and CCE limit handling and PDCCH overbooking handling on P(S)Cell - The number of unicast DCI limits for PCell/PSCell scheduling - Processing K1 unicast DCI scheduling DL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - Processing K2 unicast DCI scheduling UL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - FFS: N is based on pair of (PCell/PSCell SCS, sSCell SCS): N=1 for(15,15), (30,30), (60,60) and N=2 for (15,30), (30,60) and N=4 for (15, 60) - Same numerology between sSCell and P(S)Cell or sSCell SCS is larger than P(S)Cell SCS - USS set(s) for DCI format 0_1,1_1 configured on sSCell for CCS from sSCell to PCell/PSCell and USS set(s) for DCI format 0_2,1_2 configured on sSCell for CCS from sSCell to PCell/PSCell if UE supports FG 11-1 (dci-Format1-2And0-2-r16) - PDCCH monitoring occasion(s) - FFS: frame boundary alignment between PCell/PSCell and sSCell Note: The SCell configured with Cross-carrier scheduling to PCell/PSCell is referred to as ‘sSCell’ | 6-5 | Yes | N/A | Cross-carrier scheduling from SCell to PCell/PSCell (Type B) is not supported | Per BC | No | Applicable to FR1 only | No | Candidate value set: One or more of supported SCS combinations ({P(S)Cell SCS in kHz, sSCell SCS in kHz}) from following set are indicated by the UE: {15,15}, {15,30}, (15, 60), [{30,30}, {30,60},{60,60})] [Candidate value set 2: frequency band pair(s) for {PCell/PSCell, sSCell}] Component 4 candidate values: (K1, K2) = {(1,1) for FDD P(S)Cell; (K1, K2) = (1,2) for TDD P(S)Cell} Component 7 candidate values: [Value 1: PDCCH monitoring occasion(s) on PCell/PSCell and on sSCell for cross-carrier scheduling to PCell/PSCell is within the first 3 OFDM symbols of a PCell/PSCell slot. Value 2: PDCCH monitoring occasion(s) on PCell/PSCell and on sSCell for cross-carrier scheduling to PCell/PSCell is not restricted to the first 3 OFDM symbols of a PCell/PSCell slot] Note: The CCS from sSCell to Pcell is applicable to FR1 only but there can be other Scells in FR2 configured for the UE Note: The SCell configured with Cross-carrier scheduling to PCell/PSCell is referred to as ‘sSCell’ | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG34-3 (new): Non-aligned frame boundaries between PCell/PSCell and sSCell

| 34. NR_DSS | 34-3 | Non-aligned frame boundaries between PCell/PSCell and sSCell | CA with non-aligned frame boundaries for PCell/PSCell and sSCell in inter-band CA - Candidate value set 1: One or more of supported SCS combinations ({P(S)Cell SCS in kHz, sSCell SCS in kHz}) from following set are indicated by the UE: {15,15}, {15,30}, (15, 60), [{30,30}, {30,60}, {60,60})] - Candidate value set 2: frequency band pair(s) for {PCell/PSCell, sSCell} | 34-1 or 34-2 |  |  | Non-aligned frame boundaries between PCell/PSCell and sSCell is not supported | Per BC | No | Applicable to FR1 only |  | Indicates whether the UE supports carrier aggregation operation where the frame boundaries of the PCell/PSCell and the sSCell are not aligned, while the slot boundaries are. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG34-4 (new): disabling scaling factor a when sSCell is deactivated

| 34. NR_DSS | 34-4 | Disabling the scaling factor a when sSCell is deactivated | For a UE configured for CCS from sSCell to PCell/PSCell, scaling factor a is not applied for PDCCH overbooking/BD/CCE limit computation when sSCell is deactivated; otherwise scaling factor a is applied | 34-1 or 34-2 |  |  | Scaling factor a is applied even when sSCell is deactivated | Per BC | No | Applicable to FR1 only |  | - Timing for disabling scaling factor α when sSCell is deactivated follows sSCell deactivation timing in current specifications, i.e., no later than the minimum requirement defined in TS 38.133 as captured in 38.213 subclause 4.3 - Note: It is up to UE implementation whether/when to apply the scaling factor α during sSCell activation | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG34-5 (new): disabling scaling factor a when sSCell is switched to dormant BWP

| 34. NR_DSS | 34-5 | Disabling the scaling factor a when sSCell is switched to dormant BWP | For a UE configured for CCS from sSCell to PCell/PSCell, scaling factor a is not applied for PDCCH overbooking/BD/CCE limit computation when sSCell is switched to dormant BWP; otherwise scaling factor a is applied | 34-1 or 34-2 |  |  | Scaling factor a is applied even when sSCell is swiched to dormant BWP | Per BC | No | Applicable to FR1 only |  | - Timing for disabling scaling factor α follows the non-dormant to dormant BWP switching delay in current specifications (TS 38.133) - Note: It is up to UE implementation whether/when to apply the scaling factor α during sSCell BWP switch | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 35. LTE_NR_DC_enh2 | 35-1 | Aperiodic CSI-RS for tracking for fast SCell activation | - Aperiodic CSI-RS for tracking for fast SCell activation is triggered by enhanced SCell activation/deactivation MAC CE - Aperiodic CSI-RS configurations for tracking for fast SCell activation is triggered within the BWP indicated by firstActiveDownlinkBWP-Id for the sSCell - Maximum number of aperiodic CSI-RS resource set configurations for tracking for fast SCell activation that can be configured to UE per CC in a reported band - FFS: Maximum number of aperiodic CSI-RS resource sets for tracking for fast SCell activation that can be configured to UE across CCs in a reported band - FFS: Maximum number of aperiodic CSI-RS for tracking for fast SCell activation by a MAC-CE | 6-5 | Yes | N/A | Aperiodic CSI-RS for tracking for fast SCell activation is not supported | [Per UE/Per BC/Per band] | [No/Yes] N/A | [No/Yes] N/A | N/A | Component 3 candidate values: FFS {[1,] 8,16,32,48,64,128,255} Up to RAN2 Component 4 candidate values: FFS {[1,] 8,16,32,64,128,256,512,1024} Up to RAN2 Note: In component 3, the candidate component values {FFS} do not apply to FR2 Note: In component 4, the candidate component values {FFS} do not apply to FR2 Note: component 3 and 4 candidate values refer to the number of RS configurations for fast SCell activation that can be indicated by the MAC CE Component 5 candidate values: FFS The NZP-CSI-RS configured as temporary RS for tracking for fast SCell activation are not considered when counting the maximum NZP-CSI-RS configurations of FG2-33 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG35-2 (new): supported aperiodic CSI-RS bandwidth for tracking for fast SCell activation for 10MHz UE channel bandwidth

| 35. LTE_NR_DC_enh2 | 35-2 | aperiodic CSI-RS bandwidth for tracking for fast SCell activation for 10MHz UE channel bandwidth | Indicates the UE supported TRS bandwidths for fast SCell activation, in addition to 52 RBs, for a 10MHz UE channel bandwidth. This only applies for the BWPs configured with 52 RBs size and 15kHz SCS, in FDD bands. - Candidate values of Set 1: 28, 32, 36, 40, 44, 48 RBs - Candidate values of Set 2: 32, 36, 40, 44, 48 RBs | 35-1 |  |  |  | Per band | FDD only | FR1 only | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |