3GPP TSG RAN WG1 #109-emeeting	     					   R1-2205012

e-Meeting, May. 9th  – May. 20th, 2022

Agenda item:	8.16.17

Source: 	Qualcomm Incorporated

Title: 	UE features for Rel-17 UL Tx switching

Document for:	Discussion/Decision

# Introduction

RAN plenary approved the work item R17 NR FR1 which includes the R17 UL Tx switching [1]. In this paper, we discussed the UE capabilities and express our views.

# New UE capability - uplinkTxSwitching-OptionSupport17

In the RAN1#108-emeeting, the proposals were discussed in email thread [108-e-R17-UE-features-Others-01]. Two proposals from different proponents were summarized by FL [5] as below and the difference was highlighted.

|  | Proposal #1 |  | Proposal #2 |  |
| --- | --- | --- | --- | --- |
| RAN2 FG | FG 22-1 (Rel-16) | Rel-17 2Tx-2Tx | FG 22-1 (Rel-16) | Rel-17 2Tx-2Tx |
| Support | Option 1 | Option 1 | Option 1 | Option 1 |
| Support | Option 1 and 2 | Option 1 | Option 1 and 2 | Option 1 |
| Support | Option 2 | Option 2 without new per-BC FG | Option 2 | Option 2 with new per-BC FG |
| Support | Option 1 and 2 | Option 1 and 2 with new per-BC FG | Option 1 and 2 | Option 1 and 2 with new per-BC FG |

The consensus is to define a new capability for inter band CA Option 2. The difference between Proposal #1 and #2 are how a Rel-17 Option 2 only UE to report its capability. Proposal #1 suggests to use Rel-16 Option 2 capability together with the Rel-17 2Tx-2Tx switching capability, while Proposal #2 suggests directly report the new capability.

The proponent of Proposal #1 claims Proposal #1 would save the newly defined capability bit by report the legacy Rel-16 Option 2 capability for a Rel-17 Option 2 only UE. However, we can’t agree with this. Based on our understanding, as this new capability is to be defined as an optional capaiblity, this bit would be consumed regardless for a band combination where the UE supports 2Tx-2Tx.

Therefore, we support Proposal # 2 above and propose to adopt the following proposal while the highlighted capability could be revisited by RAN2.

- Specify a new per-BC FG for Rel-17 2Tx-2Tx UL Tx switching CA Option 2

  - UE supports this new FG needs to support Rel-16 CA Option 2

- UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] and indicating “option 1” or “both option 1 and option 2” support for Rel-16 1Tx-2Tx UL Tx switching via FG 22-1 supports Rel-17 2Tx-2Tx UL Tx switching Option 1 as well.

- UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] supports Rel-17 2Tx-2Tx UL Tx switching Option 2, if the UE indicates either one of the following,

  - Indicating “option 2” via the new per-BC FG

  - Indicating “both option 1 and option 2” via FG 22-1 and the new per-BC FG (i.e. support “option 2” for Rel-17 2Tx-2Tx UL Tx switching when “both option 1 and option 2” is indicated via FG 22-1)

Proposal: Agree on the following proposal for Rel-17 UE capability.

- Specify a new per-BC FG for Rel-17 2Tx-2Tx UL Tx switching CA Option 2

  - UE supports this new FG needs to support Rel-16 CA Option 2

- UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] and indicating “option 1” or “both option 1 and option 2” support for Rel-16 1Tx-2Tx UL Tx switching via FG 22-1 supports Rel-17 2Tx-2Tx UL Tx switching Option 1 as well.

- UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] supports Rel-17 2Tx-2Tx UL Tx switching Option 2, if the UE indicates either one of the following,

  - Indicating “option 2” via the new per-BC FG

  - Indicating “both option 1 and option 2” via FG 22-1 and the new per-BC FG (i.e. support “option 2” for Rel-17 2Tx-2Tx UL Tx switching when “both option 1 and option 2” is indicated via FG 22-1)

# Conclusions

For Rel-17 UL Tx switching UE capability, we have the following proposal:

Proposal: Agree on the following proposal for Rel-17 UE capability.

- Specify a new per-BC FG for Rel-17 2Tx-2Tx UL Tx switching CA Option 2

  - UE supports this new FG needs to support Rel-16 CA Option 2

- UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] and indicating “option 1” or “both option 1 and option 2” support for Rel-16 1Tx-2Tx UL Tx switching via FG 22-1 supports Rel-17 2Tx-2Tx UL Tx switching Option 1 as well.

- UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] supports Rel-17 2Tx-2Tx UL Tx switching Option 2, if the UE indicates either one of the following,

  - Indicating “option 2” via the new per-BC FG

  - Indicating “both option 1 and option 2” via FG 22-1 and the new per-BC FG (i.e. support “option 2” for Rel-17 2Tx-2Tx UL Tx switching when “both option 1 and option 2” is indicated via FG 22-1)

# Reference

- RP-210899, revised WID: RF requirements enhancement for NR frequency range 1 (FR1), Mar. 2021

- R2-2109226, CR to TS 38.306 Introduction of Rel-17 Tx switching enhancements, Aug. 2021

- R1-2109720, Summary on other UE feature related discussions	Moderator (NTT DOCOMO, INC.)

- R1-2112147, Summary on other UE feature related discussions	Moderator (NTT DOCOMO, INC.)

- R1-2201522, Summary on other UE feature related discussions	Moderator (NTT DOCOMO, INC.)