3GPP TSG-RAN WG1 Meeting #109-e	R1-2205015

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.1.1.2

Source: 	Qualcomm Incorporated

Title: 	Supporting two TAs for multi-DCI based mTRP

Document for:	Discussion/Decision

# Introduction

The Rel-18 WID on NR MIMO evolution for downlink and uplink is approved [1], which includes the following objective:

- Study, and if justified, specify the following

  - Two TAs for UL multi-DCI for multi-TRP operation

  - …

For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

In Rel-16/17 mTRP, there can be only one single TA per CC, and it is further assumed at the received signals from different TRPs are withing the CP at the UE side. Such assumption limits the deployment scenario in FR2 given the small CP duration. In this contribution, we discuss framework and possible enhancements to enable two TAs in a CC that is configured with multi-DCI based mTRP operation.

# Discussions

The need for different TA’s for UL transmissions toward different TRPs can be motivated by two reasons:

- Different DL timing at the two TRPs due to imperfect time synchronization

- Different propagation delays from the two TRPs to a given UE

The former alone may not be enough to justify the need for multiple TAs in a CC as cell phase synchronization accuracy is within 3µsec as specified in RAN4 spec. However, even with perfect DL time synchronization across the two TRPs, different propagation delays to the UE (which can be due to different distance and/or different LOS/NLOS conditions) may necessitate multi-TA enhancements as illustrated in Figure 1.

Figure 1: Different TA’s due to different propagation delays.

For UL-CA, multiple TAGs in different UL CCs have been supported since Rel-15, where each serving cell can be configured with a TAG-ID, and MAC-CE can indicate a TA command for a given TAG-ID as shown below:

Figure 2: Legacy UL-CA framework for multiple TAs.

Hence, we think the same framework can be directly reused for introducing two TAs in a CC that is configured with multi-DCI based mTRP, where the two TAs are associated with the two coresetPoolIndex values, and association between UL channels/signals and TAGs can be configured directly or determined based on coresetPoolIndex value. For dynamic UL channels/signals such association already exists (based on the scheduling DCI). For periodic/semi-persistent UL channels/signals such association can be defined by RRC configuration.

Proposal 1: A CC configured with multi-DCI based mTRP can be configured with two TAG IDs. Existing UL-CA framework is used to indicate TA values associated with different TAG IDs.

- The two TAG IDs are associated with the two coresetPoolIndex values.

- Association between UL channels/signals and the two TAG IDs is determined based on coresetPoolIndex value:

  - For dynamic UL channels/signals: Reuse Rel-16 rule for associated coresetPoolIndex value

  - For periodic/semi-persistent UL channels/signals: RRC configures the associated coresetPoolIndex value

Another basic aspect that needs to be addressed is the maximum difference between the two TA values in a CC. Similar to the discussion above, we propose to reuse the UL-CA. In RAN4 specifications (38.133, Section 7.5.4, the maximum uplink transmission timing difference for inter-band NR CA (between different TAGs) is specified in as shown in the table below:

As it can be seen, within a given FR, the maximum difference is 34.6 µsec in FR1 and 8.5 µsec in FR2. The same values should be reused for maximum uplink transmission timing difference between two TAs in a given CC depending on the frequency range of the CC:

Proposal 2: The maximum uplink transmission timing difference between two TAs in a given CC is the same as inter-band NR CA, which depends on the frequency range of the CC.

As discussed above, the TA command in MAC-CE can be indicated per TAG ID, which is used for the relative (accumulative) TA value. In legacy systems for connected mode UEs, PDCCH order can be used to trigger contention-free random access to acquire UL timing when it goes out-of-sync. The PDCCH order for this purpose can be used either for the PCell (for pTAG) or SCell (for sTAG). In either case, UE transmits the PRACH in the cell for which the UL timing re-sync is required in response to the PDCCH order, and then RAR PDCCH/PDSCH is received on the PCell. The RAR PDSCH indicates the absolute TA value (12 bits) for the cell in which the PRACH is transmitted on. This is illustrated in Figure 3.

Figure 3: CFRA triggered by PDCCH order to acquire UL timing for PCell / SCell.

In the case of two TAs in a CC (either PCell or SCell), some further study may be needed with respect to impact to the PDCCH order for acquiring UL timing for the two TRPs. In particular, UE needs to know whether the indicated TA command in RAR PDSCH corresponds to the first TA or the second TA in the CC.

Proposal 3: Study the impact of two TAs per CC to CFRA triggered by PDCCH order for acquiring the UL timing for each of the two TAs.

Another issue is related to overlaps between two UL channels / signals due to different TAs in a same CC. For example, if UL channel 1 and UL channel 2 do not overlaps in logical time (w/o considering TA) but they overlap in actual time due to different TA values, the following possibilities should be further studied:

- If UE can transmit the two UL channels simultaneously in the CC, then there is no problem to be solved. This should be at least one of possible modes of operation as a UE transmitting UL channels with different TAs is likely using different Tx chains / PAs.

- If UE cannot transmit the two UL channels simultaneously in the CC:

  - First, it should be discussed if such a mode of operation should be supported for multi-TA case (in one CC).

  - If yes, it should be discussed whether the issue can be avoided by network implementation. In theory, at least for dynamic signals/channels, if gNB knows the two TA values applied by the UE perfectly, the gNB scheduler can take care of this and avoid overlap. However, in practice, gNB does not know the exact TA values applied at the UE and/or may not be able to avoid the issue for periodic UL signals/channels.

  - Otherwise (if the issue cannot be avoided by network implementation), one of the overlapping UL channels should be dropped or partially transmitted at the UE side.

Note that the issue above exists also in legacy but only across different slots and only due to a new TA command. The issue is addressed by reducing the duration of a slot after applying a TA command. This is captured in 38.213 as “If two adjacent slots overlap due to a TA command, the latter slot is reduced in duration relative to the former slot”.

In comparison, in the case of multi-TA in one CC:

- UE may be able to transmit two different UL signals/channels in a CC simultaneously,

- The overlap is not only across different slots but can be also within a given slot,

- The overlap is not only occasionally due to a new TA command but can be also frequent due to the fact that different UL signals/channels are associated with different TAs.

Hence, further study on aspects discussed above is needed in the case of two TAs in one CC.

Proposal 4: Study whether/how to address the case when two different UL signals/channels overlap in actual time (but do not overlap in logical time) due to two TAs in a same CC.

Finally, some consistency for TA configurations is needed across different CCs. Note that in UL-CA, we can have up to 4 TAGs. Among multiple CCs, some may be configured with two coresetPoolIndex values (with multi-DCI based mTRP) while others may be configured with a single value of coresetPoolIndex. For example, for the purpose of HARQ-Ack (separate and joint), there is relationship between coresetPoolIndex value(s) of different CCs for the purpose of HARQ-Ack reporting and codebook construction. In the case of multi-TA in a CC, TAGs configured for different CCs need to be consistent across coresetPoolIndex values of different CCs. Figure 4 illustrates a few examples for consistent and inconsistent configurations, where it is assumed that UE can support up to 4 TAGs overall.  As it can be seen, the top-right and bottom-right examples are not consistent as a same TAG is used in the two CCs but are associated with different coresetPoolIndex values across the two CCs.

Figure 4: Consistency of TA configurations across different CCs.

Proposal 5: Study necessary conditions to ensure consistency of TA configurations across different CCs. Use case for each configuration needs to be justified.

# Conclusion

In this contribution, we proposed:

Proposal 1: A CC configured with multi-DCI based mTRP can be configured with two TAG IDs. Existing UL-CA framework is used to indicate TA values associated with different TAG IDs.

- The two TAG IDs are associated with the two coresetPoolIndex values.

- Association between UL channels/signals and the two TAG IDs is determined based on coresetPoolIndex value:

  - For dynamic UL channels/signals: Reuse Rel-16 rule for associated coresetPoolIndex value

  - For periodic/semi-persistent UL channels/signals: RRC configures the associated coresetPoolIndex value

Proposal 2: The maximum uplink transmission timing difference between two TAs in a given CC is the same as inter-band NR CA, which depends on the frequency range of the CC.

Proposal 3: Study the impact of two TAs per CC to CFRA triggered by PDCCH order for acquiring the UL timing for each of the two TAs.

Proposal 4: Study whether/how to address the case when two different UL signals/channels overlap in actual time (but do not overlap in logical time) due to two TAs in a same CC.

Proposal 5: Study necessary conditions to ensure consistency of TA configurations across different CCs. Use case for each configuration needs to be justified.

- References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung.