3GPP TSG RAN WG1 #109-e			    R1-2205020

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.1.4.2

Source: 	Qualcomm Incorporated

Title: 	Enhancements for 8 Tx UL transmissions

Document for:	Discussion/Decision

# Introduction

In RAN plenary #94e, the work item on MIMO Evolution for Downlink and Uplink was approved [1]. One of the main objectives of the work item is enabling 8 Tx UL transmission, which is listed as below

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

  - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In this contribution, the schemes to support enhancement for 8 Tx UL transmissions are discussed.

# Evaluation methodology for 8 Tx UL transmission

Introducing necessary enhancements to support for 8 antenna ports for UL transmission can offer improvement for UL coverage and average throughput. To investigate the performance gains, system level simulations (SLS) should be conducted, as it studies the performance of a scheme at a system level including multiple cells/UEs with link adaptation, power control, etc.

Before conducting the SLS study, one important prerequisite is settling down the assumptions for the antenna layout/model for 8 Tx UE, as the SLS results would heavily depends on the UE antenna layout. Unfortunately, UE’s antenna layout heavily depends on the UE type/size/form factor. For example, Tx antenna layout on a CPE device could be totally different from antenna layout on a vehicular. Given that the objective in WID [1],  CPE, FWA, vehicle, and industrial devices are all included. Among those UE, it is desired to prioritize the study of 8 Tx for CPE/FWA devices because of their relatively important use cases, mature market demand, and feasibility to support straightforward antenna layout.

The typical use case of CPE/FWA is utilizing cellular data (4G/LTE and/or 5G) connection to provide broadband internet to a home or office as shown in Fig 1, without the need for cable connection. This use case has been widely acknowledged by operators and NW/UE device companies in the area of cellular communications.

For CPE/FWA devices, the size could vary but is normally within the range of 10 to 20 centimetres in length, width, and depth. Therefore, to model the antenna layout of 8 Tx CPE/FWA, we can assume the antenna elements are regularly deployed on one surface of the CPE product, following conventional uniform linear array (ULA) or the uniform planar antenna array (UPA). In this way, cross-polarized antenna elements are uniformly spaced in the horizontal direction and vertical direction with a spacing of 0.5 λ, such as (M,N,P)=(1,4,2) or (M,N,P)=(2,2,2). These assumed antenna layout on CPE/FWA follows similar structure as gNB antenna arrays, while with smaller number of antennas on CPE/FWA.

For CPE/FWA, low mobility can be assumed to model the channel between the CPE/FWA terminal and the base station, as those devices typically are stationary once installed. A very low mobility, e.g., 3km/h, can be assumed to model the environment change around the devices.

For other devices such as vehicular and industrial devices, it is not clear that what are the typical use cases. Furthermore, it is not clear what should be the typical assumptions for antenna layout/structure on those devices. With these uncertainty in mind, it is reasonable to deprioritize the 8-Tx study on those devices.

Fig 1: Examples of two CPE/FWAs with 8-Tx typical antenna layout

Based on the above analysis, we have the following proposals.

Proposal 1: Study the performance of 8 Tx PUSCH via SLS. Prioritize 8Tx CPE/FWA devices in this study.

Proposal 2: Adopt the following SLS assumptions as a starting point to study the performance of 8 Tx PUSCH.

| Parameter | Value |
| --- | --- |
| Metric | UL throughput |
| Baseline | Rel-16 UL NCB and CB based PUSCH with 4Tx |
| Carrier frequency | 3.5GHz |
| SCS | 30KHz |
| System bandwidth | 20MHz/40MHz/100MHz |
| UE antennas layout | (M, N, P) = (1,4,2) for ULA; (M, N, P) = (2,2,2) for UPA, (dH,dV) = (0.5, 0.5)λ |
| UE speed | 3km/h |
| Scenario | Umi/Uma |

# Scope for 8 Tx PUSCH

The work of RAN 1 standardization on 8 Tx is very heavy, because of the broad initial scope of this objective defined in the Rel-18 MIMO WID [1]. In our view, the required large amount of work is due to the following two aspects.

- New codebook design is needed for coherent 8 Tx. However, the codebook design heavily depends on the antenna layout of the coherent 8 Tx, which can vary significantly based on devices type, size, form factor. With large uncertainty on device antenna layout, the new codebook design for coherent 8 Tx precoders is very challenging. On the other hand, supporting non-coherent and partial coherent 8 Tx transmissions can be very straightforward, via reusing and concatenating the existing Rel-15 2 Tx or 4 Tx PUSCH precoders, as shown in Section 4.1 in detail.

- Supporting 2 CWs with more than 4 layers PUSCH has large specification impact (as shown in Section 6 in detail), which would require a large amount of standard effort and TU. Furthermore, based on system level studies, more than 4 layers do not provide significant gain over 4 layers. Fig 2 and Table 1 show the UL throughput comparison between 8 Tx with up to 4 layers and 8 Tx with up to 8 layers. The x-axis of Fig 2 is the median Iot (interference over thermal) measured at gNB, which reflects the UE UL Tx power. It is observed that with increased Iot, the average UE throughput firstly increases due to the increase of actual UE transmit power, then decreases due to the increase of the inter-cell interference. Nevertheless, across the Iot range simulated, only marginal gain can be achieved with more than 4 layers.

Table 1: Normalized Average TPUT

| Normalized Average TPUT (w.r.t up to 4 layers) with power control Alpha 0.9 and NCB PUSCH |  |  |  |
| --- | --- | --- | --- |
|  | Median IoT: 8 dB | Median IoT: 11 dB | Median IoT: 14 dB |
| Maximal rank 4 (Baseline) | 1x | 1x | 1x |
| Maximal rank 8 | +1.0% | +2.2% | +3.0% |

Fig 2: Throughput comparison between maximal rank 4 and maximal rank 8

Table 2: Simulation assumptions for the above SLS result

| Parameter | Value |
| --- | --- |
| Scenario | Uma |
| Carrier frequency | 3.5 GHz |
| Simulation BW | 25MHz |
| gNB antenna array | 64 TxRUs, (M, N, P) = (4, 8, 2), dH = 0.5λ, dV = 0.8λ, ±45° x-pol |
| UE max Tx power | 23 dBm |
| Number of UE antennas | (M, N, P) = (1,4,2) for ULA, (dH,dV) = (0.5, -)λ, cross-polarized (0/+90 deg) |
| UE distribution | 20% UEs are outdoor and 80% UEs are indoor |
| UE speed | indoor (3km/h) and outdoor (30km/h) |
| Traffic Models | Full buffer |
| Modulation | Up to 256-QAM |
| UL precoder | Non-codebook based PUSCH |

Based on the above analysis, for 8 Tx codebook design, it is desired to prioritize the specification for partial coherent and non-coherent 8 Tx precoders, to secure a baseline support for 8 Tx transmission in Rel-18. With partial coherent and non-coherent 8 Tx precoders, at least the objective in the WID is achieved to support 8 Tx in Rel-18. The design of 8 Tx coherent precoders may optimize the performance of 8 Tx, which can be viewed as second stage objective. Of course, RAN1 should still aim an ideal outcome and strive to complete the second stage objective in Rel-18.

Furthermore, it is desired to prioritize the specification for 8 Tx transmission with single CW and up to 4 layers. Similarly, supporting two CWs with more than 4 layers can be reviewed as second stage objective, which RAN1 should still strive to complete as an ideal outcome.

With the above rationale, to secure a baseline specification to support 8 Tx transmission in Rel-18, the following proposals are made.

Proposal 3: Rel-18 prioritize the specification for 8 Tx PUSCH with up to 4 layers.

Proposal 4: For codebook based 8 Tx PUSCH in Rel-18, prioritize the specification for non-coherent and partial coherent 8 Tx precoders.

Last but not least, for 8 Tx PUSCH transmissions, codebook and non-codebook based PUSCH are of similar importance. Therefore, both should be supported, just as Rel-15 supports 4 Tx PUSCH for both codebook and non-codebook based.

Proposal 5: Rel-18 support both codebook and non-codebook based 8 Tx PUSCH.

# 8 Tx codebook based PUSCH

For 8 Tx codebook based PUSCH, the key question is designing the 8 Tx precoders codebook. For the 8 Tx precoders, there are three cases need to be studied as below, similar as 4 Tx precoders,

- Non coherent 8 Tx precoders

- Partial coherent 8 Tx precoders

- Full coherent 8 Tx precoders

## Partial coherent and non-coherent precoders with 8 Tx

On CPE/FWA implementation, the easiest way to build 8 Tx might be installing multiple of the existing 2 Tx or 4 Tx antenna modules on a CPE/FWA. The multiple antenna modules can be installed on a same or a few different surfaces of the CPE/FWA devices. Across different antenna modules, it is usually very challenging, if not impossible, to keep coherence. Within a same antenna module, depends on that the antenna module itself consists of coherent or noncoherent antennas, those antennas can be coherent or noncoherent.

Two examples of above-mentioned implementation are provided in Fig 3. In the first example (in left sub-figure), a CPE/FWA is equipped with 2 antenna modules. Antenna module 1 consists of 4 coherent antennas. Antenna module 2 also consists of 4 coherent antennas. The two modules are noncoherent due to hardware limitations, as they are two separated modules. For this CPE/FWA with partial coherent 8 Tx antennas, the most straightforward way to build 8 Tx precoder is selecting a precoder A for antenna module 1 from Rel-15 4 Tx codebook, and separately selecting a precoder B for antenna module 2 from Rel-15 4 Tx codebook. By concatenating two Rel-15 4 Tx precoders, an 8 Tx precoder with a block diagonal structure can be formed, as shown in left sub-figure of Fig 3. Similarly, if a CPE/FWA is equipped with 4 antenna modules, where each consists of 2 coherent antennas, a block diagonal 8 Tx precoder can be built by concatenating four of Rel-15 2 Tx precoders.

Of course, under this umbrella of building 8 Tx precoders via diagonally concatenating Rel-15 2 Tx and 4 Tx precoders, a more complicated cases such as mixed concatenating 2 Tx and 4 Tx precoders can be considered. However, for the progress of this work item, it is recommended to prioritize the study of the following two example scenarios as a starting point.

Fig 3: Examples of partial coherent 8 Tx precoders based on Rel-15 4 Tx or 2 Tx precoders

With the above analysis, the following proposal is made.

Proposal 6: NR Rel-18 reuse and concatenate existing Rel-15 2 Tx and/or 4 Tx PUSCH precoders to support 8 Tx PUSCH precoders with partial coherent or noncoherent 8 Tx.

- Prioritize the specification of the following two cases.

  - Concatenate two 4 Tx precoders to build an 8 Tx precoder.

  - Concatenate four 2 Tx precoders to build an 8 Tx precoder.

- FFS details on signalling to reuse and concatenate existing Rel-15 precoders.

## Full coherent precoders with 8 Tx

For full coherent precoders with 8 full coherent Tx, the simple concatenation approach as in 4.1 cannot be used. To fully explore the potential gain of 8 coherent Tx, a new precoder codebook is desired.

In general, there are at least two principles RAN1 can follow to design the new 8 Tx coherent precoders.

The first principle is designing the 8 Tx coherent precoder based on DFT matrix, assuming the 8 Tx antenna layout follows the regular structure of ULA or UPA as shown in Fig 1. Following this principle, one example of 8 Tx precoder can be similar to DL type-I 8 Tx codebook, which is based on DFT codebook with oversampling factor 2 and a co-phasing factor φ.

The second principle is designing the 8 Tx coherent precoder based on Householder matrices, assuming the channels are spatially uncorrelated.  A similar co-phasing factor φ can be introduced with the cross polarized antennas.

Proposal 7: As a starting point, Rel-18 study the new precoder codebook for PUSCH with fully coherent 8 Tx based on DFT matrix or Householder matrix.

# 8 Tx non-codebook based PUSCH

For non-codebook based PUSCH, the enhancement for 8 Tx is relatively simple, as there is no precoder codebook design involved. The only enhancement identified identified is on the enhancement of SRI signalling to indicate the up to 8 SRS ports to transmit PUSCH.

The necessary SRI enhancements are heavily dependent on the adopted SRS enhancements for 8 Tx, which are discussed in detail in an accompanied contribution [2].  In below, a summary of SRS enhancement for 8 Tx is provided.

If 8 SRS ports are sounded via a single SRS resource set, as illustrated in Fig 4. The SRI enhancement is simply expanding the bit width the SRI field in DCI to indicate the total number of combinations of X ports, which is:X=18X8=255, where X8 denotes the number of combinations of choose X ports out of 8 ports.

Fig 4: 8 SRS ports are sounded via a single SRS resource for NCB based PUSCH

If 8 SRS ports are sounded via multiple SRS resource sets, as illustrated in Fig 5. Multiple SRI fields, e.g., 2, might be needed in the DCI scheduling 8 Tx NCB PUSCH. The details of bit-width of SRI field for each SRS resource set can be further studied.

Fig 5: 8 SRS ports are sounded via two SRS resource sets for NCB based PUSCH

With the above discussion, the following proposal is made for non-codebook based PUSCH with 8 Tx.

Proposal 8: Rel-18 specify SRS and SRI enhancement to support non-codebook based PUSCH with 8 Tx.

- FFS details on SRI enhancement for 8 SRS ports sounding via a single SRS resource set.

- FFS details on SRI enhancement for 8 SRS ports sounding via multiple SRS resource set, each sounding less than 8 ports.

# Specification enhancement for more than 4 layers PUSCH

With more than 4 layers 8 Tx PUSCH transmission, similar to PDSCH, it is naturally to support 2 codeword (CW) PUSCH transmissions. With 2 CWs PUSCH transmission, it opens up many new issues for study in Rel-18, which are discussed in this section.

The first issue is CW to layer mapping. It is natural to follow the similar mapping as in PDSCH with 2 CW, i.e., the first CW maps to layer 0-3, and the second CW maps to layer 4-7.

Proposal 9: For 2 CWs PUSCH with 8 layers in Rel-18, reuse Rel-15 2 CWs PDSCH CW to layer mapping procedure.

The second issue is on what are the necessary UL HARQ enhancement to support 2 CW transmission and retransmission, which could include the following aspects.

- NDI, RV, MCS signaling for the second CW

- CBG based PUSCH with 2 CWs

- Dynamic switch between 2 CW and single CW PUSCH

It is apparent that the UL grant supporting 2 CWs PUSCH needs to be expanded to include NDI/RV/MCS fields for the second CW. The signalling to enable/disable each of the two CWs, i.e., switch between single CW and 2 CWs PUSCH transmissions need to be specified as well. CBG based PUSCH specification needs to be extended to cover 2 CW PUSCH.

Based on the above analysis, the follow proposal is made for HARQ enhancement with 2 CWs PUSCH.

Proposal 10: Study, and if necessary, specify HARQ enhancement to support two codewords PUSCH with 8 Tx including at least the following aspects

- NDI, RV, MCS signaling for the second CW

- CBG based PUSCH with 2 CWs

- Dynamic switch between 2 CW and single CW PUSCH

With 2 CWs on PUSCH, another important issue needs study is the UCI multiplexing on PUSCH. With an initial scoping, the following aspects need to be address.

- Multiplex UCI only on one of the CWs or both CWs

- Whether allowing different beta offset values for the two CWs

Proposal 11: Study, if necessary, specify the UCI-multiplexing enhancement to support UCI multiplexing on two codewords PUSCH with 8 Tx including at least the following aspects

- Multiplex UCI only on one of the CWs or both CWs

- Whether allowing different beta offset values for the two CWs

In Rel-15 up to 4 layers PUSCH transmission, the DMRS ports is signaling via “antenna ports” field in the scheduling DCI. With more than 4 layers PUSCH transmission, apparently, enhancement is needed to indicate more than 4 DMRS ports.

Proposal 12: Study, if necessary, specify the signaling on DMRS port indication in DCI to support PUSCH with more than 4 layers.

# 8 Tx full power UL transmission

For non-coherent and partial-coherent 8 Tx PUSCH, the full power transmission specified in Rel-16 can be extended to support full power transmission for 8 Tx in a straightforward way with minimum spec impact.

For full power mode 0, a UE with 8 Tx, where each Tx is equipped with full power PA, can simply transmit full power with any 8 Tx precoders supported by Rel-18 specification to be standardized. Mode 1 supports full power by allowing non-coherent/partial coherent UEs to transmit with full coherent 8 Tx precoders via UE implementation transparent to specification such S-CDD. Mode 2 virtualize SRS port(s) to support full power, following a similar way as in Rel-16.

Fig 6: Examples for full power mode 0/1/2 with 8 Tx PUSCH

With the above analysis, the following proposal is made.

Proposal 13: Support reusing Rel-16 full power transmission schemes for PUSCH with 8 Tx, with necessary enhancement if identified.

- Strive to minimize spec impact.

# Conclusions

In summary, we have the following proposals for 8 Tx UL transmission for Rel-18 MIMO evolution.

Proposal 1: Study the performance of 8 Tx PUSCH via SLS. Prioritize 8Tx CPE/FWA devices in this study.

Proposal 2: Adopt the following SLS assumptions as a starting point to study the performance of 8 Tx PUSCH.

| Parameter | Value |
| --- | --- |
| Metric | UL throughput |
| Baseline | Rel-16 UL NCB and CB based PUSCH with 4Tx |
| Carrier frequency | 3.5GHz |
| SCS | 30KHz |
| System bandwidth | 20MHz/40MHz/100MHz |
| UE antennas layout | (M, N, P) = (1,4,2) for ULA; (M, N, P) = (2,2,2) for UPA, (dH,dV) = (0.5, 0.5)λ |
| UE speed | 3km/h |
| Scenario | Umi/Uma |

Proposal 3: Rel-18 prioritize the specification for 8 Tx PUSCH with up to 4 layers.

Proposal 4: For codebook based 8 Tx PUSCH in Rel-18, prioritize the specification for non-coherent and partial coherent 8 Tx precoders.

Proposal 5: Rel-18 support both codebook and non-codebook based 8 Tx PUSCH.

Proposal 6: NR Rel-18 reuse and concatenate existing Rel-15 2 Tx and/or 4 Tx PUSCH precoders to support 8 Tx PUSCH precoders with partial coherent or noncoherent 8 Tx.

- Prioritize the specification of the following two cases.

  - Concatenate two 4 Tx precoders to build an 8 Tx precoder.

  - Concatenate four 2 Tx precoders to build an 8 Tx precoder.

- FFS details on signalling to reuse and concatenate existing Rel-15 precoders.

Proposal 7: As a starting point, Rel-18 study the new precoder codebook for PUSCH with fully coherent 8 Tx based on DFT matrix or Householder matrix.

Proposal 8: Rel-18 specify SRS and SRI enhancement to support non-codebook based PUSCH with 8 Tx.

- FFS details on SRI enhancement for 8 SRS ports sounding via a single SRS resource set.

- FFS details on SRI enhancement for 8 SRS ports sounding via multiple SRS resource set, each sounding less than 8 ports.

Proposal 9: For 2 CWs PUSCH with 8 layers in Rel-18, reuse Rel-15 2 CWs PDSCH CW to layer mapping procedure.

Proposal 10: Study, and if necessary, specify HARQ enhancement to support two codewords PUSCH with 8 Tx including at least the following aspects

- NDI, RV, MCS signaling for the second CW

- CBG based PUSCH with 2 CWs

- Dynamic switch between 2 CW and single CW PUSCH

Proposal 11: Study, if necessary, specify the UCI-multiplexing enhancement to support UCI multiplexing on two codewords PUSCH with 8 Tx including at least the following aspects

- Multiplex UCI only on one of the CWs or both CWs

- Whether allowing different beta offset values for the two CWs

Proposal 12: Study, if necessary, specify the signaling on DMRS port indication in DCI to support PUSCH with more than 4 layers.

Proposal 13: Support reusing Rel-16 full power transmission schemes for PUSCH with 8 Tx, with necessary enhancement if identified.

- Strive to minimize spec impact.

# References

- 3GPP TSG RAN Meeting #94e, RP-213598 “New WID: MIMO Evolution for Downlink and Uplink”, Samsung, Electronic Meeting, Dec. 6 - 17, 2021.

- 3GPP TSG RAN WG1 #109-e, R1-2205018, “SRS enhancement for TDD CJT and 8 Tx operation”, Qualcomm, e-Meeting, May 9th – 20th, 2022