3GPP TSG RAN WG1 #109-e			R1-2205024

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.2.2.1

Source: 	Qualcomm Incorporated

Title: 	Evaluation on AI/ML for CSI feedback enhancement

Document for:		Discussion and Decision

# Introduction

At RAN #94, a new study item on artificial intelligence/machine learning for NR air interface has been approved [1] with a goal of studying the 3GPP framework for AI/ML for air-interface.  CSI feedback enhancement has been identified as one of the use cases in the initial set of use cases. The following aspects have been identified in [1] related to the evaluation methodology:

“Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set:

  - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations.

    - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed.

    - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study.

    - Need for common assumptions in dataset construction for training, validation and test for the selected use cases.

    - Consider adequate model training strategy, collaboration levels and associated implications

    - Consider agreed-upon base AI model(s) for calibration

    - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes

  - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases.

    - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline

    - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered.”

In this document, we discuss aspects related to the evaluation methodology for the CSI enhancement use case. We also present initial results on the benefits of a machine learning based approach for CSI enhancement.

# Discussion on Evaluation Methodology and KPIs

Evaluation Methodology: Dataset

Past 3GPP RAN1 studies for several features, including channel state information (CSI) feedback, were based on channel models defined in TR 38.901 [2]. The same approach can be used for evaluations in the current study on AI/ML for CSI feedback enhancement. In comparison with using field data, using a synthetic dataset derived from a pre-defined channel model and agreed assumptions has the benefit of ensuring consistency in modeling channel features and covering a wide range of scenarios. Also, for data generated using agreed 3GPP channel models, there is no need for calibration of the dataset across companies. For data from other sources, some statistics derived from the data may be reported along with the results.

Proposal 1: The channel model in TR 38.901 is adequate for the evaluation on AI/ML for CSI feedback enhancement.

Proposal 2: The dataset can be generated using TR 38.901 channel model and an agreed table of assumptions for dataset generation.

Proposal 3: If the dataset is derived based on TR 38.901 models, there is no need for calibration of the data across companies. For datasets from other sources, statistics about the data used for training the machine learning models can be reported along with the results.

Evaluation Methodology: KPIs

Consider the sub use case of channel compression and reconstruction using a cross-node machine learning approach, discussed in [3]. The performance of a machine learning based approach to CSI feedback can be characterized in two ways:

- Standalone ML model performance evaluation: Here, the evaluation is focused on the accuracy of the ML model. For example, for the sub use case of compressing the channel features on the UE-side and reconstructing them on the network-side, the performance of the ML model can be quantified using the reconstruction error between the original and reconstructed channel features. A tradeoff between the CSI feedback overhead and the reconstruction error can be considered as the key performance indicator.

- End-to-end performance evaluation: Here, the goal is to quantify the impact of the ML-based CSI feedback enhancement on the final user throughput. A tradeoff between the CSI feedback overhead and the user throughput can be considered as the key performance indicator. Such evaluation would require link-level simulations (LLS) or system-level simulations (SLS). We propose that both SU-MIMO and MU-MIMO scenarios should be evaluated to better understand the impact of the enhancement techniques in those scenarios.

Both the above methods should be considered as both could provide useful insights on the performance of CSI feedback enhancement techniques.

Proposal 4: Companies should be encouraged to share evaluation results on standalone ML model performance and end-to-end performance using LLS/SLS for ML-based CSI feedback enhancement methods.

Proposal 5: For standalone ML model performance evaluation, the KPI can be the tradeoff between the feedback overhead and the accuracy of the ML model output.

- For the sub use case of channel compression and reconstruction, the ML model accuracy can be represented by the reconstruction error metric of the channel features.

Proposal 6: For end-to-end performance, the KPI can be the tradeoff between the feedback overhead and user throughput.

Proposal 7: End-to-end performance characterization should include SU-MIMO and MU-MIMO scenarios.

Observation 1: Inference latency may be difficult to quantify as it would be implementation dependent.

# Performance results: Feedback overhead vs. reconstruction accuracy

In this section, we provide initial results on the standalone machine learning model performance for the use case of channel compression and reconstruction using a cross-node machine learning approach [3].

Figure 1and Figure 2 below show the cosine loss performance of channel compression based on ML model vs. eType2. We use the 3GPP UMI and INH-Open Office scenarios to generate channel realizations. Table 1 below captures the simulation assumptions. The compression is performed on the non-whitened channel.

Table 1: Simulation Assumptions

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 3.5 GHz |
| Bandwidth | 20 MHz |
| gNB TxRUs | 32: (M, N, P) = (4, 4, 2) |
| UE TxRUs | 4: (M, N, P) = (1, 2, 2) |
| Channel Estimation | Ideal |

Assume that the channel realization “H” has a right-singular matrix “V”. We use the strongest singular vector “v” as the input to the ML model’s encoder. The ML decoder reconstructs the vector as v. The ML model tries to maximize the dot-product between the two vectors. This is equivalent to minimizing the negative of cos(θ) between the two vectors. The cosine loss between vectors v (ideal singular vector) and v (reconstructed singular vector) is defined as follows:

Cosine Loss v, v=-Cos(θ(v, v)).

The Cosine Loss has been used as the training loss function. The output of the ML model’s encoder is quantized, and the quantized indices are sent to the decoder.

In both the plots below, the X-axis only contains the PMI segment of the overhead required for eType2, ignoring the wideband CQI/RI as well as the sub-band CQI feedback bits. The Y-axis shows the equivalent MSE in the dB domain which measures the per-RB reconstruction accuracy of the singular vectors (v vs v) ignoring the random phase rotation that does not affect precoding. The equivalent MSE is directly related to the cosine loss and defined as follows:

Equivalent MSE v, v=argminϕMSEv,eiϕv=argminϕv-veiϕ2=2-2Cos(θ(v, v)).

Figure 1: ML Based CSF vs eType2 (UMI)

For the UMI channel (above), at a MSE of -7.4 dB, ML based CSF gives an overhead reduction of 32 bits (64 vs 96 bits for eType2), or 33% reduction in overhead.

For the INH open office channel (below), at a MSE of -11.6 dB, ML based CSF gives an overhead reduction of 22 bits (16 vs 38 bits for eType2), or 58% reduction in overhead.

Figure 2: ML Based CSF vs eType2 (INH open)

Both the above plots show that:

- Overhead gains can be achieved through ML-based CSF over eType2 baseline performance, at a given accuracy.

- Magnitude of gain in overhead is larger in certain channels (INH Open) vs others (such as UMI), thereby allowing us to customize the ML-based-CSI report to the channel scenario.

Observation 2: For the same reconstruction accuracy, X-node ML-based CSI feedback requires significantly less feedback overhead compared to Rel-16 Type II baseline:

- For the UMI channel model, at a loss of -7.4 dB, ML based CSF gives a 33% overhead reduction (32 fewer bits – 64 bits for ML-based vs. 96 bits for eType2).

- For the INH open office channel model, at a loss of -11.6 dB, ML based CSF gives a 58% overhead reduction (22 fewer bits – 16 bits for ML-based vs. 38 bits for eType2).

# Conclusions

In this document, we have discussed aspects related to the evaluation methodology for the CSI enhancement use case. We also presented initial results on the benefits of a machine learning based approach for CSI enhancement. We made the following proposals and observations:

Proposal 1: The channel model in TR 38.901 is adequate for the evaluation on AI/ML for CSI feedback enhancement, including single shot and temporal compression.

Proposal 2: The dataset can be generated using TR 38.901 channel model and an agreed table of assumptions for dataset generation.

Proposal 3: If the dataset is derived based on TR 38.901 models, there is no need for calibration of the data across companies. For datasets from other sources, statistics about the data used for training the machine learning models can be reported along with the results.

Proposal 4: Companies should be encouraged to share evaluation results on standalone ML model performance and end-to-end performance using LLS/SLS for ML-based CSI feedback enhancement methods.

Proposal 5: For standalone ML model performance evaluation, the KPI can be the tradeoff between the feedback overhead and the accuracy of the ML model output.

- For the sub use case of channel compression and reconstruction, the ML model accuracy can be represented by the reconstruction error metric of the channel features.

Proposal 6: For end-to-end performance, the KPI can be the tradeoff between the feedback overhead and user throughput.

Proposal 7: End-to-end performance characterization should include SU-MIMO and MU-MIMO scenarios.

Observation 1: Inference latency may be difficult to quantify as it would be implementation dependent.

Observation 2: For the same reconstruction accuracy, X-node ML-based CSI feedback requires significantly less feedback overhead compared to Rel-16 Type II baseline:

- For the UMI channel model, at a loss of -7.4 dB, ML based CSF gives a 33% overhead reduction (32 fewer bits – 64 bits for ML-based vs. 96 bits for eType2).

- For the INH open office channel model, at a loss of -11.6 dB, ML based CSF gives a 58% overhead reduction (22 fewer bits – 16 bits for ML-based vs. 38 bits for eType2).

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, 3GPP RAN Plenary #94, Dec 2021.

- TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz,” 3GPP Release 16, Dec 2019.

- R1-2205025, “Other aspects on AI/ML for CSI feedback enhancement”, Qualcomm Incorporated, 3GPP TSG RAN WG1 #109-e, May 2022.