3GPP TSG RAN WG1 #109-e		R1- 2205028

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.2.4.1

Source: 	Qualcomm Incorporated

Title: 	Evaluation on AI-ML for positioning accuracy enhancement

Document for:		Discussion and Decision

Introduction

At RAN #94, a new study on artificial intelligence/machine learning for NR air interface was approved [1] with the following evaluation goals briefly summarized as below:

- AI/ML positioning sub use cases

  - Decide on what AI/ML positioning sub use cases to be evaluated

- Data modelling and generation:

  - Decide on any required extensions to 3GPP evaluation methodology and 3GPP channel modelling, (from TR 38.901 [2] and TR 38.857 [3][positioning]) for AI/ML positioning use case

  - Decide if any field and/or raytracing data generation is required for AI/ML positioning use case and whether this is required for any robustness studies related to AI/ML positioning

  - Decide if having common datasets should be required for AI/ML positioning evaluation

- Training strategy and generalization requirements:

  - Decide on any requirements for separating training, validation, and testing data for AI/ML positioning use case

  - Decide on whether common AI model is required for cross-checking and discuss related calibration

  - Decide on training assumptions that need to be reported by companies

  - Decide on any requirements to evaluate the generalization of trained AI/ML models

- KPIs:

  - Decide on AI/ML positioning KPIs for the selected sub use cases

  - Decide on benchmarking scheme to evaluate the selected sub use cases

  - Decide on KPIs related to AI/ML model operation (e.g., latency, complexity, hardware requirements – memory, power, etc.) and comparing them with baseline benchmarking scheme

In this document, we discuss various aspects related to evaluating AI/ML positioning enhancements, including evaluation methodology, generation of training and inference datasets, and proposed KPIs. We also provide performance evaluations to both indoor and outdoor scenarios with data generated from synthetic statistical channel models, field experiments, and raytracing modelling.

Prominent sub use cases for AI/ML positioning are discussed in [4]. In this evaluation document, we study the following sub use cases and approaches, ML-based RF fingerprinting (RFFP) positioning and ML likelihood fusion for DL-TDoA positioning.

# Discussion on Evaluation Methodology and KPIs

## 2.1 Scope of evaluation

AI/ML can play significant role in positioning, including enhancing positioning accuracy in challenging multipath and NLOS conditions, providing confidence metrics for the estimated position, provide useful assistance data etc. The evaluation study should show the significance of AI/ML to enhance positioning accuracy, offering a strong motivation to study AI/ML impacts to the specification.

We propose to study both indoor and outdoor scenarios with challenging multipath & NLOS conditions and show how AI/ML can help enhancing positioning accuracy in these scenarios when compared to classical approaches. The evaluation methodology should focus on understanding data collection and training requirements to achieve certain accuracy target and protocol aspects to enable scalable adoption of AI/ML methods for positioning.

For indoor environments, one candidate evaluation scenario can be the InF-DH deployment with extreme clutter settings. For outdoor scenarios, candidates for evaluation scenarios can be UMi/UMa deployments with small LOS probability.

Proposal 1: The evaluation should focus on demonstrating positioning accuracy enhancements using AI/ML methods in challenging multipath and NLOS conditions in both indoor and outdoor conditions.

Proposal 2: For evaluation scenarios, consider InF-DH deployment with extreme clutter conditions for indoor scenarios and UMi/UMa with small LOS probability for outdoor scenarios). Companies to agree on updated clutter and LOS settings for proposed scenarios.

## 2.2 Evaluation methodology

AI/ML methods can offer significant enhancement to positioning accuracy in challenging multipath & NLOS conditions as they can take advantage of prominent spatial and temporal features of the wireless channel and learn mapping between these features and ground truth position. When it comes to model the channel for use case of AI/ML positioning, it is quite important to preserve the temporal and spatial properties of the wireless channel and ensure they are consistent across various locations. Without spatial consistency, the channel would be arbitrarily different at nearby UE locations in a drop, and it would be exceedingly difficult for the ML models to learn meaningful features from the data.

3GPP channel models, specified in TR 38.901, have multiple levels of spatial consistency. For example, the basic channel model, used in link level evaluations, have simple consistency in the form of large-scale parameters. In specific, the LOS/NLOS state and pathloss are a function of distance (see Section 7.4.1 and 7.4.2 in 38.901 [2]) and the showing fading applied on top of the path loss is also correlated and an exponentially decaying correlation as a function of distance is specified in Section 7.4.4 of 38.901. Some correlation parameters are also specified as a function of distance for shadow fading (SF), Rician Factor (K), Delay spread (DS) and Angular spreads (ASA, ASD, ZSA, ZSD). However, it is unknown whether such a correlation modeling across UEs is typically implemented in evaluations; and if implemented the exact mechanism for realizing the correlation could be different across companies.

3GPP further specifies additional components of spatial consistency for cluster specific random delay, shadowing, sign and offset for each angle (AoA, AoD, ZoA, ZoD) and some ray specific random variables (coupling, XPR, random phase) in Section 7.6.3.1 of 38.901. Furthermore, Section 7.6.3.2 also specifies spatially consistent UT/BS mobility modeling, but this model only applies to small motion as there is no birth/death model for paths and clusters. The large-scale parameters are also not updated as a function of the UE mobility. It is our understanding that such additional components are not typically employed in simulations.

In our initial evaluations using statistical channel models, we find the gain offered of AI/ML methods highly depends on the LOS probability and decorrelation distance. We find that normal values specified in specifications TR 38.857 [3] are good starting point.

We propose that 3GPP discuss and consider enabling various aspects of spatial consistency procedure already specified in the TR 38.901 for the purposes of the Rel-18 AI/ML positioning evaluations. While this addresses the modeling of the channel partially, it is also clear that proper spatially consistent channels reflecting observations in the real world cannot be produced using the existing simulation framework. The drawbacks of the existing channel models should be mentioned in the AI/ML TR, so that 3GPP may spend some effort to produce an updated channel model for such cases (AI/ML positioning, RF sensing etc.) in later releases leading up to future 6G studies.

To augment the study and provide an independent verification of the gains of AI/ML positioning methods, companies are encouraged to use results with ray tracing or field data for the positioning use case. It is prudent to note that statistical channel models still offer good flexibility and scalability to quickly simulate a wide range of scenarios while this would be an extremely time-consuming effort to do so with either field data or with ray tracing.

Observation 1: Channel models in TR 38.901 [2] are statistical and limited when it comes to modeling spatially/physically consistent propagation occurring in real deployments.

Observation 2: Modeling channels as close to physical reality as feasible is key to understanding and interpreting the performance of AI/ML positioning methods.

Observation 3: Enabling spatial consistency as specified in Section 7.6.3.1 and Section 7.6.3.2 of TR 38.901 is a good starting point for evaluations.

Proposal 3: The channel model in TR 38.901 [2] is adequate for conducting initial evaluation on AI/ML positioning enhancement. It is recommended to have spatial consistency and/or consistent temporal evolution enabled, as in Section 7.6.3.1 and 7.6.3.2 for generating the channels.

Proposal 4: Companies to agree on channel characteristics to be used for evaluating ML positioning, including LOS probability, decorrelation distance (if any), etc., only if the current values specified in TR 38.857  [3] or the agreed parameters on in Rel-17 found to be insufficient.

Proposal 5: RAN1 to recognize and document the necessity for studying improved channel models for future studies.

Proposal 6: Companies are also encouraged to submit evaluations with ray tracing or with field data to understand the performance of AI/ML positioning methods in real world scenarios due to the known drawbacks of using statistical channels.

## 2.3 Training strategy and generalization requirements

### 2.3.1 Partitioning of evaluation datasets and evaluation model assumptions

Learning and generalization for ML positioning methods depend on diversity of training samples and their similarities to testing ones. The separation requirement between training and testing samples depends on the positioning sub use case and expected operating conditions. The operating condition and positioning sub use case dictate whether training and testing samples should come from channels with same or slightly/significantly different small-scale (SSPs) and/or large-scale (LSPs) propagation parameters. We propose companies to discuss such requirement on sub use case basis. As starting point, companies can adopt similar SSP/LSP parameters for both training and testing and look at the performance to have appropriate judgement on initial model generalization capabilities. Companies can gradually relax channel parameters for testing to verify on the model generalization with new updating settings.

For example, in indoor InF-DH deployment, we can start testing with clutter density of 60%, matching the training case, and then gradually reducing it to 40% and monitoring the inference performance. Another example, in outdoor scenarios, we can consider training with limited UE height and LOS probability generated with normal shadow fading settings as specified in specifications. For testing, we start relaxing these settings to consider extended UE height and larger shadow fading settings.

Observation 4: Partitioning strategy of datasets into training, validation, testing, etc., and their SSP and LSP settings is scenario dependent and should be specified based on the use-case & goal of evaluation study.

Proposal 7: Companies should agree on LSP and SSP settings for training and testing datasets. We propose training and testing datasets to initially have same LSP and SSP settings. Subsequent evaluations can investigate requirements of different settings on scenario and sub use case bases.

A comprehensive discussion on our view regarding AI/ML model standardization is presented in [5]. There is no need to specify specific model for evaluating positioning as this it might not eventually be specified in 3GPP. The AI/ML models are proprietary.

Observation 5: Development of evaluation AI model for positioning is implementation and can be different across companies. AI model can also be different for different scenarios and channel conditions.

Proposal 8: Each company should develop its own proprietary AI model for the sake of positioning evaluation.

### 2.3.2 AI model robustness evaluation

The generalization performance of ML positioning model depends on dynamics in the wireless environment. We find that evaluation should account for generalization performance and companies need to decide on requirements to ensure proper generalization performance. For example, the testing datasets can include slightly changed LSP and/or SSP parameters as required. Companies should report robustness evaluations and agree on required modeling of different dynamics and impairments.

Observation 6: AI model performance can change due to wireless environment dynamics Studying robustness and sensitivity of AI positioning model to channel dynamics is key important to understand how frequently model should be updated and monitored.

Proposal 9: Evaluation should also consider evaluating robustness and sensitivity of AI/ML positioning to wireless environment dynamics.

The developed AI/ML model should be trained and tested while accounting for UE and network impairments. We find the modeling for UE and network impairments listed in Rel-17 studies, i.e., TR 38.857 [3], sufficient for the current study. ML positioning model should also be evaluated against potential UE and network impairments, including UE and network synchronization, UE clock drift, inter-TRP synchronization, group delays, etc.

Observation 7: AI model performance can change due to UE and network impairments (e.g., UE clock drift, group delays, TRP synchronization, etc.). Studying sensitivity of AI positioning model to network and UE impairments is key important to understand whether AI/ML can handle such impairments in real scenarios.

Proposal 10: Evaluation should also consider evaluating the robustness to UE and network impairments (e.g., UE clock drift, group delays, TRP synchronization, etc.). Companies to agree on these settings. Companies can adopt previous settings used in RAN1 Rel-17, TR 38.857 [3].

## 2.4 KPIs for positioning use cases

Companies spent considerable time in Rel-16 and Rel-17 to decide on proper KPIs and benchmarking schemes for evaluating positioning. Thus, we propose adopting KPIs and benchmarking schemes that have been agreed-on in previous releases and studies.

Proposal 11: Baseline scheme should be the state-of-art classical scheme based on RAN1 Rel-17 [3]. Companies are encouraged to provide assumptions on their baseline scheme. KPIs adopted in previous releases should be adopted in the AI/ML positioning study.

Proposal 12: The KPI can be the positioning accuracy for all AI positioning approaches and this applies to all sub use cases.

The focus of this study is to show AI/ML potential in enhancing positioning. Thus, we find it logical to provide new KPIs that relate to AI/ML aspects. However, some of the AI/ML KPIs, e.g., latency and specific model complexity, might not be interesting from specification perspective as they are related AI/ML model implementation and has less impact on specifications. We find KPIs that describe training strategy and relation between training and testing datasets are more appealing since they help assessing the validity of evaluations presented by different companies. Examples of training and testing dataset KPIs include sample area density, distance between training samples, distance between training and testing samples.

Observation 8: Inference latency may be difficult to quantify as it would be implementation dependent.

Proposal 13: Other training and inference KPIs that relate to AI positioning include:

- CDF of minimum distance between training samples à this KPI helps understanding data collection requirements for achieving certain accuracy level.

- UE area density for training set à this KPI helps understanding data collection requirements for achieving certain accuracy level.

- CDF of minimum distance between training and inference samples à this KPI can help assessing how difficult was the generalization task when compared to training.

3 Performance Results

We provide evaluations for both ML-based and ML-assisted positioning sub use cases. We consider the RFFP [4] as the ML-based positioning approach and we use it to show the gain that AI/ML can offer in extreme NLOS indoor scenarios. For the ML-assisted approach, we evaluate the ML likelihood fusion for DL-TDoA scheme and show its enhancements to outdoor positioning scenarios.

## 3.1 ML-based RFFP positioning evaluation

We consider three indoor deployments for evaluating RFFP, including 3GPP InF-DH deployment based on synthetic statistical modelling from TR 38.901, over-the-air indoor deployment, and raytracing-generated indoor deployment. The three deployments include sufficiently significant NLOS conditions that help showing gain of AI/ML in positioning. In our evaluations we consider a baseline scheme that is based on TDoA. The baseline scheme has also SNR pruning enabled and RANSAC for outliers’ rejection. The three deployments have a channel of 100 MHz bandwidth.

### 3.1.1 Performance evaluation using TR 38.901 channel model

We consider the InF-DH deployment [2], as shown in  Figure 1. To simulate extreme NLOS condition, clutter parameters are set to original settings, as shown in Table 1. We decide to revert to original clutter settings for considering extreme NLOS. Companies decided to relax InF-DH clutter settings to have less NLOS as the original one found to be stringent. The rest of simulation parameters are in line with those in TR 38.857 [3]. The spatial consistency is enabled according to Section 7.6.3, TR 38.901 [2] with decorrelation distance 10 meters and the grid-based method discussed in Section 5.3 [6].  We drop 18K UEs uniformly and use16K for RFFP training. The UE area density of training dataset is 2.2 UEs/m2. The testing set has 2K UEs widely spread in the layout. We train the ML model to pay attention to relative timing of multipath components, allowing ML model to account for UE clock drifts as it occurs in real deployments.

Table 1 InF-DH clutter parameters

|  | Clutter density | Clutter height | Clutter size |
| --- | --- | --- | --- |
| InF-DH (original) | 60% | 6 meters | 2 meters |
| InF-DH (relaxed) | 40% | 2 meters | 2 meters |

Figure 1 InF-DH deployment (red triangles: TRPs, blue dots: UE locations).

Figure 2 CDF of horizontal positioning error for RFFP and classical schemes.

In Figure 2, we plot CDF of horizontal positioning error for RFFP and classical benchmarking schemes. The RFFP scheme offers significant improvement to positioning accuracy. The 90th percentile of RFFP and classical schemes are ~3 m and ~13 m, respectively. This proves the significant enhancement that ML methods can offer in extreme NLOS environments. We also discuss next further ML enhancements that helps pushing positioning error to smaller values.

The impact of enabling spatial consistency on RFFP scheme is investigated in Figure 3. The RFFP performance shows better performance when trained on channel realizations generated according to Section 7.6.3 [2] and Section 5.3 [6]. This motivates us to propose companies to further discuss setting of spatial consistency parameters in this study.

Figure 3 CDF of horizontal positioning error for RFFP scheme under different channel modelling assumptions (blue plot: channel model involves spatial consistency as modeled in Section 7.6.3 [1] and Section 5.3 [6]; black plot: channel model is based on TR 38.901 but does not involve spatial consistency modeling in Section 7.6.3 3 [1] and Section 5.3 [6]).

AI/ML offers advantages to positioning that go beyond improving accuracy. AI/ML can be leveraged to enhance positioning performance against network and UE impairments, including UE clock drift and TRP synchronization. RFFP ML model can be trained to account for UE clock drifts and TRP mis synchronization. We evaluate the impact of accounting for these impairments on the generalization performance of RFFP scheme. In Figure 4, we plot the CDF of positioning error for RFFP scheme under two scenarios: ‘no clock drift’ and ‘clock drift’. The two scenarios have same training complexity. The ‘no clock drift’ scenario corresponds to an ideal setting where UE clock is tightly coupled and synchronized with TRPs, while the ‘clock drift’ scenario represents the RFFP testing performance when UE clock drift is set variable within [-150, 150] nanoseconds. We train RFFP ML model to account for such drift. As can be observed, RFFP ML model can be trained to be robust against UE clock drifts. It should be noted that the gap between ‘no clock drift’ and ‘clock drift’ scenarios can be further tightened by applying further optimizations.

The impact of TRP mis-synchronization is evaluated in Figure 5. For the ‘relaxed TRPs sync.’ scenario, we relax TRP synchronization assumption and add artificial synchronization error within [-10,10] nanoseconds during training. This helps accounting for potential residual clock synchronization offsets between TRPs that could happen in real deployments. RFFP ML model is can account for such error and provide comparable performance to ideal synchronization settings, as seen in the ‘relaxed TRP sync.’ scenario. In conclusion, with careful training, RFFP can be made robust to network and UE impairments. We also would like to mention that ML can be trained to learn compensating other static impairments such as group delays.

Figure 4 CDF of horizontal positioning error for RFFP scheme under different UE clock drift conditions (green plot: RFFP performance in ideal settings when no clock drift present; blue plot: training accounts for UE clock drift and testing includes UE clock drift within [-150,150] nanoseconds).

Figure 5 CDF of horizontal positioning error for RFFP scheme under different TRPs’ synchronization assumptions (blue plot: TRPs are synchronized; magenta plot: TRPs have random synchronization error within [-10, 10] nanoseconds).

The impact of UE area density on RFFP performance is investigated in Figure 6. We consider multiple realizations of the RFFP ML model with different UE area densities. Other training assumptions for the considered UE area densities are the same. We increase the UE area density from 2.2 UEs/m2 to 640 UEs/m2. As observed, increasing UE area density when doing training helps reducing positioning error significantly. The 90th percentile of error reduces to ~50 cm (see the black solid plot). Considering performance improvement as function of UE area density is important because it helps companies decide on data collection strategies and signalling requirements depending on the sub use case of interest.

Figure 6 CDF of horizontal positioning error for RFFP under different UE area densities (blue plot: sparse UE area density; black: dense UE area density).

### 3.1.2 Performance evaluation using field data

We employ field data to verify on previous findings from statistical channel models. We conduct experiment using an in-house testbed that has six TRPs and UE mounted on an Automated Ground Vehicle (AGV), as shown in Figure 7. To emulate NLOS condition, we introduce a metal sheet blocker that causes blocking to two TRPs, i.e., TRP 3 and TRP 4. This deployment considers positioning using uplink SRS signals with 100 MHz channel centred at 3.75 GHz. We conduct 3D positioning using RFFP and classical schemes. The CDFs of horizontal positioning error for the two schemes are shown in Figure 8. The RFFP scheme offers good improvement to positioning accuracy when compared to classical scheme. We would like to mention that classical scheme in this experiment applies time filtering using Kalman filter to further improve positioning performance. The RFFP performance still outperforms the classical one and aligns with our findings in previous section.

Figure 7 In-house positioning prototype.

Figure 8 CDF of horizontal positioning error for RFFP and classical schemes based on OTA field data.

### 3.1.3 Performance evaluation using raytracing data

Raytracing offers spatially consistent modelling of radio propagation for indoor and outdoor scenarios. Path gain and phase can be accurately modelled to capture the impact of different propagation phenomena, including reflection, refraction, diffraction, and diffuse scattering. We build an indoor 3D factory model and use raytracing to generate channel impulse responses, as shown in Figure 9. This layout is 100 by 150 sq. mt. and has sophisticated modelling of materials. Raytracing tool uses shoot and bounce raytracing methodology and employs sophisticated modelling of reflection and refraction using physical optics as well as diffraction using uniform theory of diffraction (UTD). Our setup has eight TRPs and dense UE deployment in the right-most middle room. In this room, UEs have extreme NLOS condition and most TRPs do not have LOS path with them. We evaluate positioning using this layout with 100 MHz channel bandwidth centred at 3.5 GHz. The CDF of horizontal positioning error is shown in Figure 10. The ML RFFP scheme still shows excellent performance when compared to the classical one. This highlights the significant improvement that AI/ML can provide for positioning and aligns with findings obtained from statistical models.

Figure 9 Indoor factory modelling using raytracing.

Figure 10 CDF of horizontal positioning error of RFFP and classical schemes based on raytracing-generated data.

## 3.2 ML likelihood fusion for DL-TDoA evaluation

Figure 11 presents a CDF of the horizontal positioning error for the UMi layout (FR1) with a 100 MHz system bandwidth. The channel model incorporates absolute time-of-arrival modelling for NLOS links, as described in Section 7.6.9 of TR 38.901 [2]. The results are shown for two schemes:

- Classical baseline scheme

- Machine learning based scheme: ML model is used to derive DL-TDoA distribution from the channel estimate. Likelihood fusion is used to combine the soft information across different measurements.

The machine learning based solution is seen to provide a significant improvement in positioning accuracy over the classical scheme. For example, the 90th percentile of the horizontal positioning error reduces from 17 m. for the classical scheme to 7.9 m. for the ML-based scheme.

Figure 11: ML-based likelihood fusion vs. classical scheme

# 4. Conclusions

In this document, we discussed various aspects related to evaluating AI/ML positioning enhancements, including evaluation methodology, generation of training and inference datasets, and proposed KPIs. Our initial evaluations show ML positioning methods offering significant gain in challenging NLOS indoor and outdoor scenarios using data generated from synthetic statistical channel models, field experiments, and raytracing modelling. The following is a list of our proposals and observations:

Proposal 1: The evaluation should focus on demonstrating positioning accuracy enhancements using AI/ML methods in challenging multipath and NLOS conditions in both indoor and outdoor conditions.

Proposal 2: For evaluation scenarios, consider InF-DH deployment with extreme clutter conditions for indoor scenarios and UMi/UMa with small LOS probability for outdoor scenarios). Companies to agree on updated clutter and LOS settings for proposed scenarios.

Proposal 3: The channel model in TR 38.901 [2] is adequate for conducting initial evaluation on AI/ML positioning enhancement. It is recommended to have spatial consistency and/or consistent temporal evolution enabled, as in Section 7.6.3.1 and 7.6.3.2 for generating the channels.

Proposal 4: Companies to agree on channel characteristics to be used for evaluating ML positioning, including LOS probability, decorrelation distance (if any), etc., only if the current values specified in TR 38.857  [3] or the agreed parameters on in Rel-17 found to be insufficient.

Proposal 5: RAN1 to recognize and document the necessity for studying improved channel models for future studies.

Proposal 6: Companies are also encouraged to submit evaluations with ray tracing or with field data to understand the performance of AI/ML positioning methods in real world scenarios due to the known drawbacks of using statistical channels.

Proposal 7: Companies should agree on LSP and SSP settings for training and testing datasets. We propose training and testing datasets to initially have same LSP and SSP settings. Subsequent evaluations can investigate requirements of different settings on scenario and sub use case bases.

Proposal 8: Each company should develop its own proprietary AI model for the sake of positioning evaluation.

Proposal 9: Evaluation should also consider evaluating robustness and sensitivity of AI/ML positioning to wireless environment dynamics.

Proposal 10: Evaluation should also consider evaluating the robustness to UE and network impairments (e.g., UE clock drift, group delays, TRP synchronization, etc.).  Companies to agree on these settings. Companies can adopt previous settings used in RAN1 Rel-17, TR 38.857 [3].

Proposal 11: Baseline scheme should be the start-of-art classical scheme used in RAN1 Rel 17 [3]. Companies are encouraged to provide assumptions on their baseline scheme. Positioning KPIs adopted in previous releases should be adopted in the AI/ML positioning study.

Proposal 12: The KPI can be the positioning accuracy for all AI positioning approaches and this applies to all sub use cases.

Proposal 13: Other training and inference KPIs that relate to AI positioning include:

- CDF of minimum distance between training samples à this KPI helps understanding data collection requirements for achieving certain accuracy level.

- UE area density for training set à this KPI helps understanding data collection requirements for achieving certain accuracy level.

- CDF of minimum distance between training and inference samples à this KPI can help assessing how difficult was the generalization task when compared to training.

Observation 1: Channel models in TR 38.901 [2] are statistical and limited when it comes to modeling spatially/physically consistent propagation occurring in real deployments.

Observation 2: Modeling channels as close to physical reality as feasible is key to understanding and interpreting the performance of AI/ML positioning methods.

Observation 3: Enabling spatial consistency as specified in Section 7.6.3.1 and Section 7.6.3.2 of TR 38.901 is a good starting point for evaluations.

Observation 4: Partitioning strategy of datasets into training, validation, testing, etc., and their SSP and LSP settings is scenario dependent and should be specified based on the use-case & goal of evaluation study.

Observation 5: Development of evaluation AI model for positioning is implementation and can be different across companies. AI model can also be different for different scenarios and channel conditions.

Observation 6: AI model performance can change due to wireless environment dynamics Studying robustness and sensitivity of AI positioning model to channel dynamics is key important to understand how frequently model should be updated and monitored.

Observation 7: AI model performance can change due to UE and network impairments (e.g., UE clock drift, group delays, TRP synchronization, etc.). Studying sensitivity of AI positioning model to network and UE impairments is key important to understand whether AI/ML can handle such impairments in real scenarios.

Observation 8: Inference latency may be difficult to quantify as it would be implementation dependent.

# References

| [1] | RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” 3GPP RAN Plenary. |
| --- | --- |
| [2] | TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz,” 3GPP, 2019. |
| [3] | TR 38.857, “Annex C: Appendix to Performance evaluations for R17 performance targets,” 3GPP, 2021. |
| [4] | R1-2205029, “Other aspects on AI-ML for positioning accuracy enhancement,” Qualcomm Incorporated, 2022. |
| [5] | R1-2205023, “General Aspects of AI/ML Framework,” Qualcomm Incorporated, 2022. |
| [6] | “5G Channel Model for bands up to 100 GHz,” 5GCMSIG, 2016. |