3GPP TSG RAN WG1 #109-e			    R1-2205030

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.3.1

Source: 	Qualcomm Incorporated

Title: 	On deployment scenarios and evaluation methodology for NR duplex evolution

Document for:	Discussion/Decision

# Introduction

In RAN plenary #94e, the following study item description on evolution of NR duplex operation was approved [1].

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum. In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1).

In this contribution, we discuss the deployment scenarios and evaluation methodology for both subband non-overlapping full duplex and dynamic/flexible TDD.

# Subband non-overlapping full duplex

## Deployment scenarios

There are different deployment scenarios that could benefit from latency reduction and coverage enhancement advantages of the subband full duplex deployment. During Rel-18 package preparation, companies had different opinions on the deployment scenarios, including macro cells, micro cells, small cells, IAB and Hetnet deployment scenarios. However, RAN1 should carefully select a subset of deployment scenarios as baseline deployment scenario that leverage subband full duplex reduced latency and improved coverage. These baseline deployment scenarios can be evaluated by all companies while other sets of optional deployment scenarios.

Observation 1: Deployment scenarios that could benefit from subband full duplex, e.g. reduced latency and improve coverage, should be considered as baseline deployment scenarios. Other deployment scenarios may be considered as optional deployment scenarios.

Massive MIMO Macro cell with large EIRP should be considered one of the baseline deployment scenarios for gNB subband full duplex to reduce network latency and improve UL coverage. Also, such UMa deployment with large Tx Power is challenging at gNB due to high self-interference which makes it important scenario for such study.

Observation 2: Subband full duplex deployment for Massive MIMO macro cell deployment with large EIRP could benefit from UL coverage gain and latency improvement and at same time is challenging deployment due to large self-interference at gNB. For FR1, UMa with 500m ISD shall be considered and for FR2, UMa with 200m ISD shall be considered.

In addition, small cells deployment with small Tx Power e.g., FR2 Indoor Hotspot could reduce gNB requirements of self-interference mitigation. However at least for FR1, indoor deployment of subband full duplex for indoor may be challenging to handle inter-UE and inter-gNB cross link interference due to close proximity between UEs and TRPs.

Observation 3: Subband full duplex deployment Indoor deployment may reduce requirements on gNB for self-interference mitigation due to small Tx Power. However, at least for FR1, it may be challenging deployment for handling cross link-interference.

For the UL/DL subbands configuration across the cells, there are two options. The first one is where difference cells deploy different division between the UL and DL subbands. Although this option may provide some flexibility for each cell, however, it may increase the cross-link interference across gNB and across UE. The other option is where all cells have same UL and DL subband configurations. In our views, the latter option is more practical from deployment perspective as same gNB hardware and interference mitigation techniques could be used across all cells. In addition, different UL/DL subbands across cells is more challenging from inter-gNB and inter-UE CLI perspective as interference will be fully overlap while in the same subband scenarios, the CLI interference is cross subbands.

Observation 4: Same UL/DL subband configurations across all cells is more practical from deployment scenario.

Proposal 1: For FR1, support UMa with 500m ISD as baseline deployment scenario for subband non-overlapping full duplex evaluation.

Proposal 2: For FR1, support UMi and InH as optional deployment scenarios.

Proposal 3: For FR2, support UMa with 200m ISD for FR2-1 and InH for FR2-1 as baseline deployment scenarios for subband non-overlapping full duplex evaluation.

Proposal 4: For FR2, support UMi with 100m ISD for FR2-1, InH for FR2-2 and IAB as optional deployment scenarios.

Proposal 5: For FR1 and FR2, support configuration of same UL/DL subbands across cells for subband full duplex study for baseline evaluation.

## Evaluation methodology

To evaluate the performance gain of subband full duplex and system level evaluation should be used as the primary tool for the duplex evolution.

Proposal 6: Support SLS as main tool for the evaluation of subband full duplex study.

To reduce the RAN1 efforts on agreeing on the simulation’s assumptions, it makes sense to leverage some of the common SLS assumptions from earlier 3GPP studies, e.g., Rel-17 XR study and Rel-17 Fe-MIMO [3] [4]. However, there are a few assumptions that are unique for the subband full duplex evaluation that could discussed separately. The next few subsections discuss the configurations of the base-station antenna/panels, the slot structure, UL/DL subband division, self-interference modeling and cross-link interference modeling for FR1 and FR2.

### Baseline deployment and comparison metric

DL/UL UPT and DL/UL transfer time could be good comparison metrics to compare subband full duplex deployment scenario with baseline legacy TDD deployment scenario and evaluate the performance benefit.

Proposal 7: For subband full duplex deployment scenario, legacy TDD deployment scenario should be used as base for the evaluation.

Proposal 8: For subband full duplex deployment scenario, support to use DL/UL UPT and DL/UL transfer time as evaluation metrics.

### Base-station antennas and panels configuration

For subband full duplex deployment scenario, base station antenna configuration should be based on two panels configuration with split of the antenna elements for simultaneous downlink transmission and uplink reception. With the split panel architecture, base station can enable larger spatial isolation is an essential component to mitigate self-interference. The physical separation typically in the range of few wavelengths. Exact number cannot be specified for the SBFD simulation evaluation, and it depends on gNB implementation so companies may report the assumed physical panels separation.

For legacy TDD deployment scenario, base station antenna configuration should be based on single panel configuration for either downlink transmission or uplink reception. The single big panel shall have the same total number of antenna elements as used for separate panels for subband full duplex deployment scenario for fair comparison and evaluation, as illustrated in Figure 2-1.

Observation 5: Support separate panels configuration with split of the antenna elements for simultaneous downlink transmission and uplink reception for subband full duplex. Companies can report assumed physical panels separation.

Observation 6: Support single panel configuration with same total number of antenna elements for downlink transmission or uplink reception for legacy TDD.

Figure 2-1 Base-station antenna/panels configuration in TDD and SBFD modes

Proposal 9: For subband full duplex deployment scenario, BS antenna configuration is based on two panels with split of the antenna elements for simultaneous downlink transmission and uplink reception.

Proposal 10: For legacy TDD deployment scenario, BS antenna configuration is based on single panel for downlink transmission or uplink reception.

For the subband full duplex evaluation, the baseline assumption is static panel partition with the two separate panels across all slots. However, among the subband full duplex slots, although simultaneous downlink and uplink is allowed on both directions on every slot, the actual scheduling to conduct simultaneous downlink and uplink, or just downlink, or just uplink is according to traffic conditions. To further optimize the performance of subband full duplex, adaptive antenna array configuration across slots could be considered in the evaluation, in which case the separate panels configuration shall be used on the subband full duplex slots with scheduled simultaneous downlink and uplink, the single panel configuration same as legacy TDD shall be used on the dynamic TDD slots with scheduled either downlink or uplink, as illustrated in Figure 2-2.

Figure 2-2 Adaptive antenna array configuration across subband full duplex slots according to traffic

Observation 7: To further optimize the performance of subband full duplex when traffic is single direction in SBFD slot, TDD-like single panel configuration could be used to improve the beamforming gain.

Proposal 11: At least for FR2, optionally support adaptive antenna array configuration across slots for the subband full duplex evaluation. According to traffic conditions, separate panels configuration shall be used on the subband full duplex slots with scheduled simultaneous downlink and uplink, and TDD-like single panel configuration shall be used on the dynamic TDD slots with scheduled either downlink or uplink.

### Slot structure and UL/DL subband configurations

For legacy TDD deployment scenario, it makes sense to leverage the agreed slot format from earlier 3GPP studies – to use DDDSU as defined in Table A.1-2 of 38.838 of Rel-17 XR study [3]. For subband full duplex deployment scenario, support to use same full duplex slot format of XXXXX (X=FD=D+U) across slots to allow for adaptive, flexible and simultaneous (lower latency) DL and UL scheduling.  From the UE perspectives, all slots can be considered as flexible slot. gNB by dynamic scheduling can grant UE UL/DL transmission or reception in UL/DL subband.  For DL CSI acquisition, gNB can utilize few symbols within a slot for wideband UL reception. Similarly, certain symbols can be used for wideband DL CSI-RS transmission.

Observation 8: For SLS evaluation, SBFD is transparent to the UE where all slots are flexible from UE perspective. gNB dynamically schedules the UE within the UL or DL subbands of the SBFD slot.

- Full band CSI (SRS and CSI-RS) can be enabled at some non-SBFD symbols

Proposal 12:  Support the following slot format configurations for the evaluation of subband full duplex.

- For subband full duplex deployment scenario, use same full duplex slot format XXXXX (X=FD=D+U).

  - Note: all slots are flexible from the UE perspective.

- For legacy TDD deployment scenario, use DDDSU as defined in Table A.1-2 of 38.838.

For FR2, for legacy TDD deployment scenario (and dynamic TDD) and subband full duplex deployment scenario, it makes sense to support periodic reserved DL-only slots and UL-only slots for common control channels. For example, assume SSB periodicity is 20ms and there are 32 SSBs, with two SSBs configured per slot, it needs at least 16 DL slots to transmit all SSBs in every 20 ms / 160 slots (SCS = 120 kHz). Consider the defined slot format has a repeated slot pattern of every 5 slots, could round the 16 SSB slots up to 20 slots reserved as DL-only slots for SSB transmissions. Similarly, assume RO periodicity is 160 ms / 1280 slots (SCS = 120 kHz) with configuration of ssb-perRACH-Occasion = 1, with 2 ROs configured per slot, it needs at least 16 slots for all ROs in every 160 ms / 1280 slots. Same as SSB, could round the 16 RO slots up to 20 slots reserved as UL-only slots for PRACH transmissions. The rest of the slots of every 20 ms / every 160 ms could follow the defined legacy TDD or subband full duplex slot format, as illustrated in Figure 2-3.

For subband full duplex deployment scenario, in the periodic reserved DL-only slots and UL-only slots, baseline option for the base station could be no simultaneous data transmission on the opposite direction. The first extension option could be to allow simultaneous DL data transmission on the periodic reserved RO UL slots. The second extension option could be to also allow simultaneous UL data transmission on the periodic reserved SSB DL slots.

Figure 2-3 Periodic reserved DL-only slots and UL-only slots for common control channels for FR2

Proposal 13: For FR2, for legacy TDD deployment scenario and subband full duplex deployment scenario,

- Support periodic reserved DL-only slots and UL-only slots for common control channels

  - E.g. 20 slots per 20 ms for SSB, 20 slots per 160 ms for PRACH

For the DL and UL subband full duplex, there could be different UL and DL subbands slots configurations as shown in Figure 2-4. For example, there could be one DL subband and one UL subbands or multile UL/DL subbands. For example, to enable inter-operator coexistence and align traffic direction at edge of the channel, the dual DL subbands at edge + UL in middle or dual UL subbands edge + DL subbands are essentials.

Figure 2-4: Different UL/DL subbands configuration within the SBFD slot

For SLS evaluation, the DL heavy SBFD slots (UL subband in middle) could be used as baseline. In general, for DL and UL subbands configuration, assume N1 is the number of RBs for DL subbands, N2 is the number of RBs for UL subbands, N3 is the number of RBs for the guard gap between the DL and UL subbands. Therefore, N1+N2+N3 is the total bandwidth and N1, N2 and N3 are configurable.

A typical configuration is 4 to 1 splits of the frequency resource between DL:UL subbands. This will map to ~40% RBs for each of the two DL RBs (N1=2x~40% RBs) and ~20% RBs for UL RBs in the middle (N2=~20% RBs) and guard band in between each DL and UL subbands as illustrated in Figure 2-5. The number of guard bands could be N or 0 RB between each DL and UL subbands (N3=2xN or 0 RB).

Other DL and UL subbands configuration options could also be considered as optional configurations, e.g. DL subband in the middle and two UL subbands on the sides configuration or single DL subband and single UL subband configuration for subband full duplex.

For FR2, with narrower beams, support fully overlapping DL and UL configuration for in-band full duplex as another option for evaluation, in which case frequency resources can be more efficiently utilized. In the fully overlapping DL and UL configuration, N1=N2 and N3=0. Initial performance evaluation results in section 2.3.2 for FR2 will show the performance gain with fully overlapping DL and UL.

The number of guard band RBs between the DL and UL subbands could be a soft threshold. The larger the guard band, the smaller the “flat” residual interference we will model, and vice versa. N RBs or 0 RB could be the choices for the number of guard band RBs. The exact residual interference level will change accordingly.

GB: N3 RBs

UL SB: N2 RBs

DL SB: N1 RBs

Figure 2-5 DL/UL band configuration

Proposal 14: For subband full duplex deployment scenario, support configurable N1 RBs DL subbands, N2 RBs UL subbands and N3 RBs as the gap between the DL and UL subbands

- Option 1: Support ~40% RBs for each of the two DL subbands (N1=2x~40% RBs) and ~20% RBs for UL subband in middle (N2=~20% RBs) and guard band in between

  - N or 0 RB for the gap between DL and UL subbands (N3=2xN or 0 RB)

- Option 2: For FR2, support fully overlapping DL/UL band configuration (N1=N2=entire BW and N3=0)

### Self-interference modelling

#### Direct leakage

The amount of residual self-interference into the UL subband depends on the Tx power of the DL signal, the value of spatial isolation and the frequency isolation as explained in  Figure 2-6. In addition to spatial and frequency isolation, gNB receiver may deploy some digital cancellation technique and/or beamforming to reduce the amount of the residual self-interference including clutter reflections. A detailed discussion on mitigation techniques of self-interference is presented in our paper of subband full duplex feasibility [2]. A first order approximation is to model the residual interference as fixed value across the UL subband.

Pself-interfence  = TxPowerDL –(Antenna isolation+ Freq isolation-Beamform nulling/isolation + DigitalIC )

Figure 2-6 Self interference leakage into the UL subband

Observation 9: The amount of residual self-interference depends on gNB spatial isolation, subband frequency isolation, digital interference cancellation and beamform nulling/isolation.

Proposal 15:  The residual self-interference at gNB receiver is modelled as fixed value across the UL subband and is given by Pself-interfence  = TxPowerDL-α

- where α is the overall self-inference reduction capability of the gNB by means of spatial isolation, subband frequency isolation, digital interference cancellation and beamform nulling/isolation.

- FFS: Frequency selective residual self-interference modelling.

#### Clutter reflection

In addition to the direct leakage, there could be some clutter reflection from objects or reflector that are near-by the base-station. The strength of these clutter reflections depends on the locations of these clutters, the orientation and the reflection coefficients which is mainly characterized by the RCS (Radar cross section). Clutter reflections could be very high as large as direct leakage when large reflector is nearby the gNB. In other scenarios, the reflected power from the clutter could be small and can be ignored.

Figure 2-7 Clutter echo

Near field self-interference with direct leakage most likely is not beam-dependent. However, far field clutter reflection shall be beam-dependent.

Consider there is no existing clutter model, to reduce the RAN1 efforts on agreeing on the clutter model assumption, at least for FR2, a simplified statistically clutter modeling can be considered for subband full duplex evaluation. For example, instead of exact clutter drop, clutter reflected paths can be modeled statistically. Individual reflected path could have random strength and AoA, within certain angular range distributed around the Tx beam direction, as illustrated in Figure 2-8. The signal transmitted from the Tx beam is reflected from objects or reflectors and absorbed by the Rx beam of the base station.

Observation 10: There is no 3GPP model for clutter modelling.

Observation 11:  Exact clutter modelling is complicated and may drain RAN1 time and efforts.

Figure 2-8 FR2 statistical clutter modeling

The simplified clutter model shall be an intra-serving-gNB clutter model and will have no impact on other links with other gNBs and other UEs in the network as shown in Figure 2-9. No new AoD paths are added between Tx and Rx of the serving gNB. For simplicity, support taking existing AoD paths and adding reflections with new AoAs with the same direction of existing AoD angles to this serving gNB only.

Observation 12: A statistical clutter model based on statistics of clutter strength and AoA is simple model.

Figure 2-9 Intra-serving-gNB clutter model

Proposal 16: At least for FR2, for subband full duplex deployment scenario, simplified statistical clutter modelling can be considered based on statistics of cluster power and AoA.

Proposal 17: For subband full duplex deployment scenario, simplified statistical clutter modelling shall be intra-serving-gNB model and shall have no impact on other gNBs and UEs in the network.

### Cross-link interference modelling (inter-gNB and inter-UE)

Figure 2-10 shows the differences of cross-link interference modelling on legacy static TDD deployment scenario, dynamic TDD deployment scenario, subband full duplex scenario and fully overlapping full duplex deployment scenario. There is no inter-gNB CLI and inter-UE CLI in legacy static TDD scenario. With evolution to dynamic/flexible TDD, there is inter-gNB CLI and inter-cell inter-UE CLI, which could reuse existing inter-gNB CLI modelling and inter-UE CLI modelling in TR 38.802/38.901. With evolution to subband full duplex, there is inter-subband inter-gNB CLI and inter-subband both inter cell and intra cell inter-UE CLI, RAN1 needs to agree on the inter-subband interference model for evaluation. With further evolution to fully overlapped full duplex, there is in-band inter-gNB CLI and both inter cell and intra-cell inter-UE CLI, could reuse existing inter-gNB CLI modelling and inter-UE CLI modelling in TR 38.802/38.901.

Observation 13: RAN1 needs to agree on the inter-subband cross-link interference model for subband full duplex evaluation.

Figure 2-10 CLI modelling in static TDD, dynamic TDD, SBFD and fully overlapped FD.

#### Inter-gNB inter-subband CLI model

Figure 2-11: inter-gNB CLI modeling

For the modelling of the inter-SB inter-gNB CLI, first the gNB-to-gNB channel can be modelled by leveraging the channel model for flexible duplex evaluation in TR in 38.802/38.901 similar to the gNB-to-UE channel model with certain changes on UE parameters to mimic a gNB. Then, the DL subband to UL subband leakage can be modelled by utilizing the frequency isolation model similar to the self-interference.

Observation 14: gNB-to-gNB channel model by leveraging gNB-to-UE channel model in TR 38.802 / 38.901 with certain changes on UE to mimic a gNB.

Proposal 18: Inter-gNB inter-subband CLI can be modelled using two steps

- Step 1: gNB-to-gNB channel model based on TR 38.802 / 38.901 as gNB-to-UE channel and height adjustment.

- Step 2: inter-SB leakage based on the frequency isolation between DL subband and UL subband.

#### Inter-UE inter-subband CLI modelling

Figure 2-12: inter-UE CLI modeling

Inter-UE inter-subband CLI modelling could be done using similar framework for inter-gNB inter-SB CLI modelling, First the UE-to-UE channel can be modelled by leveraging the channel model for flexible duplex evaluation in TR in 38.802 / 38.901. Then, the UL subband to DL subband leakage can be modelled by utilizing the frequency isolation model similar to the self-interference. The only difference is value of frequency isolation at the UE side is different than the one assumed at gNB side.

Observation 15: UE-to-UE inter-subband CLI model can be done using similar procedure for modelling inter-gNB inter-subband CLI.

Proposal 19: Inter-UE inter-subband CLI can be modelled using two steps

- Step 1: UE-to-UE channel model based on TR 38.802 / 38.901.

- Step 2: inter-SB leakage based on the frequency isolation between UL subband and DL subband.

### Detailed system level simulation assumptions for FR1 and FR2

We provide common baseline simulation assumptions for both FR1 and FR2 for both full duplex and dynamic/flexible TDD evaluations in Table 1. The baseline results are evaluated under various scenarios. Detailed FR1 and FR2 specific simulation parameters are provided in following sessions.

Table 1 common SLS assumptions for FR1 and FR2 for full duplex and dynamic/flexible TDD evaluations

| TDD Configuration | Option 1: DDDSU (Preferred) Option 2: DDDUU Note: Detailed S slot format is 10D:2F:2U. (F=gap) Note: For option 2, there is a 2-symbol gap at the end to third "D" slot of DDDUU |
| --- | --- |
| Scheduler | SU/MU-MIMO PF scheduler (company to report SU or MU), other scheduler (e.g., delay aware scheduler) is up to companies report |
| Channel Estimation /CSI acquisition | Realistic Both CSI feedback and SRS are considered Companies should report - CSI feedback delay, CSI report periodicity, whether using CSI quantization, CSI error model or not, - Assumptions on SRS: periodicity, processing gain, processing delay, etc Note: Companies may optionally use ideal channel estimation |
| PHY processing delay | Baseline: UE PDSCH processing Capability #1 Companies should report gNB processing delay, e.g. DL NACK to retransmission delay, UL previous transmission to current transmission delay and etc. |
| PDCCH overhead | Companies should report |
| DMRS overhead | Companies should report |
| Target BLER | Companies should report |
| Max HARQ transmission | Companies should report |
| Power control parameter | Companies should report |
| Transmission scheme | Companies should report |

#### FR1 SLS assumptions

The detailed proposed simulation parameters in Table 2 for FR1 subband full duplex evaluation for UMa, Umi and InH deployment scenarios.

Proposal 20: RAN 1 shall consider simulation parameters in Tables 1, and 2 for FR1 full duplex evaluation.

Table 2 Full duplex specific simulation parameters for FR1

| Parameter | Deployment Scenarios |  |  |
| --- | --- | --- | --- |
|  | UMa (38.913 w/ following parameters) 500 ISD | UMi (38.913 w/ following parameters) 200 ISD | Indoor Hotspot (38.913 w/ following parameters) [Optional] |
| Layout | 21 cells with wraparound ISD: 500m | 21 cells with wraparound ISD: 200m | 120m x 50m ISD: 20m 12 sites, 1 sector per cell site |
| Channel Model | UMa (38.901) | UMi (38.901) | InH (38.901) |
| UE Distribution | 80% indoor, 20% outdoor Note: Other UE distribution can be evaluated optionally. |  |  |
| UE Mobility | 3 Km/hr |  |  |
| Carrier frequency | 3.5 GHz |  |  |
| System bandwidth | 100 MHz |  |  |
| Subcarrier spacing | 30 kHz |  |  |
| BS height | 25m | 10m | 3m |
| UE height | The UE height for indoor UEs is updated as following based on Table 6-1 in TR 36.873. 1.5m |  | 1.5 m |
| Open-loop power control | Default: p0=18dB, alpha=0.8 |  |  |
| Processing Delays | K0 = 0, K1 = 1, K2 = 2 |  |  |
| BS/UE TX power | BS: 45dBm, UE: 23dBm |  | BS: 31 dBm, UE: 23dBm |
| BS antenna configuration | (M,N,P,Mg,Ng,Mp,Np,dV,dH)=(8,16, 2, 1, 1, 4,16, 0.8, 0.5). 128 ports (64 DL +64 UL) Upper half panel for DL TX, bottom half for UL RX |  | (M,N,P,Mg,Ng,Mp,Np,dV,dH)=(4,4, 2, 1, 1, 4,4, 0.8, 0.5). 32 ports (16 DL + 16 UL) |
| Antenna element Gain | 8 dBi |  | 5 dBi |
| UE antenna configuration | (M,N,P,Mg,Ng,Mp,Np,dV,dH) = (1, 2, 2, 1, 1, 1, 2, 0.5, 0.5). 4 ports Option 1: 4 Tx, 4 Rx Option 2: 2 Tx, 4Rx |  | (M,N,P,Mg,Ng,Mp,Np,dV,dH) = (1, 2, 2, 1, 1, 1, 2, 0.5, 0.5). 4 ports Option 1: 4 Tx, 4 Rx Option 2: 2 Tx, 4Rx |
| Resource blocks | 272 RBs - DL: Edge 204RBs, UL: Center 56RBs, GB:12RBs |  |  |
| Slot structure | For FD gNB slot pattern: XXXXX (X=FD=D+U); Downlink subband (symbols): 11 DL, 1 Guard, 1 UL Uplink subband (symbols): 2 DL, 1 Guard, 11 UL |  |  |
| Inter-UE CLI model | - A.2.1.2 of 36.843 is used to model UE to UE links - In A.2.1.2 , InH model is used for Indoor-Indoor - When UE to UE distance is greater than 50m, the UEs are assumed to be in different buildings and dual O2I building penetration loss is modeled. - When UEs are assumed to be in the same building, additional floor loss can optionally be added to model UEs on different floors - Other parameters from 38.901 |  |  |
| Inter-gNB CLI model | - 38.802 (NR Rel 14 SI) assumptions for Flexible Duplex study (Table A.2.1-11: Evaluation parameters specific to flexible duplex) - O2I Losses (No Loss) - Angular spread Params: ASA = ASD, ZSA = ZSD - Other LOS/NLOS parameters from 38.901 - FLAT ACLR 45 dBc/20 MHz |  |  |
| Intra-gNB CLI | - Spatial isolation plus Digital NLIC = 90 dB - Flat ACLR 45 dBc/20MHz - Total Isolation = Spatial Isolation + NLIC + ACLR = 135 dB |  |  |
| Metric | DL/UL User Perceived Throughput and DL/UL transfer time |  |  |
| Traffic model | Option 1: Poisson traffic model with file size and arrival rate adjusted to achieve DL system load of 30% and 50% average across all cells (e.g., 100 KB/file) UL system load of 30% and 50% average across all cells (e.g., 10 KB/file) Option 2: Full buffer traffic for DL, Poisson traffic model for UL: DL: The DL buffer is full all the time UL: File size and arrival rate adjusted to achieve UL system load of 30% and 50% average across all cells (e.g., 10 KB/file) |  |  |

#### FR2 SLS assumptions

We provide baseline simulation parameters in Table 1, Table 3 and Table 4 for FR2 full duplex evaluations. The baseline results are evaluated under various scenarios. Table 1 provides common simulation assumptions for FR1 and FR2 for full duplex and dynamic/flexible TDD evaluations. Table 3 provides the common simulation parameters for full duplex and dynamic/flexible TDD evaluations. Table 4 provides full duplex specific simulation parameters.

Proposal 21: RAN 1 shall consider simulation parameters in Tables 1, 3, and 4 for FR2 full duplex evaluation.

Table 3 Common parameters for full duplex and dynamic/flexible TDD evaluations for FR2

| Parameter | Deployment scenarios |  |  |  |
| --- | --- | --- | --- | --- |
|  | UMa (38.913 w/ following parameters) 200 ISD | UMi (38.913 w/ following parameters) 100 ISD | Indoor Hotspot (38.913 w/ following parameters) FR2-1 | Indoor Hotspot (38.913 w/ following parameters) FR2-2 |
| Layout | 21cells with wraparound ISD: 200m | 21cells with wraparound ISD: 100m | 120m x 50m ISD: 20m TRP numbers: 12 |  |
| Channel Model | UMa(38.901) | UMi(38.901) | InH(38.901) |  |
| UE Distribution | For indoor scenario: 100% indoor For outdoor scenario: 100% outdoor Note: Other UE distribution can be evaluated optionally. |  |  |  |
| Carrier frequency | 30 GHz |  | 30 GHz | 60 GHz |
| Subcarrier spacing | 120 KHz |  | 120 KHz | 120 KHz |
| BS height | 25m | 10m | 3m |  |
| UE height | The UE height for indoor UEs is updated as following based on Table 6-1 in TR 36.873. 1.5m |  | 1.5m |  |
| BS noise figure | 7 dB |  |  |  |
| UE noise figure | 13 dB |  |  |  |
| BS receiver | MMSE-IRC |  |  |  |
| UE receiver | MMSE-IRC |  |  |  |
| UE speed | 3 km/hr |  |  |  |
| MCS | Up to 256QAM |  |  |  |
| BS antenna pattern | 3-sector antenna radiation pattern, 8 dBi |  | Ceiling-mount antenna radiation pattern, 5 dBi |  |
| UE Antenna Pattern | UE antenna radiation pattern model 1, 5dBi |  |  |  |
| UE Antenna Configuration | (Follow Rel-17 evaluation methodology for FeMIMO in R1-2007151) (M, N, P)=(1, 4, 2), 3 panels (left, right, top) |  | (M, N, P) =(1, 4, 2), 3 panels (left, right, top) | (M, N, P) =(2, 8, 2), 3 panels (left, right, top) |
| Downtilt | Not specified |  | 90° (pointing to the ground) Note: Other downtilt values can also be optionally evaluated |  |
| BS Transmit Power | 40 dBm per 80 MHz. EIRP should not exceed 73 dBm Note: For system BW larger than above, Tx power scales up accordingly. | EIRP should not exceed 60 dBm | 23 dBm per 80 MHz. EIRP should not exceed 58 dBm Note: For system BW larger than above, Tx power scales up accordingly. | EIRP should not exceed 40 dBm |
| UE max Tx power | 23dBm, maximum EIRP 43 dBm |  |  |  |
| System Bandwidth | Single Carrier Evaluations: - Option 1: 100 MHz (FR2-1) - Option 2: 400 MHz (FR2-1 and FR2-2) CA Evaluations: Companies should report the CA system bandwidth if CA is configured. Note: Other system bandwidths can also be optionally evaluated |  |  |  |
| Metric | DL/UL UPT and DL/UL transfer time |  |  |  |
| Traffic model | Poisson traffic model: DL: K MB/file and offered system load of 30% and 50% average across all cells for TDD UL: M MB/file and offered system load of 30% and 50% average across all cells for TDD |  |  |  |
| Inter-UE CLI model | Existing inter-UE model in 38.901 |  |  |  |
| Inter-gNB CLI model | Existing inter-gNB model in 38.901 |  |  |  |

Table 4  Full duplex specific simulation parameters for FR2

| Parameter | Deployment scenarios |  |  |  |
| --- | --- | --- | --- | --- |
|  | UMa (38.913 w/ following parameters) 200 ISD | UMi (38.913 w/ following parameters) 100 ISD | Indoor Hotspot (38.913 w/ following parameters) FR2-1 | Indoor Hotspot (38.913 w/ following parameters) FR2-2 |
| Slot format for FD | For FD gNB slot pattern: XXXXX (X=FD=D+U) [SSB, RO slots reserved without data transmission as baseline] For legacy HD gNB slot pattern: DDDSU |  |  |  |
| DL/UL subbands and guard band for FD | Option 1: ~40% RBs for each of the two DL SBs and ~20% RBs for UL SB in middle Option 2: Fully overlapping DL/UL (N or 0 RBs as GB in between up/down DL and UL) |  |  |  |
| Antenna Configurations | FD: 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1) per direction TDD: 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,16,2,2,2;1,1) - proposal (dH, dV) = (0.5λ, 0.5λ) |  | FD: 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8, 8, 2,1,1;1,1) per direction TDD: 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (16, 8, 2,1,1;1,1) (dH, dV) = (0.5λ, 0.5λ) |  |
| SI model | Simplified clutter modeling can be considered based on statistics of AoA and strength. |  |  |  |

## Initial performance evaluation

### FR1 Initial Performance Evaluation

In this section, we discuss initial performance evaluation for the subband full duplex deployment in urban macro scenario. In this setup, the system simulations setup is summarized in Table 8 in the appendix. The baseline scenario is TDD deployment with DDDU slot format while all slots are full duplex slots for the SBFD deployment. The subband full duplex is divided in to two DL subands with 102 RBs each and one center UL subband of 56RBs with 6 RBs guard at each side as explained in Figure 2-13. For the SBFD, both intra-gNB self-interference and inter-gNB self-interference was modelled following the description in section 2.2.4 and section 2.2.5 respectively. The BS has Tx power of 45 dBm over the 100 MHz system BW.

Figure 2-13 UL and DL subbands configuration

In this analysis, we evaluate the impact inter-gNB CLI on both DL and UL performance in terms of DL and UL User perceived throughput and UL transfer time. It is worth mentioning, no solutions for handling inter-gNB CLI was modelled in this system evaluation.

Figure 2-14 UL Perceived Throughput Improvement

Figure 2-14 shows the CDF plot of the Uplink perceived throughput comparison between baseline TDD and SBFD with inter-gNB and without inter-gNB CLI.  Without Inter-gNB CLI, SBFD (red curve) shows significant performance improvement of UPT compared to TDD due to uplink duty cycle enhancement. In SBFD, there is available UL subbnad at every slot unlink TDD where uplink is limited, e.g., once in every slot for a DDDU slot format. The green curve Figure 2-15 shows the impact of inter-gNB CLI on UPT. Without any inter-gNB CLI mitigation solution, it is expected that gNB UL reception would be impacted due interference from other neighbouring base station that may have line of sight (LOS) with the victim gNB. This may require some solutions to mitigate the effect of inter-gNB CLI and restore the full gain of SBFD [2].

Observation 16: SBFD provides UL transmission opportunity in every slot which improve UL duty cycle and improves the UPT as compared to baseline TDD.

Observation 17: Inter-gNB CLI has an impact on the UPT gain for the subband full duplex deployment which require to investigate some solutions to mitigate such impact and recover the full gain of the SBFD.

SBFD results in coverage improvement

Figure 2-15 UL Coverage improvement

Figure 2-15 shows the UL median transfer time versus coupling loss for TDD and SBFD. Same UL transfer time can be achieved for SBFD with higher coupling (4-5 dB) compared to TDD which means SBFD achieves UL coverage improvement. For the SBFD scenario, the coverage limited uplink users get more opportunity to transmit in the uplink (frequent uplink resource allocation with smaller chunk of bandwidth) and can leverage the full UE transmit power across multiple slots. As the uplink allocation is frequent and small, it does not get the uplink queue to build up. Such an allocation enables the UE to transmit with max power on the smaller bandwidth. In case of legacy TDD, all the uplink resources are allocated in one slot which makes the UE to spread the power over larger uplink bandwidth thereby resulting in lower PSD and poor coverage.

Observation 18: SBFD enables coverage limited UE to get more frequent UL opportunity and leverage the full UE transmit power across multiple slots which improve UL coverage.

Figure 2-16 DL performance similar to baseline (TDD)

Finally, the comparison of DL perceived throughput for baseline TDD and SBFD scenarios with and without inter-gNB CLI is shown in Figure 2-16. Compared to TDD, the SBFD downlink perceived throughput has improved, thanks to 100% DL duty cycle as compared to TDD where DL duty cycle is 75%.  In addition, for the SBFD deployment the impact of inter-gNB CLI on DL performance is negligible.

Observation 19: The downlink perceived throughput has improved in case of SBFD. This is because DL duty cycle in SBFD is 100% (i.e., every slot has DL resources), as compared to TDD where DL duty cycle is 75%.

Observation 20: Inter-gNB CLI has almost no impact on DL perceived throughput.

### FR2 Initial Performance Evaluation

Initial performance evaluation results are provided for FR2. In this set of performance results, fully overlapping DL and UL band configuration for full duplex is used in the simulation setup. We provide fully overlapping full duplex performance results compared with baseline TDD half-duplex SU-MIMO and half-duplex MU-MIMO results. The flat fading self-interference model with a range of self-interference isolation values is used in the initial performance evaluation. Inter-UE CLI and inter-gNB CLI are both simulated with reusing the models in TR 38.901. The base station antenna array has the same number of total antenna elements for half duplex SU-MIMO, half duplex MU-MIMO and full duplex modes, as illustrated in Figure 2-17.

Figure 2-17 base-station antenna panels configuration in TDD SU-MIMO, MU-MIMO and FD modes

The main simulation parameters for initial performance evaluation of UMi deployment scenario are provided in Table 9 in the appendix.

For dense urban deployment scenario for FR2, the base station analog beam codebook used for initial performance evaluation is provided in Table 5 and corresponding base station composite codebook is provided in Figure 2-18 for half duplex SU-MIMO, half duplex MU-MIMO and full duplex modes.

Table 5 UMi base station analog beams in TDD SU-MIMO, MU-MIMO and FD modes for FR2

|  | Azimuth Angles | Elevation Angles |
| --- | --- | --- |
| 32x8 – for HD-SU | -60̊ to 60˚, at 5.6˚ separation | 101˚ to 146˚, at 22.5˚ separation |
| 32x4 – for FD and HD-MU | -60̊ to 60˚, at 5.6˚ separation | 112.5˚ and 135˚, at 22.5˚ separation |

Figure 2-18 base-station analog beam codebook in TDD SU-MIMO, MU-MIMO and FD modes

Figure 2-19 shows the downlink user perceived throughput (UPT) results with low load bursty traffic for baseline half-duplex SU-MIMO, half-duplex MU-MIMO and full duplex modes. For full duplex simulations, different curves represent different simulation runs with different parameterized values of self-interference isolation.

Figure 2-19 DL UPT with low load for TDD SU-MIMO, MU-MIMO and FD modes

The DL performance is impacted by UE-UE CLI and gNB-UE interference in a direct sense. Self-interference on the other hand increases UL load, consequently affecting DL performance via increase CLI hence causing an indirect impact.

Observation 21: The downlink perceived throughput for low load bursty traffic has ~2.6x performance gains compared with half duplex SU on the median DL UPT and ~2x performance gains compared with half duplex MU on the median DL UPT. The gains are largely a product of increased time domain resources and reduced transmission latencies enabled by full duplex.

Observation 22: The downlink perceived throughput for low load bursty traffic could achieve better performance with 110 dB or better self-interference isolation value with good isolation.

Figure 2-20 shows the downlink user perceived throughput (UPT) results with high load bursty traffic for baseline half-duplex SU-MIMO, half-duplex MU-MIMO and full duplex modes. For full duplex simulations, different curves represent different simulation runs with different configured self-interference isolation values.

Figure 2-20 DL UPT with high load for TDD SU-MIMO, MU-MIMO and FD modes

Observation 23: The downlink perceived throughput for high load bursty traffic has ~3.6x performance gains compared with half duplex SU on the median DL UPT and ~1.6x performance gains compared with half duplex MU on the median DL UPT. The sources of gain include the increased time domain resources (slot format of DDDDUUUU is used for baseline TDD half-duplex simulations) as well as decreased latencies due to slot format flexibility of full duplex. Larger performance gains are observed over baseline half duplex SU at high load system.

Observation 24: The downlink perceived throughput for high load bursty traffic could achieve better performance with 110 dB or better self-interference isolation value with good isolation.

Observation 25: Full duplex will improve the DL perceived throughput for both low load and high load systems and larger gains can be achieved at high loads.

Observation 26: Better isolation at the base station for full duplex mode can boost the downlink performance gain.

Figure 2-21 shows the uplink user perceived throughput (UPT) results with low load bursty traffic for baseline half-duplex SU-MIMO, half-duplex MU-MIMO and full duplex modes. For full duplex simulations, different curves represent different simulation runs with different configured self-interference isolation values.

Figure 2-21 UL UPT with low load for TDD SU-MIMO, MU-MIMO and FD modes

UL performance is directly affected by gNB-gNB CLI, UE-gNB and self-interference. Self-interference impacts UL directly at the gNB Rx.

Observation 27: The uplink perceived throughput for low load bursty traffic has ~2x performance gains compared with half duplex SU on the median DL UPT. The main sources of gain are the increased (2x) time domain resources as well as the slot format flexibility offered by full duplex (DDDDUUUU is used for baseline TDD half-duplex simulations).

Observation 28: Compared to DL, where the self-interference impact is indirect, UL is more sensitive to self-interference and isolation of 120 dB is needed to achieve ideal full duplex performance.

Observation 29: MU performance is limited at low loads by the availability of UEs to schedule (dynamic) while the SINR cost/loss is fixed due to half the array gain.

Figure 2-22 shows the uplink user perceived throughput (UPT) results with high load bursty traffic for baseline half-duplex SU-MIMO, half-duplex MU-MIMO and full duplex modes. For full duplex simulations, different curves represent different simulation runs with different configured self-interference isolation values.

Figure 2-22 UL UPT with high load for TDD SU-MIMO, MU-MIMO and FD modes

Observation 30: There are similar observations for the uplink perceived throughput for high load bursty traffic. Larger performance gains are observed over baseline half duplex SU at high loads.

Observation 31: Full duplex will improve the UL perceived throughput for both low load and high load systems and larger gains can be achieved in high load system.

Observation 32: UL is more sensitive to self-interference and better isolation is needed to achieve ideal full duplex performance.

To summarize the initial performance evaluation for fully overlapping full duplex for FR2:

Observation 33: gNB full duplex yields different types of advantages over half duplex

- Among them, the lower latency benefit from dynamic TDD (facilitated by full duplex) is a dominant source of gain especially at low loads

- Spatial (DL-UL) multiplexing benefits are clearly seen at both low and high loads, although the percentage of true FD slots is limited

- Larger gain at high loads is observed due to queuing delays incurred by the baseline

- It is critical to limit self-interference to preserve full duplex gains

Observation 34: The current performance results are based on fully overlapping full duplex. Subband full duplex results and also updated fully overlapping full duplex results will be provided next time.

# Dynamic/flexible TDD

## Deployment scenarios

For the potential enhancement on dynamic/flexible TDD, it is very important to find the most practical, relevant and challenging deployment scenarios that can show gain of dynamic TDD. Following same evaluation methodology for subband full duplex, RAN1 should select baseline scenarios that are essential for evaluation plus some other optional deployment scenarios. In our views, enabling dynamic TDD for urban macro cell deployment is essential.

Proposal 22: For FR1, support the following deployment scenarios for study on potential enhancement on Dynamic/flexible TDD.

- Baseline: UMa.

- Optional: HetNet (Macro and Pico)

Proposal 23: For FR2, support the following deployment scenarios for study on potential enhancement on Dynamic/flexible TDD.

- Baseline: UMa (FR2-1) macro and InH (FR2-1)

- Optional: UMi (FR2-1) and InH (FR2-2)

## Evaluation methodology

To evaluate the performance gain of potential enhancement on dynamic/flexible TDD, system level evaluation should be used as the primary tool for the duplex evolution. In addition, the valuation methodology for FR1/FR2 should be similar to the baseline legacy TDD of the SBFD study. The main difference is the slot format. All other simulation assumptions dynamic/flexible TDD evaluation can be the same as legacy TDD, e.g., antenna configuration could be the same as legacy TDD with single panel configuration for Tx or Rx.

For FR2, the bandwidth configuration dynamic/flexible TDD evaluation could be either all for DL or all for UL, as illustrated in Figure 3-1. There could be aligned and misaligned slots across cells. For the misaligned dynamic TDD slots, there will be inter-cell inter-gNB and inter-UE CLI, which could be mitigated with beam isolation, proper beam-pair selection, lower Tx Power for FR2 deployment scenarios.

Figure 3-1 Dynamic TDD with aligned and misaligned slots

Proposal 24: Support SLS as main tool for the evaluation of potential enhancement of dynamic/flexible TDD study.

Proposal 25: For dynamic/flexible TDD,

- Utilize the BS antenna configuration of legacy baseline TDD

- Slot format is all flexible FFFFF (D or U direction is picked based on traffic)

Proposal 26: All other simulation assumptions dynamic/flexible TDD evaluation can be the same as legacy TDD, e.g. antenna configuration could be the same as legacy TDD with single panel configuration for Tx or Rx. For FR2, the bandwidth configuration dynamic/flexible TDD evaluation could be either all for DL or all for UL.

### FR1 SLS assumptions

The detailed proposed simulation parameters in Table 6 for the evaluation on potential enhancement on dynamic TDD.

Proposal 27: RAN 1 shall consider simulation parameters Table 6 FR1 evaluation on Dynamic/flexible TDD

Table 6 Dynamic/flexible TDD specific simulation parameters for FR1

| Parameter | Deployment Scenarios |  |
| --- | --- | --- |
|  | UMa (38.913 w/ following parameters) 500 ISD | UMi (38.913 w/ following parameters) 200 ISD |
| Layout | 21 cells with wraparound ISD: 500m | 21 cells with wraparound ISD: 200m |
| Channel Model | UMa (38.901) | UMi (38.901) |
| UE Distribution | 80% indoor, 20% outdoor Note: Other UE distribution can be evaluated optionally. |  |
| UE Mobility | 3 Km/hr |  |
| Carrier frequency | 3.5 GHz |  |
| System bandwidth | 100 MHz |  |
| Subcarrier spacing | 30 kHz |  |
| BS height | 25 m | 10 m |
| UE height | The UE height for indoor UEs is updated as following based on Table 6-1 in TR 36.873. 1.5m |  |
| Open-loop power control | Default: p0=18dB, alpha=0.8 |  |
| Processing Delays | K0 = 0, K1 = 1, K2 = 2 |  |
| BS/UE TX power | BS: 45dBm, UE: 23dBm |  |
| BS antenna configuration | (M,N,P,Mg,Ng,Mp,Np,dV,dH)=(8,16, 2, 1, 1, 2,16, 0.8, 0.5). 64 ports |  |
| Antenna element Gain | 8 dBi |  |
| UE antenna configuration | (M,N,P,Mg,Ng,Mp,Np,dV,dH) = (1, 2, 2, 1, 1, 1, 2, 0.5, 0.5). 4 ports Option 1: 4 Tx, 4 Rx Option 2: 2 Tx, 4Rx |  |
| Slot structure | For dynamic TDD: Subband Half Duplex in all slots For legacy TDD slot pattern: DDDSU |  |
| Resource blocks | 272RBs - DL: 204RBs, UL: 56RBs, GB:12RBs |  |
| Metric | DL/UL User Perceived Throughput and DL/UL transfer time |  |
| Traffic model | Option 1: Poisson traffic model with file size and arrival rate adjusted to achieve DL system load of 30% and 50% average across all cells (e.g., 100 KB/file) UL system load of 30% and 50% average across all cells (e.g., 10 KB/file) Option 2: Full buffer traffic for DL, Poisson traffic model for UL: DL: The DL buffer is full all the time UL: File size and arrival rate adjusted to achieve UL system load of 30% and 50% average across all cells (e.g., 10 KB/file) |  |

### FR2 SLS assumptions

We provide baseline simulation parameters in Table 1, Table 3, and Table 7 for FR2 dynamic/flexible TDD evaluation. The baseline results are evaluated under various scenarios. Table 1 provides common simulation assumptions for both FR1 and FR2 for both full duplex and dynamic/flexible TDD evaluations. Table 3 provides the common simulation parameters for full duplex and dynamic/flexible TDD evaluations. Table 7 provides dynamic/flexible TDD specific simulation parameters.

Proposal 28: RAN 1 shall consider simulation parameters in Tables 1, 3, and 7 for FR2 dynamic/flexible TDD evaluation. The bandwidth configuration dynamic/flexible TDD evaluation could be either all for DL or all for UL at least for FR2.

Table 7 Dynamic/flexible TDD specific simulation parameters for FR2

| Parameter | Deployment scenarios |  |  |  |
| --- | --- | --- | --- | --- |
|  | UMa (38.913 w/ following parameters) 200 ISD | UMi (38.913 w/ following parameters) 100 ISD | Indoor Hotspot (38.913 w/ following parameters) FR2-1 | Indoor Hotspot (38.913 w/ following parameters) FR2-2 |
| Slot format | Flexible TDD slot pattern: FFFFF (D or U direction is picked based on traffic) [SSB, RO slots reserved without data transmission as baseline] For legacy TDD slot pattern: DDDSU |  |  |  |
| DL/UL band | All D or All U |  |  |  |
| Antenna Configurations | 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,16,2,2,2;1,1) (dH, dV) = (0.5λ, 0.5λ) |  | 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (16,8,2,1,1;1,1) (dH, dV) = (0.5λ, 0.5λ) |  |

## Performance Evaluation

### FR1 Initial Performance Evaluation

To enable dynamic TDD in FR1 macro cell deployment with flexile adaptation of slot format based on traffic, we considered subband half-duplex (SBHD) deployment. In asynchronous slots, where gNBs have different traffic direction, the frequency resources of this slot could be split into DL subband and UL subband as show in Figure 3-2. This subband split provides frequency isolation between aggressor and victim gNBs with help to mitigate inter-gNB CLI.

Figure 3-2: subband isolation to enable dynamic TDD

To evaluate the potential enhancement of dynamic TDD by deploying subband half-duplex operation, system level evaluation study was conducted where all slots are assumed to be flexible subband and cells can either adopt DL subband or UL subband based on traffic direction. In this study, an example of two cells deploying SBHD is shown in Figure 3-3.  The complete SLS assumptions for the SBHD deployment and baseline TDD are summarized in Table 10 at the appendix.

Figure 3-3: SBHD for enabling dynamic TDD

Figure 3-4 shows the UL median UL transfer time for Dynamic-TDD using SBHD and legacy synchronized fixed TDD pattern. Up to 5 dB improvement in UL coverage is observed over TDD. This is due to more UL UL TX opportunities for cell-edge UEs that reduces UL blocking delay and improves UL coverage.

Observation 35: Dynamic TDD based on SBHD deployment improves UL coverage as compared to static TDD due to more frequent UL Tx opportunities and UL blocking reduction.

~5 dB coverage improvement

Figure 3-4 UL transfer time for SBHD vs TDD

Figure 3-5 shows the median DL transfer time of dynamic-TDD using SBHD and static TDD. The impact on DL performance is not significant.

Figure 3-5 Median DL transfer time

Observation 36: At least for FR1, Dynamic TDD based on SBHD deployment improves has no significant loss on DL performance as compared to static TDD for small packet transmission.

Figure 3-6 Median UL UPT

Observation 37: Dynamic TDD based SBHD improvs UL User Perceived Throughput by 48% as compared to static TDD by increasing UL duty cycle

### FR2 Initial Performance Evaluation

Initial performance evaluation results are provided for FR2. We use the same initial simulation set up as described in section 2.3.2 for full duplex evaluation. The reason is that among the full duplex slots, although simultaneous downlink and uplink is allowed on both directions on every slot, the actual scheduling to conduct simultaneous downlink and uplink, or just dynamic downlink only, or just dynamic uplink only, is according to traffic conditions. With bursty traffic, there are various TDD slot patterns over time; therefore, full duplex facilities dynamic TDD. Figure 3-7 shows the slot-type statistics of split on UL-only, DL-only, FD, idle slots for the FD simulation setup with 110 dB self-interference isolation at the base station.

Figure 3-7 Duplex mode % on UL-only, DL-only, FD, idle slots

Observation 38: The actual duplexing percentage has large variation across different cells. On average in this simulation setup, among all the slots, 14% of slots are FD slots, 26% of slots are dynamic DL-only slots, 20% of slots are dynamic UL-only slots, and 40% of slots are idle slots.

Observation 39: The low percentage of full duplex slots indicates that not all the performance gain in the full duplex simulation (shown in section 2.3.2) is achieved by the true simultaneous transmissions, but partial performance gain is achieved by dynamic TDD. Enabling dynamic TDD as a natural side-effect of FD is a significant source of the performance gain.

Among all the full duplex slots, although some slots are dynamic downlink only or dynamic uplink only slots, two separate antenna subpanels are applied same as full duplex slots in this simulation setup. Therefore, in the actual dynamic TDD evaluation without full duplex slots involved, the antenna configuration shall use the single big panel configuration same as legacy TDD, which shall improve the beamforming gain and boost the actual dynamic TDD performance.

Observation 40: The dynamic TDD slots shall have the same single panel antenna configuration as baseline legacy TDD. Actual dynamic TDD results will be provided next time for FR2.

# Conclusions

In summary, we have the following observations and proposals for the evolution of NR duplex operation.

Observation 1: Deployment scenarios that could benefit from subband full duplex, e.g. reduced latency and improve coverage, should be considered as baseline deployment scenarios. Other deployment scenarios may be considered as optional deployment scenarios.

Observation 2: Subband full duplex deployment for Massive MIMO macro cell deployment with large EIRP could benefit from UL coverage gain and latency improvement and at same time is challenging deployment due to large self-interference at gNB. For FR1, UMa with 500m ISD shall be considered and for FR2, UMa with 200m ISD shall be considered.

Observation 3: Subband full duplex deployment Indoor deployment may reduce requirements on gNB for self-interference mitigation due to small Tx Power. However, at least for FR1, it may be challenging deployment for handling cross link-interference.

Observation 4: Same UL/DL subband configurations across all cells is more practical from deployment scenario.

Observation 5: Support separate panels configuration with split of the antenna elements for simultaneous downlink transmission and uplink reception for subband full duplex. Companies can report assumed physical panels separation.

Observation 6: Support single panel configuration with same total number of antenna elements for downlink transmission or uplink reception for legacy TDD.

Observation 7: To further optimize the performance of subband full duplex when traffic is single direction in SBFD slot, TDD-like single panel configuration could be used to improve the beamforming gain.

Observation 8: For SLS evaluation, SBFD is transparent to the UE where all slots are flexible from UE perspective. gNB dynamically schedules the UE within the UL or DL subbands of the SBFD slot.

- Full band CSI (SRS and CSI-RS) can be enabled at some non-SBFD symbols

Observation 9: The amount of residual self-interference depends on gNB spatial isolation, subband frequency isolation, digital interference cancellation and beamform nulling/isolation.

Observation 10: There is no 3GPP model for clutter modelling.

Observation 11:  Exact clutter modelling is complicated and may drain RAN1 time and efforts.

Observation 12: A statistical clutter model based on statistics of clutter strength and AoA is simple model.

Observation 13: RAN1 needs to agree on the inter-subband cross-link interference model for subband full duplex evaluation.

Observation 14: gNB-to-gNB channel model by leveraging gNB-to-UE channel model in TR 38.802 / 38.901 with certain changes on UE height to mimic a gNB.

Observation 15: UE-to-UE inter-subband CLI model can be done using similar procedure for modelling inter-gNB inter-subband CLI.

Observation 16: SBFD provides UL transmission opportunity in every slot which improve UL duty cycle and improves the UPT as compared to baseline TDD.

Observation 17: Inter-gNB CLI has an impact on the UPT gain for the subband full duplex deployment which require to investigate some solutions to mitigate such impact and recover the full gain of the SBFD.

Observation 18: SBFD enables coverage limited UE to get more frequent UL opportunity and leverage the full UE transmit power across multiple slots which improve UL coverage.

Observation 19: The downlink perceived throughput has improved in case of SBFD. This is because DL duty cycle in SBFD is 100% (i.e., every slot has DL resources), as compared to TDD where DL duty cycle is 75%.

Observation 20: Inter-gNB CLI has almost no impact on DL perceived throughput.

Observation 21: The downlink perceived throughput for low load bursty traffic has ~2.6x performance gains compared with half duplex SU on the median DL UPT and ~2x performance gains compared with half duplex MU on the median DL UPT. The gains are largely a product of increased time domain resources and reduced transmission latencies enabled by full duplex.

Observation 22: The downlink perceived throughput for low load bursty traffic could achieve better performance with 110 dB or better self-interference isolation value with good isolation.

Observation 23: The downlink perceived throughput for high load bursty traffic has ~3.6x performance gains compared with half duplex SU on the median DL UPT and ~1.6x performance gains compared with half duplex MU on the median DL UPT. The sources of gain include the increased time domain resources (slot format of DDDDUUUU is used for baseline TDD half-duplex simulations) as well as decreased latencies due to slot format flexibility of full duplex. Larger performance gains are observed over baseline half duplex SU at high load system.

Observation 24: The downlink perceived throughput for high load bursty traffic could achieve better performance with 110 dB or better self-interference isolation value with good isolation.

Observation 25: Full duplex will improve the DL perceived throughput for both low load and high load systems and larger gains can be achieved at high loads.

Observation 26: Better isolation at the base station for full duplex mode can boost the downlink performance gain.

Observation 27: The uplink perceived throughput for low load bursty traffic has ~2x performance gains compared with half duplex SU on the median DL UPT. The main sources of gain are the increased (2x) time domain resources as well as the slot format flexibility offered by full duplex (DDDDUUUU is used for baseline TDD half-duplex simulations).

Observation 28: Compared to DL, where the self-interference impact is indirect, UL is more sensitive to self-interference and isolation of 120 dB is needed to achieve ideal full duplex performance.

Observation 29: MU performance is limited at low loads by the availability of UEs to schedule (dynamic) while the SINR cost/loss is fixed due to half the array gain.

Observation 30: There are similar observations for the uplink perceived throughput for high load bursty traffic. Larger performance gains are observed over baseline half duplex SU at high loads.

Observation 31: Full duplex will improve the UL perceived throughput for both low load and high load systems and larger gains can be achieved in high load system.

Observation 32: UL is more sensitive to self-interference and better isolation is needed to achieve ideal full duplex performance.

Observation 33: gNB full duplex yields different types of advantages over half duplex

- Among them, the lower latency benefit from dynamic TDD (facilitated by full duplex) is a dominant source of gain especially at low loads

- Spatial (DL-UL) multiplexing benefits are clearly seen at both low and high loads, although the percentage of true FD slots is limited

- Larger gain at high loads is observed due to queuing delays incurred by the baseline

- It is critical to limit self-interference to preserve full duplex gains

Observation 34: The current performance results are based on fully overlapping full duplex. Subband full duplex results and also updated fully overlapping full duplex results will be provided next time.

Observation 35: Dynamic TDD based on SBHD deployment improves UL coverage as compared to static TDD due to more frequent UL Tx opportunities and UL blocking reduction.

Observation 36: At least for FR1, Dynamic TDD based on SBHD deployment improves has no significant loss on DL performance as compared to static TDD for small packet transmission.

Observation 37: Dynamic TDD based SBHD improvs UL User Perceived Throughput by 48% as compared to static TDD by increasing UL duty cycle

Observation 38: The actual duplexing percentage has large variation across different cells. On average in this simulation setup, among all the slots, 14% of slots are FD slots, 26% of slots are dynamic DL-only slots, 20% of slots are dynamic UL-only slots, and 40% of slots are idle slots.

Observation 39: The low percentage of full duplex slots indicates that not all the performance gain in the full duplex simulation (shown in section 2.3.2) is achieved by the true simultaneous transmissions, but partial performance gain is achieved by dynamic TDD. Enabling dynamic TDD as a natural side-effect of FD is a significant source of the performance gain.

Observation 40: The dynamic TDD slots shall have the same single panel antenna configuration as baseline legacy TDD. Actual dynamic TDD results will be provided next time for FR2.

And here is the summary of our proposals:

Proposal 1: For FR1, support UMa with 500m ISD as baseline deployment scenario for subband non-overlapping full duplex evaluation.

Proposal 2: For FR1, support UMi and InH as optional deployment scenarios.

Proposal 3: For FR2, support UMa with 200m ISD for FR2-1 and InH for FR2-1 as baseline deployment scenarios for subband non-overlapping full duplex evaluation.

Proposal 4: For FR2, support UMi with 100m ISD for FR2-1, InH for FR2-2 and IAB as optional deployment scenarios.

Proposal 5: For FR1 and FR2, support configuration of same UL/DL subbands across cells for subband full duplex study for baseline evaluation.

Proposal 6: Support SLS as main tool for the evaluation of subband full duplex study.

Proposal 7: For subband full duplex deployment scenario, legacy TDD deployment scenario should be used as base for the evaluation.

Proposal 8: For subband full duplex deployment scenario, support to use DL/UL UPT and DL/UL transfer time as evaluation metrics.

Proposal 9: For subband full duplex deployment scenario, BS antenna configuration is based on two panels with split of the antenna elements for simultaneous downlink transmission and uplink reception.

Proposal 10: For legacy TDD deployment scenario, BS antenna configuration is based on single panel for downlink transmission or uplink reception.

Proposal 11: At least for FR2, optionally support adaptive antenna array configuration across slots for the subband full duplex evaluation. According to traffic conditions, separate panels configuration shall be used on the subband full duplex slots with scheduled simultaneous downlink and uplink, and TDD-like single panel configuration shall be used on the dynamic TDD slots with scheduled either downlink or uplink.

Proposal 12:  Support the following slot format configurations for the evaluation of subband full duplex.

- For subband full duplex deployment scenario, use same full duplex slot format XXXXX (X=FD=D+U).

  - Note: all slots are flexible from the UE perspective.

- For legacy TDD deployment scenario, use DDDSU as defined in Table A.1-2 of 38.838.

Proposal 13: For FR2, for legacy TDD deployment scenario and subband full duplex deployment scenario,

- Support periodic reserved DL-only slots and UL-only slots for common control channels

  - E.g. 20 slots per 20 ms for SSB, 20 slots per 160 ms for PRACH

Proposal 14: For subband full duplex deployment scenario, support configurable N1 RBs DL subbands, N2 RBs UL subbands and N3 RBs as the gap between the DL and UL subbands

- Option 1: Support ~40% RBs for each of the two DL subbands (N1=2x~40% RBs) and ~20% RBs for UL subband in middle (N2=~20% RBs) and guard band in between

  - N or 0 RB for the gap between DL and UL subbands (N3=2xN or 0 RB)

- Option 2: For FR2, support fully overlapping DL/UL band configuration (N1=N2=entire BW and N3=0)

Proposal 15:  The residual self-interference at gNB receiver is modelled as fixed value across the UL subband and is given by Pself-interfence  = TxPowerDL-α

- where α is the overall self-inference reduction capability of the gNB by means of spatial isolation, subband frequency isolation, digital interference cancellation and beamform nulling/isolation.

- FFS: Frequency selective residual self-interference modelling.

Proposal 16: At least for FR2, for subband full duplex deployment scenario, simplified statistical clutter modelling can be considered based on statistics of cluster power and AoA.

Proposal 17: For subband full duplex deployment scenario, simplified statistical clutter modelling shall be intra-serving-gNB model and shall have no impact on other gNBs and UEs in the network.

Proposal 18: Inter-gNB inter-subband CLI can be modelled using two steps

- Step1: gNB-to-gNB channel model based on TR 38.802 / 38.901 as gNB-to-UE channel and height adjustment.

- Step2: inter-SB leakage based on the frequency isolation between DL subband and UL subband.

Proposal 19: Inter-UE inter-subband CLI can be modelled using two steps

- Step1: UE-to-UE channel model based on TR 38.802 / 38.901.

- Step2: inter-SB leakage based on the frequency isolation between UL subband and DL subband.

Proposal 20: RAN 1 shall consider simulation parameters in Tables 1, and 2 for FR1 full duplex evaluation.

Proposal 21: RAN 1 shall consider simulation parameters in Tables 1, 3, and 4 for FR2 full duplex evaluation.

Proposal 22: For FR1, support the following deployment scenarios for study on potential enhancement on Dynamic/flexible TDD.

- Baseline: UMa.

- Optional: HetNet (Macro and Pico)

Proposal 23: For FR2, support the following deployment scenarios for study on potential enhancement on Dynamic/flexible TDD.

- Baseline: UMa (FR2-1) macro and InH (FR2-1)

- Optional: UMi (FR2-1) and InH (FR2-2)

Proposal 24: Support SLS as main tool for the evaluation of potential enhancement of dynamic/flexible TDD study.

Proposal 25: For dynamic/flexible TDD,

- Utilize the BS antenna configuration of legacy baseline TDD

- Slot format is all flexible FFFFF (D or U direction is picked based on traffic)

Proposal 26: All other simulation assumptions dynamic/flexible TDD evaluation can be the same as legacy TDD, e.g. antenna configuration could be the same as legacy TDD with single panel configuration for Tx or Rx. For FR2, the bandwidth configuration dynamic/flexible TDD evaluation could be either all for DL or all for UL.

Proposal 27: RAN 1 shall consider simulation parameters Table 6 FR1 evaluation on Dynamic/flexible TDD

Proposal 28: RAN 1 shall consider simulation parameters in Tables 1, 3, and 7 for FR2 dynamic/flexible TDD evaluation. The bandwidth configuration dynamic/flexible TDD evaluation could be either all for DL or all for UL at least for FR2.

# References

- 3GPP TSG RAN Meeting #94e, RP-213591 “New SI: Study on evolution of NR duplex operation”, CMCC, Electronic Meeting, Dec. 6 - 17, 2021.

- R1-2205031, “Feasibility and techniques for subband non-overlapping full duplex”, Qualcomm Inc, RAN1 #109-e.

- 3GPP TR 38.838, “Study on XR (Extended Reality) Evaluations for NR” (Release 17)

- Chairman’s Notes, SLS assumptions at least for SRS capacity enhancements in Rel-17, RAN1#102-e

# Appendix

Table 8 FR1 SBFD SLS assumptions

| Parameter | Subband Full Duplex | Static TDD |
| --- | --- | --- |
| Scenario | UMa - 7 sites, 3 sectors/site. 10 UEs/sector |  |
| Carrier frequency | 3.5GHz |  |
| System bandwidth, SCS | 100MHz TDD, 30KHz |  |
| BS/UE TX power | BS half panel: 45dBm, UE: 23dBm | BS full panel: 45dBm, UE: 23dBm |
| BS antenna configuration | (M,N,P,Mg,Ng,Mp,Np,dV,dH)=(8,16, 2, 1, 1, 4,16, 0.8, 0.5). 128 ports (64 DL +64 UL) Upper half panel for DL TX, bottom half for UL RX | (M,N,P,Mg,Ng,Mp,Np,dV,dH)=(8,16, 2, 1, 1, 2,16, 0.8, 0.5). 64 ports |
| UE antenna configuration | (M,N,P,Mg,Ng,Mp,Np,dV,dH) = (1, 2, 2, 1, 1, 1, 2, 0.5, 0.5). 4 ports |  |
| UE location/mobility | 20% outdoor, 80% indoor, 3 km/hr |  |
| Frame format | Full Duplex in all slots | DDDU |
| Resource blocks | 272 RBs - DL: Edge 204RBs, UL: Center 56RBs, GB:12RBs | 272 RBs |
| Open-loop power control | Default: p0=18dB, alpha=0.8 |  |
| Processing Delays | K0 = 0, K1 = 1, K2 = 0 |  |
| Cross-link interference | gNB-gNB is modeled | N/A |
| Panel isolation + NLIC / ACLR | 90 dB / Flat: 45 dBc/20 MHz Freq. Flat | N/A |
| Traffic pattern | Each gNB serves 5 DL UEs and 5 UL UEs. Bidirectional Poisson. DL: 100 files/sec/UE, 20KB/file. UL: 300 files/sec/UE, 1KB/file |  |

Table 9 Full duplex initial evaluation simulation parameters for FR2

| Traffic model (Per UE offered load) | Bursty Poisson traffic, 0.5 Mbyte file size |  |
| --- | --- | --- |
|  |  | Umi 100m |
|  | Low Load (DL/UL) ~30% RU for SU-MIMO | 18/14.4 Mbps |
|  | High Load (DL/UL) ~60% RU for SU-MIMO | 28/21.6 Mbps |
| UE antenna configuration | Two 2x2 dual polarized, front and back Target beam angles: ±45° in azimuth, 45° & 135° in elevation |  |
| Carrier freq. & bandwidth | 100MHz @ 28GHz |  |
| Noise figures | gNb = 7dB, UE = 9dB |  |
| UE Tx power | 17dBm à 31 dBm EIRP |  |
| SRS Periodicity | 10 ms (80 slots) |  |
| Scheduler | Proportional fair (w/ analog BF based MU-MIMO) |  |
| Node height | gNB height = 10m for Umi UE height = 1.5m |  |
| Processing time | PDSCH to ACK = 2 slots (K1), UL grant to PUSCH = 3 slots (K2), PUSCH to retransmission DCI = 5 slots; UPT= File SizeΔT, ΔT=Last PDUSCH ACK -File Arrival Time |  |

Table 10. SLS for FR1 Dynamic TDD SBHD study

|  | - Subband Half Duplex | - Static TDD |
| --- | --- | --- |
| - Scenario | - UMa - 7 sites, 3 sectors/site. 10 UEs/sector |  |
| - Carrier frequency | - 3.5GHz |  |
| - System bandwidth, SCS | - 100MHz TDD, 30KHz |  |
| - BS/UE TX power | - BS: 45dBm, UE: 23 dBm, BS NF: 5 dB, UE NF: 9 dB - Same DL PSD per RB (SBHD does not boost power by allocating 45dBm over DL subband) |  |
| - BS antenna configuration | - (M,N,P,Mg,Ng,Mp,Np,dV,dH)=(8,16, 2, 1, 1, 2,16, 0.8, 0.5). 64 ports |  |
| - UE antenna configuration | - (M,N,P,Mg,Ng,Mp,Np,dV,dH) = (1, 2, 2, 1, 1, 1, 2, 0.5, 0.5). 4 ports |  |
| - UE location/mobility | - 20% outdoor, 80% indoor, 3 km/hr |  |
| - Frame format | - Subband Half Duplex in all slots | - SSSU |
| - Resource blocks | - 272RBs - DL: 204RBs, UL: 56RBs, GB:12RBs | - 272RBs |
| - Open-loop power control | - Default: p0=18dB, alpha=0.8 |  |
| - Processing Delays | - K0 = 0, K1 = 1, K2 = 0 (K2=0 synchronizes DL/UL scheduling for simplification) |  |
| - Cross-link interference | - gNB-gNB is modeled | - N/A |
| - ACLR | - Flat: 45 dBc/20 MHz | - N/A |
| - Traffic pattern | - Bidirectional Poisson. DL: 20KB/file, 100 files/s/UE. UL: 1KB/file, 150 files/s/UE |  |