3GPP TSG RAN WG1 #109-e			    R1-2205032

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.1.3.1

Source: 	Qualcomm Incorporated

Title: 	On potential enhancements on dynamic/flexible TDD

Document for:	Discussion/Decision

# Introduction

RAN plenary #94e, the study item on evolution of NR duplex operation was approved [1]. The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum. In this study, the followings are assumed:

- Duplex enhancement at the gNB side

- Half duplex operation at the UE side

- No restriction on frequency ranges

The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In this contribution, the feasibility and potential enhancement for dynamic/flexible TDD are discussed.

# Summary of Rel-16 study on dynamic TDD

In Rel-16, coexistence study for the dynamic TDD deployment across adjacent channels were conducted and outcome was captured in TR 38.828 [2]. The main focus of the study was the on co-existence of CLI among different operators in adjacent channels. The outcome of this study showed that there are deployment scenarios where performance is degraded and dynamic TDD is not suitable. While there are some deployment scenarios where little to no performance degradation was observed. The next is summary of the simulations results and recommendations for different scenarios.

## Deployment Scenarios with performance degradation

In FR1, performance degradation due to BS-to-BS interference is observed in macro-to-macro deployments. For indoor-to-indoor scenarios, some performance degradation is observed under high BS transmit power. For macro-to-indoor, performance degradation exists in some scenarios due to UE-to-UE interference. In FR2, BS-to-BS interference leads to some performance degradation in macro-to-macro deployments and some scenarios in indoor-to-indoor and micro-to-micro deployments. Note, in this study that interference from co-located aggressor BS leads to Rx blocking at victim BS in all scenarios and no simulation results were provided for this case.

## Deployment Scenarios without performance degradation

In FR1, no performance degradation is observed due to BS-to-BS interference for in indoor-to-macro and macro-to-indoor deployments. UE-to-UE interference has no impact on performance in macro-to-macro, indoor-to-macro, and indoor-to-indoor deployments. In FR2, no performance degradation is observed due to BS-to-BS interference in indoor-to-macro and macro-to-indoor deployments. UE-to-UE interference has no impact on performance in macro-to-macro, indoor-to-macro, indoor-to-indoor and micro-to-micro deployments.

## Outcome and Recommendation

The following recommendations are made regarding feasibility of dynamic TDD in FR1. Dynamic TDD is not suitable for macro-to-macro deployments because of performance degradation caused by BS-to-BS interference. No performance degradation is expected when operating dynamic TDD between an indoor network and a macro network and vice versa assuming sufficient isolation. For indoor-to-indoor deployments, dynamic TDD is feasible if some conditions are met.

In FR2, for macro-to-macro deployments, dynamic TDD is feasible in certain deployments with coordination between operators to avoid performance degradation caused by BS-to-BS interference. Dynamic TDD is feasible in indoor-to-macro and macro-to-indoor deployments with sufficient isolation. For Indoor-to-Indoor deployments, dynamic TDD is feasible under careful layout planning and parameterization. Finally, dynamic TDD can also be used in certain micro-to-micro deployments with careful planning.

Observation 1: For FR1, deployments scenario with large Tx Power BS suffers from inter-gNB interference.

- In general, inter-UE CLI is not an issue except for macro-to-indoor deployment.

Observation 2: For FR2, Dynamic TDD is possible under careful assumption of layout and power parameterization to avoid inter-gNB interference.

# Scope of the potential enhancement of dynamic TDD

The study item description [1] has a clear note that for potential enhancements on dynamic/flexible TDD, RAN1 should utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion. Given Rel-16 dynamic TDD study on inter-operator adjacent channel coexistence, this work should not be repeated. The focus of Rel-18 SI on potential enhancement on dynamic TDD should be limited to co-channel intra-operator deployment.

Observation 3: Rel-18 study on potential enhancement of dynamic TDD suggests utilizing the outcome of Rel-15 and Rel-16 studies outcome avoid repetition of same discussion, e.g., inter-operator Dynamic TDD coexistence study.

Proposal 1: The focus of Rel-18 study on potential enhancement for dynamic TDD should be limited to co-channel intra-operator deployment.

# Feasibility of Dynamic TDD

## Dynamic TDD using non-overlapping subbands across gNB

To enable dynamic TDD deployment in FR1 macro cell deployment with flexile adaptation of slot format based on traffic, we considered subband half-duplex (SBHD) deployment. In asynchronous slots, where gNBs have different traffic direction, the frequency resources of this slot could be split into DL subband and UL subband as show in Figure 4-1. This subband split provides frequency isolation between aggressor and victim gNBs with help to mitigate inter-gNB CLI.

Figure 4-1: subband isolation to enable dynamic TDD

In the scenario where all slots are considered flexible subband as show in Figure 4-2, it provides flexibility to adapt to traffic demand and addresses new hybrid topologies where use cases dictate use of different slot structures. In addition, the ability to select direction based on traffic profile, reducing blocking delay. Also, the availabity of fore UL TX opportunities for cell-edge UEs helps improving UL coverage. Finally, this deployment can be achieved by utilizes existing radio HW. This can prove subband operation as a viable approach to mitigate cross-link interference. The only drawback of using all slots as flexible subband is potential loss of frequency resources that may leads into some performance loss for heavily loaded system. However, SBHD could be enabled for asynchronous slots.

Figure 4-2: SBHD slots structure across cells

Observation 4: SBHD can enable dynamic TDD and mitigate the impact of inter-gNB CLI.

Observation 5: SBHD-based dynamic enables flexible adaption of slots direction based on traffic which leads to reduced latency and improved UL coverage.

To study the feasibility of macro-to-macro dynamic TDD using subband half-duplex mode, the following link-budget consider a macro base station with 45 dBm Tx Power and BS array gain of 20 dBi which gives 65 dBm of EIRP. The thermal noise power assuming 7 dB noise figure and 100 MHz system BW is -87 dBm.

|  | Out of subband blocker | subband jammer |
| --- | --- | --- |
| EIRP | 68 dBm |  |
| Inter-gNB Path Loss (500 m) | 98 dB | 98 dB |
| Subband Freq. isolation | 0 | 45 dB |
| shadowing | 5 dB | 5 dB |
| Tx/Rx beam nulling or beam isolation | 10~15 dB | 10~15 dB |
| Rx Power | - 45 ~ 50 dBm | -90 to -95 dBm |

Assuming sufficient Tx and Rx beamform nulling of 10 dB or higher, the residual interference within the UL subband is lower than thermal noise floor and amount of IoT is negligible.

Observation 6: Link budget analysis shows that SB-based dynamic TDD is feasible for macro-cell deployment.

Proposal 2: Support subband half-duplex as solution to enable dynamic TDD at least for FR1

## Qualcomm MWC21 OTA SBHD Demonstration

Qualcomm showed in MWC 21 a demonstration of 5G NR test network supporting subband half-duplex which enables Dynamic TDD and allows for more flexible service multiplexing as well as improve latency and coverage. In this OTA network operates in the 3.5 GHz spectrum with 100 MHz bandwidth, supporting two gNB with advanced multi-user massive MIMO and 5G NextGen Core. Two commercial devices, device 1 is a cell edge UE driving UL heavy bursty traffic served by gNB1 and device 2 is driving downlink heavy burst traffic served by gNB2. The Baseline scenario is fixed TDD with DDDU slot format. While in the SBHD network, different cells have different slots structure which allow adaptation of traffic needs and reduce switch latency. gNB1 has D’U’D’U’ slot format to deliver more UL opportunity while gNB2 has D’D’D’U’ slot format to serve the DL heavy user slot formant where D’ and U’ are DL and UL slots with subband mode. In slot 1 and 3 and 4 gNB1 and gNB2 are either transmitting on downlink or receiving on uplink and there is no cross-link interference. In slot 2, gNBs have different direction and the DL-to-UL interference is mitigated through the subband operation.

Compared to Device 1 uplink perceived throughput and latency is significantly improved due to increased UL duty cycle. Device 2 still gets 75% DL duty cycle from gNB2, and its downlink perceived throughput and latency are mostly not impacted.

Observation 7: Qualcomm OTA test network validated the feasibility of dynamic TDD in macro-cell deployment using subband half-duplex.

## Dynamic TDD using fully overlapping UL and DL

At least for FR2, fully overlapping UL and DL for dynamic TDD shall be considered as illustrated in Figure 4-3. There could be aligned and misaligned slots across cells. For the misaligned dynamic TDD slots, there will be inter-cell inter-gNB and inter-UE CLI, which could be mitigated with beam isolation, proper beam-pair selection, lower Tx Power for FR2 deployment scenarios.

Figure 4-3 Dynamic TDD with aligned and misaligned slots

Observation 8: In FR2, Dynamic TDD with misaligned slots format is possible where CLI could be mitigated with proper beam-pair selection and lower Tx power.

# Inter-UE Cross-link interference mitigation techniques

Leveraging/Using Rel-16 CLI framework without enhancement for dynamic/flexible TDD deployment has few limitations which are discussed in the following section.

### Limitation # 1: Latency and configuration flexibility

Rel-16 CLI framework is based on layer 3 reporting which inherently has limited flexibility. L3 reporting is carried out on PUSCH channel and is collected by gNB-CU first then communicated to the gNB-DU. This will introduce some latency on the CLI report availability to the gNB-DU. However, L1 or L2 based CLI reporting doesn’t suffer from such increased latency.  In addition, L3 reporting is based on periodic CLI measurement resources with L3 filtering. This is not suitable for enabling fast beam selection in response to interference variation as compared to L1 based beam selection. Furthermore, RRC reconfiguration is needed to update the configuration of the CLI measurement resource which is typically in order of tens of millisecond.

Observation 9: Rel-16 CLI reporting is based on L3 which has limited flexibility and slow adaptability

- This leads to increased latency in CLI reporting which is not suitable for fast adaptation to mitigate CLI

- L1/L2 report can be obtained by gNB-DU with lower latency, in turn it can better reflect current CLI

- L1 report can be sent on-demand enabling fast L1 beam adaptation

### Limitation # 2: QCL Type-D

Rel-16 CLI framework does not support signalling/configuration of Rx beam (QCL-D) for CLI measurement. Specifically, Rx beam for CLI measurement is up to UE implementation as QCL-D follows one of the latest received PDSCH and the latest monitored CORESET. In turn, gNB cannot characterize CLI for different Rx beams in order to enable CLI-aware beam management.

Observation 10: In Rel-16 CLI framework, there is no dedicated signalling or configuration of QCL-D for CLI measurement, hence not suitable for enabling CLI-aware beam management

### Limitation # 3: subband reporting

Rel-16 CLI framework does not support subband CLI reporting, i.e., reporting CLI for one or more subbands in the measurement bandwidth. It supports wideband CLI reporting which may not be suitable for measuring CLI leakage from an adjacent subbands in SBFD, since CLI leakage to adjacent subbands is non-uniform over the measurement bandwidth.

Observation 11: Rel-16 CLI framework does not support subband CLI reporting, i.e., reporting CLI for one or more subbands in the measurement bandwidth. In SBFD, CLI leakage to adjacent subbands is not uniform over the measurement bandwidth and may require subband CLI reporting.

## Possible Enhancement for inter-UE CLI

Considering that Rel-16 CLI reporting is L3 which lacks flexibility and is not suitable for measurement of dynamic CLI in dynamic/flexible TDD operation. L1 or L2 based reporting is beneficial to adapt to fast inter-UE CLI within one cell.

Proposal 3: Support L1/L2 based CLI reporting to increase flexibility and reduce reporting latency compared to Rel-16 L3 based framework.

In Rel-16 CLI framework, Rx beam for CLI measurement is up to UE implementation and can’t be used for enabling CLI measurement and reporting in different spatial directions

Proposal 4: Support UE Rx beam (QCL-D) configuration and indication per CLI measurement resource for enabling CLI-aware beam management.

In Rel-16 CLI framework, only wideband CLI reporting is supported based on CLI measurement resource bandwidth. In dynamic/flexible TDD operation, CLI leakage is non-uniform over the DL subbands and may require CLI reporting with increased frequency granularity. The UL and DL could be either fully overlapped, partially overlapped or non-overlapping as the case for subband half-duplex. This motivates the use-case

Proposal 5: Support subband-based CLI reporting to provide accurate CLI reporting in dynamic TDD in which CLI could be non-uniform across the DL.

# Inter-gNB Cross-link interference mitigation techniques

## Limitation of Rel-16 RIM

In Rel-16, remote interference management (RIM) was introduced to facilitate interference detection and mitigation from a remote gNB caused by the atmospheric ducting effect. The DL transmission from a remote gNB interferes with UL reception at victim gNB, as propagation delay experienced by a transmission from the remote gNB is larger than the guard period between DL and UL.

In Rel-16 RIM framework, RIM-RS is transmitted by the gNBs which carries the set ID of the transmitting gNB. Additionally, RIM-RS is used for conveying information about presence of ducting phenomenon and whether the applied interference mitigation is sufficient or not.

In dynamic/flexible TDD operation, gNBs may experience CLI from neighboring gNBs and this inter-gNB CLI will dependent on Tx/Rx beams used at the gNBs, especially in FR2. In Rel-16 RIM framework, there is no support for beam-based interference detection. In turn, Rel-16 RIM will have limited utility for interference management in dynamic/flexible TDD which may require advance coordination for beam management between neighboring gNBs.

Observation 12: In Rel-16 RIM framework, this no support for beam-based interference detection and mitigation which may be needed for reducing inter-gNB CLI in dynamic/flexible TDD.

Observation 13: Rel-16 RIM-RS is used for conveying information about presence of ducting phenomenon and sufficiency of the applied interference mitigation. It was not intended for enabling inter-gNB CLI channel measurement.

## Possible enhancement

Inter-gNB coordination can help reduce impact of inter-gNB CLI in dynamic/flexible TDD. Coordination schemes may include RS transmission for inter-gNB CLI measurement. Figure 6-1 illustrates the Tx/Rx beam-pairs for aggressor and victim gNBs.

Figure 6-1 Tx/Rx inter-gNB beam-pairs

Proposal 6: Support of inter-gNB coordination schemes for inter-gNB CLI mitigation in dynamic/flexible TDD to identify compatible inter-gNB beam pairs, which is enabled by inter-gNB CLI measurement and reporting per candidate DL/UL beam pair.

One possible enhancement for inter-gNB CLI mitigation in dynamic/flexible is to optimize Tx beamforming and Rx nulling at the gNBs based on knowledge of inter-gNB CLI channel.

Proposal 7: Support of inter-gNB CLI channel measurement and reporting to neighbouring gNBs for enabling Tx/Rx beamforming or nulling.

# Conclusion

In this contribution, we discussed the feasibility and potential enhancement for dynamic/flexible TDD. Below are the summary of the observations and proposals.

Observation 1: For FR1, deployments scenario with large Tx Power BS suffers from inter-gNB interference.

- In general, inter-UE CLI is not an issue except for macro-to-indoor deployment.

Observation 2: For FR2, Dynamic TDD is possible under careful assumption of layout and power parameterization to avoid inter-gNB interference.

Observation 3: Rel-18 study on potential enhancement of dynamic TDD suggests utilizing the outcome of Rel-15 and Rel-16 studies outcome avoid repetition of same discussion, e.g., inter-operator Dynamic TDD coexistence study.

Observation 4: SBHD can enable dynamic TDD and mitigate the impact of inter-gNB CLI.

Observation 5: SBHD-based dynamic enables flexible adaption of slots direction based on traffic which leads to reduced latency and improved UL coverage.

Observation 6: Link budget analysis shows that SB-based dynamic TDD is feasible for macro-cell deployment.

Observation 7: Qualcomm OTA test network validated the feasibility of dynamic TDD in macro-cell deployment using subband half-duplex.

Observation 8: In FR2, Dynamic TDD with misaligned slots format is possible where CLI could be mitigated with proper beam-pair selection and lower Tx power.

Observation 9: Rel-16 CLI reporting is based on L3 which has limited flexibility and slow adaptability

- This leads to increased latency in CLI reporting which is not suitable for fast adaptation to mitigate CLI

- L1/L2 report can be obtained by gNB-DU with lower latency, in turn it can better reflect current CLI

- L1 report can be sent on-demand enabling fast L1 beam adaptation

Observation 10: In Rel-16 CLI framework, there is no dedicated signalling or configuration of QCL-D for CLI measurement, hence not suitable for enabling CLI-aware beam management

Observation 11: Rel-16 CLI framework does not support subband CLI reporting, i.e., reporting CLI for one or more subbands in the measurement bandwidth. In SBFD, CLI leakage to adjacent subbands is not uniform over the measurement bandwidth and may require subband CLI reporting.

Observation 12: In Rel-16 RIM framework, this no support for beam-based interference detection and mitigation which may be needed for reducing inter-gNB CLI in dynamic/flexible TDD.

Observation 13: Rel-16 RIM-RS is used for conveying information about presence of ducting phenomenon and sufficiency of the applied interference mitigation. It was not intended for enabling inter-gNB CLI channel measurement.

Proposal 1: The focus of Rel-18 study on potential enhancement for dynamic TDD should be limited to co-channel intra-operator deployment.

Proposal 2: Support subband half-duplex as solution to enable dynamic TDD at least for FR1

Proposal 3: Support L1/L2 based CLI reporting to increase flexibility and reduce reporting latency compared to Rel-16 L3 based framework.

Proposal 4: Support UE Rx beam (QCL-D) configuration and indication per CLI measurement resource for enabling CLI-aware beam management.

Proposal 5: Support subband-based CLI reporting to provide accurate CLI reporting in dynamic TDD in which CLI could be non-uniform across the DL.

Proposal 6: Support of inter-gNB coordination schemes for inter-gNB CLI mitigation in dynamic/flexible TDD to identify compatible inter-gNB beam pairs, which is enabled by inter-gNB CLI measurement and reporting per candidate DL/UL beam pair.

Proposal 7: Support of inter-gNB CLI channel measurement and reporting to neighbouring gNBs for enabling Tx/Rx beamforming or nulling.

# References

- 3GPP TSG RAN Meeting #94e, RP-213591 “New SI: Study on evolution of NR duplex operation”, CMCC, Electronic Meeting, Dec. 6 - 17, 2021.

- 3GPP TR 38.828, “Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR”

- Qualcomm MWC 21 Demonstration of subband half-duplex OTA test network.