3GPP TSG RAN WG1 Meeting #109-e	                                                                          R1-2205033

e-Meeting, May 9th – May 20th, 2022

Agenda item:    9.4.1.1

Source:              Qualcomm Incorporated

Title:                  Channel access mechanism for Sidelink on Unlicensed Spectrum

Document for:  Discussion and Decision

# Introduction

In RAN #95, a Rel-18 study item for supporting sidelink on unlicensed spectrum has been approved [1]. The objective of the SI includes the following

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4]

- Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation

  - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation

    - No specific enhancements for Rel-17 resource allocation mechanisms

    - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval.

- Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum

  - The existing NR sidelink and NR-U channel structure shall be reused as the baseline.

- No specific enhancements for existing NR SL feature

- The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98

In this paper we tackle the channel access design for Rel-18 sidelink over unlicensed (SL-U), which will based on Rel-16 NR-U channel access and the main features of Rel-16/Rel-17 NR sidelink (NR SL). We envision that the main goals of the standardization effort will be centered on high throughput and high channel access efficiency, to support eMBB-like use cases. Some example of commercial use cases include:

- Interactive gaming [2]: e.g., phone to phone (peer to peer) gaming for players in the same room (in-proximity case),

- Cloud/Edge/Split rendering [2]: e.g., phone to XR headset rendering,

- Internet of Everything (IoE) based social networking [2]: e.g., users can seamlessly interact with other local users in a real-time community virtual scene setup,

- In-vehicles communications: e.g., car to displays, mobile to speakers, etc.

Some of the objectives of this paper include how to competitively coexist in the unlicensed spectrum with asynchronous systems such as WiFi, which has been proven to be challenging for synchronous systems such as NR-U and NR SL. Our perspective on the goals of the standardization effort is to design a system that can access the medium quickly, perform long transmission bursts for multiple transport blocks, and share the use of the medium as much as possible between SL-U devices. For these reasons we consider introducing more transmission starting points compared to NR SL, to improve the chances of obtaining the medium in the presence of other RATs. Some of the difficulties that we foresee are posed by the distributed nature of NR SL, and the need to tackle alternative forms of coordination compared to NR-U. For example, we see the potential need of revisiting some of the features of NR SL, like resource selection and reservation in mode 2, to better mix with the uncertainty of channel access with LBT. As for upsides, in sidelink, UEs can talk directly without tunneling data through gNBs, which enables communication between UEs not belonging to the same operator, scalability of solutions for a plethora of devices of diverse complexity, more traffic offloading and overall air interface efficiency.

Figure 1: Examples of high-throughput SL-U use cases.

Figure 2: Examples of high-throughput in-vehicle communications.

# Discussion

## NR-U and SL background

This section recaps some of the most important features from NR-U and SL, that will be called out through the paper.

### NR-U

NR-U design focuses on eMBB-like use cases. It utilizes listen before talk (LBT) as a baseline to access the unlicensed spectrum and comply with regulations [5]. NR-U uses 20MHz as the basic channel access unit (LBT bandwidth) and defines the available RBs in each LBT bandwidth as an RB-set.

The baseline for accessing the channel in NR-U is Type 1 LBT, which randomizes a counter based on a traffic priority class and a contention window (CW) length. After sensing the medium as free (according to a proper energy detection threshold) for a period of 16 μs, the device can measure the energy of the channel over the LBT BW (based on RB-sets of 20 MHz) and then performs clear channel assessments (CCAs) over 9 μs contention slots to reduce the counter. When the countdown is completed, the device, if ready, can start the transmissions. If not ready, an additional LBT (Td+Tsl=25 μs) needs to be cleared before the desired transmission starting point. Otherwise, the channel access would be declared to be failed and the procedure should start over. This additional LBT was introduced due to the synchronous nature of NR-U as a method to avoid inter-RAT collisions. In some cases, a device from another RAT (e.g., a WiFi node, which can start transmissions asynchronously) could complete its LBT and start transmitting before the NR-U node’s next transmission starting point. The additional LBT allows the NR-U node to detect this case and defer the access.

Rel-16 NR-U introduced mini-slots level channel access to increase the number of transmission starting points and therefore the chance of starting a COT after the completion of Type 1 LBT, as a method to coexist with WiFi.

This mechanism, and generally, allowing transmissions at more frequent starting points than full slot boundaries, can also provide a certain degree of intra-RAT collision resolution capability. That is, a device that completes the Type 1 LBT earlier, can start transmitting earlier.

Another NR-U feature to support eMBB-like traffic, is the capability of performing back-to-back transmissions, namely transmission burst, over a channel occupancy time (COT) up to a maximum duration (MCOT), at the condition that the activity gaps between two consecutive transmissions are no longer than 16 μs. Transmission bursts can improve channel access efficiency by reducing the number of channel accesses needed to deliver a file (e.g., if composed by multiple transport blocks). This is beneficial compared to accessing the channel for only delivering small portions of a file, which can also reduce the possibility to lose the medium due to the presence of WiFi.

One of the enablers of COT based access is COT sharing, that is, different devices can alternate to use the medium, at the condition of keeping controlled the activity gaps between a transmission and another. NR-U specified different types of COT sharing, which can be allowed via different types of LBT.

NR-U specified COT sharing with Type 2A, 2B, or 2C LBT, with activity gaps of 25 μs, 16<x<25 μs, and up to 16 μs, respectively. The gap can be controlled at the gNB side via the LBT type and CAPC indication in DCI, and the UE that the COT has been shared to can start transmissions with CPE to enforce the activity gap of the desired length.

For gNB-to-UE COT sharing, NR-U introduced a long pause, so that a UE can start transmissions after a long gap (at ms granularity) after the end of a gNB transmission. If such a large gap occurs between any two transmissions, the gNB cannot resume transmissions. In other words, a gNB can resume transmissions after a UE transmission only if the UE started its activity within a 25 μs gap. For the rules for types of LBT and gap lengths between UE and gNB transmission, the rules as in the paragraph above apply (Type 2A, 2B, or 2C LBT, with activity gaps of 25 μs, 16<x<25 μs, and up to 16 μs, respectively).

To solve potential collisions between UEs trying to access with Type 1 LBT for transmitting CG-PUSCH, NR-U introduced up to seven starting points within 1/2/4 symbols for 15/30/60 KHz sub carrier spacing (SCS), respectively. These points are achieved via CPE of the first symbol of the CG-PUSCH transmission.

COT structure information (COT-SI) is a sent by the gNB over DCI 2_0, to provide information about the medium utilization (e.g., duration and available bandwidth), and is used alongside the LBT control field in scheduling DCI to provide coordination to UEs for channel access and COT sharing.

CW adjustment is also introduced in NR-U to reduce collisions in future accesses based on the evaluation of CRC information.

All these aspects and potentially more, could be investigated in the study and potentially adapted for specification of SL-U.

### NR Sidelink

Rel-16/Rel-17 sidelink was designed for V2X traffic, and therefore channel access was built for short and potentially periodic transmissions.

Resources are allocated to UEs in terms of sub-channels and time slots, in transmission and reception resource pools (RPs). Two modes can be used, namely mode 1 and mode 2. In mode 1, resources are allocated to UEs from the gNB, while in mode 2 UEs have to perform a two-step procedure, i.e., resource selection and resource reservation. During the sensing part, the UE detects the level of RSRP of the received reservations and projects it on the reserved resource under test. During the exclusion step the said RSRP is tested against a threshold to assess the acceptability of the level of interference in case a collision may occur in the said resource under test. In the reservation step, a UE, communicates via SL control information (SCI) a limited number of selected resources (up to 2) that can potentially be used for retransmissions. Notably, due to NR SL being optimized for V2X traffic, a UE performs resource selection and reservation of a single TB at a time.

While mode 1 communications are orchestrated via scheduling (DCI 3_0 over PDCCH), in mode 2, resource selection and reservation algorithms were designed to minimize the collisions between UEs for V2X traffic, which presents smaller activity compared to eMBB, and, sometimes, periodicity.

The slot structure in NR SL, may include symbols for transmissions over Uu links for mode 1 UEs coordination, which are allocated at the beginning (PDCCH) and at the end of the slot (PUCCH). The other symbols (PSCCH/PSSCH, PSFCH, and their repetitions for AGC purposes) are the core of SL communications between UEs. Notably, activity gaps are added at the end of the SL portion of the slot, and before PSFCH symbol, to ensure enough time for switching between transmission and reception and vice versa. Further, slots containing the PSFCH symbol occur periodically with a pre-determined period.

Figure 3: Rel-16 SL slot structure (14 symbols) a) without PSFCH, b) with PSFCH

All these aspects should be revisited for the use of NR SL over unlicensed spectrum, also considering the designs adopted by NR-U.

## Medium access uncertainty issue and solutions

Recalling the WID in [1], the Uu operation for mode 1 is limited to licensed spectrum only. Therefore, according also to the slot structure as described in Section 2.1.2, it is reasonable to assume that SL-U slots will not host any Uu downlink or uplink symbol. Therefore, the transmission starting points for PSCCH/PSSCH can occur in symbol #0, whereas for PSFCH can occur in symbol #11.

Sharing the unlicensed spectrum poses new challenges for channel access and requires clearing LBT, which is required by regulations. It is to be noted that WiFi systems have the capability of starting transmission at any point in time in an asynchronous fashion thanks to the detection of a preamble that identifies the subsequent frame and can trigger decoding at receivers. For this reason, systems with fixed, pre-determined, and potentially sparse transmission starting points would end up being disadvantaged when coexisting with WiFi.

Observation 1: The presence of other RATs can impede the capability of SL-U of obtaining access to the medium, especially if such RATs can access asynchronously.

Let’s consider for example a mode 2 UE in a SL-U system that follows the LBT procedure as in NR-U. As it is depicted in the left hand side of Figure 4, the disadvantage manifests, for example, when the UE clears its LBT slightly later than the transmission starting point in a slot 0 (for example, after the boundary between symbol 13 in slot -1 and symbol 0 in slot 0) and has to wait until the next transmission starting point (e.g. at the boundary between symbol 13 in slot 0 and symbol 0 in the following slot 1) before accessing the medium. As soon as the UE is ready to transmit, it will have to clear an additional short duration LBT (see Section 2.1.1) before starting transmissions to determine if the channel is still available. If a WiFi station clears its LBT too before the UE’s transmission starting point in slot 1, then it will occupy the medium. In which case, the UE will find the medium busy during the additional LBT and will lose the COT.

Figure 4: Starvation for SL-U UE in presence of asynchronous interference (left: SL-U attempts to transmit PSCCH/PSSCH; right: SL-U attempts to transmit PSFCH).

### Multiple intra-slot opportunities for channel access

As presented in Section 2.1.1, Rel-16 NR-U introduced mini-slots for increasing the number of transmissions starting points and therefore the reliability of accessing the channel after the completion of Type 1 LBT when coexisting with WiFi. The philosophy behind NR-U channel access, that is, introducing more frequent transmission starting points, can be considered for SL-U.

More detailed designs are proposed in Section 2.1.2 of the companion physical layer design paper [8], which proposes different solutions to trade-off channel access reliability and receiver complexity.

We performed an evaluation in Appendix 1 depicted in Figure 5. The evaluation considers 2 neighboring links in ED range, i.e., a WiFi link and an SL-U link, both with COT duration equal to 6 ms, where the SL-U link is considered under the two hypotheses of

- Full-slot access: a transmission starting point every 14 symbols in a synchronous timeline

- Half-slot access: a transmission starting point every 7 symbols in a synchronous timeline.

The figure depicts user perceived throughput (UPT) [Mbps] for increasing offered load [Mbps], and shows that after the 70 [Mbps] offered load point, the full-slot-based access is broken (almost zero UPT), while the half-slot-based access presents a non-broken behavior with an UPT of 25 [Mbps].

Figure 5: Median UPT [Mbps] for a SL-U link in proximity of a WiFi link for a) full-slot access, b) half-slot access.

From this experiment, it can be observed that even if we simply introduce one more transmission starting point in the middle of the slot, we can substantially improve the performance under asynchronous interference. Therefore, we have the following proposal.

Proposal 1: Study methods to introduce more transmission starting points in SL-U other than full-slot transmission to improve channel access probability.

### Enhancements for multiple consecutive TB transmissions

In Section 2.1.2 we summarized Rel-16 mode 2 resource allocation, which allows selecting resources at the MAC layer for the transmission of one TB at a time. Considering the eMBB-like use cases for SL-U, the transmission of a large file would be inefficient when the transmission needs to be segmented into multiple TBs each transmitted in a slot with a symbol level gap in-between. The regulation (and Rel-16 NR-U channel access rules) will require separate channel access for each transmission when the gap is too long.

Rel-16 NR-U introduced back-to-back transmissions over multiple slots in a COT to improve channel access efficiency (see Section 2.1.2). In practice, a device that clears LBT (e.g., over a 20MHz RB-set), is interested in holding onto the medium as much as possible (e.g., until the maximum COT (MCOT) duration) to satisfy the eMBB traffic requirements.

To illustrate this point, we introduced an evaluation with additional parameters in Appendix 2 and depicted in Figure 6. The evaluation considers 2 neighboring links in ED range, i.e., a WiFi link with COT duration 6 ms, and an SL-U link, where the SL-U link’s UPT is plotted for different values of COT duration (five cases, ranging from 1 slot = 0.5 ms to 12 slots = 6 ms). It is shown that:

- For increasing COT duration, the supported offered load increases, and

- Given an offered load, increasing the COT duration will increase the UPT.

Figure 6: Median UPT [Mbps] for a SL-U link in proximity of a WiFi link for increasing offered load and different COT lengths.

Observation 2: The current NR SL transmission scheme (one TB per channel access) with symbol level gaps between transmissions can be inefficient with eMBB-like traffic. Back-to-back transmissions over multiple slots in a COT can improve channel access efficiency and throughput.

These observations motivate the following proposal.

Proposal 2: Study how to introduce back-to-back transmissions over multiple consecutive slots in SL-U.

More discussions for methods for ensuring continuity of transmissions, also in compliance with regulations, are provided in Section 2.1.1 of the companion physical layer design paper [8].

Another limiting aspect for back-to-back transmissions in SL-U is Rel-16 mode 2 resource allocation. An extensive discussion on the matter and on the role of resource selection and reservation in Rel-18 is provided in Section 2.3.2).

## Intra-RAT collision issue and solutions

In NR-U, the gNB that obtains the COT can schedule UEs with dynamic grants (DGs) or configured grants (CGs). In either case, the gNB can solve via scheduling potential collisions between UEs. Similarly, in SL-U, mode 1 UEs can use resources allocated by a gNB over licensed spectrum [1], which may alleviate intra-operator collisions.

Observation 3: In mode 1, the gNB scheduling via DCI 3-0 can alleviate intra-RAT intra-operator collisions.

However, this does not solve the intra-RAT collision issue for general cases, such as mode 2, or mode 1 with multiple gNBs (especially if they are from different operators and do not coordinate with each other on resource usage). On the contrary, the synchronicity of transmission starting points in current NR SL (see Section 2.1.2) is a major concern for intra-RAT collisions. This is especially true for, but not restricted to, the first transmission, for which the reserving SCI-1 from Rel-16 resource reservation has not been transmitted yet. Furthermore, the burden on resource selection can be higher with eMBB-like traffic than in V2X applications, for which Rel-16 resource selection and reservation was optimized for. This can, in principle, lead to more collisions.

Observation 4: Rel-16 NR SL channel access at synchronous transmission starting points can be prone to intra-RAT collisions especially for high load scenario.

Let us focus for now on the first transmission (for demonstrative purposes) of two mode 2 SL-U UEs, for which there is no resource reservation. The following discussion, which is considering Type 1 LBT as the baseline for channel access, can be extended to later transmissions, retransmissions, and even to COT sharing with other types of LBT. An additional discussion targeting later transmissions of reserved resources will be addressed in Section 2.3.2.

Rel-16/Rel-17 SL UEs don’t have the capability of accessing the channel at a granularity that is finer than the full slot. That is, if multiple UEs intending to perform PSCCH/PSSCH transmission complete their Type 1 LBT in the early part of such a slot, they would possibly experience collision at the transmission starting point at the beginning of the next slot.

In fact, after the countdown part of the Type 1 LBT is completed in slot 0, the additional LBT before the aligned transmission starting points of the two UEs (as outlined in Section 2.1.1) is incapable of preventing collision at the slot boundary (see Figure 7).

Figure 7: Intra-RAT collisions with current LBT in SL-U.

In order to support Observation 4, we introduced an evaluation with additional information in Appendix 3 and summarized by Table 1. The evaluation considers an SL-U deployment where UEs are randomly dropped in an indoor office and associated in pairs according to an RSRP threshold (i.e., -72 dBm). Two loading points are compared. It can be noted that increasing the load directly impact resource utilization (RU) and intra-RAT collision ratio.

Table 1: Resource utilization and intra-RAT collision ratio in SL-U network.

### Channel access at contention slot granularity

On the collision resolution capability of NR-U, it is partially handled by the Type 1 LBT at the contending node side. In NR-U, the contending node can access the channel with mini-slots (see Section 2.1.1), which allows, in principle, for a node that completes LBT earlier to occupy the channel near to the termination of the Type 1 LBT. This mechanism provides a certain degree of intra-RAT collision resolution capability, in layman terms, a node that clears the LBT earlier can occupy the medium earlier, thus potentially blocking other neighboring nodes from clearing their LBT.

Furthermore, NR-U applied the same principle for UEs accessing the channel to transmit CG-PUSCH. NR-U specified up to seven starting points for CG-PUSCH (multiple starting points at 9 μs granularity before the first symbol and early transmission with CPE, as outlined in Section 2.1.1). These ideas can in principle be applied in SL-U too.

One possible design for an intra-RAT collision resolution method based on LBT is depicted in Figure 8. The design follows the principle applied in NR-U for CG-PUSCH (see Section 2.1.1). That is, divide symbol #13 and symbol #0 into 9 μs contention slots, each UE can select one of those contention slots as transmission starting point according to a criterion, synchronize the additional LBT to that point, and start transmission therein upon CCA. If the selected contention slot occurs in symbol #13, transmissions can start with CPE. Conversely, if the selected contention slot occurs in symbol #0, the UE can start transmissions via puncturing the AGC symbol.

Proposal 3: Study starting transmissions in one of a set of contention slots with granularity 9 μs (e.g., with the use of CPEs of different lengths or AGC symbol puncturing).

Figure 8: UE accessing the channel after LBT completion at one of a set of contention slot of length 9 μs around the slot boundary.

### Resource selection and reservation enhancements

In the unlicensed spectrum, the uncertainty introduced by LBT can impact Rel-16 mode 2 resource allocation, which may need enhancements to achieve the high throughput targeted by SL-U. We present the following two arguments:

The first reason is related to the potential inefficiency of the process of resource selection and LBT. In fact, a UE, after firstly selecting a set of resources, could fail LBT, which would trigger an LBT failure at the MAC layer. A possible way of handling such an LBT failure could be to issue resource re-selection at the MAC layer, which may introduce severe overhead and impede throughput.

Observation 5: Rel-16 resource selection can introduce severe overhead due to re-selection in conjunction with LBT failure and lead to low throughput.

Figure 9: Rel-16 resource selection multiple TBs transmissions (with potential re-selections needed due to LBT failures).

The second reason is related to the system-level impact of unused reserved resources due to LBT failure. Consider for example a UE1 that successfully selects resources and broadcasts its reservation SCI 1. Such a UE1 can potentially fail to clear the LBT for channel access corresponding to future reserved resources. Other UEs that decode the reservation SCI 1 from UE1 can exclude the said resources according to the procedure outlined in Section 2.1.2, which can be wasteful. Therefore, we propose to jointly study LBT and mode 2 resource allocation.

Observation 6: Rel-16 resource reservation may negatively impact the system throughput due to unused reserved resources due to LBT failure.

Enhancements to resource selection employing a less restrictive exclusion step could be, in principle, introduced to allow more UEs to select the same resource, and limit the system-wide inefficiency related to LBT uncertainty.

Proposal 4: Study enhancements to the exclusion step in resource selection to limit the system throughput losses due to the issue of unused reserved resources due to LBT failures.

Enhancements to resource reservation to enable the overbooking of multiple consecutive slots could be used which can, in principle, be used to silence other UEs in the reserved time, and therefore facilitate the LBT procedure with respect to the competition from other SL-U UEs and improve LBT reliability. Also, in case a transmission burst is used for multiple TBs in a COT, the reservation of multiple consecutive resources can add extra protection and relief transmissions from intra-RAT interference.

Proposal 5: Study enhancements to resource reservation to reserve multiple consecutive slots as a way to silence other SL-U UEs and benefit in terms of LBT success probability and interference over the COT.

In Section 2.1.1 we discussed COT sharing as one of the key enablers for high throughput in NR-U. Resource reservation could facilitate COT sharing via enabling a distributed coordination between SL-U UEs. For example, a UE that obtains a COT could signal the TDRA and FDRA (potentially different FDRA in different slots) within the COT duration/bandwidth. This way other UEs could exploit this information to select the resources for transmission accordingly and try to access the channel via COT sharing. The UEs may need also a COT structure information (COT-SI) in order to understand the composition of the set of resources that are up for sharing. Further discussions on the COT-SI will be tackled in Section 0.

Proposal 6: Study enhancements to resource reservation to signal the TDRA and FDRA within a COT and facilitate TDM and FDM COT sharing to other SL-U UEs.

Figure 10: Resource reservation indicating TDRA within overall COT to facilitate sharing to other UEs.

## COT utilization

In Rel-16 NR-U, channel utilization is in unit of COTs. The initiating node can perform Type 1 LBT to acquire a COT for its own transmission and can share the COT with responding nodes for their transmission. The same design can be leveraged to Rel-18 SL-U operation.

Rel-16 NR SL specified periodic PSFCH occasions, where the PSFCH symbol is placed after a single symbol gap after the PSCCH/PSSCH (see Section 2.1.2). Rel-18 SL-U may have issues in reliably transmitting the PSFCH. Let us assume Type 1 LBT as the default access method for PSFCH transmission. When one of the periodic opportunities occurs for a UE1, if another neighbor UE2 was already transmitting PSCCH/PSSCH in the same slot, it would be extremely unlikely for UE1 to complete the Type 1 LBT in the one symbol of activity gap. Such an activity gap may even become smaller than one symbol in case UE2 itself wants to transmit a PSFCH. In that case UE2 may adopt some methods to ensure tight activity gap between its first transmission (PSCCH/PSSCH) and the second (PSFCH), to avoid the risk that other devices (potentially from other RATs) complete the LBT and cause interference. This can cause large delays for UE1’s PSFCH and undermine throughput.

Observation 7: PSFCH transmissions in unlicensed spectrum can be unreliable when assuming periodic opportunities from Rel-16 NR SL baseline and Type 1 LBT for channel access, which can degrade throughput.

### COT sharing

Regulations [9] allow a receiver of a device that acquired the channel via LBT to respond to the said device. Rel-16 NR-U COT sharing (see Section 2.1.1) was introduced to achieve high channel access efficiency (by minimizing the number of LBT attempts across the board), resource utilization, and improve system throughput, with the added benefit of facilitating channel access in the presence of other RATs. Such motivations can apply to SL-U as well, nevertheless SL-U presents its own peculiarities due to its distributed nature.

Since per the WID [1] the gNB-to-UE and UE-to-gNB links (namely Uu links) are restricted to licensed spectrum, only sharing over Pc5 links need to be addressed.

UE-to-UE COT sharing can use, in principle, the baseline provided by gNB-to-UE or UE-to-gNB COT sharing from NR-U.

NR SL supports three basic transmission scenarios, which should be considered while studying COT sharing.

Observation 8: To allow COT sharing, the UE that is trying to use shared resources should be a valid receiver of the COT initiator, which should be evaluated separately for unicast, groupcast, and broadcast.

Proposal 7: Conditions can be considered and can be studied to determine when a UE is a valid receiver of a COT initiator UE. The study can include (without being restricted to) the following definitive conditions: 1) decoded SCI, 2) is within a range from the initiator

Some COT sharing behaviors are considered more fundamental and of higher priority for potential specification in SL-U, and are listed in the following:

- COT sharing for PSFCH, which can solve the problem outlined in Observation 7.

  - PSFCH towards COT initiator: can directly exploit Uu baselines

  - PSFCH towards a UE other than the COT initiator: can exploit Uu baselines with the analogy to UE-to-gNB sharing, where the gNB in NR-U can send control messaging to other UEs than the COT initiator (e.g., initiating a COT for CG-PUSCH)

- COT sharing for PSCCH/PSSCH, which can push further the concept of channel access efficiency via back-to-back transmissions (see Observation 2) to more than one UE.

  - PSCCH/PSSCH towards a group of UEs including COT initiator: can directly exploit Uu baselines if the UE that is trying to use shared resources is a valid receiver of the COT initiator.

    - Can be unicast back to the COT initiator or groupcast/broadcast to a group on UEs including the COT initiator.

Proposal 8:  Introduce UE-to-UE COT sharing in SL-U with R16 NR-U as baseline, for at least the following behaviors:

- PSFCH to COT initiator

- PSFCH to UE other than COT initiator

- PSCCH/PSSCH to a group of UEs including the COT initiator

Further, it may be beneficial to study mechanisms to help SL-U UEs to coordinate towards making the most out of the shared COT available resources. For example, we introduced Proposal 6 as a method for the UE to provide indications on the resources that it plans to use during the COT. A complementary role would be fulfilled by the signaling of the COT structure information.

Proposal 9: Study how to introduce COT-SI indication in SL-U, to provide indication of the bandwidth and duration of an ongoing COT.

Figure 11: UE to UE COT sharing for PSFCH and PSCCH/PSSCH.

### Resuming transmissions after a long gap

Due to the Rel-16 NR SL slot structure with periodic PSFCH (see Section 2.1.2), there might be a problem related to using transmission burst (see Section 2.1.1) for multiple TBs over a COT, as outlined in Section 2.2.2. In fact, if a UE that is transmitting a PSCCH/PSSCH in a slot with PSFCH does not have a PSFCH to transmit, it may lose the COT and with it the possibility of transmitting another PSCCH/PSSCH in the next slot.

Observation 9: Due to Rel-16 NR SL slot structure with periodic PSFCH, a UE may have issues if attempting to transmit multiple TBs over consecutive slots, in the case of absence of an owned PSFCH to transmit, which can cause the loss of the COT and inefficiency in channel access.

Observation 10: TDM UEs transmissions of PSCCH/PSSCH (UE1) to PSFCH (UE2) to PSCCH/PSSCH (UE1) via COT sharing can facilitate transmissions of multiple TBs from UE1 without performing additional Type1 LBT.

NR-U specified how a UE can transmit after a long gap (ms level) after a gNB terminates transmissions in a gNB initiated COT (see Section 2.1.1). This can be potentially introduced in SL-U for UE to UE COT sharing at least to address the gaps left by a PSFCH resource.

Proposal 10: Study methods to enable for initiating UE to resume transmissions in its COT after a gap longer than 25 μs.

## Evaluation Methodology

The use cases of major interest outlined above involve eMBB-like traffic in potentially densely populated networks, and therefore the goals for optimization will be mainly high throughput and efficiency of channel access. We advocate following earlier studies on NR-U [3,4] as much as possible, and consider modification of the evaluation methodology as needed. Such a study used two scenarios, indoor and outdoor, with two operators. For example, due to the focus on eMBB-like traffic, we can use FTP3 models from [3] for traffic and user perceived throughput (UPT) as the key performance indicator (KPI).

Proposal 11: Reuse evaluation methodology from Rel-16 NR-U study as much as possible with possible adjustments.

Drawing from the WID [1], the study and specification for SL-U should assume Uu links to be limited to licensed spectrum only. Therefore, we propose to restrict the scope of system evaluations to Pc5 links.

Proposal 12: Studies can focus on Pc5 links over the unlicensed spectrum BW.

The performance of SL-U only should be studied in the presence of interference from other RATs. In this regards we can exploit the 2 operators split in [4] to model two RATs at a time. As mentioned before and per the WID [1], extensive studies for coexistence studies between WiFi and NR-U have been carried out in Rel-16 and given that SL-U access will be designed in compliance with regulations and NR-U access schemes, we propose to restrict the scope of system evaluations to SL-U performance, without specifically requiring performance evaluations from the WiFi point of view. Nevertheless, as was developed in Section 2.2, systems operating with asynchronous access (i.e., at any time) pose a real threat to the ability of SL-U of obtaining the use of the medium. Therefore, we can evaluate SL-U in the presence of NR-U or WiFi. Evaluations of different versions of SL-U should not be precluded, as for example

We can assign the two operators to study different cases:

- Case 1: Operator 1 is NR-U and Operator 2 is SL-U, see Figure 12

- Case 2: Operator 1 is WiFi and Operator 2 is SL-U, see Figure 12

- Case 3: Operator 1 is SL-U and Operator 2 is SL-U, see Figure 13

Proposal 13: Study layouts with two operators, where a first SL-U system is evaluated in the presence of one of an NR-U system, a WiFI system, or a second SL-U system.

The topology of NR-U and WiFi can be treated in the same way, and specifically can be obtained by keeping one of the two operators in the layouts from [4], while the second operator is replaced by SL-U, for both indoor (see Figure 12 and Figure 13) and outdoor. Specifically, the SL-U deployment should adopt a uniformly random dropping of UEs, which is detailed later in the section.

Figure 12: Layout for indoor scenario, Case 1 and 2. (1) SL-U with pair topology. (2) SL-U with star topology. Dashed arrows indicate SL-U links. In the figure a = 20m, b = 60m, c = 20m, d = 80 m.

Figure 13: Layout for indoor scenario, Case 3. (1) SL-U with pair topology. (2) SL-U with star topology. Dashed arrows indicate SL-U links. In the figures d = 80m, e = 120m.

Proposal 14: Reuse layouts for indoor and outdoor from the Rel-16 NR-U study, where NR-U and WiFI operators are deployed as in [4], and the SL-U operator (or operators) are deployed uniformly at random in the area.

SL-U topology:

For the SL-U topology we propose to focus on pairs of UEs, to model the peer-to-peer communications in SL-U. Nevertheless, we do not exclude evaluations where UEs are associated in such a way to form stars (i.e., a central UE associated with more than one peripheral UEs), which may be useful for studying the benefits of more coordination in SL-U channel access and communications. The sidelink units (either in pairs or star topology) should be randomly deployed in the area.

To form SL-U links – either between the peer UEs in the network of pairs described in (a), or between the central UE and the peripheral UEs in the network of stars described in (b) – we propose to follow the technique in [5], that associates UEs for communications only if they have RSRP above an association threshold. Therefore, we propose uniform dropping of UEs in the area of interest, with RSRP thresholding to associate two UEs, and re-dropping if the RSRP thresholding fails. The number of elementary units (pair or star) and the RSRP threshold can be determined according to the level of interference to be captured in the study.

Proposal 15: Support the following SL-U topology in system evaluation

- Pair-wise SL-U topology (baseline), where pairs for SL-U UEs are dropped uniformly in the region of interest

- Star topology (optional), where the central SL-U UE is associated with multiple peripheral SL-U UEs

For the remaining general evaluation assumptions we refer to [4], with potential adjustments, as summarized in the following tables.

Table 2: Evaluation parameters for indoor scenarios

| Carrier Frequency | 5 or 6 GHz |
| --- | --- |
| Carrier Channel Bandwidth | 20MHz baseline, 80 MHz optional |
| Number of carriers | 1 |
| Number of users per operator | Operator 1 (NR-U/WiFi): 5 UEs/STAs associated per each gNB/AP per 20 MHz Operator 2 (SL-U pairs): 3, 5, or 10 pairs of UEs per 20 MHz Operator 2 (SL-U stars): 3 Hub UEs with 5 associated peripheral UEs per 20MHz |
| SCS | 30 KHz or 15 KHz |
| Channel Model | NR InH Mixed or Open Office |
| gNB/AP Tx Power | 23 dBm |
| NR-U UE/STA | 18 dBm |
| SL-U UE | 18 dBm (for 5GHz carrier frequency) 14 dBm (for 6GHz carrier frequency) |
| gNB/AP Antenna gain | 0 dBi |
| UE/STA Antenna gain | 0 dBi |
| gNB/AP Noise Figure | 5 dB |
| UE/STA Noise Figure | 9 dB |
| Minimum received power from serving cell for UE dropping | -82 dBm |
| UE receiver | MMSE-IRC as baseline |
| gNB/AP antenna array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | Baseline Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ Optional Tx/Rx: (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | Use 36.889 Table A.1.1. Note: Results based on the mixed traffic models can be used to determine the design. |
| UE/STA to UE/STA link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |
| gNB to gNB link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |

Table 3: Evaluation parameters for outdoor scenario

| Carrier Frequency | 5 or 6 GHz |
| --- | --- |
| Carrier Channel Bandwidth | 20MHz baseline, 80 MHz optional |
| Number of carriers | 1 |
| Number of users per operator | Operator 1 (NR-U/WiFI): 5 UEs/STAs associated per each gNB/AP per 20 MHz Operator 2 (SL-U pairs): follow 36.843 Option 6 Operator 2 (SL-U stars): follow NR-U/WiFi |
| SCS | 30 KHz or 15 KHz |
| Channel Model | NR Umi street canyon |
| gNB/AP Tx Power | 23 dBm |
| UE (NR-U) /STA | 18 dBm |
| UE (SL-U) | 18 dBm (for 5GHz carrier frequency) 14 dBm (for 6GHz carrier frequency) |
| gNB/AP Antenna gain | 0 dBi |
| UE/STA Antenna gain | 0 dBi |
| gNB/AP Noise Figure | 5 dB |
| UE/STA Noise Figure | 9 dB |
| Minimum received power from serving cell for UE dropping | -82 dBm |
| UE receiver | MMSE-IRC as baseline |
| gNB/AP antenna array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | Baseline Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ Optional Tx/Rx: (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | Use 36.889 Table A.1.1. Note: Results based on the mixed traffic models can be used to determine the design. |
| UE/STA to UE/STA link pathloss model | Directly use UMi street canyon pathloss model with proper d_3D with UMi street canyon LOS probability |
| gNB to gNB link pathloss model | Directly use UMi street canyon pathloss model with proper d_3D with UMi street canyon LOS probability |

# Summary

Observation 1: The presence of other RATs can impede the capability of SL-U of obtaining access to the medium, especially if such RATs can access asynchronously.

Proposal 1: Study methods to introduce more transmission starting points in SL-U other than full-slot transmission to improve channel access probability.

Observation 2: The current NR SL transmission scheme (one TB per channel access) with symbol level gaps between transmissions can be inefficient with eMBB-like traffic. Back-to-back transmissions over multiple slots in a COT can improve channel access efficiency and throughput.

Proposal 2: Study how to introduce back-to-back transmissions over multiple consecutive slots in SL-U.

Observation 3: In mode 1, the gNB scheduling via DCI 3-0 can alleviate intra-RAT intra-operator collisions.

Observation 4: Rel-16 NR SL channel access at synchronous transmission starting points can be prone to intra-RAT collisions especially for high load scenario.

Proposal 3: Study starting transmissions in one of a set of contention slots with granularity 9 μs (e.g., with the use of CPEs of different lengths or AGC symbol puncturing).

Observation 5: Rel-16 resource selection can introduce severe overhead due to re-selection in conjunction with LBT failure and lead to low throughput.

Observation 6: Rel-16 resource reservation may negatively impact the system throughput due to unused reserved resources due to LBT failure.

Proposal 4: Study enhancements to the exclusion step in resource selection to limit the system throughput losses due to the issue of unused reserved resources due to LBT failures.

Proposal 5: Study enhancements to resource reservation to reserve multiple consecutive slots as a way to silence other SL-U UEs and benefit in terms of LBT success probability and interference over the COT.

Proposal 6: Study enhancements to resource reservation to signal the TDRA and FDRA within a COT and facilitate TDM and FDM COT sharing to other SL-U UEs.

Observation 7: PSFCH transmissions in unlicensed spectrum can be unreliable when assuming periodic opportunities from Rel-16 NR SL baseline and Type 1 LBT for channel access, which can degrade throughput.

Observation 8: To allow COT sharing, the UE that is trying to use shared resources should be a valid receiver of the COT initiator, which should be evaluated separately for unicast, groupcast, and broadcast.

Proposal 7: Conditions can be considered and can be studied to determine when a UE is a valid receiver of a COT initiator UE. The study can include (without being restricted to) the following definitive conditions: 1) decoded SCI, 2) is within a range from the initiator

Proposal 8:  Introduce UE-to-UE COT sharing in SL-U with R16 NR-U as baseline, for at least the following behaviors:

- PSFCH to COT initiator

- PSFCH to UE other than COT initiator

- PSCCH/PSSCH to a group of UEs including the COT initiator

Proposal 9: Study how to introduce COT-SI indication in SL-U, to provide indication of the bandwidth and duration of an ongoing COT.

Observation 9: Due to Rel-16 NR SL slot structure with periodic PSFCH, a UE may have issues if attempting to transmit multiple TBs over consecutive slots, in the case of absence of an owned PSFCH to transmit, which can cause the loss of the COT and inefficiency in channel access.

Observation 10: TDM UEs transmissions of PSCCH/PSSCH (UE1) to PSFCH (UE2) to PSCCH/PSSCH (UE1) via COT sharing can facilitate transmissions of multiple TBs from UE1 without performing additional Type1 LBT.

Proposal 10: Study methods to enable for initiating UE to resume transmissions in its COT after a gap longer than 25 μs.

Proposal 11: Reuse evaluation methodology from Rel-16 NR-U study as much as possible with possible adjustments.

Proposal 12: Studies can focus on Pc5 links over the unlicensed spectrum BW.

Proposal 13: Study layouts with two operators, where a first SL-U system is evaluated in the presence of one of an NR-U system, a WiFI system, or a second SL-U system.

Proposal 14: Reuse layouts for indoor and outdoor from the Rel-16 NR-U study, where NR-U and WiFI operators are deployed as in [4], and the SL-U operator (or operators) are deployed uniformly at random in the area.

Proposal 15: Support the following SL-U topology in system evaluation

- Pair-wise SL-U topology (baseline), where pairs for SL-U UEs are dropped uniformly in the region of interest

- Star topology (optional), where the central SL-U UE is associated with multiple peripheral SL-U UEs

# References

- 3GPP RP-220300, NR sidelink evolution.

- 3GPP TR 22.842, Study on network controlled services

- 3GPP TR 36.889, Study on Licensed-Assisted Access to Unlicensed Spectrum

- 3GPP TR 38.889, Study on NR-based access to unlicensed spectrum

- 3GPP TR 36.843, Study on LTE D2D proximity services

- 3GPP TR 37.213, Physical layer procedures for shared spectrum channel access

- 3GPP TR 38.211, Physical channels and modulations

- 3GPP R1-2205034, Channel access mechanism in unlicensed spectrum for NR sidelink evolution, Qualcomm Inc.

- ETSI EN 301 893, 5 GHz RLAN

# Appendix 1

Refer to the layout in Figure 14 and the simulation parameters in Table 4.

Figure 14: Layout Simulation 1, representing two neighbor links, one SL-U and one WiFi.

Table 4: Simulation 1 Parameters

# Appendix 2

Refer to the layout in Figure 14 and the simulation parameters in Table 5.

Table 5: Simulation 2 Parameters (changes from Simulation 1)

# Appendix 3

Refer to the layout in Figure 15 and the simulation parameters in Table 4.

Resource utilization (RU) is defined as # slots with transmissions# slots in the simulation . The intra-RAT collision ratio is defined as AB where for each slot of the simulation A is incremented by K if K is the number of pairs of links for which the RSRP between the devices performing the LBT is above threshold (threshold is assumed -72 dBm). B is the count of total number of COTs in the simulation.

Figure 15: Layout Simulation 3, SL-U network with 12 pairs. Left: RSRP threshold = -72 dBm. Right: RSRP threshold = - 82 dBm.