3GPP TSG-RAN WG1 #109-e	R1-2205034

e-Meeting, May 9th – May 20th, 2022

Agenda item:	9.4.1.2

Source: 	Qualcomm Incorporated

Title: 	Physical Channel Design for Sidelink on Unlicensed Spectrum

Document for:	Discussion/Decision

# Introduction

In RAN #95, a Rel-18 study item for supporting sidelink on unlicensed spectrum has been approved [1]. The objective of the SI including the following

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4]

- Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation

  - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation

    - No specific enhancements for Rel-17 resource allocation mechanisms

    - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval.

- Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum

  - The existing NR sidelink and NR-U channel structure shall be reused as the baseline.

- No specific enhancements for existing NR SL feature

- The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98

As a part of the study item, the study on physical channel design framework is focused on the required changes to NR sidelink physical channel structure and procedures to operate on unlicensed spectrum.

NR-U design has been focusing on the eMBB-like use cases. Supporting SL operations in unlicensed band allows more eMBB use cases of direct UE to UE communication without tunneling data through gNB, e.g. phone to XR headset for rendering, phone to phone gaming (peer 2 peer) for people in the same room, phone to wearable. This is particularly useful for UEs not belonging to the same operator.

One aspect of our design touches upon the optimization on slot structure to transmit as early as possible and occupy the medium continuously for eMBB data burst after clearing the LBT. This allows SL-U to better coexist with WiFi and NR-U nodes. NR-U introduces search space switching which allow the transmitter to transmit with mini-slot after clearing the LBT instead of waiting until slot boundary so other node cannot jump in the gap and steal the COT.

In some unlicensed bands (e.g., 5GHz), there are transmission PSD limitations. For example, the regulation on PSD limitation with 5GHz band is listed below [2]:

- 5150-5350 MHz with TPC: 10dBm/MHz

- 5250-5350 MHz without TPC: 7dBm/MHz

- 5150-5250 MHz without TPC: 10dBm/MHz

- 5470-5725 MHz with TPC: 17dBm/MHz

- 5470-5725 MHz without TPC: 14dBm/MHz

Under the PSD limitation, in order for a node to transmit with higher power, it needs to occupy wider bandwidth. This can lead to wasted resources unless careful design is considered. NR-U uplink channel introduces interlaced waveform which allows the UE to transmit at full transmit power and allow multi-user multiplexing on different interlaces. Similarly, we will introduce interlaced waveforms for PSCCH, PSSCH and PSFCH.

Just as NR-U, the SL-U needs to coexist with WiFi in 5/6GHz unlicensed band and the WiFi channel access is in 20MHz unit. NR-U uses 20MHz as the basic channel access unit (LBT bandwidth) and defines the available RBs in each LBT bandwidth as an RB-set. Hence, SL-U may reuse the concept of RB-set and necessary enhancements for SL-U resource allocation based on interlace and RB-set are considered.

Furthermore, to satisfy the minimum OCB requirement in 5GHz band, S-SSB waveform may need to be reconsidered. In NR-U, the SSB transmission relies on PDCCH/PDSCH multiplexing to fulfill the OCB constraint. The Rel’16 full slot S-SSB does not allow PSCCH/PSSCH multiplexing as S-SSB are excluded from the resource pool. One may want to consider reuse the NR SSB pattern such that PSCCH/PSSCH multiplexing is possible, i.e., include new S-SSB pattern in the resource pool

To accommodate eMBB traffic, there may also be a need to introduce new PSFCH which can carry HARQ codebook (reusing existing PUCCH formats) and type-1/2/3 HARQ codebook could be considered.

Below is the summary of NR-U design aspects that SL-U could reuse from

- Mini-slot switching to allow transmitter to hold the channel early when clearing the LBT

- Interlace waveforms for PSD and minimum OCB

- RB-set configurations

- Resource allocation: FDRA for both interlace and RB-set

- PUCCH format 0 (Rel’16 SL only supported format) and format 2

- SSB pattern which allows PDCCH/PDSCH multiplexing to satisfy the minimum OCB constraint

- Multiple SSB candidate locations (DRS window)

# Discussion

## Slot Structure

The Rel’16 SL transmission is slot based in which any transmission needs to start on a slot boundary. SL nodes may face starvation issue in the presence of asynchronous WiFi since WiFi could transmit right away once it clears the LBT and may block the SL transmission. In this section, we discuss the optimization in slot structure to fill the Tx/Rx gaps or PSFCH gaps for long data burst transmission and creating more transmission starting positions with mini-slot resource pool, which allows UE to transmit as early as possible after clearing the LBT.

### Optimization for contiguous data burst

For unlicensed band operation, a SL node may prefer continuous transmission of data burst to avoid losing the COT. If there is a gap ≥16/25us in the middle of the transmission burst, an additional type 1 LBT is required. The Rel’16 SL slot structure contains a gap symbol at symbol #13 if the slot does not have PSFCH, or at symbols #10 and #13 if PSFCH is included in the slot as shown in Figure 1.

Additionally, for long data burst in eMBB traffic, some optimization in control signaling, DMRS, AGC and gap symbols could be considered to improve the spectral efficiency.

Figure 1: Rel'16 SL slot structure a) without PSFCH, b) with PSFCH

#### Close the gap between two contiguous slots

The Rel’16 SL slot structure has one symbol gap at the end of the slot for Tx/Rx switching. The transmitter (Txer) may prefer to occupy the gap symbol between two adjacent slots for contiguous transmission. One alternative is to consider using CP extension (CPE) to either fill the entire gap symbol (full AGC symbol repetition) or part of it (but at least long enough to make sure the gap is less than 16us, to keep the contiguous access of the channel).

Proposal 1: Study how to use CP extension (CPE) of the AGC symbol to fill into the gap symbol of the previous slot so that the one symbol transmission gap in between the slots becomes narrower

The gap and AGC symbols between two contiguous slots within the data burst can be used for data transmission to improve the spectral efficiency. Instead of filling the gap symbol with CPE, one can rate match PSSCH to the gap symbol. Considering the AGC is already trained at the beginning of the burst and no other close by transmitter can clear LBT and start transmission in the middle of the burst, the transmitter may also choose to rate match PSSCH to the AGC symbol after the 1st slot of the burst. The same principles can be also applied in the case of multiple contiguous slots, discussed later in Sec.2.1.1.4.

Proposal 2: In addition to CPE method, study how to rate match PSSCH into the gap symbol or/and AGC symbol

Figure 2: Closing the gap in between slots: a) use of CP extension b) PSSCH rate matching

#### Close the gap before the PSFCH symbol

If the slot contains the PSFCH, there is an additional gap symbol at symbol #10. The PSFCH transmission for Ack/Nak may want to share the same COT with the PSCCH/PSSCH in the same slot and the data burst may want to continue with the same COT after the PSFCH. In this case, we may need to close the gap symbol #10.

For the PSFCH transmission, from the experience of NR-U, the following are possible

- Share another SL transmission COT with Type 2C LBT if gap is no more than 16us

- Share another SL transmission COT with Type 2B LBT if gap is 16us

- Share another SL transmission COT with Type 2A LBT if gap is longer than or equal to 25us

- Acquire its own COT with Type 1 LBT

For PSFCH to share another SL transmission’s COT, small gap in symbol #10 could prevent WiFi from jumping in and block the PSFCH transmission. If the COT initiating Txer wants to resume the COT in the following slot after the PSFCH symbol, the gap at symbol #10 needs to be ≤16 us and CPE is needed to fill the gap at symbol #13. To allow the PSSCH transmitter to control the gap, SCI triggering the PSFCH transmission may be responsible to indicate the channel access type and CP extension duration for PSFCH transmission just as the DCI indicates the channel access type and CPE for the PUCCH in NR-U.

Proposal 3: For the gap before PSFCH, study how to use CP extension to maintain the right length gap to match the channel access type or keep the COT

Figure 3: Close the gap before the PSFCH: a) 25us gap, b)16us gap, c) no CPE

#### Close the gap in the PSFCH symbol

For the eMBB case, the Txer schedules TBs in a burst and may not expect Ack/Nak at the beginning of the bursts. However, the PSFCH occasions are common across all links in the network, there could be some un-used PSFCH instances at the beginning of the data burst and could cause COT termination. These PSFCH occasions could potentially be used by other links, but the COT initiating transmitter may or may not transmit/receive PSFCH to/from other SL nodes. If the COT initiator cannot guarantee or assume there is a transmission in the PSFCH occasion with ≤16us gap, either from the COT initiating transmitter or other SL nodes, the COT will be terminated and additional LBT overhead would be undesirable. In Figure 4, the COT-initiating UE (UE#0) is transmitting 4 TBs to its receiver. TB #0 and #1 are associated with the 2nd  PSFCH instance and the 1st PSFCH instance is unused by the UE#0 and its Rxers. Then, the 1st COT of UE#0 is terminated at the 1st PSFCH instance and additional LBT or another COT may be required for UE #0 to finish the data burst transmission.

Figure 4: Unused PSFCH instances causes additional LBT

Observation 1:  If there exists an unused PSFCH instance in the middle of data burst, additional LBT may be required by SL transmitter to continue the remaining transmission

Observation 2: If the COT-initiating transmitter could transmit or its receiver could be scheduled to transmit some signals at the unused PSFCH instances, we can reduce the LBT overhead

#### Multiple contiguous slots transmission

Towards making the most out of a cleared LBT attempt, a transmitter may utilize multiple contiguous slots for transmission of large size data. This avoids multiple LBTs for multiple standalone slots and improves channel access efficiency. The gap reduction mechanisms discussed above are applied the same for all slots within a burst (except, possibly the last gap symbol of the last transmitted slot, to allow for RX switching).

With multiple contiguous slots per transmissions there is potential for transmission overhead reduction.

- Instead of having each slot transmitting PSCCH as per Rel’16, a single PSCCH can appear in the first slot of the burst with a single SCI scheduling/reserving all slots in the burst (multi-TTI SCI grant), as shown in the example of Figure 5. This may require a change from the PSCCH-to-PSFCH resource mapping in Rel’ 16 to a PSSCH-to-PSFCH  resource mapping

- DMRS overhead reduction: For example, with all slots directed towards the same UE(s), reduction of DMRS symbols in some slot(s) may be possible under low mobility conditions with no performance cost, leveraging DMRS symbols available in adjacent slots

- Utilization of gap and AGC symbols for PSSCH: last gap symbol between consecutive slots can be utilized as and additional PSSCH symbol and the AGC symbol of all slots following the first in the burst could also be used for PSSCH transmission. These two features may be activated/indicated in first slot SCI

Observation 3: Multi-slot transmission improves efficiency of LBT-restricted channel access and provides opportunities for overhead reduction

Proposal 4: Study multiple contiguous slot transmissions with minimum signaling overhead

PSSCH

PSSCH

PSSCH

PSSCH

Embedded SCI schedules 4 TBs for single receiver

No gap or 16/25us gap

SCI-1

SCI-2

Figure 5: Multi-TTI SCI grant

### Mini-slot/sub-slot structure for LBT channel access

SL transmission is slot based in which any transmission needs to start on a slot boundary. However, WiFi can transmit immediately upon clearing the LBT and may block the SL-U transmission. The SL-U may face channel access starvation issue in the presence of WiFi in some scenarios [3]. NR-U has introduced mini-slot and slot search space switching which allows the transmitter to transmit with mini-slot after clearing the LBT instead of waiting until slot boundary.

To create more transmission starting position across a SL slot, we can introduce mini-slot or sub-slot structure with AGC symbol and PSCCH/PSSCH which is like a miniature of a full SL slot. For example, the SL transmitter could be configured with full slot resource pool (RP#0 in Figure 6) and mini-slot RP or sub-slot RP (RP#1). In a mini-slot RP, the full-slot is partitioned into multiple (e.g., two) mini-slots to create multiple starting points. A sub-plot RP is the same as a mini-slot RP with the first mini-slot whose starting point coincides with the starting point of the full slot RP omitted. The combination of the two types of RPs provides the UE with additional starting points. The transmitter may choose the RP with the earliest transmission starting point after clearing the LBT and switch to full slot-based RP after the 1st slot in the COT for better spectrum efficiency as shown in  Figure 7.

The more starting points we introduce, the more PSCCH candidate locations the receiver need to monitor. It is basically a tradeoff between the receiver PSCCH decoding complexity and LBT channel access successful rate. We envision two starting positions could offer a good tradeoff.

There may be some concerns that mini-slot access UE’s transmission may cause AGC problem for UEs trying to receive a full-slot. The mini-slot transmission is conditioned on LBT, so if the mini-slot transmitter is very close to the transmitter with ongoing full-slot transmission, the LBT will gate the mini-slot transmission if the mini-slot transmitter wants to start in the middle of the ongoing full-slot transmission. Also, WiFi will anyway start its transmission anywhere in the middle of an ongoing full-slot SL transmission if it clears the LBT and do nothing to protect SL-U. Hence, the AGC problem is the nature of the unlicensed band.

Observation 4: Slot-based access in NR Sidelink reduces channel access opportunities compared to other RATs

Observation 5: Mini-slots or sub-slots can increase channel access opportunities

Proposal 5: Study “mini-slot or sub-slot” channel access as a method to increase channel access opportunities

Mini-slot

1 slot

RP#0

RP#1

1 slot

RP#0

RP#1

sub-slot

Figure 6: a) mini-slot structure, b) overlapping mini-slot/subslot RP (RP#1) and slot-based RP (RP#0)

Figure 7: Txer starts the COT with earliest starting position and switches to full-slot transmission

## Physical Waveform

### RB-set configuration considerations

In NR-U, the DL/UL RB-sets are RRC configured by gNB per UE. The RB-set config specifies the guard band in between the 20MHz LBT subbands. For SL mode 2, we cannot always rely on gNB for the SL RB-set configurations. Two options could be considered

Option 1: Common RB-set config for all SL nodes in the network via UE profile or gNB config.

Option 2: Allow RB-set config to be set up per link via RRC messages

Option 1 seems to be simple solution for deployment. However, different UEs from different vendors could have different implementations and guard band requirements could be different. Having a conservative common RB-set could be a solution but sacrifices the spectral efficiency as the interlace RBs in the guard band cannot be used for transmission even when two SL nodes in a link with good implementation do not require that much of a guard band.

For option 2, distributed SL UE to UE communication needs to have common assumptions of RB-set config before RB-set configs can be delivered via RRC message exchanges. Hence, some common minimum RB-set config needs to be there via network config or UE profile so two nodes can communicate before the per link RB-set config is available. The common minimum RB-set configuration could be the intersection of all the supported RB-set configs in the specification.

Observation 6: The RB-set config for 20MHz subbands could be either common config for all SL nodes in the NW or configurable per link via RRC message exchange

Observation 7: If RB-set config per link is configurable, one may need to define the common minimum RB set for a 20MHz subband for the initial RRC message exchange to set up the RB-set config

Figure 8: Common minimum RB-set config

### PSCCH and PSSCH

#### Interlaced waveforms

To maximize the transmit power under the PSD limit and fulfil the minimum OCB requirement in the unlicensed band, NR-U has introduced the interlaced waveform. Interlaced waveforms for PSCCH/PDDSCH could directly leverage from the NR-U UL waveforms. For 15/30KHz SCS, PSCCH/PSSCH has 10/5 interlaces for resource allocation.

In NR-U, the basic unit of frequency resource allocation is one interlace and one RB-set. The basic unit for SL resource allocation in frequency domain is one subchannel. For SL-U, it is natural to reuse the basic unit of resource allocation from NR-U and hence one can define 1 interlace in one subband (RB-set) to be one subchannel. Depending on the guard band setting, the number of RBs in a subchannel is not fixed. It is approximately 10 RBs but could be more or less.

Proposal 6: Study interlaced waveforms for PSCCH/PSSCH

Proposal 7: Study the basic unit for resource allocation in terms of interlace and RB-set

Figure 9: Interlace waveform for PSCCH/PSSCH

#### PSCCH resource mapping

As the basic resource allocation unit for SL-U is 1 interlace in one RB-set, one PSCCH will only occupy 1 interlace in one RB-set as in Figure 10. Within a 20MHz LBT subband, the PSCCH is rate matched to the common (minimum) RB-set. Even if we allow RB-set to be configurable per link via RRC messages, confining the rate matching to the common minimum RB-set is necessary since PSCCH contains reservation information need to be understood by other SL nodes with different RB-set configs.

Proposal 8: Study PSCCH resource mapping in terms of interlace and RB-set

Figure 10: PSCCH resource mapping

#### PSSCH resource mapping

The FDRA in SCI-1 could allocate multiple interlaces and RB-sets for PSSCH transmission in which the intra-cell guard bands in the middle of scheduled RB-sets could be occupied by PSSCH as shown in Figure 10.

In Rel’16 SL, the first subchannel that PSSCH occupies is the leading/starting subchannel. The scheduling PSCCH is also contained in the leading subchannel. Hence, the PSSCH in SL-U could TDM or/and FDM with the scheduling PSCCH. And the PSCCH interlace and RB-set will be the leading/starting interlace and RB-set for the scheduled PSSCH interlace(s) and RB-set(s).

In case the RB-set is configurable per link, the SCI-2 is rate matched to the common minimum RB-set(s) (instead of the configured RB-set(s)) indicated by SCI-1. From receiver point of view, different transmitters could apply different RB-set config and the receiver needs to identify the transmitter’s ID in SCI-2 to adjust the RB-set assumption. If the NW only allows a global common RB-set config per 20MHz subband as in option 1 in Sec. 2.2.1, then the SCI-2 is rate matched to the common RB-set(s) indicated by SCI-1.

For the data PSSCH REs, data could be rate matched to the configured RB-set of the link for better efficiency if the link configures the RB-set via initial RRC message exchanges. Otherwise, data PSSCH is rate matched to the common RB-set(s) or common minimum RB-set(s) scheduled by SCI-1

Observation 8: It is natural for PSSCH to TDM or/and FDM with the scheduling PSCCH and the PSCCH interlace/RB-set being the leading interlace/RB-set of the scheduled PSSCH interlace(s)/RB-set(s)

Proposal 9: Study the resource mapping of PSSCH in terms of interlace and RB-set

#### PSSCH resource allocation

NR-U uses two part FDRA (X,Y) for resource allocation in which X indicates the interlace(s) allocated and Y indicate the RB-set(s) allocation. SL-U may reuse the two part FDRA, i.e., (X,Y), for resource allocation since similar interlaced waveform for PSSCH and LBT channel access unit are reused from NR-U.

Proposal 10: Study how to reuse NR-U two part FDRA, i.e., (X,Y), for resource allocation in which X indicates the interlace(s) allocated and Y indicate the RB-set(s) allocation

However, in Rel.16 SL, the FDRA is in the form of relative allocation starting from the current SCI-1 reception subchannel and FDRA indicates how many subchannels are allocated while (X,Y) resource allocation in NR-U indicates absolute interlace/RB-set index. The relative allocation could be useful when two SL nodes have different assumptions on the absolute interlace or RB-set index, e.g., two nodes (UE0 and UE1 in Figure 11) are configured resource pools with partially overlapping RB-sets. If one defines Y allocation relative to the SCI reception RB-set, two nodes could still communicate via overlapping RB-sets.

Figure 11: UEs with different RB-set configs

Observation 9: Defining (X,Y) relative to the SCI-1 reception interlace and RB-set is helpful to enable communication between SL nodes with different interlace/RB-set assumption

### PSFCH

#### Interlaced waveforms and PSFCH formats

To address PSD limit and minimum OCB requirement, we may need interlaced waveform for PSFCH just as PUCCH in NR-U. Just as NR-U introduces interlaced waveform to PUCCH format 0 (PF0), one can introduce interlaced waveform to legacy PSFCH which is based on PUCCH format 0 as in Figure 12. One Ack/Nak bit is carried via one interlace and one cyclic shift pair. As in NR-U, cyclic shift ramping could be applied across interlaced RBs to reduce the PAPR.

Proposal 11: Study interlace waveform for PUCCH format 0 based PSFCH, which carries one-bit Ack/Nak

The interlaced PF0 based PSFCH is inefficient for eMBB use cases in which it can only carry one bit Ack/Nak with one interlace(9/10/11 interlaced RBs) and one CS pair. Suppose the PSSCH receiver has multiple Ack/Nak’s to reports to the same transmitter. It would be more efficient to arrange Ack/Nak’s in a HARQ codebook, e.g., type-1/2/3, and transmit with some multiple-bit PSFCH format with one PSFCH transmission instead of multiple PF0 PSFCH transmissions. Note that the number of simultaneous PF0 PSFCH could depend on UE capability.

In NR-U, PUCCH format 2 (PF2) with interlaced waveform is introduced to carry multiple Ack/Nak bits. We may try to reuse the interlaced PUCCH format 2 (PF2) design for SL-U. The new PF2-based PSFCH could occupy one symbol and one interlace with one AGC symbol ahead just as the PF0 based PSFCH as in Figure 12. To improve user multiplexing, frequency domain OCC-2/4 on PF2 PSFCH data REs and DMRS REs could be applied just as in NR-U PF2.

Proposal 12: Study multi-bit PSFCH format and its interlace waveform

Figure 12: PSFCH interlaced waveform and PF2 based PSFCH

### S-SSB

Rel.16 SL introduced a new S-SSB structure with uniform 11 RB and full slot transmission (14 symbols including a gap symbol) as shown in Figure 13. The S-SSB does not satisfy the OCB requirement by itself and cannot be transmitted along with PSCCH/PSSCH in the PSSCH resource pool. There could be a couple of options to consider

- S-SSB multiplexed with other SL transmission from the same transmitter

- Interlaced S-SSB waveform

- S-SSB frequency domain repetition or long PSS/SSS sequence

Interlaced S-SSB waveform (option 2) has bad impact on timing acquisition, and option 3 may drastically increase the   receiver searcher complexity with the increased S-SSB bandwidth. As mentioned in the later section, we may want to introduce multiple S-SSB candidate slots to counter the LBT uncertainty. Option 3 could cost too much overhead as the PSCCH/PSSCH from other transmitter could not be FDM’ed in the S-SSB candidate slots or occupy the RBs with potential S-SSB frequency domain repetitions.  NR-U takes option 1 as the solution to meet to OCB requirement. The SSB transmission relies on PDCCH/PDSCH multiplexing to fulfill the minimum OCB constraint.

Proposal 13: Study S-SSB multiplexed with other SL transmissions as an option to fulfill OCB requirement

But taking the Rel’16 S-SSB full slot pattern has some problems. First of all, S-SSB is not part of the resource pool. If we introduce multiple S-SSB candidate locations, the overhead of S-SSB is increased. Even if PSCCH may be able to rate match around S-SSB, this involves blind detection of rate matching given that a receiver does not know the PSCCH is multiplexed with S-SSB or not.

In order to transmit S-SSB in SL-U, we will need a new solution with the following requirements

- The PSS/SSS bandwidth stays the same (127 REs or 11RBs) to avoid increasing searching complexity

- The transmission goes together with PSCCH/PSSCH transmissions to approximately fill at least 80% of 20MHz (satisfy the OCB requirement)

- Avoid/reduce transmit power variation over time to reduce the risk of losing phase continuity

To solve this problem, we may want to study different S-SSB pattern for SL-U instead of using Rel’16 13-symbol S-SSB. One solution is to reuse NR SSB structure (4 symbols and 20 RBs) in Figure 13. However, single S-SSB per slot with S-SSB avoiding PSCCH, DMRS of PSSCH and SCI2 may be preferred. In an example shown in Figure 14, SSB avoids first 4 symbols (AGC symbol, up to 3 symbols for PSCCH) and symbols 1, 6, and 11, which are the three DMRS locations. One may not need to support all DMRS patterns for PSSCH in the S-SSB slots and may only choose the DMRS patterns which have at least 4-symbol gap in between/after the DMRS symbols. In this way, the S-SSB transmission does not impact the SCI and DMRS of PSSCH and could be multiplexed with PSCCH/PSSCH transmission to fulfill the OCB requirement. The PSSCH data REs could perform rate matching around the S-SSB. The multiplexed PSSCH could either take the full subband or a couple of interlaces which allows other S-SSB transmitters to FDM PSSCH in a different interlace.

Observation 10: To fulfil the OCB requirement in S-SSB transmission slot, reusing NR Uu SSB pattern for SL-U allows S-SSB to go together with PSCCH/PSSCH transmission without affecting the rate-matching of PSCCH

Figure 13: a) Rel'16 S-SSB b) NR SSB

Figure 14: S-SSB for SL-U

## Physical channel procedures

### Multiple PSFCH opportunities for PSFCH

Legacy PSFCH HARQ timeline has one opportunity for PSFCH transmission. If LBT failed, the transmitter would treat it as a Nack and introduce PSSCH retransmission overhead. We may want to study how to configure multiple PSFCH opportunities associated with a PSSCH and an example is shown in in Figure 15. The PSSCH receiver may continue to try LBT on different PSFCH opportunities until it clears the LBT. If the interference at different slots or LBT subbands are uncorrelated, the PSFCH will have more chances to clear the LBT.

Proposal 14: To address LBT uncertainty of PSFCH, study how to configure multiple PSFCH opportunities associated with a PSSCH

Figure 15: Multiple PSFCH opportunities illustration (interlace structure not shown)

### HARQ codebook

For eMBB like traffic, transmitting multiple Ack/Nak’s from one PSSCH receiver to one PSSCH transmitter with interlaced PF0 PSFCH could be inefficient or not even possible as the number of simultaneous PF0-based PSFCH transmissions is based on UE capability. HARQ codebook is needed so the Ack/Nak’s for multiple TBs can be carried via PF2 PSFCH in one PSFCH transmission.

As in Rel’16 SL, all the SL nodes in the network may share common PSFCH occasions with configurable periodicity of 0/1/2/4 slots. The HARQ timeline may follow the legacy timeline with MinTimeGapPSFCH constraint. Instead of sending multiple Ack/Nak’s via different PF0 PSFCH resources, the Ack/Nak’s are collected to form a HARQ codebook and transmitted via multi-bit carrying PSFCH with one PSFCH transmission. Type-3 like HARQ could also be introduced to enable triggered Ack/Nak retransmission.

Observation 11: Introducing HARQ codebooks and conveying the Ack/Nak’s via multi-bit carrying PSFCH could improve Ack/Nak feedback efficiency and help with Aak/Nak retransmission

Figure 16: Type-1 HARQ codebook

### Multiple S-SSB candidate locations

In Rel’16 SL, the number of S-SSB instance (K) within one S-SSB period is as follows

  - FR1, SCS = 15 kHz: K=1

  - FR1, SCS = 30 kHz: K=1, 2

  - FR1, SCS = 60 kHz: K=1, 2, 4

Due to LBT uncertainty, the SL S-SSB transmitting node may need more opportunities to transmit S-SSB. We may also want to limit the multiple opportunities be within a configured window to limit the S-SSB searcher’s complexity instead of letting the searcher to be on throughout the entire S-SSB period.  One can reuse NR-U DRS window design with multiple candidate locations clustered in the DRS window. The receiver may only search for the S-SSB within the configured DRS window.

Among the M candidate locations, the S-SSB transmitter may transmit up to L S-SSB slots, e.g., E.g. L=4/8 in M=10/20 candidate locations. Multiple L S-SSB transmissions could be S-SSB repetition to improve the coverage as in Uu.

Figure 17: S-SSB Transmission within S-SSB candidate locations

Proposal 15: To address the LBT uncertainty for S-SSB transmission, we may want to study how to reuse NR-U DRS window design with multiple candidate locations

# Summary

In this section, we summarize the observations and proposals on different design aspects on SL-U.

Observation 1:  If there exists an unused PSFCH instance in the middle of data burst, additional LBT may be required by SL transmitter to continue the remaining transmission

Observation 2: If the COT-initiating transmitter could transmit or its receiver could be scheduled to transmit some signals at the unused PSFCH instances, we can reduce the LBT overheadError! Reference source not found.

Observation 3: Multi-slot transmission improves efficiency of LBT-restricted channel access and provides opportunities for overhead reduction

Observation 4: Slot-based access in NR Sidelink reduces channel access opportunities compared to other RATs

Observation 5: Mini-slots or sub-slots can increase channel access opportunities

Observation 6: The RB-set config for 20MHz subbands could be either common config for all SL nodes in the NW or configurable per link via RRC message exchange

Observation 7: If RB-set config per link is configurable, one may need to define the common minimum RB set for a 20MHz subband for the initial RRC message exchange to set up the RB-set config

Observation 8: It is natural for PSSCH to TDM or/and FDM with the scheduling PSCCH and the PSCCH interlace/RB-set being the leading interlace/RB-set of the scheduled PSSCH interlace(s)/RB-set(s)

Observation 9: Defining (X,Y) relative to the SCI-1 reception interlace and RB-set is helpful to enable communication between SL nodes with different interlace/RB-set assumption

Observation 10: To fulfil the OCB requirement in S-SSB transmission slot, reusing NR Uu SSB pattern for SL-U allows S-SSB to go together with PSCCH/PSSCH transmission without affecting the rate-matching of PSCCH

Observation 11: Introducing HARQ codebooks and conveying the Ack/Nak’s via multi-bit carrying PSFCH could improve Ack/Nak feedback efficiency and help with Aak/Nak retransmission

Proposal 1: Study how to use CP extension (CPE) of the AGC symbol to fill into the gap symbol of the previous slot so that the one symbol transmission gap in between the slots becomes narrower

Proposal 2: In addition to CPE method, study how to rate match PSSCH into the gap symbol or/and AGC symbol

Proposal 3: For the gap before PSFCH, study how to use CP extension to maintain the right length gap to match the channel access type or keep the COT

Proposal 4: Study multiple contiguous slot transmissions with minimum signaling overhead

Proposal 5: Study “mini-slot or sub-slot” channel access as a method to increase channel access opportunities

Proposal 6: Study interlaced waveforms for PSCCH/PSSCH

Proposal 7: Study the basic unit for resource allocation in terms of interlace and RB-set

Proposal 8: Study PSCCH resource mapping in terms of interlace and RB-set

Proposal 9: Study the resource mapping of PSSCH in terms of interlace and RB-set

Proposal 10: Study how to reuse NR-U two part FDRA, i.e., (X,Y), for resource allocation in which X indicates the interlace(s) allocated and Y indicate the RB-set(s) allocation

Proposal 11: Study interlace waveform for PUCCH format 0 based PSFCH, which carries one-bit Ack/Nak

Proposal 12: Study multi-bit PSFCH format and its interlace waveform

Proposal 13: Study S-SSB multiplexed with other SL transmissions as an option to fulfill OCB requirement

Proposal 14: To address LBT uncertainty of PSFCH, study how to configure multiple PSFCH opportunities associated with a PSSCH

Proposal 15: To address the LBT uncertainty for S-SSB transmission, we may want to study how to reuse NR-U DRS window design with multiple candidate locations

# References

- RP-213678, “New WID on NR sidelink evolution”, RAN #94, OPPO, LG Electronics

- ETSI EN 301 893 V2.1.1

- RP- 2205033, “Channel Access Mechanism for SL-U”, RAN #109e, Qualcomm