3GPP TSG RAN WG1 #109-e			R1-2205036

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.5.1.1

Source: 	Qualcomm Incorporated

Title: 	Sidelink Positioning Scenarios and Requirements

Document for:		Discussion and Decision

# Introduction

The study on expanded and improved NR positioning introduces sidelink positioning as an objective [1]:

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2] - Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1] - Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required] Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

In this contribution, we discuss the scnarios and associated requirements for evaluating V2X, public safety, commercial, and IIoT applications.

# General Concepts

In Release 16 and 17, and as proposed in out companion contribution [6] for Release-18, the CDF of vertical position, horizontal position, and range error is used as a performance metric for positioning and ranging evaluations. Hence, we also also porpose to use the CDF, more specifically a percentile, to define the target requirements for the different scenarios and use-cases.

Similarly, for relative positioning, we propose to use a percentile in the CDF of horizontal and vertical displacement error between two UEs to define the target requirements.

Proposal 1: The target requirement is defined to be a particular accuracy at a a particular percentile in the CDF of the error in estimated horizontal position, veritical position, range, horizontal displacement, and vertical displacement.

# V2X Use Cases and Requirements

In V2X deployments, the out-of-coverage scenario is the primary one since the V2X system must work regardless of whether a network is present. Hence, we propose to study V2X sidelink positioning in out-of-coverage scenarios.

Proposal 2: Study out-of-coverage scenarios for V2X sidelink positioning and ranging.

3GPP identified three sets of positioning requirements for V2X [2] based on inputs from different 3GPP working groups and other industry forums:

- Set 1: 10 – 50 m with 68 – 95 % confidence level. This includes Group 1 in [5] and Service level 1 in [3]. - Set 2: 1 – 3 m with 95 – 99 % confidence level. This includes Group 2 in [5], Service level 2, 3, 4 in [3]. - Set 3: 0.1 – 0.5 m with 95 – 99 % confidence level. This includes Group 3 in [5], Service level 5, 6, 7 in [3], the requirements in [4].

Of note was the input from SAE AA TC emphasizing the necessity of high accuracy positioning at the sub-meter level [4]. Hence, we propose to use Set 3 requirements as baseline for V2X sidelink positioning and ranging evaluations. Other requirement sets can be optionally used.

Proposal 3: 0.5m accuracy at the 90th percentile is used as baseline target requirement for V2X sidelink positioning and ranging evaluations.

Currently, only one LTE band and two NR bands are defined for V2X applications: b47, n47, and in some regions, n38. NR V2X bands are defined with a maximum bandwidth configuration of 40 MHz and when Uu band n71 is defined to be used concurrently with n47 for V2X, the maximum bandwidth configuration for n71 is 20 MHz [3].

Table 5.3.5-1 Channel bandwidths for each NR band [3]

| NR band / SCS / UE Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n38 | 15 | Yes | Yes10 | Yes | Yes10 | Yes | Yes10 | Yes10 |  |  |  |  |  |  |
|  | 30 |  | Yes10 | Yes | Yes10 | Yes | Yes10 | Yes10 |  |  |  |  |  |  |
|  | 60 |  | Yes10 | Yes | Yes10 | Yes | Yes10 | Yes10 |  |  |  |  |  |  |
| n47 | 15 |  | Yes10 |  | Yes10 |  | Yes10 | Yes10 |  |  |  |  |  |  |
|  | 30 |  | Yes10 |  | Yes10 |  | Yes10 | Yes10 |  |  |  |  |  |  |
|  | 60 |  | Yes10 |  | Yes10 |  | Yes10 | Yes10 |  |  |  |  |  |  |
| NOTE 10: These UE channel bandwidths are applicable to sidelink operation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.3E.2-1: Inter-band con-current V2X configurations [3]

| V2X con-current operating band Configuration | NR Bands | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz | 40 MHz | 50 MHz | Maximum bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X_n71A-n47A | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  | 60 | 0 |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |
|  | n47 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  | 30 |  | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  | 60 |  | Yes |  | Yes | Yes | Yes |  |  |  |

Observation 1: The maximum bandwidth available in dedicated (ITS) spectrum for V2X is 40 MHz.

Observation 2: The maximum bandwidth available for Uu operation in licensed spectrum to be concurrently used with a sidelink band for V2X is 20 MHz.

Observation 3: There is no licensed spectrum available for V2X sidelink operation.

Proposal 4: V2X simulations should use a maximum of 40 MHz bandwidth in dedicated spectrum.

# Public Safety Use Cases and Requirements

TR 38.845 sumamrizes the positioning requirements for public safety, which had been defined in TS 22.261, TS 22.280, and TS 22.261:

1 m horizontal accuracy, 2 m (absolute) or 0.3 m (relative) vertical accuracy, 95 – 98 % positioning service availability. TS 22.280 [7] specifies some qualitative positioning requirements in its Clause 5.11, 6.12, and 7.8. These requirements are applicable to both relative and absolute positioning.

Since, as will be discussed later, both commercial and IIoT use cases use 0.2m relative vertical accuracy, the same value could be used for public safety use case to simplify evaluations.

Proposal 5: The following requirements are used for relative and absolute positioning for public safety use cases: 1 m horizontal accuracy, 2 m (absolute) or 0.2 m (relative) vertical accuracy, at the 90th percentile.

We propose to use the same requirements in the RAN1 study item. In terms of coverage, both in-coverage and out-of-coverage scenarios should be studied for public safety. As part of in-coverage scenarios, joint Uu and SL positioning should be studied as a method for enhancing accuracy.

Proposal 6: Both in-converage and out-of-coverage scenarios are studied for public safety positioning.

Proposal 7: Joint Uu and SL positioning is studied for public safety in-coverage scenarios.

# Commerical Use Cases and Requirements

The following positioning service levels and their associated requirements are defined:

| Positioning service level | Absolute(A) or Relative(R) positioning | Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Coverage, environment of use and UE velocity |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Horizontal Accuracy | Vertical Accuracy (note 1) |  |  | 5G positioning service area | 5G enhanced positioning service area (note 2) |  |
|  |  |  |  |  |  |  | Outdoor and tunnels | Indoor |
| 1 | A | 10 m | 3 m | 95 % | 1 s | Indoor - up to 30 km/h Outdoor (rural and urban) up to 250 km/h | NA | Indoor - up to 30 km/h |
| 2 | A | 3 m | 3 m | 99 % | 1 s | Outdoor (rural and urban) up to 500 km/h for trains and up to 250 km/h for other vehicles | Outdoor (dense urban) up to 60 km/h Along roads up to 250 km/h and along railways up to 500 km/h | Indoor - up to 30 km/h |
| 3 | A | 1 m | 2 m | 99 % | 1 s | Outdoor (rural and urban) up to 500 km/h for trains and up to 250 km/h for other vehicles | Outdoor (dense urban) up to 60 km/h Along roads up to 250 km/h and along railways up to 500 km/h | Indoor - up to 30 km/h |
| 4 | A | 1 m | 2 m | 99,9 % | 15 ms | NA | NA | Indoor - up to 30 km/h |
| 5 | A | 0,3 m | 2 m | 99 % | 1 s | Outdoor (rural) up to 250 km/h | Outdoor (dense urban) up to 60 km/h Along roads and along railways up to 250 km/h | Indoor - up to 30 km/h |
| 6 | A | 0,3 m | 2 m | 99,9 % | 10 ms | NA | Outdoor (dense urban) up to 60 km/h | Indoor - up to 30 km/h |
| 7 | R | 0,2 m | 0,2 m | 99 % | 1 s | Indoor and outdoor (rural, urban, dense urban) up to 30 km/h Relative positioning is between two UEs within 10 m of each other or between one UE and 5G positioning nodes within 10 m of each other (note 3) |  |  |
| NOTE 1: The objective for the vertical positioning requirement is to determine the floor for indoor use cases and to distinguish between superposed tracks for road and rail use cases (e.g. bridges). NOTE 2: Indoor includes location inside buildings such as offices, hospital, industrial buildings. NOTE 3: 5G positioning nodes are infrastructure equipment deployed in the service area to enhance positioning capabilities (e.g. beacons deployed on the perimeter of a rendezvous area or on the side of a warehouse). |  |  |  |  |  |  |  |  |

Release-17 Uu positioning used the following requirements for positioning in commercial applications: 1m abolute positioning accuracy at the 90th percentile. Those requirements can be reused in Release-18.

The positioning SID in Release-18 includes relative positioning in addition to absolute positioning and we propose to use the 0.2m accurage requirement from the table above for these new scenarios.

Proposal 8: The following requirements are used for relative and absolute positioning for commercial use cases: 1 m horizontal accuracy, 2 m (absolute) or 0.2 m (relative) vertical accuracy, at the 90th percentile.

As we discuss in our companion contribution [7], both in-coverage and out-coverage scenarios are relevant for commercial applications, and we propose to study both. There we also also discuss the benefits of joint Uu/SL positioning for such applications.

Proposal 9: Both in-coverage and out-of-coverage scenarios are studied for commercial application positioning.

Proposal 10: Joint Uu and SL positioning is studied for commercial in-coverage scenarios.

# IIoT Use Cases and Requirements

The following requirements are defined for the different industrial IoT positioning use cases [5]:

| Scenario | Horizontal accuracy | Vertical accuracy | Availability | Heading | Latency for position estimation of UE | UE speed | Corresponding Positioning Service Level in TS 22.261 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mobile control panels with safety functions (non-danger zones) | < 5 m | < 3 m | 90 % | n/a | < 5 s | n/a | Service Level 2 |
| Process automation – plant asset management | < 1 m | < 3 m | 90 % | n/a | < 2 s | < 30 km/h | Service Level 3 |
| Flexible, modular assembly area in smart factories (for tracking of tools at the work-place location) | < 1 m (relative positioning) | n/a | 99 % | n/a | 1 s | < 30 km/h | Service Level 3 |
| Augmented reality in smart factories | < 1 m | < 3 m | 99 % | < 0.17 rad | < 15 ms | < 10 km/h | Service Level 4 |
| Mobile control panels with safety functions in smart factories (within factory danger zones) | < 1 m | < 3 m | 99.9 % | < 0.54 rad | < 1 s | n/a | Service Level 4 |
| Flexible, modular assembly area in smart factories (for autonomous vehicles, only for monitoring purposes) | < 50 cm | < 3 m | 99 % | n/a | 1 s | < 30 km/h | Service Level 5 |
| Inbound logistics for manufacturing (for driving trajectories (if supported by further sensors like camera, GNSS, IMU) of indoor autonomous driving systems)) | < 30 cm (if supported by further sensors like camera, GNSS, IMU) | < 3 m | 99.9 % | n/a | 10 ms | < 30 km/h | Service Level 6 |
| Inbound logistics for manufacturing (for storage of goods) | < 20 cm | < 20 cm | 99 % | n/a | < 1 s | < 30 km/h | Service Level 7 |

From the above table, the strictest accuracy requirement is 20 cm, which is defined for the inbound logistics scenario. Since this target was used in Release-17, we propose to use it in Release-18 as well.

Proposal 11: The following requirements are used for relative and absolute positioning in IIoT use cases: 0.2m absolute and relative, horizontal and vertical accuracy at the 90th percentile.

For industrial IoT, the most relevant cases are where SL positioning is deployed to assist in Uu-positioning to enhance accuracy and/or reliability. Hence, we propose to study in-coverage scenarios and joint Uu-SL positioning for IIoT scenarios.

Proposal 12: In-coverage scenarios are studied for IIoT positioning.

Proposal 13 Joint Uu and SL positioning is studied for IIoT in-coverage scenarios.

# Conclusions

Observation 1: The maximum bandwidth available in dedicated (ITS) spectrum for V2X is 40 MHz.

Observation 2: The maximum bandwidth available for Uu operation in licensed spectrum to be concurrently used with a sidelink band for V2X is 20 MHz.

Observation 3: There is no licensed spectrum available for V2X sidelink operation.

Proposal 1: The target requirement is defined to be a particular accuracy at a a particular percentile in the CDF of the error in estimated horizontal position, veritical position, range, horizontal displacement, and vertical displacement.

Proposal 2: Study out-of-coverage scenarios for V2X sidelink positioning and ranging.

Proposal 3: 0.5m accuracy at the 90th percentile is used as baseline target requirement for V2X sidelink positioning and ranging evaluations.

Proposal 4: V2X simulations should use a maximum of 40 MHz bandwidth in dedicated spectrum.

Proposal 5: The following requirements are used for relative and absolute positioning for public safety use cases: 1 m horizontal accuracy, 2 m (absolute) or 0.2 m (relative) vertical accuracy, at the 90th percentile.

Proposal 6: Both in-converage and out-of-coverage scenarios are studied for public safety positioning.

Proposal 7: Joint Uu and SL positioning is studied for public safety in-coverage scenarios.

Proposal 8: The following requirements are used for relative and absolute positioning for commercial use cases: 1 m horizontal accuracy, 2 m (absolute) or 0.2 m (relative) vertical accuracy, at the 90th percentile.

Proposal 9: Both in-coverage and out-of-coverage scenarios are studied for commercial application positioning.

Proposal 10: Joint Uu and SL positioning is studied for commercial in-coverage scenarios.

Proposal 11: The following requirements are used for relative and absolute positioning in IIoT use cases: 0.2m absolute and relative, horizontal and vertical accuracy at the 90th percentile.

Proposal 12: In-coverage scenarios are studied for IIoT positioning.

Proposal 13 Joint Uu and SL positioning is studied for IIoT in-coverage scenarios.

# References

- RP-213588 “Revised SID on Study on expanded and improved NR positioning,”  Intel (Email discussion moderator), RAN #94-e.

- 3GPP TR 38.845 “Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases.”

- 3GPP TS 38.101-1 “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone.”

- RP-210036 "Reply LS to 3GPP TSG RAN on requirements of in-coverage, partial coverage and out-of-coverage positioning use cases," (source: SAE Advanced Applications Technical Committee).

- 3GPP TS 22.104 “Service requirements for cyber-physical control applications in vertical domains.”

- R1-2205037 “Sidelink Positioning Evaluation Assumptions and Results,” Qualcomm Inc. RAN1 #109.

- R1-2205038 “Potential Solutions for Sidelink Positioning,” Qualcomm Inc. RAN1 #109.