3GPP TSG RAN WG1 #109-e		R1- 2205037

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.5.1.2

Source: 	Qualcomm Incorporated

Title: 	Sidelink Positioning Evaluation Assumptions and Results

Document for:		Discussion and Decision

# Introduction

The study on expanded and improved NR positioning introduces sidelink positioning as an objective [1]:

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2] - Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1] - Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required] Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

In this contribution, we discuss evaluation methodology and assumptions for the different supportred use-cases. In addition, we provide simulation results illustrating the impact of available bandwidth on positioning accuracy.

- Evaluation Methodology and Assumptions

In section, we discuss general evaluation methodology aspects affecting all sidelink positioning use-cases, then provide additional details for each use-case in its corresponding sub-section.

The first topic is to decide is whether PRS can be FDMed with other sidelink transmissions. It is preferable to transmit SL PRS on as wide a bandwidth as possible and allowing other sidelink communications to be FDMed with SL PRS would limit the bandwidth available for SL PRS; and increase interference and collision probability with SL PRS. This would result in degraded positioning performance. For similar reasons, PRS is not FDMed with other transmissions in Uu positioning. Hence, we propose to follow the Uu positioning approach and not FDM other sidelink communications with SL PRS.

Proposal 1: Sidelink PRS and other sidelink communications cannot be FDMed with each other, i.e. they can only be TDMed.

The next issue is whether there is uncertainty in an absolute positioning anchor’s location coordinates. In general, anchors such as gNBs and RSUs can know their location with very high accuracy, hence, it can be assumed that they have no uncertainty in their location coordinates as baseline as was done in the Uu positioning study. Whether other anchors have uncertainty can be discussed as an optional case.

Proposal 2: As baseline for absolute positioning, there is no uncertainty in the sidelink anchors location coordinates.

Proposal 3: Optionally for absolute positioning, consider that the SL anchors have a location coordinate uncertainty.

Similar to the agreement in the Rel-16 positioning study item, all links are to be simulated and network synchronization follows a truncated Gaussian distribution when applied.

Proposal 4: Explicit simulation of all links, individual parameters estimation is applied. Companies should provide description of applied algorithms for estimation of signal location parameters. FFS whether a common algorithm is to be agreed upon.

Proposal 5: Network synchronization error. Network sync error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an gNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, the range of timing errors is [-T2, T2], T2 = 2*T1. FFS whether the same is applied to UE-to-UE synchronization error, e.g. for TDoA evaluations.

In terms of which metrics to use, the CDF of horizontal and, where applicable, vertical positioning errors are used. In addition, since ranging is also included part of the SID, the CDF of range error is used as a metric for ranging analysis.

Proposal 6: The CDF of horizontal position, vertical position, and range error is used as a performance metric depending on the evaluated scenario.

Finally, the SID states to study solutions for relative positioning, ranging and absolute positioning. For brevity, we use the term positioning, however all evaluation methodology and assumptions should apply to all three.

Proposal 7: Evaluation methodology and assumptions apply to relative positioning, ranging and absolute positioning.

## V2X Scenarios

V2X evaluations in Releases 16 and 17 used two defined deployment scenarios: highway and urban grid [3]. The same UE drop models could be used in this study as well.

Proposal 8: Use the highway and urban grid UE drop models defined in TS 37.885 for V2X scenarios.

Similarly, the antenna configurations, channel models, fading parameters from TS 37.885 can be used.

Proposal 9: Use the antenna configurations, channel models, fading parameters from TS 37.885 for V2X scenarios.

The maximum bandwidth defined by RAN4 for a V2X band is 40 MHz. Hence, we propose to use the same value for ITS bands [4]. It should be noted that no licensed bands are defined for V2X applications.

Proposal 10: The maximum bandwidth to use for an ITS band is 40 MHz.

Observation 1: No licensed bands are available for V2X applications.

## Public Safety Scenarios

In this section, we discuss evaluation assumptions related to public safety scenarios based on [6] and the agreements made in the Release 17 Sidelink Enhancements work item [5].

Two deployment layouts can be considered:

- Urban macro layout with 1732m ISD (Option 5 from [6])

- Urban macro layout with 500m ISD (Option 3 from [6])

Proposal 11: Use urban macro layout with 1732m ISD (Option 5 from TS 36.843) and/or urban macro layout with 500m ISD (Option 3 from TS 36.843) for public safety evaluations.

Additional details are also included in [6] for a variety of scenarios. We propose to use the following set of parameters:

| PARAMETER | VALUE |
| --- | --- |
| Carrier Frequency | 700MHz |
| System Bandwidth | 10MHz |
| UE Mobility | 3kmph (baseline) and 60kmph (optional) |
| Maximum TX Power | 23dBm or 31 dBm |
| Number of Antennas | 1T2R |
| Antenna Gain | 0dBi |
| Noise figure | 9dB |
| Uniform Drop (for Option 3 and 5 of [6]) | All UEs are randomly and uniformly dropped throughout the layout. All UEs are outdoors, and no buildings are dropped. |
| Minimum distance between UEs | >= 3m |
| Number of UEs per sector | 10 |

Proposal 12: For public safety scenarios, evaluate out-of-coverage cases as well as in-coverage.

To perform joint Uu-SL positioning, gNB-UE channel models must also be defined. For this part, it would be more efficient to use the same set of models for both public safety and commercial scenarios. We provide more details in Section 2.3.

Proposal 13: For joint SL-Uu positioning in public safety scenarios, use the same UE-gNB channel models and the same UE-gNB band and bandwidth as the commercial scenarios.

## Commercial Scenarios

For general commercial use cases, Rel-16 scenarios and channel models in TR 38.855 for FR1 and FR2 can be reused with regards Uu channels and deployment options. For the absolute time of arrival modelling in IOO, UMa, UMi, sources may provide the details of their model, if any.

Proposal 14: For general commercial use cases, for gNB-UE channels, Rel-16 scenarios and channel models in TR 38.855 are reused for Uu channels. These are included for reference in the Appendix.

- For the absolute time of arrival modelling in IOO, UMa, UMi, sources may provide the details of their model.

Proposal 15: Use TS 36.843 A.2.1.2 channel models for UE-UE Outdoor to Outdoor, Indoor to Indoor channels.

- For UMI/UMA scenarios, for both FR1, FR2, support the following UE drop assumption:

  - Baseline Scenario: 10 UEs per sector, uniform drop of UEs wherein all UEs randomly and uniformly dropped throughout the macro geographical area. All UEs are dropped outdoors. No buildings are dropped

- Optional: For InH scenarios, for both FR1, FR2

  - Uniformly at random drop of [X] indoor UEs

As discussed in our companion contributions [7][8], there are scenarios where commercial devices would perform positioning using sidelink only and other scenarios where joint Uu/SL positioning is used. We propose to evaluate both sets of scenarios.

Proposal 16: Evaluate both sidelink-only and joint Uu/SL positioning for commercial scenarios.

Following channel models shall be used for D2D. Outdoor to Outdoor; Indoor to Indoor. Pathlossa; PL_B1_tot(d) = max(PLfreespace(d), PL_B1(d)) where d is distance between UEs PLfreespace is free space path loss (Eq. 4.24 in [6]), PL_B1 is the Winner + B1b ([7] Table 4-1) channel model for hexagonal layout with the following offsets - LOS offset = 0 dBc - NLOS offset = -5 dBc While calculating Winner + B1 pathloss the following values shall be used h_BS = h_MS = 1.5m, h_BS' = h_MS' = 0.8m; Dual strip ([3] Table A.2.1.1.2-8) for Layout Option 2 InH ([3] Table A.2.1.1.5-1) for remaining layout optionsd. LOS Probabilitye; Winner II-B1 ([6] Table 4-7); ITU-R IMT UMi ([8] Table A1-3) for InH N/A for Dual Strip. Shadowing standard deviation; 7 dB log-normal; UEs are in same building: LOS: 3 dB log-normal NLOS: 4 dB log-normal UEs are in different building: 10 dB log-normal. Shadowing correlation; i.i.d.. Fast Fadingf; ITU-R IMT UMi ([8] Annex 1.3.2) LOS and NLOS; ITU-R IMT InH ([8] Annex 1.3.2) LOS and NLOS. - a) Pathloss should be defined for 700 MHz in addition to 2 GHz (by applying 20log(fc) correction for 700 MHz if not otherwise specified) - b) Winner+B1 is assumed to be valid up to a minimum distance of 3m - c) The offsets are assumed to be valid for all the frequencies of interest. - d) For calculating the indoor to indoor path loss between a virtual indoor UE and another virtual indoor UE or indoor UE use the InH model for UEs inside different buildings - e) LOS probability: some pathloss models do not specify a LOS/NLOS region – the LOS Probability would not be used for such models - f) Additional modifications needed to incorporate changes in Doppler modeling due to dual mobility are described in Section A.2.1.2.1. Further study can be undertaken on the dual-scattering described in [9]. NOTE: These models are being adopted for the purpose of relative comparisons of D2D techniques, without necessarily having measurement support.

## IIoT Scenarios

For IIoT scenarios, the Rel-17 scenarios and channel models in TR 38.857 Section 6 are reused for the purpose of Uu gNB-UE channel models. These are included for reference in the Appendix.

Proposal 17: For IIoT scenarios, the Rel-17 scenarios and channel models in TR 38.857 Section 6 are reused for the purpose of Uu gNB-UE channel models. These are included for reference in the Appendix.

Proposal 18: Use TS 36.843 A.2.1.2 channel models for UE-UE Indoor to Indoor channels, with uniformly at random drop of [X] indoor UEs. FFS X.

Similar to commercial scenarios, both sidelink-on and joint Uu/SL positiong scenarios arise in IIoT and we propose to evaluate both.

Proposal 19: Evaluate both sidelink-only and joint Uu/SL positioning for commercial scenarios.

# Evaluation Results

## 3.1 Bandwidth Requirements

In this section, we provide evaluation results for ranging operations in a V2X highway scenario. These results are shown in Figure 1, which shows the CDF of the error observed when performing ranging operations using sidelink positioning. From the figure, it can be observed that the bandwidth available in ITS bands, i.e. 20 MHz and 40 MHz, cannot meet Set 2 requirements for V2X applications.

Figure 1 CDF of range estimation error in a highway V2X scenario.

Observation 2: Set 2 requirements for V2X application cannot be met by the bandwidth available for ITS bands, i.e. 40 MHz.

The trend shown in results is consistent with the theoretical analysis based on the Cramer-Rao bound in the following. Assuming a peak-finding algorithm for estimating the time of arrival (ToA), the variance of such and estimator is lower-bounded by:

σToA2≥18π2⋅M⋅SNRT⋅Brms2

Where

- M is the number of antennas, assumed to be 2 in this ranging evaluation.

- SNRT is the signal energy of the direct path over the noise spectral density.

- Assumed uniform PSD for the SL-PRS signal, then Brms2=B2/3 with B= 20, 40, 100 MHz in the evaluations.

For SL-RTT, the time uncertainty is twice as much as ToA.

Using the above bound, we obtain the lower bound on range uncertainty in an AWGN setting shown in Figure 2. We note that the simulations in Figure 1 used a fading channel model and using that a more realistic channel model instead of AWGN for the lower bound analysis would further increase the lower bound.

Figure 2 Theoretical lower bound SL-RTT range uncertainty in AWGN with for SL-PRS bandwidths 20, 40, and 100 MHz

## 3.1 Joint Uu-SL Positioning Evaluations

For the scenario of joint Uu-SL Positioning, we provide a first set of simulation results for the scenario of 4 GHz, UMI, outdoor UEs, with 10 UEs per sector randomly and uniformly dropped in the xy plane. All UEs are assumed to be in-coverage.

- A target UE transmits and receives SL-PRS with up to 1 or 2 of the nearest UEs, which we assume have genie location knowledge. SL-PRS bandwiwdth is 100 MHz, positioning method is SL-RTT

- The same UE is also configured with Uu-RTT, receives DL-PRS and transmits SRS for Positioning

- All the measurements, both SL Rx-Tx and UE Rx-Tx are sent to one entity for the purpose of positioning estimation using an outlier rejection approach (RANSAC).

In the results shown below we observe that, even adding a single SL anchor could improve the performance of Uu-Only positioning.

Observation 3: For in-coverage scenarios, SL anchors can assist Uu-Positioning by providing additional anchors and measurements for location determination.

# Conclusions

Observation 1: No licensed bands are available for V2X applications.

Observation 2: Set 2 requirements for V2X application cannot be met by the bandwidth available for ITS bands, i.e. 40 MHz.

Observation 3: For in-coverage scenarios, SL anchors can assist Uu-Positioning by providing additional anchors and measurements for location determination.

Proposal 1: Sidelink PRS and other sidelink communications cannot be FDMed with each other, i.e. they can only be TDMed.

Proposal 2: As baseline for absolute positioning, there is no uncertainty in the sidelink anchors location coordinates.

Proposal 3: Optionally for absolute positioning, consider that the SL anchors have a location coordinate uncertainty.

Proposal 4: Explicit simulation of all links, individual parameters estimation is applied. Companies should provide description of applied algorithms for estimation of signal location parameters. FFS whether a common algorithm is to be agreed upon.

Proposal 5: Network synchronization error. Network sync error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an gNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, the range of timing errors is [-T2, T2], T2 = 2*T1. FFS whether the same is applied to UE-to-UE synchronization error, e.g. for TDoA evaluations.

Proposal 6: The CDF of horizontal position, vertical position, and range error is used as a performance metric depending on the evaluated scenario.

Proposal 7: Evaluation methodology and assumptions apply to relative positioning, ranging and absolute positioning.

Proposal 8: Use the highway and urban grid UE drop models defined in TS 37.885 for V2X scenarios.

Proposal 9: Use the antenna configurations, channel models, fading parameters from TS 37.885 for V2X scenarios.

Proposal 10: The maximum bandwidth to use for an ITS band is 40 MHz.

Proposal 11: Use urban macro layout with 1732m ISD (Option 5 from TS 36.843) and/or urban macro layout with 500m ISD (Option 3 from TS 36.843) for public safety evaluations.

Proposal 12: For public safety scenarios, evaluate out-of-coverage cases as well as in-coverage.

Proposal 13: For joint SL-Uu positioning in public safety scenarios, use the same UE-gNB channel models and the same UE-gNB band and bandwidth as the commercial scenarios.

Proposal 14: For general commercial use cases, for gNB-UE channels, Rel-16 scenarios and channel models in TR 38.855 are reused for Uu channels. These are included for reference in the Appendix.

- For the absolute time of arrival modelling in IOO, UMa, UMi, sources may provide the details of their model.

Proposal 15: Use TS 36.843 A.2.1.2 channel models for UE-UE Outdoor To Outdoor, Indoor to Indoor channels.

- For UMI/UMA scenarios, for both FR1, FR2, support the following UE drop assumption:

  - Baseline Scenario: 10 UEs per sector, uniform drop of UEs wherein all UEs randomly and uniformly dropped throughout the macro geographical area. All UEs are dropped outdoors. No buildings are dropped

- Optional: For InH scenarios, for both FR1, FR2

  - Uniformly at random drop of [X] indoor UEs

Proposal 16: Evaluate both sidelink-only and joint Uu/SL positioning for commercial scenarios.

Proposal 17: For IIoT scenarios, the Rel-17 scenarios and channel models in TR 38.857 Section 6 are reused for the purpose of Uu gNB-UE channel models. These are included for reference in the Appendix.

Proposal 18: Use TS 36.843 A.2.1.2 channel models for UE-UE Indoor to Indoor channels, with uniformly at random drop of [X] indoor UEs. FFS X.

Proposal 19: Evaluate both sidelink-only and joint Uu/SL positioning for commercial scenarios.

# References

- RP-213588 “Revised SID on Study on expanded and improved NR positioning,” Intel (Email discussion moderator), RAN #94-e.

- 3GPP TS 38.885, “Study on NR Vehicle-to-Everything (V2X) (Release 16)”

- 3GPP TS 37.885, “Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR (Release 15)”

- 3GPP TS 38.101-1 “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone.”

- RAN1 Chair Notes, RAN1 #103 Eelectronic Meeting.

- 3GPP TS 36.843, “Study on LTE device to device proximity services (Release 12)”

- R1-2205036, “Sidelink Positioning Scenarios and Requirements,” Qualcomm, Inc. RAN1 #109.

- R1-2205038, “Potential Solutions for Sidelink Positioning,” Qualcomm, Inc. RAN1 #109.

# Appendix

## 6.1 Commercial Scenarios for Positioning evaluation

For evaluating baseline performance, scenarios (with various options/configurations) are defined below for RAT-dependent positioning techniques for NR positioning study

-	Scenario 1. Indoor Office for FR1 and FR2 (Open office)

-	Scenario 2. UMi street canyon for FR1 and FR2 (ISD 200m)

-	Scenario 3. UMa (ISD 500m) for FR1 only (Macro cell only deployment scenario)

The evaluation methodology does not define any baseline reference signals. The existing channel models as provided in the TR 38.901 are used for NR positioning evaluation as a baseline.  The results from the evaluations can potentially be used to select a subset of the reference signals studied as part of the evaluations when considering design options. Companies can provide additional results and describe modifications made for NLOS channel model.

The scenario parameters common to all scenarios are detailed in table 6.1.1-1. Parameters specific to scenario 1,2 and 3 are detailed in table 6.1.1-3, 6.1.1-4 and 6.1.1-6, respectively.

Table 6.1.1-1: Common scenario parameters applicable for all scenarios

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 2GHz, 4GHz – Note 1 | 30 GHz – Note 1 |
| Bandwidth, MHz | 5MHz, 50MHz for 2GHz 100MHz for 4GHz | 100MHz, 400MHz |
| Subcarrier spacing, kHz | 15kHz for 5MHz and 50MHz 30kHz for 100MHz | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU - Optional: Provided by company |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |

Table 6.1.1-2: UE antenna radiation pattern model 1 (FR2)

| Parameter | Values |
| --- | --- |
| Antenna element radiation pattern in θ'' dim (dB) |  |
| Antenna element radiation pattern in φ'' dim (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5dBi |
|  | Note: are in local coordinate system. |

Table 6.1.1-3: Scenario 1. Indoor office scenario parameters

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Layout | Indoor floor: (12BSs per 120m x 50m), TRP number per floor:12, Inter-gNB distance = 20m - Note 1 |  |
| Total gNB TX power, dBm | 24dBm | 24dBm EIRP should not exceed 58 dBm |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |
| gNB antenna radiation pattern | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |
| Channel model | Indoor open office – Note 2 |  |
| Peneteration loss | 0dB |  |
| Number of floors | 1 |  |
| UE drop procedure | 100% indoor, uniformly distributed over the horizontal area |  |
| UE mobility | 3km/h |  |
| UE antenna height | 1.5m |  |
| Min gNB-UE distance (2D), m | 0m |  |
| gNB antenna height | 3m |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |

Table 6.1.1-4: Scenario 2: Urban micro (UMi) scenario parameters

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Layout | Hexagonal grid, 19 or 7 macro sites, 3 sectors per site, ISD = 200m, Note 1 Wrap-around is applied, Note 2 |  |
| Total gNB TX power, dBm | 44dBm | 37dBm per panel EIRP should not exceed 73dBm |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1), (dH, dV) = (0.5, 0.8)λ, - Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2), (dH, dV) = (0.5, 0.5)λ, (dg,H,dg,V) = (4.0, 2.0)λ, - Note 1 |
| gNB antenna radiation pattern | Directional, 8dBi – Note 1 Table 6.1.1-5 | Directional, 8dBi – Note 1 Table 6.1.1-5 |
| Channel model | UMi Street Canyon – Note 3 |  |
| Penetration loss | For outdoor UEs: 0dB |  |
| Number of floors | All UEs are on the ground. |  |
| UE drop procedure | 100% outdoor uniformly distributed over the horizontal area |  |
| UE mobility (for modeling Doppler effects) | Outdoor: 3km/h |  |
| UE height, m | 1.5m |  |
| Min. gNB-UE distance (2D), m | 10m |  |
| gNB antenna height | 10 m by default. Companies can bring results with uniform distribution [5-20]m​ |  |
| Note 1: According to 3GPP TR 38.802 Note 2: In case if interference considerations are not properly taken into account for 7 sites companies are encouraged to provide results for 19 sites. Note 3: According to 3GPP TR 38.901 |  |  |

Table 6.1.1-5: gNB antenna patterns for UMi and UMa

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8dBi |

Table 6.1.1-6: Scenario 3: Outdoor macro (UMa) scenario parameters

|  | FR1 Specific Values |
| --- | --- |
| Layout | Hexagonal grid, 3 sectors per site, 7 or 19 macro sites, ISD = 500m – Note 1 Wrap-around is applied. Note 2 |
| Total gNB TX power, dBm | 49dBm |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1), (dH, dV) = (0.5, 0.8)λ – Note 1 Applicable for 2GHz and 4 GHz carrier frequency. |
| gNB antenna radiation pattern | Directional, 8dBi – Note 1, Table 6.1.1-5 |
| Channel model | UMa scenario – Note 3 |
| Penetration loss | For outdoor UEs: 0dB For indoor UEs: 20dB+0.5d2D-in – Note 3 |
| Number of floors, (floor height) | 8, (3m) |
| Antenna Height: | Uniformly distributed [20-50] m – Note 4 25m + α, where α~uniform[-5, 25] |
| UE Height | 3(nfl – 1) + 1.5 m – Note 4 where, nfl ~ uniform(1,Nfl) and Nfl = 8 |
| UE dropping procedure | 50% indoor and 50% outdoor uniformly distributed over the horizontal area (separate statistic) |
| Min. gNB-UE distance (2D), m | 35m |
| UE mobility (for modeling Doppler effects) | For indoor UEs: 3km/h For outdoor UEs: 60km/h |
| Note 1: According to 3GPP TR 38.802 Note 2: In case if interference considerations are not properly taken into account for 7 sites companies are encouraged to provide results for 19 sites. Note 3: According to 3GPP TR 38.901 Note 4: According to 3GPP TR 37.857 |  |

## IIoT Scenarios Uu Channel Models

For evaluating performance, the following scenarios are defined for RAT-dependent positioning techniques for the NR positioning enhancements study

-	Scenario 1. InF-SH for FR1 and FR2

-	Scenario 2. InF-DH for FR1 and FR2

In the evaluation of all scenarios, the absolute-time-of arrival model defined in TR 38.901 is considered, without modification. Parameters specific to scenario 1and 2 are detailed in table 6.1-1.

Table 6-1: Common scenario parameters applicable for all scenarios

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 3.5GHz | 28GHz |
| Bandwidth, MHz | 100MHz | 400MHz |
| Subcarrier spacing, kHz | 30kHz for 100MHz | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Baseline: Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU Optional: 4-panels UE: - The antenna elements of the same polarization of the same panel is virtualized into one TXRU |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 in TR 38.855 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns (Optional) |  |
| UE/gNB RX and TX timing error | (Optional) The UE/gNB RX and TX timing error, in FR1/FR2, can be modeled as a truncated Gaussian distribution with zero mean and standard deviation of T1 ns, with truncation of the distribution to the [-T2, T2] range, and with T2=2*T1: - T1: X ns for gNB and Y ns for UE - X and Y are up to sources - Note: RX and TX timing errors are generated per panel independently Apply the timing errors as follows: - For each UE drop, - For each panel (in case of multiple panels) - Draw a random sample for the Tx error according to [-2*Y,2*Y] and another random sample for the Rx error according to the same [-2*Y,2*Y] distribution. - For each gNB - For each panel (in case of multiple panels) - Draw a random sample for the Tx error according to [-2*X,2*X] and another random sample for the Rx error according to the same [-2*X,2*X] distribution. - Any additional Time varying aspects of the timing errors, if simulated, can be left up to each company to report. - For UE evaluation assumptions in FR2, it is assumed that the UE can receive or transmit at most from one panel at a time with a panel activation delay of 0ms. |  |
| Note 1: According to TR 38.802 Note 2: According to TR 38.901 |  |  |

Table 6.1-1: Parameters common to InF scenarios

|  |  | FR1 Specific Values |  | FR2 Specific Values |
| --- | --- | --- | --- | --- |
| Channel model |  | InF-SH, InF-DH |  | InF-SH, InF-DH |
| Layout | Hall size | InF-SH: (baseline) 300x150 m (optional) 120x60 m InF-DH: (baseline) 120x60 m (optional) 300x150 m |  |  |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |  |  |
|  | Room height | 10m |  |  |
| Total gNB TX power, dBm |  | 24dBm | 24dBm EIRP should not exceed 58 dBm |  |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |  |
| gNB antenna radiation pattern |  | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |  |
| Penetration loss |  | 0dB |  |  |
| Number of floors |  | 1 |  |  |
| UE horizontal drop procedure |  | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area should be - (baseline) at least the convex hull of the horizontal BS deployment. - (optional) It can also be the whole hall area if the CDF values for positioning accuracy is obtained from whole hall area. |  |  |
| UE antenna height |  | Baseline: 1.5m (Optional): uniformly distributed within [0.5, X2]m, where X2 = 2m for scenario 1(InF-SH) and X2= for scenario 2 (InF-DH) |  |  |
| UE mobility |  | 3km/h |  |  |
| Min gNB-UE distance (2D), m |  | 0m |  |  |
| gNB antenna height |  | Baseline: 8m (Optional): two fixed heights, either {4, 8} m, or {max(4,), 8}. |  |  |
| Clutter parameters: {density , height ,size } |  | Low clutter density: {20%, 2m, 10m} High clutter density: - Baseline): {40%, 2m, 2m} for fixed UE antenna height and gNB antenna height - (Optional): {40%, 3m, 5m} - (Optional): {60%, 6m, 2m} |  |  |
| Note 1: According to Table A.2.1-7 in TR 38.802 |  |  |  |  |

## Simulation Assumptions

This section includes the simulation assumptions used for the V2X evaluations in Section 3.

| UE drop | Highway |
| --- | --- |
| Lanes | 6, 3 in each direction. |
| Number of UEs | 42 |
| UE speed | 140 km/h |
| Ranging method | SL-RTT, single-shot, genie calibration. |
| Antenna configuration | 2x2 |