3GPP TSG RAN WG1 #109-e																	R1-2205040

e-Meeting, May 9th  – 20th, 2022

Agenda item:	9.5.2.2

Source: 	Qualcomm Incorporated

Title: 	Phase Measurements in NR Positioning

Document for:		Discussion and Decision

# Introduction

The Rel-18 Positioning Study Item RP-213588 [1] includes the following:

Regarding higher accuracy, two promising techniques identified in earlier studies will be considered in Rel-18: one is to take the advantage of the rich 5G spectrum to increase the bandwidth for the transmission and reception of the positioning reference signals based on PRS/SRS bandwidth aggregation for intra-band carriers, and the other is to use the NR carrier phase measurements. GNSS carrier phase positioning has been used very successfully for centimetre-level positioning but is limited to outdoor applications. NR carrier phase positioning has the potential for significant performance improvements for indoor and outdoor deployments in comparison with the existing NR positioning methods, as well as shorter latency and lower UE power consumption in comparison with RTK-GNSS outdoors. - Study solutions for accuracy improvement based on NR carrier phase measurements [RAN1, RAN4] - Reference signals, physical layer measurements, physical layer procedures to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning [RAN1] - Focus on reuse of existing PRS and SRS, with new reference signals only considered if found necessary

In this paper, we present our views on studying incorporating carrier-phase techniques into NR positioning.

# Carrier phase in NR

## Basics of carrier phase positioning

Carrier-phase techniques have been studied and deployed in GNSS systems, but are being considered for the first time now for a cellular positioning scenario in NR Rel-18. Hence, we summarize in this section, at a basic level, what constitutes a carrier-phase based technique and what distinguishes it from timing-based or angle-based techniques that have been standardized in NR Rel-16/17.

Consider a baseband positioning waveform p(t), 0 ≤  t ≤ T [such as DL PRS or UL SRS-for-positioning], which is transmitted by a device ‘A’ to device ‘B’. Device ‘A’ begins the transmission at the start of its internal clock that counts time at its transmit antenna, which is at offset TA with respect to the global/absolute time ‘t’, and uses upconversion to RF carrier frequency fc. Thus, the transmit waveform at Tx antenna of device ‘A’, and corresponding receive waveform at Rx antenna of device ‘B’, are as follows:

p(t-TA) exp(-j(2πfct +θA))     [transmit waveform at Tx antenna of device ‘A’]

h*p(t- τ- TA) exp(-j(2πfc(t- τ) +θA)) [receive waveform at Rx antenna of device ‘B’]

where θA is the initial phase offset at the transmitter, and h is the (complex) gain and τ is the delay of the propagation path from A to B. Note here that for simplicity we consider a single path, but with multiple paths we just need to sum all the receive waveforms corresponding to each path. Also note that we could have written the argument of the exp() function in the transmit waveform alternatively as -j(2πfc(t-TA ) +θA) to emphasize that the transmission starts at time TA, but this is equivalent upon redefinition of θA to include 2πfcTA.

The receiver then demodulates it, essentially by multiplying it with a carrier signal exp(j(2πfct +θB)) generated based on its own internal clock that counts time at its receive antenna, and is at offset TB with respect to the global/absolute time ‘t’. Thus, the device ‘B’ baseband waveform is:

h*p(t- τ- TA) exp(jθcp), where θcp = 2πfcτ + θB -θA [receive waveform at baseband of device ‘B’]

This equation allows us to highlight the key difference between timing-based and carrier-phase-based positioning techniques. Both techniques ultimately try to estimate in some form, the propagation delay τ , which can then be converted into an estimate of distance and thus position. Timing-based techniques do this by trying to determine the time t = τ + TA of arrival of the start of the positioning waveform at the receiver, whereas carrier-phase techniques do this by estimating the phase θcp, which also clearly depends on τ.

More specifically, timing-based technique often involve a receive correlator Ct= u=0Trup*(u-t-TB) whose output C(t) is then fed to a peak-energy-detector. The timing of the peak is at t= τ + TA  - TB, thus detecting this timing can extract the required delay τ, while completely ignoring the phase of the correlator output. Carrier-phase based technique on the other hand notices that the phase of the peak-energy correlator output is θcp = 2πfcτ + θB -θA (modulo 2π) which also allows extracting the required delay τ. In both techniques, this delay of interest comes combined with ‘initial offset terms’ (TA  - TB in timing-based and θB -θA in phase-based techniques), which can be handled by differential techniques (elaborated in Section 2.2). However, the carrier-phase technique presents some novel features already immediately observable:

- Phase added by channel: Note that in the above analysis the phase of the peak-energy correlator should also include the phase of the complex channel gain coefficient h, which we have ignored above. This includes the gain of the actual over-the-air propagation path from Tx antenna to Rx antenna, as well as any phase added by the antenna response at both Tx and Rx. Consider first the simple model where there is only a single propagation path going directly between the single Tx and Rx which are both ideal omnidirectional antennas. In this case, the OTA propagation path gain phase depends on the propagation distance (or equivalently, propagation time), and this dependency is precisely the one that we are trying to exploit to uncover the propagation time, and it is already captured in the expression for θcp . Thus, in this case, indeed h is not complex but purely real (because the ‘complex’ part is absorbed in θcp ). In case of multipath with different delays, the same observation holds, and as long as we can estimate the phase corresponding to the path delay of interest (usually earliest path in case of positioning), we can again use the same analysis. However, if the antennas are not ideal omni [for example, the antenna is itself created out of an antenna array with beamforming over the array elements], then the coefficient ‘h’ includes the phase due to the antenna phase-response, and further that phase may depend on the AoD (from Tx) and AoA (at Rx). It may be possible in some cases to treat this ‘phase of h’ in the same way as the θB -θA, using differential techniques, but as we describe in Section 2.2, there are specific conditions to allow it, and they will very often not hold in the cellular context. Section 2.2 also shows an example of the dependence of the phase of h on AoD/AoA for the case of a 4-element uniform linear array

- Integer ambiguity: The delay τ of interest is present in the expression for the phase θcp, which is the argument of the exp() function. So, since we only know the phase modulo 2π, we thus only know the delay modulo 1 time-period, or the propagation distance modulo 1 wavelength. The (usually very large) number of integer wavelengths of propagation distance is unknown. The promise of centimeter-accurate location using carrier phase can only be realized if this can be determined. A rough initial guess for this number can be determined from an initial coarse position, and this has to then be further refined using the carrier phase estimates. This is discussed in more detail in Section 2.3.

λ

Fractional λ

Figure 1: Illustrating integer ambiguity with carrier phase

- Accuracy comparison: The accuracy of TDOA is limited by the accuracy of identifying the timing of the correlation peak, which is inversely proportional to the bandwidth B of the positioning waveform p(t). [The Cramer-Rao lower bound on the variance of the peak detector in AWGN is proportional to 1/(B*sqrt(SNR))]. Whereas with carrier phase, assuming the integer ambiguity can be perfectly removed, the accuracy is then within one wavelength, e.g., 1cm at fc=30GHz. Of course the accuracy with which integer ambiguity itself can be removed will depend on the accuracy of the carrier phase measurement, and this is what the study item should aim to explore.

- Accuracy of phase measurement: Besides the AWGN noise which is added to the received signal and will thus impact the phase measurement, there are also noise/error sources that are added directly to the phase of the received signal, namely phase-noise and frequency offset. Phase noise is a random noise added to the phase, caused due to non-ideal VCO/PLL for the modulation/demodulation. Frequency offset occurs when the fc value used at the transmitter and the receiver are not exactly the same, which causes the received signal in baseband as derived above to be further multiplied by a ‘phase ramp in time’, exp(j2πfΔt) (where fΔ is the frequency offset between the Tx and Rx carrier frequencies), which is similar to the phase-noise but deterministic instead of random. These noises/errors should be small enough over the integration time T of the correlator (i.e., the positioning waveform duration), and will impact the accuracy of phase measurements.

It has been suggested that carrier phase may potentially need low bandwidth because it uses measurement of phase rather than timing of the received signal. The above analysis however should readily make it clear that this argument only applies in a single-path channel. In case of multipath, the ideally desired measurement is that of the phase of the path of interest (the LOS or earliest path in case of positioning). Closely spaced paths will impact each others’ phases and render this desired measurement inaccurate. Larger BW will be needed to resolve closer-spaced paths and thus observe the phase of the earliest path. Further, as we show in Section 2.3, the resolution of the integer ambiguity problem raised above requires an initial coarse position, and higher accuracy of this position likely translates into higher accuracy and/or lower complexity of the IAR solution. Large BW can help to achieve a more accurate initial coarse position used for carrier phase positioning. Thus, as per the SID description that says, “Focus on reuse of existing PRS and SRS, with new reference signals only considered if found necessary”, we propose the following:

Proposal 1: Reuse Rel-16/17 PRS and SRS-for-positioning waveforms, and do not introduce a lower-BW positioning reference signal for carrier phase positioning.

We also note the contrast between carrier-phase and phase-difference-of-arrival (PDOA) based schemes. PDOA measures the difference between the received phases of the same transmission at different Rx antenna elements, or alternatively, the difference between received phases corresponding to different Tx antenna elements at the same Rx antenna.

Consider first, the case of a common transmission received with multiple Rx antennas. Assuming that (1) the antenna elements are close enough relative to their distance from the transmitting device, so that they both see the same channel paths and thus the same AoAs, and (2) they are both driven by sufficiently identical downconversion/demodulation chains that the measured PDOA corresponds mainly to the difference in propagation distances to the two antennas, the measured PDOA is then a measure of exp(j2πfcτdelta) where τdelta = d*cos(θAoA)/c where c=speed of light, d=distance between the antenna elements, and θAoA = angle between the direction of arrival and the direction of the line joining the antenna elements. Thus, phase measurements can be used to extract the AoA, which can then be used to compute position. However, this approach is more appropriately classified as an angle-based technique rather than as a carrier-phase-based technique: It uses phase measurements (as opposed to RSRP measurements) to estimate the the angle of arrival, rather than to estimate range (or double-differenced range) to within a single wavelength. Note that PDOA as described here could already be supported by gNB by implementation in UL AoA method. It is also closely related to RSRP-based AoA/AoD: If the receiver does Rx beamforming, the optimal combiner co-phases all the individual antenna elements by undoing these phase-differences. Thus, comparing RSRPs of different combiners can be seen as an indirect way of comparing the phases at the Rx antennas.

Next, consider the case of distinguishable transmisssions from multiple Tx antennas being received by a common Rx antenna, with the Rx device measuring the PDOA between the two Tx antennas. The geometry of the situation is identical to the earlier case (common Tx and multiple Rx antennas), and the difference between the phases now again corresponds to exp(j2πfcτdelta) where τdelta = d*cos(θAoD)/c, as shown in the figure below.  If the Rx device has the knowledge of the Tx device antenna geometry, it can thus extract the AoD from the PDOA measurement. Otherwise, it has to report the PDOA to the entity that has that knowledge and can compute the AoD. In the NR context, the former represents the UE-based DL-AoD, which requires new assistance data on the TRP’s antenna geometry, while the latter represents UE-assisted DL-AoD, which requires a new PDOA report. Although this report is not technically a report of carrier phase [as described earlier], it is still a report of a phase, and the same reporting formats needed for carrier phase can be used to report this PDOA as well. Section 3 shows the performance benefits from using this PDOA report.

Figure 2: PDOA with multiple Tx and single Rx antenna. Note: target UE is assumed to be in the far-field of the transmitter (so AoDs from both Tx antennas can be assumed to be the same)

## Differential schemes and need for PRUs

In the previous section, the timing based techniques estimated τ + TA  - TB. This section analyzes the ‘offset’ term TA  - TB and ways to handle it. TA and TB are offsets of the clocks of devices A and B relative to the global/absolute time. They may themselves vary with time (e.g., from one positioning occasion to another), due to clock drifts/errors at the two devices. Similarly, the carrier phase measurement estimates 2πfcτ + θB -θA , and we need to handle the offset terms θB -θA, where θB , θA represent initial phase-offsets at the transmitter A and receiver B, which may also vary with time due to phase drift/errors.

RTT may be thought of as a way to handle the clock offsets in a ‘differential’ manner, where the ‘differential’ of the Rx clock at B is with respect to its Tx clock, and similarly the differential of Tx clock at A is with respect to its Rx clock. All the clocks here are supposed to be counting out time at the respective antenna (Rx/Tx) of the respective device. Thus, the antenna-to-baseband group-delays are then included in the Rx-Tx measurements, and need to be calibrated, and residual errors after calibration can be managed by the TEG framework adopted in Rel-17. Apart from that, there is fundamentally no difference between ‘Rx clock’ and ‘Tx clock’ at the same device, and thus the method is quite robust to clock drifts. However, combining of measurements made at different times at the two devices A and B could still be error-prone if the clock at device B is adjusted based on transmissions or commands from device A in between those two times (as happens in NR Uu with TA commands or UE-autonomous corrections).

There is no easy analog of RTT that can be applied to carrier-phase, because a device may not necessarily have a notion of a master ‘phase reference’ that is common to both its transmit chain and its receive chain, analogous to a common ‘device clock’ that serves as both Tx clock and Rx clock of the device. Events such as DRX/DTX or Rx-Tx transitions at a UE may result in loss of phase coherence even within one chain (Tx chain or Rx chain), and even for devices that keep two separate chains ‘always on’, there is not necessarily any coupling between the Tx phase and Rx phase, because there is no requirement or advantage of forcing such a coupling for data communications.

TDOA is a way to handle the Rx clock offset TB: By differencing measurements from two different transmitter devices A and A’, the TB term subtracts out, provided both measurements are ‘close enough in time’ that TB has not changed much in between them (i.e., the Rx clock drift is small enough). As the Rx clock is with respect to time at the antenna, any difference in group-delay between antenna and baseband (where the measurements are made) should also be calibrated to ensure that the TB term subtracts out leaving sufficiently small residual error. With this differencing, the Tx clock offset TA then becomes a ‘differential offset’ TA - TA’ , which represents the synchronization offset between the Tx antennas of the two transmit devices A and A’ at the times that they were measured by the receiving device B. This offset could itself be due to multiple causes such as uncalibrated group-delays between the baseband and antenna of one or both of the Tx nodes (A and A’), clock drifts, etc. This offset needs to be tracked sufficiently tightly to achieve the targeted TDOA accuracy. Any residual errors in this tracking may be further minimized by a double-differential scheme: Differencing two such single-difference measurements from two receiving devices B and B’ that both observed the same transmissions from A and A’ removes the synchronization offset altogether. If B and B’ observed different transmissions (e.g., different PRS occasions) from A and/or A’, then again, the same idea applies provided the variation in the relevant time offsets across those different transmissions is small enough. Some coordination between the receiver nodes B and B’ could potentially be useful to ensure this.

It is readily seen that both the single and double-difference concepts in TDOA can directly be applied to carrier phase as well, to remove the effects of the initial phase offsets θB , θA. Further, unlike in TDOA where the time-synchronization between transmitters may potentially be tightly tracked (avoiding the need for differencing out the synchronization errors using two receive devices), with carrier phase there may be no easy way to synchronize or track the initial transmit phase offsets of two devices. Thus, the double-difference scheme becomes much more essential. Hence, the standardization needed to enable PRUs, which was considered in Rel-17 but delayed to Rel-18, will become an essential part of Rel-18 if carrier-phase based methods have to be adopted. This should include the role of PRU and LMF in both UE-based and UE-assisted positioning.

Observation 1: PRUs are vital to carrier-phase based NR positioning schemes

Proposal 2: Study carrier-phase schemes that assume availability of PRUs

As observed in Section 2.2, even in the carrier phase measurement in the case of a single Tx/Rx device pair (A/B), in addition to the initial phase offset term θB -θA, there is also the phase of the channel coefficient h on that link, which should include the effect of Tx and Rx beamforming and antenna phase response, which itself is a function of AoD/AoA. It is now evident that double-differencing schemes will not eliminate this coefficient: The AoDs from A and A’, as well as the corresponding AoAs at B and B’, could all be different, and the coefficient for each of the 4 links depends on the corresponding AoD and AoA.

The figures below illustrate the dependence of phase on AoA/AoD by showing the beam gain and phase as a function of azimuth angle for a 4-element uniform linear array with beam coefficient of 1 used for all the 4 elements. There is a phase transition of 180 degrees when switching from main-lobe to side-lobe. In this case a coarse AoA estimation may be used to account for the phase transition, by determining whether the reception of the path of interest is via the main-lobe or the side-lobe. In more complex cases of phase variation vs. azimuth, we need both accurate characterization of this variation and accurate AoA estimation to be able to account for it. Such a characterization of antenna phase-response may be additional effort in the development of the relevant devices, which is needed only for carrier-phase positioning. Thus it may potentially be hard to justify for many classes of more general-purpose (as opposed to positioning-specific) commercial devices such as smartphones. Note that the need for such characterization applies both to individual antennas as well as to antenna arrays, and thus to both FR1 and FR2.

Figure 3: Gain and phase vs Azimuth of a 4-element ULA with identical beamforming weights

Proposal 3: Study the impact of antenna phase center calibration on carrier phase performance.

## Integer ambiguity resolution (IAR)

IAR has been well studied in the context of GNSS carrier phase, and many practical algorithms have been proposed, for example, [2]. Here we will discuss the basic principles of IAR, and suggest a simple ‘brute-force’ IAR solution, which may be considered as a possible baseline for harmonizing simulation results across companies.

Every carrier phase measurement includes an associated unknown integer number of cycles. As we have seen in the previous section, carrier phase positioning requires double-differencing, which will then result in a corresponding unknown integer double-differenced number of cycles. The goal of IAR is to solve for these unknowns. A phase measurement θ can be converted to a range measurement λ(N+ θ/2pi) under a hypothesis N for the corresponding unknown integer number of cycles. Thus, with a hypothesized IAR solution, we can convert the measured phases to a corresponding set of ranges, compute position based on those ranges, and then compute the expected measurements (e.g., of phase, or of timing of arrival, TDOA, etc) based on that position.

The hypothesis IAR solution with the ‘best match’ between these expected measurements and the actually observed measurements is the solution. Many different optimization criteria are possible for what constitutes ‘Best match’ here, e.g., best in the ML or MAP sense (which may be computationally challenging), or in the sense of minimizing the maximum or the mean error across all measurements, etc. The set of hypotheses may be chosen as some hyper-grid or subset of hypotheses ‘near’ to an initial solution. The initial solution is based on the position computed from the initial measurements that do not involve carrier-phase.

Conceptually, any practical IAR solution could be thought of as a smarter rather than exhaustive search on this hyper-grid. However, given the wide number of possible variations in these solutions, we propose that the above brute-force solution be considered at least as a baseline. It is easy to parameterize by specifying the resolution and range of the search hyper-grid and the objective function used, and should thus help to compare results across companies.

Proposal 4: Use brute-force integer ambiguity resolution as a baseline for comparison of simulation results.

## Cycle slips

A cycle slip is a discontinuity in a receiver’s phase lock on a satellite/TRP signal. A power loss, a very low signal-to-noise ratio, a failure of the receiver software, a malfunctioning satellite/TRP oscillator can cause a cycle slip. It can also be caused by severe ionospheric conditions. Most common, however, are obstructions such as buildings, trees, etc., that are so solid they prevent the satellite signal from being tracked by the receiver. Under such circumstances, when the satellite reappears, the tracking resumes.

Coded pseudorange measurements (and therefore, timing-based positioning methods) are not as affected by cycle slips as are carrier phase measurments.  On the other hand, carrier phase positioning accuracy suffers if cycle slips are not detected and repaired. A cycle slip causes the critical component for successful carrier phase positioning, a resolved integer cycle ambiguity, N, to become instantly unknown again. In other words, lock is lost. When that happens, correct positioning requires that N be re-established.

In the NR context, a cycle-slip can be thought of as a loss of phase coherence. This could happen for example with DTX/DRX at the UE side. There was discussion on maintaining phase continuity across slots for DMRS bundling for coverage extension in Rel-17. Similarly, the conditions under which phase continuity over time can be assumed should be discussed for carrier-phase positioning as well. Note that similar discussion is also needed related to phase-continuity across multiple component carriers or positioning frequency layers in the context of PRS bandwidth aggregation, which will be studied in RAN4 in Rel-18, and can also be useful for high accuracy Redcap positioning, to be studied in RAN1 [3].

Proposal 5: Study the need for phase coherence across time when evaluating carrier-phase based techniques. Simulation evaluations should clarify assumptions made on phase-coherence at both transmitter and receiver.

Figure 4: Illustrating cycle-slips

## Simulation framework, initial results, and considerations on accuracy

As the SID mentions, carrier phase has potential in both indoor and outdoor use cases, so we propose at least including Indoor Hotspot, Indoor-factory and UMi simulation scenarios as defined in Rel-16.

Proposal 6: For carrier phase positioning, reuse Simulation Assumptions of NR Rel-16/17 for InH, InF, UMi for FR1 and FR2.

Proposal 7: For carrier phase positioning, consider any new scenarios that are introduced for other Rel-18 positioning techniques.

The figure below shows an initial simulation result in InF-SH scenario in FR2 (30GHz) using NR Rel-16 simulation methodology, illustrating the potential gains from carrier phase technique. The dotted blue curve shows performance of DL-TDOA without any timing errors (perfect sync across the TRPs and no group-delay errors). With truncated Gaussian (with range [-2T1, 2T1]) modelling of timing errors – T1=10ns for TRP synchronization, 1ns for gNB Tx group-delay and 2ns for UE Rx group-delay, we observe from the dotted brown line that double-differenced TDOA is able to correct for the timing errors to a good extent, producing a slight degradation relative to the case of perfect timing. The carrier phase measurement at the UEs is modeled as genie phase + additive phase-noise, where genie phase corresponds to the propagation time only (i.e., to θcp in Section 2.1), and the phase-noise is uniform in range [-θ, θ]. The initial location as computed with double-differenced TDOA (dotted brown curve) is used to determine the initial integer number of cycles corresponding to all the carrier phase measurements. Then, the brute-force IAR described in Section 2.3 is used, with search space ranging from -4 to +4 for each double-differenced integer ambiguity. The search optimization criterion for ‘best match’ as described in that section was to minimize the maximum of the ‘mismatch’ in the double-differenced phase measurement. We observe that carrier-phase can produce significant improvements and lead to sub-millimeter accuracy for more than 80th percentile of the users, even when θ=10 degrees. However, the performance floors out at higher percentiles, eventually getting comparable to DD-TDOA without carrier phase. This is because the brute-force search region is not large enough to include the correct integer-ambiguities in some cases. With no additive phase noise, we verified that using the genie IAR solution resulted in ‘perfect’ positioning (errors of the order of picometers, limited only by the precision of the double-precision arithmetic in the simulation code), and the figure shows a small error-floor at a very high percentile (around 97th percentile), caused due to the same reason – insufficiently large IAR search-space.

Figure 5: Initial simulation result showing gains from carrier-phase positioning over double-difference TDOA.

Observation 2: Carrier phase has potential for millimeter-level accuracy at 80th percentile if the errors in the phase measurement are limited to ±10 degrees.

Observation 3: Carrier phase measurement errors can occur due to a combination of channel estimation error, antenna phase center calibration inaccuracy, and receiver imperfections such as phase noise and frequency offset.

Proposal 8: Study carefully the sources and impacts of carrier phase measurement errors.

Regarding the accuracy targets for carrier phase positioning, we observe that unlike in Rel-16 and Rel-17, there have been no explicit new targets/use-cases defined by 3GPP SA1 for positioning accuracy that are tighter than the Rel-17 target of 20cm for some IIoT use cases. From a technology perspective, however, it is quite evident that achieving tighter accuracy will naturally result in enabling new applications that can exploit this accuracy. An analogy may be drawn with increasing data rates in cellular networks, wherein many of the applications that exploit these higher data rates arise only after the networks themselves are built out. Thus, higher data rates are an intrinsically desirable goal even without a specific target application in mind, because it is quite obvious that there will be such applications to follow once the data rates are made available. The same may be said for higher accuracy positioning, wherein applications such as precise robotic manipulation (e.g., finding and driving a small screw into its desired location) easily provide avenues for potential exploitation of millimeter-level positioning accuracy. As shown from the initial simulation result, carrier phase has potential for accuracy that is comparable to or smaller than the carrier wavelength. However, as noted with this result, realizing this potential is critically dependent on containing the errors within the phase measurement, which should therefore be studied carefully:

Proposal 9: A desired accuracy target for carrier phase positioning should be of the order of the carrier wavelength [e.g., 1cm for 30GHz, or 7.5cm for 4GHz], in presence of realistic sources of error in the carrier phase measurements.

# Phase-Difference AoD: PDOA

The following agreement was made during NR Rel-17 WI:

Agreement: - For both UE-based and UE-assisted DL-AOD study the following enhancements that enable the UE to measure and report (for UE-assisted) information related to the first arriving path - Option 1: Information corresponds to PRS-RSRP of the first arriving path - Option 2: Information corresponds to the angle of departure of the first arriving path - Option 3: Information corresponds to the arrival time of the first path - Option 4: Information corresponds to phase of the CIR corresponding to the first arriving path - Option 5: Information corresponds to received signal value (amplitude and phase of the channel estimated from the first path which can be achieved as a combination of option 1 and option 4) of the first arriving path - FFS: Reporting of additional path to the first arriving path. - FFS: Measurement definition details - FFS: additional assistance data to support these enhancements - FFS: how the “first path” is selected among PRS resources in a PRS resource set - Note 1: Supporting multiple options as well as none of the options above is not precluded.

A phase-difference based DL-AoD would correspond to the following method: The transmitting device sends multiple PRS resources, each PRS resource via each of the physical antennas. As each PRS from the antennas in the array arrives at the receiver’s single antenna, it is phase shifted from the previous PRS due to the different distance it has traveled from the transmitter as shown graphically in the figure below. In the simple scenario shown in the figure below, one can estimate the angle of departure by measuring the phase difference between the PRS resources using a simple formula.

Figure 6: PDOA with multiple Tx and single Rx antenna. Note: target UE is assumed to be in the far-field of the transmitter (so AoDs from both Tx antennas can be assumed to be the same)

In such a method, there is no need to know what are the beam responses of the PRS resources. The receiver is only required to know the mapping of the PRS resources into the physical antennas, along with the antenna (e.g., ULA, UPA, single or multi-panel) configuration of the TRP and the relative distance of the antennas (the dH and dV parameters, as usually referred to in the 38.901 specification). It deserves to be noted that such a method is being employed already by competing technologies (e.g., Bluetooth).

Observation 4: Phase-Difference-based AoD is a positioning method employed by competing technologies (e.g. Bluetooth) which exploits the phase difference between signals transmitted from different antennas and knowledge of the Tx antenna configuration.

For the purpose of clarifying this method, and get a better understanding on what specification effort is needed to enable such a DL-AoD method, one can envision to reuse the CSI framework and configuration already specified in 38.214; i.e. the Type I codebooks and available UPA configurations:

To see this, consider for simplicity the example of Single-panel codebooks from 38.214, which uses constant modulus DFT codebooks tailored for a dual-polarized 2D UPA: DFT precoders are used, where, each precoder vector can be used to precode a single-port transmission using a single-polarized uniform linear array (ULA) with N antennas defined as

w1Dk=1Nej2π⋅0⋅kQNej2π⋅1⋅kQN⋮ej2π⋅N-1⋅kQN,

where k=0,1,…QN-1 is the precoder index and Q is an integer oversampling factor.

If the UE is provided with such a codebook configuration (as already done for CSI feedback), and receives multiple single-port PRS resources (as already needed for the RSRP-based DL-AoD method), a UE, instead of just computing the RSRP vector, would be searching for the PMI of the codebook that maximizes the received power of the first path (i.e. use the N ports to derive a Delay-Angle Response, by computing the product of the received channels by each PMI in the configured codebook, and identify the earliest path together with the strongest peak for that earliest path). It should be noted that the multiple single-port PRS resources will need to be phase-coherent for this method to work, but this is controlled by the network: If the network maps the PRS resources in a such a way that enables a phase-difference DL-AoD method, the network could signal to the UE to employ such a measurement.

Note the similarity to the CSI feedback procedure: In 3GPP procedures on CSI feedback, a UE measures multiple ports of a single CSI-RS resource, and performs a PMI sweep to identify which PMI maximizes a spectral efficiency metric, whereas in this phase-difference DL-AoD method, the UE measures multiple ports (each one associated with a different PRS resource) for the purpose of sweeping the PMIs and identify the PMI index that corresponds to the maximum received power at an estimated TOA. Such a method would exploit additional measurements, and enable the UE to report the angle that is really associated with the earliest path, and not just an RSRP measurement which can be the result of multiple paths being summed at the receiver.

In other words, for the nth received time-domain tap,  a UE can calculate the received energy across all the configured PMIs of the codebook, and choose the PMI l that maximizes the received energy. In UE-assisted DL-AoD method, a UE would report the index l corresponding to the earliest tap, and for UE-based DL-AoD, a UE would map the index l back to an angle ϕl using the knowledge of dH and the ULA config as follows:

1Nhor⋅ej2π⋅k⋅lQ⋅Nhor=1Nhor⋅ej2π⋅k⋅dH⋅sin⁡(ϕl)→lNhor=dH⋅sinϕl→

ϕl=arcsin⁡(ldH⋅Q⋅Nhor)

It should be noted that one assumption of such a method is that the TRPs would be mapping each PRS resource to a physical antenna element; an assumption that is reasonable for a variety of FR1 deployment scenarios, but not in some high-band scenarios, like FR2 cases.

It should also be noted that a UE may perform AOD oversampling during the digital PMI sweeping procedure described above (e.g., in the case of the Codebooks already specified in TS 38.214, this would corresponds to oversampling factor), to increase the precision even when there are few number of antennas at the gNBs.  Such a method is expected to have better performance than the RSRP-based DL-AoD, as shown in the simulation example below (InH 4 GHz, 100 MHz), while having a minimal assistance data overhead, i.e., antenna configuration information  with a Coodebook configuration for the UE to perform the PMI sweeping.

Such a method does not require any additional PRS resources to be sent (compared to RSRP-based DL-AoD), since the UE would be performing the angle-oversampling digitally, and reaping significant benefits in performance, as shown for example in the figure below (UMI, 4 GHz, 100 MHz) for different oversampling factors (Q={1,2,4,8,16,32}):

Observation 5: In a phase-difference-based DL-AoD method, the UE is only required to be provided a PMI codebook (e.g. Type-I codebook already in 38.214), measure multiple single-port PRS resources, perform PMI search on the received PRS ports to identify the PMI index that maximizes the power associated to the earliest arriving path.

Proposal 10: Support enhancing the UE-A/UE-B DL-AoD method with UE measurements that provide phase-difference and Angle of Departure information related to the first arrival path:

- Assistance Data Enhancement:

  - gNBs’ antenna Configuration, PMI Codebook configuration & their association to the transmitted PRS resources, PMI to DL-AoD Mapping Table (for UE-B).

- UE Measurement Enhancement:

  - Support a UE measuring multiple single-port PRS resources, sweeping a PMI codebook across the measured ports and determining the PMI index that maximizes the power associated with the earliest arriving path.

# Comparison of phase-difference AoD with Earliest RSRP AoD

In this section, we provide a simulation comparison of DL-AoD estimation performance for the InH 4 GHz scenario between the following 3 options:

- “Legacy RSRP” corresponds to the NR Rel-16 RSRP measurement

- “Earliest RSRP” corresponds to the following measurement:

  - For each PRS resource, the UE determines the earliest TOA, and reports the relative power of the earliest TOA over the maximum earliest power across all the PRS resources of the set

- “Phase Difference AoD” corresponds to the method described in the previous section.

- CDF comparison of the DL-AOD Error across all the 12 links for each UE, across all the UEs.

- CDF comparison of the DL-AOD Error across the best 5 links for each UE, across all the UEs.

- Fraction of UEs having at least X links with an AoD error of less than 2 degrees, wherein X = {1,2,…, 12}.

- Fraction of UEs having at least X links with an AoD error of less than 1 degrees, wherein X = {1,2,…, 12}

We observe that, even though the “earliest RSRP” method provides performance improvements over the “Legacy RSRP” method, additional gains are achieved if one considers the phase-difference-based method for DL-AoD.

Observation 6: At least in the InH 4 GHz scenario, Phase-Difference AoD demonstrates better performance than the “Earliest RSRP” methods, which demonstrates better performance than the “Legacy RSRP” DL-AoD.

# Conclusions

In this paper, we have discussed the following:

- We described the basics of carrier phase positioning in Section 2.1, and showed its relationships, similarities and differences from other schemes such as timing, angle, and phase-difference based positioning.

- In Section 2.2, we showed how double-differencing schemes, and therefore the deployment of PRUs, are vital to enable carrier-phase positioning in Section 2.2. We showed the importance of antenna phase-center calibration for carrier-phase positioning, and showed how it is required even if double-differencing is used

- In Section 2.3, we discussed the principles of integer ambiguity resolution, and proposed a simple ‘brute-force’ scheme as a baseline.

- In Section 2.4, we described the importance of clarifying the assumptions made on phase coherence over time in carrier-phase schemes

- In Section 2.5, we showed initial simulation results that show the potential gains achievable by carrier phase positioning, and highlighted all the underlying assumptions that need to be carefully studied to allow actual realization of these gains.

- In Section 3, we presented details of PDOA positioning, which, while not technically a carrier phase method, can reuse the same definitions of phase measurements that are needed for carrier phase positioning, and allow estimating DL AoD with better accuracy than that achieved by using the RSRP or 1st-path RSRP for AoD estimation as in Rel-16/17

We made the following observations and proposals:

Proposal 1: Reuse Rel-16/17 PRS and SRS-for-positioning waveforms, and do not introduce a lower-BW positioning reference signal for carrier phase positioning.

Observation 1: PRUs are vital to carrier-phase based NR positioning schemes

Proposal 2: Study carrier-phase schemes that assume availability of PRUs

Proposal 3: Study the impact of antenna phase center calibration on carrier phase performance.

Proposal 4: Use brute-force integer ambiguity resolution as a baseline for comparison of simulation results.

Proposal 5: Study the need for phase coherence across time when evaluating carrier-phase based techniques. Simulation evaluations should clarify assumptions made on phase-coherence at both transmitter and receiver.

Proposal 6: Reuse Simulation Assumptions of NR Rel-16/17 for InH, InF, UMi for FR1 and FR2, for carrier phase positioning.

Proposal 7: For carrier phase positioning, consider any new scenarios that are introduced for other Rel-18 positioning techniques.  Introduce an additional FDD scenario targeting scenarios with small number of antennas at the TRPs.

Observation 2: Carrier phase has potential for millimeter-level accuracy at 80th percentile if the errors in the phase measurement are limited to ±10 degrees.

Observation 3: Carrier phase measurement errors can occur due to a combination of channel estimation error, antenna phase center calibration inaccuracy, and receiver imperfections such as phase noise and frequency offset.

Proposal 8: Study carefully the sources and impacts of carrier phase measurement errors.

Proposal 9: A desired accuracy target for carrier phase positioning should be of the order of the carrier wavelength [e.g., 1cm for 30GHz, or 7.5cm for 4GHz], in presence of realistic sources of error in the carrier phase measurements.

Observation 4: Phase-Difference-based AoD is a positioning method employed by competing technologies (e.g. Bluetooth and WiFi) which exploits the phase difference between signals transmitted from different antennas and knowledge of the Tx antenna configuration.

Observation 5: In a phase-difference-based DL-AoD method, the UE is only required to be provided a PMI codebook (e.g. Type-I codebook already in 38.214), measure multiple single-port PRS resources, perform PMI search on the received PRS ports to identify the PMI index that maximizes the power associated to the earliest arriving path.

Proposal 10: Support enhancing the UE-A/UE-B DL-AoD method with UE measurements that provide phase-difference and Angle of Departure information related to the first arrival path:

- Assistance Data Enhancement:

  - gNBs’ antenna Configuration, PMI Codebook configuration & their association to the transmitted PRS resources, PMI to DL-AoD Mapping Table (for UE-B).

- UE Measurement Enhancement:

  - Support a UE measuring multiple single-port PRS resources, sweeping a PMI codebook across the measured ports and determining the PMI index that maximizes the power associated with the earliest arriving path

# References

- RP-213588, “Revised SID on Study on expanded and improved NR positioning ”, RAN#94e, December 6-17, 2021.

- “The least squares ambiguity decorrelation adjustment: A method for fast GPS integer ambiguity estimation”. P.J.G. Teunissen, Journel of Geodesy.

- R1-2205042, “Discussion on Positioning for Redcap UEs”, RAN1#109-e, May 9-20, 2022