3GPP TSG RAN WG1 #109-e																		R1-2205041

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.5.2.3

Source: 	Qualcomm Incorporated

Title: 	Requirements, Evaluations, Potential Enhancements for Low Power High Accuracy Positioning

Document for:		Discussion and Decision

# Introduction

At RAN #94 a new SID has been approved for expanded and improved NR positioning which includes the following goal with regards to the Reduced Capability UEs:

- Study the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the evaluation, and, if found beneficial, study potential enhancements to help address any limitations [RAN2, RAN1] - Study is limited to a single representative use case (use case 6 as defined TS 22.104). The choice of selected use case can be reviewed at the start of the study. - Study is limited to enhancements to RRC_INACTIVE and/or RRC_IDLE state

SA1 has introduced requirements for LPHAP (Low Power High Accuracy Positioning) for industrial IoT scenarios. A typical scenario of interest is use case 6 as defined TS 22.104, which corresponds to tracking of workpiece (in- and outdoor) in assembly area and warehouse with a target accuracy of <1m, a positioning interval of 15-30 seconds, and a battery life of 6-12 months.

While Rel-17 NR positioning has introduced support for positioning in RRC_INACTIVE state, there is a need to evaluate whether the current system allows LPHAP requirements to be met.

In this paper, we present our views on LPHAP positioning, requirements, evaluation assumptions for power consumption, and potential enhancements within RRC Inactive and/or RRC Idle positioning.

# LPHAP & Use case 6

Low power high accuracy positioning is an integral part of a considerable number of industrial applications. Examples of target applications for low power high accuracy positioning are asset tracking in process automation, tracking of vehicles, and tool tracking.

| Use Case # | Horizontal accuracy | Corresponding service level (22.261) |  | Positioning interval/ duty cycle |  | battery life time/ minimum operation time |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 m | Service Level 1 |  | on request |  | 24 months |  |
| 2 | 2 m to 3 m | Service Level 2 |  | < 4 seconds |  | > 6 months |  |
| 3 | < 1 m | Service Level 3 |  | no indication |  | 1 work shift - 8 hours (up to 3 days, 1 month for inventory purposes) |  |
| 4 | < 1 m | Service Level 3 |  | 1 second |  | 6 - 8 years |  |
| 5 | < 1 m | Service Level 3 |  | 5 seconds - 15 minutes |  | 18 months |  |
| 6 | < 1 m | Service Level 3 |  | 15 s to 30 s |  | 6 - 12 months |  |
| 7 | 30 cm | Service Level 5 |  | 250 ms |  | 18 months |  |
| 8 | 30 cm | Service Level 5 |  | 1 second |  | 6 - 8 years (no strong limitation in battery size) |  |
| 9 | 10 m |  | Service Level 1 | 20 minutes | 12 years (@20mJ/position fix) |  |

Use case 6 (Flexible modulare assembly area: Tracking of workpiece (in- and outdoor) in assembly area and warehouse) has a horizontal accuracy <1m with a positioning duty cycle of 15s to 30s, and a battery life time of 18 months. Given all the set of the use cases shown above, we believe that indeed use case 6 is a good example to be evaluated.

Proposal 1: Keep use case 6 (and the corresponding requirements) as the representative use case to be evaluated.

# Power Consumption Evaluation Assumptions

The following agreements were made during NR Rel-17 SI with regards to the power consumption:

Agreement (Rel-17 SI): UE power consumption for NR positioning can be optionally evaluated in the SI. Note: It is up to each company on how to evaluate the power consumption for positioning. The UE power consumption models developed in TR38.840 can be considered as the starting point for defining the UE power consumption model for the evaluation for NR positioning Conclusion: For power consumption evaluation, it is up to each company to detail their methodology (including power model) for evaluation.

We think that in this release, it may be useful to agree on more details for the power consumption methodology in order to achieve a common understanding of the power consumption of different methods and procedures.

Proposal 2: Support agreeing on a common power consumption methodology, using TR38.840 as a starting point, for evaluating at least the following methods:

- RRC Inactive UE-based Positioning

  - DL-TDOA/DL-AoD with DL-PRS reception at the UE

- RRC Inactive network-based Positioning

  - UL-AoA/UL-TDOA with SRS for Positioning transmission by the UE

- RRC Inactive UE-assisted Positioning

  - M-RTT with SRS for Positioning transmission and DL-PRS reception.

## Power Consumption Modeling

With regards to the power consumption modelling, following the TR38.840, we make the following suggestions for the relative powers of different Power states:

Proposal 3: For RRC Inactive Positioning and power consumption modelling consider the following relative Powers:

| Power State | Relative Power (Idle/inactive) |
| --- | --- |
| Deep Sleep (PDS) | 1 |
| Light Sleep (PLS) | 20 |
| Micro sleep (PMS) | 45 |
| PDCCH-only (PPDCCH) | 50 |
| PDCCH+PDSCH | 120 |
| SSB serv. cell | 50 |
| Long PUSCH Tx | 250 (0 dBm), 700 (23 dBm) |
| SRS Tx | 0.3*UL Power |
| PRACH | 175 |
| Power scaling to 20MHz reception BW according to the rule in Section 8.1.3 of TR 38.840: max{reference power * 0.4, 50}. |  |

Proposal 4: For power consumption modelling for RRC inactive positioning, consider the following relative powers with regards to DRX and other main systems-related parameters:

- I-DRX cycle to evaluate: 1.28 sec, 10.248 sec

  - 8 msec on-duration timer, 100 msec inactivity timer

- SCS and Bandwidth:

  - FR1: 30 KHz SCS, 100 MHz PRS/SRS

  - FR2: 120 KHz SCS, 400MHz PRS/SRS

- PRS/SRS Periodicities to be evaluated: 160 msec, 1.28 sec, 10.248 secs

- No data traffic is assumed in the power cycle computation

### DL-PRS processing Average Power Consumption per slot (Pfr)

| Scenario | Value (Pfr) |
| --- | --- |
| Serving cell SSB or CSI-RS processing, 30 KHz SCS, System BW 100 MHz, Single-CC, Table 18 of 38.840 | 100 |
| Intra-frequency RRM measurement, 8 TRPs, 2 SSBs per slot, Synchronous (Table 22 of 38.840) | 150 |
| Intra-frequency RRM measurement, 8 TRPs, 2 SSBs per slot, combined neighbor cell measurements and cell search (Table 24 of 38.840) | 200 |
| PRS Measurement, 8 TRPs, 30 KHz SCS, 6ms PRS length, 100 MHz (R1-2007576) | 200 |
| PRS Measurement, 8 TRPs, 30 KHz SCS, 4ms PRS length, 100 MHz (R1-2007664) | 610 |

Observation 1: Average Power consumption per slot of PRS depends significnaatly on the UE implementation, whether the UE is in a tracking or acquisition mode, number of TRPs to be measured, type of measurement report, etc.

Proposal 5: Suggest evaluating multiple average DL PRS power consumption per slot in the range of [100,200].

## RRC Inactive UE-based DL-only Positioning

With regards to RRC Inactive UE-based Positioning, the main procedures that may need to be included in the evaluation power consumption modelling includes the following:

- Paging occasion

- Serving cell SSB

- PRS processing

Example of such a flow is shown below:

As a first example, for FR1, a typical case of paging occasion may be 2 msec with an average power unit of 57 (*10% Paging rate: 50*0.9+120*0.1 = 57), a serving cell SSB of 2 msec, and PRS instance of X msec, e.g, X = 1-4 msec.

| RRC Idle/Inactive UE-based DL Positioning | Power unit (per slot) |
| --- | --- |
| Paging Occasion ([2] ms) | 57*1 |
| SSB serv. cell ([2] ms) | 50 |
| Light sleep | 20 |
| PRS Instance ([1-4] ms) | PPRS |
| Deep sleep | 1 |
| Deep State transit | 450 |
| Light State transit | 100 |

Proposal 6: For UE-based DL Positioning power consumption modeling, consider the scenario that the UE is expected to process Paging occasion, measure SSB from serving cell, and DL PRS, with the following  aspects:

- Different  PPRS (power consumption per slot for PRS) including at least PPRS=100

- M samples of PRS reception per Positioning fix, with M = {1,4}

- PRS instance as small as one slot

- SSB are only needed to be measured once per paging cycle

## 3.2 RRC Inactive network-based UL-AoA/UL-TDOA

With regards to network-based Positioning, one main item that is different compared to the DL-only Positioning is the need of measuring SSBs before the SRS instance for the purpose of determing power control parameters and SRS power. Furthermore, the power consumption should consider the fact that a long SRS (e.g. 12-symbol SRS) should be associated with a power unit similar to that of PUSCH (power unit of 700), whereas a short SRS (e.g., 1-2 symbols) could be associated with a cost of 210 (0.3*700=210, as suggested in TS 38.840).

| RRC Idle/Inactive network-based Positioning | Power unit |
| --- | --- |
| Paging Occasion ([2] ms) | 57*1 |
| SSB serving cell ([2] ms) | 50 |
| Light sleep | 20 |
| SRS ([1] slot) | 210 (short SRS) 700 (long SRS) |
| Deep sleep | 1 |
| Light Sleep State transit | 100 |
| Deep Sleep State transit | 450 |

Proposal 7: For network-based UL Positioning, consider the scenario that the UE is only expected to transmit SRS, measure SSB from serving cell, process a paging occasion, with the following  aspects:

- Power consumption evaluation for both short SRS and long SRS in slot

- M samples of SRS transmissions per Positioning fix, with M = {1,4}

- Reception of SSBs for the purpose of power control before each SRS instance Transmission

## RRC Inactive UE-assisted Positioning (DL, UL, DL+UL)

With regards to RRC Inactive UE-assisted M-RTT, the 5GC-MT-LR Procedure with SDT is shown in the figure below.

Overall, two SDT procedures are needed to complete a DL+UL positioning flow, where in the first, the SRS Configuration is provided to the UE, and the second is for the purpose of measurement reporting in RRC Inactive. A few main steps to note:

- When event is triggered, UE sends a request for SRS using SDT (Step 3)

- SRS Config is provided in RRC Release (Step 10) which completes the 1st SDT

- After performing DL measurements, UE reports measurements using 2nd SDT procedure (Step 12)

- UL-only positioning would use SDT#1 only

- DL-only positioning would used SDT#2 only

With the above, a procedure that shows the PHY layer channels transmitted and received is shown below:

The corresponding power units per slot are included in the table below:

| RRC Inactive Positioning | Power unit |
| --- | --- |
| SSB serv. cell (2ms) | 50 |
| Light Sleep | 20 |
| PRS | PPRS |
| SRS | 210/700 |
| Coreset0+SIB1 | 120 |
| PRACH (short format) | 175 |
| RAR (PDCCH+PDSCH) | 120 |
| Msg3 (PUSCH) | 700 |
| Msg4 (PDCCH+PDSCH) | 120 |
| RRC release (PDCCH+PDSCH) | 280 |
| Micro sleep | 45 |
| Deep sleep | 1 |
| Deep State transit | 450 |
| Light State transit | 100 |

Proposal 8: For RRC Inactive UE-assisted positioning, consider the scenario with 2 SDT procedures (1st used to configure SRS, 2nd used for measurement reporting), with a 4-step RACH procedure and the following aspects included:

- Different  PPRS (power consumption per slot for PRS) including at least PPRS=100

- M samples of PRS reception / SRS transmission per Positioning fix, with M = {1,4}

- PRS instance as small as one slot

- Power consumption evaluation for both short SRS and long SRS in slot

- Reception of SSBs for the purpose of power control before each SRS instance Transmission

# Potential Enhancements

## 4.1 DL PRS waveform enhancements: Unstaggered DL-PRS resources

A typical way of reducing power consumption is to make sure that a device wakes up as little as possible. All DL-PRS configurations in NR Rel-16/17 are fully staggered, which means that, multiple symbols within a slot need to be occupied. For example, if the deployed PRS is comb-12/12-symbols, 12 TRPs can be FDMed but the UE is expected to be awake for 12 symbols in a slot. If the deploye dPRS is comb-2/2-symbols, in order to transmit the same number of TRPs, again 12 symbols would need to be used. The way to reduce the need of the UE staying awake for long time, unstaggered DL-PRS resources could be specified in NR:

Proposal 9: Study whether and how much unstaggered/partially-staggered DL-PRS could reduce the power consumption of DL PRS processing

- Study methods of mitigating time-domain aliasing due to unstaggered PRS measurements

## 4.2 RRC Idle Positioning

An easy and useful extension with regards to RRC Idle and Inactive Positioning is to extend the UE’s measurements in RRC Idle state. Up to NR Rel-17, all the measurements can happen in RRC connected or RRC Inactive state.

Proposal 10: Support Positioning measurements in RRC Idle state.

## 4.3 DL+UL RRC Inactive Positioning

With regards to DL+UL RRC inactive positioning, we make the following observations:

- The procedure requires 2 SDTs, one for configuring the SRS and one for reporting the measurements.

  - Even though the 2nd SDT appears unavoidable, since the UE has to report the measurements for UE-assted RTT, the 1st SDT procedure may be unnecessary. For example, instead of having to do a MSG3-based request of Location services, RAN1 could work on a MSG1-based request. Furthermore, instead of configuring/triggering the SRS in the RRC Release message, one could envision a paging-type of operation, where a UE only monitors a PDCCH+PDSCH.

- Furthermore, power consumption could be reduced if the UE avoids retuning between PRS and SRS every time it needs to receive and transmit. Requesting PRS and SRS to have the same center frequency may reduce the power consumption of the DL+UL RRC Inactive operation. Another similar constraint could be that PRS and SRS are close by in time so that the UE really wakes up once for both.

- Lastly, even though intra-band RTT has been specified in NR Rel-16/17, having PRS and SRS on different bands is another direction, that depending on the scenario, may provide power saving gains. To be more precise, if we are in a scenario that DL-PRS & UL-SRS in FR2 is being used, but the SRS is power/coverage limited, if a device is calibrated across bands, it could enable transmitting the SRS on an FR1 band so that it can fight against the increased pathloss of FR2.

Proposal 11: Study at least the following enhancements in RRC Inactive Positioning for the purpose of reducing power consumption:

- Study PRS/SRS configuration restrictions & corresponding new UE capabilities for enabling reduced power consumption for RTT positioning.

- Study ways of avoiding configuring an SDT procedure for the purpose of SRS configuration/triggering (e.g. RACH-based SRS request from the UE, paging-based SRS activation).

- Study ways for SRS transmission continuation after cell change in RRC Inactive (e.g., continuity of the configured SRS across cell change).

# Conclusions

With regards to the topic of LPHAP positioning we make the following observations and proposals:

Proposal 1: Keep use case 6 (and the corresponding requirements) as the representative use case to be evaluated.

Proposal 2: Support agreeing on a common power consumption methodology, using TR38.840 as a starting point, for evaluating at least the following methods:

- RRC Inactive UE-based Positioning

  - DL-TDOA/DL-AoD with DL-PRS reception at the UE

- RRC Inactive network-based Positioning

  - UL-AoA/UL-TDOA with SRS for Positioning transmission by the UE

- RRC Inactive UE-assisted Positioning

  - M-RTT with SRS for Positioning transmission and DL-PRS reception.

Proposal 3: For RRC Inactive Positioning and power consumption modelling consider the following relative Powers:

| Power State | Relative Power (Idle/inactive) |
| --- | --- |
| Deep Sleep (PDS) | 1 |
| Light Sleep (PLS) | 20 |
| Micro sleep (PMS) | 45 |
| PDCCH-only (PPDCCH) | 50 |
| PDCCH+PDSCH | 120 |
| SSB serv. cell | 50 |
| Long PUSCH Tx | 250 (0 dBm), 700 (23 dBm) |
| SRS Tx | 0.3*UL Power |
| PRACH | 175 |
| Power scaling to 20MHz reception BW according to the rule in Section 8.1.3 of TR 38.840: max{reference power * 0.4, 50}. |  |

Proposal 4: For power consumption modelling for RRC inactive positioning, consider the following relative powers with regards to DRX and other main systems-related parameters:

- I-DRX cycle to evaluate: 1.28 sec, 10.248 sec

  - 8 msec on-duration timer, 100 msec inactivity timer

- SCS and Bandwidth:

  - FR1: 30 KHz SCS, 100 MHz PRS/SRS

  - FR2: 120 KHz SCS, 400MHz PRS/SRS

- PRS/SRS Periodicities to be evaluated: 160 msec, 1.28 sec, 10.248 secs

- No data traffic is assumed in the power cycle computation

Observation 1: Average Power consumption per slot of PRS depends significnaatly on the UE implementation, whether the UE is in a tracking or acquisition mode, number of TRPs to be measured, type of measurement report, etc.

Proposal 5: Suggest evaluating multiple average DL PRS power consumption per slot in the range of [100,200].

Proposal 6: For UE-based DL Positioning power consumption modeling, consider the scenario that the UE is expected to process Paging occasion, measure SSB from serving cell, and DL PRS, with the following  aspects:

- Different  PPRS (power consumption per slot for PRS) including at least PPRS=100

- M samples of PRS reception per Positioning fix, with M = {1,4}

- PRS instance as small as one slot

- SSB are only needed to be measured once per paging cycle

Proposal 7: For network-based UL Positioning, consider the scenario that the UE is only expected to transmit SRS, measure SSB from serving cell, process a paging occasion, with the following  aspects:

- Power consumption evaluation for both short SRS and long SRS in slot

- M samples of SRS transmissions per Positioning fix, with M = {1,4}

- Reception of SSBs for the purpose of power control before each SRS instance Transmission

Proposal 8: For RRC Inactive UE-assisted positioning, consider the scenario with 2 SDT procedures (1st used to configure SRS, 2nd used for measurement reporting), with a 4-step RACH procedure and the following aspects included:

- Different  PPRS (power consumption per slot for PRS) including at least PPRS=100

- M samples of PRS reception / SRS transmission per Positioning fix, with M = {1,4}

- PRS instance as small as one slot

- Power consumption evaluation for both short SRS and long SRS in slot

- Reception of SSBs for the purpose of power control before each SRS instance Transmission

Proposal 9: Study whether and how much unstaggered/partially-staggered DL-PRS could reduce the power consumption of DL PRS processing

- Study methods of mitigating time-domain aliasing due to unstaggered PRS measurements

Proposal 10: Support Positioning measurements in RRC Idle state.

Proposal 11: Study at least the following enhancements in RRC Inactive Positioning for the purpose of reducing power consumption:

- Study PRS/SRS configuration restrictions & corresponding new UE capabilities for enabling reduced power consumption for RTT positioning.

- Study ways of avoiding configuring an SDT procedure for the purpose of SRS configuration/triggering (e.g. RACH-based SRS request from the UE, paging-based SRS activation).

- Study ways for SRS transmission continuation after cell change in RRC Inactive (e.g., continuity of the configured SRS across cell change).

# References

[1] RP-213588, “Revised SID on Study on expanded and improved NR positioning ”, RAN#94e, December 6-17, 2021.