3GPP TSG RAN WG1 #109-e			R1-2205042

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.5.3

Source: 	Qualcomm Incorporated

Title: 	Positioning for Reduced Capabilities UEs

Document for:		Discussion and Decision

# Introduction

At RAN #94 a new SID has been approvedoth for expanded and improved NR positioning which includes the following goal with regards to the Reduced Capability UEs:

… - Positioning support for RedCap UEs, considering the following: - Evaluate positioning performance of existing positioning procedures and measurements with RedCap UEs [RAN1] - Based on the evaluation, assess the necessity of enhancements and, if needed, identify enhancements to help address limitations associated with for RedCap UEs [RAN1, RAN2]

NR Release-17 has specified support for RedCap UEs with reduced bandwidth support and reduced complexity including reduced number of receive chains. Such UEs could support NR positioning functionality but there is a gap in that the core and performance requirements have not been specified for the positioning related measurements performed by RedCap UEs, and no evaluation was performed to see how the reduced capabilities of RedCap UEs might impact position accuracy.

In this paper, we present our views on positioning performance of existing procedures and measurements with RedCap UEs and potential enhancements.

# Redcap device comparison with NR eMBB devices

Based on the study for UE complexity reduction, NR RedCap devices can achieve cost reduction up to 70% in FR1. The main enablers for cost reduction include BW reduction, RX branch reduction, DL modulation order relaxation and Half Duplex FDD (HD-FDD). A summary of UE capability comparison between eMBB UE (assuming single-carrier operation for the ease of comparison) and RedCap UE is shown in the following table:

| UE Capabilities (FR2) | NR R17 RedCap UE | NR R15/16 eMBB UE |
| --- | --- | --- |
| Max UE BW (MHz) | 100 | 400 |
| Duplex Mode | Same (TDD) |  |
| Carrier Aggregation | Do not support | Support |
| Dual Connectivity | Do not support | Support |
| Maximum Number of RX Branches | 1 | 2 |
| DL Peak Data Rate for Single Carrier (Mbps) | Up to 845 Mbps (TDD 3:1, Rank 2, 256 QAM) | Up to 3.4 Gbps (TDD 3:1, Rank 2, 256 QAM) |
| UL Peak Data Rate for Single Carrier (Mbps) | Up to 115 Mbps (TDD 3:1, Rank 1, 64 QAM) | Up to 465 Mbps (TDD 3:1, Rank 1, 64 QAM) |
| Max Modulation Order | Same |  |
| Channel Coding - control | Same |  |
| Channel Coding – data | Same |  |
| MCS/CQI Tables | Same |  |
| Power Class | Same |  |
| Max Number of HARQ Processes | Same |  |
| UE Processing Capability/Timeline for N1/N2/Z/Z’ | Same |  |

In the next Section we are going to discuss the NR Rel-16/17 positioning features and how these are compatible with the Redcap UE capabilities.

# NR Positioning Support Status – Redcap UEs

Although NR Rel-16/17 did not include specific enhancements for RedCap UEs, in this section we investigate whether the 3GPP standards have already covered the limitations related to UE capabilities.

- Limited PRS processing bandwidth

  - A UE may report a maximum PRS BW as small as 5 MHz in FR1. Therefore, the already specified capabilities are enough to cover the limited PRS processing bandwidth of Redcap devices.

Maximum DL PRS bandwidth in MHz, which is supported and reported by UE. - a)FR1 bands: {5, 10, 20, 40, 50, 80, 100} - b) FR2 bands: {50, 100, 200, 400} - Per Band capability reporting - Notes: UE is not expected to support DL PRS bandwidth that exceeds the reported DL PRS bandwidth value

- Limited PRS processing capabilities

  - A UE may already report very limited UE capabilities as shown below. For example, it can support as small as processing of a single PRS resource in each slot, and only process a few msec of PRS every hundreds of msec.

- Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. - T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms - N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms - Max number of DL PRS resources that UE can process in a slot under it - FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz - FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz - Notes: - UE reports one combination of (N, T) values per band, where N is a duration of DL PRS symbols in ms processed every T ms for a given maximum bandwidth (B) in MHz supported by UE - UE DL PRS processing capability is defined for a single positioning frequency layer. - UE DL PRS processing capability is agnostic to DL PRS comb factor configuration - The reporting of (N, T) values for maximum BW in MHz is not dependent on SCS

- Limited PRS resource configuration capabilities. A UE can already provide UE capabilities for the following max number of positioning frequency layers, PRS resource sets, resources and TRPs:

  - Max number of positioning frequency layers across all supported methods and all bands: {1, 2, 3, 4}

  - Max number of PRS resource sets per TRP per frequency layer: {1, 2}

  - Max number of positioning frequency layers supported per UE: {1, 2, 3, 4}

  - Max number of TRPs across all positioning frequency layers per UE: down to 4

- Limited capabilities to support simultaneous processing of different positioning methods

  - A UE can declare to the Location Server the following capabilities with regards to simultaneous processing of different methods within NR or across NR/LTE:

| Scenario | UE Capability |
| --- | --- |
| DL-AOD & DL-TDOA | Support of simultaneous processing for DL AoD and Multi-RTT measurements - Per Band capability - If it is not indicated, a UE is not expected to perform simultaneously the processing for deriving DL AoD and M-RTT measurements |
| DL-AoD & M-RTT | Support of simultaneous processing for DL AoD and DL-TDOA measurements - Per Band capability - If it is not indicated, a UE is not expected to perform simultaneously the processing for deriving DL AoD and DL TDoA measurements |
| NR PRS & LTE PRS | Support of parallel processing of LTE PRS and NR PRS - Per UE capability |

- Reduced number of RX branches

  - 2 RX antennas are assumed in the NR RAN4 performance. Requirements are derived using simulation assumptions where companies provide results (R4-2113156). 2 Rx is assumed at the UEs.

  - However, 1 Rx performance & requirements are not specified.

- PRS Resource Sharing by Different UE Types

  - Since the wideband DL PRS sequence is configured with respect to point A by a RRC parameter dl-PRS-PointA, the same PRS resource can be shared by RedCap UE and non-RedCap UE, as illustrated in the next Figure.

PRS Resource Sharing by Different UE Types

Observation 1: NR Rel-16/17 Work Items did not specifically include positioning enhancements associated to Redcap devices, but several of the features have already been defined for the purpose of being applicable to UEs with:

- Limited PRS Processing Bandwidth

- Limited PRS processing capabilities

- Limited PRS resource configuration capabilities

- Limited Simultaneous Positioning method capabilities

Observation 2: Performance requirements for the case of single Rx antennas have not been included in NR Rel-16/17 RAN4 specification.

Proposal 1: Send LS to RAN4 to ask them to include positioning requirements derived using simulation assumptions wherein 1 Rx is assumed at the UE.

# Evaluation Assumptions -  Redcap UEs

For TDD scenarios, we propose to reuse NR Rel-16/17 Evaluation assumptions for InH, and UMI FR1 scenarios. Consider adding scenarios wherein DeltaTau modeling similar to the InF case is being simulated.

Proposal 2: For TDD scenarios,

- We propose to reuse NR Rel-16/17 Evaluation assumptions for InH, and UMI FR1 scenarios as agreed in RAN1 #94-Bis for 4 GHz and 2 GHz.

  - Support adding optionally to evaluate the UMI FR1 cases with DeltaTau modeling similar to the one introduced for InF scenarios.

- For FR1 FDD scenario introduce a new Urban Macro at 700 MHz with 500m ISD according to the simulation assumptions shown below.

- For FR2 TDD Redcap, reuse the NR Rel-16/17 FR2 simulation assumptions with 100 MHz PRS bandwidth.

For FR1 FDD scenarios, consider the following additional scenario:

- Urban Macro at 700 MHz with 500m ISD

| Parameters | UMa scenario at 700 MHz with 500m ISD |
| --- | --- |
| System parameters |  |
| Carrier Frequency, GHz | 700 MHz |
| Bandwidth, MHz | 20 MHz |
| Subcarrier spacing, kHz | 15 KHz |
| gNB model parameters |  |
| gNB Noise Figure, dB | 5dB |
| Total gNB TX Power, dBm | 49 dBm |
| gNB Antenna Configuration | (M, N, P, Mg, Ng) = (8, 2, 2, 1, 1), (dH, dV) = (0.5, 0.8)λ, [TR 38.802], 4 TXRUs or (M, N, P, Mg, Ng) = (8, 4, 2, 1, 1), (dH, dV) = (0.5, 0.8)λ, [TR 38.802], 8 TXRUs |
| gNB Antenna Radiation Pattern | Directional, 8dBi [TR 38.802] |
| UE model parameters |  |
| UE Max. TX Power, dBm | 23dBm |
| UE Noise Figure, dB | 7 dB |
| UE Antenna Configuration | Panel model 1 [TR 38.802], 1 Rx UE: (M, N, P, Mg, Ng) = (1, 1, 1, 1, 1) 2 Rx UE: (M, N, P, Mg, Ng) = (1, 2, 1, 1, 1) |
| Number of UE Tx Chains | 1 |
| UE Antenna Radiation Pattern | Omni, 0dBi |
| Propagation characteristics |  |
| Channel Model | According to 3GPP TR 38.901 (UMa scenario) |
| Penetration Loss | For outdoor UEs: 0 dB, For indoor UEs: 20 dB + 0.5 d2D-in, [TR 38.901] |
| Layout considerations |  |
| Layout | Hexagonal grid, 19 or 7 macro sites, 3 sectors per site, ISD = 500m, [TR 38.802] Wrap-around is applied Note: in case if interference considerations are not properly taken into account for 7 sites companies are encouraged to provide results for 19 sites |
| Number of floors, (floor height) | 8, (3m), [TR 37.857] |
| UE drop procedure | 50% indoor and 50% outdoor uniformly distributed over the horizontal area (separate statistic) |
| UE mobility (for modelling Doppler effects) | Indoor: 3km/h / Outdoor: 60km/h |
| UE antenna height | 1.5 m [TR 37.857] |
| Min. gNB - UE distance (2D), m | 35m [TR 37.857] |
| gNB antenna height | Uniformly distributed [20-50] m |
| PHY/Link Level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |
| Network Synchronization assumption | Network synchronization error. Network sync error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an gNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, the range of timing errors is [-T2, T2], T2 = 2*T1. T1 = 0 (perfectly synchronized) and T1 =50 ns are evaluated |

# Initial Simulation Results

We now provide initial evaluation results, according to the NR Rel-16/17 scenarios, for Redcap devices.

# 1 Rx vs. 2 Rx for 4 GHz UMI

Using the NR Rel-16/17 Evaluation assumptions for 4 GHz UMI scenario, we provide the following comparison for DL-TDOA with 1 Rx and 2 Rx UEs.

Figure 1: Positioning Accuracy of RedCap UE (TDD Evaluation Assumptions) for 1 and 2 Rx for UMI 20 MHz 4 GHz

Observation 3: At the 80% percentile, for DL-TDOA, without network synchronization error, in 4 GHz UMI TDD, without Deltautau modelling, ,and 20 MHz UEs, the horizontal accuracy degrades from 11.3 meters to 12.2 meters due to reducing the number of Rx antennas from 2 to 1.

# 1 Rx vs. 2 Rx for 4 GHz InH

Using the NR Rel-16/17 Evaluation assumptions for 4 GHz InH scenario, we provide the following comparison for DL-TDOA with 1 Rx and 2 Rx UEs.

Figure 2: Positioning Accuracy of RedCap UE (TDD Evaluation Assumptions) for 1 and 2 Rx for InH 20 MHz 4 GHz

Observation 3: At the 80% percentile, for DL-TDOA, in 4 GHz InH TDD, without network synchronization error, without Deltautau modelling, and 20 MHz UEs, the horizontal accuracy degrades from 6.7 meters to 7.2 meters due to reducing the number of Rx antennas from 2 to 1.

# 1 Rx vs. 2 Rx for InF-SH 28 GHz

Using the NR Rel-17 Evaluation assumptions for FR2 28GHz InF-SH scenario, we provide the following comparison for DL-TDOA with 1 Rx and 2 Rx UEs. Based on the previous assumption, each UE has two Rx chains. The following simulation result contain four cases.

Case1: Target UE uses tw Rx chains for PRS reception.

Case2: Target UE uses Rx chain 1 only for all PRS reception.

Case3: Target UE uses Rx chain 2 only for all PRS reception.

Case4: Target UE randomly select one out of 2 Rx chains for PRS reception in one realization.

As can be seen form the simulation results, 2 Rx chains are better than other cases. Among case 2-4, case 2 is better than case 3&4. This performance difference can be explained by the fact UE and gNB orientation assumption may result that UE’s and gNB’s polarization are better aligned for Rx chain 1. Case 4 is more realistic in the sense that different UEs may have different orientations.

Figure 3: Positioning Accuracy of RedCap UE (TDD Evaluation Assumptions) for 1 and 2 Rx for InF-SH 100 MHz 28 GHz

Observation 4: At the 80% percentile, for DL-TDOA, without network synchronization error, in 28 GHz InF-SH TDD, and 100 MHz UEs, the horizontal accuracy degrades from 0.07 meters to 0.20 meters (case 4) due to reducing the number of Rx antennas from 2 to 1.

# Simulation Results for Redcap UEs

The following table compares the positioning accuracy achievable by a RedCap UE using M-RTT, DL-TDoA and DL-AoD estimation.

| Scenario | Horizontal Error (m) |
| --- | --- |
| UMI, 1 Rx, 20 MHz, RTT, No DeltaTau modeling | 9.5 |
| UMI, 2 Rx, 20 MHz, DL-TDOA, No DeltaTau modeling, No sync error | 11.3 |
| UMI, 1 Rx, 20 MHz, DL-TDOA, No DeltaTau modeling, No sync error | 12.2 |
| InH, 1 Rx, 20 MHz, DL-TDOA, No DeltaTau modeling, No sync error | 7.2 |
| InH, 2 Rx, 20 MHz, DL-TDOA, No DeltaTau modeling, No sync error | 6.7 |
| InH, 1 Rx, 20 MHz, DL-AoD, No DeltaTau modeling | 2.5 |
| UMI, 2 Rx, 100 MHz, RTT, No DeltaTau modeling | 2.3 |
| InH, 2 Rx, 100 MHz, RTT, No DeltaTau modeling | 1.3 |

Positioning Accuracy Comparison of RedCap UE and eMBB UE (TDD FR1 4 GHz Assumptions)

Observation 5: In the 3GPP UMI 4 GHz Scenario, without DeltaTau modeling, Horizontal accuracy <10m is achievable with 20 MHz, 1 Rx with M-RTT and outlier rejection positioning engine for outdoor UEs.

- Reducing the number of antennas from 2 Rx to 1 Rx, has smaller impact in the performance compared to reducing the bandwidth

- M-RTT seems to be demonstrating better performance compared to DL-TDOA on the same scenario.

Observation 6: Reducing from (100 MHz, 2 Rx) to (20 MHz, 1 Rx), results in a horizontal performance degradation from the [1-3]m range to the [7-12]m range, with the reduction in Bandwidth playing the main role in this performance reduction

| Scenario | Horizontal Error (m) |
| --- | --- |
| InF-SH, 2 Rxs, 100 MHz, DL-TDOA, No sync error | 0.07 |
| InF-SH, 1 Rx, 100 MHz, DL-TDOA, No sync error | 0.20 |
| InF-SH, 2 Rxs, 400 MHz, DL-TDOA, No sync error | <0.01 |
| UMi, 2 Rxs, 100 MHz, DL-TDOA, No sync error | 0.26 |
| UMi, 1 Rx, 100 MHz, DL-TDOA, No sync error | 0.82 |
| UMi, 2 Rxs, 400 MHz, DL-TDOA, No sync error | 0.02 |

Positioning Accuracy Comparison of RedCap UE and eMBB UE (TDD FR2 28 GHz Assumptions)

Observation 7: Reducing from (400 MHz, 2 Rx) to (100 MHz, 1 Rx), results in a horizontal performance degradation from the 0.0x m range to the 0.x0 m range, with the reduction in Bandwidth playing the main role in this performance reduction.

Proposal 3: Support Enhancements for Redcap devices, in both FR1 and FR2, to enable reaching a horizontal accuracy performance close to the NR Rel-17 positioning requirements at least in a subset of evaluation scenarios.

# Potential Enhancements

## 6.1 Receiver DL PRS Frequency Hopping

In scenarios of small bandwidth, a typical solution to solve the problem of reduced timing resolution is to look into ways of performing frequency hopping and enable coherent processing across the hops.

In LTE FeMTC Positioning, frequency hopping is supported for 1.4 MHz PRS BW, however no coherent processing is assumed, and therefore the gains of frequency hopping are relatively limited (mainly due to increased diversity). Specifically, PRS starts from the center of system BW and hops every PRS occasion. Up to 16 hops are supported, wherein a UE can hop 2 or 4 out of the 16 hops.

prsHoppingInfo-r14 CHOICE { nb2-r14 INTEGER (0.. maxAvailNarrowBands-Minus1-r14), nb4-r14 SEQUENCE (SIZE (3)) OF INTEGER (0.. maxAvailNarrowBands-Minus1-r14) } OPTIONAL -- Cond PRS-FH prsHoppingInfo This field specifies the PRS frequency hopping configuration (TS 36.211 [16]). The choice nb2 indicates hopping between 2 narrowbands; the choice nb4 indicates hopping between 4 narrowbands. The first PRS positioning occasion of the first PRS occasion group that starts after the beginning of SFN=0 of the assistance data reference cell is located at the centre of the system bandwidth. The frequency band of each subsequent PRS occasion is indicated by nb2 or nb4, respectively, which defines the narrowband index as specified in TS 36.211 [16]. If this field is absent, no PRS frequency hopping is used.

Enabling receiver’s PRS frequency hopping would allow sharing the legacy PRS across eMBB and Redcap devices. Specifically, assume a TRP transmits a PRS with 100 MHz bandwidth, and it is being shared between an eMBB and a Redcap device. If such PRS already has some level of repetitions, either inter-slot (e.g., each PRS resource is configured to be transmitted over multiple slots within a period), or intra-slot repetition (e.g. comb-2/12-symbols has 6 repetitions of the basic comb-2/2-symbol pattern), then a Redcap device could perform fast RF Rx switching and perform positioning measurements with increased time-domain resolution.

In other words, the already specified inter-slot repetition of a single PRS resource, or intra-slot repetition of a basic comb-based pattern could be reused for frequency hopping and fast switching amongst the hops. Note that current purposes of repetition of a DL PRS in NR Rel-16/17 are the following: Rx beam sweeping across repetitions, Combining gains for coverage extension, Intra-instance Muting.

Observation 8: Enabling receiver’s PRS hopping would allow sharing the legacy PRS across eMBB and Redcap devices.

With regards to potential specification impact of such PRS receive hopping, even though the PRS waveform does not change, at least the the measurement period formulation, additional UE capabilities, achieved accuracy and performance requirements would have to be revisited.

Proposal 4: Study inter & intra-slot repetition of a DL PRS resource for the purpose of enabling receive PRS hopping for both FR1 and FR2.

## 6.2 Transmit SRS Frequency Hopping

Similarly to the scenario of PRS frequency hopping, using SRS frequency hopping for SRS for Positioning could increase the bandwidth, if coherent transmission and reception is possible, or if partially . We suggest to also study hopping for SRS for Positioning.

Proposal 5: Study inter & intra-slot repetition of a SRS resource for positioning for the purpose of enabling transmit SRS hopping for both FR1 and FR2.

## 6.3 Phase Offset Compensation in Frequency Hopping scenarios

h(f2,t1) is related to h(f2,t2) through a phase offset and phase slope

- Phase slope is a linear function of timestamps

- Phase discontinuity could depend on many RF-related factors

hf2+f,t2=hf2+f,t1⋅e-j⋅f⋅(ϵ2-ϵ1)⋅ejθ

One way to allow for phase offset estimation is by using overlapping tones across the hops, assuming the hops are close-by in time and the device is not in a high mobility scenario. For example, Let L overlapping tones be h11 h12…h1L and h21 h22…h2L. The device could compute gk=minh1k,hnkei(angleh1k-anglehnk) and estimate θ=anglekg(k).

In the figures below, we show the performance of frequency hopping and coherently stitching between the hops for a TDD scenario with 30 KHz and 24 PRB PRS processing in each hop where, an unknown and uncompensated phase offset (θ) exists between the hops randomly chosen between the hops.

Figure 4: Positioning Accuracy of RedCap UE (TDD Evaluation Assumptions) for UMI with 2 Hops (30 KHz SCS) and phase offset chosen uniformly in [-θ,θ] with (θ = {0,8,12,15,20,30} degrees) amongst the 2 hops

Figure 5: Positioning Accuracy of RedCap UE (TDD Evaluation Assumptions) for UMI with 2 Hops (30 KHz SCS) and phase offset chosen uniformly in [-θ,θ] with (θ = {0,8,12,15,20,30} degrees) amongst the 4 hops

Figure 6: Positioning Accuracy of RedCap UE (TDD Evaluation Assumptions) for UMI with 2 Hops (30 KHz SCS) and phase offset chosen uniformly in [-θ,θ] with (θ = {0,8,12,15,20,30} degrees) amongst the 8 hops

With regards to FR2 Redcap devices, we first show the impact of phase shift in PRS band stitching. We assume that phase difference between two consecutive hops θi+1  -θi  follows a uniform distribution between [-θ,θ]  and 4 PRS hops, 100 MHz each, are processed coherently at the receiver. As can be seen from the following Figure, PRS frequency hopping could improve the accuracy until the phase difference is larger than 16 deg with 2 Rx chains, or 32 deg with 1 Rx chain in the InF-SH channel.  In the UMi channel, PRS frequency hoppings is better than the baseline case.

Figure 7: Positioning Accuracy of RedCap UE (TDD Evaluation Assumptions) with 2 Rx chains for InF-SH FR2 with 4 Hops (120 KHz SCS) and phase offset chosen uniformly in [-θ,θ] with (θ = {0, 1, 2, 4, 8, 16,32} degrees) between two consecutive hops

Figure 8: Positioning Accuracy of RedCap UE (TDD Evaluation Assumptions) with 1 Rx chain for InF-SH FR2 with 4 Hops (120 KHz SCS) and phase offset chosen uniformly in [-θ,θ] with (θ = {0, 1, 2, 4, 8, 16,32} degrees) between two consecutive hops

Figure 9: Positioning Accuracy of RedCap UE (TDD Evaluation Assumptions) with 1 Rx chain for FR2 Umi DL-TDoA with 4 Hops (120 KHz SCS) and phase offset chosen uniformly in [-θ,θ] with (θ = {0, 1, 2, 4, 8, 16,32} degrees) between two consecutive hops

Figure 10: Positioning Accuracy of RedCap UE (TDD Evaluation Assumptions) with 1 Rx chain for FR2 UMi UL-TDoA with 4 Hops (120 KHz SCS) and phase offset chosen uniformly in [-θ,θ] with (θ = {0, 1, 2, 4, 8, 16,32} degrees) between two consecutive hops

In Figure. 11, we show the comparison between the PRS frequency hopping without overlapping tones across the hops and the PRS frequency hopping with overlapping tones across the hops, assuming the same resource overhead. We assume 4 PRS hops, 100MHz each, and the overlapping BW is 25 MHz.  Therefore, the total BW is 325 MHz with overlapping tones. With the same phase shift error range, the PRS band stitching with phase offset estimation is inferior to the stitching without phase offset estimation due to the lower total BW. With overlapping tones, receiver can estimate phase offset by using overlapping tones across the hops, thus reduce the phase offset and improve positioning accuracy. For example, the positioning accuracy of PRS hopping with overlapping tones (θ=4) is better than the one without overlap (θ=8).

Figure 11: Positioning Accuracy of RedCap UE (TDD Evaluation Assumptions) with 1 Rx chain for InF-SH FR2 with 4 Hops (120 KHz SCS) , with and without overlapping tones

Observation 9: The phase discontinuity introduced due to PRS frequency hopping results a in performance degradation which could be mitigated by using  frequency hopping with overlapping tones.

Proposal 6: Study PRS/SRS overlapping configuration for the purpose of enabling phase estimation across PRS hops both FR1 and FR2.

## 6.5 Transmitter DL PRS Frequency hopping

In the receiver PRS hopping, enabling receiver’s PRS hopping would allow PRS sharing the legacy PRS across eMBB and Redcap devices. For Redcap only system, legacy DL-PRS with repetitons cannot efficiently utilize all RF resources since receiver can measure a subset of RBs within each repetition. Therefore, we propose that transmitter could also transmit PRS with a frequency hopping pattern to save the system resource. The hopping pattern should be known to both transmitter and receiver.

One additional advantage of transmitter PRS hopping is that it may increase EPRE of combined PRS. Assume a transmitter applies the same Tx power in each transmission occasion, the total Tx power of N hopping is N times compared with single transmission. In other word, the Tx power of transmitter PRS hopping with N hops can be N times the Tx power of single shot PRS transmission, assume the total bandwidths are the same.  This could benefit more for the large area positioning and UL where the link budget is limited.

In Figure. 12&13 we show the positioning accuracy with increased Tx power. Similar to the previous simulation assumption, 4 PRS hops, 100 MHz each, are processed coherently at the receiver. We increase the Tx power by 6dB to approximate the Tx power gain. As can be seen from both figures, the accuracy can be improved with Tx boost, especially with the UMi channel.

Figure 12: Positioning Accuracy of RedCap UE (TDD Evaluation Assumptions) for FR2 InF-SH DL-TDOA with 4 Hops (120 KHz SCS) , with and without Tx boost.

Figure 13: Positioning Accuracy of RedCap UE (TDD Evaluation Assumptions) for FR2 UMI DL-TDOA with 4 Hops (120 KHz SCS) and phase offset chosen uniformly in [-θ,θ], with and without Tx boost

Observation 10: Enabling transmitter’s PRS hopping could improve the Tx power, and further improve the positioning accuracy.

Proposal 7: Study DL-PRS/ SRS resource configuration for the purpose of enabling transmitter PRS hopping.

## 6.4 Phase-Difference DL-AoD

The following agreement was made during NR Rel-17 WI:

Agreement: - For both UE-based and UE-assisted DL-AOD study the following enhancements that enable the UE to measure and report (for UE-assisted) information related to the first arriving path - Option 1: Information corresponds to PRS-RSRP of the first arriving path - Option 2: Information corresponds to the angle of departure of the first arriving path - Option 3: Information corresponds to the arrival time of the first path - Option 4: Information corresponds to phase of the CIR corresponding to the first arriving path - Option 5: Information corresponds to received signal value (amplitude and phase of the channel estimated from the first path which can be achieved as a combination of option 1 and option 4) of the first arriving path - FFS: Reporting of additional path to the first arriving path. - FFS: Measurement definition details - FFS: additional assistance data to support these enhancements - FFS: how the “first path” is selected among PRS resources in a PRS resource set - Note 1: Supporting multiple options as well as none of the options above is not precluded.

A phase-difference based DL-AoD would correspond to the following method: The transmitting device sends multiple PRS resources, each PRS resource via each of the physical antennas. As each PRS from the antennas in the array arrives at the receiver’s single antenna, it is phase shifted from the previous PRS due to the different distance it has traveled from the transmitter as shown graphically in the figure below. In the simple scenario shown in the figure below, one can estimate the angle of departure by measuring the phase difference between the PRS resources using a simple formula.

In such a method, there is no need to know what are the beam responses of the PRS resources. The receiver is only required to know the mapping of the PRS resources into the physical antennas, along with the antenna (e.g., ULA, UPA, single or multi-panel) configuration of the TRP and the relative distance of the antennas (the dH and dV parameters, as usually referred to in the 38.901 specification). It deserves to be noted that such a method is being employed already by competing technologies (e.g., Bluetooth ).

Observation 11: Phase-Difference-based AoD may be a useful additional method for scenarios with unprecoded tranmissions from gNB side (e.g. FDD cases).

Proposal 8: For the purpose of Redcap positioning enhancements, study supporting Phase-Difference AoD.

## 6.5 SSB, TRS & SRS-MIMO for Positioning Measurements

Redcap devices may not have strict positioning requirements, and in many scenarios, may correspond to IoT devices with very reduced features and the need of only crude positioning. In this case, these devices might not implement the additional features of DL-PRS and SRS for Positioning, and instead, prefer to support a simplified version wherein any positioning measurement is derived using SSBs, TRS, or SRS for MIMO. For this purpose, we think that it is useful to study whether such enhancements are useful to be included in NR Rel-18:

Proposal 9: For the purpose of Redcap positioning enhancements, study supporting Positioning measurements (RSTD, UE Rx-Tx, RSRPP) derived on SSB, TRS.

Proposal 10: For the purpose of Redcap positioning enhancements, study supporting M-RTT using SRS-MIMO.

# Conclusions

Overall, we make the following observations and proposals:

Observation 1: NR Rel-16/17 Work Items did not specifically include positioning enhancements associated to Redcap devices, but several of the features have already been defined for the purpose of being applicable to UEs with:

- Limited PRS Processing Bandwidth

- Limited PRS processing capabilities

- Limited PRS resource configuration capabilities

- Limited Simultaneous Positioning method capabilities

Observation 2: Performance requirements for the case of single Rx antennas have not been included in NR Rel-16/17 RAN4 specification.

Proposal 1: Send LS to RAN4 to ask them to include positioning requirements derived using simulation assumptions wherein 1 Rx is assumed at the UE.

Proposal 2: For TDD scenarios,

- We propose to reuse NR Rel-16/17 Evaluation assumptions for InH, and UMI FR1 scenarios as agreed in RAN1 #94-Bis for 4 GHz and 2 GHz.

  - Support adding optionally to evaluate the UMI FR1 cases with DeltaTau modeling similar to the one introduced for InF scenarios.

- For FR1 FDD scenario introduce a new Urban Macro at 700 MHz with 500m ISD according to the simulation assumptions shown below.

- For FR2 TDD Redcap, reuse the NR Rel-16/17 FR2 simulation assumptions with 100 MHz PRS bandwidth.

Observation 3: At the 80% percentile, for DL-TDOA, without network synchronization error, in 4 GHz UMI TDD, without Deltautau modelling, ,and 20 MHz UEs, the horizontal accuracy degrades from 11.3 meters to 12.2 meters due to reducing the number of Rx antennas from 2 to 1.

Observation 4: At the 80% percentile, for DL-TDOA, without network synchronization error, in 28 GHz InF-SH TDD, and 100 MHz UEs, the horizontal accuracy degrades from 0.07 meters to 0.20 meters (case 4) due to reducing the number of Rx antennas from 2 to 1.

Observation 5: In the 3GPP UMI 4 GHz Scenario, without DeltaTau modeling, Horizontal accuracy <10m is achievable with 20 MHz, 1 Rx with M-RTT and outlier rejection positioning engine for outdoor UEs.

- Reducing the number of antennas from 2 Rx to 1 Rx, has smaller impact in the performance compared to reducing the bandwidth

- M-RTT seems to be demonstrating better performance compared to DL-TDOA on the same scenario.

Observation 6: Reducing from (100 MHz, 2 Rx) to (20 MHz, 1 Rx), results in a horizontal performance degradation from the [1-3]m range to the [7-12]m range, with the reduction in Bandwidth playing the main role in this performance reduction

Observation 7: Reducing from (400 MHz, 2 Rx) to (100 MHz, 1 Rx), results in a horizontal performance degradation from the 0.0x m range to the 0.x0 m range, with the reduction in Bandwidth playing the main role in this performance reduction.

Proposal 3: Support Enhancements for Redcap devices, in both FR1 and FR2, to enable reaching a horizontal accuracy performance close to the NR Rel-17 positioning requirements at least in a subset of evaluation scenarios.

Observation 8: Enabling receiver’s PRS hopping would allow sharing the legacy PRS across eMBB and Redcap devices.

Proposal 4: Study inter & intra-slot repetition of a DL PRS resource for the purpose of enabling receive PRS hopping for both FR1 and FR2.

Proposal 5: Study inter & intra-slot repetition of a SRS resource for positioning for the purpose of enabling transmit SRS hopping for both FR1 and FR2.

Observation 9: The phase discontinuity introduced due to PRS frequency hopping results a in performance degradation which could be mitigated by using  frequency hopping with overlapping tones.

Proposal 6: Study PRS/SRS overlapping configuration for the purpose of enabling phase estimation across PRS hops both FR1 and FR2.

Observation 10: Enabling transmitter’s PRS hopping could improve the Tx power, and further improve the positioning accuracy.

Proposal 7: Study DL-PRS/ SRS resource configuration for the purpose of enabling transmitter PRS hopping.

Observation 11: Phase-Difference-based AoD may be a useful additional method for scenarios with unprecoded transmissions from gNB side (e.g. FDD cases).

Proposal 8: For the purpose of Redcap positioning enhancements, study supporting Phase-Difference AoD.

Proposal 9: For the purpose of Redcap positioning enhancements, study supporting Positioning measurements (RSTD, UE Rx-Tx, RSRPP) derived on SSB, TRS.

Proposal 10: For the purpose of Redcap positioning enhancements, study supporting M-RTT using SRS-MIMO.

# References

[1] RP-213588, “Revised SID on Study on expanded and improved NR positioning ”, RAN#94e, December 6-17, 2021.