3GPP TSG RAN WG1 Meeting #109-e       	                R1-2205044

E-meeting, May 9th – May 20th, 2022

Source:	Qualcomm Incorporated

Title:	Evaluation for eRedCap SI

Agenda Item:	9.6.2

Document for:	Discussion/Decision

# Introduction

In RAN #94e, a new Rel-18 study item was approved to study the reduction of the complexity of RedCap devices [1]. As described in the study item objective shown below, RAN1 is expected to provide performance evaluation for potential solutions from the perspectives of complexity reduction, network/UE impacts, and coexistence with different device types.

### 4.1 Objective of SI To further reduce the complexity of RedCap devices, the following should be studied, and the results should be captured in TR 38.8xx: - Study further UE complexity reduction techniques based on Rel-17 evaluation methodology in TR 38.875 [RAN1] - Consider network impact, coexistence of Rel-17 and Rel-18 RedCap and non-RedCap UEs in a cell, UE impact, specification impact - Potential solutions, which may complement each other, for reducing device complexity are focusing on: - UE bandwidth reduction to 5MHz in FR1, - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - reduced UE peak data rate in FR1, - Possibly including restricted bandwidth for PDSCH and/or PUSCH - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - Notes: - Rel-15 SSB should be reused and L1 changes minimized. - Operation in BWP with/without SSB and without/with RF retuning should be considered. - It is not precluded that some solutions for FR1 can be applied to FR2 in WI stage. - Aim to define a single Rel-18 RedCap UE type for further UE complexity reduction.

In this contribution, we discuss required performance evaluations for successful and timely completion of the study item. As the study item duration and allocated time units are extremely limited, it is important to focus on simulations which are necessary and critical for the study.

# Discussions on Required Evaluations

This section discusses required simulations or analysis to evaluate the proposed solutions. Basically, there were similar evaluation done for Rel-17 RedCap discussions and many comprehensive evaluation results were summarized in corresponding TR 38.875 [3]. As the allowed time is limited for the study, we need to avoid any duplicated evaluation from TR 38.875. If needed, we can directly capture some results from TR 38.875 without redoing the same evaluation.

Proposal 1: For general evaluation

- Do not duplicate the same evaluation that was already done in Rel-17

- If needed, we can directly capture some results from TR 38.875 without redoing the same evaluation

## 2.1 Complexity analysis

As the main objective of this study is seeking further UE complexity reduction techniques, the complexity analysis has to be provided for each solutions. Potential complexity reduction solutions are listed below and details of each scheme are discussed in the companion contribution [2]

- UE bandwidth reduction to 5MHz

- Reduced UE peak rate

- Relaxed UE processing time

- Reduced PDCCH monitoring

- Half-duplex FDD

As capture in the SID, it is proposed that we reuse the Rel-17 complexity evaluation methodology in [3] as much as possible. The cost breakdown table used for the estimated relative device cost needs to be reused. In Rel-17 RedCap study item, Rel-15 eMBB device was used as the reference device for cost analysis. In Rel-18 eRedCap study, the reference device will be the Rel-17 RedCap device defined as follows.

- Maximum bandwidth: 20 MHz for DL and UL

- Antennas: 1Rx/1Tx

- Power class: PC3

- Processing time: Capability 1

- Modulation: 64QAM for DL/UL

- Access: Direct DL/UL access between UE and gNB

- HD-FDD type A for FDD scenario

Proposal 2: For complexity analysis

- Study the complexity analysis of the following cost reduction techniques

  - UE bandwidth reduction to 5MHz

  - Reduced UE peak rate

  - Relaxed UE processing time

  - Reduced PDCCH monitoring

  - Half-duplex FDD

- Reuse the cost breakdown table as in Rel-17 RedCap TR 38.875

- Use the Rel-17 RedCap device defined as follows as reference devices for the cost comparison

  - Maximum bandwidth: 20 MHz for DL and UL

  - Antennas: 1Rx/1Tx

  - Power class: PC3

  - Processing time: Capability 1

  - Modulation: 64QAM for DL/UL

  - Access: Direct DL/UL access between UE and gNB

  - HD-FDD type A for FDD scenario

## 2.2 Link budget analysis

One of the main discussion points for reducing the device cost is whether the bandwidth is further reduced from 20MHz to 5MHz or we keep the bandwidth as 20MHz. If we study differences in bandwidth size, there would naturally be coverage impacts to existing channels, at least PDSCH/PDUCH/PDCCH. As we did similar studies during Rel-17 RedCap study item, we simply reuse the evaluation methodology in TR38.875 [3]. The channels and messages used in link budget evaluations have to include at least PDCCH, PDSCH, and PUSCH. Assumptions for gNB and UE are provided in Table 1 and Table 2.

Table 1: gNB assumptions for coverage evaluation

| Parameters | values |
| --- | --- |
| Channel model | TDL-C |
| Delay spread | 300ns |
| UE velocity | 3 km/h |
| Antenna correlation | Low |
| # gNB Tx chains | 2 or 4 |
| # gNB Rx chains | 2 or 4 |

Table 2: UE assumptions for coverage evaluation

| Parameters | Reference UE | eRedCap UE |
| --- | --- | --- |
| # UE Tx chains | 1 | 1 |
| # UE Rx chains | 1 or 2 | 1 or 2 (same as reference) |
| UE bandwidth | 20 MHz | Depending on solutions |

Proposal 3: For link budget evaluation

- Study the link budget of at least for PDSCH/PUSCH/PDCCH

- Reuse the evaluation methodology of as in Rel-17 RedCap TR 38.875

- Use the gNB and UE assumptions provided in Table 1 and Table 2

## 2.3 PDCCH blocking rate

Among the UE cost reduction techniques listed in section 2.1, UE bandwidth reduction and reduced PDCCH monitoring brings potential impacts to PDCCH blocking rate. We had comprehensive analysis on reduced PDCCH monitoring for 20MHz RedCap devices during Rel-17 study item. We suggest to simply extend the work to 5MHz BW assumption as given in Table 3. One note is that if we compare 5MHz scenario and 20MHz scenario, the number of users needs to be adjusted considering potential user distributions over multiple BWPs, which is more fair comparison, e.g. 4 times larger number of users for 20MHz BW compared to 5MHz BW.

For the PDCCH AL distributions, we may reuse the configurations of Table 6.2-5 of [3]. However, at least 5MHz BW option, some of the ALs may not be supported. So any modification or different handling on the AL distribution has to be reported by companies.

Table 3: Parameters for the PDCCH blocking rate evaluation

| Parameters | Assumptions |
| --- | --- |
| SCS/BW | 15 kHz/5 MHz, 15KHz/20MHz |
| CORESET duration | 3 symbols |
| DCI size | 40 bits (Not including CRC) |
| Delay toleration (Slot) | 1 (1: implies that PDCCH is blocked if it can't be scheduled in the given slot) |
| Number of users | 4 times larger number is assumed for 20MHz BW compared to 5MHz BW |

Proposal 4: For PDCCH blocking rate

- Use the parameter given in Table 3

- Reuse the PDCCH AL distributions as in Rel-17 RedCap TR 38.875

  - Any modification of AL distributions to be reported by companies (e.g., restriction on some ALs by BW reduction)

## 2.4 System level evaluation

During the Rel-17 RedCap study item, there were studies with system level simulation for the evaluation of how complexity reduction techniques give impacts to network capacity and spectral efficiency. The main motivation of the system level evaluation in Rel-17 time frame was to see the impacts of the reduced number of rx antennas. However, reduced number of rx antennas has been already reduced to one and no further impacts need to be evaluated in Rel-18.

Also, discussions on system-level simulation requires longer time for including discussions on evaluation assumption, deployment scenario, traffic model, evaluation metrics, and so on compared to other evaluation. However, we have limited time unit and limited number of meetings for the study item. To safely meet the timeline for the study item, it is challenging to include the work which requires system level evaluation. It is suggested that RAN1 does not work on system level simulation for this study item.

Proposal 5: For system level evaluation

- Do not include system level evaluation for Rel-18 eRedCap study item

# Conclusion

In this contribution, we discussed required evaluations for eRedCap study item. Based on the discussions above, we have following proposals:

Proposal 1: For general evaluation

- Do not duplicate the same evaluation that was already done in Rel-17

- If needed, we can directly capture some results from TR 38.875 without redoing the same evaluation

Proposal 2: For complexity analysis

- Study the complexity analysis of the following cost reduction techniques

  - UE bandwidth reduction to 5MHz

  - Reduced UE peak rate

  - Relaxed UE processing time

  - Reduced PDCCH monitoring

  - Half-duplex FDD

- Reuse the cost breakdown table as in Rel-17 RedCap TR 38.875

- Use the Rel-17 RedCap device defined as follows as reference devices for the cost comparison

  - Maximum bandwidth: 20 MHz for DL and UL

  - Antennas: 1Rx/1Tx

  - Power class: PC3

  - Processing time: Capability 1

  - Modulation: 64QAM for DL/UL

  - Access: Direct DL/UL access between UE and gNB

  - HD-FDD type A for FDD scenario

Proposal 3: For link budget evaluation

- Study the link budget of at least for PDSCH/PUSCH/PDCCH

- Reuse the evaluation methodology of as in Rel-17 RedCap TR 38.875

- Use the gNB and UE assumptions provided in Table 1 and Table 2

Proposal 4: For PDCCH blocking rate

- Use the parameter given in Table 3

- Reuse the PDCCH AL distributions as in Rel-17 RedCap TR 38.875

  - Any modification of AL distributions to be reported by companies (e.g., restriction on some ALs by BW reduction)

Proposal 5: For system level evaluation

- Do not include system level evaluation for Rel-18 eRedCap study item

# Reference

- RP-213661, “New SID on Study on further NR RedCap UE complexity reduction,” RAN#94e, December, 2021.

- R1-2205043, “Further complexity reduction for eRedCap device,” Qualcomm, RAN1 #109e

- 3GPP TR 38.875, “Study on support of reduced capability NR devices (Release 17)”

- Study on Support of Reduced Capability NR Devices (R17), 3GPP TR 38.875, Dec. 2020.