3GPP TSG RAN WG1 #109-e			R1-2205047

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.8.1

Source: 	Qualcomm Incorporated

Title: 	On side control information for network controlled repeaters (NCR)

Document for:	Discussion/Decision

# 1 Introduction

This contribution provides our views on the following objective from the NR network-controlled repeaters SID [1]:

“Study and identify which side control information below is necessary for network-controlled repeaters including assumption of max transmission power [RAN1]

- Beamforming information

- Timing information to align transmission / reception boundaries of network-controlled repeater

- Information on UL-DL TDD configuration

- ON-OFF information for efficient interference management and improved energy efficiency

- Power control information for efficient interference management (as the 2nd priority)”

Section 3 provides a high-level description of network-controlled repeaters (NCR). Section 4 discusses the benefits of each listed side control information. Appendix provides performance evaluation results.

# 3 Network-controlled Repeaters: Architecture and Functionality

In this section, we provide a high-level model and description of the network-controlled repeaters (NCR).

For repeater to receive side control info from the network, it needs to have a UE functionality (e.g., NCR-UE) to establish an interface to the gNB. Apart from the NCR-UE, the repeater has an amplify-and-forward (A&F) functionality to forward the signals between the network and UEs.

The interface between NCR-UE and the network, which is essentially based on Uu interface, is primarily used for the DL and UL communications related to the control and configuration of the A&F component.

Proposal 1

A network-controlled repeater (NCR) comprises the following two components:

- NCR-UE: a UE functionality that establishes an interface to the network to support DL/UL communication of side control and related information.

- A&F: an amplify-and-forward component that provides an analog pass-through for the UEs’ DL/UL signals.

Figure 1 Network-controlled repeaters (NCR) simple model

There are questions on the detailed architecture of the repeater (NCR-UE) – for example, whether NCR-UE supports the full-stack protocol or only lower-layers. In our view, such discussions are outside the RAN1’s scope. What RAN1 should focus on is identifying the side control, deciding the granularity and how dynamic this information should be provided and identifying the required lower-layer (L1/L2) signaling to carry the side control info. Aspects related to upper-layer, e.g., whether some configurations can be provided by RRC or by implementation (e.g., OAM) should be left to RAN2/3.

Our RAN1 assumption is that the repeater has an NCR-UE that at least supports communicating L1/L2 signals with the network.

Proposal 2

NCR-UE, at least, supports the lower-layer (L1/L2) functionalities.

It is outside RAN1 scope to discuss and decide about the repeater’s architecture, and such discussions should be left to RAN2/3.

# 3 Side Control Information

In what follows, we discuss the benefits of the identified side control information elements to enable the repeater “to perform its amplify-and-forward operation in a more efficient manner” [1]. We also provide some performance evaluation results to support the claimed benefits. More details of the sim assumptions can be found in the Appendix.

## UL-DL TDD configuration

Given the info about UL-DL TDD configuration, the repeater forwards the signals in the right direction (one of DL or UL) in any given symbol. Lack of this information may have different consequences depending on the repeater’s implementation.

Example implementation 1

A repeater may be able to forward the signals only in one direction at any given time – e.g., when the repeater has a single amplification chain and is equipped with switches to select the desired input/output ports.

Without side control info about the UL-DL TDD configuration, such a repeater can effectively operate only within those symbols whose UL/DL direction is known [semi]statically. This may work for some deployments with static TDD, but not in all deployments, where there are “flexible” resources whose DL/UL direction are decided later/dynamically.

Example implementation 2

A repeater may have the capability of forwarding signals in both DL and UL directions simultaneously – e.g., when the repeater has two separate amplification chains with enough separation.

One may argue no TDD information is required for such a repeater. However, we should note

- Such a repeater has a higher cost due to implementation of multiple amplification chains.

- There will be coupling/interference between two adjacent active amplification chains. This may lead to oscillation and instability, and to avoid such issues, the repeater should set its amplification gains properly. This will lead to operating at a gain level lower (e.g., X [dB]) than the maximum level supported by the repeater on each chain independently. In practice, X can be more than 20 dB.

Our studies show TDD-awareness (i.e., side control info about UL-DL TDD configuration) increases the coverage (by ~10%) and improves the end-to-end SINR (by ~4.5 dB).

Observation 1

To support efficient repeater’s operation, UL-DL TDD information should be provided to the repeater.

- In a system with static TDD (i.e., without “flexible” resources), repeater may be preconfigured, or semi-statically acquire this information (from reading SIB1, or from OAM).

- In a system with dynamic TDD (i.e., with “flexible” resources), side control info should provide UL-DL TDD information to the repeater.

  - Otherwise, the repeater either cannot be active during the “flexible” resources or would have to forward the signals blindly in both DL and UL directions.

Observation 2

A repeater with a capability of forwarding signals simultaneously in both DL/UL directions has a larger implementation cost. It may also have to back off from its max amplification gain to address the coupling issue across the two active amplification chains and avoid oscillation/instability.

Observation 3

A TDD-aware repeater (i.e., with full information about UL-DL TDD configuration) has a superior performance compared to a repeater that forwards signals blindly in both DL/UL directions.

- It can increase the coverage (e.g., by ~10%), and improve the throughput (e.g., a median SINR improvement of ~4.5dB).

Proposal 3

Support side control info to provide UL-DL TDD configuration information, especially for the deployments with dynamic TDD (“flexible” resources).

## ON-OFF information

As indicated in the SID objective, being able to turn off the repeater within resources (symbols/slots) that it is not needed can lead to less interference, and lower power consumption of the repeaters.

Our studies show dynamic ON-OFF information can improve the throughput in deployments where interference is not non-negligible. For example, the median SINR may improve by ~2dB.

Observation 4

Being able to dynamically turn the repeater ON and OFF can save power and reduce the interference.

- e.g., the median SINR can improve by ~2dB.

Proposal 4

Support side control info to provide dynamic ON-OFF information.

## Beamforming information

When talking about the beamforming configuration, we should keep in mind that the repeater supports simultaneous communications on two links:

- gNB-repeater link (we may call this link “backhaul” for the discussions)

- repeater-UE link (we may call this link “access” for the discussions)

The conventional repeater (e.g., Rel-17 repeaters), with no side control, may not support adaptive beamforming on either of its backhaul or access links.

One may argue that for some deployments, the backhaul link can be LOS and stable and hence an initial [fine] beam can be set properly and adaptive backhaul beamforming may not be needed. We acknowledge such an argument but notice that the assumption may not hold in general – especially when the repeater is deployed at a lower height and in a more dynamic scattering environment.

RAN1 should further discuss whether it can leverage NCR-UE’s procedures and functionalities to support adaptive backhaul beamforming.

Observation 5

Adaptive backhaul (gNB-side) beamforming will be beneficial/needed when there are multiple paths between the repeater and the gNB with time-varying path gains (e.g., when the repeater is deployed in a dynamic scattering environment such as a low-height deployment).

Proposal 5

Support adaptive backhaul (gNB-side) beamforming.

- RAN1 to discuss whether NCR-UE’s procedures and functionalities are sufficient.

Access-link beamforming is of a great importance. The conventional repeaters cannot locate/track the scheduled UE(s). Therefore, they may have a fixed, and rather wide, beam towards the UEs, and cannot support adaptive access-link beamforming. The benefits of adaptive [fine] beamforming on the access link are obviously larger beamforming gains, lower interference and hence better link strength and quality.

Our studies show adaptive access-link beamforming can significantly improve the throughput. For example, the median SINR may improve by ~11.5 dB.

Observation 6

Adaptive access-link (UE-side) beamforming will offer significant performance gain by providing a larger beamforming gain and reducing the interference (due to use of narrower beams).

- e.g., the median SINR can improve by 11dB.

Proposal 6

Support side control info to provide dynamic access-link (UE-side) beamforming configuration.

## Transmission/reception timing alignment information

A TDD repeater should have its UL-DL timing properly set to make sure it can forward the UL/DL symbols in full and will not cause additional interference.

Observation 7

A TDD repeater should have its UL-DL timing properly set to make sure it can forward the UL/DL symbols in full and will not cause additional interference.

Figure 2 shows the TX/RX timings of UL and DL signals sent via the repeater. We note that the repeater may add an extra latency, group delay TGD, as it receives and forwards the signals. The group delay can be in the order of 10(s) nsec, or more depending on the architecture and implementation of the repeater.

Figure 2 TX/RX timing references

Observation 8

A repeater may cause an extra latency, group delay TGD, as it receives and forwards the signals. The group delay can be in the order of 10(s) nsec or more.

Assuming the repeater can track the DL RX timing from the gNB, it should use the same timing reference for forwarding the DL signals to the UE.

Observation 9

A repeater uses DL RX timing from the gNB for forwarding DL signals to the UE.

For forwarding UL signals,

- The repeater can utilize the available guard periods to open its UL forwarding window wide enough to fully capture UE’s UL signal for a various range of propagation delays on repeater-UE link (Figure 3 (a)). However, this may lead to extra interference.

- Assuming the repeater has received an UL timing advance for its own control-related communications with the gNB, it can use its UL TX timing reference to better align its UL forwarding window (Figure 3 (b)). This helps reducing the size of the UL forwarding window and hence the interference.

  - We should note that the UL forwarding timing should be selected to accommodate the repeater’s group delay TGD.

- (b)

Figure 3 UL forwarding window

Observation 10

A repeater may leverage the available guard periods, its own timing advance value, and/or knowledge of group delay to properly set its UL forwarding window.

Proposal 7

A repeater is expected to properly set its DL and UL forwarding time, based on the available information including DL RX timing (e.g., by tracking DLRS like SSBs), NCR-UE’s UL TX timing (e.g., via UL TAC received by NCR-UE), and knowledge about the repeater’s group-delay.

## Power control information

Power control is identified as a 2nd priority side control info in the SID. Repeater’s power adaptation, under the network’s control, can reduce interference and power consumption at the repeater and/or UEs.

We should note that the power adjustment of a repeater could be subject to (a) a max output TX power, and/or (b) a max amplification gain.

Observation 11

Repeater’s power adjustment may be subject to (a) a max TX power, and/or (b) a max amplification gain.

In our view, power control can prove to be useful side control info. RAN1 may further consider supporting this side control, after discussing other higher-priority side control information.

Proposal 8

RAN1 to consider supporting power control information, as a 2nd priority and after discussing other higher-priority side control information.

# 4 Conclusion

In this contribution, we provided our views on the high-level architecture of a network-controlled repeater and the benefits and necessities of identified side control information.

The following observations and proposals are made.

Proposal 1

A network-controlled repeater (NCR) comprises the following two components:

- NCR-UE: a UE functionality that establishes an interface to the network to support DL/UL communication of side control and related information.

- A&F: an amplify-and-forward component that provides an analog pass-through for the UEs’ DL/UL signals.

Proposal 2

NCR-UE, at least, supports the lower-layer (L1/L2) functionalities.

It is outside RAN1 scope to discuss and decide about the repeater’s architecture, and such discussions should be left to RAN2/3.

--- UL-DL TDD information ---

Observation 1

To support efficient repeater’s operation, UL-DL TDD information should be provided to the repeater.

- In a system with static TDD (i.e., without “flexible” resources), repeater may be preconfigured, or semi-statically acquire this information (from reading SIB1, or from OAM).

- In a system with dynamic TDD (i.e., with “flexible” resources), side control info should provide UL-DL TDD information to the repeater.

  - Otherwise, the repeater either cannot be active during the “flexible” resources or would have to forward the signals blindly in both DL and UL directions.

Observation 2

A repeater with a capability of forwarding signals simultaneously in both DL/UL directions has a larger implementation cost. It may also have to back off from its max amplification gain to address the coupling issue across the two active amplification chains and avoid oscillation/instability.

Observation 3

A TDD-aware repeater (i.e., with full information about UL-DL TDD configuration) has a superior performance compared to a repeater that forwards signals blindly in both DL/UL directions.

- It can increase the coverage (e.g., by ~10%), and improve the throughput (e.g., a median SINR improvement of ~4.5dB).

Proposal 3

Support side control info to provide UL-DL TDD configuration information, especially for the deployments with dynamic TDD (with “flexible” resources).

--- ON-OFF information ---

Observation 4

Being able to dynamically turn the repeater ON and OFF can save power and reduce the interference.

- e.g., the median SINR can improve by ~2dB.

Proposal 4

Support side control info to provide dynamic ON-OFF information.

--- beamforming information ---

Observation 5

Adaptive backhaul (gNB-side) beamforming will be beneficial/needed when there are multiple paths between the repeater and the gNB with time-varying path gains (e.g., when the repeater is deployed in a dynamic scattering environment such as a low-height deployment).

Proposal 5

Support adaptive backhaul (gNB-side) beamforming.

- RAN1 to discuss whether NCR-UE’s procedures and functionalities are sufficient.

Observation 6

Adaptive access-link (UE-side) beamforming will offer significant performance gain by providing a larger beamforming gain and reducing the interference (due to use of narrower beams).

- e.g., the median SINR can improve by 11dB.

Proposal 6

Support side control info to provide dynamic access-link (UE-side) beamforming configuration.

--- TX/RX timing alignment information ---

Observation 7

A TDD repeater should have its UL-DL timing properly set to make sure it can forward the UL/DL symbols in full and will not cause additional interference.

Observation 8

A repeater may cause an extra latency, group delay TGD, as it receives and forwards the signals. The group delay can be in the order of 10(s) nsec or more.

Observation 9

A repeater uses DL RX timing from the gNB for forwarding DL signals to the UE.

Observation 10

A repeater may leverage the available guard periods, its own timing advance value, and/or knowledge of group delay to properly set its UL forwarding window.

Proposal 7

A repeater is expected to properly set its DL and UL forwarding time, based on the available information including DL RX timing (e.g., by tracking DLRS like SSBs), NCR-UE’s UL TX timing (e.g., via UL TAC received by NCR-UE), and knowledge about the repeater’s group-delay.

--- Power control information ---

Observation 11

Repeater’s power adjustment may be subject to (a) a max TX power, and/or (b) a max amplification gain.

Proposal 8

RAN1 to consider supporting power control information, as a 2nd priority and after discussing other higher-priority side control information.

# 5 References

[1] RP-213700, “New SI: Study on NR Network-controlled Repeaters”

# 6 Appendix

This section provides assumptions and simulation results of our system-level analysis.

In order to investigate the benefits of different side control info, we have evaluated and compared the performance of different types of repeaters based on the availability of side control information. Table 1 summarizes different repeater types.

Table 1 Different types of repeaters based on the available side control info

| Type of Repeater (the names are arbitrary and for the reference) | Side control information |  |  |
| --- | --- | --- | --- |
|  | TDD awareness (full info about UL-DL TDD config) | ON-OFF information | Adaptative access-link (UE-side) beamforming |
| Class A (conventional) | Not available | Not available | Not available |
| Class A+ | Available | Not available | Not available |
| Class A++ | Available | Semi-static ON-OFF (repeater is OFF if there is no associated UE) | Not available |
|  |  | Dynamic ON-OFF (repeater is OFF within slots where there is no associated scheduled UE) |  |
| Class B | Available | Available | Available |

The performance of these repeaters are evaluated for an urban grid deployment (Figure 1), where 84 gNBs are deployed on every other streets, and a total of 156 repeaters are deployed on the streets with no gNBs. The deployment setup is intended to study the benefits of repeaters to extend the coverage of gNBs by bending over their signals on the streets with no/minimal direct gNBs’ coverage. UEs (a total of 252) are dropped outdoor along the streets with no deployed gNBs. Other assumptions can be found in Table 2.

Figure 4 Urban grid deployment

Table 2 Simulation assumptions

| Parameters | Values |
| --- | --- |
| Repeater’s max amplification gain | 50 dB for Class A (w/ no TDD info and hence forwarding signals in both DL and UL directions) 70 dB for TDD-aware repeaters (forwarding signals only in one of DL or UL directions) |
| Repeater’s antenna configuration | 2 backhaul arrays (facing gNBs), 2 access arrays (facing UEs). Each array comprises 16x4 antenna elements. For repeaters with no adaptive access beamforming, a fixed beam with max BF gain of 10dB is assumed. |
| gNB’s antenna configuration | 4 sectors (arrays), each comprising 16x4 antenna elements (PA = 7dBm, and peak element pattern gain = 4dB) |
| Frequency configuration | Carrier freq = 28GHz DL BW = 800MHz, UL BW = 100MHz |
| Traffic type | Full-buffer |

The simulation results are shown in Figure 5 and summarized in Table 3.

(a) DL SINR  					(b) UL SINR

Figure 5 CDF of DL and UL SINR

Table 3 Summary of performance results

| Performance​ | Downlink​ |  |  | Uplink​ |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Coverage​* | 50% SINR​ | 50% rate**​ | Coverage​* | 50% SINR​ | 50% rate**​ |
| Class A (conventional)​ | 81%​ | 5.8dB​ | 1.14​ | 80%​ | 4.85​ | 1​ |
| Offered gain compared to class A​ |  |  |  |  |  |  |
| Class A+ (TDD awareness)​ | 1.1X​ | +4.6dB​ | 1.9X​ | 1.18X​ | +4​ | 1.8X​ |
| Class A++ (TDD awareness + semi-static ON-OFF info)​ | 1.16X​ | +5.54dB​ | 2.1X​ | 1.21X​ | +5.55​ | 2.2X​ |
| Class A++ (TDD awareness + dynamic ON-OFF info)​ | 1.2X​ | +6.4dB​ | 2.3X​ | 1.22X​ | +6​ | 2.3X​ |
| Class B (TDD awareness + dynamic ON-OFF + spatial info)​ | 1.23X​ | +17.5dB​ | 5.32X​ | 1.25X​ | +17.15​ | 5.7X​ |

* coverage is defined based on 0dB SINR threshold

** spectral efficiency (bits/sec/Hz)