3GPP TSG RAN WG1 #109-e			R1-2205048

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.8.2

Source: 	Qualcomm Incorporated

Title: 	On L1-L2 signaling for side control information for NCR

Document for:	Discussion/Decision

# 1 Introduction

This contribution provides our views on the following objective from the NR network-controlled repeaters SID [1]:

“Study and identify L1/L2 signaling (including its configuration) to carry the side control information [RAN1]”

In our companion paper [2], we discussed about the side control information. Section 2 summarizes the main proposals from [2] for easier reference. Section 3 discusses L1/L2 signaling to carry the side control info.

# 2 Side Control Information

This section summarizes our main viewpoints, discussed in more details in [2].

## High-level model of a network-controlled repeater

A network-controlled repeater (NCR) comprises the following two components (Figure 1):

- NCR-UE: a UE functionality that establishes an interface to the network to support DL/UL communication of side control and related information.

  - NCR-UE, at least, supports the lower-layer (L1/L2) functionalities.

- A&F: an amplify-and-forward component that provides an analog pass-through for the UEs’ DL/UL signals.

Figure 1 Network-controlled repeaters (NCR) simple model

## Side-control information

Below is our view on the identified side control information.

- UL-DL TDD configuration

  - Support indication of the UL-DL TDD configuration, especially for the deployments with dynamic TDD (i.e., with “flexible” resources)

- ON-OFF information

  - Support indication of dynamic ON-OFF information

- Beamforming information

  - Support indication of dynamic access-link (UE-side) beamforming info

  - Support adaptive backhaul (gNB-side) beamforming

    - FFS whether NCR-UE’s procedures and functionalities can be reused

- Transmission/reception timing alignment

  - A repeater is expected to set its DL and UL forwarding time, based on the available information including DL RX timing (e.g., by tracking DLRS like SSBs), NCR-UE’s UL TX timing (e.g., via UL TAC received by NCR-UE), and knowledge about the repeater’s group-delay.

  - No new explicit side control information is needed.

- Power control information

  - Can be supported, after discussing other higher-priority side control information.

# 3 L1/L2 Signalling

For the design of L1/L2 signaling carrying the side-control, we first need to agree on the main principals.

As discussed in [2] and summarized in the previous sections, the network can get the most benefit from a repeater when it can adapt the repeater’s operation/configuration dynamically and with a sufficient level of granularity. For example, network should be able to change the repeater’s [access-link] beamforming dynamically and with symbol-level granularity.

Observation 1

Network gets the most benefit from a repeater when it can adapt the repeater’s operation/configuration dynamically and with a sufficient level of granularity.

Proposal 1

Support dynamic and granular (symbol-level) side control info indication.

Figure 2 shows an example in which gNB uses a repeater to serve UE1 and UE2. For example, if the gNB wants to send a downlink signal to UE1, it needs to first send a control message, in advance, to repeater to provide the required side control info (e.g., the beamforming info, forwarding direction (UL/DL), and the associated time-domain resources).

Figure 2 gNB using a repeater (R) to serve remote UEs

Given the above example, and Proposal 1, we should at least support L1 (DCI-based) signaling to enable fast adaptation of repeater’s configurations.

On the other hand, the repeater may be used to forward semi-static/periodic signals (such as SSBs/RMSI/CSI-RS/RACH, etc.). An L2 (MAC-CE based) or L3 (RRC based) signaling to carry the related side control information will be more efficient.

Observation 2

L1 (DCI-based) signaling is needed to support fast adaptation of repeater’s configurations.

L2 (MAC-CE) and/or L3 (RRC) signaling may be more efficient for configuring the repeater’s forwarding operation associated with semi-static/periodic signals.

Proposal 2

At least, support L1 (DCI-based) signaling to carry side control info.

RAN1 to also discuss L2/L3 (MAC-CE or RRC based) signaling to carry side control info associated with forwarding semi-static/periodic signals (such as SSB/RMSI/CSI-RS/RACH/etc.)

It should be clear, but still noteworthy, that the signals carrying the side control information should further indicate the time-domain resources (e.g., symbol/slot indices) for which the indicated control info (e.g., beamforming, ON-OFF, etc.) should be adopted.

Proposal 3

The signaling that carries the side control info should also indicate the associated time-domain resources (e.g., symbol/slot indices) within which the repeater is to adopt the indicated side control info.

Another important aspect is the required information that the repeater should share with the network.

As shown in Figure 1, a repeater has two components: (a) NCR-UE, and (2) A&F.

NCR-UE supports features/capabilities, where some of which may need to be reported to the network. Such features/capabilities and their reporting would depend on the repeater’s architecture and should be discussed later pending the discussions/decisions about the latter.

Observation 3

NCR-UE’s features/capabilities and their reporting would depend on the NCR-UE’s architecture (e.g., whether it supports full-stack or only lower layers).

Proposal 4

NCR-UE’s features/capabilities should be discussed later, after discussing/deciding about the architecture.

The repeater also has some features/configurations associated with its A&F component. Beam-related information (such as the number of beams NCR can support on its access-link side) is one such example. Other possible examples are power-related or frequency-related information. The repeater should be able to report such information to the network.

A&F related information may be reported along with NCR-UE’s features/capabilities (e.g., by extending the legacy capability reporting framework). Alternatively, A&F related information may be reported separately, and especially via lower-layer (L1/L2) signaling.

Observation 4

A repeater has features/configurations associated with its A&F component – such as beam-related (e.g., number of supported TX/RX beams), power-related, and frequency-related information.

Proposal 5

Support indication of A&F specific information to the network.

- At least including beam-relate info (e.g., the number of supported TX/RX beams)

- FFS: indication of other information such as power-related and/or frequency-related info.

- FFS: signaling method (e.g., new L1/L2 signaling, or L3 signaling with a possibility of joint report with other repeater’s features/capabilities associated with NCR-UE)

# 4 Conclusion

In this contribution, we discussed the aspects related to signaling design to carry the side control and other repeater’s information. The following observations and proposals are made.

Observation 1

Network gets the most benefit from a repeater when it can adapt the repeater’s operation/configuration dynamically and with a sufficient level of granularity.

Proposal 1

Support dynamic and granular (symbol-level) side control info indication.

Observation 2

L1 (DCI-based) signaling is needed to support fast adaptation of repeater’s configurations.

L2 (MAC-CE) and/or L3 (RRC) signaling may be more efficient for configuring the repeater’s forwarding operation associated with semi-static/periodic signals.

Proposal 2

At least, support L1 (DCI-based) signaling to carry side control info.

RAN1 to also discuss L2/L3 (MAC-CE or RRC based) signaling to carry side control info associated with forwarding semi-static/periodic signals (such as SSB/RMSI/CSI-RS/RACH/etc.)

Proposal 3

The signaling that carries the side control info should also indicate the associated time-domain resources (e.g., symbol/slot indices) within which the repeater is to adopt the indicated side control info.

Observation 3

NCR-UE’s features/capabilities and their reporting would depend on the NCR-UE’s architecture (e.g., whether it supports full-stack or only lower layers).

Proposal 4

NCR-UE’s features/capabilities should be discussed later, after discussing/deciding about the architecture.

Observation 4

A repeater has features/configurations associated with its A&F component – such as beam-related (e.g., number of supported TX/RX beams), power-related, and frequency-related information.

Proposal 5

Support indication of A&F specific information to the network.

- At least including beam-relate info (e.g., the number of supported TX/RX beams)

- FFS: indication of other information such as power-related and/or frequency-related info.

- FFS: signaling method (e.g., new L1/L2 signaling, or L3 signaling with a possibility of joint report with other repeater’s features/capabilities associated with NCR-UE)

# 5 References

[1] RP-213700, “New SI: Study on NR Network-controlled Repeaters”

[2] R1-2205047, “On side control information for network controlled repeaters (NCR)”, Qualcomm.