3GPP TSG RAN WG1 Meeting #109-e    	            R1-2205049

E-meeting, May 9th – 20th, 2022

Source:	Qualcomm Incorporated

Title:	NR PDCCH reception in symbols with LTE CRS REs

Agenda Item:	9.9.1

Document for: 	Discussion and Decision

# Introduction

The 1st objective of Rel-18 WI enhancement of NR DSS is to study and if necessary, specify NR PDCCH reception in symbols with LTE CRS REs.

- Study and if needed specify NR PDCCH reception in symbols with LTE CRS REs. [RAN1] - Investigate enabling LTE CRS to puncture NR PDCCH, including the impact to NR PDCCH DMRS if there is the performance gain from the additional PDCCH resources.

In this contribution, we provide our views on this objective.

# Background

TS 38.213 has specified that if at least one RE of a PDCCH candidate for a UE on the serving cell overlaps with at least one RE of lte-CRS-ToMatchAround, or of LTE-CRS-PatternList, the UE is not required to monitor the PDCCH candidate. Since any PDCCH candidate is mapped to all REs in integer number of REGs, a UE does not monitor any PDCCH candidate if the PDCCH monitoring occasion (identified via search space monitoring occasion and duration of the associated CORESET) overlaps with at least one LTE CRS symbol identified through lte-CRS-ToMatchAround or LTE-CRS-PatternList. Because of this, PDCCH monitoring occasion in a DSS carrier is in reality limited to the 3rd OFDM symbol of a slot using 1-symbol CORESET for a carrier with 4-port CRS (for a carrier with 1 or 2-port CRS, PDCCH can be on the 2nd and 3rd OFDM symbols). For 5MHz and 10MHz UE channel bandwidths, the number of CCEs for 1-symbol CORESET is only up to 4 and 8, respectively.

A number of solutions have been specified so far as summarized in Fig. 1. In Rel-15, FG3-2 for PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot was specified (as known as PDCCH case 1-2). For such UE, any non-LTE CRS symbols can be configured for PDCCH monitoring, e.g., on symbols #2-#3, #5-#6, #9-#10, or #12-#13. At the RAN1#108-e meeting, another option of the UE capability for PDCCH case 1-2 was agreed as FG22-12 for Rel-16. With this FG, 2 OFDM symbols can be ensured for the UE even in a DSS carrier with 4-port CRS. In Rel-17, cross-carrier scheduling from sSCell to P(S)Cell was supported. With this, SCell PDCCH resources can be used for P(S)Cell data scheduling.

| (a) Basic UE | (b) Rel-15 FG3-2 |
| --- | --- |
| (c) Rel-16 FG22-12 | (d) Rel-17 FG34-1/34-2 |

Fig. 1	PDCCH reception in a DSS carrier with legacy features

In Rel-18, support of PDCCH on LTE-CRS symbol(s) is to be considered. Since legacy UEs do not monitor such PDCCH, the potential Rel-18 solution (if supported) is not available for any of legacy UEs, and hence, the network has to take care of both legacy UEs and Rel-18 UEs in different ways. Benefits highly depend on the ratio/amount of Rel-18 UEs supporting the potential Rel-18 solution. In addition, for UEs supporting FG6-2, 22-12, and/or 34-1/34-2, the potential Rel-18 solution would not be essential. Therefore, the benefit depends also on the ratio of UEs supporting and configured with FG6-2, 22-12, and/or 34-1/34-2.

Observation:

- There have been a number of optional features that address the issue of PDCCH resource limitation in a DSS carrier

  - The overall benefit of potential Rel-18 solution depends on various factors such as ratio/amount of UEs supporting legacy FGs or the potential Rel-18 solution

# Principles for possible solution

In this section, potential Rel-18 solution is discussed.

For PDCCH, maximum numbers of blind decodes and non-overlapped CCEs have been specified to cap the UE complexities regarding PDCCH blind decodes and channel estimation. For Rel-18 DSS, complexities regarding PDCCH blind decodes and channel estimation shall not be increased. Potential solutions that increase these complexities may lead to a need of reducing BD/CCE budget, increasing UE processing time, and so on.

In particular, careful investigation is necessary for PDCCH DMRS design/handling. For example, puncturing some PDCCH DMRS REs due to collision with CRS REs leads to unequally distant DMRS RE mapping in frequency-domain after the puncturing. For unequally distant DMRS REs, averaging/filtering that has been used for legacy PDCCH/CORESET channel estimation may not work. We believe potential Rel-18 solution shall ensure that PDCCH DMRS is mapped on equally distant REs in frequency-domain. Unlike LTE CRS, legacy NR PDCCH DMRS has not introduced DMRS RE shift in frequency-domain. Introducing such RE shift in frequency-domain in Rel-18 increases the patterns/configurations of PDCCH/CORESET. Therefore, such RE shift should also be avoided.

Proposal 1:

- Potential solution (if supported) shall not increase the UE complexity for PDCCH blind decodes and channel estimation

  - For PDCCH channel estimation, the UE uses DMRS mapped on equally distant REs in frequency-domain

  - The DMRS mapping/pattern does not depend on the LTE-CRS pattern/configuration or PDCCH monitoring occasion

As described in Section 2, if a UE supports PDCCH monitoring case 1-2 (or case 2), support of NR PDCCH reception in LTE-CRS symbol(s) is no longer essential. For this Rel-18 WI, we propose to limit the scope to the scenarios where a UE does not support FG3-2 (or other FGs enabling PDCCH monitoring case 2). With this, the discussion can be limited to PDCCH reception in the first [3 or 4] OFDM symbols in a slot. The value between 3 and 4 can be concluded once the specification impact and potential benefit is clearer.

Proposal 2:

- Potential solution (if supported) is for PDCCH reception in the first [3 or 4] OFDM symbols in a slot

  - PDCCH reception on the other spans/positions in a slot is not the scope of this WI

# Possible designs

Within the first [3 or 4] OFDM symbols of a slot, first 1 or 2 OFDM symbols have LTE CRS while remaining does not. One question that would impact on the design discussion is whether to support NR PDCCH reception that fully overlapped with LTE CRS. We think this is not a reasonable scenario. The first OFDM symbol (i.e., symbol #0) of an LTE subframe typically carries not only LTE-CRS/LTE-PDCCH but also PCFICH/PHICH and hence should be dedicated to LTE operation of the DSS carrier. With this understanding, the question is limited to whether to support NR PDCCH reception within symbol #1. 1-symbol CORESET with a fixed PDCCH DMRS that is equal-distantly mapped in frequency-domain is not possible due to LTE-CRS v-shift. Therefore, we consider that the potential solution here should be for the case where at least 1 OFDM symbol of a PDCCH monitoring occasion does not overlap with LTE CRS.

Proposal 3:

- Potential solution (if supported) for PDCCH reception in symbol(s) with LTE CRS REs shall be for the cases where at least 1 OFDM symbol of a PDCCH monitoring occasion does not overlap with LTE CRS

  - I.e., PDCCH reception for the case where all the symbols of a PDCCH monitoring occasion overlap with LTE CRS is not considered

  - FFS: the case where 2 OFDM symbols of a PDCCH monitoring occasion do not overlap with LTE CRS

With all these above in mind, we propose to consider the following as the potential solution for PDCCH reception in symbol(s) with LTE CRS REs:

- PDCCH spans 2 [or 3] consecutive symbols where the first 1 [or 2] OFDM symbol(s) overlaps with LTE CRS REs but at least the last OFDM symbol of the PDCCH does not

- On symbols where LTE CRS REs are present, PDCCH DMRS is not transmitted

- On symbols where LTE CRS REs are not present, PDCCH DMRS is mapped in the same way as for legacy PDCCH/CORESET (i.e., DMRS on REs #1, #5, #9 in each REG)

- PDCCH payload is mapped on REs used for monitored PDCCH and not used for the associated PDCCH DMRS in the same way as for legacy PDCCH/CORESET, and then is punctured at REs colliding with LTE CRS REs

- Following will not be changed from legacy PDCCH/CORESET:

  - REG-bundle sizes

  - CCE-to-REG mapping

  - Aggregation levels

The structure with the REG-bundle size of 2 is illustrated as an example in Fig. 2. Red REs represent PDCCH DMRS while yellow REs are for PDCCH payload. With this, DMRS mapping/location is equal distant and is not affected by CRS v-shift. The UE uses DMRS in the same way as for legacy PDCCH/CORESET. PDCCH payload is mapped on REs used for monitored PDCCH and not used for the associated PDCCH DMRS and then some REs are punctured due to the collision with LTE CRS REs.

Fig. 2	PDCCH reception in symbol(s) with LTE CRS

Note that network has to accommodate Rel-18 UEs supporting this PDCCH/CORESET structure and legacy UEs on the same DSS carrier. Therefore, it makes sense to consider that Type-0/0A/1/2 CSS sets are configured to be monitored on symbol #2 using 1-symbol legacy CORESET for both Rel-18 UEs and legacy UEs.

Proposal 4:

- Consider following if a solution is specified for Rel-18:

  - PDCCH spans 2 [or 3] consecutive symbols where the first 1 [or 2] OFDM symbol(s) overlaps with LTE CRS REs but at least the last OFDM symbol of the PDCCH does not

    - On symbols where LTE CRS REs are present, PDCCH DMRS is not transmitted

    - On symbols where LTE CRS REs are not present, PDCCH DMRS is mapped in the same way as for legacy PDCCH/CORESET (i.e., DMRS on REs #1, #5, #9)

    - PDCCH payload is mapped on REs used for monitored PDCCH and not used for the associated PDCCH DMRS in the same way as for legacy PDCCH/CORESET, and then is punctured at REs colliding with LTE CRS REs

  - Following are not changed from legacy PDCCH/CPRESET:

    - REG-bundle sizes

    - CCE-to-REG mapping

    - Aggregation levels

  - Type-0/0A/1/2 CSS-sets are transmitted using legacy PDCCH/CORESET on non-LTE CRS symbol(s)

    - E.g., these SS sets are associated with 1-symbol CORESET and monitored on symbol #2

# Conclusion

In this contribution, NR PDCCH reception in symbols with LTE CRS REs was discussed. Observation and proposals are following.

Observation:

- There have been a number of optional features that address the issue of PDCCH resource limitation in a DSS carrier

  - The overall benefit of potential Rel-18 solution depends on various factors such as ratio/amount of UEs supporting legacy FGs or the potential Rel-18 solution

Proposal 1:

- Potential solution (if supported) shall not increase the UE complexity for PDCCH blind decodes and channel estimation

  - For PDCCH channel estimation, the UE uses DMRS mapped on equally distant REs in frequency-domain

  - The DMRS mapping/pattern does not depend on the LTE-CRS pattern/configuration or PDCCH monitoring occasion

Proposal 2:

- Potential solution (if supported) is for PDCCH reception in the first [3 or 4] OFDM symbols in a slot

  - PDCCH reception on the other spans/positions in a slot is not the scope of this WI

Proposal 3:

- Potential solution (if supported) for PDCCH reception in symbol(s) with LTE CRS REs shall be for the cases where at least 1 OFDM symbol of a PDCCH monitoring occasion does not overlap with LTE CRS

  - I.e., PDCCH reception for the case where all the symbols of a PDCCH monitoring occasion overlap with LTE CRS is not considered

  - FFS: the case where 2 OFDM symbols of a PDCCH monitoring occasion do not overlap with LTE CRS

Proposal 4:

- Consider following if a solution is specified for Rel-18:

  - PDCCH spans 2 [or 3] consecutive symbols where the first 1 [or 2] OFDM symbol(s) overlaps with LTE CRS REs but at least the last OFDM symbol of the PDCCH does not

    - On symbols where LTE CRS REs are present, PDCCH DMRS is not transmitted

    - On symbols where LTE CRS REs are not present, PDCCH DMRS is mapped in the same way as for legacy PDCCH/CORESET (i.e., DMRS on REs #1, #5, #9)

    - PDCCH payload is mapped on REs used for monitored PDCCH and not used for the associated PDCCH DMRS in the same way as for legacy PDCCH/CORESET, and then is punctured at REs colliding with LTE CRS REs

  - Following are not changed from legacy PDCCH/CPRESET:

    - REG-bundle sizes

    - CCE-to-REG mapping

    - Aggregation levels

  - Type-0/0A/1/2 CSS-sets are transmitted using legacy PDCCH/CORESET on non-LTE CRS symbol(s)

    - E.g., these SS sets are associated with 1-symbol CORESET and monitored on symbol #2

# Reference

- RP-213575, ‘New WI: Enhancement of NR Dynamic spectrum sharing (DSS),’ Ericsson