3GPP TSG RAN WG1 Meeting #109-e    	            R1-2205050

E-meeting, May 9th – 20th, 2022

Source:	Qualcomm Incorporated

Title:	UE support for two overlapping CRS rate matching patterns

Agenda Item:	9.9.2

Document for: 	Discussion and Decision

# Background

The 2nd objective of Rel-18 WI enhancement of NR DSS is to support two overlapping CRS rate matching patterns regardless of support or configuration of multi-TRP.

- Allow a UE to support, and be configured with, two overlapping CRS rate matching patterns regardless of support or configuration of multi-TRP [RAN1, RAN2]

In this contribution, we provide our views on this.

# Support of two overlapping CRS rate-matching patterns

Existing specifications related to two overlapping CRS rate matching patterns are copied below.

TS 38.331

lte-CRS-PatternList1 A list of LTE CRS patterns around which the UE shall do rate matching for PDSCH. The LTE CRS patterns in this list shall be non-overlapping in frequency. The network does not configure this field and lte-CRS-ToMatchAround simultaneously.

lte-CRS-PatternList2 A list of LTE CRS patterns around which the UE shall do rate matching for PDSCH scheduled with a DCI detected on a CORESET with CORESETPoolIndex configured with 1. This list is configured only if CORESETPoolIndex configured with 1. The first LTE CRS pattern in this list shall be fully overlapping in frequency with the first LTE CRS pattern in lte-CRS-PatternList1, The second LTE CRS pattern in this list shall be fully overlapping in frequency with the second LTE CRS pattern in lte-CRS-PatternList1, and so on. Network configures this field only if the field lte-CRS-ToMatchAround is not configured and there is at least one ControlResourceSet in one DL BWP of this serving cell with coresetPoolIndex set to 1.

TS38.214

- If the UE is configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet and is also configured by the higher layer parameter lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 in ServingCellConfig, the following REs are declared as not available for PDSCH: - if the UE is configured with crs-RateMatch-PerCoresetPoolIndex, REs indicated by the CRS pattern(s) in lte-CRS-PatternList1-r16 if the PDSCH is associated with coresetPoolIndex set to '0', or the CRS pattern(s) in lte-CRS-PatternList2-r16 if PDSCH is associated with coresetPoolIndex set to '1'; - otherwise, REs indicated by lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16, in ServingCellConfig.

TS38.306

| overlapRateMatchingEUTRA-CRS-r16 Indicates whether the UE supports two LTE-CRS overlapping rate matching patterns within a part of NR carrier using 15 kHz SCS overlapping with a LTE carrier. If the UE supports this feature, the UE needs to report multipleRateMatchingEUTRA-CRS-r16. | Band | No | N/A | FR1 only |
| --- | --- | --- | --- | --- |

| separateCRS-RateMatching-r16 Indicates whether the UE supports rate match around configured CRS patterns which is associated with CORESETPoolIndex (if configured) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same value of CORESETPoolIndex. The UE that indicates support of this feature shall support multiDCI-MultiTRP-r16 and overlapRateMatchingEUTRA-CRS-r16. This is only applicable for 15kHz SCS. | Band | No | N/A | FR1 only |
| --- | --- | --- | --- | --- |

To enable two overlapping CRS rate matching patterns without multi-TRP configuration/capability, the yellow part can simply be deleted. However, in this case, a UE indicating support of overlapRateMatchingEUTRA-CRS-r16 is supposed to support two overlapping CRS rate-matching patterns without configuration of coresetPoolIndex = 1. Therefore, if the yellow parts are to be deleted, it should be clarified that support of two overlapping CRS rate-matching patterns according to the legacy UE capability overlapRateMatchingEUTRA-CRS-r16 is subject to support of multiDCI-Multi-TRP-r16 and the UE indicating this legacy UE capability overlapRateMatchingEUTRA-CRS-r16 can be configured with lte-CRS-PatternList2 only if there is at least one ControlResourceSet in one DL BWP of this serving cell with coresetPoolIndex set to 1.

Proposal:

- A UE can be configured with lte-CRS-PatternList1 and lte-CRS-PatternList2 without coresetPoolIndex being configured

  - The n-th LTE CRS pattern in lte-CRS-PatternList2 shall be fully overlapping in frequency with the n-th LTE CRS pattern in lte-CRS-PatternList1 (same rule as the existing multiple overlapping LTE-CRS patterns for m-DCI m-TRP)

- Introduce corresponding new UE capability

  - NW can configure lte-CRS-PatternList1 and lte-CRS-PatternList2 without coresetPoolIndex only if the UE indicates support of this capability

- Clarify that the Rel-16 UE capability overlapRateMatchingEUTRA-CRS-r16 is subject to support of multiDCI-Multi-TRP-r16 and the UE indicating this Rel-16 UE capability can be configured with lte-CRS-PatternList2 only if there is at least one ControlResourceSet in one DL BWP of this serving cell with coresetPoolIndex set to 1

Currently, CRS rate matching pattern is based on semi-static/RRC configuration. Since 2nd CRS rate matching pattern is to address inter-cell interference from neighbor cell, in general, it should be enabled only if the UE observes strong inter-cell interference. A certain flexibility is necessary to achieve the benefit of 2nd CRS rate matching pattern in reality. DCI indication of CRS rate matching pattern(s) could be an option to achieve this.

# Conclusion

In this contribution, support for two overlapping CRS rate-matching patterns was discussed. Following is our proposal.

Proposal:

- A UE can be configured with lte-CRS-PatternList1 and lte-CRS-PatternList2 without coresetPoolIndex being configured

  - The n-th LTE CRS pattern in lte-CRS-PatternList2 shall be fully overlapping in frequency with the n-th LTE CRS pattern in lte-CRS-PatternList1 (same rule as the existing multiple overlapping LTE-CRS patterns for m-DCI m-TRP)

- Introduce corresponding new UE capability

  - NW can configure lte-CRS-PatternList1 and lte-CRS-PatternList2 without coresetPoolIndex only if the UE indicates support of this capability

- Clarify that the Rel-16 UE capability overlapRateMatchingEUTRA-CRS-r16 is subject to support of multiDCI-Multi-TRP-r16 and the UE indicating this Rel-16 UE capability can be configured with lte-CRS-PatternList2 only if there is at least one ControlResourceSet in one DL BWP of this serving cell with coresetPoolIndex set to 1

# Reference

- RP-213575, ‘New WI: Enhancement of NR Dynamic spectrum sharing (DSS),’ Ericsson