3GPP TSG RAN WG1 #109-emeeting	     R1-2205052

e-Meeting, May 9th – May 20th, 2022

Agenda item:	9.10.2

Source: 	Qualcomm Incorporated

Title: 	Discussion on Rel-18 UL Tx switching

Document for:	Discussion/Decision

# Introduction

RAN #94 emeeting approved the work item Rel-18 Multi-carrier enhancements for NR, which includes UL Tx switching for up to four bands [1]. We list the scope below and provide our views in this paper.

2. Study and if necessary specify following enhancements for multi-carrier UL operation [RAN1, RAN2, RAN4] - UL Tx switching schemes across up to 3 or 4 bands with restriction of up to 2 Tx simultaneous transmission for FR1 UEs, including mechanisms to enable more configured UL bands than its simultaneous transmission capability and to support dynamic Tx carrier switching across the configured bands for both single TAG and multiple TAGs configurations (RAN1, RAN4) - UE capability and RRC configuration related signaling (RAN2) - Note: strive for RAN1/2 design agnostic with the number of bands, i.e., common design between 3 and 4 bands - Note: no additional TAG is introduced for UL transmission on a carrier without corresponding DL carrier - Note: this objective does not target to extend the SUL framework to support more than 1 SUL for 1 NUL - Switching time and other RF aspects, and RRM requirements for above UL Tx switching schemes across up to 3 or 4 bands (RAN4) - Note: Prioritize UL Tx switching across up to 3 bands is to be addressed first and then that for up to 4 bands can also be addressed

# General switching scenarios

In Rel-16 and Rel-17, RAN1 specified the UL Tx switching between two bands and we list the major features below.

Table 1 Summary of Rel-16 and Rel-17 switching

|  | Switching band combinations | Multi-carrier scenarios |
| --- | --- | --- |
| Rel-16 | 1 Tx band + 2 Tx band Single CC on each band | CA, SUL and EN-DC |
| Rel-17 | 2Tx band + 2 Tx band One of the bands is with up to 2 contiguous CC | CA, SUL |

For Rel-18, RAN1 agreed to study and, if necessary, specify the enhancement for multi-carrier UL operation with up to four bands.

Table 2 summarizes the general switching cases for up to 4 bands. Due to the expansion in the number of bands, it would be difficult to list all the possible switching case combinations like in Rel-16 and Rel-17. In Table 2, we list three possible cases – Case 1 for 1Tx on two bands, Case 2 for all Tx on single band among all the bands, and Case 3 for all Tx on another band (different from Case 2). The fractions a, b, c and d are used to indicate which band(s) take the Tx chain.

Table 2 General switching cases for Rel-18

|  | Tx status of each band, may be contiguous CA of some band (Band A, B, C, D) |  |
| --- | --- | --- |
| Case 1 | at + bT + cT + dT | Two out of {a, b, c, d} are “1” and the rest are “0” |
| Case 2 | aT + bT + cT + dT | One out of {a, b, c, d} is “1” or “2” and the rest are “0” |
| Case 3 | aT + bT + cT + dT | Another one of {a, b, c, d} is “1” or “2” and the rest are “0” |

Proposal 1: Use the switching cases in Table 2 for Rel-18 UL Tx switching discussion.

# CA

From UE perspective, a CA capable UE needs to keep sufficient memory and processing capability to handle data streams in all aggregated carriers. Once MIMO is introduced, the required resource would be doubled on that carrier. When UL Tx switching is introduced, the UE would be required for extra resource for “dynamic” switching. In worst case, the required resource would be doubled compared with no UL Tx switching if UE doesn’t want to flush the memory during dynamic switching. When the band number increases, this required extra resource would increase exponentially.

Due to the above reasons, we have concerns on dynamic switching among more than 2 bands. If the majority companies still want to define this feature, we would need to discuss how to simplify the implementation.

One method to simplify the UE implementation is to define an anchor band in the switching band combination. For any RF state switch, either the switch to or switch from carrier/band must be the anchor band.

To avoid the required resource increased exponentially, we think there should be no restriction on the UEs choice of MIMO capability on any of the bands/CCs involved in the Rel-18 UL Tx switching band combination. Meanwhile, as the switching bands increase, the UE needs to monitor more switching decisions we propose to avoid frequent scheduling within 14 consecutive symbols.

Proposal 2: For inter-band UL CA, adopt following for UL Tx switching among 3 or 4 bands.

- Identify an anchor band in the switching band combination.

- For any RF state switch, either the switch-to or switch-from carrier/band must be the anchor band.

- No restriction on the UEs choice of MIMO capability on any of the bands/CCs involved in the UL Tx switching band combination

- After one RF state switch, the next RF state switch must occur after 14 symbols or later.

  - Which SCS assumed for symbol duration is TBD.

In the rest part of this section, we provide views on the two options.

## CA Option 1

As we explained above, Rel-18 extends to 3 or 4 bands. To avoid the required resource increase exponentially, there should be no restriction on the UEs choice of MIMO capability on any of the bands/CCs involved in the Rel-18 UL Tx switching band combination. Case 2 is where the anchor band takes all Tx and Case 3 is the non-anchor band takes all Tx.

Table 3 CA Option 1 switching cases

|  | Tx status of each band, may be contiguous CA of some band (Band A, B, C, D) |  |
| --- | --- | --- |
| Case 2 | aT + bT + cT + dT | The anchor band is “1” or “2” and the rest are “0” |
| Case 3 | aT + bT + cT + dT | A non-anchor band is “1” or “2” and the rest are “0” |

In Rel-16 and 17, RAN1 approved the mapping table between switching cases and transmission layers for each band. However, it would be difficult to list all the possible transmission layers for the switching combination cases. Based on Proposal 2 above, we propose RAN1 using a generic description for the mapping tables as in Table 4.

Table 4 CA Option 1 mapping between Tx state and Tx layers

|  | Tx state of each band, may be contiguous CA of some band (Band A, B, C, D) |  | Transmission layers |
| --- | --- | --- | --- |
| Case 2 | aT + bT + cT + dT | The anchor band is “1” or “2” and the rest are “0” | Anchor band: ≥1 layer |
| Case 3 | aT + bT + cT + dT | The non-anchor band is “1” or “2” and the rest are “0” | Non-anchor band: ≥1 layer |

Proposal 3: Adopt Table 4 for CA Option 1 mapping between Tx state and Tx layers.

## CA Option 2

In Rel-16 and Rel-17, CA Option 2 allows simultaneous transmission from two bands/carriers, which is the major difference with Option 1.

As we explained above, Rel-18 extends to 3 or 4 bands. To avoid the required resource increase exponentially, we propose to not restrict the UEs choicing of MIMO capability on any of the bands/CCs involved in the UL Tx switching band combination in Table 5.

- The Case 1 is anchor and non-anchor simultaneous Tx.

- The Case 2 is the anchor band takes all Tx.

- The Case 3 is non-anchor band takes all Tx.

Table 5 CA Option 2 switching cases

|  | Tx status of each band, may be contiguous CA of some band (Band A, B, C, D) |  |
| --- | --- | --- |
| Case 1 | aT + bT + cT + dT | The anchor and one non-anchor band are “1” and rest are “0” |
| Case 2 | aT + bT + cT + dT | The anchor band is “1” or “2” and the rest are “0” |
| Case 3 | aT + bT + cT + dT | The anchor band is “1” or “2” and the rest are “0” |

With considering the simultaneous transmissions, we make the following proposal for Rel-18 CA Option 2.

Proposal 4: For inter-band UL CA Option 2, adopt following for UL Tx switching among 3 or 4 bands.

- Identify an anchor band in the switching band combination.

- For any RF state switch, either the switch-to or switch-from carrier/band must be the anchor band.

- No restriction on the UEs choice of MIMO capability on any of the bands/CCs involved in the UL Tx switching band combination.

- If simultaneous Tx is supported between 2 bands, one of the two bands is always the switching anchor band

  - Among the other bands, i.e. other than the switching anchor band, any one may or may not support simultaneous Tx.

- After one RF state switch, the next RF state switch must occur after 14 symbols or later.

  - Which SCS assumed for symbol duration is TBD.

We translate the above proposal into Table 6 the mapping between Tx status and Tx layers of CA Option 2. We propose to adopt the Table 6 for Option 2 mapping between Tx status and Tx layers.

Table 6 CA Option 2 mapping between Tx state and Tx layers

|  | Tx state of each band, may be contiguous CA of some band (Band A, B, C, D) |  | Transmission layers |
| --- | --- | --- | --- |
| Case 1 | aT + bT + cT + dT | The anchor and one non-anchor band are “1” and rest are “0” | Anchor band: 1 layer Non-anchor band: 1 layer |
| Case 2 | aT + bT + cT + dT | The anchor band is “1” or “2” and the rest are “0” | Anchor band: ≥ 1 layer |
| Case 3 | aT + bT + cT + dT | The non-anchor band is “1” or “2” and the rest are “0” | Non-anchor band: ≥ 1 layer |

Proposal 5: Adopt Table 6 for CA Option 2 mapping between Tx state and Tx layers.

# SUL + CA

In Rel-16 and Rel-17, switching between SUL and the serving cell is supported. In Rel-17, the non-SUL band could be 2 contiguous carriers and one of it should be the NUL paired with SUL. Another change is the UL MIMO is enabled on SUL carrier in Rel-17.

For Rel-18, the combined bands are extended to upto four. For three bands combination, one SUL could be configured and its NUL would be aggregated with another band as normal CA. For four bands combination, there would be two possible combinations in theory as below.

- (1 SUL + 1 NUL) + 2 CA bands

- (1 SUL + 1 NUL) + (1 SUL + 1 NUL)

Our understanding is option b) has never been formally discussed & specified in the standard. Some key issues are unclear like whether two SUL could simultaneously transmit, whether SUL and its non-serving cell could simultaneously transmit. Given the limited TU, we think it would be very difficult to get consensus on these new UE behaviors. We suggest precluding Option b) and only considering Option a) for Rel-18.

Proposal 6: For SUL + CA combination, when the band combination includes four bands, Rel-18 precludes 2 SUL and 2 NUL cases and only specifies 1 SUL and up to 3 CA bands.

# Multiple TAG

In the WID, the switching bands could be with either single TAG or multiple TAGs configurations. RAN#95 discussed Multiple TAG on 2 bands and concluded it’s in the Rel-18 scope and the work is limited to RAN4.

RP-221008 Moderator's summary for discussion [95e-39-R17-TEIs] RAN4 Chair (Huawei) Replaces RP-220899 conclusion: proposals #1, #2, #3 of RP-221008 are endorsed Extension of switching to multiple TAG for UL CA: Proposal #3: To support Tx switching with multiple TAG on 2 bands, it is proposed to add the following note in Rel-18 WI on multi-carrier enhancements in RAN#96, and consider release independence for the switching band pairs. Note: Extension of TX switching for 2 bands to multiple TAG configurations is included in the scope. The work is limited to RAN4.

However, we think there might be different understanding on which WGs should take lead of multiple TAG for more than 2 bands. There are two alternative understandings below:

- Alternative 1: Multiple TAG for both 2 bands and more than 2 bands would be discussed in RAN4.

- Alternative 2: Multiple TAG for more than 2 bands should be discussed in RAN1.

We are ok with either of the above alternative understandings, as long as Multiple TAG would be well discussed & solved. However, we hope companies could discuss and try to reach the consensus whether we would like to take lead of M-TAG for more than 2 bands switching.

Proposal 7: RAN1 to discuss and conclude whether RAN1 would like to take the lead of M-TAG for more than two- band switching or not.

Given RAN4 doesn’t have TUs before June, we want to share our high-level principles on the supported number of TAGs.

Multiple TAGs would largely increase the complexity of UE as the UE needs to perform timing tracking, slot/symbol time stamp calculations for each TAG. The number of tasks to be handled, chance of channel collision, and Tx AGC control efforts would increase exponentially with each addition of TAG. In this sense, we propose to limit the TAG number to two.

Proposal 8: Limit to no more than two TAGs supported for Rel-18 UL Tx switching.

# Conclusions

In this paper, we have provided our views on Rel-18 UL Tx switching and made following proposals.

Proposal 1: Use the switching cases in Table 2 for Rel-18 UL Tx switching discussion.

Proposal 2: For inter-band UL CA, adopt following for UL Tx switching among 3 or 4 bands.

- Identify an anchor band in the switching band combination.

- For any RF state switch, either the switch-to or switch-from carrier/band must be the anchor band.

- No restriction on the UEs choice of MIMO capability on any of the bands/CCs involved in the UL Tx switching band combination

- After one RF state switch, the next RF state switch must occur after 14 symbols or later.

  - Which SCS assumed for symbol duration is TBD.

Proposal 3: Adopt Table 4 for CA Option 1 mapping between Tx state and Tx layers.

Proposal 4: For inter-band UL CA Option 2, adopt following for UL Tx switching among 3 or 4 bands.

- Identify an anchor band in the switching band combination.

- For any RF state switch, either the switch-to or switch-from carrier/band must be the anchor band.

- No restriction on the UEs choice of MIMO capability on any of the bands/CCs involved in the UL Tx switching band combination.

- If simultaneous Tx is supported between 2 bands, one of the two bands is always the switching anchor band

  - Among the other bands, i.e. other than the switching anchor band, any one may or may not support simultaneous Tx.

- After one RF state switch, the next RF state switch must occur after 14 symbols or later.

  - Which SCS assumed for symbol duration is TBD.

Proposal 5: Adopt Table 6 for CA Option 2 mapping between Tx state and Tx layers.

Proposal 6: For SUL + CA combination, when the band combination includes four bands, Rel-18 precludes 2 SUL and 2 NUL case and only specifies 1 SUL and 3 CA bands.

Proposal 7: RAN1 to discuss and conclude whether RAN1 would like to take the lead of M-TAG for more than two- band switching or not.

Proposal 8: Limit to no more than two TAGs supported for Rel-18 UL Tx switching.

# Reference

- RP-213577, Multi-carrier enhancements for NR, Dec. 2021