3GPP TSG RAN WG1 #109-e			R1-2205055

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.11.1

Source: 	Moderator (Qualcomm Incorporated)

Title: 	Moderator Summary#1 on XR specific power saving techniques

Document for:	Discussion and Decision

# Introduction

This contribution summarizes and discusses companies’ proposals and views on the XR-specific power saving techniques for Rel-18 XR study item [1]. In the papers [2] to [25] submitted to RAN1 #109-e, companies identified the following major issues and design challenges for XR specific power saving techniques. Based on these observations, companies proposed potential design enhancements.

- Mismatch between the CDRX cycle and XR traffic periodicity

  - The XR video frame periodicity has a non-integer number of ms. Time offset between CDRX cycle and frame periodicity varies and accumulates over time. This leads to capacity loss due to larger latency (i.e., data arrival outside CDRX on duration) or larger UE power consumption (i.e., dense and short CDRX cycles).

- Mismatch between the PDCCH monitoring periodicity and XR traffic periodicity

  - Like the CDRX issue above, the misalignment in time causes capacity loss or consumes additional power due to frequent PDCCH monitoring.

- Random data arrival time in presence of jitter

  - To meet the PDB requirement of XR traffic, large On Duration timer needs to be configured to cover the entire jitter range. This leads to extra delay for early data arrival or power consumption for late arrival.

- Misalignment between DL transmission and UL transmission

  - For example, UL pose/control information has a generation periodicity of integer number of ms (e.g., 4ms) which does not align with DL video traffic in time. More power is consumed due to limited UE sleep time.

- Multiple data flows for XR applications

  - A single CDRX configuration does not fit multiple flows. Then either multiple CDRXs or XR dedicated PDCCH monitoring window is needed besides the existing CDRX mechanism.

- High data rate, large and varying packet size, tight PDB

  - These are XR unique characteristics different than the eMBB or uRLLC traffic.

This contribution includes the following additional sections to facilitate the email and GTW discussions:

- Section 2 discusses evaluation methodology for Rel-18 XR power saving evaluations

- Section 3 summarizes proposals and initial evaluation results from companies

  - Section 3.1 summarizes proposed CDRX enhancements

  - Section 3.2 summarizes proposed PDCCH monitoring enhancements

  - Section 3.3 summarizes the other proposals

  - Section 3.4 provides observations based on companies’ evaluation results

  - Section 3.5 discusses priority of topics for XR power saving enhancements

- Section 4 captures potential proposals based on discussions among companies

For the 1st round email discussion, please check the discussions and questions with tag [RD1] in Section 2, Section 3.1.5, Section 3.2.4 and Section 3.3.3. Please provide feedback by UTC 3:00am, Thursday, May 12th.

For the 2nd round email discussion, please check the discussions and questions with tag [RD2] in Section 2, Section 3.1.5, Section 3.2.4 and Section 3.3.3. Please provide feedback by UTC 11:58pm May 13th, i.e., before quite period starts.

# Evaluation Methodology

In Rel-17 XR SI, power saving performance for XR was evaluated based on the multi-cell system level simulation [26]. The evaluation methodology includes XR traffic model, deployment scenarios, UE configurations, BS configurations, TDD UL-DL slot format pattern, etc. In addition, it was agreed UE power and capacity are jointly evaluated [27].

In this meeting, companies also discussed in their papers about the evaluation methodology for potential power saving enhancements of the Rel-18 XR SI. The general view is that Rel-17 evaluation methodology should be taken as baseline. The related proposals are summarized in Table 1.

Table 1: Companies’ views on evaluation methodology

| Company | Views |
| --- | --- |
| Ericsson | Observation 7 Extensive power saving features exist for PDCCH monitoring in Rel-17. Proposal 1 XR traffic characteristics, i.e. non-integer traffic period, multiple traffic flows, and traffic jitter range, should be jointly taken into account when configuring and evaluating C-DRX solutions. Proposal 6 Rel-17 PDCCH power saving features are baseline for Rel-18 enhancements. Proposal 8 For performance evaluation of candidate power saving methods, reuse as much as possible the evaluation methodology and simulation assumptions in TR 38.838. |
| OPPO | Proposal 2: Layer-1 based power saving schemes discussed in Rel-17 power saving WI should be the starting point for further Rel-18 XR study. |
| TCL | Proposal 3: PDCCH skipping and search space group switching can be considered for XR power saving. |
| InterDigital | Proposal 3: RAN1 to perform evaluations of XR-specific enhancement techniques (e.g. adaptive CDRX and DCI-based PDCCH monitoring adaptations) for evaluating the power saving performance Proposal 4: Reuse the baseline evaluation assumptions (e.g. deployment scenarios, SLS parameters, traffic models) in TR 38.838 when evaluating performance of power saving gain |
| CMCC | Proposal 4. PDCCH skipping and SSSG switching can be considered to be applied when DRX is not configured to save PDCCH monitoring power consumption. |
| DOCOMO | Proposal 3: Study the necessity of enhancements on the existing schemes, e.g. SSSG switching, PDCCH skipping, WUS, taking into account the relationship between PDCCH monitoring enhancement scheme and CDRX enhancement scheme. |
| LGE | Observation 1: High throughput and capacity should also be taken into account for XR-specific PS enhancements. Observation 3: Discussion on the baseline CDRX scheme that can serve as the baseline for potential enhancements would be useful. Observation 4: Rel-17 PDCCH monitoring adaptation in connected mode can also be applied to the UE not being configured with the CDRX, with enhancements if necessary. |
| Nokia, NSB | Observation 1: KPIs of further UE power saving schemes should be compared to the achievable performance with existing Rel-15/16/17 functionalities. Observation 2: Earlier releases contain multiple functionalities that should be accounted when further UE power saving enhancements are considered for XR, such as: - CDRX operation with short- and longDRX-cycle and MAC-CEs commands - DCP - Minimum scheduling offset restriction - PDCCH monitoring adaptation for skipping PDCCH monitoring and SSSG switching Observation 6: When proposing power saving enhancements, capacity loss should be minimized. Proposal 1: Use the PS evaluation methodology from TR 38.838 as a starting point, when developing the methodology for PS studies in Release 18 XR work. Proposal 2: To facilitate efficient study of applicable PS schemes, narrow the scope of the considered deployments and setups to a minimal sufficient set. Proposal 3: The proposed unified set of setups for PS evaluations is summarized in the following Table: Proposal 4: In addition to the overall power saving gain of the evaluated enhancement, we propose to report the (i) power saving gain of the Rel-15/16/17 power saving scheme without the evaluated enhancement, and (ii) the extra power saving gain of the evaluated enhancement. Proposal 5: For Rel-18 XR PS evaluations, define a Release 17-compliant baseline setup that includes all the applicable power saving features, including but not limited to DRX and PDCCH monitoring adaptation that should be accounted in evaluation of possible benefits. Proposed set of evaluation setups for XR PS studies in Release 18. Data rate [Mbps]; Packet arrival rate [fps]; PDB [ms]. DL; AR/VR; 30; 60; 10. CG; 30; 60; 15. UL; VR/CG: Pose/control; 0.2; 250; 10. AR: Option 1 (single stream model); 10; 60; 30. |
| Intel | Proposal 3: Rel-17 PDCCH skipping and SSSG switching solutions should be considered as starting point for PDCCH monitoring enhancements. |

Based on companies’ input, the following question points are provided.

[RD1] Question 2-1: Rel-17 evaluation methodology for XR power saving captured in TR 38.838 is used as the baseline evaluation methodology for UE power evaluation of Rel-18 SI on XR enhancements.

| Company | Comments |
| --- | --- |
| MTK | We are generally fine with the proposal. Besides, in the Rel-17 TR of the XR study, for baseline assumption, the following three traffic flows are modelled: - DL video traffic is assumed to be periodical arrival with 16.67 ms period (60Hz) - UL pose control is assumed to be periodical with 4 ms period (250Hz) - UL video (AR only) traffic is assumed to be periodical with 16.67 ms period (60Hz) Hence, we suggest RAN1 to adopt Figure 1 below as a common UL/DL traffic arrival model to evaluate the power saving gain under the potential future enhancements: Figure 1 |
| Xiaomi | Generally fine with the proposal. |
| Samsung | Rel-17 introduced SSSG switching and PDCCH skipping for UE PS. We think those mechanisms are indeed very useful and have to be considered; otherwise, any conclusions may not be valid. Other than that, fine with the Rel-17 evaluation methodology. |
| vivo | Generally, we are supportable to reuse the evaluation methodology provided in TR 38.838. Additional evaluation methodology can be reported by companies, if needed. |
| LGE | We are okay that Rel-17 evaluation methodology for XR power saving captured in TR 38.838 is used as the baseline evaluation methodology for UE power evaluation of Rel-18 SI on XR enhancements. |
| Nokia, NSB | We agree to re-use Rel17 XR SI evaluation methodology to evaluate power saving enhancements. We also propose to down scope or choose the minimum submitted set of the simulation parameters (e.g., traffic parameters) and/or scenarios to be submitted by companies. As quoted above and given in our contribution we would propose to focus following set traffic parameters: Data rate [Mbps]; Packet arrival rate [fps]; PDB [ms]. DL; AR/VR; 30; 60; 10. CG; 30; 60; 15. UL; VR/CG: Pose/control; 0.2; 250; 10. AR: Option 1 (single stream model); 10; 60; 30. In order to facilitate the comparison among companies, we propose to choose at least one scenario to be submitted by all companies: e.g., Dense Urban or InH. Our preference would be Dense Urban. It will not prevent companies submitting other scenarios agreed as baseline in Rel17 XR SI, just give more chances for companies to compare their results. |
| Intel | Support the proposal. Baseline and optional models considered during Rel-17 study should still apply for Rel-18 evaluations. |
| CATT | We support the proposal to use the Rel-17 evaluation methodology for Rel-18 XR study to minimize the duplicated discussion. We had used Rel-15/16/17 power saving techniques for our proposal in UE power saving for XR in Rel-17 based on the evaluation methodology. We should continue it in Rel-18 XR study. |
| OPPO | Agree. |
| SONY | We support the proposal |
| InterDigital | We support FL’s proposal |
| ZTE | Basically we agree to follow the baseline evaluation methodology in TR 38.838, and additional realistic jitter value should be considered. |
| ITRI | We support the proposal. |
| NTT DOCOMO | We support the proposal |
| Spreadtrum | We agree with the proposal. |
| Lenovo | agree |
| CMCC | Support |
| Huawei, HiSilicon | Due to limited TU in for R18 XR, Rel-17 evaluation methodology for XR power saving in TR 38.838 shall be reused. There is no need to re-open discussions on evaluation methodology, which is also not in R18 XR scope. If some companies have strong interest in some new simulation assumptions, it can be left to company report. |
| Moderator | Please check follow-up discussion in the round 2 at Proposal 2-1 and Proposal 2-2. |

Summary of 1st round discussion on Question 2-1:

Companies in general agree to use the Rel-17 evaluation methodology for XR power saving captured in TR 38.838 is used as the baseline valuation methodology for Rel-18 XR power saving study. MTK suggests that companies should evaluate the DL+UL scenario. Nokia suggests focussing on the following set of the traffic parameters:

|  |  | Data rate [Mbps] | Packet arrival rate [fps] | PDB [ms] |
| --- | --- | --- | --- | --- |
| DL | AR/VR | 30 | 60 | 10 |
|  | CG | 30 | 60 | 15 |
| UL | VR/CG: Pose/control | 0.2 | 250 | 10 |
|  | AR: Option 1 (single stream model) | 10 | 60 | 30 |

There was a RAN1 #105-e where this set of parameters was recommended to be used by companies when they are submitting evaluation results.

Agreement When companies are submitting evaluation results to RAN1, it is recommended to submit results at least the following parameters in the below table. - Note 1: This is only intended to have more results from more companies at least for the corresponding configuration. RAN1 agreements regarding baseline vs. optional for simulation scenarios, configurations, parameters, remain the same. - Note 2: Companies are encouraged to submit results for other baseline/optional configurations as much as they can. Data rate [Mbps]; Packet arrival rate [fps]; PDB [ms]. DL; AR/VR; 30; 60; 10. CG; 30; 60; 15. UL; VR/CG: Pose/control; 0.2; 250; 10. AR: Option 1 (single stream model); 10; 60; 30.

Based on the inputs from companies on evaluation methodology, the following updated proposal is made for evaluation methodology. Please directly discuss Proposal 2-1 in the email thread [109-e-R18-XR-02].

[RD2] Proposal 2-1: Rel-17 evaluation methodology for XR power saving captured in TR 38.838 is used as the baseline evaluation methodology for UE power evaluation of Rel-18 SI on XR enhancements

- Companies are suggested to follow Rel-17 joint power saving gain and capacity evaluation for the proposed power saving enhancements

  - Same as Rel-17, capacity is defined as the maximum number of UEs per cell for which the percentage of satisfied UEs is larger than 90%

- Companies are encouraged to submit evaluation results for joint DL and UL evaluation

- When companies are submitting evaluation results to RAN1, it is recommended to submit results at least for the following parameters in the below table which was agreed in RAN1 #105-e during Rel-17

|  |  | Data rate [Mbps] | Packet arrival rate [fps] | PDB [ms] |
| --- | --- | --- | --- | --- |
| DL | AR/VR | 30 | 60 | 10 |
|  | CG | 30 | 60 | 15 |
| UL | VR/CG: Pose/control | 0.2 | 250 | 10 |
|  | AR: Option 1 (single stream model) | 10 | 60 | 30 |

Besides, Nokia suggests defining a common scenario to facilitate the comparison among companies. A similar suggestion was made by CATT to have one baseline DL traffic model with one DRX configuration for the calibration purpose of the UE power saving results from all companies. The following proposal is made to define a common setup for performance calibration across companies. Please directly discuss Proposal 2-2 in the email thread [109-e-R18-XR-02].

[RD2] Proposal 2-2: The following traffic model, scenario and DRX configuration are to be used for calibration of Rel-18 XR UE power saving evaluation results from all companies.

|  |  | Data rate [Mbps] | Packet arrival rate [fps] | PDB [ms] | Scenario | CDRX configuration |
| --- | --- | --- | --- | --- | --- | --- |
| DL | AR/VR | 30 | 60 | 10 | Dense Urban | {DRX cycle, ODT, IAT} = {16, 8, 4}ms |

[RD1] Question 2-2: which of the following features or any other features should be used in the performance reference to compare with the proposed enhancements?

- Rel-15/16 CDRX

- Rel-17 PDCCH adaptation

- Etc.

| Company | Reasons |
| --- | --- |
| MTK | We think Rel-15/16 CDRX should be used as reference for R18 eCDRX for XR. Rel-17 PDCCH adaptation should be used as reference for R18 PDCCH enhancement for XR. |
| Xiaomi | R17 PDCCH skipping/SSSG switching is a more dynamic and flexible feature, and has generally better performance than R15/16 C-DRX. Since we are doing enhancement in R18 for XR, We think it is more meaningful to compare the proposed power saving schemes with Rel-17 PDCCH adaptation. But for eCDRX, if it is majority’s view, the baseline can be Rel-15/16 CDRX |
| Samsung | Fine to consider both CDRX and SSSG switching/PDCCH skipping. Also, possibly for faster progress, fine to consider only the latter as it is anyway more “powerful” than CDRX. |
| Vivo | To evaluate the relative performance compared to the existing power saving features, Rel-15/16 CDRX and Rel-17 PDCCH adaptation should be evaluated together with the proposed CDRX and PDCCH monitoring enhancements. The details of evaluation assumption can be reported by companies as well. |
| LGE | Both Rel-15/16 CDRX and Rel-17 PDCCH adaptation should be used in the performance reference. |
| Nokia, NSB | To motivate further enhancements for power saving, the proposed schemes should be compared to the existing schemes. We propose the following schemes to be used when comparing the enhancements: - Rel-15/16 CDRX (including short and long DRX cycle, MAC CE and DCP) - Rel-17 PDCCH adaptation We understand that it might be time consuming to come up with common configuration for the parametrisation etc. for all the schemes, so it could be possible to leave this to subject to individual company discretion with proper justification. Thus, we would propose to adopt approach where the power saving gain (and capacity impact) of the company selected ‘best Rel-17 baseline’ and the suggested enhancement(s) are compared to the ‘AlwaysOn’ baseline to show that there is meaningful improvement. This seems to be the approach many companies have used already. |
| Intel | Support the proposal. We think WUS is included as part of Rel-16 CDRX as a reference? |
| CATT | We support the proposal with Rel-15 C-DRX, Rel-16 DRX adaptation with WUS, and Rel-17 PDCCH monitoring adaptation. |
| OPPO | In our view, the power saving benefit is per UE, so the performance reference should be the one for what a particular UE currently supports prior to new-release XR. We see both Rel-15/16 CDRX and Rel-17 PDCCH adaptation are equally reasonable choices as the reference method. For the SI TR purpose, it may be ok to use the one reference that performs less well from power saving perspective (given the UE may not implement the other), but we think it is also a reasonable decision to use both references in TR. |
| SONY | At least Rel-15/16 CDRX is used as the baseline. |
| InterDigital | We think Rel-15/16 CDRX can be considered as a reference scheme for comparing the performance of Rel-18 enhancement schemes with respect to the Always-On baseline. |
| Qualcomm | These two can be used to provide the reference performance. A proposed enhancement is supposed to outperform these two legacy features. |
| ZTE | As to evaluation of eCDRX, we think it is reasonable to compare with legacy Rel-15/16 CDRX. |
| ITRI | We support use both Rel-15/16 CDRX and Rel-17 PDCCH adaptation as the performance reference. |
| NTT DOCOMO | At least Rel-15/16 CDRX can be considered for the reference. |
| Spreadtrum | We agree to use Rel-15/16 CDRX and Rel-17 PDCCH adaptation as the reference method for the proposed CDRX and PDCCH monitoring enhancements. |
| Lenovo | The proposed enhancements should be able to provide meaningful gains over any combinations of the existing Rel-15/16/17 power saving features. |
| CMCC | Both Rel-15/16 CDRX and Rel-17 PDCCH adaptation should be considered. |
| Huawei, HiSilicon | Baseline should be AlwaysOn (R17 XR baseline as in TR 38.838). This is enough for simulation results comparison and further discussions, since every company has the same baseline. R17 XR had lengthy discussions on what is baseline, what is optional, etc. There is no need to re-open such discussions. Otherwise, RAN1 may spend a lot of time discussing which parameters of C-DRX/PDCCH adaptations are baseline, which parameters are optional, etc. Rel-15/16 CDRX and Rel-17 PDCCH adaptation can be up to company report when studying the CDRX enhancements and PDCCH monitoring enhancements. |
| Moderator | Please check follow-up discussion in the round 2 at Proposal 2-3. |

Summary of 1st round discussion on Question 2-2:

In the first round discussion, some companies suggested at least using Rel-15/16 CDRX and some other companies suggested at least using Rel-17 PDCCH monitoring adaptations for generating the reference power saving gain. While majority of companies suggested that both Rel-15/16 CDRX and Rel-17 PDCCH monitoring adaptations as performance reference. Based on this, the following proposals is made for further discussion. Please directly discuss Proposal 2-3 in the email thread [109-e-R18-XR-02].

[RD2] Proposal 2-3: Both of the following Rel-15/16/17 features are to be used to generate reference performance and compare with the proposed enhancements for Rel-18 XR power saving evaluations. Power saving gain for the reference performance and for proposed power saving enchantments are calculated w.r.t. AlwaysOn. Companies are supposed to configure these Rel-15/16/17 features to obtain the best reference performance

- Rel-15/16 CDRX including long DRX cycle, short DRX cycle and DRX command MAC CE and DCP

- Rel-17 PDCCH adaptation including PDCCH skipping and SSSG switching

[RD1] Question 2-3: is there any other issue to be discussed for evaluation methodology for UE power saving enhancements of Rel-18 XR SI?

| Company | Comments |
| --- | --- |
| MTK | We think RAN1 to adopt a common UL/DL traffic arrival model would be essential for companies to evaluate the power saving gain with aligned setting under various enhancements. |
| LGE | We think that wake-up signal can be discussed for evaluation methodology, which is not considered in TR38.838. We think WUS enhancement can be an option for XR specific power saving techniques. |
| Nokia, NSB | It would be preferable to evaluate the capacity as the maximum number of UEs for which the percentage of satisfied UEs is larger than ~90% (i.e., using the definition of capacity in capacity evaluation methodology) for each scheme individually to better reflect the capacity impact of different schemes. |
| CATT | It would be useful to have one baseline DL traffic model with one DRX configuration for the calibration purpose of the UE power saving results from all companies |
| Qualcomm | Better to clarify how power saving gain is computed for better alignment of companies’ evaluation results. There are two ways: - For both legacy power saving features and proposed enhancements, PSG is calculated w.r.t. to AlwaysOn, and then compare with each other to show the additional gain by these proposed enhancements PSG of proposed enhancements is calculated w.r.t. power consumption of legacy power saving features |
| ZTE | The jitter value can be a larger value given realistic random packet arrival (Reference [R1-2103278]). Hence, a larger jitter range (e.g., [-8ms, 8ms]) can also be evaluated for enhancements. [Moderator] Thanks for the comments. As always, additional parameters can be assumed and reported by company. Given the majority view, the larger jitter range can be optional rather than a mandatory requirement. |
| Huawei, HiSilicon | No. We suggest RAN1 do not re-open discussion on evaluation methodology, which is also not in R18 XR scope. If some companies have strong interest in some new simulation assumptions, it can be left to company report. |
| Moderator | Companies’ comments on Question 2-3 are addressed in Proposal 2-1/2/3 above. Please check the corresponding proposals. |

# Power Saving Enhancements

In the following, power saving enhancements proposed in companies’ contributions are summarized and categorized based on the Rel-18 XR SI scope and Rel-17 TR 38.838.

CDRX Enhancements

### Periodicity Alignment for CDRX

According to TR 38.838, enhanced CDRX (eCDRX) is a set of enhanced CDRX mechanisms which allow adjusting DRX On duration start time offset to be aligned with each XR DL traffic arrival time, or configure a CDRX cycle pattern with different cycle values instead of only one CDRX cycle, etc.

Table 3 summarizes proposals and evaluation results provided from companies for eCDRX.

Table 3: Proposals and evaluation results for eCDRX

| Company | Views |
| --- | --- |
| Huawei, HiSi | Proposal 1: Further study power saving techniques to address the following XR-specific issues: - Non-integer periodicity - Jitter Proposal 2: When studying the potential enhancements to address XR-specific issues, the following general principles are considered - Principle 1: The solutions should be forward compatible, e.g., applicable to potential new frame rate in the future. - Principle 2: Solution(s) that can address multiple or all the identified issues are more preferred. Proposal 3: Further study the following directions for C-DRX and PDCCH monitoring enhancements to address the non-integer periodicity issue: - Adjusting the start offset of DRX OnDuration and/or the start offset of PDCCH monitoring - Adjusting the periodicity, e.g. configuring a cycle pattern - Multiple C-DRX configurations |
| Spreadtrum | Figure 2. Combine the DRX patterns to match the XR traffic sProposal 1: It is recommended adopting semi-static DRX patterns adjustments or combinations multiple DRX cycles to match XR traffic. Figure 4. Dynamic adjust DRX pattern, such as early trigger or postpone the DRX starting position We also evaluated the power consumption of UE. The power saving gain (PSG) is determined from A: the enhanced CDRX mechanisms which allow adjusting DRX On duration start time offset to be aligned with each XR DL traffic arrival time and B: the power consumption of baseline (AlwaysOn) case; PSG = (B-A)/B×100%. From our evaluation results, the PSG provided by the enhanced CDRX mechanism is shown in Table 1. The specific parameters can be found in the appendix. Table 1: Evaluation results DRX configuration; PSG. DRX cycle (ms); On duration timer (ms); Inactivity timer (ms); Mean (%); Range (%). 16; 10; 4; 13.71; 5.5~27.5. Proposal 2: Dynamic DRX pattern adjustment should be studied, such as early trigger or postponing the DRX starting position. Proposal 3: To solve the issues about the non-integer periodicities and jitter, UE can be configured with multiple DRX configurations associated with different DRX parameters, such as DRX cycle and/or On Duration. |
| vivo | Figure 4 Two DRX configuration approaches to solve the mismatch issue Observation 2: It is beneficial to introduce higher layer based DRX enhancement to align DRX cycle with XR traffic periodicity with the following approaches: - Approach 1: multiple DRX configurations with different drx-StartOffset values - Approach 2: single DRX configuration with multiple DRX ondurations Observation 3: It is beneficial to introduce DCI based DRX enhancement (i.e., adjusting drx-startOffset by a DCI) to align DRX cycle with XR traffic periodicity. Figure 5 Power saving gain of R15/16 DRX and Enhanced DRX Observation 4: For DL XR traffic for FR1 in Indoor Hotspot scenario, compared to R15/16 DRX power saving scheme, enhanced DRX scheme can achieve additional {19.78%~24.71%} power saving gain. Proposal 1: Study DRX enhancements for XR traffic in conjunction with RAN2. |
| ZTE, Sanechips | Observation 2: Dynamic change of CDRX configuration will provide additional signaling overhead and may cause more power consumption on PDCCH monitoring. Proposal 1: Semi-static RRC configuration for CDRX enhancement can be considered to align the CDRX cycle and XR traffic periodicity. • FFS: dynamic update of the CDRX configuration. Proposal 2: RAN 1 to study solutions to match periodicities of XR traffic and CDRX via semi-static RRC configuration, e.g., multiple CDRX configurations, non-integer CDRX cycle, CDRX periodicity resynchronization. Observation 4: In FR1, DL only, InH, high load, VR30, comparing to the scheme(UE always on), following is observed: - for R15 CDRX power saving scheme, the power saving gain is in the range of {6.2%,7.1%} with 3.03% of satisfied UE loss. - for CDRX alignment power saving scheme, the power saving gain is in the range of {33.8%-35.4%} with 3.03% of satisfied UE loss. - for eCDRX with flexible additional PDCCH monitoring, the power saving gain is in the range of {29.8%,31.78%} with 1.51% of satisfied UE loss. Observation 5: In FR1, DL only, InH, high load, CG30, comparing to the scheme(UE always on), following is observed: - for R15 CDRX power saving scheme, the power saving gain is in the range of {4.2%,6.1%} with 7.64% of satisfied UE loss. - for CDRX alignment power saving scheme, the power saving gain is in the range of {20.6%,34.5%} with {7.64%-12.56%} of satisfied UE loss. - for eCDRX with flexible additional PDCCH monitoring, the power saving gain is in the range of {32.4%,34.5%} with 8.34% of satisfied UE loss. Observation 6: In FR1, DL only, InH, high load, VR45, comparing to the scheme(UE always on), following is observed: - for R15 CDRX power saving scheme, the power saving gain is in the range of {7.4%,8.1%} with 4% of satisfied UE loss. - for CDRX alignment power saving scheme, the power saving gain is in the range of {22.4%-23.5%} with 2% of satisfied UE loss. Observation 7: For CDRX alignment power saving scheme, a larger drx-onDurationTimer value will lead to a larger capacity, while the power saving gain will be decreased. Observation 9: In FR1, DL+UL, InH, high load, VR30, ccomparing to the scheme(UE always on), following is observed: - for R15 CDRX power saving scheme, the power saving gain is in the range of {6%,6.65%} with 3.03% of satisfied UE loss. - for CDRX alignment power saving scheme, the power saving gain is in the range of {23.2%,24.6%} with 3.03% of satisfied UE loss. - for eCDRX with flexible additional PDCCH monitoring, the power saving gain is in the range of {20.5%,21.6%} with 1.51% of satisfied UE loss. Observation 10: In FR1, DL+UL, InH, high load, CG30, comparing to the scheme(UE always on), following is observed: - for R15 CDRX power saving scheme, the power saving gain is in the range of {4.5%,6.3%} with 7.64% of satisfied UE loss. - for CDRX alignment power saving scheme, the power saving gain is in the range of {14.7%,23.7%} with {7.64%-12.53%}% of satisfied UE loss. - for eCDRX with flexible additional PDCCH monitoring, the power saving gain is in the range of {21.3%,22.7%} with 8.34% of satisfied UE loss. Observation 11: In FR1, DL+UL, InH, high load, VR45, comparing to the scheme(UE always on), following is observed: - for R15 CDRX power saving scheme, the power saving gain is in the range of {7.22%,7.83%} with 4% of satisfied UE loss. - for CDRX alignment power saving scheme, the power saving gain is in the range of {19.6%,20.5%} with 2% of satisfied UE loss. |
| Ericsson | - (b) Figure 3. Results for (a) mean UE power consumption and (b) fraction of satisfied UEs, for DRX matched to the video traffic period of 16.67 ms with drx-onDurationTimer 10 ms (blue) and standard Long DRX with a cycle of 10 ms and drx-onDurationTimer 8 ms (cyan). Three flows are assumed: one DL video flow (60 fps, PDB of 10 ms), one DL audio flow (10 ms periodicity, PDB of 30 ms), and one UL pose flow (4 ms periodicity, PDB of 10 ms). The duration of drx-InactivityTimer is 2 ms for all configurations. Observation 1 Existing C-DRX solutions do not achieve a good delay/power consumption tradeoff. Solutions that meet the delay requirements for marginal capacity decrease achieve only limited power saving gains of up to 5%, while solutions that save more power increase the delay and thus decrease capacity. Observation 2 In order to enhance C-DRX power saving features, the network needs information about the XR traffic flows: application packet periodicity (generation rate) and periodicity changes for each application traffic flow; application packet periodicity start time per application flow, to understand where to align the DRX cycles; application packet jitter information, e.g. range, per application flow; delay budget (PDB) of the application packet for radio interface, per application flow. Observation 3 Matching the DRX cycle with the non-integer video application packet inter-arrival time achieves power saving gains of up to 5% w.r.t. standard DRX, while maintaining a high (almost ideal) capacity. Proposal 2 Adopt enhancements to C-DRX to achieve a better delay/UE power consumption tradeoff. Proposal 3 Adopt DRX enhancements to match various XR traffic periods, including non-integer (ms) video traffic periods. |
| China Telecom | Proposal1: C-DRX cycles should support XR-specific non-integer period, such as 8.33ms, 16.67ms etc. |
| Sony | Observation 3 – UE power saving can be optimally obtained by arranging the DRX configuration: Proposal 2 – Consider to enable dynamic DRX parameters of the pre-assigned DRX configuration, such as the start of DRX ON duration can be advancing / delaying. |
| Samsung | Proposal #1: Study PDCCH triggered dynamic adaptation on C-DRX configuration, including - C-DRX cycle, - OnDurationTimer, - InActivityTimer, or - Start of ON duration Power saving scheme; Average PSG; Packet success rate; C-DRX configuration (T, ODT, OAT). Always-on; 0; 100%; (0, 0, 0). R15/16 CDRX; 32.3%; 100%; (16, 8, 4). R15/16 CDRX; 42.4%; 97.2%; (16, 6, 4). R15/16 CDRX; 51.5%; 85.9%; (16, 4, 4). Dynamic adaptation on C-DRX cycle; 43.13%; 98.9%; (16, 6, 4) and (18, 6, 4). LP WUS for dynamic start of onDuration; 63.4%; 100%; (16, 2, 1). Observation 6: Dynamic adaptation on C-DRX cycle increase packet success rate by aligning C-DRX cycle with non-integer periodicity of XR traffic. |
| NEC | Proposal 4: Study the mechanism to address the mismatch between DRX cycle and the non-integer periodicity of XR packet arrival time. |
| OPPO | Observation 2: There exists periodicity mismatch issue between traffic arrival and DRX cycle, which may lead to larger scheduling latency Figure 2: Two DRX cycles are applied in a semi-static pattern {17ms, 17ms, 16ms} Figure 3: drx-startoffset change periodically Proposal 1: The following DRX configuration enhancements can be further studied to solve periodicity mismatch issue: - Configure a DRX cycle pattern with two cycle values; - Change drx-startoffset periodically. |
| InterDigital | Proposal 5: Starting point for potential enhancement techniques for power savings can be those identified during Rel-17 SI (e.g. eCDRX, supporting multiple CDRX configurations, enhanced DCI-based PDCCH monitoring adaptations) |
| TCL | Proposal 1: A fixed pattern for DRX within an integer periodicity for XR power saving can be considered. |
| CMCC | Proposal 1. Multiple DRX parameters for one DRX configuration can be considered for XR service, e.g., multiple DRX cycles or multiple DRX ON duration start positions to align the XR DL traffic arrival and DRX ON. Proposal 2. Both explicit and implicit ways can be considered to indicate UE which DRX configuration parameter to be used. - Explicit way: using dedicated signalling to indicate the used DRX parameter. - Implicit way: defining DRX parameter selection rule for each DRX cycle. |
| DOCOMO | Proposal 2: Study following potential CDRX enhancement schemes for XR: - semi-static/dynamic adaptation of CDRX on duration - new candidate values of CDRX cycle |
| Lenovo | In Rel-17 XR study item, it has been suggested to adjust some of the C-DRX cycle start offsets to reduce the additional power consumption. Such an adjustment could be done dynamically or semi-statically. Observation 1: C-DRX cycle start offset adjustment needs to solve the mismatch issue in the presence of frame arrival jitter, otherwise the benefit of such adjustment would be impacted. |
| ITRI | Proposal 1: Further study on the cycle applied to the alignment between C-DRX and XR traffic method. Proposal 2: Further study on how to against the various traffic period and the jitter effect of XR DL service in the “configure a C-DRX cycle pattern with different cycle values” method. |
| ETRI | Proposal 1: To efficiently serve XR traffics having non-integer periodicities, study an extension of C-DRX configuration to guarantee that one DRX cycle or a bunch of DRX cycles is aligned with multiple traffic arrivals. |
| Nokia, NSB | Figure 2 – Example of drift evolution assuming two DRX cycles of duration equal to 16ms (DRX 1) and 17ms (DRX 2). Yellow lines represent the PDB of VR/AR applications. After 16 and 31 cycles, the accumulated gap of configurations DRX 1 and DRX 2 have exceeded the PDB of VR/AR traffic. Observation 3: The non-integer and recurrent periodicity of XR traffic and the integer periodicity of the DRX cycle results in the misalignment between XR traffic and DRX cycle. This misalignment causes a time drift between the XR frame arrivals and the start of the DRX cycles. The drift accumulates over time and eventually the start of the DRX cycle falls outside expected arrival interval of the XR traffic. Proposal 6: We propose to introduce L1/L2 signalling and new IEs to realign the DRX cycle with the XR traffic and control the drift between the DRX cycle and XR traffic. The realignment can be execute synchronously using the parameters of the new IE or asynchronously by the network using the new L1/L2 signalling mechanism. |
| Intel | Proposal 1: RAN1 studies designs to align CDRX ON duration with XR packet arrival. Table 1: Power consumption and capacity performance of enhanced DRX and legacy C-DRX. Results are presented for VR and CG in Dense Urban scenario, 4 UEs, DL only with SU-MIMO scheduler. DRX (Cycle, IAT, ON). Power Saving Scheme; Jitter ON; Jitter OFF. Power consumption (PC); #satisfied UEs per cell/ #UEs per cell PDB 10ms; #satisfied UEs per cell/ #UEs per cell PDB 15ms; Power consumption (PC); #satisfied UEs per cell/ #UEs per cell PDB 10ms; #satisfied UEs per cell/ #UEs per cell PDB 15ms. Baseline; Optional; Baseline; Optional. Mean PC; PC of 5%-tile UE in PC CDF; PC of 50%-tile UE in PC CDF; PC of 95%-tile UE in PC CDF; Mean PC; PC of 5%-tile UE in PC CDF; PC of 50%-tile UE in PC CDF; PC of 95%-tile UE in PC CDF. Always On; 116.76; 108.24; 116.13; 127.31; 3.93/4; 3.95/4; 115.19; 107.43; 114.42; 126.9; 3.97/4; 3.99/4. Legacy CDRX; DRX (8,6,6); 111.36; 98.98; 109.41; 127.46; 3.72/4; 3.81/4; 109.67; 98.19; 107.4; 126.03; 3.80/4; 3.89/4. DRX (16,4,14); 112.99; 102.69; 111.63; 126.18; 3.82/4; 3.89/4; 111.58; 101.94; 109.78; 125.95; 3.84/4; 3.93/4. w/ eDRX; DRX (8,6,6); 108.36; 98.04; 107.36; 121.26; 3.83/4; 3.9/4; 103.05; 93.34; 102.26; 116.97; 3.97/4; 3.99/4. DRX (16,4,14); 111.43; 102.32; 110.63; 123.13; 3.89/4; 3.93/4; 105.55; 97.76; 104.77; 117.26; 3.97/4; 3.99/4. |
| Qualcomm | Observation 2: By adopting the rational number in DRX cycle and adding the floor operations in DRX formulas, DRX cycles could be adjusted to address the mismatch between DL traffic arrival times and DRX on-duration start times. Proposal 1: Introduce non-integer rational numbers in short/long DRX cycles and add floor operations in DRX formulas for CDRX enhancement in Rel.18 with minimal spec impact. Table 1: Enhanced CDRX, FR1, DL+UL joint evaluation, DU, VR30 Power saving scheme; CDRX cycle (ms); ODT (ms); IAT (ms); Load H/L; #UE /cell; floor (Capacity); % of DL + UL satisfied UE; Mean PSG of all UEs (%). Always On; H; 11; 11; 97.14%; 0%. Rel15/16 CDRX; 16; 12; 12; H; 11; 11; 69.87%; 1.78%. eCDRX; 16/17/17; 10; 10; H; 11; 11; 82.86%; 9.43%. eCDRX; 16/17/17; 12; 12; H; 11; 11; 94.20%; 4.51%. Genie; H; 11; 11; 97.14%; 24.62%. Table 2: Enhanced CDRX, FR2, DL-only evaluation, InH, VR30 Power saving scheme; CDRX cycle (ms); ODT (ms); IAT (ms); Load H/L; #UE /cell; floor (Capacity); % of DL satisfied UE; Mean PSG of all UEs (%). Always On; H; 7; 7; 90%; 0.00%. Rel15/16 CDRX; 16; 4; 4; H; 7; 7; 0%; 28.44%. Rel15/16 CDRX; 16; 8; 8; H; 7; 7; 50%; 9.64%. Rel15/16 CDRX; 16; 8; 16; H; 7; 7; 65%; 4.10%. eCDRX; 16/16/15; 4; 4; H; 7; 7; 27%; 25.10%. eCDRX; 16/16/15; 8; 8; H; 7; 7; 84%; 8.28%. eCDRX; 16/16/15; 8; 16; H; 7; 7; 88%; 2.43%. Genie; H; 7; 7; 90%; 73.5%. Figure 4 SFN wraparound mismatch between XR DL traffic (60fps) and enhanced CDRX start offset Figure 5 SFM modulo addressing SFM wraparound mismatch problem Observation 3: In R15/16/17 DRX operation, the mismatch happens between DRX on-duration times and XR DL traffic arrivals when SFN returns to 0 every hyper frame 10,240ms. This would lead to XR capacity loss due to larger latency and/or larger UE power consumption to keep the same latency performance. Proposal 2: Introduce a timing reference value of SFN_M which can be updated as SFN_M = (SFN_M + 1) mod M when SFN is changed. The modulo number M can be configured as 1000 for XR and CG applications. This is required for CDRX alignment with XR periodicities. Observation 4: By using SFN_M instead of SFN in DRX formulas, the SFN wraparound issue can be resolved with the least spec impact. Proposal 3: For CDRX cycle alignment with XR periodicity, adopt the TPs in the Appendix. |

During Rel-17 SI, enhanced CDRX was used to refer to the set of techniques that align the CDRX On Duration with XR traffic occasion in time. Companies have proposed to achieve the alignment by both semi-static and dynamic approaches. Below is a summary of these proposals.

Configuration of CDRX cycle pattern

[Huawei, HiSi], [Spreadtrum], [vivo], [ZTE, Sanechips], [NEC], [OPPO], [TCL], [CMCC], [DOCOMO], [Lenovo], [ITRI], [ETRI], [Nokia, NSB] and [Intel] proposed to use a semi-static pattern of CDRX cycles to align the DRX On Duration with XR traffic periodicity. There are multiple options to realize this:

- Configure a periodic pattern of consecutive DRX cycles with two DRX cycle values, e.g., {17, 17, 16}ms

- Periodically change the drx-startoffset over DRX cycles

- Configure a single DRX configuration with multiple On Durations each matching with a XR traffic occasion

- Use multiple small CDRX cycles to compose a longer CDRX cycle that align with multiple consecutive XR traffic occasions in time

New non-integer CDRX cycle

[ZTE, Sanechips], [Ericsson], [China Telecom], [NEC], [DOCOMO] and [Qualcomm] proposed to introduce the new non-integer CDRX cycle (e.g., 50/3 ms) that directly matches with the XR traffic periodicity. The existing formula is modified to determine the CDRX start offset in time unit of slot or ms.

Timing reference to resolve SFN wraparound mismatch

[Qualcomm] brought up the SFN wraparound mismatch issue when CDRX cycle is matched to XR traffic periodicity. The issue is because 1024 frames do not contain an integer multiple of CDRX cycles. A timing reference value SFN_M is proposed to resolve the issue with SFN_M = (SFN_M + 1) mod M as SFN increments, e.g., M = 1000 for XR and CG applications.

Multiple active CDRX configurations

[Huawei, HiSi], [Spreadtrum], [vivo], [ZTE, Sanechips] and [ETRI] proposed to use multiple active CDRX configurations to transfer the same XR video data flow. These CDRX configurations have the same periodicity (e.g., 50ms) and staggered start offsets (e.g.., {0, 17, 34}ms). As a result, assemble of On Durations across the multiple CDRX configurations will match with the XR traffic periodicity.

Dynamic adjustment of DRX start offset

[Huawei, HiSi], [Spreadtrum], [vivo], [Sony], [Samsung], [NEC], [CMCC], [DOCOMO], [Lenovo], [Nokia, NSB] and [Qualcomm] proposed to dynamically adjust the start offset and/or cycle of one or multiple upcoming DRX cycles to align the DRX On Duration with XR traffic occasions in time. The adjustment is provided by L1 signaling in DCI or L2 signaling received in or outside the DRX active time.

### Jitter Handling for CDRX

Jitter causes extra delay to XR traffic if data arrives before the DRX On Duration start time or extra power consumption if data arrives after the DRX On Duration start time. For existing CDRX, jitter can be compensated if long DRX On Duration is configured to cover the entire jitter range. In this meeting, companies proposed enhancements for better power saving and latency performance.

Table 4 captures companies’ views on jitter handling for CDRX.

Table 4: Proposals and views on jitter handling for CDRX

| Company | Views |
| --- | --- |
| Huawei, HiSilicon | Figure 3. Jitter distribution Observation 1: C-DRX OnDuration or the length of search space Duration needs to be large enough to cover the whole or most parts of the jitter range, resulting in extra power consumption. Proposal 4: Further study the following mechanisms to allow skipping unnecessary PDCCH monitoring caused by jitter: - Configuring a PDCCH monitoring pattern within DRX OnDuration or Search Space duration to match jitter distribution; - Skipping unnecessary PDCCH monitoring once the XR frame is received correctly; - UE monitors PDCCH sparsely before the data arrival and monitors PDCCH densely once the data arrives, e.g., implicit SSSG switching based on data arrival. |
| Spreadtrum | Observation 2: Considering the impact of jitter, the XR DL traffic arrival time is random. Early DL traffic arrival will increase the packet transmission delay, and late DL traffic arrival will increase the UE power consumption. From our evaluation results, the PSG provided by the enhanced CDRX mechanism is shown in Table 1. The specific parameters can be found in the appendix. Table 1: Evaluation results DRX configuration; PSG. DRX cycle (ms); On duration timer (ms); Inactivity timer (ms); Mean (%); Range (%). 16; 10; 4; 13.71; 5.5~27.5. Proposal 2: Dynamic DRX pattern adjustment should be studied, such as early trigger or postponing the DRX starting position. Proposal 3: To solve the issues about the non-integer periodicities and jitter, UE can be configured with multiple DRX configurations associated with different DRX parameters, such as DRX cycle and/or On Duration. Proposal 5: Rel-17 PDCCH skipping can be a starting point to solve XR jitter effects. |
| vivo | Leveraging R17 search space set group (SSSG) switching indication, as depicted in Figure 8 (a) and Figure 8 (b), before a reference time, i.e., the -jitter boundary of the next traffic burst, e.g., frame, UE switch to the default SSSG with sparse PDCCH monitoring periodicity via the expiry of searchSpaceSwitchTimer-r17 or another explicitly indication. Once the UE detect the scheduling PDCCH, it can accordingly switch to a dense SSSG. And with the successful transmission of the last packet of the frame, a PDCCH skipping indication will be sent to UE to skip the remaining PDCCH monitoring until the -jitter boundary of the next frame. Observation 5: SSSG switching mechnasim can be used for jitter handling. Figure 10 Jitter handling method based on LP-WUS Proposal 5: Study LP-WUS for PDCCH monitoring reduction. |
| ZTE, Sanechips | Figure 3 Flexible additional PDCCH monitoring Observation 4: In FR1, DL only, InH, high load, VR30, comparing to the scheme(UE always on), following is observed: - for R15 CDRX power saving scheme, the power saving gain is in the range of {6.2%,7.1%} with 3.03% of satisfied UE loss. - for CDRX alignment power saving scheme, the power saving gain is in the range of {33.8%-35.4%} with 3.03% of satisfied UE loss. - for eCDRX with flexible additional PDCCH monitoring, the power saving gain is in the range of {29.8%,31.78%} with 1.51% of satisfied UE loss. Observation 5: In FR1, DL only, InH, high load, CG30, comparing to the scheme(UE always on), following is observed: - for R15 CDRX power saving scheme, the power saving gain is in the range of {4.2%,6.1%} with 7.64% of satisfied UE loss. - for CDRX alignment power saving scheme, the power saving gain is in the range of {20.6%,34.5%} with {7.64%-12.56%} of satisfied UE loss. - for eCDRX with flexible additional PDCCH monitoring, the power saving gain is in the range of {32.4%,34.5%} with 8.34% of satisfied UE loss. Observation 8: Compared with CDRX alignment power saving scheme, eCDRX with flexible additional PDCCH monitoring power saving scheme can provide larger capacity under similar power saving gain. Observation 9: In FR1, DL+UL, InH, high load, VR30, ccomparing to the scheme(UE always on), following is observed: - for R15 CDRX power saving scheme, the power saving gain is in the range of {6%,6.65%} with 3.03% of satisfied UE loss. - for CDRX alignment power saving scheme, the power saving gain is in the range of {23.2%,24.6%} with 3.03% of satisfied UE loss. - for eCDRX with flexible additional PDCCH monitoring, the power saving gain is in the range of {20.5%,21.6%} with 1.51% of satisfied UE loss. Observation 10: In FR1, DL+UL, InH, high load, CG30, comparing to the scheme(UE always on), following is observed: - for R15 CDRX power saving scheme, the power saving gain is in the range of {4.5%,6.3%} with 7.64% of satisfied UE loss. - for CDRX alignment power saving scheme, the power saving gain is in the range of {14.7%,23.7%} with {7.64%-12.53%}% of satisfied UE loss. - for eCDRX with flexible additional PDCCH monitoring, the power saving gain is in the range of {21.3%,22.7%} with 8.34% of satisfied UE loss. - Proposal 3: Enhancements of handling jitter through introducing flexible additional PDCCH monitoring, or introducing a new wake up signal can be studied. |
| Ericsson | Figure 6. Illustration of two-stage DRX. - (b) Figure 8. Results for (a) mean UE power consumption and (b) fraction of satisfied UEs, for multi-flow & two-stage DRX (magenta) and a standard DRX configuration with a Long cycle of 10 ms (cyan). Three flows are assumed: one DL video flow (30 fps, PDB of 10 ms), one DL audio flow (10 ms periodicity, PDB of 30 ms), and one UL pose flow (4 ms periodicity, PDB of 10 ms). The duration of drx-InactivityTimer is 2 ms in all cases. For multi-flow DRX, one DRX configuration matches the video traffic with two-stage DRX {outer DRX cycle equals video traffic period, 10 ms outer onDurationTimer, 4 ms inner DRX cycle, 2 ms inner onDurationTimer} and the second DRX configuration matches the audio period with drx-onDurationTimer 2 ms. Observation 6 When multiple XR flows are present and one of them suffers from jitter, two-stage DRX with multi-flow DRX saves 13% UE power w.r.t. the existing DRX, while achieving a nearly equal capacity. Proposal 5 Adopt the two-stage DRX solution to handle jitter for quasi-periodic XR traffic flows. |
| China Telecom | Proposal 2: Specify a dynamic or semi-static C-DRX mechanism to adjust the starting time of C-DRX on duration adapted to the random jitter. Proposal 3: The start-offset indication monitoring should be supported at least during C-DRX On Duration. Besides, start-offset indication monitoring can also be supported during DRX dormancy, when DCP is enabled. Proposal 4: Option 1 with single C-DRX On Duration or DRX Cycle affected should be supported, while option 3 needs FFS. |
| Sony | Observation 4 – Adjusting only the length of the ON duration can result either in a long delay or unnecessary increased power consumption. Proposal 3 – Consider dynamic DRX configuration to delay or advance DRX ON duration based on the experienced jitter. Proposal 4 – Two level DRX configuration with finer granularity inactive indication during DRX ON can be considered for further power saving. |
| Samsung | Observation 1: Current C-DRX configuration does not fit well for XR traffic with i) non-integer periodicity, and ii) varying packet arrival time due to jitter effect. Proposal #1: Study PDCCH triggered dynamic adaptation on C-DRX configuration, including - C-DRX cycle, - OnDurationTimer, - InActivityTimer, or - Start of ON duration Observation 7: LP WUS for dynamic start of ON duration allows short onDurationTimer and inactivityTimer, and thus provides large PSG and high packet success rate. Proposal 2: Study sequence-based LP WUS using Rel-17 signals to indicate start of ON duration for C-DRX. |
| NEC | Proposal 2: Support DCI based dynamic adaptation of XR specific PDCCH monitoring offset such as ‘jitter’. Proposal 5: Study the enhancement of the starting time of the DRX on-duration and the WUS monitoring window to get good balance between the power saving and packet delay. |
| OPPO | Proposal 3: An additional DRX-on duration time can be dynamically indicated to UE to solve jitter issue. |
| TCL | Proposal 2: An additional PDCCH monitor occasion can be configured on DRX-OFF state. |
| CMCC | Proposal 3. A new "early wake-up indication" field in DCI format 2_6 to indicate UE whether to wake up early from DRX ON can be considered for XR service. |
| LGE | To minimize the additional power consumption with this approach, the extension may need to be conditioned on the event that the UE does not receive any data during the periodic DRX OnDuration, or the UE receives a triggering indication. Proposal 2: Study PS enhancement techniques to tackle the jitter in arrival time of XR packets. Proposal 5: Consider WUS outside DRX Active Time as well as the DCI inside DRX Active Time as a candidate for indicating the PDCCH monitoring adaptation. |
| ETRI | Proposal 5: Consider WUS outside DRX Active Time as well as the DCI inside DRX Active Time as a candidate for indicating the PDCCH monitoring adaptation. |
| MediaTek | Proposal 5: The UE to stop the On-Duration-Timer at the first data arrival and to rely on the Inactivity Timer while waiting for the next packet of the same media frame. |
| Qualcomm | Observation 7: Early CDRX reduces the active part of the CDRX cycle without impacting packet latency resulting in UE power saving. Observation 8: The power savings achieved by Early CDRX on top of eCDRX are 4.9% and 10.4% for 60 and 120 FPS respectively for [-4, +4]ms jitter. Similarly, for [-5, +5] ms jitter, these savings are 7.8% and 13.3% respectively. Observation 9: Compared to Early CDRX, Release 17 PDCCH skipping with 3 skip durations scheme requires 12.9% and 22.2% additional PDCCH slots per frame for [-4, +4] ms and [-5, +5] ms jitter respectively resulting in higher power consumption. Proposal 5: UE can transition to the CDRX inactive state after reception of the DL frame. The transition can happen within the configured active part of the CDRX cycle if the Early CDRX conditions are satisfied. |

Various jitter handling techniques were proposed by companies to better align the UE reception time of XR traffic with the random data arrival time when data becomes available at the gNB. Due to the random nature of jitter, the proposals generally rely on certain dynamic mechanism to detect and compensate the jitter effect. In the meanwhile, the proposals have tried to minimize the power overhead when the XR data is not available to be received by the UE yet and to minimize the extra delay when the XR data is already available. Below is a summary of these proposals.

PDCCH monitoring occasions matched to jitter distribution

[Huawei, HiSi] proposed to configure a non-uniform pattern of PDCCH monitoring occasions within the DRX On Duration or search space set duration that is matched the statistical distribution of jitter. Then UE monitors PDCCH more frequently around the centre of jitter distribution (i.e., expected data arrival time) and less frequently in the tail of jitter distribution.

Dynamic adjustment of DRX starting time

[Spreadtrum], [China Telecom], [Sony], [Samsung], [NEC], [OPPO] and [LGE] proposed to dynamically advance or delay the DRX starting time based on UE experienced jitter. It intends to realize a fast adjustment rather than using the RRC reconfiguration. Note that a similar proposal for dynamic adjustment of DRX start offset was also proposed to resolve the misalignment between DRX On Duration and XR traffic occasion under the eCDRX topic.

Jitter handling by WUS

[vivo], [ZTE, Sanechips], [Samsung], [NEC], [CMCC], [LGE] and [ETRI] proposed to compensate the jitter effect by using wake-up signal (WUS). This can be considered as a special realization of the general “dynamic adjustment of DRX starting time” technique above. Some proposals suggested to design a low power (LP) such as sequence based WUS so that UE overall power consumption for jitter handling is reduced. DRX starting time is adjusted by a configured offset from where WUS is detected or is indicated by the WUS payload content. Note that the LP-WUS design is in the scope of the Rel-18 SI on low power WUS and receiver for NR.

Adjustment of CDRX on duration conditioned on data reception

Three techniques are proposed for the adjustment of CDRX on duration conditioned whether UE has received data in the CDRX cycle. They belong to two types, the additional On Duration for PDCCH monitoring and early switching to DRX OFF.

For the additional On Duration technique, [ZTE, Sanechips] [TCL] [LGE] proposed UE monitors PDCCH in additional time duration if data is not received in the On Duration. The On Duration is not necessarily increased first to cover the range of jitter by CDRX configuration. The additional time duration is used to receive the late XR data packet caused by jitter and may not be adjacent to the On Duration.

For the early switching to DRX OFF technique, [MediaTek] proposed early termination of the On-Duration-Timer and [Qualcomm] proposed early CDRX for PDCCH monitoring in presence of jitter. The On Duration is increased first to cover the range of jitter by CDRX configuration. The On Duration then can be much longer than the Inactivity Timer. In [MediaTek] proposal, UE stops the On-Duration-Timer at the first data arrival and relies on the Inactivity Timer while waiting for the next packet of the same media frame. In [Qualcomm] proposal, UE performs early switching to DRX OFF when DL packets are served and certain conditions (minimum offset to next CDRX On Duration, inactivity duration, number of received grants) are satisfied.

Two stage/level CDRX configuration

Two companies [Ericsson] and [Sony] proposed similar mechanism for reduced PDCCH monitoring within CDRX On Duration when the On Duration is enlarged to cover the jitter range. In the proposals, the existing DRX configuration is called the Outer/1st level DRX cycle and is aligned with XR traffic period. On top of that, introduce Inner/2nd level DRX cycles within On Duration of the Outer/1st level DRX. The UE monitors the PDCCH only during the On Duration time of the Inner/2nd level cycles.

Implicit SSSG switching conditioned on data reception

[Huawei, HiSi] and [vivo] proposed the implicit SSSG switching from sparse to dense PDCCH monitoring after the random data arrival due to jitter is detected within DRX On Duration.

### Multiple CDRXs for Multiple Flows

XR traffic is characterized by multiple data flows with different periodicities, data rate and QoS requirements. Companies proposed to adopt multiple active CDRX configurations to support multi-flow XR traffic.

Table 5 captures companies’ views on multiple CDRX configurations for the support of multiple XR traffic flows.

Table 5: Proposals and views on multiple CDRX for multiple XR traffic flows

| Company | Views |
| --- | --- |
| Ericsson | Figure 4. Illustration of multi-flow DRX with two configurations, each matching a DL traffic flow: the cycle and drx-onDurationTimer in green follow the video flow (16.67 ms period) and the DRX configuration in magenta follows the audio flow (10 ms period). The video traffic experiences a random jitter of ±4 ms with a truncated Gaussian distribution around the mean traffic period. - (b) Figure 5. Results for (a) mean UE power consumption and (b) fraction of satisfied UEs, for multi-flow DRX with two configurations (red) and a standard DRX configuration with a cycle of 10 ms (cyan). Three flows are assumed: one DL video flow (60 fps, PDB of 10 ms), one DL audio flow (10 ms periodicity, PDB of 30 ms), and one UL pose flow (4 ms periodicity, PDB of 10 ms). The duration of drx-InactivityTimer is 2 ms for all configurations. For multi-flow DRX, one DRX configuration matches the video period with drx-onDurationTimer 10 ms and the second configuration the audio period with drx-onDurationTimer 2 ms. Observation 4 Multiple simultaneous DRX configurations, each matching a traffic flow, achieve power saving gains 9% w.r.t. standard DRX, while maintaining a low delay and almost ideal capacity, especially if the PDBs are shorter than the DRX cycles. Proposal 4 Adopt multiple simultaneous DRX configurations for XR traffic with multiple (quasi-)periodic XR traffic flows. |
| China Telecom | Proposal 5: Double period or multiple periods, or sequence based DRX On Duration/DRX Cycle should be studied for XR-specific power saving. |
| Sony | Figure 1 - Illustration of multi DRXs operation Proposal 1 - Application aware multi C-DRX configuration can be considered as an enhanced C-DRX mechanism to further reduce power consumption in UEs with XR traffic. |
| NEC | Proposal 6: Study the enhancement of multiple DRX configurations to better support XR services with multiple traffic flows. |
| InterDigital | Proposal 5: Starting point for potential enhancement techniques for power savings can be those identified during Rel-17 SI (e.g. eCDRX, supporting multiple CDRX configurations, enhanced DCI-based PDCCH monitoring adaptations) |
| Apple | Observation: Rel-18 NR enhancements for XR should be motivated by XR services’ traffic characteristics, especially the multiple data flow aspects. Proposal 3-1: consider the support of multiple DRX configurations to support multiple data flows in DL/UL with different periodicities. |
| LGE | Proposal 3: Study enhancements to the secondary DRX group taking into account the XR traffic characteristics such as non-integer periodicity and jitter. |
| ETRI | Proposal 2: To efficiently serve multi-flow XR traffic, study feasibility and benefit of DRX operation based on multiple DRX layers (i.e., multiple DRX configurations) within a DRX group. |
| Intel | Proposal 2: RAN1 studies multiple active DRX configurations to support XE media with multiple flows. |

Companies proposed CDRX enhancements with the intention to better match the PDCCH monitoring ON/OFF duration determined by CDRX with the multiple flow traffic so that UE power consumption can be reduced with no or marginal impact to capacity. Below is a summary of these proposals.

Multiple active CDRX configurations

[Ericsson], [Sony], [NEC], [InterDigital], [Apple], [LGE], [ETRI] and [Intel] proposed to configure multiple simultaneous DRX configurations for multiple XR traffic flows.

Please note that the multi-CDRX is also proposed to resolve the misalignment between CDRX cycles and XR traffic. The difference is that to support the multi-flow XR traffic, the multiple CDRX configurations may generally have different periodicities. While for periodicity alignment, the multiple CDRX configurations have same periodicity but staggered start offsets.

Multiple periods or sequence based DRX

[China Telecom] proposed to configure multiple periods in a sequence for the DRX On Duration and cycle. This proposal can be a more general mechanism for the “Configuration of CDRX cycle pattern” under the eDRX topic. To support multiple XR traffic flows, the configuration of DRX On Duration and cycle could be more flexible.

### Other CDRX Enhancements

Table 6 captures the other CDRX related enhancements for XR-specific power saving.

Table 6: Proposals and views on the other CDRX enhancements

| Company | Views |
| --- | --- |
| vivo | To address this problem, we need to enhance specification so that DRX timers do not decrement during UL slots. Figure 6 DRX configuration in TDD deployment Proposal 2: Study skipping a CG transmission if it falls outside DRX active time or falls into PDCCH skipping duration for XR traffic. Proposal 3: Study enhanced DRX scheme to prevent DRX timers decrementing over UL slots. |
| China Telecom | UE will receive all SPS PDSCHs during C-DRX On Duration, but only receive part of the SPS PDSCHs (e.g., with high priority or XR-specific) during C-DRX dormancy. Proposal 6: C-DRX enhancements for SPS PDSCHs of different priorities should be studied. |
| Samsung | Proposal #1: Study PDCCH triggered dynamic adaptation on C-DRX configuration, including - C-DRX cycle, - OnDurationTimer, - InActivityTimer, or - Start of ON duration |
| Nokia, NSB | (a) CG in FR1 at 30Mbps with X=99%.; (b) AR/VR in FR1 at 30Mbps with X=99%.. Figure 4 – DL capacity of ADRX versus CDRX for CG traffic (Video Single-Stream) and AR/VR (Video Single-Stream) in Dense Urban deployment in FR1 with X=99% of frames received within PDB and cells evenly loaded. Figure 5 – Power saving gain of ADRX versus CDRX for CG traffic (Video Single-Stream) and AR/VR (Video Single-Stream) in Dense Urban deployment in FR1 with X=99% of frames received within PDB and cells evenly loaded. Table 4 – Evaluation of enhanced ADRX enhancement for {Dense Urban, CG, DL Only, 30Mbps, FR1}. Option 1: PS gain computed with All UEs. Power Saving Scheme; Power Saving Gain (PSG) compared to ‘Always ON’; #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS; Capacity with PS [#satisfied UEs/cell with PS]; Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS. Baseline; Optional. Mean PS gain; PS gain of 5%-tile UE in PSG CDF; PS gain of 50%-tile UE in PSG CDF; PS gain of 95%-tile UE in PSG CDF. Always ON; -; -; -; -; - / 8; 8; -. CDRX(16,8,8); 13.6; 16.8; 14.4; 8.3; 4 / 8; 6; 50%. ADRX(16,1,16); 7.5; 8.1; 7.2; 3.1; 5.6 / 8; 4; 70%. Table 5 – Evaluation of enhanced ADRX enhancement for {Dense Urban, AR/VR, DL Only, 30Mbps, FR1}. Option 1: PS gain computed with All UEs Power Saving Scheme; Power Saving Gain (PSG) compared to ‘Always ON’; #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS; Capacity with PS [#satisfied UEs/cell with PS]; Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS. Baseline; Optional. Mean PS gain; PS gain of 5%-tile UE in PSG CDF; PS gain of 50%-tile UE in PSG CDF; PS gain of 95%-tile UE in PSG CDF. Always ON; -; -; -; -; - / 6; 6; -. CDRX(16,8,8); -; -; -; -; 0 / 6; 0; 0%. ADRX(16,1,16); 7.5; 8.1; 7.2; 3.1; 4 / 6; 4; 66%. Observation 4: A single CDRX configuration for multiple XR applications, which have different QoS characteristics, results in suboptimal performance. For example, the CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(16, 8, 8) cannot satisfy any UE when PDB changes from 15ms to 10ms. Observation 5: The possibility to quickly reconfigure the parameters of the CDRX configuration or selecting a CDRX configuration among a set of pre-configured CDRX configurations like in Adaptive DRX (ADRX) reduces the capacity loss while maintaining a 10% power saving gain with respect to the baseline “always ON”. Proposal 7: We propose to extend DRX configuration to have multiple adjustable parameters and/or configurations as well as to introduce L1/L2 signalling to quickly change the CDRX configuration. Observation 6: When proposing power saving enhancements, capacity loss should be minimized. Table 6 – Evaluation of enhanced TA-WUS enhancement for {Dense Urban, CG, DL Only, 30Mbps, FR1}. Option 1: PS gain computed with All UEs. Power Saving Scheme; Power Saving Gain (PSG) compared to ‘Always ON’; #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS; Capacity with PS [#satisfied UEs/cell with PS]; Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS. Mean PS gain. Always ON; -; - / 8; 8; 96%. CDRX(8,0,4); 16.7; 6.72 / 8; 7; 84%. CDRX(8,0,4) + 1 WUS; 17; 5.6 / 8; 6; 70%. CDRX(4,0,2); 18; 7.04 / 8; 7; 88%. CDRX(4,0,2) + 1 WUS; 20; 6.32 / 8; 7; 79%. CDRX(4,0,2) + 2 WUS; 22.5; 6.08 / 8; 7; 76%. Table 7 – Evaluation of enhanced TA-WUS enhancement for {Dense Urban, AR/VR, DL Only, 30Mbps, FR1}. Option 1: PS gain computed with All UEs. Power Saving Scheme; Power Saving Gain (PSG) compared to ‘Always ON’; #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS; Capacity with PS [#satisfied UEs/cell with PS]; Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS. Mean PS gain. Always ON; -; - / 6; 6; -. CDRX(8,0,4); 18; 3.66 / 6; 5; 61%. CDRX(8,0,4) + 1 WUS; 20; 2.7 / 6; 4; 45%. CDRX(4,0,2); 20; 4.98 / 6; 5; 83%. CDRX(4,0,2) + 1 WUS; 22.5; 3.9 / 6; 5; 65%. CDRX(4,0,2) + 2 WUS; 25.3; 3.9 / 6; 5; 64%. Observation 7: Dense and short DRX configurations reduce capacity loss with respect to R17 baseline of XR evaluation (UE “always ON”). Therefore, dense and short DRX configurations represent good candidates as baseline for power saving evaluation. Observation 8: While dense and short DRX configurations reduce capacity loss with respect to R17 baseline of XR evaluation (UE “always ON”), they also limit the power saving gain since the UE can enter only micro sleep mode. Observation 9: When a frame is successfully delivered to the UE, future On Durations of short and dense DRX cycles can be skipped when no traffic is expected. Observation 10: Traffic awareness, like knowledge of the boundaries of the XR frame (i.e., first and last PDU in a PDU Set), is beneficial to use the Wake-Up signal to skip the On Durations when no traffic is expected to be transmitted. Proposal 8: We propose to evaluate enhancements, which might be on traffic awareness ,to skip the On Durations when no traffic is expected to be transmitted and the XR frame has been successfully delivered. |
| Qualcomm | Observation 5: Using fixed C-DRX parameters may have negative impact on delays (PDB) and power consumption for XR traffic. Proposal 4: For XR, consider studying methods to dynamically adapt the C-DRX parameters to the traffic bursts |

Below is a summary of the remaining proposals related to CDRX enchantments for UE power saving.

Dynamic adaptation on CDRX configuration

[Samsung], [Nokia, NSB] and [Qualcomm] proposed the dynamic adaptation on general CDRX configuration parameters including CDRX cycle, OnDurationTimer, InActivityTimer, start offset etc. [Nokia, NSB] proposed to extend CDRX configuration to have multiple adjustable parameters to facilitate the dynamic adaptation. Note that dynamic adaptation of CDRX configuration focused on start offset adjustment is also proposed under the eCDRX topic for the alignment with XR traffic periodicity and under the jitter handling topic.

UL slot handling for CDRX timers

[vivo] proposed to not decrement the various CDRX timers during UL slots based on the observation that when much short DRX timers are configured for XR service due to stringent latency and high data rate, overlapping between UL slots and the CDRX timers leads to less PDSCH scheduling opportunities.

SPS and CG skipping for CDRX

[China Telecom] proposed to support CDRX dormancy for the configured SPS PDSCHs. UE receives all SPS PDSCHs during On Duration, and only receives part of the SPS PDSCHs (e.g., with high priority or XR-specific) during CDRX dormancy. [vivo] proposed to skip CG transmission outside DRX active time or within PDCCH skipping duration for XR traffic.

Traffic aware WUS

[Nokia, NSB] proposed the traffic aware WUS for UE to skip the CDRX On Duration when XR frame has been successfully delivered.

### Summary

Evaluation results related to CDRX enhancements are collected in Tables 3.1-1 to 3.1-10 below.

Table 3.1-1 CDRX related power saving evaluation results: FR1, DL only, Indoor Hotspot, VR 30Mbps

| Tdoc # | Power Saving Scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Load H/L | avg # UEs/Cell | floor (Capacity) | % of satisfied UE | Mean PSG of all UEs | Mean PSG of satisfied UEs | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2203585 | Always On | - | - | - | L | 5 | 10 | 100% | - | - |  |
| R1-2203585 | R15/16 DRX | 16 | 14 | 4 | L | 5 | 10 | 100% | 3.67% | 3.67% |  |
| R1-2203585 | eCDRX | 16 | 4 | 4 | L | 5 | 10 | 100% | 28.38% | 28.38% |  |
| R1-2203585 | Always On | - | - | - | H | 10 | 10 | 92.50% | - | - |  |
| R1-2203585 | R15/16 DRX | 16 | 14 | 4 | H | 10 | 10 | 91.81% | 3.24% | 3.46% |  |
| R1-2203585 | eCDRX | 16 | 4 | 4 | H | 10 | 10 | 91.25% | 23.84% | 24.11% |  |
| R1-2203606 | Always On | - | - | - | H | 11 | 11 | 93.18% | - | - |  |
| R1-2203606 | R15 CDRX | 10 | 8 | 4 | H | 11 | 11 | 90.15% | 6.2% | 7.1% |  |
| R1-2203606 | eCDRX | 16 | 6 | 4 | H | 11 | 11 | 90.15% | 33.8% | 35.4% |  |
| R1-2203606 | eCDRX with flexible additional PDCCH monitoring | 16 | 6 | 4 | H | 11 | 11 | 91.67% | 29.8% | 31.78% |  |
| R1-2203484 | Baseline: Always On | - | - | - | H | 12 | 12 | 95.83% | - | - |  |
| R1-2203484 | XR-PMW with go-to-sleep | - | - | - | H | 12 | 12 | 90% | 24.01% | 24.39% |  |
| R1-2203484 | XR-PMW with PDCCH skipping and go-to-sleep | - | - | - | H | 12 | 12 | 89.17% | 29.44% | 29.92% |  |
| R1-2203484 | Baseline: CDRX | 16 | 8 | 4 | H | 12 | 12 | 54.17% | - | - |  |
| R1-2203484 | XR-PMW with go-to-sleep | - | - | - | H | 12 | 12 | 90% | 13.08% | 13.34% | Note 1 |
| R1-2203484 | XR-PMW with PDCCH skipping and go-to-sleep | - | - | - | H | 12 | 12 | 89.17% | 19.30% | 19.68% | Note 1 |
| Note 1: Baseline is C-DRX (16,8,4) |  |  |  |  |  |  |  |  |  |  |  |

Table 3.1-2 CDRX related power saving evaluation results: FR1, DL only, Indoor Hotspot, VR 45Mbps

| Tdoc # | Power Saving Scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Load H/L | avg # UEs/Cell | floor (Capacity) | % of satisfied UE | Mean PSG of all UEs | Mean PSG of satisfied UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2203585 | Always On |  |  |  | L | 3 | 5 | 100% | - | - |
| R1-2203585 | R15/16 DRX | 16 | 14 | 4 | L | 3 | 5 | 100% | 3.46% | 3.46% |
| R1-2203585 | eCDRX | 16 | 4 | 4 | L | 3 | 5 | 100% | 26.74% | 26.74% |
| R1-2203585 | Always On | - | - | - | H | 5 | 5 | 96.67% |  |  |
| R1-2203585 | R15/16 DRX | 16 | 14 | 4 | H | 5 | 5 | 94.44% | 2.83% | 3.14% |
| R1-2203585 | eCDRX | 16 | 4 | 4 | H | 5 | 5 | 93.89% | 22.61% | 23.62% |
| R1-2203606 | Always On |  |  |  | H | 7 | 7 | 91% | - | - |
| R1-2203606 | R15 CDRX | 10 | 8 | 4 | H | 7 | 7 | 87% | 7.4% | 8.1% |
| R1-2203606 | eCDRX | 16 | 8 | 6 | H | 7 | 7 | 89% | 22..4% | 23.5% |

Table 3.1-3 CDRX related power saving evaluation results: FR1, DL only, Indoor Hotspot, CG 30Mbps

| Tdoc # | Power Saving Scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Load H/L | avg # UEs/Cell | floor (Capacity) | % of satisfied UE | Mean PSG of all UEs | Mean PSG of satisfied UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2203606 | Always On | - | - | - | H | 12 | 12 | 96.53% | - | - |
| R1-2203606 | R15 CDRX | 10 | 8 | 4 | H | 12 | 12 | 88.89% | 4.2% | 6.1% |
| R1-2203606 | eCDRX | 16 | 6 | 4 | H | 12 | 12 | 84% | 30.9% | 34% |
| R1-2203606 | eCDRX | 16 | 8 | 6 | H | 12 | 12 | 88.89% | 20.6% | 23.1% |
| R1-2203606 | eCDRX with flexible additional PDCCH monitoring | 16 | 6 | 4 | H | 12 | 12 | 88.19% | 32.4% | 34.5% |

Table 3.1-4 CDRX related power saving evaluation results: FR1, DL only, Dense Urban, VR 30Mbps

| Tdoc # | Power Saving Scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Load H/L | avg # UEs/Cell | floor (Capacity) | % of satisfied UE | Mean PSG of all UEs | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2204674 | Always On | - | - | - | H | 6 | 6 | - | - |  |
| R1-2204674 | CDRX | 16 | 8 | 8 | H | 6 | 6 | 0% | - |  |
| R1-2204674 | ADRX | 16 | 16 | 1 | H | 6 | 6 | 66% | 7.5% |  |
| R1-2204674 | Always On | - | - | - | H | 6 | 6 | - | - |  |
| R1-2204674 | CDRX | 8 | 4 | 0 | H | 6 | 6 | 61% | 18% |  |
| R1-2204674 | CDRX + 1 WUS | 8 | 4 | 0 | H | 6 | 6 | 45% | 20% |  |
| R1-2204674 | CDRX | 4 | 2 | 0 | H | 6 | 6 | 83% | 20% |  |
| R1-2204674 | CDRX + 1 WUS | 4 | 2 | 0 | H | 6 | 6 | 65% | 22.5% |  |
| R1-2204674 | CDRX + 2 WUS | 4 | 2 | 0 | H | 6 | 6 | 64% | 25.3% |  |
| R1-2204818 | Always On | - | - | - | H | 4 | 4 | 98.25% | - |  |
| R1-2204818 | R15 CDRX | 8 | 6 | 6 | H | 4 | 4 | 93% | 4.6% |  |
| R1-2204818 | R15 CDRX | 16 | 14 | 4 | H | 4 | 4 | 95.5% | 3.2% |  |
| R1-2204818 | eCDRX | 8 | 6 | 6 | H | 4 | 4 | 95.75% | 7.2% |  |
| R1-2204818 | eCDRX | 16 | 14 | 4 | H | 4 | 4 | 97.25% | 4.6% |  |
| R1-2204818 | Always On | - | - | - | H | 4 | 4 | 99.25% | - | Note 1 |
| R1-2204818 | R15 CDRX | 8 | 6 | 6 | H | 4 | 4 | 95% | 4.8% | Note 1 |
| R1-2204818 | R15 CDRX | 16 | 14 | 4 | H | 4 | 4 | 96% | 3.1% | Note 1 |
| R1-2204818 | eCDRX | 8 | 6 | 6 | H | 4 | 4 | 99.25% | 10.5% | Note 1 |
| R1-2204818 | eCDRX | 16 | 14 | 4 | H | 4 | 4 | 99.25% | 8.4% | Note 1 |
| Note 1: without jitter |  |  |  |  |  |  |  |  |  |  |

Table 3.1-5 CDRX related power saving evaluation results: FR1, DL only, Dense Urban, CG 30Mbps

| Tdoc # | Power Saving Scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Load H/L | avg # UEs/Cell | floor (Capacity) | % of satisfied UE | Mean PSG of all UEs | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2204674 | Always On | - | - | - | H | 8 | 8 | - | - |  |
| R1-2204674 | CDRX | 16 | 8 | 8 | H | 8 | 8 | 50% | 13.6% |  |
| R1-2204674 | ADRX | 16 | 16 | 1 | H | 8 | 8 | 70% | 7.5% |  |
| R1-2204674 | Always On | - | - | - | H | 8 | 8 | 96% | - |  |
| R1-2204674 | CDRX | 8 | 4 | 0 | H | 8 | 8 | 84% | 16.7% |  |
| R1-2204674 | CDRX + 1 WUS | 8 | 4 | 0 | H | 8 | 8 | 70% | 17% |  |
| R1-2204674 | CDRX | 4 | 2 | 0 | H | 8 | 8 | 88% | 18% |  |
| R1-2204674 | CDRX + 1 WUS | 4 | 2 | 0 | H | 8 | 8 | 79% | 20% |  |
| R1-2204674 | CDRX + 2 WUS | 4 | 2 | 0 | H | 8 | 8 | 76% | 22.5% |  |
| R1-2204818 | Always On | - | - | - | H | 4 | 4 | 98.75% | - |  |
| R1-2204818 | R15 CDRX | 8 | 6 | 6 | H | 4 | 4 | 95.25% | 4.6% |  |
| R1-2204818 | R15 CDRX | 16 | 14 | 4 | H | 4 | 4 | 97.25% | 3.2% |  |
| R1-2204818 | eCDRX | 8 | 6 | 6 | H | 4 | 4 | 97.5% | 7.2% |  |
| R1-2204818 | eCDRX | 16 | 14 | 4 | H | 4 | 4 | 98.25% | 4.6% |  |
| R1-2204818 | Always On | - | - | - | H | 4 | 4 | 99.75% | - | Note 1 |
| R1-2204818 | R15 CDRX | 8 | 6 | 6 | H | 4 | 4 | 97.25% | 4.8% | Note 1 |
| R1-2204818 | R15 CDRX | 16 | 14 | 4 | H | 4 | 4 | 98.25% | 3.1% | Note 1 |
| R1-2204818 | eCDRX | 8 | 6 | 6 | H | 4 | 4 | 99.75% | 10.5% | Note 1 |
| R1-2204818 | eCDRX | 16 | 14 | 4 | H | 4 | 4 | 99.75% | 8.4% | Note 1 |
| Note 1: without jitter |  |  |  |  |  |  |  |  |  |  |

Table 3.1-6 CDRX related power saving evaluation results: FR1, DL+UL, Indoor Hotspot, VR 30Mbps

| Tdoc # | Power Saving Scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Load H/L | avg # UEs/Cell | floor (Capacity) | % of satisfied UE | Mean PSG of all UEs | Mean PSG of satisfied UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2203606 | Always On | - | - | - | H | 11 | 11 | 93.18% | - | - |
| R1-2203606 | R15 CDRX | 10 | 8 | 4 | H | 11 | 11 | 90.15% | 6% | 6.65% |
| R1-2203606 | eCDRX | 16 | 6 | 4 | H | 11 | 11 | 90.15% | 23.2% | 24.6% |
| R1-2203606 | eCDRX with flexible additional PDCCH monitoring | 16 | 6 | 4 | H | 11 | 11 | 91.67% | 20.5% | 21.6% |

Table 3.1-7 CDRX related power saving evaluation results: FR1, DL+UL, Indoor Hotspot, VR 45Mbps

| Tdoc # | Power Saving Scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Load H/L | avg # UEs/Cell | floor (Capacity) | % of satisfied UE | Mean PSG of all UEs | Mean PSG of satisfied UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2203606 | Always On | - | - | - | H | 7 | 7 | 91% | - | - |
| R1-2203606 | R15 CDRX | 10 | 8 | 4 | H | 7 | 7 | 87% | 7.22% | 7.82% |
| R1-2203606 | eCDRX | 16 | 8 | 6 | H | 7 | 7 | 89% | 19.6% | 20.5% |

Table 3.1-8 CDRX related power saving evaluation results: FR1, DL+UL, Indoor Hotspot, CG 30Mbps

| Tdoc # | Power Saving Scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Load H/L | avg # UEs/Cell | floor (Capacity) | % of satisfied UE | Mean PSG of all UEs | Mean PSG of satisfied UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2203606 | Always On | - | - | - | H | 12 | 12 | 96.53% | - | - |
| R1-2203606 | R15 CDRX | 10 | 8 | 4 | H | 12 | 12 | 88.89% | 4.5% | 6.3% |
| R1-2203606 | eCDRX | 16 | 6 | 4 | H | 12 | 12 | 84% | 21.2% | 23.7% |
| R1-2203606 | eCDRX | 16 | 8 | 6 | H | 12 | 12 | 88.89% | 14.7% | 16.6% |
| R1-2203606 | eCDRX with flexible additional PDCCH monitoring | 16 | 6 | 4 | H | 12 | 12 | 88.19% | 21.3% | 22.7% |

Table 3.1-9 CDRX related power saving evaluation results: FR1, DL+UL, Dense Urban, VR 30Mbps

| Tdoc # | Power Saving Scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Load H/L | avg # UEs/Cell | floor (Capacity) | % of satisfied UE | Mean PSG of all UEs | Mean PSG of satisfied UEs | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2205054 | Always On | - | - | - | H | 11 | 11 | 97.14% | - |  |  |
| R1-2205054 | R15 CDRX | 16 | 12 | 12 | H | 11 | 11 | 69.87% | 1.78% |  |  |
| R1-2205054 | eCDRX | 16/17/17 | 10 | 10 | H | 11 | 11 | 82.86% | 9.43% |  |  |
| R1-2205054 | eCDRX | 16/17/17 | 12 | 12 | H | 11 | 11 | 94.20% | 4.51% |  |  |
| R1-2203638 | Always On | - | - | - | L | 3 | 7 | 99.6% | - | - |  |
| R1-2203638 | R15/16 DRX | 10 | 8 | 2 | L | 3 |  | 99.6% | 4.7% | 4.7% |  |
| R1-2203638 | R15/16 DRX | 4 | 2 | 2 | L | 3 |  | 99.6% | 10.4% | 10.6% |  |
| R1-2203638 | eCDRX | 16.6 | 10 | 2 | L | 3 |  | 99.6% | 8.8% | 9.0% |  |
| R1-2203638 | Always On | - | - | - | H | 7 | 7 | 91.3% | - | - |  |
| R1-2203638 | R15/16 DRX | 10 | 8 | 2 | H | 7 |  | 89.1% | 4.1% | 4.4% |  |
| R1-2203638 | R15/16 DRX | 4 | 2 | 2 | H | 7 |  | 85.6% | 9.5% | 8.7% |  |
| R1-2203638 | eCDRX | 16.6 | 10 | 2 | H | 7 |  | 89.7% | 7.9% | 7.6% |  |
| R1-2203638 | Always On | - | - | - | H | 5 | 5 | 92.2% | - |  | Note 1 |
| R1-2203638 | R15/16 DRX | 10 | 8 | 2 | H | 5 |  | 89.8 | 4.2% | 4.6% | Note 1 |
| R1-2203638 | multi-flow DRX | 33.3 | 10 | 2 | H | 5 |  | 91.2% | 12.1% | 11.6% | Note 1 |
|  |  | 10 | 2 | 2 |  |  |  |  |  |  |  |
| R1-2203638 | multi-flow and two-stage DRX | outer DRX: 33.3; inner DRX: 4; | outer ODT: 10; inner ODT: 2 | 2 | H | 5 |  | 87.2% | 15.4% | 15.0 | Note 1 |
|  |  | 10 | 2 | 2 |  |  |  |  |  |  |  |
| Note 1: DL video frame rate is 30 fps |  |  |  |  |  |  |  |  |  |  |  |

Table 3.1-10 CDRX related power saving evaluation results: FR2, DL only, Indoor Hotspot, VR 30Mbps

| Tdoc # | Power Saving Scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Load H/L | avg # UEs/Cell | floor (Capacity) | % of satisfied UE | Mean PSG of all UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2205176 | Always On | - | - | - | H | 7 | 7 | 90% | - |
| R1-2205176 | R15 CDRX | 16 | 4 | 4 | H | 7 | 7 | 0% | 28.44% |
| R1-2205176 | R15 CDRX | 16 | 8 | 8 | H | 7 | 7 | 50% | 9.64% |
| R1-2205176 | R15 CDRX | 16 | 8 | 16 | H | 7 | 7 | 65% | 4.10% |
| R1-2205176 | eCDRX | 16/17/17 | 4 | 4 | H | 7 | 7 | 27% | 25.10% |
| R1-2205176 | eCDRX | 16/17/17 | 8 | 8 | H | 7 | 7 | 84% | 8.28% |
| R1-2205176 | eCDRX | 16/17/17 | 8 | 16 | H | 7 | 7 | 88% | 2.43% |

Based on the companies’ evaluation results in Table 3.1-1 to Table 3.1-10, FL provides the following observation.

[RD1] Observation 3.1-1: observations are removed after 1st round discussion based on companies’ feedback

[RD1] Question 3.1-1: companies’ views on Table 3.1-1 to Table 3.1-10 and Observation 3.1-1.

| Company | Comments |
| --- | --- |
| MTK | The captured power saving gains in Observation 3.1-1 for CDRX enhancements seem a bit too large from our understanding. We suggest to only capture the power saving gain results with “% of satisfied UE > 90%”. |
| Xiaomi | We had a check on R17 XR TR 38.838, the captured evaluation results are all for satisified UE >=90%, Maybe we should keep the same standard for R18 TR |
| Samsung | Agree with MTK that some PS gains from eCDRX seem excessive and would be good to discuss first before capturing. Suggest to conclude at the next meeting. |
| vivo | Thanks for the great effort of moderator to summarize the evaluation results. We are supportive for the form of these Tables and Observations. Besides, there seems to be some mistakes in Table 3.1-2 for our results, and we correct them in revision mode. [Moderator] Thanks for the careful check. |
| LGE | - We have similar view with MediaTek. 20% loss in capacity seems not marginal loss. - As listed below there are many types of potential CDRX enhancement schemes and we may need to compare power saving benefits of them during SI phase. Observation 3.1-1 would be good starting point to understand baseline power saving benefits, but it seems that more details are needed to compare each of the potential methods. |
| Nokia, NSB | We appreciate the FL hard work gathering of results, and it is beneficial to see the summarised results but it is not 100% clear if we yet need to make any actual observation at this stage. We think that we should consider further before trying to do any conclusions based on these once we have some related assumptions clarified. Also we can then later consider what results and how to capture those on the TR, based on further agreements on the schemes. It would be also good to have common understanding that (in general, not just context of CDRX enhancements) that we should account the Rel-17 and earlier functionalities (e.g. Rel-16 DCP) for power saving when trying to conclude on the candidate enhancements for Rel-18. Thus, when legacy C-DRX is noted to be assumed it would seem to refer ‘plain vanilla’ long-DRXcycle operation without short-DRXcycle or MAC CE use. It might be good to clarify this. As commented in Section 2 we would prefer to evaluate the capacity as the maximum number of UEs for which the percentage of satisfied UEs is larger than ~90% (i.e., using the definition of capacity in capacity evaluation methodology) for each scheme individually and evaluate the effect in more comparable manner. In addition, as commented by other companies, maybe we should consider also what we define as a marginal loss in capacity. |
| Intel | We think capturing observations based on submitted results will be too rushed considering this is first meeting. We suggest to agree to evaluation assumptions and methodologies, and some common framework first. Agree with comment from Nokia |
| CATT | We appreciate the effort of capturing all results. However, CATT’s proposal of XR-PMW in R1-2203484 is the proposal for dynamic scheduling to battle the periodicity misalignment of C-DRX and delay jitter. The power saving gain of XR-PMW in Table 2 and 3 of R1-2203484 should be included in the Table. We observe that some of the power saving gains is from the large power consumption in the baseline assumption. It would be clear if the reference configuration, such as always on or DRX configuration, should be included in the table. Moreover, adding a column of “note” or “remark” to capture some observations of the results would make the results self-explanable. [Moderator] The missing evaluation results are added now. |
| OPPO | We appreciate the efforts from companies to run the evaluations and from FL to collect/organize the results in tables. But it is a bit surprising to see that RAN1 tries to agree some evaluation observations for the TR in the very first meeting in SI, at the same time as or even before agreeing the evaluation assumptions. We suggest to wait for one or two meeting cycles to collect results from sufficient number of companies, and then do a batch observation, instead of making numerous observations in each of meetings upon every new result arrival. |
| InterDigital | We share similar understanding with Nokia, Intel and Oppo to first confirm/agree on the evaluation methodology and assumptions before making any observations related to the enhancement scheme. |
| ZTE | In our understanding, the tables and observations are for result collection so far, and companies are expected to provide more evaluations. Regarding Note 2, it is actually follow the decision in Rel-17 SI, and we’re fine with it. |
| ITRI | We share the similar view with OPPO. |
| NTT DOCOMO | Thanks for the effort of summarizing companies’ evaluation results. Although we think the table is generally good and beneficial, the evaluation methodology and assumption including what is a marginal loss in capacity can be considered and clarified before trying to agree it. |
| Spreadtrum | We appreciate the effort from companies to run the evaluations and moderator to summarize the results. We think that notes should be added to the table to introduce more details of the potential methods. In addition, there seems to be a mistake with "ADXR" in table 3.1-4 and table 3.1-5. We correct them with "ADRX" in revision mode [Moderator] Thanks for the careful check. For the notes, companies have included baseline in their results. The other notes are removed together with the Observations as mentioned below. |
| Huawei, HiSilicon | We are unclear about the FL’s intention/plan to give these observations. If the intention/plan is to collect companies’ views just for information, and no agreement/conclusion will be made in this meeting, then we are fine with this. If the intention/plan is to draw conclusions on these observations and capture them the into TR, then we have some concerns since it’s too pre-mature: - The evaluation methodology for Rel-18 XR study is still under discussion and not stable yet (see Question 2-1/2-2/2-3 above). Since the evaluation methodology itself is not stable, it’s hard to judge current simulation results comply with R18 XR evaluation methodology or not. - Only a few companies provided evaluation results. RAN1 does not have enough time to discuss these simulation results, compare and validate them, etc. It is too early to draw conclusion in this meeting. We suggest to wait for 1~2 meetings for companies to further submit simulation results. Conclusion/observation can be made when the evaluation methodology is stable and most of the companies can provide their evaluation results. - The details of the enhanced CDRX is not clear. This should be further clarified. |
| Moderator | The missing results from CATT and Ericsson are also added in table above now. Some companies have concerns on making observations in this meeting based on evaluation results from a limited number of companies without performance calibration. To address these concerns, FL removed the Observations to only keep the raw evaluation results in the Tables. |

Based on the various enhancements proposed in Section 3.1.1, 3.1.2, 3.1.3 and 3.1.4, the following main issues are identified with CDRX enhancements:

- Topic 1: CDRX enhancements

  - Issue 1-1: Alignment between CDRX and XR traffic for resolving the mismatch between CDRX cycle and XR traffic periodicity. The proposed solutions include 3 semi-static and 1 dynamic techniques

    - Non-uniform CDRX cycle pattern, e.g., {16,17,17}ms

    - Uniform non-integer CDRX cycle, e.g., 50/3ms

    - Fix to SFN wraparound mismatch with new time reference

    - Dynamic adjustment of CDRX start offset

  - Issue 1-2: Jitter handling for CDRX. The proposed solutions include

    - Non-uniform PDCCH monitoring occasions within On Duration matched to jitter distribution

    - Dynamic indication of DRX On Duration starting time by DCI or WUS

    - Adjustment of DRX On Duration conditioned on data reception

    - PDCCH monitoring adaptation within On Duration

  - Issue 1-3: CDRX enhancements for multiple XR traffic flows. The proposed enhancements include

    - Multiple simultaneous CDRX configurations

    - Multi-period CDRX cycle or On Duration sequence of a CDRX cycle

[RD1] Question 3.1-2: companies’ views on Topic 1 and the Issues.

| Company | High/low sub-topics and reasons |
| --- | --- |
| MTK | We think the issues listed in Topic 1 are essential and worth further discussions. |
| Xiaomi | OK with the proposed issues, and for the proposed enhancements, maybe currently, we can leave a door to other schemes, and saying “other schemes are not precluded” |
| NEC | We think all the three issues are essential and should be further studied. For the Issue 1-1, we prefer to further study the first three sub-bullets, since they may do not need explicit L1/L2 signalling, therefore may be better for UE complexity and the overhead. For the Issue 1-2, we are open to the listed candidate techniques. For the Issue 1-3, we prefer the first sub-bullet, because it can be realized based on the enhancement of the existing multiple CDRX mechanism, therefore less standard effort is needed compared to the second sub-bullet. |
| Samsung | OK in general with the suggested enhancements for further discussion although we think they can be reduced after some discussion at this meeting. For example, the first and fourth items for Issue 1-2 seem possible using Rel-17. [Moderator] Thanks for the comments. “PDCCH monitoring adaptation within On Duration” is removed now, and a note is added under “Non-uniform PDCCH monitoring occasions within On Duration matched to jitter distribution” to respond. Also, existence of multiple XR traffic flows does not necessarily motivate Issue 1-3 as the flows will need to be non-overlapping in time with large separation and there is also availability of the Rel-17 solutions for PDCCH monitoring that can be used. Suggest to first identify XR-specific issues that cannot be sufficiently addressed by Rel-17. |
| vivo | We are supportive of focusing on the identified issues for Topic 1. First of all, we think the description of some of the solutions seem to be over detailed. Since this is the first meeting, we kindly suggest to provide some high level solutions for addressing XR issues and more concrete solutions can be discussed further. Besides, we also note that some proposed enhancements could be adopted for addressing different issues. Hence, we give the following modification suggestions for moderator consideration. 1, The solution “Multiple simultaneous CDRX configurations” listed below issue 1-3 can also be used to solve issue 1-1. As analysed in our contribution [R1-2203585], by adopting multiple CDRX configurations, the same power saving effect can be achieved as that of Non-uniform CDRX cycle pattern, dynamic adjustment of CDRX start offset etc. 2, The solution “Dynamic adjustment of CDRX start offset” listed below issue 1-1 is duplicated with the solution “Dynamic indication of DRX On Duration starting time by DCI or WUS”. Hence, we can remove one of them. 3, The solution “Non-uniform PDCCH monitoring occasions within On Duration matched to jitter distribution” can be included into the solution “PDCCH monitoring adaptation within On Duration”. 4, The solution “jitter handling by WUS” proposed by lots of companies can also be studied and we suggest to add it into the list below issue 1-1. 5, Combine the solution “Adjustment of DRX On Duration conditioned on data reception” into the solution “Dynamic or condition based adjustment of CDRX start offset”. [Moderator] Thanks for the comments. From companies’ proposals, the “Adjustment of DRX On Duration conditioned on data reception” is to adjust On Duration length rather than the adjustment. This is clarified in the updated proposal for round 2 discussion. 6, Add some notes for clarification. - Topic 1: CDRX enhancements [Suggested by vivo] - Issue 1-1: Alignment between CDRX and XR traffic for resolving the mismatch between CDRX cycle and XR traffic periodicity. The proposed solutions include 3 semi-static and 1 dynamic techniques - Non-uniform CDRX cycle pattern, e.g., {16,17,17}ms - Uniform non-integer CDRX cycle, e.g., 50/3ms - Fix to SFN wraparound mismatch with new time reference - Dynamic or condition based adjustment of CDRX start offset [1] - Issue 1-2: Jitter handling for CDRX. The proposed solutions include - Non-uniform PDCCH monitoring occasions within On Duration matched to jitter distribution - Dynamic indication of DRX On Duration starting time by DCI or WUS - Jitter handling by WUS - Adjustment of DRX On Duration conditioned on data reception - PDCCH monitoring adaptation within On Duration - Issue 1-3: CDRX enhancements for multiple XR traffic flows. The proposed enhancements include - Multiple simultaneous CDRX configurations [2] - Multi-period CDRX cycle or On Duration sequence of a CDRX cycle Notes: [1]: it can also be adopted for addressing issue 1-2 [2]: it can also be adopted for addressing issue 1-1 |
| LGE | In general, we are ok with further study issues listed above. Also, as commented by Xiaomi, it would be better not to preclude other potential schemes. For Issue 1-2, we prefer to remove ‘matched to jitter distribution’ in the first sub-bullet. We don’t need to restrict potential enhancements for a specific scenario. |
| Nokia, NSB | We are supportive of trying to identify the main issues to be addressed, i.e. alignment of periodicity and jitter, to enable further power saving. As noted above we should also try to ensure that the considered enhancements provide benefit on top of the existing functionalities (in Rel-17 and earlier) in terms of power saving (or capacity). While listing schemes can also be done, it is probably premature to close door from other methods (or combination of them), thus schemes could exemplary. Also, when going forward, if we can address some issues identified in terms of power saving impact for example via C-DRX enhancement (e.g. periodicity), while other via PDCCH monitoring enhancement (e.g. jitter), there would not be need to have multiple solutions for same issue (eat the same cake). Thus, we should consider the proposed solutions holistically so that we can conclude to limited set of enhancements to ensure focused specification phase after the study. These could also be considered, if relevant, in terms of compatibility with proposed capacity enhancements |
| Intel | Some revisions are suggested. It is expected that non-uniform PDCCH monitoring occasions within ON duration falls under PDCCH monitoring adaptation within ON duration. - Topic 1: Identified issues related to CDRX enhancements are as follows, but not limited to, [Intel] - Issue 1-1: Alignment between CDRX and XR traffic for resolving the mismatch between CDRX cycle and XR traffic periodicity. Solutions include the following: - Non-uniform CDRX cycle pattern - Uniform non-integer CDRX cycle - Fix to SFN wraparound mismatch with new time reference - Dynamic adjustment of CDRX start offset - Other considerations are not precluded - Issue 1-2: Jitter handling for CDRX. The proposed solutions include - Dynamic indication of DRX On Duration starting time by DCI or WUS - Adjustment of DRX On Duration conditioned on data reception - PDCCH monitoring adaptation within On Duration - Other considerations are not precluded - Issue 1-3: CDRX enhancements for multiple XR traffic flows. The proposed enhancements include - Multiple simultaneous/active CDRX configurations - Multi-period CDRX cycle or On Duration sequence of a CDRX cycle or non-uniform CDRX cycle pattern - Other considerations are not precluded |
| CATT | We are OK with the categorization of the XR-specific C-DRX power saving techniques. CATT’s proposal of XR-PMW is a dynamic scheduling of MAC PDCCH monitoring control with DRX configuration customized for the XR and other traffic. The customized XR PDCCH monitoring window is controlled by DRX at MAC to have periodic PDCCH decoding based on the XR packet generation rate and a window duration of PDCCH monitoring based on the network delay jitter in both DRX ON and OFF (similar to SPS fixed resource). Thus, our suggest is as follows, - Topic 1: CDRX enhancements - Issue 1-1: Alignment between CDRX and XR traffic for resolving the mismatch between CDRX cycle and XR traffic periodicity. The proposed solutions include 3 semi-static and 1 dynamic techniques - Non-uniform CDRX cycle pattern, e.g., {16,17,17}ms - Uniform non-integer CDRX cycle, e.g., 50/3ms - Fix to SFN wraparound mismatch with new time reference - Dynamic adjustment of CDRX start offset - Periodic XR-specific PDCCH monitoring in both DRX ON and OFF based on XR packet generation cycle - Issue 1-2: Jitter handling for CDRX. The proposed solutions include - Non-uniform PDCCH monitoring occasions within On Duration matched to jitter distribution - Dynamic indication of DRX On Duration starting time by DCI or WUS - Adjustment of DRX On Duration conditioned on data reception - PDCCH monitoring adaptation within On Duration - XR-specific PDCCH monitoring window at each cycle in both DRX ON and OFF based on network delay jitter - Issue 1-3: CDRX enhancements for multiple XR traffic flows. The proposed enhancements include - Multiple simultaneous CDRX configurations - Multi-period CDRX cycle or On Duration sequence of a CDRX cycle |
| OPPO | - Issue 1-1: In our understanding, the 4th candidate solutions “dynamic adjustment of CDRX start offset” includes both “dynamic adjustment with dynamic signaling” and “dynamic adjustment w/o dynamic signaling (e.g., change drx_startoffset periodically upon a configuration)”. We would like to ask FL to make this clear in the summary/proposal; otherwise we would like to ask to have a separate bullet for “periodic change of drx_startoffset based on configuration”. Moderator: Thanks for the comment. A new 5th bullet is added, and the 4th is updated to clarify it is based on dynamic signaling. - Issue 1-2: We prefer to modify the second bullet as follows: (This is because the current 2nd bullet just include to dynamically adjust the start time of DRX-ON, but does not include dynamically add another DRX on duration time which can be used to solve late arrival issue as summarized in section 3.1.2) - Dynamic indication of DRX On Duration starting time and/or another DRX On Duration time by DCI or WUS |
| SONY | - We support to continue study the aforementioned issues (i.e, 1-1, 1-2, and 1-3). Specifically on Issue 1-2. We propose to include two stage/level CDRX configuration. - Issue 1-2: Jitter handling for CDRX. The proposed solutions include - Non-uniform PDCCH monitoring occasions within On Duration matched to jitter distribution - Dynamic indication of DRX On Duration starting time by DCI or WUS - Adjustment of DRX On Duration conditioned on data reception - PDCCH monitoring adaptation within On Duration - Two stage/level CDRX configuration |
| InterDigital | We are generally ok with the listed issues under Topic 1. For issue 1-1, our preference is to focus on the solutions described in first and fourth sub-bullets as they seem to be less complex and could potentially address the issue with lower standardization effort For issue 1-2, we think the last three sub-bullets are more robust to handle jitter issue, and hence prefer excluding the first sub-bullet For issue 1-3, we are open for considering both solutions in the first and second sub-bullets Additionally, for all three Issues, we share similar understanding with Xiomi to not preclude other enhancement schemes at this stage. |
| Qualcomm | Generally, at this point, we are fine to keep the options open for study. The examples mentioned in Topic 1 are a good starting point, with the understanding that other options are not precluded. In general, as explained in our paper, using fixed (e)CDRX parameters may have negative impact on delays (PDB) and power consumption for XR traffic. For some cases (e.g., misalignment), the gNB may need to buffer packets and transmit them on the next CDRX ON duration having delay impacts. In other cases (e.g., data shorter than Active Time), CDRX ON or inactivity timer durations may be longer than what is needed, hence, wasting power. In such cases, it may be helpful to study dynamically changing CDRX parameters. From the list of examples, we prefer to focus on studying eCDRX for DRX cycle misalignment issue as well as dynamically adjusting CDRX parameters (e.g., start offset, ON duration, inactivity timer, etc…), hence we can add: - Topic 1: CDRX enhancements - Issue 1-1: Alignment between CDRX and XR traffic for resolving the mismatch between CDRX cycle and XR traffic periodicity. The proposed solutions include 3 semi-static and 1 dynamic techniques - Non-uniform CDRX cycle pattern, e.g., {16,17,17}ms - Uniform non-integer CDRX cycle, e.g., 50/3ms - Fix to SFN wraparound mismatch with new time reference - Dynamic adjustment of CDRX start offset - Issue 1-2: Jitter handling for CDRX. The proposed solutions include - Non-uniform PDCCH monitoring occasions within On Duration matched to jitter distribution - Dynamic indication of DRX On Duration starting time by DCI or WUS - Adjustment of DRX On Duration conditioned on data reception - PDCCH monitoring adaptation within On Duration - Issue 1-3: CDRX enhancements for multiple XR traffic flows. The proposed enhancements include - Multiple simultaneous CDRX configurations - Multi-period CDRX cycle or On Duration sequence of a CDRX cycle - Issue 1-4: CDRX enhancements to adjust to variable burst sizes Dynamic adjustment of CDRX Active Time parameters |
| ZTE | For issue 1-1, in general, we’re fine with the sub-bullets except that 1 more semi-static solution in section 3.1.1 is missing - Multiple CDRX configurations [Moderator] Thanks for the comments. [Note 2] is added in the updated proposal for this. For the SFN wraparound mismatch issue, we don’t think we need to limit solutions to a new reference, other solutions are more precluded at this stage. Therefore, we suggest the following update - Fix to SFN wraparound mismatch with, e.g., new time reference For issue 1-2, it seems “On Duration” is used for all the sub-bullets, we think “On” Duration is too specific, and a general description such as [active time] can be used to capture all the proposed solutions. [Moderator] On Duration is used to reflect the original proposals from companies. Sorry if some proposals are mis-captured, but this may have to done case by case. For issue 1-3, multiple CDRX configurations or single CDRX configuration can be studied for accommodating multiple flows |
| ITRI | We support further study on listed issues. Also we agree with Xiaomi on saying “other schemes are not precluded”. |
| NTT DOCOMO | We are generally fine with the listed issues. Since this is the first meeting, other schemes should not be precluded, which needs to be mentioned in proposal. For 4th candidate solution in Issue 1-1 “Dynamic adjustment of CDRX start offset”, we prefer to modify “Dynamic adjustment of CDRX start offset/duration”. |
| Spreadtrum | We think the issues of Topic 1 are worth discussing. We share similar understanding with vivo that the first meeting should discuss some general solutions to solve XR issues, and study the details in the following meeting. |
| Lenovo | 1-1: low 1-2: high 1-3: medium In our view, solutions for issue 1-2 may be able to address issue 1-1, especially considering the jitter range. Also, solutions for issue 1-2 along with potential application awareness (e.g., flow dependency in case of multiple flow) and scheduler implementation may be able to address issue 1-3. |
| CMCC | Generally, we are fine about the listed solutions and other solutions are not precluded. Regarding the Issue 1-2, we have two comments. The first is that we think Non-uniform PDCCH monitoring occasions within On Duration matched to jitter distribution is one of the PDCCH monitoring adaptation schemes, and the second comment is what is the difference between PDCCH monitoring adaptation within On Duration and Rel-17 PDCCH monitoring adaptation? [Moderator] Thanks for the comments. “Non-uniform PDCCH monitoring...” proposal is moved to PDCCH monitoring enhancements as suggested. The “PDCCH monitoring adaptation within On Duration” is now removed based on multiple companies’ comments. Regarding the issue 1-3, we think “Multiple simultaneous CDRX configurations” needs more clarification, does it preclude that RRC configure multiple CDRX configurations and another signalling indicate the active or enabled CDRX configuration(s)? [Moderator] The dynamic enabling is not precluded under the general proposal of multi-CDRX configurations. This is clarified in the list. |
| Huawei, HiSilicon | For Issue 1-1, 1) In addition to dynamic adjustment of CDRX start offset by using L1/L2 signalling, we think adjustment of CDRX start offset in a semi-static manner can also be considered (e.g., RRC configuration, pre-defined manner). Since XR traffic is periodic and the traffic arrival is somehow predictable, dynamic adjustment of CDRX start offset by using L1/L2 signalling may cause extra signalling overhead. Semi-static adjustment may be more preferred. Both can be further studied. 2) For SFN wraparound mismatch, this issue is not XR-specific. Other CDRX cycles, which cannot be divided by 10240, will also cause SFN wraparound mismatch. This issue has been discussed in RAN2 in RAN2#105, the conclusion of which is summarized in R2-1903001, copied as follows: Copied from R2-1903001 (RAN2#105 meeting report): R2-1901921 Correction to start of drx-onDurationTimer Huawei, HiSilicon CR Rel-15 38.321 15.4.0 0632 - F NR_newRAT-Core - QC and Intel think this can be handled by network configuration. LG agrees, and the problem is very small. - QC think that this is non-backwards compatible. - LG point out that the “misconfiguration” is allowed in LTE and as long as network and UE has the same understanding there is no problem. - Interpretation: When the TS says “start” a timer while timer is running then nothing will happen. LG thikn this is already captured. - Chair: There is no support to fix. Not pursued. To save workload, we think this issue can be handled by RAN2 if necessary, since they have already discussed this before and are familiar with this. RAN1 does not need to discuss it again from zero ground. [Moderator] Thanks for the comments. As the proponent points out an essential issue, FL suggest to capture it in the first round outcome and leave the down selection in the following rounds where down selection based on in-depth technical discussions on whether it is within scope. This also applies to other proposals. 3) In addition to the list solutions, we think multiple active CDRX configurations can also be considered to address the mismatch between CDRX cycle and XR traffic periodicity. For example, these CDRX configurations can have the same periodicity (e.g., 50ms) and staggered start offsets (e.g.., {0, 17, 34}ms). Then the On Durations across the multiple CDRX configurations will match with the XR traffic periodicity. This solution can be a more general mechanism, which can also be used to support multiple XR traffic flows with different periodicities. Since the identified XR-specific issues (e.g. non-integer periodicity, multiple flows) exist simultaneously in real XR applications, solution(s) that can address multiple or all the identified issues are more preferred. [Moderator] please check note 2. For Issue 1-2, the second and the third bullets are similar. Both are to indicate of DRX On Duration starting time, either explicitly by DCI/WUS or implicitly conditioned on data reception. At this stage, we do not need to agree on such detailed things. These two bullets can be uniformed as “adjustment of DRX On Duration starting time”. For the fourth bullet, since the data transmission may also be outside OnDuration where the InactivityTimer runs. Changing ‘within On Duration’ to ‘within Active Time’ is more accurate. For Issue 1-3, we think multiple XR traffic flows is a key issue that need to be considered. Multiple active CDRX configurations with different periodicities and start offsets can be used to handle this issue efficiently. Furthermore, since the identified XR-specific issues (e.g. non-integer periodicity, jitter, multiple flows) exist simultaneously in real XR applications, solution(s) that can address multiple or all the identified issues are more preferred. It needs to further study how the solutions or the combination of the solutions can handle all the identified issues. In summary, we suggest the following changes in red: == - Topic 1: CDRX enhancements - Issue 1-1: Alignment between CDRX and XR traffic for resolving the mismatch between CDRX cycle and XR traffic periodicity. The proposed solutions include 3 semi-static and 1 dynamic techniques - Non-uniform CDRX cycle pattern, e.g., {16,17,17}ms - Uniform non-integer CDRX cycle, e.g., 50/3ms - Fix to SFN wraparound mismatch with new time reference - Dynamic adjustment of CDRX start offset - Multiple simultaneous CDRX configurations - Issue 1-2: C-DRX enhancements to handle jitter Jitter handling for CDRX. The proposed solutions include - Non-uniform PDCCH monitoring occasions within On Duration matched to jitter distribution - Dynamic indication adjustment of DRX On Duration starting time by DCI or WUS - Adjustment of DRX On Duration conditioned on data reception - PDCCH monitoring adaptation within Active Time On Duration - Issue 1-3: CDRX enhancements for multiple XR traffic flows. The proposed enhancements include - Multiple simultaneous CDRX configurations - Multi-period CDRX cycle or On Duration sequence of a CDRX cycle - FFS how the solutions or the combination of the solutions can handle all the identified issues. |
| Apple | - Consistent with the tasks given by SID ( study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: - C-DRX enhancement. - PDCCH monitoring enhancement), we propose to have issue-1-4. For latency sensitive transmission, if retransmission is not expected, then monitoring for PDCCH for retranmsission can be turned off. Note under issue 1-2 ther are solutions playing with DRX timers, logically 1-2 and 1-4 can be combined. - Topic 1: CDRX enhancements - Issue 1-1: Alignment between CDRX and XR traffic for resolving the mismatch between CDRX cycle and XR traffic periodicity. The proposed solutions include 3 semi-static and 1 dynamic techniques - Non-uniform CDRX cycle pattern, e.g., {16,17,17}ms - Uniform non-integer CDRX cycle, e.g., 50/3ms - Fix to SFN wraparound mismatch with new time reference - Dynamic adjustment of CDRX start offset - Issue 1-2: Jitter handling for CDRX. The proposed solutions include - Non-uniform PDCCH monitoring occasions within On Duration matched to jitter distribution - Dynamic indication of DRX On Duration starting time by DCI or WUS - Adjustment of DRX On Duration conditioned on data reception - PDCCH monitoring adaptation within On Duration - Issue 1-3: CDRX enhancements for multiple XR traffic flows. The proposed enhancements include - Multiple simultaneous CDRX configurations - Multi-period CDRX cycle or On Duration sequence of a CDRX cycle - Issue 1-4: low latency handling: - SPS/CG configuration specific handling of DRX timers. - DRX timers for DG transmission |
| Moderator | Please check follow-up discussion below and Proposal 3.1-1. |

Summary of 1st round discussion on Question 3.1-2:

FL would like to thank companies for the valuable inputs on the improvement of the list of issues. Based on these inputs, the following updated list is provided. There indeed were some conflicts between different companies on the same technique, e.g., one company prefers to remove a proposed enhancement, but another company proposed to keep or update a proposed enhancement. In this case, FL will keep the proposed enhancement and add a note to reflect the second company’s concern. There can be further down selection of the proposed enhancements after the 1st round discussion of this meeting and in future meetings.

[RD2] Proposal 3.1-1: For power saving study of Rel-18 XR SI, the following areas for potential CDRX enhancements are identified

- Issue 1-1: Alignment between CDRX and XR traffic for resolving the mismatch between CDRX cycle and XR traffic periodicity. The proposed solutions include the following:

  - Non-uniform CDRX cycle pattern

  - Uniform non-integer CDRX cycle

  - Fix to SFN wraparound mismatch, e.g., new time reference

  - Dynamic signaling indicated adjustment of CDRX start offset/duration [Note 1]

  - Periodic change of drx_startoffset based on configuration

  - Periodic XR-specific PDCCH monitoring in both DRX ON and OFF based on XR packet generation cycle

  - Other considerations are not precluded

- Issue 1-2: C-DRX enhancements to handle jitter. The proposed solutions include

  - Jitter handling by dynamic indication of DRX On Duration starting time and/or another DRX On Duration time by DCI or WUS

  - Adjustment of DRX On Duration conditioned on data reception

    - Note: the adjustment is applied to the duration

  - XR-specific PDCCH monitoring window at each cycle in both DRX ON and OFF based on network delay jitter

  - Two stage/level CDRX configuration

  - Other considerations are not precluded

- Issue 1-3: CDRX enhancements for multiple XR traffic flows. The proposed enhancements include

  - Multiple active CDRX configurations [Note 2]

    - Dynamic activation of CDRX configuration(s) is not precluded

  - Multi-period CDRX cycle or On Duration sequence of a CDRX cycle or non-uniform CDRX cycle pattern

  - Other considerations are not precluded

- Issue 1-4: CDRX enhancements to adjust to variable burst sizes

  - Dynamic adjustment of CDRX Active Time parameters

- Issue 1-5: low latency handling:

  - SPS/CG configuration specific handling of DRX timers.

  - DRX timers for DG transmission

FFS how the solutions or the combination of the solutions can handle all the identified issues.

Note 1: it can also be adopted for addressing issue 1-2

Note 2: it can also be adopted for addressing issue 1-1

Note 3: these enhancements are subject to down selection based on technical discussions on the necessity and evaluation results

Companies view on [RD2] Proposal 3.1-1

Goals of this round of discussion:

- Identify high and lower priority issues under Proposal 3.1-1

  - Especially issues that you do not think should be studied in Rel-18 XR SI

- Identify proposed enhancements for that there is existing spec solution achieving same or similar effect

| Company | Comments/Reasons |
| --- | --- |
| Apple | 1-3 and 1-5 are of high priority |

PDCCH Monitoring Enhancements

### Periodicity Alignment for PDCCH Monitoring

Similar to the periodicity mismatch between DRX cycle and XR traffic, there is a mismatch between search space set periodicity and XR traffic periodicity. The enhanced PDCCH monitoring was proposed by companies to address this periodicity mismatch problem.

Table 7 captures companies’ views on alignment for PDCCH monitoring and XR traffic in time.

Table 7: Proposals and views on PDCCH monitoring alignment with XR traffic in time

| Company | Views |
| --- | --- |
| Huawei, HiSi | Proposal 3: Further study the following directions for C-DRX and PDCCH monitoring enhancements to address the non-integer periodicity issue: - Adjusting the start offset of DRX OnDuration and/or the start offset of PDCCH monitoring - Adjusting the periodicity, e.g. configuring a cycle pattern - Multiple C-DRX configurations |
| Spreadtrum | Proposal 4: According to the mismatch of XR periodicity and PDCCH monitoring occasions, semi-static PDCCH monitoring periodicity and PDCCH monitoring skipping should be studied to match XR traffic. |
| NEC | A Search Space Set occurs during slot(µ, s, f) within Frame(f) when the following equality is true: For 30KHz SCS, there are 20 slots per frame (Frame(f) x 20 + slot(µ, s, f) – offset – k(µ) + jitter) mod Periodicity = 0 where p is the index of periodicity, p=0, 1, 2, 3, 4, 5… k(µ) = ⌈ p *(1000ms/fps*2^µ - Periodicity) ⌉ - The ‘fps’ (frame per second) is the frame rate for XR. - ‘jitter’ is an offset which may be added for the jitter. A DCI field (such as the reserved value of PDCCH monitoring adaptation field) may be used to set the jitter offset from time to time (start offset of PDCCH monitoring may shift according to +/- ‘jitter offset’ value). Proposal 1: Specify XR specific PDCCH monitoring offset parameters such as k(µ) and jitter in Search Space Set configuration. Proposal 3: Specify a higher layer parameter of ‘frame per second’ for the frame rate of XR traffic. |
| OPPO | Proposal 4: The following search space set configuration enhancements can be further studied to solve periodicity mismatch between XR traffic and search space monitoring: - Configure a monitoring periodicity pattern with two periodicity values; - Change monitoring offset periodically. |
| ITRI | configure a MonitoringSlotPeriodicity pattern with different MonitoringSlotPeriodicity values to fit the XR DL traffic arrival pattern. For example, MonitoringSlotPeriodicity pattern is set as {17, 17, 16}ms. Proposal 3: Further study on the impacts caused by the uncertainty of XR DL traffic arrival and jitter to the PDCCH monitoring periodicity modification method. |

Semi-static pattern for PDCCH monitoring

[Huawei, HiSi], [Spreadtrum], [OPPO] and [ITRI] proposed to configure a semi-static pattern of PDCCH monitoring that is matched to the non-integer XR traffic periodicity. [NEC] and [OPPO] proposed to periodically change the start offset of PDCCH monitoring in a semi-static manner to match with XR traffic period.

Dynamically adjust the start offset of PDCCH monitoring

[Huawei, HiSi] and [Spreadtrum] proposed to dynamically adjust the start offset of PDCCH monitoring, e.g., by PDCCH skipping to match XR traffic periodicity.

### Enhancements to PDCCH Monitoring Adaptation

Table 8 captures companies’ views on enhancements of PDCCH monitoring adaptation.

Table 8: Proposals and views on enhancements of PDCCH monitoring adaptation

| Company | Views |
| --- | --- |
| CATT | Figure 1: The procedure of the PDCCH skipping with 2 bits indication in non-scheduling DCI Proposal 1: Rel-17 PDCCH skipping adaptation should be enhanced to further reduce UE power consumption for XR services, e.g., non-scheduling DCI based PDCCH skipping Proposal 2: The extension of non-scheduling DCI format design could reuse the existing DCI format 1_1 in Rel-16 and not increase the size of DCI format with additional function in extending the PDCCH monitoring adaptation in PCell without introducing additional information field. |
| Vivo | Figure 9 Power saving gain of PDCCH skipping Observation 6: For DL XR traffic for FR1 in Indoor Hotspot scenario, compared to R17 PDCCH skipping scheme, enhanced PDCCH skipping scheme can achieve an additional {4.83%~7.15%} power saving gain. Observation 7: For DL XR traffic for FR1 in Indoor Hotspot scenario, compared to R17 PDCCH skipping with SSSG switching scheme, enhanced PDCCH skipping with SSSG switching scheme can achieve an additional {6.31%~8.14%} power saving gain. Proposal 4: Study XR specific PDCCH skipping enhancement, e.g. extention of skipping duration. Proposal 6: The following leftover enhancements of R17 power saving WI can also be further studied for XR traffic, if needed: - Non-scheduling DCI based PDCCH monitoring adaptation - Single DCI indicates monitoring adaptation for multiple cells - PDCCH monitoring adaptation applying for Type 0-2 CSS sets (depending on the outcome of R17 Power Saving WI) - PDCCH skipping and interaction with HARQ retransmission (depending on the outcome of R17 Power Saving WI) |
| Ericsson | Furthermore, although existing PDCCH monitoring adaptation features are promising for saving power, they could also introduce significant signaling overhead, since SSSG switching and PDCCH skipping are based on dynamic DCI signaling. C-DRX based solutions could manage with less signaling overhead, due to the overall (quasi-) deterministic characteristics of the XR traffic, for which semi-static PDCCH monitoring patterns with C-DRX are suitable. Observation 7 Extensive power saving features exist for PDCCH monitoring in Rel-17. Proposal 6 Rel-17 PDCCH power saving features are baseline for Rel-18 enhancements. Proposal 7 Investigate potential enhancements to Rel-17 PDCCH monitoring, considering both cost (e.g., DCI overhead) and benefit (e.g., power saving gain). |
| Sony | When DCI#1 is successfully decoded, the linkage information will guide the sub-sets selection for the decoding of DCI#2. Compared to normal decoding of DCI#2 in whole SS, proposed scheme can reduce BD attempts to one sub-set. Proposal 5 – Adding DCI linkage information in DCI can be considered to reduce PDCCH BD attempts for power saving. |
| OPPO | Proposal 5: The value of monitoringSlotPeriodicityAndOffset and duration for a search space set can be changed dynamically by gNB to cope with jitter and packet size variation. |
| Apple | Proposal 3-2: investigate DRX enhancement to support data flows with different latency budget so packets with stringent latency budget don’t trigger PDCCH monitoring in the case they are not received in their first transmission. |
| CMCC | Proposal 5. Non-scheduling DCI can also be considered as PDCCH skipping and SSSG switching indication, e.g., repurpose some fields in DL/UL grant. |
| Lenovo | Proposal 1: Study impact of ADU dropping on DRX timers (including drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL). Proposal 2: PDCCH skipping duration is selected from a larger set of RRC configured skipping durations to accommodate XR traffic jitter or varying video frame size. |
| LGE | The indication can be carried by a scheduling DCI for XR packet or some other DCI format. Observation 4: Rel-17 PDCCH monitoring adaptation in connected mode can also be applied to the UE not being configured with the CDRX, with enhancements if necessary. Proposal 5: Consider WUS outside DRX Active Time as well as the DCI inside DRX Active Time as a candidate for indicating the PDCCH monitoring adaptation. Proposal 6: Consider enhancements on PDCCH monitoring adaptation in connected mode to mimic the CDRX function for a UE not being configured with the CDRX. Proposal 7: For enhancements on PDCCH monitoring adaptation in connected mode, consider PDB margin as well as periodicity and jitter distribution of the XR traffic arrival time. |
| MediaTek | Proposal 1: If the UE is indicated to skip PDCCH monitoring, then if any NACK is transmitted the UE resumes the monitoring and cancel the indication of PDCCH. Proposal 2: If the UE is indicated to skip PDCCH monitoring, then if any NACK is transmitted or if UL data is transmitted, the UE resumes the monitoring for a configured duration. Proposal 6: Selective retransmissions for UL pose/control information can help reduce power consumption without impacting performance. |
| Intel | Since PDB of XR media can be short, it may not be desirable that retransmission is delayed since skipping starts from next slot of DCI reception. One option can be to support non-scheduling DCI providing the trigger for PDCCH skipping, i.e., when packet delivery is complete gNB can use a non-scheduling DCI to indicate PDCCH skipping so that UE can enter sleep state immediately. Proposal 3: Rel-17 PDCCH skipping and SSSG switching solutions should be considered as starting point for PDCCH monitoring enhancements. Proposal 4: RAN1 considers supporting non-scheduling DCI to trigger PDCCH monitoring enhancements. |
| CATT | Proposal 3: DRX enhancement for XR service should not affect other data services. UE monitors only XR scheduled PDCCH during a preconfigured PDCCH monitoring window during both DRX ON and OFF similar to that of DL SPS when C-DRX is configured. The XR-PMW schemes allows UE periodic monitoring PDCCH with resource allocation for various XR packet size through dynamic scheduling grant when C-DRX is configured as the power saving techniques of other services with robust traffic arrival type. Figure 2: XR-dedicated PDCCH monitoring window scheme Table 1: Evaluation results of XR-PMW with PDCCH skipping and go-to-sleep compared to Always-on Configuration; Considered UE set; Mean PSG compared to Always-on; #satisfied UEs per cell / #UEs per cell. Baseline: Always-on; -; -; 11.5 / 12. XR-PMW with go-to-sleep; All UEs; 24.01%; 10.8 / 12. Satisfied UEs; 24.39%. XR-PMW with PDCCH skipping and go-to-sleep; All UEs; 29.44%; 10.7 / 12. Satisfied UEs; 29.92%. Table 2: Evaluation results of XR-PMW with PDCCH skipping and go-to-sleep compared to C-DRX Configuration; Considered UE set; Mean PSG compared to C-DRX; #satisfied UEs per cell / #UEs per cell. Baseline: C-DRX(16,8,4); -; 0.0%; 6.5 / 12. XR-PMW with go-to-sleep; All UEs; 13.08%; 10.8 / 12. Satisfied UEs; 13.34%. XR-PMW with PDCCH skipping and go-to-sleep; All UEs; 19.30%; 10.7 / 12. Satisfied UEs; 19.68%. Proposal 4: The XR-dedicated PDCCH monitoring window should be studied and supported for XR UE power saving due to the good balance between system capacity and UE power saving. Table 3: The performance comparison between some enhanced schemes and XR-PMW with PDCCH skipping and go-to-sleep Scheme; Power saving; System capacity; Impact on existing service; Note. Mean PSG (%); Mean percentage of DL satisfied UE; Capacity (#Ues per cell); Dedicated to XR services. XR-PMW with PDCCH skipping and go-to-sleep; 29.68%; 89.2%; 12; Yes; Note1. Traffic arrival offset staggering[5]; 2.64~9.94%; 100%; 7~9; Yes; Note2,Note 3. e-PDCCH monitoring[6]; 5.08%~21.84%; 60%~84%; 5; No; Note2. eDRX[7]; 9.36%~23.84%; 91.25%~91.94%; 10; No; Note1. eDRX with additional active time[8]; 29.43%; 86.36%; 11; No; Note1. Note1: The evaluated scenario is indoor hotspot. Note2: The evaluated scenario is dense urban. Note3: With different traffic arrival offset, i.e., random or evenly spaced, system capacity is different.. Proposal 5: The XR-dedicated PDCCH monitoring window should be studied and supported for XR UE power saving due to the advantages that it doesn’t affect other data services and is achievable. |
| ITRI | Proposal 4: Further study on impacts of the “XR-dedicated PDCCH monitoring window” method to other UE behaviors. |
| Qualcomm | Observation 15: using multiple time windows with different configurations can help adapt to PDCCH monitoring density depending on the XR traffic patterns with reduced signaling overhead, latency, and complexity Proposal 11: For XR, consider studying a configuration of multiple time windows have different configurations for messages, signals, and/or operations, where switching between windows can be configured, dynamic, or implicit. Observation 12: Multi-PDSCH/PUSCH scheduling DCI is suitable for XR video data transmission for jitter handling, scheduling of large and variable size of video frame data and UE power saving by PDCCH monitoring reduction. Observation 13: If data of a XR video frame is scheduled by the same DCI, the number of schedulable PDSCHs may need to be increased beyond 8. Proposal 7: To support more efficient scheduling of XR video data, the single DCI scheduling multiple independent PDSCHs feature can be extended to SCS = 15, 30 and 60 kHz. Proposal 8: When more than 8 PDSCHs are scheduled by the same DCI, the RV field can be removed from the scheduling DCI to avoid the increase of DCI size. Observation 14: Disabling the retransmission timer allows the reduction of PDCCH monitoring and provides an average power saving gain of 21.0%. Proposal 9: NR supports disabling retransmission timer for the indicated Configured Grants(CG). Proposal 10: For XR, consider studying ways to have additional DL control signalling opportunities between sparsely configured semi-static SS set occasions by: - Dynamically configuring SS set occasions - Piggy-backing/multiplexing DL control signalling on already existing SCH messages (DG or SPS) |
| CATT | The configured period SPS occasion provides wake-up timing for XR UE during DRX OFF, and subsequent dynamic grant window ensures timely transmission of XR packets. When XR packet arrives after the wake-up timing, i.e., SPS occasion, gNB could indicate UE to perform PDCCH skipping until XR packet arrival and to transform to sleep state after XR packet transmission finish Proposal 6: The SPS enhancement with PDCCH skipping and go-to-sleep should be supported for XR UE power saving. |
| Samsung | For example, a UE may only receive one PDSCH for a XR traffic frame per DRX ON duration. Then, using a scheduling DCI format for triggering adaptation within DRX active time as in R17 may not work. For example, the triggering can be a GC-PDCCH, such as DCI format 2_6, before the next DRX ON duration. Observation 3: Rel-17 PDCCH monitoring adaptation triggered based on scheduling DCI format within active time of C-DRX may not work considering a short ON duration for XR service. Proposal 4: Support PDCCH monitoring adaptation triggered outside C-DRX ON duration for XR. Proposal 5: Support SSSG switching per TRP for XR A delay aware scheduler is one candidate solution for XR capacity enhancements. If the latency of a packet would exceed the PDB, there is no need to schedule (re)transmissions. Then, there is no need for the UE to monitor the PDCCH scheduling (re)transmissions. The corresponding enhancement for C-DRX can also be considered Proposal 3: Study C-DRX enhancements to reduce PDCCH monitoring time considering enhancements for the following - SPS PDSCH/CG PUSCH - Delay aware scheduler |
| Nokia | Figure 7 – Example of adaptive PDCCH sparse monitoring for values ks = 1 and ks > 1 Observation 11: XR services could benefit from an adaptive PDCCH monitoring approach, with monitoring periodicity changing depending on the traffic behaviour. Proposal 9: Study the influence of adapting the PDCCH monitoring periodicity in XR services according to traffic characteristics and behaviour, focusing also on the impact the delays/interruptions associated with changing the PDCCH monitoring periodicity could have on the system capacity. |

Below is a summary of all proposals that are generally related to adaptation of UE PDCCH monitoring. Some of the proposals are related to discussions in Rel-17 WI on potential extension(s) to Rel-16 DCI-based power saving adaptation during DRX ActiveTime.

PDCCH monitoring adaptation triggered by non-scheduling DCI

[CATT], [CMCC], [LGE], [Intel] and [Samsung] proposed in the Rel-18 XR agenda to use non-scheduling DCI format to trigger the PDCCH monitoring adaptation. The intention is to allow UE to wait for potential additional scheduling (and avoid extra latency for the scheduled data) and skip PDCCH monitoring by the non-scheduling DCI in case the scheduling did not occur. [CATT], [CMCC] and [vivo] proposed to provide the triggering indication by repurposing DCI field in existing DL/UL data scheduling DCI such as format 1_1 to avoid DCI size increase. [LGE] [Samsung] proposed to use WUS outside the CDRX ON duration to indicate the PDCCH monitoring adaptation. This topic was discussed in Rel-17 UE power saving WI.

XR-dedicated PDCCH monitoring window

[CATT], [ITRI] and [Qualcomm] proposed to introduce the XR-dedicated PDCCH monitoring window. Network configures the XR-dedicated PDCCH monitoring windows for XR service and CDRX for other services. This intends to match the overall (by the window and CDRX) PDCCH monitoring ON/OFF pattern with multiple XR traffic flows without introducing multiple CDRX configurations. [Qualcomm] proposed to extend the XR-dedicated window mechanism to general UL and DL activities including SPS, CG, TRS and SRS, etc. and adaptively switch between multiple windows.

Additional PDCCH skipping durations

[vivo] and [CMCC] proposed to configure a larger number (more than the three values supported by Rel-17) of PDCCH skipping duration values to achieve the one-shot skipping from the last scheduling grant to the jitter boundary of the next XR traffic arrival. This intends to have a seamless PDCCH skipping duration more often rather than concatenation of multiple shorter PDCCH skipping monitoring durations so that UE has longer contiguous sleep duration.

PDCCH skipping interaction with HARQ retransmission

This topic was discussed in Rel-17 UE power saving WI. [MediaTek] and [vivo] proposed to continue discussing the interaction between PDCCH skipping with HARQ retransmission in the Rel-18 XR context. [MediaTek] proposed to cancel the skipping of PDCCH monitoring if a NACK is transmitted or if UL data is transmitted after gNB has indicated PDCCH monitoring skipping.

Dynamic adaptation of PDCCH monitoring configuration

[OPPO], [Nokia], [Qualcomm] proposed to dynamically adapt periodicity, offset, duration of PDCCH monitoring.  [Qualcomm] proposed to add PDCCH monitoring occasions during sparse PDCCH monitoring. The mechanism can be applied to search space set configuration or XR-specific PDCCH monitoring window. This is a more general adaptation mechanism in compared to “Dynamically adjust the start offset of PDCCH monitoring” technique proposed under the enhanced PDCCH monitoring topic.

PDCCH monitoring adaptation indication by SPS

[CATT] and [Qualcomm] proposed the enhanced SPS providing PDCCH monitoring adaptation indication in both DRX ON and OFF to activate additional PDCCH monitoring to receive additional data or to skip PDCCH monitoring until the next XR data arrival after the current XR data packet is completely received.

Disabling UL retransmission for PDCCH monitoring reduction

[MediaTek] and [Qualcomm] proposed to disable the retransmission for UL pose/control information, e.g., when UL pose/control information is transmitted by UL CG. This intends to save UE power otherwise would be used for PDCCH monitoring when retransmission timer is running.

PDCCH monitoring skipping by delay awareness

[Samsung], [Apple], [Lenovo] and [LGE] proposed to use delay awareness for UE to skip monitoring PDCCH that schedules (re)transmissions if the latency of a packet would exceed the PDB.

### Other Enhancements

Blow is a summary of proposals for PDCCH monitoring adaptation submitted from a single company.

Single DCI indicating monitoring adaptation for multiple cells

[vivo] proposed to continue discussing the leftover topic from the Rel-17 UE power saving WI.

PDCCH monitoring adaptation applying for Type 0-2 CSS sets

[vivo] proposed to continue discussing the leftover topic from the Rel-17 UE power saving WI.

One DCI scheduling more than 8 PDSCHs/PUSCHs

[Qualcomm] proposed to extend the Rel-17 FR2-2 multi-PDSCH/PUSCH scheduling feature to FR1 and FR2-1 for XR traffic scheduling. Remove RV field in the scheduling DCI format to avoid DCI size increase when the number of schedulable PDSCHs/PUSCHs is increased beyond the largest number 8 supported by Rel-17.

DCI linkage for BD reduction

[Sony] proposed to add the linkage information in the first DCI so that once the first DCI is detected, reduced PDCCH monitoring of the second DCI can be performed within subset of PDCCH BD attempts.

SSSG switching per TRP

[Samsung] proposed SSSG switching per TRP for XR if SSSG is predetermined per TRP or per group of CORESETs according to associated CORESETPoolIndex. It intends to achieve the fast adaptation on spatial domain resources.

### Summary

Evaluation results related to PDCCH monitoring enhancements are collected in Tables 3.2-1 to 3.2-3 below.

Table 3.2-1 PDCCH monitoring adaptation related power saving evaluation results: FR1, DL only, Indoor Hotspot, VR 30Mbps

| Tdoc# | Power Saving scheme | avg # Ues/Cell | floor (Capacity) | % of satisfied Ues | Mean PSG of all Ues | Mean PSG of satisfied Ues | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2203585 | R17 PDCCH skipping | 5 | 10 | 100% | 18.15% | 18.15% | Note 1 |
| R1-2203585 | R17 PDCCH skipping and SSSG switching | 5 | 10 | 100% | 22.04% | 22.04% | Note 2,4 |
| R1-2203585 | R17 PDCCH skipping and SSSG switching | 5 | 10 | 100% | 23.73% | 23.73% | Note 2,5 |
| R1-2203585 | R17 PDCCH skipping and SSSG switching | 5 | 10 | 100% | 24.60% | 24.60% | Note 2,6 |
| R1-2203585 | Enhanced PDCCH skipping | 5 |  | 100% | 25.30% | 25.30% | Note 3,7 |
| R1-2203585 | Enhanced PDCCH skipping and SSSG switching | 5 | 10 | 100% | 30.73% | 30.73% | Note 4,7 |
| R1-2203585 | Enhanced PDCCH skipping and SSSG switching | 5 | 10 | 100% | 32.55% | 32.55% | Note 5,7 |
| R1-2203585 | Enhanced PDCCH skipping and SSSG switching | 5 | 10 | 100% | 33.44% | 33.44% | Note 6,7 |
| R1-2203585 | R17 PDCCH skipping | 10 | 10 | 92.50% | 15.48% | 15.67% | Note 1 |
| R1-2203585 | R17 PDCCH skipping and SSSG switching | 10 | 10 | 92.50% | 19.17% | 19.38% | Note 2,4 |
| R1-2203585 | R17 PDCCH skipping and SSSG switching | 10 | 10 | 92.50% | 20.78% | 20.99% | Note 2,5 |
| R1-2203585 | R17 PDCCH skipping and SSSG switching | 10 | 10 | 92.50% | 21.58% | 21.83% | Note 2,6 |
| R1-2203585 | Enhanced PDCCH skipping | 10 | 10 | 92.50% | 21.01% | 21.27% | Note 3,7 |
| R1-2203585 | Enhanced PDCCH skipping and SSSG switching | 10 | 10 | 92.50% | 26.10% | 26.38% | Note 4,7 |
| R1-2203585 | Enhanced PDCCH skipping and SSSG switching | 10 | 10 | 92.50% | 27.80% | 28.09% | Note 5,7 |
| R1-2203585 | Enhanced PDCCH skipping and SSSG switching | 10 | 10 | 92.50% | 28.65% | 28.94% | Note 6,7 |
| Note 1: three candidate PDCCH skipping durations are configured {6ms 4ms 2ms} Note 2: two candidate PDCCH skipping durations are configured {6ms 4ms} Note 3: PDCCH monitoring periodicity in the sparse SSSG is 1 slot Note 4: PDCCH monitoring periodicity in the sparse SSSG is 2 slot Note 5: PDCCH monitoring periodicity in the sparse SSSG is 3 slot Note 6: PDCCH monitoring periodicity in the sparse SSSG is 4 slot Note 7: skip to the -jitter boundary of the next frame |  |  |  |  |  |  |  |

Table 3.2-2 PDCCH monitoring adaptation related power saving evaluation results: FR1, DL only, Indoor Hotspot, VR 45Mbps

| Tdoc# | Power Saving scheme | avg # Ues/Cell | floor(Capacity) | % of satisfied Ues | Mean PSG of all Ues | Mean PSG of satisfied Ues | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2203585 | R17 PDCCH skipping | 3 | 5 | 100% | 16.47% | 16.47% | Note 1 |
| R1-2203585 | R17 PDCCH skipping and SSSG switching | 3 | 5 | 100% | 20.28% | 20.28% | Note 2,4 |
| R1-2203585 | R17 PDCCH skipping and SSSG switching | 3 | 5 | 100% | 21.91% | 21.91% | Note 2,5 |
| R1-2203585 | R17 PDCCH skipping and SSSG switching | 3 | 5 | 100% | 22.73% | 22.73% | Note 2,6 |
| R1-2203585 | Enhanced PDCCH skipping | 3 | 5 | 100% | 22.59% | 22.59% | Note 3,7 |
| R1-2203585 | Enhanced PDCCH skipping and SSSG switching | 3 | 5 | 100% | 27.79% | 27.79% | Note 4,7 |
| R1-2203585 | Enhanced PDCCH skipping and SSSG switching | 3 | 5 | 100% | 29.52% | 29.52% | Note 5,7 |
| R1-2203585 | Enhanced PDCCH skipping and SSSG switching | 3 | 5 | 100% | 30.38% | 30.38% | Note 6,7 |
| R1-2203585 | R17 PDCCH skipping | 5 | 5 | 96.67% | 14.40% | 14.62% | Note 1 |
| R1-2203585 | R17 PDCCH skipping and SSSG switching | 5 | 5 | 96.67% | 18.01% | 18.23% | Note 2,4 |
| R1-2203585 | R17 PDCCH skipping and SSSG switching | 5 | 5 | 96.67% | 19.55% | 19.78% | Note 2,5 |
| R1-2203585 | R17 PDCCH skipping and SSSG switching | 5 | 5 | 96.67% | 20.38% | 20.58% | Note 2,6 |
| R1-2203585 | Enhanced PDCCH skipping | 5 | 5 | 96.67% | 19.20% | 19.45% | Note 3,7 |
| R1-2203585 | Enhanced PDCCH skipping and SSSG switching | 5 | 5 | 96.67% | 24.14% | 24.41% | Note 4,7 |
| R1-2203585 | Enhanced PDCCH skipping and SSSG switching | 5 | 5 | 96.67% | 25.80% | 26.08% | Note 5,7 |
| R1-2203585 | Enhanced PDCCH skipping and SSSG switching | 5 | 5 | 96.67% | 26.60% | 26.89% | Note 6,7 |
| Note 1: three candidate PDCCH skipping durations are configured {6ms 4ms 2ms} Note 2: two candidate PDCCH skipping durations are configured {6ms 4ms} Note 3: PDCCH monitoring periodicity in the sparse SSSG is 1 slot Note 4: PDCCH monitoring periodicity in the sparse SSSG is 2 slot Note 5: PDCCH monitoring periodicity in the sparse SSSG is 3 slot Note 6: PDCCH monitoring periodicity in the sparse SSSG is 4 slot Note 7: skip to the -jitter boundary of the next frame |  |  |  |  |  |  |  |

Table 3.2-3 PDCCH monitoring adaptation related power saving evaluation results: FR1, DL+UL, Dense Urban, VR 30Mbps

| Tdoc# | Power Saving scheme | avg # Ues/Cell | floor(Capacity) | % of satisfied Ues | Mean PSG of all Ues | Mean PSG of satisfied Ues | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2205176 | Retransmission-less CG | 1 | - | 97.14% | 21.0% | - | Note 1 |
| R1-2205176 | Retransmission-less CG | 10 | - | 92.62% | 20.0% | - | Note 1 |
| Note 1: eCDRX (cycle, on duration timer, inactivity timer, retransmission timer) = (16/17/17, 4, 4, 4)ms |  |  |  |  |  |  |  |

Based on the companies’ evaluation results in Table 3.2-1 to Table 3.2-3, FL provides the following observation.

[RD1] Observation 3.2-1: observations are removed after 1st round discussion based on companies’ feedback.

[RD1] Question 3.2-1: companies’ views on Table 3.2-1 to 3.2-3 and Observation 3.2-1

| Company | Comments |
| --- | --- |
| MTK | We are generally fine with Table 3.2-1 to 3.2-3 and Observation 3.2-1. It would be better to briefly describe the mechanism of “Legacy PDCCH monitoring adaptation” and “Enhancements to PDCCH monitoring adaptation”. For example, which one is based on the currently available R17 power saving techniques (for PDCCH monitoring adaptation)? |
| Xiaomi | OK with the proposal. And to make it more clear, “legacy PDCCH monitoring adaptation ” can be changed as “R17 PDCCH monitoring adaptation ”, if my understanding for “legacy PDCCH monitoring adaptation ” is right. |
| Samsung | OK with Tables 3.2-1 to 3.2-3 and Observation 3.2-1 |
| LGE | We are generally fine with FL’s observations, but how to capture in the TR can be discussed later. |
| Nokia, NSB | As commented, while we appreciate the FL hard work and it is interesting to see the summarised results, but it does not seem necessary to agree actual observation at this stage. At least, we should consider further before trying to do any conclusions based on these. Also we can later consider what results and how to capture those on the TR, based on further agreements on the schemes. As commented in Section 2 we would prefer to evaluate the capacity as the maximum number of Ues for which the percentage of satisfied Ues is larger than ~90% (i.e., using the definition of capacity in capacity evaluation methodology) for each scheme individually and evaluate the effect in more comparable manner. |
| Intel | Similar comment as mentioned in response to 3.1- 1. Collection of observations can be done after agreeing to evaluation assumptions, and a common framework, including baselines. |
| CATT | We had shown the results of power saving gain and system capacity of XR-PMW comparing to always on and C-DRX configuration. It would be nice to show the reference configuration of each power saving techniques. The MAC control go-to-sleep in DRX configuration is one of the XR-specific solution for reducing PDCCH monitoring as gNB indicate UE to sleep when XR specific packets has been completed in transmission at each cycle. |
| OPPO | Same concern/comment as provided in Question 3.1-1. We suggest to wait for one or two meeting cycles to collect results from sufficient number of companies, and then do a batch observation, instead of making numerous observations in each of meetings upon every new result arrival. |
| InterDigital | We prefer to first confirm/agree on evaluation assumptions and methodology before discussing any observations on enhancement schemes. |
| ZTE | In our understanding, the tables and observations are for result collection so far, and companies are expected to provide more evaluations. And corresponding power saving scheme e.g., R17 PDCCH skipping and/or SSSG switching should be noted. |
| ITRI | We share the same view as InterDigital. |
| NTT DOCOMO | Thanks for the effort of summarizing companies’ evaluation results. Although we think the table is generally good and beneficial, the evaluation methodology and assumption including what is a marginal loss in capacity can be considered and clarified before trying to agree it. |
| Spreadtrum | We are fine with Table 3.2-1 to 3.2-3 and Observation 3.2-1. |
| Huawei, HiSilicon | Similar comment as in Question 3.1-1, we this it’s pre-mature to draw conclusions on simulation results. We suggest to drop such discussions in this meeting. |
| Moderator | Companies showed concerns on making observations in this meeting based on evaluation results from a limited number of companies without performance calibration. To address these concerns, FL removed the Observations to only keep the raw evaluation results in the Tables. Please provide comment if you do not agree with this. |

Based on various proposed enhancements in Section 3.2.1, 3.2.2 and 3.2.3, the following main issues and PDCCH monitoring enhancements are identified:

- Topic 2: PDCCH monitoring enhancements

  - Issue 2-1: Alignment between PDCCH monitoring and XR traffic to resolve the mismatch between PDCCH monitoring periodicity and XR traffic periodicity. The proposed solutions include

    - Semi-static pattern of PDCCH monitoring

    - Dynamic adjustment of the start offset of PDCCH monitoring

  - Issue 2-2: XR-dedicated PDCCH monitoring window. The enhancements include

    - Using window based PDCCH monitoring to replace CDRX for scheduling XR traffic

    - Window-based transmission for general UL/DL channel and signal

  - Issue 2-3: Enhancements to Rel-17 PDCCH monitoring adaptation. The solutions include

    - PDCCH monitoring adaptation triggered by non-scheduling DCI

    - Additional PDCCH skipping durations (i.e., >3 values)

    - PDCCH skipping interaction with HARQ retransmission by cancelling PDCCH skipping or disabling retransmission

[RD1] Question 3.2-2: companies’ views on Topic 2 and the Issues.

| Company | Comments |
| --- | --- |
| MTK | We are generally fine with Topic 2 and the Issues. Suggest the following revision for Issue 2-3: - Issue 2-3: Enhancements to Rel-17 PDCCH monitoring adaptation. The solutions include - PDCCH monitoring adaptation triggered by non-scheduling DCI - Additional PDCCH skipping durations (i.e., >3 values) - PDCCH skipping interaction with HARQ retransmission by cancelling PDCCH skipping or disabling retransmission or an additional monitoring window for retransmission during the skipping period |
| Xiaomi | Similar as our comments above, maybe we can add another bullet saying other schemes are not precluded. |
| NEC | We think the Issue 2-1 is more essential than others. On alignment for PDCCH monitoring and XR traffic in time domain, considering the lower delay tolerance and shorter XR traffic periodicity, we propose to specify XR specific PDCCH monitoring offset parameters such as k(µ) and jitter in search space set configuration, signalling overhead is reduced significantly compared to Rel-17 based dynamic search space switching/skipping. K(µ) = ⌈ p *(1000ms/fps*2^µ - Periodicity) ⌉ - The ‘fps’ (frame per second) is the frame rate for XR. A DCI field (such as the reserved value of PDCCH monitoring adaptation field) may be used to set the jitter offset from time to time (start offset of PDCCH monitoring may shift according to +/- ‘jitter offset’ value). Proposal 1: Specify XR specific PDCCH monitoring offset parameters such as k(µ) and jitter in Search Space Set configuration. Proposal 2: Support DCI based dynamic adaptation of XR specific PDCCH monitoring offset such as ‘jitter’. Proposal 3: Specify a higher layer parameter of ‘frame per second’ for the frame rate of XR traffic. |
| Samsung | OK with the outline for further discussion but we would like to again suggest to start considering some reduction at this meeting. For example, we do not think Issue 2-2 needs to be further considered as there cannot be XR-dedicated PDCCH monitoring window because a UE will need to monitor PDCCH for several other purposes such as for SI or for Type3-PDCCH CSS. There are also the Rel-17 mechanisms for SSSG switching and PDCCH skipping that can in principle achieve any windowing. The same also largely applies for Issue 2-1. It is noted that DCI overhead is negligible compared to the size of XR traffic and there will be very few XR Ues at a given time. [Moderator] Thanks for the comments. Please provide further input in the following rounds discussions where some down selection could be made. |
| Vivo | We are fine with the identified issue 2-1. For issue 2-2 and issue 2-3, it seems current descriptions are based on the solution itself. What use cases and what XR specific issues these solutions tend to address need further clarifications. [Moderator] Thanks for the comments. Some clarification is added to explain the issues to resolve. Besides, as commented in Question 3.1-2, the description of some of the solutions seem to be much detailed, so we kindly suggest to provide some high level solutions for addressing XR issues and more concrete solutions can be discussed further. |
| LGE | We are generally okay with Topic 2 and the Issues. |
| Nokia, NSB | Like noted earlier, we are supportive of identifying the key issues from power saving perspective e.g. during onDuration with jitter, but it is probably premature to close door from other methods (or combination of them), thus listed schemes could be more high level and exemplary. Also as commented earlier, if we for example consider enhancement to C-DRX to address the non-integer periodicity, the need for other solutions to address the same issue should well justified. |
| Intel | For Issue 2-1 and Issue 2-2, it needs to be clarified whether Rel-17 PDCCH monitoring adaptation can solve the issues or achieve similar intended outcome or not. In our view, we need to first list potential issues that cannot be solved using existing methods. [Moderator] Thanks for the comments. Notes are added to trigger further discussion in the following rounds. |
| CATT | We are OK with the categories. However, MAC control go-to-sleep indication is part of Issue 2-1 and Issue 2-2 |
| OPPO | Issue 2-1: The 2nd sub-bullet is restricting. We prefer not to perform down selection at this stage for the PDCCH monitoring parameters applicable to dynamic adjustment. So we prefer the following revision. - Dynamic adjustment of the start offset/duration/periodicity of PDCCH monitoring |
| SONY | We are fine with the scope to be studied on topic 2. In addition, we propose DCI linkage for BD reduction as part of issue 2-3. - Issue 2-3: Enhancements to Rel-17 PDCCH monitoring adaptation. The solutions include - PDCCH monitoring adaptation triggered by non-scheduling DCI - Additional PDCCH skipping durations (i.e., >3 values) - PDCCH skipping interaction with HARQ retransmission by cancelling PDCCH skipping or disabling retransmission - DCI linkage for BD reduction |
| InterDigital | We share similar understanding with NEC that Issue 2-1 is more important and hence, we believe focusing on the solutions described under Issue 2-1 can be beneficial in terms of evaluation and potential standards effort. Regarding Issues 2-2 and 2-3, we are open to considering the solutions listed under these issues, although further clarification/analysis is needed on whether and how such solutions are different and advantages over those listed under Issue 2-1. |
| Qualcomm | As mentioned earlier, we prefer to keep study options open at this point. So we are fine with the list above. In addition, as mentioned in our paper, for XR traffic patterns framework, there may be a need to sparse configure search space sets for power savings (e.g., multi-PDSCH/PUSCH scheduling). In such cases, there may still be the need to accommodate some low latency traffic that may arise un-expectedly within this sparse SS sets. Hence, along with the sparse SS set occasions, there may be a need to have additional DCIs, where additional temporary DL control occasions are allocated within. We would like to propose studying ways to have additional DL control signalling opportunities between sparsely configured semi-static SS set occasions (e.g., dynamically configuring SS set occasions, or piggy-backing/multiplexing DL control signalling on SCH messages). We suggest the following addition: - Topic 2: PDCCH monitoring enhancements - Issue 2-1: Alignment between PDCCH monitoring and XR traffic to resolve the mismatch between PDCCH monitoring periodicity and XR traffic periodicity. The proposed solutions include - Semi-static pattern of PDCCH monitoring - Dynamic adjustment of the start offset of PDCCH monitoring - Issue 2-2: XR-dedicated PDCCH monitoring window. The enhancements include - Using window based PDCCH monitoring to replace CDRX for scheduling XR traffic - Window-based transmission for general UL/DL channel and signal - Issue 2-3: Enhancements to Rel-17 PDCCH monitoring adaptation. The solutions include - PDCCH monitoring adaptation triggered by non-scheduling DCI - Additional PDCCH skipping durations (i.e., >3 values) - PDCCH skipping interaction with HARQ retransmission by cancelling PDCCH skipping or disabling retransmission - Additional DL control signalling opportunities between sparsely configured semi-static SS set occasions (e.g., dynamically configuring SS set occasions, or piggy-backing/multiplexing DL control signalling on SCH messages) |
| ZTE | For issue 2-1, it should be differentiated with the issue in section 3.1.1. And duplicated study should be avoided. For issue 2-2, as to XR-dedicated PDCCH monitoring window, the motivation need to be further identified in the group. For issue 2-3, regarding the Rel-17 leftover issues, since they’re still under discussion in Rel-17 maintenance phase, therefore they can be postponed. |
| ITRI | We support the listed issues and Xiaomi’s proposal on saying “other schemes are not precluded”. |
| NTT DOCOMO | We are generally fine with the listed issues. Since this is the first meeting, other schemes should not be precluded, which needs to be mentioned in proposal. Also, the relationship/linkage between PDCCH monitoring enhancement scheme and CDRX enhancement scheme should be considered, e.g., both of PDCCH monitoring enhancement scheme and CDRX enhancement scheme can be configured and they can work jointly, or either of them is configured depending on XR traffic characteristics and so on. |
| Spreadtrum | We are fine with Topic 2 and the corresponding solutions listed under each issue. |
| Lenovo | Considering additional PDCCH skipping durations (i.e., >3 values) seems to be the easiest/simplest enhancement for PDCCH monitoring in the presence of traffic jitter |
| CMCC | Regarding the issue 2-3 we think a unified solution can be studied merged with issue 1-2 in section 3.1.5 which a unified PDCCH monitoring adaption can be used in CDRX scenario and non-CDRX scenario. Regarding the issue 2-2, we think current SS design e.g., dedicated SS for XR PDCCH monitoring can be used to address this issue, more discussion is needed. |
| Huawei, HiSilicon | For Issue 2-1, it is similar to Issue 1-1 for CDRX enhancement, which is to address the mismatch between CDRX/PDCCH monitoring periodicity and XR traffic periodicity. This issue can be discussed together with CDRX enhancement Issue 1-1 and maybe have a unified design to save workload. We have similar comments here. Both dynamic and semi-static adjustment of start offset can be further studied. For Issue 2-2, similar to R15/R16 CDRX, the existing search space configurations allow UE to periodically monitor PDCCH for some time (i.e., a PDCCH monitoring window), which is realized by configuring a periodicity, an offset, and a duration. We are not clear what is “XR-dedicated PDCCH monitoring window” and what enhancements are needed to support “XR-dedicated PDCCH monitoring window”. We are wondering whether the intention of “XR-specific PDCCH monitoring window” is to align PDCCH monitoring duration with XR traffic. If so, this issue can be discussed together with Issue 2-1 to address the mismatch between PDCCH monitoring periodicity and XR traffic periodicity. For Issue 2-3, the enhancements to Rel-17 PDCCH monitoring adaptation can be considered for further studied, which can allow skipping unnecessary PDCCH monitoring caused by XR traffic jitter. RAN1 should only focus on XR-specific issues. In summary, we suggest the following changes in red: == - Topic 2: PDCCH monitoring enhancements - Issue 2-1: Alignment between PDCCH monitoring and XR traffic to resolve the mismatch between PDCCH monitoring periodicity and XR traffic periodicity. The proposed solutions include - Semi-static Non-uniform pattern of PDCCH monitoring, e.g., {16,17,17}ms - Dynamic adjustment of the start offset of PDCCH monitoring - Issue 2-2: XR-dedicated PDCCH monitoring window. The enhancements include - Using window based PDCCH monitoring to replace CDRX for scheduling XR traffic - Window-based transmission for general UL/DL channel and signal - Issue 2-3: Enhancements to Rel-17 PDCCH monitoring adaptation. The solutions include - PDCCH monitoring adaptation triggered by non-scheduling DCI - Additional PDCCH skipping durations (i.e., >3 values) - PDCCH skipping interaction with HARQ retransmission by cancelling PDCCH skipping or disabling retransmission |
| Apple | In our view, SPS/CG are useful features for XR. To tackle variying packet size and other issues, we suggest the followingadditions: - Topic 2: PDCCH monitoring enhancements - Issue 2-1: Alignment between PDCCH monitoring and XR traffic to resolve the mismatch between PDCCH monitoring periodicity and XR traffic periodicity. The proposed solutions include - Semi-static pattern of PDCCH monitoring - Dynamic adjustment of the start offset of PDCCH monitoring - Issue 2-2: XR-dedicated PDCCH monitoring window. The enhancements include - Using window based PDCCH monitoring to replace CDRX for scheduling XR traffic - Window-based transmission for general UL/DL channel and signal - PDCCH monitoring accomanying SPS/CG configuration. - Issue 2-3: Enhancements to Rel-17 PDCCH monitoring adaptation. The solutions include - PDCCH monitoring adaptation triggered by non-scheduling DCI - Additional PDCCH skipping durations (i.e., >3 values) - PDCCH skipping interaction with HARQ retransmission by cancelling PDCCH skipping or disabling retransmission for both DG and SPS PDSCH, and DG and CG PUSCH |
| Moderator | Please check updated proposal in Proposal 3.2-1. |

Summary of 1st round discussion on Question 3.1-2:

FL would like to thank companies for the valuable inputs on the improvement of the list of issues. Based on these inputs, the following updated proposal is provided. Again, if a proposal is both supported and objected by companies, FL will keep it from 1st round discussion and leave the down selection to further discussions and evaluation results.

[RD2] Proposal 3.2-1: For power saving study of Rel-18 XR SI, the following areas for potential PDCCH monitoring enhancements are identified

- Issue 2-1: Alignment between PDCCH monitoring and XR traffic to resolve the mismatch between PDCCH monitoring periodicity and XR traffic periodicity. The proposed solutions include

  - Non-uniform pattern of PDCCH monitoring, e.g., {16,17,17}ms

  - Adjustment of the start offset/duration/periodicity of PDCCH monitoring

  - MAC control go-to-sleep indication

  - Other considerations are not precluded

  - Note: some companies suggest clarifying whether Rel-17 PDCCH monitoring adaptation can solve issue 2-1 or achieve similar intended outcome or not

- Issue 2-2: XR-dedicated PDCCH monitoring window to supplement CDRX for multi-flow traffic. The enhancements include

  - Using window based PDCCH monitoring to replace CDRX for scheduling XR traffic

  - Window-based transmission for general UL/DL channel and signal

  - MAC control go-to-sleep indication

  - PDCCH monitoring accompanying SPS/CG configuration

  - Other considerations are not precluded

  - Note: some companies suggest clarifying whether Rel-17 PDCCH monitoring adaptation can solve issue 2-2 or achieve similar intended outcome or not

- Issue 2-3: Enhancements to Rel-17 PDCCH monitoring adaptation. The solutions include

  - PDCCH monitoring adaptation triggered by non-scheduling DCI to avoid missing additional data scheduling

  - Additional PDCCH skipping durations (i.e., >3 values) for maximizing UE sleep duration

  - PDCCH skipping interaction with HARQ retransmission by cancelling PDCCH skipping or disabling retransmission or an additional monitoring window for retransmission during the skipping period for both DG and SPS PDSCH, and DG and CG PUSCH

    - This is to avoid additional latency for retransmission due to PDCCH skipping.

  - DCI linkage for BD reduction

  - Additional DL control signalling opportunities between sparsely configured semi-static SS set occasions (e.g., dynamically configuring SS set occasions, or piggy-backing/multiplexing DL control signalling on SCH messages)

  - Non-uniform PDCCH monitoring occasions within On Duration matched to jitter distribution

  - Other considerations are not precluded

  - Note: when to discuss some enhancements may depend on the outcome of Rel-17 PDCCH monitoring adaptation maintenance

Note 1: these enhancements are subject to down selection based on technical discussions on the necessity and evaluation results

Companies view on [RD2] Proposal 3.2-1

Goals of this round of discussion:

- Identify high and lower priority issues under Proposal 3.2-1

  - Especially issues that you do not think should be studied in Rel-18 XR SI

- Identify proposed enhancements for that there is existing spec solution achieving same or similar effect

| Company | Comments/Reasons |
| --- | --- |

Other Power Saving Enhancements

This section summarizes remaining proposals for XR power saving enhancements.

### UL Power Saving Enhancements

Table 10 captures companies’ views on UL related power saving.

Table 10: Proposals and views on UL power saving

| Company | Views |
| --- | --- |
| ZTE, Sanechips | If the UL signaling transmitted irrelevant with the DL signaling, the sum of power consumption will be increased. For example, if UE transmits a SR outside DRX active time, UE will wake up and monitor PDCCH for potential UL grant, and increase the monitoring occasion of PDCCH. Therefore, UL power saving scheme should also be considered, especially regarding DL & UL alignment schemes. Some UL power saving schemes and corresponding evaluation results are provided in our companion contribution [5]. Proposal 4: For UL, power saving schemes and corresponding evaluations should be considered. |
| NEC | Proposal 7: Study the mechanisms for DL and UL alignment to further reduce the power consumption. |
| CATT | Figure 5: Illustration of PUSCH transmission and HARQ feedback indicated by the same DCI. Proposal 8: How to reduce UL power consumption caused by UL data transmission and UL control information feedback should be studied for potential power saving gain for XR service. Proposal 9: Alignment between PUSCH transmission and HARQ feedback should be studied and supported to reduce UL power consumption for XR UE, e.g., single DCI indicating UL transmission and DL transmission (including HARQ feedback for DL reception) simultaneously. |
| Lenovo | Proposal 3: Study the power-saving vs. delay trade-off of postponing HARQ-ACK of a PDSCH to a nearest UL resource that is close to a later PDSCH. |
| MediaTek | Figure 1: An example for three AR DL/UL traffic flows Figure 2: An example for three AR DL/UL traffic flows, with 4.167 ms period (240Hz) UL pose/control Observation 1: The UL pose/control traffic doesn’t align with the video traffic in time. Observation 2: The UL pose/control period does not strictly have to be 4ms but can depend on the application implementation. Observation 3: In cell phone industry, the display refresh period is an integer multiple of touch (pose/control) refresh period. Observation 4: UL/DL alignment can be achieved with an UL pose/control of 4.167 ms period (240Hz). Proposal 3: RAN1 to revisit the UL pose/control period assumption. One of two options to adopt: - Option 1: Modify UL pose/control period assumption to be 4.167 ms (240Hz). - Option 2: RAN1 to draw a conclusion that UL/DL alignment can be handled by application and RAN1 does not have to address the alignment issue. Proposal 4: RAN1 to adopt Figure 1 or Figure 2 as a common UL/DL traffic arrival model to evaluate the power saving gain under the potential future enhancements. |
| Apple | Proposal 3-3: investigate inband signaling to indicate varying packet size and absence/presence of SPS PDSCH. Figure 5 Using CG-UCI to adapt the transport block size |
| Qualcomm | Figure 17: An overallocation of PUSCH is partially utilized for transmission of TB with UL data only and no padding Observation 16: CG reduces the overhead of a scheduling DCI and provides lower latencies compared to SR/BSR scheduling. However, semi-static configuration of the resource allocation may not adapt to varying packet size and may not closely approximate the required resources to transmit the available data at the UE. An overallocation results in an increase of PUSCH transmit power which increases power consumption. Observation 17: Rel-15 Uplink Skipping allows a UE to completely skip a transmission in the UL but does not allow the UE to transmit over a sufficient allocation just enough to transmit the UL data. Observation 18: A UE that can transmit over a subset of the configured grant allows larger power savings. For VR application with uplink pose/control, a ~21% decrease in power consumption is observed assuming pose/control flow in UL. Observation 19: Partial PUSCH transmission can allow power savings for both CG and DG. Observation 20: For proper demodulation in the UL, gNB is required to know which of the UL resources the UE has utilized or skipped. Proposal 12: To reduce power consumption, study partial uplink transmission and investigate necessary signaling to enable it. |

Besides the CDRX and PDCCH monitoring enhancements, companies also proposed power saving enhancements when UL transmission is involved. Below is a summary of the proposals.

Power saving enhancements by DL & UL alignment

[ZTE, Sanechips], [NEC], [Lenovo] and [MediaTek] proposed to reduce the overall UE power consumption by aligning DL & UL activities so that UE has longer sleep time. [Lenovo] proposed to postpone HARQ-ACK of a PDSCH to a nearest UL resource that is close to a later PDSCH. [MediaTek] proposed to modify UL pose/control period to be 4.167 ms so that every 4th UL pose/control transmission is aligned with DL video reception.

PUSCH and PUCCH alignment by single DCI scheduling UL and DL transmissions

[CATT] proposed to use a single DCI to indicate UL transmission and DL transmission (including HARQ feedback for DL reception) simultaneously so that PUSCH transmission can be aligned with HARQ feedback for the DL reception.

Uplink transmission adaptation for CG and DG

[Apple] and [Qualcomm] proposed to indicate to gNB the resources where UL data is transmitted. The intention is to avoid power consumption otherwise would be used for the transmission of padding bits when more UL resources are allocated needed for DG or CG.

### Other Enhancements

Table 12: Proposals and views on remaining enhancements

| Company | Views |
| --- | --- |
| CATT | Table 4: Evaluation of power saving cases with gNB scheduling awareness of UE playout buffer Power saving cases with UE playout buffer; Considered UE set; Mean Power Saving Gain (PSG) compared to Always-on; #satisfied UEs per cell / #UEs per cell. Baseline; -; -; 11.5 / 12. C-DRX(16,8,4) with go-to-sleep; All UEs; 26.43%; 11.3 / 12. Satisfied UEs; 26.78%. C-DRX(16,8,4) with PDCCH skipping and go-to-sleep; All UEs; 34.56%; 11.2 / 12. Satisfied UEs; 34.77%. PDCCH skipping; All UEs; 30.26%; 11 / 12. Satisfied UEs; 30.36%. Observation 1: Large power saving gain can be obtained by UE playout buffer with negligible capacity degradation. Proposal 7: gNB awareness of UE playout buffer should be studied and supported for XR UE power saving. |
| China Telecom | Proposal 7: CBG based SPS transmission supporting multiple TBs should be studied. The enhancements of HARQ-ACK feedback for CBG based SPS PDSCH transmission and HARQ codebook construction need FFS. Proposal 8: Repetitions in frequency domain may need FFS. |
| Apple | Proposal 3-3: investigate inband signaling to indicate varying packet size and absence/presence of SPS PDSCH. Proposal 3-4: for UE power saving, support non-integer periodicity for periodic/semi-persistent CSI measurement resources, CSI reporting and periodic/semi-persistent SRS transmission. |
| Qualcomm | Observation 10: In R15/16/17 MG operation, all configured MGs are always activated with a higher priority than PDSCH and PUSCH except Msg2/3/4/A/B, which may cause a frequent XR traffic interruption and highly degrades the user experience. Observation 11: When CDRX is configured with MG, UE may enter the inactivity state during MG, and the remained packets should be buffered and delivered when the next DRX on-duration cycle starts. Proposal 6: For XR traffic, MG should be enhanced by handling the priority of data packets or dynamically activating/deactivating the MG occasions from gNB. Proposal 10: For XR, consider studying ways to have additional DL control signalling opportunities between sparsely configured semi-static SS set occasions by: - Dynamically configuring SS set occasions - Piggy-backing/multiplexing DL control signalling on already existing SCH messages (DG or SPS) |

Below is a summary of all remaining proposals from companies’ contributions.

Dynamic indication of SPS PDSCH

[Apple], [Qualcomm] proposed inband/piggy-backed signaling to indicate varying packet size and absence/presence of SPS PDSCH. It intends to avoid unnecessary PDSCH detection in excessive resources or the SPS occasion without transmitted PDSCH.

UE playout buffer for XR

[CATT] proposed to introduce XR-application awareness of UE playout buffer size at the gNB scheduler with the intension to schedule the XR traffic in better favour of UE power saving with relaxed PDB requirements.

CBG based SPS transmission

[China Telecom] proposed to introduce the CBG based SPS transmission, HARQ-ACK and retransmission with the intention for better resource efficiency for XR traffic.

Repetitions in frequency domain

[China Telecom] proposed repetition in frequency domain for high data rate and high reliability XR data transmission with low latency.

Periodicity alignment for CSI

[Apple] brought out the periodicity misalignment issue between existing CSI framework and XR traffic period. The proposal is to introduce non-integer periodicity for the CSI measurement resources, CSI reporting and periodic/semi-persistent SRS transmission.

Note that companies have proposed enhanced CDRX and enhanced PDCCH monitoring for similar issue to CDRX and PDCCH monitoring.

Enhanced measurement gap

[Qualcomm] proposed for XR, MG should be enhanced based on the higher priority of XR data packets or dynamically activating/deactivating the MG occasions.

### Summary

Evaluation results for the other enhancements are collected in Tables 3.3-1 below.

Table 3.3-1 the table content is removed based on companies’ comments

Table 3.3-2 Power saving evaluation results with gNB scheduling awareness of UE playout buffer: FR1, DL only, Indoor Hotspot, VR 30Mbps

| Tdoc # | Power Saving Scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Load H/L | avg # UEs/Cell | floor (Capacity) | % of satisfied UE | Mean PSG of all UEs | Mean PSG of satisfied UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2203484 | AlwaysOn | - | - | - | H | 12 | 12 | 95.83% | - | - |
| R1-2203484 | C-DRX with go-to-sleep | 16 | 8 | 4 | H | 12 | 12 | 94.17% | 26.43% | 26.78% |
| R1-2203484 | C-DRX with PDCCH skipping and go-to-sleep | 16 | 8 | 4 | H | 12 | 12 | 93.33% | 34.56% | 34.77% |
| R1-2203484 | PDCCH skipping | - | - | - | H | 12 | 12 | 91.67% | 30.26% | 30.36% |

[RD1] Observation 3.3-1: the observation is removed based on companies’ comments.

[RD1] Question 3.3-1: companies’ views on Table 3.3-1 and Observation 3.3-1.

| Company | Comments |
| --- | --- |
| MTK | We do not prefer to capture Observation 3.3-1 unless the proposing companies can provide capacity results. Power saving gain without a guaranteed marginal capacity loss does not seem very useful to us. |
| Samsung | Agree with MTK. Prefer to not capture single company results, at least without any prior discussion for their overall relevance or side effects. |
| LGE | We are not sure that Observation 3.3-1 is helpful for evaluating XR specific power saving without capacity results. |
| Nokia, NSB | Like commented in previous questions, we think it might be premature to make observation on these until we have further clarified and aligned the assumptions |
| Intel | Agree with Nokia comment |
| CATT | It would be clear to capture the results and its baseline system configuration. |
| OPPO | Same concern/comment as provided in Question 3.1-1. |
| ZTE | In our understanding, the tables and observations are for result collection so far, and companies are expected to provide more evaluations. |
| ITRI | Agree with Nokia. |
| Huawei, HiSilicon | Since these results are not based on Rel-17 XR power saving evaluation methodology and the capacity results are not provided, the results are not compatible with CDRX and PDCCH monitoring enhancements. It may be hard to draw a conclusion on the power saving gain of these enhancements. We suggest to drop this discussion in this meeting. |
| Moderator | As companies have concerns on power saving evaluation results without capacity results, these results are removed from this document. Please check Proposal 2-1 for joint power saving and capacity evaluation. Missing evaluation results are added in Table 3.3-2 for R1-2203484. |

Based on various proposed enhancements in Section 3.3.1 and 3.3.2, the following main issues and solutions are identified:

- Topic 3: Other enhancements

  - Issue 3-1: Misalignment with UL transmission. The proposed solutions include

    - Alignment between DL and UL transmissions

    - Alignment between PUCCH HARQ-ACK and PUSCH by a single PDSCH scheduling DCI

  - Issue 3-2: Power saving by XR-aware scheduling. The proposed enhancements include

    - PDCCH monitoring skipping by delay-deadline aware scheduling

    - UE playout buffer with XR-awareness

  - Issue 3-3: Unnecessary data transmission in allocated resources. The proposed solutions include

    - Avoidance of unnecessary UL transmission in over-allocated resources otherwise filled by padding based on dynamic UL indication

    - SPS and CG occasion skipping during DRX OFF

    - Dynamic indication of SPS PDSCH

[RD1] Question 3.3-2: companies’ views on Topic 3 and the Issues.

| Company | Comments |
| --- | --- |
| MTK | We are generally fine to discuss Topic 3 and the listed Issues. |
| Xiaomi | Generally fine with the proposal. Similar as our comments above, maybe we can add another bullet saying other schemes are not precluded. |
| NEC | We think at least Issue 3-1 should be further studied, because UL may be more power consuming than DL for some type of devices, for example, AR glasses. |
| Samsung | OK with the outline for further discussion but a same comment as before applies to discuss and aim to reduce proposed enhancements at this meeting. For example, we do not see any benefit or applicability from enhancements under Issue 3-3 (e.g. no relevance/benefit of SPS for XR, or no meaningful benefit from not using overallocated CG resources) while some enhancements under Issue 3-1 or 3-2 can be enabled by the NW using Rel-17. [Moderator] Thanks for the comments. Please provide continued input in the following rounds of discussions where some down-selection can be made. |
| Vivo | Before discussing these solutions, we would like to know how do we plan to deal with Topic 3: other enhancements? From our perspective, study on identifying the issues and the necessity to address the issues in the bullets need to be conducted first. Moreover, there needs additional clarifications for these solutions (such as how to using a single PDSCH scheduling DCI to achieve the alignment between PUSCH HARQ-ACK and PUSCH) which can facilitate companies to understand? [Moderator] Thanks for the comments. From paper of the proponent, the single DCI is used to schedule both PUSCH and PDSCH. |
| LGE | We are fine with Topic 3 and the Issues. |
| Nokia, NSB | Again as noted on earlier questions, we support the approach of identifying the issue to be considered prior concluding the enhancements. On the XR-aware scheduling, it is good to try to ensure that we do not do parallel work with RAN2, whom will be looking the application layer attributes. Also, maybe the focus should be on identifying the issue to be addressed, rather than concluding set of schemes. [Moderator] Thanks for the comments. A note is added for the XR-aware scheduling. Please provide continued input for following rounds of discussions where down selection can be made for enhancements without clear motivation. |
| Intel | Agree with Nokia comment |
| CATT | We are OK with the categorization of other power saving techniques. |
| OPPO | For the 2nd sub-bullet in issue 3-1 (Alignment between PUCCH HARQ-ACK and PUSCH by a single PDSCH scheduling DCI), we agree the alignment between UL transmissions of PUCCH and PUSCH may bring some power saving gain, but we suppose “a single DCI” is not the only mechanism, so we prefer to have following revision to avoid missing anything: - Alignment between PUCCH HARQ-ACK and PUSCH by a single PDSCH scheduling DCI [Moderator] Thanks for the comments. This is addressed. |
| InterDigital | We are fine with the Issues identified under Topic 3 and the corresponding solutions listed under each issue |
| Qualcomm | Issue 3-1: the alignment between DL and UL problem can be studied. Issue 3-3: “Dynamic indication of SPS PDSCH” better to be studied under capacity agenda, where multiple companies proposed similar enchantments in capacity agenda. [Moderator] Thanks for the careful check. Given the same proposal is submitted to capacity agenda from multiple sources, it is removed from Issue 3-3. |
| ZTE | Issue 3-1: It is also coupled with the issue of multi-flows, and we think it needs to be discussed. Issue 3-3: Could the FL clarify a bit more about the meaning of unnecessary data transmission? It seems we’re talking about the issue of overprovision of resource and how to mitigate it. However, for the second sub-bullet, i.e., SPS and CG occasion during DRX off, it is unreasonable to say these transmissions are not necessary without any justification. [Moderator] Issue 3-3 sub-bullets is updated for clarification. |
| ITRI | Agree with Nokia. |
| NTT DOCOMO | We are generally fine with the listed issues. Since this is the first meeting, other schemes should not be precluded, which needs to be mentioned in proposal. |
| Spreadtrum | We are OK on Topic 3 and the Issues. |
| Lenovo | These schemes can be further studied after discussing schemes in section 3.1 and 3.2. Wondering if considering additional PDCCH skipping values can address PDCCH monitoring skipping by delay-deadline aware scheduling? [Moderator] additional PDCCH skipping is included under Topic 2 for PDCCH monitoring adaptation. |
| CMCC | Issue 3-1: can be studied Issue 3-2: RAN2 led issue Issue 3-3: can be studied in capacity enhancement agenda |
| Huawei, HiSilicon | According to the SID scope, RAN1/RAN2 should focus on C-DRX enhancements and PDCCH monitoring enhancements to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Other enhancements should be down-prioritized. For Issue 3-1, the UL/DL alignment can be realized by gNB implementation. For Issue 3-2, the delay-aware scheduling seems the enhancement to improve capacity, rather than power saving, which may be discussed in XR capacity improvements. For UE playout buffer with XR-awareness, we are not very clear why gNB could have additional PDB for resource allocation of XR packet if the size of the playout buffer is fed back to the gNB scheduler and what is the relationship between playout buffer size and the additional PDB. This issue seems related to XR traffic modelling, which has not been considered in Rel-17 XR. For Issue 3-3, these issues and solutions are not XR-specific, which can be down-prioritized. In summary, we suggest to remove this part. == - Topic 3: Other enhancements - Issue 3-1: Misalignment with UL transmission. The proposed solutions include - Alignment between DL and UL transmissions - Alignment between PUCCH HARQ-ACK and PUSCH by a single PDSCH scheduling DCI - Issue 3-2: Power saving by XR-aware scheduling. The proposed enhancements include - PDCCH monitoring skipping by delay-deadline aware scheduling - UE playout buffer with XR-awareness - Issue 3-3: Unnecessary data transmission in allocated resources. The proposed solutions include - Avoidance of unnecessary UL transmission in over-allocated resources otherwise filled by padding based on dynamic UL indication - SPS and CG occasion skipping during DRX OFF - Dynamic indication of SPS PDSCH |
| Apple | We identifiy CS measurment and CSI reporting alignment with DL/UL data /control as useful. We suggest addition marked red: - Topic 3: Other enhancements - Issue 3-1: Misalignment with DL/UL transmission. The proposed solutions include - Alignment between DL and UL transmissions - Alignment between PUCCH HARQ-ACK and PUSCH by a single PDSCH scheduling DCI - Alignment between CSI measurement, CSI reporting with DL/UL transmission. - Issue 3-2: Power saving by XR-aware scheduling. The proposed enhancements include - PDCCH monitoring skipping by delay-deadline aware scheduling - UE playout buffer with XR-awareness - Issue 3-3: Unnecessary data transmission in allocated resources. The proposed solutions include - Avoidance of unnecessary UL transmission in over-allocated resources otherwise filled by padding based on dynamic UL indication - SPS and CG occasion skipping during DRX OFF - Dynamic indication of SPS PDSCH |
| Moderator | Please check updated proposal in [RD2] Proposal 3.3-1. |

Summary of 1st round discussion on Question 3.3-1:

FL would like to thank companies for the valuable inputs on the improvement of the list of issues. Based on these inputs, the following updated proposal is provided. Similar to Topic 1 and 2, if a proposal is supported and objected by different companies, FL will keep it from the 1st round discussion and leave the down selection to further discussions.

[RD2] Proposal 3.3-1: For Rel-18 XR power saving enhancements, the following areas for potential enhancements are proposed by companies

- Issue 3-1: Misalignment with UL transmission. The proposed solutions include

  - Alignment between DL and UL transmissions

  - Alignment between PUCCH HARQ-ACK and PUSCH

  - Other considerations are not precluded

  - Alignment between CSI measurement, CSI reporting with DL/UL transmission

- Issue 3-2: Power saving by XR-aware scheduling. The proposed enhancements include

  - PDCCH monitoring skipping by delay-deadline aware scheduling

  - UE playout buffer with XR-awareness

  - Other considerations are not precluded

  - Note: RAN1 should avoid parallel work with RAN2

- Issue 3-3: Unnecessary data transmission in allocated resources. The proposed solutions include

  - Avoidance of unnecessary UL transmission in over-allocated resources otherwise filled by padding based on dynamic UL indication

  - SPS and CG occasion skipping during DRX OFF when there is no data transmission

  - Other considerations are not precluded

Note 1: these enhancements are subject to down selection based on technical discussions on the necessity and evaluation results

Note 2: Rel-18 XR SI objective only has CDRX enhancements and PDCCH monitoring enhancements as focus of RAN1 study

Note 3: Some companies have concerns on the benefits of Issue 3-3 and think some enhancements under Issue 3-1 and 3-2 can be enabled by network

Companies view on [RD2] Proposal 3.3-1

Goals of this round of discussion:

- Identify high and lower priority issues under Proposal 3.3-1

  - Especially issues that you do not think should be studied in Rel-18 XR SI

- Identify proposed enhancements for that there is existing spec solution achieving same or similar effect

| Company | Comments/Reasons |
| --- | --- |
| Apple | Issues 3-1 and 3-3 are of high priority, but open to discussion on others as well. |

# Summary of Potential Proposals

Evaluation Methodology

In the second round of email discussions, three versions of the evaluation methodology are proposed. Each version is supported by the following companies

- Proposal 2-1a:

  - Supported by Nokia, NSB, Qualcomm, Huawei, HiSi, vivo, LGE, Xiaomi (?),CATT (2nd)

  - Proposal 2-1a is the Rel-17 baseline

- Proposal 2-1b:

  - Supported by ZTE, Sanechips, MTK, CATT (1st)

  - Proposal 2-1b = Proposal 2-1a + joint DL and UL evaluation

- Proposal 2-1c:

  - Supported by Apple, Intel

  - Poprosals 2-1c = Proposal 2-1b + AR 2 streams

Proposal 2-1a: Rel-17 evaluation methodology for XR power saving captured in TR 38.838 is used as the baseline evaluation methodology for UE power evaluation of Rel-18 SI on XR enhancements

- Companies are suggested to follow Rel-17 joint power saving gain and capacity evaluation for the proposed power saving enhancements

  - Same as Rel-17, capacity is defined as the maximum number of UEs per cell for which the percentage of satisfied UEs is larger than 90%

- When companies are submitting evaluation results to RAN1, it is recommended to submit results at least for the following parameters in the below table which was agreed in RAN1 #105-e during Rel-17

|  |  | Data rate [Mbps] | Packet arrival rate [fps] | PDB [ms] |
| --- | --- | --- | --- | --- |
| DL | AR/VR | 30 | 60 | 10 |
|  | CG | 30 | 60 | 15 |
| UL | VR/CG: Pose/control | 0.2 | 250 | 10 |
|  | AR: single stream model | 10 | 60 | 30 |

Proposal 2-1b: Rel-17 evaluation methodology for XR power saving captured in TR 38.838 is used as the baseline evaluation methodology for UE power evaluation of Rel-18 SI on XR enhancements

- Companies are suggested to follow Rel-17 joint power saving gain and capacity evaluation for the proposed power saving enhancements

  - Same as Rel-17, capacity is defined as the maximum number of UEs per cell for which the percentage of satisfied UEs is larger than 90%

- Companies are encouraged to submit evaluation results for joint DL and UL evaluation

- When companies are submitting evaluation results to RAN1, it is recommended to submit results at least for the following parameters in the below table which was agreed in RAN1 #105-e during Rel-17

|  |  | Data rate [Mbps] | Packet arrival rate [fps] | PDB [ms] |
| --- | --- | --- | --- | --- |
| DL | AR/VR | 30 | 60 | 10 |
|  | CG | 30 | 60 | 15 |
| UL | VR/CG: Pose/control | 0.2 | 250 | 10 |
|  | AR: single stream model | 10 | 60 | 30 |

Proposal 2-1c: Rel-17 evaluation methodology for XR power saving captured in TR 38.838 is used as the baseline evaluation methodology for UE power evaluation of Rel-18 SI on XR enhancements

- Companies are suggested to follow Rel-17 joint power saving gain and capacity evaluation for the proposed power saving enhancements

  - Same as Rel-17, capacity is defined as the maximum number of UEs per cell for which the percentage of satisfied UEs is larger than 90%

- Companies are encouraged to submit evaluation results for joint DL and UL evaluation

- When companies are submitting evaluation results to RAN1, it is recommended to submit results at least for the following parameters in the below table

|  |  | Data rate [Mbps] | Packet arrival rate [fps] | PDB [ms] |
| --- | --- | --- | --- | --- |
| DL | AR/VR | 30 | 60 | 10 |
|  | CG | 30 | 60 | 15 |
| UL | VR/CG: Pose/control | 0.2 | 250 | 10 |
|  | AR: Option 2(two streams) | 0.2 for stream 1; 10 for stream 2 | 250 for stream 1; 60 for stream 2 | 10 for stream 1; 30 for stream 2 |
|  | AR: Option 1 (single stream model) | 10 | 60 | 30 |

Performance Calibration

The next proposal is for calibration of evaluation results across companies.

- Supported by: CATT, vivo, ZTE, Sanechips

- Objected by: Intel (?), Nokia, NSB (?), Huawei, HiSi

Proposal 2-2: The following traffic model, scenario and DRX configuration are to be used for calibration of Rel-18 XR UE power saving evaluation results from all companies.

|  |  | Data rate [Mbps] | Packet arrival rate [fps] | PDB [ms] | Scenario | CDRX configuration |
| --- | --- | --- | --- | --- | --- | --- |
| DL | AR/VR | 30 | 60 | 10 | Dense Urban | {DRX cycle, ODT, IAT} = {TBD}ms |

{DRX cycle, ODT, IAT}

- CATT: {16, 8, 4}ms

- Intel: {8, 6, 6}ms or {8, 4, 6}ms

Performance Reference

There is no objection for the general proposal. Wording of proposal 2-3 was adjusted based on companies’ comments in email discussion.

Proposal 2-3: Companies are encouraged to compare performance of the following Rel-15/16/17 features with the proposed enhancements for Rel-18 XR power saving evaluations. Power saving gain is calculated w.r.t. the AlwaysOn baseline.

- Rel-15/16 CDRX including long DRX cycle, short DRX cycle and DRX command MAC CE and DCP

- Rel-17 PDCCH adaptation including PDCCH skipping and SSSG switching

Note: up to companies to report the configuration of the Rel-15/16/17 features

Areas for Study

Goals of 2nd round of discussion:

- Identify high and lower priority issues under each Proposal

  - Especially issues that you do not think should be studied in Rel-18 XR SI

- Identify proposed enhancements for that there is existing spec solution (e.g., by PDCCH skipping and SSSG switching) achieving same or similar effect

Proposal 3.1-1: For power saving study of Rel-18 XR SI, the following areas for potential CDRX enhancements are identified

- Issue 1-1: Alignment between CDRX and XR traffic for resolving the mismatch between CDRX cycle and XR traffic periodicity. The proposed solutions include the following:

  - Non-uniform CDRX cycle pattern

  - Uniform non-integer CDRX cycle

  - Fix to SFN wraparound mismatch, e.g., new time reference

  - Dynamic signaling indicated adjustment of CDRX start offset/duration [Note 1]

  - Periodic change of drx_startoffset based on configuration

  - Periodic XR-specific PDCCH monitoring in both DRX ON and OFF based on XR packet generation cycle

  - Other considerations are not precluded

- Issue 1-2: C-DRX enhancements to handle jitter. The proposed solutions include

  - Jitter handling by dynamic indication of DRX On Duration starting time and/or another DRX On Duration time by DCI or WUS

  - Adjustment of DRX On Duration conditioned on data reception

    - Note: the adjustment is applied to the duration

  - XR-specific PDCCH monitoring window at each cycle in both DRX ON and OFF based on network delay jitter

  - Two stage/level CDRX configuration

  - Other considerations are not precluded

- Issue 1-3: CDRX enhancements for multiple XR traffic flows. The proposed enhancements include

  - Multiple active CDRX configurations [Note 2]

    - Dynamic activation of CDRX configuration(s) is not precluded

  - Multi-period CDRX cycle or On Duration sequence of a CDRX cycle or non-uniform CDRX cycle pattern

  - Other considerations are not precluded

- Issue 1-4: CDRX enhancements to adjust to variable burst sizes

  - Dynamic adjustment of CDRX Active Time parameters

- Issue 1-5: low latency handling:

  - SPS/CG configuration specific handling of DRX timers.

  - DRX timers for DG transmission

FFS how the solutions or the combination of the solutions can handle all the identified issues.

Note 1: it can also be adopted for addressing issue 1-2

Note 2: it can also be adopted for addressing issue 1-1

Note 3: these enhancements are subject to down selection based on technical discussions on the necessity and evaluation results

Proposal 3.2-1: For power saving study of Rel-18 XR SI, the following areas for potential PDCCH monitoring enhancements are identified

- Issue 2-1: Alignment between PDCCH monitoring and XR traffic to resolve the mismatch between PDCCH monitoring periodicity and XR traffic periodicity. The proposed solutions include

  - Non-uniform pattern of PDCCH monitoring, e.g., {16,17,17}ms

  - Adjustment of the start offset/duration/periodicity of PDCCH monitoring

  - MAC control go-to-sleep indication

  - Other considerations are not precluded

  - Note: some companies suggest clarifying whether Rel-17 PDCCH monitoring adaptation can solve issue 2-1 or achieve similar intended outcome or not

- Issue 2-2: XR-dedicated PDCCH monitoring window to supplement CDRX for multi-flow traffic. The enhancements include

  - Using window based PDCCH monitoring to replace CDRX for scheduling XR traffic

  - Window-based transmission for general UL/DL channel and signal

  - MAC control go-to-sleep indication

  - PDCCH monitoring accompanying SPS/CG configuration

  - Other considerations are not precluded

  - Note: some companies suggest clarifying whether Rel-17 PDCCH monitoring adaptation can solve issue 2-2 or achieve similar intended outcome or not

- Issue 2-3: Enhancements to Rel-17 PDCCH monitoring adaptation. The solutions include

  - PDCCH monitoring adaptation triggered by non-scheduling DCI to avoid missing additional data scheduling

  - Additional PDCCH skipping durations (i.e., >3 values) for maximizing UE sleep duration

  - PDCCH skipping interaction with HARQ retransmission by cancelling PDCCH skipping or disabling retransmission or an additional monitoring window for retransmission during the skipping period for both DG and SPS PDSCH, and DG and CG PUSCH

    - This is to avoid additional latency for retransmission due to PDCCH skipping.

  - DCI linkage for BD reduction

  - Additional DL control signalling opportunities between sparsely configured semi-static SS set occasions (e.g., dynamically configuring SS set occasions, or piggy-backing/multiplexing DL control signalling on SCH messages)

  - Non-uniform PDCCH monitoring occasions within On Duration matched to jitter distribution

  - Other considerations are not precluded

  - Note: when to discuss some enhancements may depend on the outcome of Rel-17 PDCCH monitoring adaptation maintenance

Note 1: these enhancements are subject to down selection based on technical discussions on the necessity and evaluation results

Proposal 3.3-1: For Rel-18 XR power saving enhancements, the following areas for potential enhancements are proposed by companies

- Issue 3-1: Misalignment with UL transmission. The proposed solutions include

  - Alignment between DL and UL transmissions

  - Alignment between PUCCH HARQ-ACK and PUSCH

  - Other considerations are not precluded

  - Alignment between CSI measurement, CSI reporting with DL/UL transmission

- Issue 3-2: Power saving by XR-aware scheduling. The proposed enhancements include

  - PDCCH monitoring skipping by delay-deadline aware scheduling

  - UE playout buffer with XR-awareness

  - Other considerations are not precluded

  - Note: RAN1 should avoid parallel work with RAN2

- Issue 3-3: Unnecessary data transmission in allocated resources. The proposed solutions include

  - Avoidance of unnecessary UL transmission in over-allocated resources otherwise filled by padding based on dynamic UL indication

  - SPS and CG occasion skipping during DRX OFF when there is no data transmission

  - Other considerations are not precluded

Note 1: these enhancements are subject to down selection based on technical discussions on the necessity and evaluation results

Note 2: Rel-18 XR SI objective only has CDRX enhancements and PDCCH monitoring enhancements as focus of RAN1 study

Note 3: Some companies have concerns on the benefits of Issue 3-3 and think some enhancements under Issue 3-1 and 3-2 can be enabled by network

# References

- RP-220285, Revised SID: Study on XR Enhancements for NR

- R1-2203131, Discussion on XR-specific power saving techniques, Huawei, HiSilicon

- R1-2203348, Discussion on XR specific power saving techniques, Spreadtrum Communications

- R1-2203484, UE Power saving techniques for XR, CATT

- R1-2203585, Discussion on XR specific power saving enhancements, vivo

- R1-2203606, Discussion on XR specific power saving techniques, ZTE, Sanechips

- R1-2203638, Discussion on power saving enhancements for XR, Ericsson

- R1-2203666, Discussion on XR enhancement for NR, China Telecom

- R1-2203744, Considerations on power saving techniques for XR, Sony

- R1-2203927, Considerations on XR-specific Power Savings, Samsung

- R1-2203940, Discussion on XR specific power saving techniques, NEC

- R1-2204028, Discussion on XR specific power saving techniques, OPPO

- R1-2204123, Discussion on XR specific power saving enhancements, InterDigital, Inc.

- R1-2204177, XR specific power saving techniques, TCL Communication Ltd.

- R1-2204264, Views on XR specific power saving techniques, Apple

- R1-2204326, Discussion on XR-specific power saving techniques, CMCC

- R1-2204400, Discussion on XR specific power saving techniques, NTT DOCOMO, INC.

- R1-2204414, XR-specific power saving techniques, Lenovo

- R1-2204444, Discussion on XR specific power saving techniques, ITRI

- R1-2204633, Discussion on XR-specific power saving techniques, LG Electronics

- R1-2204655, Discussion on power saving techniques for XR, ETRI

- R1-2204674, Discussion on XR-specific power saving enhancements, Nokia, Nokia Shanghai Bell

- R1-2204698, On XR specific power saving techniques, MediaTek Inc.

- R1-2204818, Discussion on power saving enhancements for XR applications, Intel Corporation

- R1-2205176 (revision of R1-2205054), Power saving techniques for XR, Qualcomm Incorporated

- TR 38.838, Study on XR (Extended Reality) evaluations for NR

- RAN1 #104-e Chairman’s Notes, January 2021

# Appendix

Objective of SI

Objective of the Rel-18 XR enhancements SI is as follows

The study is to be based on Release 17 TR 38.838, on corresponding Release 17 work from SA4 (as per SP-210043) and on Release 18 work from SA2 (as per SP-211166). Objectives on XR-awareness in RAN (RAN2): - Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of. - Study how the above information aids XR-specific traffic handling. Objectives on XR-specific Power Saving (RAN1, RAN2): - Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: - C-DRX enhancement. - PDCCH monitoring enhancement. Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements; - Dynamic scheduling/grant enhancements.

Full List of Proposed Enhancements

Below is a full of power saving enhancements proposed in companies’ contributions [2] to [25] to RAN1 #109-e.

CDRX enhancements - Periodicity alignment for CDRX - Configuration of CDRX cycle pattern - New non-integer CDRX cycle - Timing reference to resolve SFN wraparound mismatch - Multiple active CDRX configurations - Dynamic adjustment of DRX start offset - Jitter handling for CDRX - PDCCH monitoring occasions matched to jitter distribution - Dynamic adjustment of DRX starting time - Jitter handling by WUS - Adjustment of CDRX on duration conditioned on data reception - Two stage/level CDRX configuration - Implicit SSSG switching conditioned on data reception - Multiple CDRXs for multiple flows - Multiple active CDRX configurations - Multiple periods or sequence based DRX - Other CDRX enhancements - Dynamic adaptation on CDRX configuration - UL slot handling for CDRX timers - SPS and CG skipping for CDRX - Traffic aware WUS PDCCH monitoring enhancements - Periodicity alignment for PDCCH monitoring - Semi-static pattern for PDCCH monitoring - Dynamically adjust the start offset of PDCCH monitoring - Enhancements to PDCCH monitoring adaptation - PDCCH monitoring adaptation triggered by non-scheduling DCI - XR-dedicated PDCCH monitoring window - Additional PDCCH skipping durations - PDCCH skipping interaction with HARQ retransmission - Dynamic adaptation of PDCCH monitoring configuration - PDCCH monitoring adaptation indication by SPS - Disabling UL retransmission for PDCCH monitoring reduction - PDCCH monitoring skipping by delay awareness - Other PDCCH monitoring adaptation enhancements - Single DCI indicating monitoring adaptation for multiple cells - PDCCH monitoring adaptation applying for Type 0-2 CSS sets - One DCI scheduling more than 8 PDSCHs/PUSCHs - DCI linkage for BD reduction - SSSG switching per TRP Other power saving enhancements - UL power saving enhancements - Power saving enhancements by DL & UL alignment - PUSCH and PUCCH alignment by single DCI scheduling UL and DL transmissions - Uplink transmission adaptation for CG and DG - Other enhancements - Dynamic indication of SPS PDSCH - UE playout buffer for XR - CBG based SPS transmission - Repetitions in frequency domain - Periodicity alignment for CSI - Enhanced measurement gap