3GPP TSG RAN WG1 #109-e			R1-2205056

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.11.2

Source: 	Qualcomm Incorporated

Title: 	Capacity Enhancement Techniques for XR

Document for:	Discussion and Decision

# Introduction

5G eXtended Reality (XR) and Cloud Gaming (CG) traffics are characterized by the quasi-periodic data generation, tight delay budget and high data rate. In 3GPP Rel-17, performance of the 5G XR service was assessed based on system level evaluation methodology. During the study item, traffic models and deployment scenarios were defined for the evaluation of capacity, UE power consumption, coverage and mobility. In Rel-18, 3GPP will continue with the study of potential enhancements on XR-awareness, UE power saving and capacity improvements for the better support of NR based XR services [1].

In this paper, we will discuss potential enhancements for XR-specific capacity improvements with the focus on SPS and CG enhancements and dynamic scheduling and grant enhancements.

# SPS and CG Enhancements

XR traffic consists of traffic bursts which are periodic with some time jitter in the arrival. These traffic bursts may have different packet sizes (lengths) and different number of packets within each burst. Dynamic grant-based operation can be used to handle the XR traffic but at the expense of additional signalling (PDCCH) and additional power consumption (PDCCH decoding). Given the periodic nature of XR traffic, more power efficient periodic schemes like SPS/CG are very likely candidates to be used for XR operation. However, in addition to the above issues, there may be a tempo mismatch between the XR traffic and the SPS/CG cycles. These may cause some issues as in the following cases:

- Case 1: variable number and sizes of packets per burst (period), hence SPS/CG occasion may either be smaller or larger than the required resources, leading to:

  - Wasted resources if more resources than needed are allocated

  - Delays if less resources than needed are allocated

  - Unnecessary gNB beam direction reservation especially for UL if more resources than needed are allocated

- Case 2: jitter and tempo mismatch between SPS/CG and XR bursts, leading to:

  - Delays if resources are not aligned with traffic

Hence, having preconfigured and fixed SPS/CG parameters may not be efficient, and we may need to have some enhancements. In this section, some techniques are introduced to mitigate these issues.

CG/SPS Set Switching and Skipping

As the traffic is periodically generated by XR application, it is suitable to use configured grant (CG) for the UL XR video data transmission. Rel-16 supports activation of one CG at a time by the DCI and joint release of multiple active CGs by one DCI. When multiple Type 2 CGs are configured for a UE, HARQ process number field in the activation DCI indicates the index of the activated CG. Release of multiple Type 2 CGs relies on a list of sets of configured CGs. When the list is configured, HARQ process number field in the release DCI indicates the index of a set of Type 2 CGs to be released. For DL video transmission, a similar use case exists for DL SPS based PDSCH.

For XR, the large video frame data size may require more than one PUSCHs to be transmitted in each video frame period. Enhancements for UL CG can then be introduced to create multiple PUSCH opportunities around the same CG occasion. It would be convenient to use the Rel-17 multi-PUSCH scheduling DCI to provide the resource allocation information for these PUSCHs. Two mechanisms can achieve the similar effect either based on multiple PUSCHs activated for the same CG occasion or multiple CGs jointly activated by the same DCI.

Proposal 1: For XR UL/DL video data transmission, use a single activation DCI for the following cases based on the multi-PUSCH/PDSCH scheduling DCI

- Case 1: activate a single CG/SPS with multiple PUSCHs/PDSCHs on a CG/SPS occasion

- Case 2: activate multiple CGs/SPSs with one PUSCH/PDSCH on an occasion of each CG/SPS

Case 1

Case 2

Figure 1: Multiple CG PUSCHs activated simultaneously

For dynamic data scheduling, NR Rel-17 has introduced the PDCCH adaptation mechanism including PDCCH skipping and search space set group (SSSG) switching. By PDCCH adaptation, network can switch between high efforts and low efforts for UE PDCCH monitoring according to the traffic condition. This allows for both UE power saving and efficient data communications. There is no similar mechanism for CG based UL data transmission. Given joint release of multiple CGs is already supported by NR, if one DCI can activate a set of CGs (i.e., by the proposal above) and release another set of CGs simultaneously, this will result in a CG set switching mechanism. Similar to SSSG switching, this allows for adaptive adjustment of CG periodicity and adaptation of other scheduling information to better fit the channel and traffic conditions. For example, the number of PUSCHs and CG periodicity may be adjusted to reflect the when the video encoding algorithm is adaptative to the varying channel conditions or the application.

CG set switching can be triggered by a DCI that activates multiple CGs simultaneously and in the meanwhile releases the current set of active CGs that are not included in the CGs to be activated. Similar to the SSSG switching, timer-based switching can be defined. I.e., UE switches back to the previous set of active CGs before the CG set switching is triggered after a timer expires when UE stays with the current set of active CGs. The timer value can be configured by RRC. Consider that the dynamic scheduling function has been replaced by CG, the timer value for CG set switching can also be indicated by the activation DCI by repurposing the “PDCCH monitoring adaptation indication” field.

Proposal 2: Introduce the CG/SPS set switching mechanism to simultaneously activate one set of CGs/SPSs and deactivate another set of CGs/SPSs for adaptative CG/SPS configuration. Timer based switching can be introduced.

Similar to Rel-17 PDCCH adaptation which has both SSSG switching and PDCCH skipping, CG adaptation can be realized by a CG skipping mechanism by temporarily disabling a set of currently active CGs for a time duration. CG or SPS set skipping can be realized by the CG release DCI that jointly releases a set of CGs. In this case, the current NR design can be reused to indicate the set of CG by the HARQ process number field in the DCI. The time duration when the CGs are temporarily deactivated can be either configured by RRC or in the release DCI by repurposing the “PDCCH monitoring adaptation indication”.

Proposal 3: Introduce the CG/SPS set skipping mechanism to temporarily deactivate a set of CGs/SPSs and reactivate it after a timer expires.

In addition, for a single SPS/CG with multiple PDSCHs/PUSCHs on a SPS/CG occasion (i.e., case 1 above), we can adapt to the variable number and sizes of packets per burst. This is mainly to reduce the resource wastage (if more than needed PDSCHs/PUSCHs in an occasion are configured) or delays (if less than needed PDSCHs/PUSCHs in an occasion are configured). In these cases, it may be a good addition to the proposal to let the gNB or the UE signal some modifications to the PDSCHs/PUSCHs in these occasions. Such modifications may include skipping, changing parameters (e.g., MCS, # RBs, …), or adding extra PDSCHs/PUSCHs. This can be indicated using DCI/UCI or MAC-CE on dedicated resources (PDCCH or PUCCH) or piggy-backed on previous a PDSCH/PUSCH in the same of different occasion. An SPS example is shown in Figure 2.

Proposal 4: For single SPS/CG with multiple PDSCHs/PUSCHs on a SPS/CG occasion, consider studying methods to skip, modify, or add extra PDSCHs/PUSCHs in an occasion.

Figure 2: Single SPS with multiple PDSCHs on a SPS occasion

Dynamic Configuration of SPS/CG Occasions

If the SPS and/or CG configuration are allowed to be changed dynamically by indication from the gNB and/or UE, they can be adapted to the XR traffic characteristics. Parameters may include:

- Cancelling occasions

  - If one or more occasions do not contain data, an indication may be sent earlier to cancel them

  - May be resource efficient

  - Frees UL beams which may have been reserved for that UE

- Adding extra occasions

  - If the SPS/CG occasion resources is not enough for the burst

  - May lead to less delays

- Shifting occasions in time (advance or delay)

  - This can be very useful in cases of jitter and/or mis-alignment due to tempo mismatch

  - May lead to less delays

- Shifting occasions in frequency

  - This can be very useful in cases of time domain collision between UEs

  - May lead to less delays

- Modifying the occasion parameters (e.g., MCS, # RBs, # symbols, TCI state, etc.)

  - If the SPS/CG occasion resources do not match the XR burst

  - May be resource efficient

  - May lead to less delays

Different methods for indicating these modifications/adaptations can be used. These may include: WUS (can be sequence or channel), DMRS, DCI, RMI (UL message for resource modification indication), MAC-CE (of DG/SPS/CG), piggy-backed DCI/UCI on DG/SPS/CG, PUCCH, and SR.

The modifications/adaptation may be done for:

- For one or more specific occasions

- In the form of occasions utilization (ON/OFF) pattern for the upcoming N occasions

In addition, to allow for additional un-expected traffic, for the cycles with skipped SPSs or CGs cycles, the gNB may configure the UE to implicitly add additional PDCCH monitoring or additional SR locations in these canceled cycles.

Figure 3 shows an example of operation with SPS dynamic adaptation.

Proposal 5: For XR, consider studying methods to dynamically adapt the SPS and CG parameters to the traffic bursts.

Figure 3: SPS dynamic adaptation

Enhancements to Combined SPS/CG and Dynamic Grant Operation

In some cases, the XR traffic payload per burst may exceed the allocation for the corresponding SPS or CG occasion. In this case, to meet the PDB and avoid additional latency, additional dynamic grant may be needed for that burst. Hence, it may be reasonable to account for such cases and have some increased PDCCH monitoring occasions or additional SR opportuneness immediately before or after a SPS and/or CG occasion. This configuration can be implicitly assumed from the UE side.

In addition, an SPS or CG occasion may piggy-back control information to grant a DG or request for more data as shown in the CG example in Figure 4.

An example gNB implementation could be to:

- Use DG if the packet arrives > T before next SPS

- Use SPS if the packet arrives ≤ T before next SPS and packet fits in SPS

- Use SPS + DG if the packet arrives ≤ T before next SPS and packet > SPS

- Value of T is chosen based on the PDB of the application

- If DG comes before SPS occasion and SPS is not needed, skip indication in DG (PDCCH/PDSCH)

Proposal 6: For XR, consider studying enhancement methods for combined SPS/CG and DG operation:

- Implicitly increased PDCCH/SR opportunities before and/or after the SPS/CG occasions

- Implicitly increased PDCCH/SR opportunities in cancelled cycles of SPS/CG occasions

- SPS/DG piggy-back control information to assist with possible future dynamic grants

Figure 4: SPS/CG + dynamic grant

Short Signal-Based SPS

To mitigate jitter and/or tempo mismatch between the XR traffic and SPS periods, consider a feature where a UE monitors for a short control signal within a window from a default location within the SPS cycle giving the UE some information on the upcoming SPS. The density of the short signal within the window is set to trade-off latency (packet arrival and transmission at the gNB) with power consumption of the short signal detection. If the UE detects the short control signal, then the UE assumes the SPS occasion is at a fixed time offset from the short control signal. However, if the UE does not detect the short signal, then the UE skips SPS decode attempt. This has buffering advantages compared to SPS DMRS detection where a UE has to buffer samples in case SPS is sent and can empty the buffer only after detecting no PDSCH in that SPS from DMRS.

To further enhance this feature, the following can be considered (Figure 5):

- The short signal can also include additional information to modify future SPS occasions (e.g., cancel, modify, etc.)

- Additional PDCCH monitoring can be implicitly assumed after the SPS occasion (as explained in the previous section)

- To reduce the latency, during the short signal detection, the monitoring window could be increased dynamically if it is not detected during the initial monitoring. In this case, the UE monitors the short signal past the initial location of the SPS occasion and even into the PDCCH monitoring window, until a short signal is detected, and a packet is received using the SPS resources

Proposal 7: For XR, consider studying a design where a short control signal can be sent within a window before the SPS and can be used to cancel, control the occasion start time, or control other SPS parameters.

Figure 5: Short signal-based SPS

XR Periodicity Alignment

R16/17 CG/SPS periodicity values mismatch with the periodicity of XR UL/DL traffic arrivals. This means that there is a tempo mismatch between them. The typical XR traffic periodicities are 60, 120 frames per seconds (fps) or Hz, of which frame periodicities are 16.67ms, 8.33ms. Since CG/SPS periodicity values support only integer multiples of slot, no matter which periodicity is chosen from currently available values from TS38.331, it cannot be exactly aligned with UL/DL traffic arrival timing. Figure 6 illustrates the case of XR periodicity mismatch between 60fps XR traffic and CG/SPS periodicity of 16 slots and 17 slots in 15kHz SCS. This would lead to XR capacity loss due to the packet delay caused by the timing difference between CG/SPS resources and actual XR traffic. In TDD system, UL/DL resource pattern also needs to be considered for CG/SPS resources.

Observation 1: There is a time mismatch issue between periodic XR UL/DL traffic and R16/17 CG/SPS configuration. This will lead to XR capacity loss due to the packet delay caused by the timing difference between CG/SPS resources and actual XR traffic.

Figure 6 Mismatch between XR UL/DL traffic (60fps) and R16/17 CG/SPS periodicity (16 slots or 17 slots)

Proposal 8: RAN1 should discuss a solution to address the time mismatch between R16/R17 CG/SPS configuration. The solution can be like those under consideration for a similar issue that exists for CDRX.

# Dynamic Scheduling and Grant Enhancements

HARQ ACK Enhancement for Dynamic Grant-Based Scheduling

HARQ allows to recover transport blocks that are not properly decoded by means of retransmissions. It can be leveraged to use MCS close to what the radio channel can support while maintaining high reliability. However, because of the HARQ round trip delay, it can have a significant impact on latency.

The gNodeB chooses the MCS for a transmission based on its knowledge of the channel conditions, but it cannot always precisely predict how they evolve. Therefore, the gNodeB may sometimes overestimates the MCS, leading to multiple retransmissions before proper decoding of the transport block. The resulting latency can impact XR performance.

Reference [3] presented the feature of soft HARQ-ACK, where the UE provides enhanced HARQ-ACK feedback beyond the single bit ACK/NACK status in the form of a Delta MCS based on PDSCH decoding. By allowing the UE to provide additional information based on reception of a transport block, soft HARQ-ACK allows the gNodeB to adapt the scheduling of retransmissions and thereby allows the UE to decode the transport block without waiting for too many additional HARQ round trips.

Large bit-rate requirements of XR traffic may require wideband RB allocations. When adapting the scheduling based on soft HARQ-ACK, if the RB allocation is already wideband, it is not possible to increase the frequency resources further. One possibility is to use the time domain instead – the gNodeB can repeat the retransmission multiple times based on the soft HARQ-ACK feedback. We next present simulation results showing gains of such a scheme over a baseline scheme without HARQ-ACK enhancements.

### Simulation Assumptions

HARQ round trip delay depends on UE and gNodeB processing delays, as well as TDD configuration. Here are our assumptions:

- TDD configuration: DDDSU

- UE processing delay: Capability 1

- gNodeB processing delay (from HARQ feedback to retransmission) = 4, 6 or 8 slots

  - When this processing delay is set to K, that means a NACK received in slot n will trigger a retransmission in the downlink slot after slot (n + K) in the TDD configuration

Other simulation assumptions follow the baseline as suggested by TR 38.838 [4]. We show XR capacity results for Dense Urban and Indoor Hotspot scenarios. The traffic consists of 60Mbps, 60Hz video with 10ms PDB, over downlink only.

### Simulation Results

Figure 7 and Figure 8 show the ratio of satisfied UEs (based on ≥ 99% packet completion rate) as a function of the number of UEs in the cell for the dense urban and indoor hotspot scenarios respectively. Results are presented for soft HARQ-ACK as well as baseline HARQ-ACK for 4, 6, and 8 slots gNodeB processing delay. (Simulation results summary is provided in the Appendix section).

Figure 7: Capacity comparison between soft HARQ-ACK and baseline HARQ-ACK for 4, 6, and 8 slots NACK to retransmission delay, Dense Urban, 60Mbps, 60fps DL stream, 10ms PDB

Figure 8: Capacity comparison between soft HARQ-ACK and baseline HARQ-ACK for 4, 6, and 8 slots NACK to retransmission delay, Indoor Hotspot, 60Mbps, 60fps DL stream, 10ms PDB

These figures show the benefit brought by soft HARQ-ACK feedback. A significant increase in XR capacity is observed in these conditions for the soft HARQ-ACK based approach as compared to the baseline HARQ-ACK scheme.

The gain of soft HARQ-ACK relative to baseline HARQ-ACK increases when the HARQ round trip delay increases (which is modeled here by increasing gNodeB processing delay).

Observation 2: Soft HARQ-ACK is observed to provide a significant gain in XR capacity over baseline HARQ-ACK.

Observation 3: The gain of soft HARQ-ACK relative to baseline HARQ-ACK increases when the HARQ round trip delay increases.

Enhancements to Single DCI Multi-PDSCH/PUSCH Scheduling

To adapt to the XR traffic characteristics, dynamic grants may be used in certain cases. However, dynamic grants are flexible but at the expense of increased PDCCH monitoring and hence increased power consumption. In our companion paper [2], we proposed enhancements to the multi-PDSCH/PUSCH grants from power reduction point of view. In this section, we consider enhancements for the single DCI multi-PDSCH/PUSCH from capacity improvement point of view.

To start, we review the support for multi-PDSCH in the current NR specs (Rel-17):

- FR: PDSCH enhancements (especially for scheduling and HARQ) for NR FR2-2 (SCS 120/480/960 kHz)

- Number of HARQ Process: a maximum of 32 HARQ processes per cell for the cases of 480/960 kHz

- SLIV and K0: each PDSCH has a separate SLIV, mapping type and K0

For multi-PUSCH, the following is supported (in Rel-17):

- TB per PUSCH

- Gap between adjacent PUSCH allowed (unlike Rel.16 multi-PUSCH grant), even slot level gaps

- No maximum gap limitation, except those from range of RRC parameters

- Most parameters shared, except HARQ process ID, RVID, NDI, TDRA

XR traffic may be such that it has jitter and/or non-integer cycles and different number and lengths of packets per burst. The current single DCI multi-PDSCH/PUSCH design may be restrictive for XR. Consider the following cases:

- Case 1: an XR traffic burst consists of packets of different sizes

  - With the current NR design, since the sizes of the PDSCHs should be the same, a DCI would grant multiple PDSCHs dimensioned over the maximum packet size  causes resource waste

- Case 2: at the time of the granting DCI, the gNB may know some of the traffic patterns (or some time into the future) but may not know the rest (or cannot predict so far into the future). To mitigate this issue:

  - The gNB may grant a fewer number of PDSCH/PUSCH  causes more DCIs to be sent + more delays

  - The gNB may over-grant resources to account for un-expected resources  causes resource waste and un-necessary UL beam reservation

Observation 4: The existing single DCI multi-PDSCH/PUSCH framework can cause resource waste, additional delays, or more control signalling.

To mitigate these issues, the following enhancements can be made:

- Enhancement 1: allow for different configurations per PDSCH/PUSCH in a single DCI grant:

  - Multi-PDSCH

    - Varying MCS

    - One or more PDSCH per PUCCH (to reduce the latency)

    - Extend to FR1 and FR2-1

    - Extend 32 max. HARQ processes to lower SCS

    - Single DCI for multiple PDSCH to be used over multiple data cycles

  - Multi-PUSCH

    - SR: request for resources: number of PUSCHs, MCS, etc.

    - Varying number of PUSCHs, MCS, etc.

    - Indicator to make resources persistent for duration of time

- Enhancement 2: resource efficient single DCI Multi-PDSCH/PUSCH (examples in Figure 9)

  - After the single DCI grant, the gNB may change the behavior of one or more of the already granted PDSCHs/PUSCHs, e.g.:

    - Skipping or activating resource

    - Change TCI state/spatial relation

    - Change MCS, FDRA, TDRA, #RBs, etc…

    - Change from new TB to repetition

    - Change DMRS configuration

  - The behavior change can be:

    - Explicitly signaled: WUS (wake-up-signal), PDCCH (DCI), MAC-CE (using DG or SPS/CG), piggy-back DCI, SR, PUCCH, or RRC

    - Implicit: e.g.: cancel pre-granted DL PDSCH after a HARQ-ACK

The following proposal can be made:

Proposal 9: For XR, consider studying enhancements for single DCI multi-PDSCH/PUSCH grants including:

- Allowing for different configurations per PDSCH/PUSCH in a single DCI grant

- Allowing the gNB to change the behavior of one or more of the already granted PDSCHs/PUSCHs after the granting DCI

Figure 9: Enhanced single-DCI multi-PDSCH

Dynamically Triggered Pre-Configured Traffic Resources

Since the number of packets per burst (period) may not be constant, a SPS/CG occasion may sometimes fit the burst, but some other times does not and would need additional resources per period. In section 2.3, an enhanced SPS/CG + dynamic grant proposal was made to resolve this issue.

However, in certain cases, signalling overhead reduction may be desirable to reduce power or to reduce overhead. In addition, in some cases, in order to reduce signalling and overhead, the gNB may be able to use preconfigured resource that can be activated (Figure 10):

- Using explicit signalling: WUS (wake-up-signal), PDCCH (DCI), MAC-CE (using DG or SPS/CG), or RRC

- Implicitly: e.g.:

  - Activate pre-configured DL re-tx resource after a HARQ-NACK

  - Activate pre-configured UL resource after a SR/CG with specific payload length

These pre-configured resources can be used for 1st transmissions or re-transmissions.

Proposal 10: For XR, consider studying introducing pre-configured data resources that can be activated or deactivated either explicitly or implicitly.

Figure 10: Pre-configured resources

# Conclusions

In this contribution, we have provided the following observations and proposals for the XR-specific capacity improvements for NR Rel-16 XR enhancements:

Observation 1: There is a time mismatch issue between periodic XR UL/DL traffic and R16/17 CG/SPS configuration. This will lead to XR capacity loss due to the packet delay caused by the timing difference between CG/SPS resources and actual XR traffic.

Observation 2: Soft HARQ-ACK is observed to provide a significant gain in XR capacity over baseline HARQ-ACK.

Observation 3: The gain of soft HARQ-ACK relative to baseline HARQ-ACK increases when the HARQ round trip delay increases.

Observation 4: The existing single DCI multi-PDSCH/PUSCH framework can cause resource waste, additional delays, or more control signalling.

Proposal 1: For XR UL/DL video data transmission, use a single activation DCI for the following cases based on the multi-PUSCH/PDSCH scheduling DCI

- Case 1: activate a single CG/SPS with multiple PUSCHs/PDSCHs on a CG/SPS occasion

- Case 2: activate multiple CGs/SPSs with one PUSCH/PDSCH on an occasion of each CG/SPS

Proposal 2: Introduce the CG/SPS set switching mechanism to simultaneously activate one set of CGs/SPSs and deactivate another set of CGs/SPSs for adaptative CG/SPS configuration. Timer based switching can be introduced.

Proposal 3: Introduce the CG/SPS set skipping mechanism to temporarily deactivate a set of CGs/SPSs and reactivate it after a timer expires.

Proposal 4: For single SPS/CG with multiple PDSCHs/PUSCHs on a SPS/CG occasion, consider studying methods to skip, modify, or add extra PDSCHs/PUSCHs in an occasion.

Proposal 5: For XR, consider studying methods to dynamically adapt the SPS and CG parameters to the traffic bursts.

Proposal 6: For XR, consider studying enhancement methods for combined SPS/CG and DG operation:

- Implicitly increased PDCCH/SR opportunities before and/or after the SPS/CG occasions

- Implicitly increased PDCCH/SR opportunities in cancelled cycles of SPS/CG occasions

- SPS/DG piggy-back control information to assist with possible future dynamic grants

Proposal 7: For XR, consider studying a design where a short control signal can be sent within a window before the SPS and can be used to cancel, control the occasion start time, or control other SPS parameters.

Proposal 8: RAN1 should discuss a solution to address the time mismatch between R16/R17 CG/SPS configuration. The solution can be like those under consideration for a similar issue that exists for CDRX.

Proposal 9: For XR, consider studying enhancements for single DCI multi-PDSCH/PUSCH grants including:

- Allowing for different configurations per PDSCH/PUSCH in a single DCI grant

- Allowing the gNB to change the behavior of one or more of the already granted PDSCHs/PUSCHs after the granting DCI

Proposal 10: For XR, consider studying introducing pre-configured data resources that can be activated or deactivated either explicitly or implicitly.

# References

- RP-220285, Revised SID: Study on XR Enhancements for NR

- R1-2205054, “Power Saving Techniques for XR,” 3GPP TSG RAN1#109-e, May 2022

- R1-2107337, “CSI enhancement for IOT and URLLC”, Qualcomm Incorporated, Aug 2021

- TR 38.838, Study on XR (Extended Reality) evaluations for NR

# Appendix

Table 1: FR1, DL, DU, VR/AR 60Mbps, MU-MIMO

| Capacity Enhancement Scheme | NACK to Retx delay (slot) | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Soft HARQ-Ack | 4 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 4.6 | 4 | 94.50% |
| Baseline | 4 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 0 | 0 | N.A. |
| Soft HARQ-Ack | 6 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 2.8 | 2 | 92.90% |
| Baseline | 6 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 0 | 0 | N.A. |
| Soft HARQ-Ack | 8 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 2 | 2 | 90.10% |
| Baseline | 8 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 0 | 0 | N.A. |

Table 2: FR1, DL, InH, VR/AR 60Mbps, MU-MIMO

| Capacity Enhancement Scheme | NACK to Retx delay (slot) | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Soft HARQ-Ack | 4 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 2.93 | 2 | 97.70% |
| Baseline | 4 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 0 | 0 | N.A. |
| Soft HARQ-Ack | 6 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 2.1 | 2 | 91.25% |
| Baseline | 6 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 0 | 0 | N.A. |
| Soft HARQ-Ack | 8 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 1.17 | 1 | 91.25% |
| Baseline | 8 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 0 | 0 | N.A. |