# 3GPP TSG RAN WG1 #109-e							R1-2205059

# e-Meeting, May 09th – May 20th, 2022

Agenda item:	9.12.2

Source: 	Qualcomm Incorporated

Title: 	Disabling HARQ Feedback for IoT-NTN

Document for:	Discussion and Decision

# Background

In RAN#94-e, the following objectives with RAN1 impact were included in the WID for eMTC/NB-IoT over NTN in Release 18:

IoT-NTN Performance Enhancements in Rel-18 to address remaining issues from Rel-17

This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below:

- Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2]

- Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption [RAN1]

In this contribution, we provide our initial views on aspects related to disabling of HARQ feedback for IoT-NTN

# Need for feedback disabled HARQ processes

In Table 1, we depict a scenario corresponding to a Set 1 GEO deployment, with a central beam centre DL SNR of -2.98 dB and a central beam edge DL SNR of -3.24 dB. For our link-level evaluations, we use an AWGN channel, which is realistic in the line-of-sight settings that characterize GEO NTN networks. We note that, under this situation, an NPDCCH transmission with a 1% BLER requires 4 ms (R = 4), while an NPDSCH transmission, for a TBS of 1544 bits (ITBS  = 9, NSF=10) , with 10% BLER requires 40 ms. The RTD—for the case where the timing reference is at the ground station—is 541.46 ms. In our analysis, we also account for the maximum delay in aligning with a NPDCCH search space (we assume Rmax=8), when determining the overall NPDCCH duration, in Table 1.

Table 1: Downlink SNRs and required transmission durations in Set 1 GEO deployments.

| Round Trip Delay RTD | SNRedge | SNRcentre | NPDCCH duration including max. scheduling time (1% BLER@SNRedge, AWGN) TNPDCCH | NPDSCH duration (10% BLER@SNRedge, AWGN, TBS=1544 bits) TNPDSCH |
| --- | --- | --- | --- | --- |
| 541.46 ms | -3.24 dB | -2.98 dB | 4+7=11ms | 4×10=40ms |

In Table 2, we depict the throughput/latency penalty that is incurred, if we do not support any HARQ process with HARQ-ACK feedback disabled. The TNPDCCH and TNPDSCH values are taken from Table 1. In our calculations, we assume a 4 ms delay from NPDCCHNPDSCH, and the minimum value of the NPDSCHHARQ-ACK delay, i.e., k0=13, for a fair comparison. The timelines used for the calculations are depicted in Fig. 1 for a NB-IoT UE with a single HARQ process.

Figure 1: Scheduling timelines for a NB-IoT UE with a single HARQ process. The top figure depicts scheduling without feedback disabling, while the bottom figure depicts scheduling when feedback disabling is supported.

Further, for the case of 2 HARQ processes without feedback disabling, we assume that the 2nd TB can be transmitted within the RTD that starts after the HARQ ACK for the 1st TB is sent—this also ensures fairness, since the time taken to transmit 2 TBs without feedback disabling is the same as that taken to transmit 1 TB.

In Table 2, we see the significant impact on throughput/latency due to the large RTD (>11x for 1 HARQ process, and >5.5x for 2 HARQ processes), when feedback disabling is not supported. In practice, due to the much inferior uplink budgets, the TACK value may be quite large, thereby exacerbating the impact further.

Table 2: Throughput/Latency penalty from not supporting feedback disabled HARQ processes.

| Number of HARQ processes (H) | HARQ Feedback Mode | Time to transmit H TBs ms | Throughput loss = Latency penalty (to transmit the same payload) |
| --- | --- | --- | --- |
| 1 | With disabling | TNPDCCH+4+TNPDSCH = 55 ms | - |
|  | No disabling | TNPDCCH+4+TNPDSCH+ (k0-1)+TACK + RTD =608.46+TACK ms | 608.46+TACK55>11.06x |
| 2 | With disabling | 2×(TNPDCCH+4+TNPDSCH) =110 ms | - |
|  | No disabling | TNPDCCH+4+TNPDSCH+ (k0-1)+TACK +RTD =608.46+TACK ms | 608.46+TACK110>5.53x |

Observation 1: For GEO Set 1 deployments, lack of feedback disabled HARQ process(es) results in a throughput/latency penalty of > 11x for UEs with one HARQ process and > 5.5x for UEs with two HARQ processes.

Proposal 1: Specify feedback-disabled HARQ processes for eMTC and NB-IoT over NTN

Further, for NB-IoT, currently there are only a maximum of 2 HARQ processes available. Keeping in mind that certain types of transmissions (e.g., MAC-CEs) will always need to have HARQ feedback enabled, we propose to increase the number of HARQ processes for NB-IoT over NTN to four (from two), to provide scheduling flexibility to the base-station so that HARQ stalling can be avoided, with the constraint that only up to two HARQ processes can have HARQ feedback enabled. This way, the soft-buffer requirements for NB-IoT UEs will not be impacted.

Proposal 2: RAN1 to support two additional feedback-disabled HARQ process for NB-IoT over NTN, resulting in a total of up to four HARQ processes.

- At most two out of these four HARQ processes will have HARQ feedback enabled, and the soft-buffer storage requirements for NB-IoT UEs will not be impacted.

# Minimum gap between transmissions

For eMTC over NTN, analogous to the parameter ‘T_proc’ for NR over NTN, there needs to be a minimum time gap between successive PDSCH transmissions over the same feedback disabled HARQ process. For NB-IoT, a 12-millisecond gap for this purpose is already captured in the specifications, presumably in the context of feedback-less SC-PTM communications.

Per usual eMTC processing timelines, we propose to introduce a gap of 4-milliseconds between successive PDSCH transmissions over the same HARQ process for eMTC over NTN.

Proposal 3: For eMTC over NTN, introduce a 4-millisecond gap between successive PDSCH transmissions over the same feedback disabled HARQ process.

# Impact to Multi-TB scheduling

Currently, for eMTC and NB-IoT, multi-TB scheduling assumes that all the TBs scheduled by a multi-TB DCI have HARQ feedback enabled. With the introduction of feedback-less HARQ processes into the mix (for NTN), RAN1 would need to revisit certain aspects related to multi-TB scheduling, such as how to transmit HARQ feedback for a multi-TB block where some TBs (or TB bundles) have feedback enabled, while some others have feedback disabled.

Proposal 4: RAN1 to discuss the impact of introducing feedback-less HARQ processes on multi-TB scheduling for eMTC and NB-IoT.

# Conclusion

In this contribution we presented our views on enhancements to NB-IoT/eMTC over NTN as it relates to disabling of HARQ feedback. We summarize our proposals below.

Observation 1: For GEO Set 1 deployments, lack of feedback disabled HARQ process(es) results in a throughput/latency penalty of > 11x for UEs with one HARQ process and > 5.5x for UEs with two HARQ processes.

Proposal 1: Specify feedback-disabled HARQ processes for eMTC and NB-IoT over NTN

Proposal 2: RAN1 to support two additional feedback-disabled HARQ process for NB-IoT over NTN, resulting in a total of up to four HARQ processes.

- At most two out of these four HARQ processes will have HARQ feedback enabled, and the soft-buffer storage requirements for NB-IoT UEs will not be impacted.

Proposal 3: For eMTC over NTN, introduce a 4-millisecond gap between successive PDSCH transmissions over the same feedback disabled HARQ process.

Proposal 4: RAN1 to discuss the impact of introducing feedback-less HARQ processes on multi-TB scheduling for eMTC and NB-IoT.