# 3GPP TSG RAN WG1 #109-e							R1-2205060

# e-Meeting, May 09th – May 20th, 2022

Agenda item:	9.12.3

Source: 	Qualcomm Incorporated

Title: 	Improved GNSS Operations for IoT-NTN

Document for:	Discussion and Decision

# Background

In RAN#94-e, the following objectives with RAN1 impact were included in the WID for eMTC/NB-IoT over NTN in Release 18:

IoT-NTN Performance Enhancements in Rel-18 to address remaining issues from Rel-17

This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below:

- Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2]

- Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption [RAN1]

In this contribution, we provide our initial views on aspects related to improved GNSS operations for IoT-NTN

# Baseline power consumption due to GNSS fixes

  - Parameters for evaluation of power consumption

To evaluate the power consumption impact of reading GNSS to maintain uplink synchronization (by appropriately pre-compensating the time and frequency offsets), we provide some reference parameters associated with the different transmit and receive operations that a UE may perform during establishing and maintaining a connection.

The key elements in evaluating power consumption associated with a transmit/receive operation at the UE are the current drawn by the UE (e.g., in milliamperes) and time-duration of the operation (e.g., in milliseconds). In Table 1 below, we report these numbers (with current values relative to a reference downlink reception current) for a typical NB-IoT over NTN scenario (e.g., a good coverage LEO satellite setting for Set 2 in [1]) corresponding to a downlink SNR (for 15 kHz 1 PRB reception) of 0 dB and an uplink SNR (for 15 kHz 1 PRB transmission) of -5 dB (with a PC5 UE transmitting at the max. power of 20 dBm).

Table 1: Parameters for evaluating power consumption in IoT over NTN.

| Operation | Current (Referenced to downlink current CDL) | Duration |
| --- | --- | --- |
| GNSS reception | CGNSS=1.1×CDL | 2000 ms |
| Downlink Reception | CDL | - PDCCH: 40 ms - PDSCH (RAR): 8 ms - PDSCH (Msg4): 8 ms - PDSCH (Conn. Release): 8 ms |
| Uplink Transmission | CUL=4.3×CDL | - PRACH: 1.6 ms - Msg3: 8 ms - PUSCH (data): 8 ms per ~1000 bits (simulated with 8000 bits per ON-duration) - HARQ-ACK: 8 ms |
| Sleep | Csleep=5.5×10-5×CDL | - PSM: 8 hrs - CDRX: ~10s |

  - Power consumption—short, sporadic connections

We model a short, sporadic connection according to Fig. 1 below. Essentially, the IoT UE is assumed to transmit its payload once every 8 hrs, once every 24 hrs, etc; after transmitting the payload, the UE’s connection is released, and it goes back into deep sleep mode, until before the next transmission occasion.

Figure 1: Short, sporadic transmissions for IoT over NTN.

In this setting, we observe that, acquiring a GNSS fix before every connection results in ~34% of the UE’s total power consumption resulting from GNSS alone. While there may be subtle differences depending on exact implementation, the general observation that GNSS consumes a significant chunk of the UE’s power—and thereby, directly reduces the expected battery life of the UE—is valid.

Observation 1: Under the studied scenario of short, sporadic connections, a GNSS fix before every connection consumes approximately 34% of the UE’s total available energy.

For UEs that are mobile, e.g., say tracking devices, etc., that are operating in this short, sporadic connection paradigm, this power penalty due to GNSS cannot be mitigated significantly, under the purview of Release 17 assumptions of GNSS-based uplink pre-compensation.

However, for UEs that are fixed, e.g., smart meters, etc, these UEs may be able to save power by having a much more relaxed (e.g., once a week, or once a month, depending on the setting) GNSS fixing schedule.

  - Power consumption—long connections (e.g., based on CDRX)

We model an example long connection according to Fig. 2 below. In this instance, the IoT UE may remain in connected mode for a significantly longer duration of time than the short, sporadic connections described above. These may be facilitated e.g., via connected mode DRX (CDRX), wherein much larger payloads are transmitted or received by the UE during the longer connection.

Figure 2: Long connection with connected mode DRX for IoT over NTN.

In this setting, we observe that a GNSS fix before every uplink transmission occasion (which, in the absence of other mechanisms, may end up being required to maintain uplink synchronization accuracy within specified limits) results in ~45% of the UE’s total power consumption resulting from GNSS fixes. While (as described above) we can mitigate this somewhat for fixed UEs, for mobile UEs (especially UEs moving at high speeds), without other enhancements to connected mode synchronization, we may not be able to avoid this hit to the UE’s power consumption on account of GNSS fixes.

Observation 2: Under the studied scenario of a long connection employing connected mode DRX (with a DRX cycle of ~10s), a GNSS fix before every uplink transmission consumes approximately 45% of the UE’s total available energy without additional enhancements w.r.t uplink synchronization.

- This is especially true for mobile UEs that cannot depend on a prior acquired GNSS fix

# Closed loop time and frequency correction

We show, in this section, that the above penalty to UE power consumption (during long connections) from GNSS fixes can be mitigated significantly by facilitating closed-loop time and frequency corrections issued by the base-station.

This may be achieved by a (potentially periodic, or prior to each uplink transmission) NPRACH transmission from the UE, followed by a timing and/or frequency correction command issued by the network in a response message. Especially in the setting where these NPRACH resources are robust to time and frequency synchronization errors (see, e.g., the design proposed in Section 4 of this contribution), such solutions can dramatically reduce the power consumption penalty from GNSS fixing during a long connection, by essentially replacing a GNSS fix with an NPRACH followed by a closed loop correction.

Such a strategy is (approximately) schematically depicted in Fig. 3 below.

Figure 3: Relaxed GNSS fixing using (N)PRACH-based closed loop corrections.

In Table 2, we demonstrate how the power penalty due to GNSS fixes diminishes with an increase in the GNSS relaxation factor, which essentially denotes—relative to the baseline scheme in Fig. 2—the factor by which we reduce the total number of GNSS fixes required and replace it with adequate close-loop corrections.

Table 2: Reduction in power penalty due to GNSS, by relaxing required GNSS fixes and enabling (N)PRACH-driven closed-loop time and frequency corrections.

| GNSS Relaxation factor | Power penalty due to GNSS in connected mode for long connections |
| --- | --- |
| No Relaxation (Baseline, as shown in Fig. 4) | 45% |
| 2 | 29% |
| 4 | 17% |

Based on the results above, we make the following observations and proposals with regards to supporting long connections in eMTC and NB-IoT over NTN.

Observation 3: For long connections in eMTC and NB-IoT over NTN, (N)PRACH-driven closed-loop time and frequency corrections lowers the GNSS power penalty from 45% to 17% (with a GNSS relaxation factor of 4), w.r.t a baseline without closed-loop corrections.

- An NPRACH design that is robust to time and frequency errors (e.g., the one based on restricted preambles in Section 4 of this contribution) is especially suitable for this.

Proposal 1: RAN1 to consider specifying (N)PRACH-driven closed-loop time and frequency corrections, to mitigate UE power consumption on account of GNSS fixes.

# Robust NPRACH design to enable reducing GNSS fixes

For the following aspects:

- Accessing a cell from IDLE mode (without essentially being required to always get a GNSS fix immediately before attempting to transmit PRACH)

- (N)PRACH-driven closed loop corrections in connected mode (e.g., during a long connection, after some inactive time)

a robust (N)PRACH design that can correct relatively large time and frequency offsets without significant specification impact is important.

Such a design may allow a relaxation of the time/frequency synchronization requirements (for (N)PRACH transmission) for a UE, which would translate to a reduction in the number and frequency of GNSS fixes required before and during each connection.

Proposal 2: RAN1 to consider specifying (at least a subset of) NPRACH resources with increased robustness to time and frequency errors, to facilitate:

- Accessing a cell from IDLE mode, while relaxing the requirement of an “immediately preceding” GNSS fix in all instances.

- Closed-loop corrections (e.g., after periods of UE inactivity), thereby reducing the number of GNSS fixes required during a connection.

One such design is to restrict NPRACH starting subcarriers (e.g., over a subset of all available starting subcarriers) that a UE can use for contention-based random access (CBRA). The simplest example is shown in Fig. 4 below, where “alternate starting subcarriers” can be selected by UEs for CBRA.

Figure 4: Example of "restrictions" on starting NPRACH subcarriers for CBRA. Alternate starting subcarriers may be selected for NPRACH transmission by a UE.

We show in Fig. 5 that with this restriction, the robustness of NPRACH to time and frequency errors improves significantly. This is due to the increased resiliency to ICI among preambles that is afforded by this design. Even with relatively large initial uplink frequency errors (e.g., up to 1kHz), the base station can determine and correct for these errors effectively (as demonstrated by the CDFs of residual timing and frequency errors after base-station processing of the NPRACH preamble). In the CDFs in Fig. 5, we see that with the alternate subcarrier restriction scheme (the red curve), the probability of the residual timing errors (after base-station processing) being outside +/- CP/2 (of NPUSCH) is very close to that of “unloaded” transmission—i.e., when only one preamble is transmitted—even with an up to 1 kHz initial frequency synchronization error. In contrast, in the “fully loaded” setting—i.e., when all preambles are used for NPRACH transmission by (e.g., other) UEs (represented by the cyan curve), the probability of residual timing errors after base-station processing being greater than +/- CP/2 increases significantly.

Figure 5: Cumulative distribution functions (CDFs) of residual timing error after base-station processing of NPRACH preambles transmitted by UE(s). In this setting, initial frequency errors can be up to +/- 1 kHz, and the power levels of different UEs are within +/- 10 dB of each other.

Observation 4: Restricting alternate starting subcarriers for NPRACH transmissions allows to correct for potentially large initial uplink frequency synchronization errors (e.g., up to 1 kHz)

- Such a scheme may facilitate UE power savings by relaxing the frequency and accuracy of GNSS fixes required.

- Such a scheme may also facilitate NPRACH-driven closed-loop corrections of time and frequency errors in connected mode, thereby reducing the power penalty from frequent GNSS fixes.

# Conclusion

In this contribution we presented our views on enhancements to NB-IoT/eMTC over NTN as it relates to improved GNSS operations. We summarize our proposals below.

Observation 1: Under the studied scenario of short, sporadic connections, a GNSS fix before every connection consumes approximately 34% of the UE’s total available energy.

Observation 2: Under the studied scenario of a long connection employing connected mode DRX (with a DRX cycle of ~10s), a GNSS fix before every uplink transmission consumes approximately 45% of the UE’s total available energy without additional enhancements w.r.t uplink synchronization.

- This is especially true for mobile UEs that cannot depend on a prior acquired GNSS fix

Observation 3: For long connections in eMTC and NB-IoT over NTN, (N)PRACH-driven closed-loop time and frequency corrections lowers the GNSS power penalty from 45% to 17% (with a GNSS relaxation factor of 4), w.r.t a baseline without closed-loop corrections.

- An NPRACH design that is robust to time and frequency errors (e.g., the one based on restricted preambles in Section 4 of this contribution) is especially suitable for this.

Proposal 1: RAN1 to consider specifying (N)PRACH-driven closed-loop time and frequency corrections, to mitigate UE power consumption on account of GNSS fixes.

Proposal 2: RAN1 to consider specifying (at least a subset of) NPRACH resources with increased robustness to time and frequency errors, to facilitate:

- Accessing a cell from IDLE mode, while relaxing the requirement of an “immediately preceding” GNSS fix in all instances.

- Closed-loop corrections (e.g., after periods of UE inactivity), thereby reducing the number of GNSS fixes required during a connection.

Observation 4: Restricting alternate starting subcarriers for NPRACH transmissions allows to correct for potentially large initial uplink frequency synchronization errors (e.g., up to 1 kHz)

- Such a scheme may facilitate UE power savings by relaxing the frequency and accuracy of GNSS fixes required.

- Such a scheme may also facilitate NPRACH-driven closed-loop corrections of time and frequency errors in connected mode, thereby reducing the power penalty from frequent GNSS fixes.

# References

[1] R1-2103070, “Scenarios applicable to NB-IoT/eMTC” by Qualcomm Incorporated, RAN1 104-bis-e.