3GPP TSG RAN WG1 #109-e	R1-2205075

e-Meeting, May 9th – May 20th, 2022

Agenda Item:	9.2.1

Source: 	Fujitsu

Title: 	Discussions on general aspects of AI/ML framework

Document for:	Discussion/Decision

Introduction

In RAN#94e, AI/ML for air interface has been approved as a SI of Rel-18 [1]. The goal of this SI is to identify a common AI/ML framework through use cases studies, which could be used in subsequent projects. In this contribution, we discuss general aspects of AI/ML framework, including terminologies, collaboration-level-based AI/ML framework, and life cycle management of AI/ML model. Besides, we also show our views on the common aspects of evaluation methodology, sub use case selection and STD impacts. In order to ease the discussion, “AI/ML for air interface” is abbreviated as “AI4AI”.

Terminologies

Although there were many studies on AI4AI in both academia and industry, we think it is still necessary to introduce common terminologies for this SI at the beginning stage, in order to align the understanding on the general functions and aspects of AI/ML framework among companies. In RAN3, the basic framework and terminologies have been concluded in the technical report TR 37.817 [2] for FS_NR_ENDC_data_collect. Most of terms defined in 37.187 can be reused, and with necessary updates, to adapt to the requests from three use cases in this SI.

Fig-1. Functional Framework for RAN Intelligence (TR 37.817)

Besides, the functional framework for RAN intelligence (as shown in Fig-1) given in 37.817 is a good reference to the discussions of framework for AI4AI. However, the big challenge in the design of framework for AI4AI is the AI related functions may talk to each other and work together over the air. Depending on different assumptions on how AI@gNB and AI@UE work together, and different RAN1/2 procedures related to three use cases, different type of frameworks may need to be developed. Even though, the framework of 37.817 can be used as the starting point for further framework investigation of AI4AI.

Proposal 1: The following aspects in RAN intelligence can be leveraged to facilitate the study of AI4AI.

- The terminologies defined for RAN intelligence in TR 37.817 can be reused for AI4AI with necessary updates to adapt to the three use cases of this SI.

- The functional framework for RAN intelligence in TR 37.817 can be used as the starting point for the framework studied in AI4AI.

Collaboration levels

In the objectives of the SI, the part on framework investigation is attached as follows:

AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment, model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate

There were many discussions on collaboration levels of AI/ML since Rel-18 workshop, different companies shared their understanding and considerations on this issue. However, there is no common understanding on the collaboration levels reached during or after the discussion. In our view, instead of classifying by how heavy the AI-related signaling is, collaboration levels can be categorized with regard to whether the AI/ML model(s) is(are) deployed to the gNB and UE jointly or separately.

3.1 Collaboration level 0 (Colla-0)

For the use case belongs to Colla-0, AI is either deployed at gNB or UE side. It is assumed that no additional signal/signaling is needed compared to conventional methods. In previous discussions, there were two sub-categories for Colla-0 proposed, 0a and 0b. 0b has a little standard impact, while 0a is purely an implementation algorithm. Some companies think it is difficult to differentiate 0b and Colla-1. In our view, Colla-0 is for the use cases in which AI or non-AI methods used in TX side is unaware to its RX side, or vice versa. In this sense, there is no additional signals or signaling introduced for supporting AI operation with respects to STD impacts.

One typical use case of Colla-0 is that gNB utilizes AI for beam management. In this case, instead of beam sweeping over all beams, gNB only down-selects several beams, and asks UEs’ feedback about beam measurement results. With the help of AI, gNB can derive all beams’ strength, and find the strongest beam for the UE. UE is unaware of the existence and operations of AI@gNB since no additional signaling compared to non-AI methods.

3.2 Collaboration level 1 (Colla-1)

For the use case belonging to Colla-1, AI is deployed at gNB side or at UE side independently. AI assist information (e.g., AI-related configuration information, AI-aided information, AI-labeled like information) is needed to enhance AI model operation and performance.

In collaboration level 1, it can be further classified as:

Colla-1/gNB: In this use case, AI resides in gNB side only, with AI assist information transmitted from UE;

Colla-1/UE: In this use case, AI resides in UE side only, with AI assist information transmitted from gNB.

For the use case of gNB-based AI for beam management as for Colla-0, with AI assist information such as the beam information from UE side, it evolves to the use case of Colla-1/gNB. It is expected that the additional AI assist information offers gain in terms of time consumption for beam alignment between gNB and UE.

Similarly, beam management can also be realized by the AI of Colla-1/UE, if assisted by beam information from gNB side. Compared to Colla-1/gNB, additional AI specific report configuration needs to be configured to enable the UE indicating the best beam which is not transmitted from gNB.

It is not a special case, use cases belong to Colla-1/UE will have more standard impacts than those of Colla-1/gNB. Besides, they have big differences with respect to their lifecycle management procedure as discussed in Section 4.1. While for the use cases belonging to each of them, there are common aspects and similarities. Therefore, we think it is necessary to have Colla-1/UE and Colla-1/gNB in the framework studies.

3.3 Collaboration level 2 (Colla-2)

In the use case belonging to Colla-2, one AI network is used at UE side (i.e., TX side), and its paired AI network is used at gNB side (i.e., RX side), as illustrated in Fig-2. The AI network at UE side and that at gNB side should be trained together offline before deployment.

Fig-2. A paired AI network

Y=fX

Y'=h(Y)+n                                                           (Eq1)

X'=g(Y')

X-X'→0

The training procedure can be formulated as Eq1, where f(∙) is the transfer function from input signal, X is source information of the neural network at TX, Y is its output signal, h(∙) is the channel response coefficient between TX and RX, n is Gaussian noise, g(∙) is the transfer function from the input signal, Y’ is the input signal of the neural network at RX, and X’ is its output signal. In the iterative training procedure, the loss function is to make X’ to be approached to X as closely as possible, i.e., X-X'→0. The loss function may use cos similarity, NMSE, or MSE as the metric. After training, the TX neural network and the RX neural network should be deployed at TX side (e.g., UE side) and RX side (e.g., gNB side), respectively, and communicate with each other through radio interface over the air. One typical use case of Colla-2 is CSI overhead reduction, AI encoder/decoder is used to compress/de-compress the CSI feedback bits from UE to gNB.

In this SI phase, the focus of the studies should be put on (sub) use cases of Colla-1 and Colla-2, and take the (sub) use cases of Colla-0 as a reference.

Proposal 2: The use cases for AI4AI can be categorized into cases with the following three collaboration levels:

- Colla-0: in this use case, AI resides in either gNB or UE, with no additional information exchanged compared to conventional methods.

- Colla-1: in this use case, AI resides in either gNB or UE, with assistant information exchanged for AI operation.

  - Colla-1/gNB: AI only resides in gNB, with AI assistant information transmitted from UE;

  - Colla-1/UE: AI only resides in UE, with AI assistant information transmitted from gNB.

- Colla-2: AI network at UE side operates jointly with its paired AI network at gNB side. AI networks in UE and gNB are trained together and operated together.

Framework and lifecycle management of AI model

From the descriptions of aforementioned collaboration levels between UE and gNB, it is difficult to use one common framework to cover all three levels. Therefore, collaboration-level specific framework can be studied at first. For each collaboration level, the framework can be studied with the focus on three parts:

Model inference: It is the function of AI/ML model operating, and provides output signals based on its input signals. It is an indispensable function for every use case in each collaboration level.

Model training: It is the function of AI/ML model online training, targeting to update AI models. Data collection, model deployment/update are the issues revolving this key function.

Model monitoring: It is the function for monitoring the performance of AI/ML operation. For AI4AI, we think that it is necessary to have this function in the framework, since the performance of a model deployed at TX side may need to be monitored by RX side. On the other hand, as a data driven technique, it is difficult for an offline trained AI model to work well in diverse radio scenarios of NR applications all the time. Therefore, there is a need to terminate the AI model operation and return to the non-AI based method when the AI model performance is not desirable. Even if model updating is supported, there is a need to ensure that the model update procedure will not interrupt the real-time signal processing in these AI4AI use cases. As a result, model ON/OFF, the outputs of a model monitor, are needed to command the operation of the model inference, which may reside either in its same side, or its opposite side.

Besides, the interface of these functions should be studied as well.

4.1 Functional framework of Colla-0

As illustrated in Fig-3, the functional framework of Colla-0 is composed by the following components.

Fig-3. Functional Framework of Colla-0

Model inference:

It is the operation part of the AI function, outputting inference results of the input signals according to the model trained.

Both the AI inputs and outputs should reuse legacy NR signals, and thus the communication peer device is unaware of the existence of the AI.

Model monitor:

It relies on the implementation for Colla-0, since whether the processing is done by AI or non-AI methods at one end is transparent to its communication end.

Model training and update:

It is an implementation issue as well.

4.2 Functional framework of Colla-1

Please refer to Fig-4 for the functional framework of Colla-1.

Fig-4. Functional Framework of Colla-1

Model inference:

The input and output signals of model inference are the same as those of conventional non-AI methods. The distinct feature of Colla-1 is that AI-aid signal(s) is(are) needed to assist the AI/ML model operation. Meanwhile, the design of AI-aid signals for Colla-1/gNB and Colla-1/UE should be studied for each selected use case and sub use cases therein.

Model monitor:

For Colla-1/gNB, the model monitor may supervise the AI model@gNB or the AI model@UE. If it is used to monitor the model at UE, it may send signals to stop the operation of the AI model at UE, depending on the performance of the model@UE.

For Colla-1/UE, the model monitor may supervise the AI model@UE or the AI model@gNB. If it is used to monitor the model at gNB, it may send signals to gNB as a stop indication or stop request, depending on the performance of the model@gNB.

For both of the two cases, the inputs of monitor may be feedback signals such as ACK/NACK, or may be monitor-aid signals which is transmitted to its peer side.

Model training and updating:

For Colla-1/gNB, model training and updating can be done in gNB side. When model performance detected by the model monitor is poor, it can trigger data collection and model online training. The way of doing data collection, online training and testing is outside the scope of this SID. But the signals for training procedure need to be studied. In this contribution, we name it as training-aid signals, which can be label like signals for further optimization of the model parameters. Besides, the procedure of model updating also needs to be clarified if there are impacts on the performance or STD.

In our view, the training procedure is a non-real-time procedure and thus can be done when there is lack of traffic. The distinctive advantage of online training at gNB is that the AI@gNB can be optimized by each UE that it talked to. Also, the overhead from the training-aid signals can be regarded as negligible, considering the fact that the model updating is an infrequent event for a gNB. For an online trained AI model, its performance should be better than the model being used at least. The metric and the procedure of model updating should be studied to ensure that there is no performance degradation resulting from the update.

For Colla-1/UE, whether online training and model updating need to be supported should be clarified first. For an AI model deployed at a commercial UE, the performance of its AI model would have passed relevant RAN4 test cases. If online training and model updating is supported, how to ensure its performance being complied to RAN4 request is problematic. Besides, it is also challengeable to support this feature with regard to hardware limitation in UE side.

4.2 Functional framework of Colla-2

Please refer to Fig-5 for the functional framework of Colla-2.

Fig-5. Functional Framework of Colla-2

Model inference:

For Colla-2, model deployed at UE side operates with its paired model at gNB side. The output of model inference at UE is fed into the input of model inference at gNB side through the air interface.

Model monitor:

One unique feature of the paired AI network is to align the operations between the AI@UE and the AI@gNB. Besides studying the signals and the procedures for the performance monitoring, the way of synchronizing the switch of the paired models should be studied.

Model training and updating:

Similar to Colla-2, online training and model updating are challengeable because the paired models are deployed separately at UE and gNB. It seems not feasible to support joint online training which introduces excessively high overhead to air interface. Similar to the discussion for Colla-1, it remains as a problem to ensure that its performance complies with RAN4 test cases, which needs to be studied for model updating. One possible way is to transfer an AI model from gNB to UE, which can be used for both initial deployment of AI model at UE side and model updating. But the overhead introduced by transmitting the AI model over air should be restricted.

From the discussion above, we think that the study on functions of model inference and model monitoring should be prioritized. While for the study on life cycle management of AI model, such as online training and model update, further clarification is needed in order to reach a common understanding among companies. For example, one way to do AI model updating is through downloading the model released from the core network. But it seems out of the scope of current SID.

Proposal 3: Collaboration-level specific functional framework should be clarified through this SI phase.

Proposal 4: The “ON/OFF” feature of AI/ML models should be supported in order to switch between an AI-based method and a non-AI-based method.

Proposal 5: The model monitoring unit should be added into the functional framework, and it is at least responsible for monitoring model performance and command the ON/OFF of AI/ML model.

Proposal 6: As for the AI/ML framework, the study should focus on the following three functions and the input/output interfaces among them.

- Model inference.

- Model monitoring.

- Model training.

Proposal 7: The scope of the AI-model lifecycle management, including online training and model updating, should be further clarified in the study of the AI/ML frameworks.

Evaluation methodology

To evaluate the performance gain of AI/ML based algorithms for the agreed use cases in the final representative set, methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations, are agreed as the descriptions in the SID.

Besides, additional AI/ML specific evaluation methodology should be studied and concluded. Here, we share our view on how to do calibration and dataset construction for the evaluation.

5.1 Calibration

AI4AI is a brand-new technique of NR air interface, as a whole Rel-18 SI, there is a need to calibration on the evaluation of AI/ML’s gains over baseline method among companies.

Our views for the calibration are shown as follows:

- As the reference for performance comparison, the baseline method should be aligned.
For calibration, each company should use the same non-AI method as the baseline method. It is desirable that the calibration on the non-AI method can be done first.

- Common datasets for each use case are not recommended. The implemented AI/ML models for each company may vary, therefore, the input and output data dimensions, the necessary data pre-processing strategies, and the AI/ML model capabilities may be different for different companies. In the calibration phase, it is not feasible to construct common datasets for all kind of models. Instead, a common set of simulation parameters and scenarios should be defined for each use case based on TR 38.901 and TR 38.857.

- For the AI/ML model side, we suggest have one reference model for each use case, the reference model should be general and easy to be implemented, which requires few extra knowledge to use it. The way to conclude the reference model need to be discussed further.

5.2 Dataset and field data

On dataset: The amount of the data samples in one dataset should be large enough to cover all possible variations in the expected scenarios, in which the AI/ML model will be applied. However, too much data may increase the complexity, and makes it harder for quick performance calibration at this stage. Therefore, a decent dataset size range can be discussed and defined for each use case.

The dataset should be split into training and validation/testing subsets. The split ratio may be flexible, but the homogeneity among those subsets must be assured.

In general, as to LLS, data sets covering a wide range of delay spread, doppler frequency shift, and SNR range should be constructed.

For SLS, the evaluation should take AI residing in UE or AI residing in gNB into account. Considering UE’s mobility, AI’s robustness and generalization capabilities should be well studied for the use case assuming of AI@UE because UE may encounter various wireless environments with a wide range of channel characteristics, so mixed (simulation) scenarios characterizing the UE mobility, in which reasonable randomness of the channel parameters is taken into account in the dataset construction.

While for AI@gNB, the verification of the generalization capability of the AI model may be relaxed because the gNB does not move. So, statistical models from TR 38.901 and TR 38.857 would be enough for its dataset constructions.

simple scenarios based on conventional models in TR 38.901 can be used directly to construct datasets since it is not moved.

On field data: our view is to exclude the usage of field data for evaluation at this stage. To utilize sufficient field data is desirable to conclude AI’s gain. However, there are too many indefinite factors and uncertainties related on how to use field data. For example, the characteristics of field data, the way to collect enough field data, the hardware and software diverse in the data collections. All these issues make it infeasible for this SI phase.

Proposal 8: For this brand-new technique, calibration among companies is needed in order to compare AI/ML with the baseline method.

Proposal 9: The baseline method for reference should be the same in calibration.

Proposal 10: For datasets and AI/ML model(s) used in calibration:

- Instead of a common dataset, common scenarios with common simulation parameters can be determined for calibration of AI performance evaluation.

- Reference AI model is needed, and companies should provide simulation results by using the reference AI model.

Proposal 11: The follows are considered for evaluations with AI@UE and AI@gNB.

- To evaluate the use case with the assumption of AI@UE, mixed scenarios should be used considering that a UE usually experiences different radio environments.

- To evaluate the use case with the assumption of AI@gNB, scenarios with the statistical models from TR 38.901 and TR 38.857 can be used.

Sub use case selection

For each use case, representative sub use cases should be finalized by RAN1#111. Considering heavy workload for investigation and evaluation of the three use cases in different areas, minimizing the number of sub use cases to be studied is desired. Since the commonalities within a collaboration level framework as addressed in Section 4, selecting one sub use case for each collaboration level would be sufficient for STD impact’s study.

Proposal 12: In order to limit the workload, only one sub use case is selected for one collaboration level in a use case.

STD impacts

STD impacts can be studied from two aspects:

Part-1: Model inference related STD impacts can be discussed first with high priority.

Part-2: Model training and updating related issues can be discussed later, after a common understanding on lifecycle management of AI/ML model has been reached among companies.

Proposal 13: For STD impacts, model inference related STD impacts should be prioritized and discussed first. Model training and model updating related STD impacts can be studied later after common understanding on lifecycle management of AI/ML model is reached among companies.

Conclusions

Proposal 1: The following aspects in RAN intelligence can be leveraged to facilitate the study of AI4AI.

- The terminologies defined for RAN intelligence in TR 37.817 can be reused for AI4AI with necessary updates to adapt to the three use cases of this SI.

- The functional framework for RAN intelligence in TR 37.817 can be used as the starting point for the framework studied in AI4AI.

Proposal 2: The use cases for AI4AI can be categorized into cases with the following three collaboration levels:

- Colla-0: in this use case, AI resides in either gNB or UE, with no additional information exchanged compared to conventional methods.

- Colla-1: in this use case, AI resides in either gNB or UE, with assistant information exchanged for AI operation.

  - Colla-1/gNB: AI only resides in gNB, with AI assistant information transmitted from UE;

  - Colla-1/UE: AI only resides in UE, with AI assistant information transmitted from gNB.

- Colla-2: AI network at UE side operates jointly with its paired AI network at gNB side. AI networks in UE and gNB are trained together and operated together.

Proposal 3: Collaboration-level specific functional framework should be clarified through this SI phase.

Proposal 4: The “ON/OFF” feature of AI/ML models should be supported in order to switch between an AI-based method and a non-AI-based method.

Proposal 5: The model monitoring unit should be added into the functional framework, and it is at least responsible for monitoring model performance and command the ON/OFF of AI/ML model.

Proposal 6: As for the AI/ML framework, the study should focus on the following three functions and the input/output interfaces among them.

- Model inference.

- Model monitoring.

- Model training.

Proposal 7: The scope of the AI-model lifecycle management, including online training and model updating, should be further clarified in the study of the AI/ML frameworks.

Proposal 8: For this brand-new technique, calibration among companies is needed in order to compare AI/ML with the baseline method.

Proposal 9: The baseline method for reference should be the same in calibration.

Proposal 10: For datasets and AI/ML model(s) used in calibration:

- Instead of a common dataset, common scenarios with common simulation parameters can be determined for calibration of AI performance evaluation.

- Reference AI model is needed, and companies should provide simulation results by using the reference AI model.

Proposal 11: The follows are considered for evaluations with AI@UE and AI@gNB.

- To evaluate the use case with the assumption of AI@UE, mixed scenarios should be used considering that a UE usually experiences different radio environments.

- To evaluate the use case with the assumption of AI@gNB,  scenarios with the statistical models from TR 38.901 and TR 38.857 can be used.

Proposal 12: In order to limit the workload, only one sub use case is selected for one collaboration level in a use case.

Proposal 13: For STD impacts, model inference related STD impacts should be prioritized and discussed first. Model training and model updating related STD impacts can be studied later after common understanding on lifecycle management of AI/ML model is reached among companies.

- References

[1] RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, 3GPP TSG RAN Meeting #94e, E-meeting, Dec 6th-17th, 2021.

[2] TR 37.817, “Study on enhancement for data collection for NR and ENDC”, Rel-17, Mar. 2022.