3GPP TSG RAN WG1 #109-e		R1-2205078

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.2.3.1

Source:	Fujitsu

Title:	Evaluation on AI/ML for beam management

Document for:	Discussion/Decision

Introduction

According to SID [1], the performance benefits of the AI/ML method should be evaluated for agreed use cases. This contribution presents the evaluation results of beam prediction in spatial domain including views on evaluation methodology and KPIs.

Evaluation on beam prediction in spatial domain

For analogue beam selection in FR2 band, base station shall configure RSs for beam measurement. And UE needs to measure RSRPs of a lot of beams to identify the potential best beam pairs. It causes large RSs overhead and latency of beam selection.

With AI/ML model, the RSRPs for the sub-set of beam pairs are measured and AI/ML model will predict the RSRPs of all beam pairs. Then, the potential best beam pair is selected among the predicted RSRPs of all beam pairs. In this case, the RSs overhead and latency of beam selection will be reduced. The function of AI/ML model are shown as Figure 1.

Beams in transmitter

Beams in receiver

Not measured beam

measured beam

AI/ML

Model

RSRPs for sub-set of beam pairs

RSRPs for all beam pairs

Beams in transmitter

Beams in receiver

Not measured beam

measured beam

AI/ML

Model

RSRPs for sub-set of beam pairs

RSRPs for all beam pairs

Figure 1: the function of AI/ML model

Evaluation methodology

Calibration

To compare the potential gains of the AI/ML methods among companies without confusion, a calibration procedure for evaluation is recommended.

In terms of calibration for the evaluation of AI/ML method, one important factor is the dataset since AI/ML model’s performance is highly dependent on it. Considering that the dataset is generated by SLS, it’s recommended to have a common set of simulation condition.

Proposal 1: To compare the performance of the AI/ML method among companies, the common simulation condition shall be used.

Another important factor for the calibration is the AI/ML model. Different companies may have their dedicated models, but it will cause the difficulty for alignment of the simulation results. On the other hand, the dedicated models may involve many implementation issues which are beyond our discussion. To calibrate the simulation results, a common reference model such as full connection neural network is recommended in SI phase.

Proposal 2: To calibrate the performance of the AI/ML method, a common reference model is recommended, and its generation procedure needs to be studied.

KPI

To evaluate the performance of beam prediction on AI/ML method, the following KPIs are used.

- Probability of correct prediction

The correct prediction is defined as that the predicted best beam pair equals to the actual best beam pair. In some cases, the RSPRs on several strongest beams are almost the same, so the probability of the predicted best beam pair coincides with one of the top N strongest beams is also calculated as a reference.

- Average L1-RSRP difference

This KPI is defined as the average L1-RSRP difference between predicted best beam pair and the actual best beam pair.

On the other hand, the complexity of AI/ML model is another factor to be evaluated. And the following KPIs are used.

- Computational complexity of AI/ML model inference

- Memory size for AI/ML model storage

Proposal 3: The following KPIs are recommended for performance evaluation of beam prediction.

  - Probability of correct prediction

  - Average L1-RSRP difference

Proposal 4: The following KPIs are recommended for complexity of AI/ML models.

  - Computational complexity of AI/ML model inference

  - Memory size for AI/ML model storage

Simulation results

In our simulation, gNBs are assumed to be configured with 64 antennas which support 12 transmitting beams (2 beams in vertical and 6 beams in horizonal). UEs are configured with 16 antennas which support 8 receiving beams (2 beams in vertical and 4 beams in horizonal). The details about the simulation parameters are shown in Appendix.

For the dataset generated by SLS, 80% data are used for model training and 20% data are used for model test. The AI/ML model is configured as a 5-layer fully connected neural network. The loss function of the network is MSE which gauges the differences between the predicted RSRPs and the ground truth.

With AI/ML model, the performance of 2 cases is evaluated with different down-scaling rates of the subsets of beam pairs.

Case 1:  down-scaling rate: 4

6 transmitting beams and 4 receiving beams (total 24 beams) are used to predict all the 96 beams.

Beams in transmitter

Beams in receiver

Not measured beam

Measured beam

Beams in transmitter

Beams in receiver

Not measured beam

Measured beam

Figure 2: subset of beam pairs for case 1

Case 2: down-scaling rate: 6

4 transmitting beams and 4 receiving beams (total 16 beams) are used to predict all the 96 beams.

Beams in transmitter

Beams in receiver

Not measured beam

Measured beam

Beams in transmitter

Beams in receiver

Not measured beam

Measured beam

Figure 3: subset of beam pairs for case 2

The simulation results are shown in the following table.

Table 1: simulation results

|  | Overhead/latency reduction | Average RSRP difference(dB) | TOP2 (%) | TOP4 (%) | TOP8 (%) |
| --- | --- | --- | --- | --- | --- |
| Case 1 | 75% | 2.6 | 82.9 | 93.0 | 97.3 |
| Case 2 | 83% | 4.3 | 51.5 | 72.9 | 86.3 |

Comparing to the optimal beam selection method, total 96 beams (12 transmitting beams and 8 receiving beams) are measured while the AI/ML method only measures 24 and 16 beam pairs for case 1 and 2. In this case, the RSs overhead and latency of beam selection are reduced 75% and 83%, respectively.

From the simulation results, the smaller down-scaling rate of subsets of beam pairs achieves better performance. When down-scaling rate equals to 4, the probability of the predicted best beam pair coinciding with the top 2 strongest beam pairs is larger than 80%, and the average RSRP difference is less than 3dB.

For the complexity of AI/ML model,

- The computational complexity of AI/ML model is about 20K FLOPs.

- The model size is about 45KB with pytorch model file format.

Observation:

- Compared to the optimal beam selection method, the overhead and latency of the AI/ML method are reduced 75% and the performance has less than 3dB degradation.

- For the complexity of AI/ML model, the computational complexity is about 20K FLOPs, and the memory size for model storage is about 45KB.

- Conclusion

In this contribution, we present our evaluation results of beam predication on AL/ML beam management. For the discussion, we have the following proposals and observation.

Proposal 1: To compare the performance of the AI/ML method among companies, the common simulation condition shall be used.

Proposal 2: To calibrate the performance of the AI/ML method, a common reference model is recommended, and its generation procedure needs to be studied.

Proposal 3: The following KPIs are recommended for performance evaluation of beam prediction.

  - Probability of correct prediction

  - Average L1-RSRP difference

Proposal 4: The following KPIs are recommended for complexity of AI/ML models.

  - Computational complexity of AI/ML model inference

  - Memory size for AI/ML model storage

Observation:

- Compared to the optimal beam selection method, the overhead and latency of the AI/ML method are reduced 75% and the performance has less than 3dB degradation.

- For the complexity of AI/ML model, the computational complexity is about 20K FLOPs, and the memory size for model storage is about 45KB.

- Appendix

Table 2:  Parameters of SLS for beam prediction

| Parameters | Value |
| --- | --- |
| Scenario | TR38.901: Uma, 19 sites, 3 sectors per site (ISD=500m) |
| Carrier frequency | 30GHz |
| System bandwidth | 50MHz |
| Sub-carrier space | 120KHz |
| TXRU mapping to antenna elements | One TXRU per panel per polarization |
| gNB TX power | 43dBm |
| gNB antenna configuration | [Mg, Ng, M, N, P] = [1, 1, 4, 8 ,2], (dH,dV) = (0.5, 0.5)λ |
| UE antenna configuration | [Mg, Ng, M, N, P] = [1, 1, 2, 4, 2], (dH,dV) = (0.5, 0.5)λ |
| Noise figure | 9dB |
| gNB beam pattern | [-60, 60] in azimuth domain and [90, 160] in zenith domain Azimuth angle (degree) = [-56.25, -33.75, -11.25, 11.25, 33.75, 56.25] Zenith angle (degree) = [112.5, 157.5] Total 12 beams = 6(H)*2(V), DFT beams |
| UE beam pattern | Azimuth angle (degree) = [-67.5, -22.5, 22.5, 67.5] Zenith angle (degree) = [45.0, 135.0] Total 8 beams = 4(H)*2(V), DFT beams |
| Beam selection method | L1-RSRP |
| Dataset | 85.5K samples = 19 sites * 3 sectors per site * 10 users per s *150 drops |

- Reference

# [1] RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, 3GPP TSG RAN Meeting #94e, E-meeting, Dec 6th-17th, 2021