3GPP TSG RAN WG1 #109-e		R1-2205079

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.2.3.2

Source:	Fujitsu

Title:	Sub use cases and Spec. impact on AI/ML for beam management

Document for:	Discussion/Decision

Introduction

In RAN#94e e-meeting, the SID [1] on AI/ML for air interface has been agreed. Three use cases have been selected as initial set during study phase.

Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

This contribution presents our views on AI/ML for beam management including representative sub use case and the potential specification impact.

Discussions and Proposals

Sub use cases selection

As described in SID [1], the possible sub use cases for beam management may include

- Beam prediction in time domain

- Beam prediction in spatial domain

- Beam selection accuracy improvement

The specification impacts are similar for above possible sub use cases since all of them can be categorized to collaboration level 0 or 1. To limit the workload, only one sub use case is selected for further study. Among the above possible sub use cases, the beam prediction in spatial domain is more suitable for the AI/ML method to observe the benefits of AI/ML model.

Proposals 1: To limit the workload for evaluation, beam predication in spatial domain can be selected as the only sub use case for beam management.

Potential specification impact

In terms of beam prediction in spatial domain, the function of AI/ML model can be treated as the spatial interpolation with the measurement results of the subset of beam pairs. The parameters like beam number of transmitting and receiving beams, 3D-beam pattern (vertical and horizonal beam number) are critical for AI/ML model training and inference. If AI/ML model locates in UE, UE should know the transmitting beam number and the beam pattern for the model training and inference. However, current specification does not support it and new signaling is required. On the other hand, if AI/ML model locates in gNB, even though UE reports its capability to gNB about the maximal receiving beam numbers, the receiving beam pattern is unknown to gNB. New signaling is required to carry such information.

Proposal 2: New signaling carrying beam information in spatial domain should be studied.

- Conclusion

In this contribution, we present our views on sub use cases of AI/ML beam management. For the discussion, we have the following proposals.

Proposals 1: To limit the workload for evaluation, beam predication in spatial domain can be selected as the only sub use case for beam management.

Proposal 2: New signaling carrying beam information in spatial domain should be studied.

- Reference

# [1] RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, 3GPP TSG RAN Meeting #94e, E-meeting, Dec 6th-17th, 2021