3GPP TSG RAN WG1 #109-e	R1-2205085

e-Meeting, May 9th – 20th, 2022

Source: 	Fujitsu

Title:	Side control information for NR network-controlled repeaters

Agenda item:	9.8.1

Document for:	Discussion and decision

- Introduction

In RAN#94e, the study item on NR network-controlled (NC) repeaters was approved [1]. To enable the network-controlled functionality of repeaters, the following candidates of side control information were suggested for study.

Study and identify which side control information below is necessary for network-controlled repeaters including assumption of max transmission power [RAN1] - Beamforming information - Timing information to align transmission / reception boundaries of network-controlled repeater - Information on UL-DL TDD configuration - ON-OFF information for efficient interference management and improved energy efficiency - Power control information for efficient interference management (as the 2nd priority)

In this contribution, we will discuss the standardization impacts of NR NC repeaters and the side control information for the NC repeaters.

- Network-controlled repeaters and standardization aspects

| (A) An RF repeater amplifies and forwards signal between the gNB and a UE. （The antenna direction of RF repeaters is almost not adjustable.） | (B) RF repeaters are transparent to both the gNB and UEs. （F link stands for forwarding link between the gNB and UEs.） |
| --- | --- |

Figure 1 RF repeaters

NR RF repeaters were specified in Rel-17. They have two antenna panels on opposite sides, as shown in Figure 1 (A). The beam direction of RF repeater antenna panels is fixed and need a manual setup. Usually, one antenna panel is set towards the gNB and the other one is set towards UEs. After the setup, the directions of beams/antenna panels are rarely changed. RF repeaters keep receiving signal from one side, then amplifying and forwarding the received signal to the other side, as shown in Figure 1 (B). RF repeaters are transparent to both the gNB and the UEs.

| (A) An NC repeater amplifies and forwards signal between a gNB and a UE. The beams of the NC repeater are adjusted and controlled by the gNB. | (B) NC repeaters are transparent to UEs. NC repeaters can communicate with the gNB to obtain side control information, such as beamforming indication. （‘F link’ stands for forwarding link between the gNB and UEs. ‘C link’ stands for communication link between the gNB and NC repeaters.） |
| --- | --- |

Figure 2 NR network-controlled (NC) repeaters

The drawbacks of RF repeaters and the expectations for NC repeaters were clearly described in the SID. We will not repeat it here. In our understanding, as an enhancement of RF repeaters, the essential difference of an NC repeater is its beam direction can be adjusted and controlled by the gNB. More specifically, we have following observation.

Observation 1: The essential difference between a network-controlled repeater and an RF repeater include:

- Network-controlled repeaters can follow the beam indications from the gNB and dynamically/semi-statically/periodically sweep Tx/Rx beams towards the gNB and Rx/Tx beams towards the UE(s), while RF repeaters cannot.

- Network-controlled repeaters can receive the side control information from the gNB which at least includes beam indications, while RF repeaters cannot.

To support NC repeaters in Rel-18, two standardization aspects should be considered and concluded in the SI phase:

- Determine how an NC repeater communicates with the gNB.

This aspect is about the communication capability of an NC repeater. It is more relevant to the SI scope about the signaling to carry the side control information.

- Determine what kinds of side control information are required for an NC repeater, in order to support its signal forwarding operation via adjustable/controllable beams.

This aspect is about the forwarding capability of an NC repeater with adjustable/controllable beams.

These two standardization aspects are dependent. “What kinds of side control information can be supported for an NC repeater” highly depends on “How the NC repeater communicates with the gNB,” and vice versa.

This contribution focuses on the second aspect which is about side control information. The first aspect about the communication capability of an NC repeater is discussed in our accompanied contribution [2].

Proposal 1: The following two standardization aspects should be considered and concluded in the study item of NR network-controlled repeaters:

- How does a network-controlled repeater communicate with the gNB? What kinds of signaling can the network-controlled repeater receive from the gNB?

- What kinds of side control information are required for a network-controlled repeater to support its signal forwarding operation via adjustable/controllable beams?

Observation 2: The standardization aspects listed in Proposal 1 are dependent on each other. “What kinds of side control information can be supported for a network-controlled repeater” highly depends on “How the network-controlled repeater communicates with the gNB”, and vice versa.

- Side control information and standardization impacts

According to the SID [1], the following side control information will be considered and evaluated in the SI phase.

- Beamforming information.

- Timing information to align transmission / reception boundaries of network-controlled repeater.

- Information on UL-DL TDD configuration.

- ON-OFF information for efficient interference management and improved energy efficiency.

- Power control information for efficient interference management (as the 2nd priority).

All the candidates of side control information are beneficial for NC repeaters as we discussed in RANP before the SI was approved. Given that the TUs of this item is limited, the key factor to whether introduce them may be their required STD effort and their effect. In the following, we will discuss the standardization impacts of these candidates one by one.

It is noticed that all side control information is for forwarding link (F link). The control information for communication link (C link) should be discussed as the configuration of signaling to carry side control information, i.e., another RAN1 scope which is discussed in our accompanied contribution [2].

For the convenience of the discussion, a feasible example of the behaviour/procedure of an NC repeater is given in Example 1. The example is for the help to clarify when side control information is received.

Example 1 A feasible example of behaviour of an NC repeater

Taking Figure 3 as a reference, one feasible example of an NC repeater is given below: As shown in Figure 3 (A), probably acting like a UE, an NC repeater: 1. Receives SSB/SIB1, at least to get the cell-specific UL/DL TDD configuration. 2. Sends Msg 1 to the gNB and receives Msg 2 from the gNB to get a rough timing (a rough timing advance). 3. [Possibly]Completes the initial access procedure and establishes an RRC connection with the gNB. 4. [Possibly]Follows the indications of the gNB to do beam calibration for C beams. 5. Gets ready to receive the side control information. 6. Receives the side control information when necessary. As shown in Figure 3 (B), acting as a Repeater, the NC repeater: 7. Starts to amplify and forward the signal between the gNB and the UE, following the side control information.

  - Beamforming information

| (A) An NC repeater obtains side control information from the gNB via C beams. Signal received or transmitted via C beams is recognizable for the NC repeater. (‘C beam’ stands for the communication beams of the NC repeater. The C beam is used to receive or transmit signal of the NC repeater’s own.) | (B) An NC repeater amplifies and forwards signal between the gNB and the UE through F beams. The NC repeater cannot recognize the signal through F beams. (‘F beam’ stands for the forwarding beams of the NC repeater including the beams at gNB side and at UE side.) |
| --- | --- |

Figure 3 NC repeaters communicate with the gNB via C beam and forward signal via F beam.

As we observed above, beam-based forwarding is the most essential/expected function of NC repeaters. The beamforming side control information should be supported. The gain of NC repeaters with beam sweeping is shown in Figure A1 in Appendix.

In Figure 3, we separate beams of NC repeaters into two kinds, i.e., the beams for communication with the gNB as shown in (A) and the beams for the signal forwarding between the gNB and UEs as shown in (B). For convenience, we use C beams to stand for the communication beams and F beams to stand for the forwarding beams.

In our understanding, for the F beams of NC repeaters, only the beam indication is needed. The potential STD impacts of F beam indication are summarized in Table 1. A detailed discussion follows.

Table 1 Potential STD impacts of beam indication for forwarding beams of network-controlled repeaters.

|  | F beams at gNB side | F beams at UE side |
| --- | --- | --- |
| Beam indication | - Same as C beams - No CA/DC - Single TRP is enough - Rel-17 unified TCI is preferred. | - New design of Tx/Rx F beam of NC repeaters is needed, such as beam index. - TCI-based beam indication does not work for F beams at UE side, because the RSs though F beams at UE side are not for NC repeaters and the NC repeaters cannot identify them. |
| Potential STD aspects | - Can be very limited, if it is agreed that F beams at gNB side can completely follow C beams. | - New feature. STD effort is required to discuss: - How to indicate a certain F beam at UE side to NC repeaters. - Signaling design |

In legacy NR, to manage the beams of UEs, the gNB instruct the UEs to measure beams and report the measurement results when necessary. The gNB select Rx/Tx beams for the UEs and indicate them to the UEs. Different logical channels may be transmitted or received via different beams because they may require different reliability/latency. Many rules for UEs are specified in legacy specs to determine Rx/Tx beam of a certain logical channel/RS. It is noted that UEs measure the RS received via a certain beam as the measurement of the beam.

As shown in Figure 3 (A), C beams for NC repeaters are like beams of UEs. C beams are used to transmit/receive the NC repeaters’ own information/signal.

F beams are very different from C beams. The measurement of F beams is neither necessary nor possible for NC repeaters. F beams are like some pipes, as shown in Figure 3 (B). The Tx/Rx signal through F beams cannot be differentiated by the NC repeater. Therefore, the NC repeater can hardly measure anything based on the signal through F beams.

The only thing about F beams an NC repeater needs to know is which F beam should be used at which time. The gNB can determine F beams based on the measurement result done by itself and by the UEs served by the NC repeater. Hence, we observe:

Observation 3: For network-controlled repeaters, the beam indication of the forwarding beams is enough to be used as side control information of beamforming. The measurement of the forwarding beams done by the network-controlled repeaters is neither necessary nor possible.

    - Beam indication for F beams at gNB side

As shown in Example 1, the information of C beams is already available when NC repeaters get ready to receive side control information. The beam indication of F beams at gNB side can directly reuse the beam indication of C beams. F beams at gNB side and C beams have close relation. F-beams at gNB side can be the same as C-beams, if they are produced by the same antenna panel. Or F-beams at gNB side and C beams are relevant if they are produced by different antenna panels. In the latter case, F-beams at gNB side and C-beams are physically co-located and signal through them are in-band.

Observation 4: For network-controlled repeaters, the beam indication of the communication beams can be directly reused as the beam indication of the forwarding beams at gNB side, because:

- The forwarding beams at gNB side can be the same as the communication beams, if they are produced by the same antenna panel.

- The forwarding beams at gNB side and the communication beams are relevant if they are produced by different antenna panels. The forwarding beams at gNB side and the communication beams are physically co-located and signal through them are in-band.

Proposal 2: For network-controlled repeaters, the beam indication of the communication beams can be reused directly for the forwarding beams at gNB side.

- No new side control information of the beam indication of the forwarding beams at gNB side is needed.

    - Beam indication for F beams at UE side

Different from F beams at gNB side, the beam indication of F beams at UE side needs specific side control information.

A new design is required for the indication of F beams at UE side. The existing TCI-based beam indication for UEs does not work for F beams. The existing beam indication for UEs is based on QCL relation between the target signal an

d a reference signal. However, for NC repeaters, there is neither target signal nor reference signal of F beams. NC repeaters cannot differentiate signal through F beams. Hence, defining/configuring TCI states for F beams at UE side is not feasible.

Observation 5: A new design is required for the indication of the forwarding beams at UE side. The existing TCI-based beam indication for UEs does not work for the forwarding beams.

The main standardization aspects relevant to the indication of F beams at UE side may be as following,

Observation 6: The possible standardization aspects for the beam indication of the forwarding beams at UE side are:

- How the gNB indicates a certain forwarding beam at UE side to NC repeaters?

- How to design a new signaling? Possibly the following factors need to be considered:

- The gNB controls network-controlled repeaters to do beam calibration with UEs.

- Network-controlled repeaters forward SSB signal/Msg2/Msg4 from the gNB to UEs and Msg1/Msg3 from UEs to the gNB.

Regarding the first factor in Observation 6, we prefer the most straight forward way, i.e., to give each F-beam at UE side an index.

Proposal 3: To support the beam indication of the forwarding beams at UE side of network-controlled repeaters, an index is defined for each forwarding beam at UE side and used to identify the beams.

- This is the most straight forward way and can save STD efforts.

  - Information of timing and information of UL-DL TDD configuration

The reason we consider information of timing and information of UL-DL TDD configuration together is because they are highly dependent. Reliable timing is the premise to apply a UL-DL TDD configuration.

In our understanding, there is no need to introduce new signaling for the support of timing and TDD config information for NC repeaters. The existing signaling of TDD config and timing advance for UEs can be reused if necessary.

Now the biggest issue may be the accuracy requirement of timing and TDD config is unclear, which is the key factor that which signaling should be reused. For example, if the rough timing obtained in Msg 2 and the TDD config obtained in SIB is enough, the support of timing and TDD config will probably have no STD impact as shown in Example 1. This is also what RF repeaters can do according to the basic assumptions made in Rel-17 RAN4 discussion. If the flexible symbols of the TDD config in SIB need to further clarify as UL or DL symbols, the specific RRC-based TDD config can be reused for NC repeaters. If NC repeaters are expected to be able to work with dynamic TDD, the NC repeater should be able to receive DCI format 2_0. More details are summarized in Table 2.

Observation 7:

- The accuracy requirement for side control information of timing and UL DL TDD configuration should be discussed and clarified first.

- No new signaling is required for the support of timing and UL DL TDD configuration. The existing indication signaling of timing advance and UL DL configuration for UEs can be reused for network repeaters. Which signaling should be reused depends on the conclusion of the discussion on the accuracy requirement.

Proposal 4: RAN1 discusses and clarifies the requirement of timing and UL DL TDD configuration for network-controlled repeaters.

Table 2 Potential STD impacts of the side control information of timing and UL-DL configuration.

|  | Requirement for NC repeaters | Potential STD impacts |
| --- | --- | --- |
| Timing obtained in Msg2 and TDD config in SIB are enough. | - No more than RF repeaters. | - Almost no. |
| Flexible symbols in SIB1 TDD config need to further clarify as UL or DL. | - NC repeaters have MAC/RRC layers. The existing signaling of timing advance and specific TDD config for UEs can be reused as side control information. - The requirement for the switching time of UL/DL forwarding of NC repeaters will be a little higher than RF repeaters. | - Some STD effort may be required for RAN4. The discussion on the potential enhancement of forwarding signal may be necessary, such as the UL/DL switching time. |
| Dynamic TDD is expected for Rel-18 NC repeaters | - NC repeaters have MAC/RRC layers. The existing signaling of timing advance and dynamic TDD config for UEs can be reused as side control information. - The requirement for the switching time of UL/DL forwarding of NC repeaters will be a little higher than RF repeaters. |  |

  - ON-OFF information

| (A) An RF repeater is always ON. (RF repeaters cannot know anything about the realistic signal transmission between the gNB and the UE.) | (B) The default behaviour of network-controlled repeaters can be OFF when no signal needs to be forwarded. (NC repeaters follow the beam indications from the gNB. No signal, no indication. When no beam indication, NC repeaters don’t forward anything.) |
| --- | --- |

Figure 4 RF repeaters are always on. NC repeaters can be OFF when no forwarding beam is indicated.

As shown in Figure 4 (A), RF repeaters have no other choice but always-on even when no signal needs to be forwarded. This is because RF repeaters have no timely assistance information and, hence, don’t know the realistic situation of the signal transmission between the gNB and the UEs. Besides, the beams/antenna panels of an RF repeater are fixed. Whenever the RF is turned ON, the fixed beams are utilized.

On the contrary, in a sense, an NC repeater cannot be always ON. F beams of the NC repeater are controlled by the gNB. The NC repeater follows beam indications of the gNB to activate F beams at both gNB side and UE side. A reasonable assumption is if signal forwarding in a certain slot/symbol is unnecessary the gNB does not indicate any F beam to the NC repeater. Accordingly, the NC repeater does not forward any signal in the slot/symbol as shown in Figure 4 (B). The NC repeater is kind of OFF in this way and dynamic ON/OFF may not be necessary for it.

Observation 8: When signal forwarding is not required for a network-controlled repeater, the gNB will not indicate any forwarding beams to the repeater. The network-controlled repeater is kind of OFF at that moment.

Proposal 5: It should be concluded that network-controlled repeaters do not forward any signal in a certain time slot/symbol if the gNB does not indicate a forwarding beam to the repeater for that time slot/symbol.

- No dynamic ON/OFF is needed for Rel-18 network-controlled repeaters.

Besides the natural OFF state of NC repeaters as discussed above, some ON/OFF mechanism like DRX of UEs may also be considered to improve the power efficiency of NC repeaters when necessary. Regarding to the DRX-like ON/OFF mechanism, the standardization aspects may be:

- Aspect 1: Whether an alignment between its F link and C link is necessary when an NC repeater is in an OFF state. For example,

  - Possibility 1: Keep the C link ON when the F link is OFF.

  - Possibility 2: The C link can be ON or OFF, e.g., like a common UE with DRX function, while the F link is OFF.

  - Possibility 3: An OFF state means that the C link and the F are both OFF.

- Aspect 2: The definition of an OFF state of F link, i.e., the off level.

  - Power control information

The side control information of power control can help to manage the interference of an NC repeater to other UEs or repeaters. However, the standardization of power control information may be struggling. Being declared as the second priority, the power control information may have less time to be discussed. It is preferred to minimize the potential standardization effort. And if the power control information is concluded to further discuss in WI, the relevant scope has to be as clear as possible to save time.

In our view, to improve the discussion efficiency, before we consider potential candidate solutions of F link power control, some basic assumptions should be clarified, for example:

- Is the power control of forwarded downlink and forwarded uplink assumed to be separated?

- Is it the total transmission power of forwarded signal or the amplification gain to be configured via the side control information from the gNB?

If any common sense about the above factors can be achieved, the candidate solutions of F link power control will be reduced and the further discussion will be more focused.

Proposal 6: RAN1 clarifies the basic assumptions of the power control for network-controlled repeater forwarding link:

- Is it assumed that the power controls for forwarded downlink and forwarded uplink are separate?

- Is it assumed that the total transmission power of forwarded signal or the amplification gain is configured via the side control information from the gNB?

- Further consideration on signal forwarding for initial access

| (A) SSB pattern in time domain. |  |
| --- | --- |
| (B) DL signal forwarded by the RF repeater to UEs includes SSB #m-1. | (C) DL signal forwarded by the RF repeater to UEs includes SSB #m. |

Figure 5 SSB signal in DL signal forwarded by RF repeaters.

(① The RF repeater forwards all DL signal it receives to UEs. The DL signal may or may not include SSB. ② The fixed beams of the RF repeater are wide. Once forwarding can be received by all UEs served by the RF repeater.)

Figure 6 A network-controlled repeater forwards SSB signal according to the indication of the gNB.

(① The forwarding of NC repeaters is fully controlled by the gNB. NC repeaters doesn’t know the forwarded signal includes SSB, but the gNB does. ② The forwarding beams of NC repeaters are usually narrow. Once forwarding through a narrow beam covers a part of UEs. To let all UEs served by the NC repeater can receive SSB at the same time, the beam of NC repeaters has to be forced wide as shown here. The cost is the coverage of the signal forwarded through the wide beam may be smaller than that of other signal forwarded through the narrow beams.)

As shown in Figure 5, RF repeaters use wide beams. Once forwarding through the wide beams can cover all UEs served by the RF repeater. To RF repeaters, the forwarding of initial access signal is as same as that of other signals.

The forwarding beams of NC repeaters are usually narrow as shown in Figure 3 (B). Once forwarding though a narrow beam covers a part of UEs. To let all UEs served by the NC repeater can receive initial access signal (such as SSB) at the same time, like what an RF repeater does, the beam of NC repeaters must be forced wide as shown in Figure 6. The cost is the coverage of the initial access signal forwarded through the wide beam may be smaller than that of other signals forwarded through the narrow beams due to the energy dispersion of wide beams. From the network deployment point of view, it is somewhat abnormal. Usually, the coverage of SSB should be larger than that of common signal such as PDSCH/PDCCH. The SSB signal is not only for the UEs already served by the gNB, but some UEs to be served also, e.g., to be handed over to the gNB.

Another feasible scheme of forwarding the initial access signal is to adopt narrow beam sweeping, as shown in

Figure 7. This scheme can guarantee the coverage of SSB is not smaller than the coverage of common signal.

| (A) SSB pattern in time domain. |  |
| --- | --- |
| (B) The NC repeater uses beam A to forward SSB #n. | (C) The NC repeater uses beam B to forward SSB #n+1. |
| (D) The NC repeater uses beam C to forward SSB #n+2. | (E) The NC repeater uses beam D to forward SSB #n+3. |

Figure 7 According to indications from the gNB, a network-controlled repeater forwards SSB signal to UEs by sweeping narrow beams.

- Conclusion

In this contribution, we carefully considered the side control information-relevant issues. We have following proposals and observations:

In our understanding, the essential difference between an NC repeater and an RF repeater is

Observation 1: The essential difference between a network-controlled repeater and an RF repeater include:

- Network-controlled repeaters can follow the beam indications from the gNB and dynamically/semi-statically/periodically sweep Tx/Rx beams towards the gNB and Rx/Tx beams towards the UE(s), while RF repeaters cannot.

- Network-controlled repeaters can receive the side control information from the gNB which at least includes beam indications, while RF repeaters cannot.

This item may have two main standardization aspects:

Proposal 1: The following two standardization aspects should be considered and concluded in the study item of NR network-controlled repeaters:

- How does a network-controlled repeater communicate with the gNB? What kinds of signaling can the network-controlled repeater receive from the gNB?

- What kinds of side control information are required for a network-controlled repeater to support its signal forwarding operation via adjustable/controllable beams?

Observation 2: The standardization aspects listed in Proposal 1 are dependent on each other. “What kinds of side control information can be supported for a network-controlled repeater” highly depends on “How the network-controlled repeater communicates with the gNB”, and vice versa.

Regarding to the side control information for beamforming, we have following observations and proposals:

Observation 3: For network-controlled repeaters, the beam indication of the forwarding beams is enough to be used as side control information of beamforming. The measurement of the forwarding beams done by the network-controlled repeaters is neither necessary nor possible.

Proposal 2: For network-controlled repeaters, the beam indication of the communication beams can be reused directly for the forwarding beams at gNB side.

- No new side control information of the beam indication of the forwarding beams at gNB side is needed.

Proposal 3: To support the beam indication of the forwarding beams at UE side of network-controlled repeaters, an index is defined for each forwarding beam at UE side and used to identify the beams.

- This is the most straight forward way and can save STD efforts.

Regarding to the side control information for timing and UL DL TDD configuration, we have following proposal:

Proposal 4: RAN1 discusses and clarifies the requirement of timing and UL DL TDD configuration for network-controlled repeaters.

Regarding to the side control information of ON/OFF, we have following observations and proposals:

Observation 8: When signal forwarding is not required for a network-controlled repeater, the gNB will not indicate any forwarding beams to the repeater. The network-controlled repeater is kind of OFF at that moment.

Proposal 5: It should be concluded that network-controlled repeaters do not forward any signal in a certain time slot/symbol if the gNB does not indicate a forwarding beam to the repeater for that time slot/symbol.

- No dynamic ON/OFF is needed for Rel-18 network-controlled repeaters.

Regarding to the side control information of power saving, we have following proposal:

Proposal 6: RAN1 clarifies the basic assumptions of the power control for network-controlled repeater forwarding link:

- Is it assumed that the power controls for forwarded downlink and forwarded uplink are separate?

- Is it assumed that the total transmission power of forwarded signal or the amplification gain is configured via the side control information from the gNB?

References

- RP-213700, “New SI: Study on NR Network-controlled Repeaters,” ZTE Corporation, RAN#94-e, e-Meeting, Dec. 6 - 17, 2021.

- R1-2205086, “NR network-controlled repeaters receive signaling of side control information”, Fujitsu, RAN1#109-e, e-Meeting, May 9 – 20, 2022.

Appendix

Figure A1 Comparison of SINR distribution of ‘Macro cells only’ and ‘Macro cells with network-controlled repeaters’.

(The SINR distribution of UEs can be visibly improved with the assistance of network-controlled repeaters, especially in low SINR region. At the point of 30%, the gain of network-controlled repeaters is about 8dB. At the point of 50%, the gain is about 3.5dB. At the point of 90%, the gain is about 2dB.)

Table A1 Simulation assumptions.

|  | Simulation assumptions |
| --- | --- |
| Layout | Uma (5GCM) / ISD=500m |
| Carrier frequency | 30GHz |
| Channel model | Uma (Macro – NC repeater/Macro – UE) Uma (NC repeater – UE) |
| Tx power | Macro: 30dBm NC repeater: 23dBm |
| Bandwidth | 100MHz |
| Antenna configuration | Macro : (M, N, P, Mg, Ng) = (8, 16, 2, 1, 1) (dH, dV) = (0.5, 0.5)λ NC repeater : (M, N, P, Mg, Ng) = (4, 16, 2, 1, 1) (dH, dV) = (0.5, 0.5)λ User : (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1) (dH, dV) = (0.5, 0.5)λ omnidirectional antenna |
| Antenna Height | Macro: 25m Repeater: 10m |
| Noise figure | Repeater: 7dB User :10dB |
| UE distribution | Outdoor 100% |
| Repeater distribution | 3 NC repeaters per Macro with a uniform distribution |
| Traffic model | Full buffer |