3GPP TSG RAN WG1 #109-e	R1-2205088

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.10.1

Source:		Fujitsu

Title:	Consideration on multi-cell PUSCH/PDSCH scheduling with a single DCI

Document for:	Discussion/Decision

- Introduction

One of the objectives of CA enhancement in Rel-18 is to specify a solution for multi-cell PUSCH/PDSCH scheduling with a single DCI. And the following aspects are identified in WID.

- Identify the maximum number of cells that can be scheduled simultaneously - Consider both intra-band and inter-band CA operation - Consider both FR1 and FR2 - The single DCI shall be optimized for 3 or more cells for the multi-cell PUSCH/PDSCH scheduling

According to the scope, this contribution is to share our views on how to support multi-cell PUSCH/PDSCH scheduling with a single DCI.

- Discussion

2.1 DCI format and fields

A basic question is, which DCI format is used to support multi-cell PUSCH/PDSCH scheduling with a single DCI. Since it is an enhancement, it is straight forward that non-fallback DCI format should be used. The non-fallback DCI could be DCI format 0_1/1_1, DCI format 0_2/1_2, or a new DCI format.

Compared with single-cell PUSCH/PDSCH scheduling with a single DCI, multi-cell PUSCH/PDSCH scheduling may require more DCI payload. If a legacy DCI format (e.g., DCI format 0_1/1_1) is reused, when single-cell PUSCH/PDSCH scheduling is supported with the same DCI format, the DCI payload for single-cell PUSCH/PDSCH scheduling would also be high. From this perspective, it may be preferred to use a new DCI format. However, due to DCI size budget, using a new DCI format may need extra standardization effort e.g., to align the DCI size of the new DCI format to a legacy DCI format. Considering the limited TUs for this WI, it is slightly preferred to reuse legacy DCI format.

Proposal 1: To support multi-cell PUSCH/PDSCH scheduling with a single DCI, legacy non-fallback DCI formats (DCI format 0_1/1_1, DCI format 0_2/1_2) should be used.

As aforementioned, multi-cell PUSCH/PDSCH scheduling may require more DCI payload. This comes from the necessity of introducing additional fields or extending some fields for some cases. For example, in case of inter-band operation, separate indication of FDRA, TDRA, rate matching indicator etc. may be necessary.

To support multi-cell scheduling, for each DCI field, RAN1 needs to discuss whether/how to share the field among cells or how to introduce additional fields or extend the field. In Rel-16/17, some new configurable fields are introduced in non-fallback DCI format. For discussion on DCI fields to support basic function of multi-cell PUSCH/PDSCH scheduling, we suggest taking DCI format 0_1/1_1 in Rel-15 or DCI format 0_2/1_2 in Rel-16 as the starting point for saving time. Additionally, DCI format 0_2/1_2 in Rel-16 is slightly preferred since its payload is more flexible.

Proposal 2: For discussion on DCI fields to support basic function of multi-cell PUSCH/PDSCH scheduling with a single DCI, take DCI format 0_1/1_1 in Rel-15 or DCI format 0_2/1_2 in Rel-16 as the starting point.

2.2 Cell bundling

As identified in WID, for multi-cell scheduling, the maximum number of cells that can be simultaneously scheduled by a single DCI is 3 or more. Meanwhile, both intra-band and inter-band CA operation should be considered. Both FR1 and FR2 should be considered.

Accordingly, Figure 1 gives two examples of scheduling cell and scheduled cells with the same scheduling cell for the case of multi-cell PDSCH/PUSCH scheduling. A DCI on the scheduling cell can simultaneously schedule two or more cells out of the scheduling cell and scheduled cells. Note that frequency band A and B are in same FR or different FRs.

| - 2 CCs | - 4 CCs |
| --- | --- |

Figure 1. example of scheduling cell and scheduled cells in multi-cell PDSCH/PUSCH scheduling

The number of cells that needs to be scheduled may vary with data buffer status. Therefore, it is preferred to dynamically indicate which cell(s) is scheduled. For example, a DCI for multi-cell scheduling can indicate its scheduled cell(s).

The following options can be considered to support the indication in DCI.

- Option 1: The DCI includes a single carrier indicator field (CIF). And the CIF can indicate which set of cells is scheduled. For example, a CIF value can correspond to multiple cells. When a DCI indicates the CIF value, the corresponding multiple cells are simultaneously scheduled.

- Option 2: The DCI includes a bitmap, with bits one-to-one mapping to multiple cells. For example, if a bit is “1”, its corresponding cell is scheduled, otherwise not scheduled.

- Option 3: The DCI includes multiple fields for same indication purpose (e.g., FDRA) with one-to-one mapping to multiple cells. Use a specific value of the fields to indicate not scheduling corresponding cell. For example, if all bits are “0” in such a field, the corresponding cell is not scheduled.

Proposal 3: A DCI for multi-cell PUSCH/PDSCH scheduling indicates its scheduled cells. Consider the following 3 options to support the indication.

- Option 1: The DCI includes a single carrier indicator field (CIF). And the CIF can indicate which set of cells is scheduled.

- Option 2: The DCI includes a bitmap, with bits one-to-one mapping to multiple cells.

- Option 3: When the DCI includes multiple fields for same indication purpose (e.g., FDRA) with one-to-one mapping to multiple cells, use a specific value of the fields to indicate not scheduling corresponding cell.

2.3 PDCCH monitoring

In the legacy, for a USS associated with a CORESET, the number of PDCCH candidate per aggregation level (AL) is configured by search space configuration per serving cell, and the formula below is used for determining CCEs for a PDCCH candidate. When cross-carrier scheduling (CCS) is enabled, nCI denotes the CIF value corresponding to a serving cell. As a result, for each serving cell, a corresponding PDCCH candidate set can be determined.

L⋅Yp,ns,fμ+ms,nCI(L)⋅NCCE,pL⋅Ms,maxL+nCImodNCCE,pL+i

Here is an example. Assume Yp,ns,fμ=0, NCCE,p=32, and number of PDCCH candidate per AL for each serving cell (two serving cells in total) is as shown in Table 1. The CCEs for each PDCCH candidate corresponding to each serving cells are as shown in Figure 2.

Table 1. PDCCH candidate number per AL for each serving cell

| AL | Scheduling cell (nCI=0) | Scheduled cell (nCI=1) |
| --- | --- | --- |
| 4 | 2 | 2 |
| 8 | 2 | 2 |

Figure 2. Example of PDCCH candidate set

For multi-cell PUSCH/PDSCH scheduling, since multiple cells may be scheduled by a same DCI on a PDCCH candidate, the legacy method of PDCCH candidate configuration and determination may not be reusable. It is necessary to discuss how to support PDCCH candidate configuration and determination for this new scheduling mechanism.

Observation 1：For multi-cell PUSCH/PDSCH scheduling with a single DCI, it is necessary to discuss how to support PDCCH candidate configuration and determination.

- Conclusion

According to the discussions above, we have the following observations and proposals for multi-cell PUSCH/PDSCH scheduling with a single DCI.

Proposal 1: To support multi-cell PUSCH/PDSCH scheduling with a single DCI, legacy non-fallback DCI formats (DCI format 0_1/1_1, DCI format 0_2/1_2) should be used.

Proposal 2: For discussion on DCI fields to support basic function of multi-cell PUSCH/PDSCH scheduling with a single DCI, take DCI format 0_1/1_1 in Rel-15 or DCI format 0_2/1_2 in Rel-16 as the starting point.

Proposal 3: A DCI for multi-cell PUSCH/PDSCH scheduling indicates its scheduled cells. Consider the following 3 options to support the indication.

- Option 1: The DCI includes a single carrier indicator field (CIF). And the CIF can indicate which set of cells is scheduled.

- Option 2: The DCI includes a bitmap, with bits one-to-one mapping to multiple cells.

- Option 3: When the DCI includes multiple fields for same indication purpose (e.g., FDRA) with one-to-one mapping to multiple cells, use a specific value of the fields to indicate not scheduling corresponding cell.

Observation 1：For multi-cell PUSCH/PDSCH scheduling with a single DCI, it is necessary to discuss how to support PDCCH candidate configuration and determination.