3GPP TSG RAN WG1 Meeting #109-e                                                  	      R1-2205094

Electronic Meeting, May 9th – 20th, 2022

Agenda item:	9.2.3.2

Source:	Charter Communications

Title:	Discussion on Codebook Enhancement with AI/ML

Document for:	Discussion

# Introduction

A new SI on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1] has been approved at the 3GPP TSG RAN Meeting #94e, with Rel–18 as the targeted potential release; its objectives are outlined below.

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment, model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate

In this contribution, we provide our initial views on potential codebook enhancements facilitated by deep learning (DL).

# Motivation for Codebook Enhancements

For antenna arrays, it is well-known that the direction θ,φ of a propagation path in 3-D space (w.r.t. spherical coordinates) is related to a Nr×Nt spatial channel matrix H by means of the array response vector(s) at∙,∙∈CNt×1 and ar∙,∙∈CNr×1; here Nr, Nt are the number of receive and transmit antennas respectively —in the general case arranged as planar antenna arrays (2-D antenna arrays allow beamforming w.r.t. both azimuth and elevation).  The array response vector on the transmit side is

atθt,φt=gtθt,φtexpj d1T kθt,φt⋯ expj dNtT kθt,φtT

and similarly, at the receive side; di  denote the vector of positions of the antenna elements in the respective arrays, kθ,φ=2 πλr=2 πλsinθcosφ   sinθsinφ    cosφT is the wave vector. The directionality angles will receive an additional subscript l that will refer to a path in the angular domain. gθ,φ is the complex valued pattern of the antenna elements (assumed common among elements).

Single-polarized antennas will be assumed below, for simplicity.

The goal is to establish a connection between the array response vectors and the eigenvectors of the correlation matrix at the transmit side

RH H≐EHHH

### Spatial Channel Matrix Vs. Array Response Vectors

For the case with one receive antenna and L angular paths, the channel matrix is a row vector, denoted hT∈C1×Nt, given by

hT=l=0L-1λl atTθlt,φlt

The paths are modelled above in the angular domain; λl∈C  models the complex gain along the l–th angular path due to interaction with scatterers in the propagation medium; it is possible to link the coefficients of h to multipath delays, but that is not immediately relevant to this discussion.

N.B.: it is a classic result [3] that, regardless of orthogonality assumptions, the optimum beamforming weights are proportional to

woT=l=0L-1λl* atHθlt,φlt,

which highlights the fact that the array response vectors play a key role.

Obviously, the addition of antennas at the receiver side will build a receiver-side (2-D) antenna array and cause the formation of an array response vector arθr,φr  (gain pattern grθr,φr is of course included with the array response vector); it will also ‘build’ new rows into the spatial channel matrix, and thereby model the spatial filtering on the receive side, at arrival angles θlr,φlr, one pair for each of the L angular paths departing from the transmitter. The spatial MIMO matrix H will be, clearly, expressed as

H=l=0L-1λlarθlr,φlr atTθlt,φlt

### Autocorrelation Matrix Vs. Array Response Vectors

In the tradition set by the NR specifications, the gNB and the UE need not know each other’s spatial filtering (precoding vectors).

For the purpose of coping with precoding on the transmitter side, one would like to remove the influence of arθlr,φlr, l=0,…,L-1. This can be achieved by relying on the transmit side correlation matrix since (cf. derivation in [4, Sec. 5.3.3])

Rt=ipt,iatθit,φitatHθit,φit

where pt,i is proportional with arθlr,φlr2 and magnitude of channel coefficient. More importantly, this does not depend on the subcarrier index, which makes the transmit side autocorrelation a large-scale parameter.

By inspection (cf., e.g., expansion of SVD in terms of the singular values and outer products of left and right singular vectors) this can be written as

Rt=AtPtAtH

One can now recognize that if the columns of At are at least approximately orthogonal, the Hermitian becomes an inverse and Rt is an eigenvalue decomposition, whereby good approximations to At can be found efficiently via eigenvalue decomposition. Such approximation does become better and better as the number of antennas increases.

One can assume, based on established state of the art, that the orthogonality approximation holds quite well [4], especially so  for large number of antennas,

N.B.: Without a formal, self-contained proof—but rather an outline—it is worth pointing out that At has size Nt—which, when the transmit antenna array is 2-D (uniform rectangular array), would equal the product of the number of antenna rows respectively columns. It was shown in [5] that (in one aspect) a channel with a dominant angular path (specifically, rank one) can be expressed as a Kronecker product between two ‘DFT-like’ vectors that can decouple azimuth from elevation and can serve as a basis for DFT-based precoding codebooks. A related result from [6] concludes that such a Kronecker product is an eigenvector of the channel correlation matrix.

### Basis for Codebook Enhancement

While DFT codebooks (in the form of Kronecker product of two 1-D DFT codebooks) are attractive and have proven useful, they start to fail when the angular spread of the channel increases; in essence, this is due to the fact that summing many array response vectors (see above MIMO channel expansion) is less well approximated by a DFT vector.

One remedy is to resort to SVD methods.

# Codebook Quantization via ML/DL

The main observation is that the SVD precoding vectors can, as argued above, be obtained as eigenvectors (normalized) of the transmit side correlation matrix—which are directly linked to beam directionality.

Based on the above considerations, it is possible to envision that the gNB either observe the DL channel matrix either at the gNB (during UL transmissions, when channel reciprocity holds) based on, e.g., CSI-RS, or has it fed back by the UE when it does not (e.g., FDD). In either case, the transmit side correlation matrix can be computed by the gNB, or it can be fed back from the UE (via a digital feedback channel).

The gNB builds, maintains, and updates over some recent past, datasets consisting of derived eigenvectors of Rt, collected from many UEs. These can be arranged in a 2-D, image–like shape, such as mimicking the shape of the URA. Finally, multiple ML algorithms can be employed to perform very efficient vector quantization directly on the collected eigenvectors of  Rt—such as practiced in image processing, where each element of the eigenvectors of Rt (its value) can be viewed as a pixel. One such method is described in [2]; other Ml/DL can be employed, too, perhaps more efficiently. Finally, the updated codebook can be periodically exchanged with the UE (e.g., in FDD scenarios); see a simplified, high-level illustration in Figure 1.

Figure 1 High-level diagram of gNB codebook update at gNB using ML with UE feedback (a case when channel reciprocity does not hold is illustrated)

Observation 1: DFT based codebooks break down when the angular spread increases (correlation decreases); it is possible to train a ML/DL network in order to derive and update a vector-quantized codebook for beam management on the gNB side, using ML/DL.

Proposal 1: Consider the option to enhance beam management with a dynamic vector-quantized codebook based on SVD and ML, and have it exchanged with the UE using appropriate interaction mechanisms between gNB and UE(s).

# Conclusion

In this contribution, we discuss the option to replace DFT codebooks with codebooks obtained on the gNB side via Deep Learning.

Observation 1: DFT based codebooks break down when the angular spread increases (correlation decreases); it is possible to train a ML/DL network in order to derive and update a vector-quantized codebook for beam management on the gNB side, using ML/DL.

Proposal 1: Consider the option to enhance beam management with a dynamic vector-quantized codebook based on SVD and ML, and have it exchanged with the UE using appropriate interaction mechanisms between gNB and UE(s).

# References

RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

R. Lippmann, “An introduction to computing with neural nets,” IEEE ASSP Magazine, vol. 4, Apr. 1 1987, pp. 4-22

R. A. Monzingo and T. W. Miller, “Introduction to Adaptive Arrays,” Bagley, G., Ed.; John Wiley, Chichester 1980. The Aeronautical Journal (1981), 85(847), 349-349. doi:10.1017/S0001924000030086

H. Asplund, D. Astely, P. von Butovitsch, T. Chapman, M. Frenne, F. Ghasemzadeh, M. Hagström, B. Hogan, G. Jöngren, J. Karlsson, F. Kronestedt, E. Larsson, Advanced Antenna Systems for 5G Network Deployments, Academic Press, 2020

J. Choi, K. Lee, D. J. Love, T. Kim and R. W. Heath, "Advanced Limited Feedback Designs for FD-MIMO Using Uniform Planar Arrays," 2015 IEEE Global Communications Conference (GLOBECOM), 2015, pp. 1-6, doi: 10.1109/GLOCOM.2015.7417281.

R1-150381, Samsung, “Discussions on FD-MIMO Codebook Enhancements” 3GPP TSG RAN1#80

The terminology is potentially confusing; transmit-side correlation is not meant to suggest reciprocity of the channel—rather, it is one of the two ways in which one can form a Gram matrix from H, namely of size Nt×Nt.