3GPP TSG RAN WG1 #109-e			R1-2205097

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.5

Source: 	Moderator (CATT)

Title: 	Moderator Summary for preparation phase on maintenance of Rel-17 WI on NR positioning enhancements

Document for:	Discussion and Decision

## Introduction

The issues in contributions submitted to RAN1#109e are summarized in the tables of sections 2 and 3. An initial assessment on each of the maintenance issues is provided based on the following classification:

- High priority (H): high-priority item (essential, pending issues, broken spec components) and proposed editorial changes that either enhance the clarity of the specs or correct mistakes

- Non-essential (N): all other purposes such as spec optimization and low priority issues

- Editorial (E): editorial issues that will be handled as editorial CRs (to be communicated to the editors/chairs)

## Issues for agenda item “8.5.1 Accuracy improvements”

Table 1 - Accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays

| Issue# | Issue | References | FL initial assessment | Company inputs (if any) |
| --- | --- | --- | --- | --- |
| 1-1 | Feedback to RAN4 LS R1-2203024 - UE/TRP Tx TEG framework RAN4 LS R1-2203024 provides UE/TRP Rx/RxTx TEG framework. In [1], HW proposes to define the framework for Tx TEG reporting, following the similar way as UE/TRP Rx/RxTx TEG framework. - The applicability of reported UE Rx TEG within the measurement report RAN4 LS indicates “The applicability of reported UE Rx TEG is limited to the measurements contained within the measurement report.” Given that there can be multiple measurement instances within a measurement report, there is a need to clarify whether RAN4’s agreement applies for the case when there are multiple measurement instances FL comments: Since RAN4 has defined the framework for Rx TEG/RxTx TEG, we may expect RAN4 will have further discussion on the framework for Tx TEG. RAN1 may also send a request to RAN4 for Tx TEG framework. For the applicability of reported UE Rx/RxTx TEGs, it seems there is a need to ask RAN4 on whether it is valid for all measurement instances within the measurement report, or only within one measurement instance. | R1-2203099[1] R1-2204985[24] | H | [Companies will fill input their views here] [CATT]: H. RAN1 need further discuss the LS from RAN4 and send a request to RAN4 for Tx TEG framework. OPPO: For the first issue, we can think RAN4 will continue to study it and RAN1 doesn’t need to send any request. For the second issue, we are ok to check with RAN4 QC: Agree with FL’s initial assessment ZTE: It is better to discuss this issue in RAN1. In our view, TEG is only valid in one measurement instance. Ericsson: ok with FL’s assessment. FL Comments: According to the Prep Phase discussion in AI 5, we will have separated email thread under AI 8.5.1 to discuss the response to RAN4’s LS. FL Final assessment: Further discussion under the email thread for discussing the response to RAN4 LS R1-2203024. . |
| 1-2 | UE Tx TEG Reporting (TEG changes/updates/Reset) RAN1 has made the agreement that it is up to RAN2 to consider how to deal with the changes of the UE Tx TEG when reporting the UE Tx TEG association. In [1], Huawei proposes RAN1 to further define the reporting principles for UE Tx TEG reporting. In [8], ZTE proposes to use a time span including starting time and ending time of the measurement instance is reported together with the measurement results, where no group delay change is assumed during the time span. In [15], InterDigital proposes UE Tx TEG association is reported if and only if there is a change association information between UE Tx TEG and SRS resource. FL comments: It is important to properly handle the change/updates of the UE Tx TEG association in higher-layer signaling. It is also quite complicated to properly handle the changes/updates and the Tx TEG association. However, RAN1 has made the decision to let RAN2 to handle the change/updates, and at this moment it is unclear whether RAN2 needs RAN1’s help on this issue, since so far RAN2 does not send LS to request RAN1’s inputs. | R1-2203099[1] R1-2203619[8] R1-2204127[15] R1-2204985 [24] | H | [vivo]: We can look at the discussion of RAN2 first, and try to avoid parallel discussions OPPO: RAN1 can wait until any further input/request is received from RAN2. Thus, it should be “N” at this stage. QC: Agree with FL’s initial assessment ZTE: Support discuss this issue in RAN1 to make things clearer. In addition, the discussion should be separate for Multi-RTT and UL-TDOA. Ericsson: Agree with FL’s assessment. FL Comments: Due to the importance of the issue, we will keep the initial assessemt to discuss it, although there may be some duplication with RAN2’ sdiscussion. FL Final assessment: H |
| 1-3 | UE Tx TEG Reporting (multi-RTT and UL-TDOA) For the reporting of UE Tx TEG associations, it could happen that a UE receives the request from LMF directly or from the gNB if UE is configured with both multi-RTT and UL-TDOA. In [2], Nokia proposes allowing UE to respond to a request with an indication that it will report, or has already reported, the information. FL comments: Similar proposal was presented in previous seminars. RAN2 has not completed the signaling design for the UE Tx TEG associations yet. For a UE configured with both multi-RTT and UL-TDOA, we may expect it is up to LMF on how to send the request for UE Tx TEG associations, i.e., via gNB or to UE directly. Thus, it seems not an critical issues to resolve the potential duplication of the request from the specs. | R1-2205095 [2] | N | [vivo]: The similar view as FL OPPO: Agree with FL. QC: Agree with FL’s initial assessment ZTE: Agree with FL comments Ericsson: Agree with FL. FL Final assessment: N |
| 1-4 | UE Tx TEG Reporting (additional information) By the definition of Tx TEG, the Tx timing error differences for the SRS associated to the same Tx TEG is within a given margin. In [2], it points out that the Tx timing error differences for the SRS resources associated in the same Tx TEG can be different although they are within a given margin SRS resources. Thus, it suggest including additional information (e.g., consistency information) to help LMF calculate differential measurement using measurements containing the same timing error. FL comments: The main purpose of the TEG is to group the RS resources with the same error margin. If we want to handle the case that Tx timing error differences for some SRS resources may be significantly different from the Tx timing error differences of other SRS resources, then it would be better to have different Tx TEGs. Thus, the motivation/benefits of including additional information for the SRS resources in the same Tx TEG is unclear to me. | R1- 2205095 [2] | N | [vivo]: The similar view as FL, we also think a Tx TEG should have the same error margin in an MR OPPO: Agree with FL. QC: Agree with FL’s initial assessment Ericsson: Agree with FL’s view. FL Final assessment: N |
| 1-5 | SRS port index for TEG In LS R1-2203040, RAN3 asks RAN to feedback “if information on the SRS port index needs to be signalled to LMF when SRS resource for MIMO is used.” The response to RAN3’s question was discussed in [4] and [11]. FL comments: RAN1 needs to provide the response back to RAN3 in this meeting. It is unclear at this moment on whether the issue will be discussed under AI 8.1 or in a separate email thread. | R1-2203436 [4] R1-2203864[11] | H | [vivo]:H [CATT]: H. Reply LS from RAN1 is needed. OPPO: Discussion is needed as RAN1 needs to reply RAN3 LS QC: Agree with FL’s initial assessment Ericsson: Ok with FL’s assessment. FL Comments: According to the Prep Phase discussion in AI 5, we will have separated email thread under AI 8.5 to discuss the response to RAN3’s LS. FL Final assessment: Further discussion under the email thread for discussing the response to RAN3 LS R1-2203040. |
| 1-6 | Error margins for Rx/RxTx TEGs RAN4 LS R1-2203024 provides UE/TRP Rx/RxTx TEG framework, which supports UE/TRP to provide the error margins for Rx/RxTx TEGs to the LMF. In [4], CATT suggests adding corresponding RRC parameters according to RAN4’s LS. In [6], vivo proposes adding UE capability of timing error margin for Rx TEG and RxTx TEG as per UE. In [8], ZTE proposes sending LS to RAN4 for the clarification on whether UE provides the margins trough UE capability signaling or other signaling. FL comments: There is a need for RAN1 to decide whether to introduce additional RRC parameters, or UE capability to support UE/TRP to provide its Rx/RxTx TEG margins to the LMF. | R1-2203436 [4] R1-2203515[6] R1-2203619[8] | H | [vivo]:H [CATT]: H. OPPO: RAN4 made the corresponding agreements and they will complete the design by introducing new signaling if necessary. Thus, no RAN1 work is needed so far. “H” -> “N” QC: Agree with FL’s initial assessment Ericsson: Ok with FL’s assessment. FL Comments: To OPPO’s comment, in FL’s understanding, it is RAN1’s responsibility to decide whether to introduce UE capability or based on RRC signaling. FL Final assessment: H |
| 1-7 | Reporting of Tx TEG association with all SRS resources RAN1 has made the agreement that an UE may report any one of the following with UE Rx-Tx time difference measurements: {RxTx TEG ID} {RxTx TEG ID, Tx TEG ID} {Rx TEG ID, Tx TEG ID} If a Tx TEG ID is reported with a UE Rx-Tx time difference measurement, the UE should also report the association of the Tx TEG ID to the UL SRS resource(s). With the concern that gNB may not, or may not measure the SRS resources whose associated Tx TEG is reported by the UE, it is proposed that “for each Rx-Tx time difference measurement for a PRS resource, the UE may report one or more pairs of {RxTx TEG, Tx TEG}” in [6]. FL comments: The concern mentioned in [6] can be resolved if UE reports all Tx TEG association. The similar proposal was discussed in the previous meetings w/o conclusion. It is unclear on whether we could reach a consensus if we further discuss it in this meeting. | R1-2203515[6] | N | [vivo]: 1-7, 1-12, and 1-13 are used to solve the misunderstanding /updating Tx TEG for SRS for Multi-RTT positioning, and they are contained in final proposal 2.4 in RAN1#108-e. So, we hope the following main bullet of final proposal 2.4 in RAN1#108-e can be considered, or the previous proposal 2.4 can be a start point. - If a Tx TEG ID is reported with any at least one UE Rx-Tx time difference measurement, the UE should also report the association of the Tx TEG ID to the UL SRS resource(s) that have already been transmitted and are associated with the Tx TEG ID. OPPO: Agree with FL. Moreover, UE is unlikely to use different pairs of {RxTx TEG, Tx TEG} for the same measurement result. QC: Agree with FL’s initial assessment FL Comments: To vivo’s comment, based on the discussion in previous meeting, it is unclear we could reach a consensus if we further discuss it in this meeting FL Final assessment: N |
| 1-8 | Multiple reference signals to determine the same Rx timing Multiple DL PRS resources associated can be used to determine the Rx timing of a DL subframe from a TP. In [8], ZTE proposes to add the clarification that in this case the multiple DL PRS resources are associated with the same Rx TEG ID. FL comments: Rx TEG ID is associated with the measurement, but not the reference signals. Assuming we have the common understanding that one measurement cannot be associated with more than one Rx TEG, and multiple DL PRS resources can be used to obtain the Rx timing for the measurement, it seems there is no need to say multiple DL PRS resources are associated to the same Rx TEG. | R1-2203619[8] | N | [vivo]: The similar view as FL OPPO: Agree with FL QC: Agree with FL’s initial assessment Ericsson: Agree with FL’s assemssment FL Final assessment: N |
| 1-9 | TP for terminologies of “ueRxTEG” and “ueRxTEG In [13], OPPO proposes to change “ueRxTEG” and “ueRxTEG” to “UE Rx TEG” and “UE RxTx TEG” for the consistence in the specification. FL comments: The TP seems editorial, which can be discussed directly when updating the specs. | R1-2203960 [13] | E | OPPO: The group can make agreement quickly and recommend it to the editor QC: Agree with FL’s initial assessment Ericssson: Ok. FL Final assessment: E |
| 1-10 | UE Tx TEG sweeping The support of UE Tx TEG sweeping as well as the potential benefits and drawbacks have been discussed intensively in the previous meeting w/o conclusion. In [17], CMCC proposes to support UE Tx TEG sweeping as an optional UE capability. FL comments: The issue was discussed very intensively in the previous meetings w/o conclusion. It is unclear on whether we could reach a consensus if we further discuss it in this meeting. | R1-2204275[17] | N | [vivo]: The similar view as FL [CATT]: N. It seems that no consensus can be achieved on this issue. OPPO: Agree with FL QC: Agree with FL’s initial assessment FL Final assessment: N |
| 1-11 | UE Tx TEG reporting for Multi-RTT Periodic UE Tx TEG reporting for Multi-RTT has been discussed in previous RAN1 meetings w/o conclusion. In [19], Docomo proposes supporting the Final Proposal 2.4 as discussed in the previous meeting. FL comments: The issue was discussed very intensively in the previous meetings w/o conclusion. It is unclear on whether we could reach a consensus if we further discuss it in this meeting. | R1-2204346[19] | N | QC: Agree with FL’s initial assessment Ericsson: Agree with FL’s assessment. FL Final assessment: N |
| 1-12 | Timestamp of the TxTEGSRS association In [24], Qualcomm proposes the Timestamp of the TxTEG<->SRS association is needed in the LPP measurement report for M-RTT. FL comments: The proposal is related to on-going discussion in RAN2. RAN1 may wait for RAN2’s conclusion to see if RAN2 needs RAAN1’s input to resolve the issue. | R1-2204985[24] | N | [vivo]: H, and the same view as 1-7 OPPO: Agree with FL. RAN4 discussion on the validation of Tx TEG may also impact this issue. QC: We think it is related to 1-2 and, if RAN1 indeed works more on the TxTEG<->SRS association and “the changes of the UE Tx TEG when reporting the UE Tx TEG association“, then this 1-12 is very relevant. The main way that a UE can provide information about changes in the TxTEG<->SRS is through timestamps. So, i think, the 1-12, should be included in the 1-2 discussion. FL Comments: To vivo’s comment, it is unclear to me how this it related to 1-7. To QC’s comment, yes, the timestamp is needed to indicate when TEG changes. My initial consideration is that RAN2 is working on the issue, and sofar RAN2 does not seek the help from RAN1 to resolve any issue. If there is any additional discussion related to timestamp is needed as the result on discussion of 1-2, we can have further discussion in Issue 1-2. FL Final assessment: If additional discussion on time stamp is needed as a result on discussion of 1-2, we can have further discussion in 1-2. |
| 1-13 | Maximum number of Tx TEG SRS Associations in a M-RTT measurement report In [24], Qualcomm proposes to support a maximum of 1024 Tx TEG SRS associations (up to 8 Tx TEG per band * Up to 4 bands * Up to 32 timestamps) for the Tx TEG SRS Association inside an M-RTT report. FL comments: The maximum number of UE-TxTEG is currently defined as 8 per UE. Further discussion may be needed to change it “per band”. In addition, the maximum of Tx TEG is also related to the issue (see Issue 1-15) on whether the SRS associations is unchanged in different time instances within one measurement report. | R1-2204985 [24] | H | [vivo]: H, and the same view as 1-7 OPPO: Support to discuss it QC: Agree with FL’s initial assessment FL Final assessment: H |
| 1-14 | TP for capturing RAN1’s agreement on UE Tx TEG reporting (TS 38.214) In [24], Qualcomm proposes to capture RAN1’s agreement “Based on a configured periodicity, a UE may report the UE Tx TEG association for the SRS resources for positioning that have already been transmitted during the configured period” in 38.214. FL comments: It seems the RAN1’s agreement is not fully captured into 38.214. Thus, suggest discuss how to capture the agreement in TS 38.214. | R1-2204985 [24] | H | [vivo]: Ok OPPO: Support to discuss it QC: Agree with FL’s initial assessment FL Final assessment: H |
| 1-15 | TP for capturing RAN1’s agreement on multiple measurement instances In [24], Qualcomm proposes to capture RAN1’s agreement “For each indicated positioning method in a measurement report, multiple measurement instances are associated with the indicated positioning method” in 38.214. FL comments: It seems the RAN1’s agreement is not fully captured into 38.214. Thus, suggest discuss how to capture the agreement in TS 38.214. | R1-2204985 [24] | H | [vivo]: Ok OPPO: Support to discuss it QC: Agree with FL’s initial assessment FL Final assessment: H |
| 1-16 | Maximum number of measurement instances in a report RAN1 has agreed that for each indicated positioning method in a measurement report, multiple measurement instances are associated with the indicated positioning method. In [24], Qualcomm proposes to support up to 128 measurement instances in a single measurement report. FL comments: RAN1 needs to define the Maximum number of measurement instances in a report, and include the number in the RRC parameter list. | R1-2204985 [24] | H | [vivo]: Ok QC: Agree with FL’s initial assessment FL Final assessment: H |

Table 2 - Accuracy improvements for UL-AoA positioning solutions

| Issue# | Issue | References | FL initial assessment | Company inputs (if any) |
| --- | --- | --- | --- | --- |
| 2-1 | [description of the issue] |  |  | [Companies will fill input their views here] |
|  | Proposal 1 Endorse the following definition for SRS RSRPP UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the power of the SRS channel response at the ith path delay, where - The SRS channel response is obtained by a time-domain transform of the received signal at the resource elements that carry the received UL SRS signal configured for the measurement, - UL SRS-RSRPP for 1st path delay is the power contribution corresponding to the first detected path in time. | [24]R1-2204943 | [N] We already have a definition captured in 215. | [vivo]:N OPPO: agree with the assessment of FL. FL Final assessment: we can discuss PRS RSRP wording and mirror the conclusion for SRS, if needed. |
| 2-2 | An LS was sent to RAN1 regarding the issue of Rx diversity signalling in NRPPa. RAN1 to feedback if information on the Rx Diversity options needs to be signalled to LMF and to comment on the UL-SRS-RSRPP value range, if needed? Proposal 1: the options on the used Rx branches are signaled to LMF. | R1-2203864 | [H] Can be discussed as part of LS discussion | [vivo]:H OPPO: Agree with the assessment of FL and this shall be discussed in LS discussion. QC: Agree with FL’s initial assessment FL Final assessment: H |

Table 3 - Accuracy improvements for DL-AoD positioning solutions

| Issue# |  | Issue | References | FL initial assessment | Company inputs (if any) |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3-1 |  | [2]Proposal 3: The UE can be requested to report PRS RSRP for specifically indicted PRS resources and previous time-stamp(s), if the UE reported DL PRS RSRPP for PRS resources and the time-stamp(s). [1]Proposal 5: For DL-AoD, introduce a separate DL PRS resource set ID and resource ID for the first RSRPP measurement when the resource is not same as that for the first RSRP measurement. ● The additional RSRPP measurement still takes (0..30) as the reporting range in reference to the first RSRPP measurement. | [2] R1-2203175 [1] R1-2203099 | [H] We can discuss the reporting of RSRP separately (from different resources) from RSRPP as proposed in [1]. Proposal in [2] is ok to discuss but seems an optimization. | [vivo]:H OPPO: Proposal 3 is not essential issue. Ok to discuss Proposal 5. ZTE: We are not sure the benefit of proposal 3 [2]. For the proposal 5 [5], we think it depends on if RAN4 uses double sides differential RSRP report, then there is no issue. But anyway, we are open to discuss the issues. FL Final assessment: H, focus on P5 |  |
| 3-2 |  | TP clarifying reception of the DL PRS-RSRPP in 38.214 Proposal-1: Adopt the following TP for DL-AOD measurement and reporting: ----------------Start of TP for TS38.214--------------------- #### 5.1.6.5 PRS reception procedure …… The UE may be configured to measure and report, subject to UE capability, up to 24 DL PRS-RSRP measurements on different DL PRS resources associated with the same dl-PRS-ID. When the UE reports DL PRS-RSRP measurements from one DL PRS resource set, the UE may indicate which DL PRS-RSRP measurements associated with the same higher layer parameter nr-DL-PRS-RxBeamIndex [17, TS 37.355] have been performed using the same spatial domain filter for reception if for each nr-DL-PRS-RxBeamIndex reported there are at least 2 DL PRS-RSRP measurements associated with it within the DL PRS resource set. When the UE reports multiple DL PRS-RSRP measurements for a DL PRS resource, the multiple DL PRS-RSRP measurements, which can be associated with the same or different higher layer parameter nr-DL-PRS-RxBeamIndex, may have the same or different timestamps. The UE may be configured to measure and report, subject to UE capability, up to 24 DL PRS RSRPP for the first detected path on different DL PRS resources associated with the same dl-PRS-ID. …… ----------------End of TP for TS38.214--------------------- | [4] R1-2203436 | [E] Since the spec already says “measurements” (plural), the TP does not seem to be an essential correction. | [vivo]:N The added description doesn't regulate anything, and we think it is a note that is used for explanation and may not necessarily be added to the specification [CATT]: According to previous RAN1 agreement, for a given PRS resource, multiple DL PRS RSRPs could be reported, which may correspond to same or different Rx Beam index and same or different timestamps. This feature should be specified in TS 38.214 for a clear description on UE reporting. Therefore, we prefer to discuss this TP in this meeting. OPPO: agree with assessment of FL that this is not an essential correction. FL Final assessment: E |  |
| 3-3 |  | On the maximum number of PRS resources in a subset: Proposal 3 - The maximum number of PRS resources in a PRS subset is 8. - The number of resources within a subset transmitted to UE should not be larger than the maximum number of resources that UE reporting | [6]R1-2203515 | [H] The maximum size of the subset was not discussed. | [vivo]:H OPPO: ok to discuss QC: We acknowledge there was no explicit agreement, but we don’t see the complexity problem if this value is 24 (as shown now in 37.355). In either case, we are OK to discuss, but we don’t consider it High priority. maxNumPrioResources-r17 INTEGER ::= 24 – FFS ZTE: Is this for UE capability or LPP signaling from LMF to UE? FL Final assessment: H |  |
| 3-4 |  | Angle rangle of the TRP information: Proposal 4 - Support the following angle range and angle granularity for relative Power/Angle response - [-90, 90] for omnidirectional antenna and [-60, 60] for directional antenna - 0 degree is represented as the boresight of the panel of TRP - The granularity angle can be 1 degree so that the maximum element can be reduced by 100 times | [6]R1-2203515 | [H] | [vivo]:H OPPO: ok to discuss QC: Not High priority. The 37.355 seems complete and more general; the LMF can provide the relative beam responses for any angles needed. ZTE: prefer deprioritizing this issue as the size of angle range report is already configurable based on the current 38.455. FL Final assessment: N |  |
| 3-5 |  | Refererence for additional path DL PRS RSRPP: Proposal 4: Support Alt. 2, i.e. the reference, for calculation of differential RSRPP, for the additional path PRS-RSRPP is the PRS-RSRPP of the first path of the same PRS resource as the additional path PRS-RSRPP. | [9] R1-2203620 | [H] | [vivo]:H OPPO: ok to discuss QC: Support to be classified as High priority, however, we are OK to not support differential reporting if there is no consensus on how to do so. Even if we specify absolute reporting, the specification is not broken. FL Final assessment: H |  |
| 3.6 |  | TP for 38.214 regarding the AOD window: Proposal 3: Adopt the following TP for TS 38.214 for complete specification of method of expected DL-AoD/AoA. TP for TS 38.214: Reason for change: The text specificaton on expected DL-AoD/AoA is not complete Summary of change: Add text to specify the method of calculating range of expected DL-AoD/AoA. Consequences if not approved: Ambiguity in UE behavior.. 5.1.6.5 PRS reception procedure <Unchanged parts are omitted> The UE may request to be provided with either expected DL-AoD/ZoD and uncertainty range(s) of expected DL-AoD/ZoD, or expected DL-AoA/ZoA and uncertainty range(s) of the expected DL-AoA/ZoA. The UE may be provided with expected DL-AoD/ZoD and uncertainty range(s) of the expected DL-AoD/ZoD with granularity of 1 degree. The UE may be provided with expected DL-AoA/ZoA and uncertainty range(s) of the expected DL-AoA/ZoA with granularity of 1 degree. The uncertainty range(s) of the expected DL-AoD/DL-AoA may be configured within [0, 60]. The uncertainty range(s) of expected DL-ZoD/DL-ZoA may be configured within [0, 30]. The UE may calculate the range of expected DL-AoD/DL-ZoD as (expected DL-AoD/ZoD – uncertainty range of expected DL-AoD/ZoD/2, expected DL-AoD/ZoD + uncertainty range of expected DL-AoD/ZoD/2) and may calculate the range of expected DL-AoA/DL-ZoA as (expected DL-AoA/ZoA – uncertainty range of expected DL-AoA/ZoA/2, expected DL-AoA/ZoA + uncertainty range of expected DL-AoA/ZoA/2) <Unchanged parts are omitted>. | [13] R1-2203960 | [H] Ok to discuss | [vivo]: Ok to discuss, but we prefer the related description can refer to ExpectedRSTD and be added to RNA2 specification OPPO: ok to discuss QC: OK to discuss FL Final assessment: H |  |
| 3-7 |  | Requesting reports of PRS subsets measurements separately from the main measurement: Proposal 3: When PRS is associated with PRS subsets, support reporting from the UE to include measurements related to subsets associated with PRS or PRS and its associated subsets, depending on the request from the LMF. | [15] R1-2204127 | [H] In the FL view, we have already agreed to separate reporting Maybe the proposal meant “support requesting” | [vivo]: Ok for request signaling from UE OPPO: not needed. The related issues have been discussed and previous agreement was made. [InterDigital] : Thank you FL for clarifying the proposal. Yes, it shoulld have read “When PRS is associated with PRS subsets, support requesting the UE to include measurements related to subsets associated with PRS or PRS and its associated subsets” From our understanding of the past agreement, the proposal attempts to clarify the report content. This is continuation of the disucssion from R1-2202605, FL’s summary for AoD enhancement. QC: Not essential. Even if the LMF cannot only request the “associated subset”, the additional overhead in the report, is marginal. ZTE: This seems an optimization, prefer deprioritizing it. FL Final assessment: N (given the lack of support and the number of remaining issues) |  |
| 3-8 |  | Proposed correction to the PRS RSRPP definition [24]Proposal 2 Endorse the following definition for PRS RSRPP DL PRS reference signal received path power (DL PRS-RSRPP) is defined as the power of the PRS channel response at the ith path delay, where - The PRS channel response is obtained by a time-domain transform of the resource elements that carry the received DL PRS signal configured for the measurement - DL PRS-RSRPP for 1st path delay is the power contribution corresponding to the first detected path in time. [1]Proposal 6: Adopt the following modification to the RSRPP definition. ● DL PRS reference signal received path power (DL PRS-RSRPP), is defined as the power of the linear average of the channel response at the i-th path delay on the resource elements that carry DL PRS signal configured for the measurement, where DL PRS-RSRPP for the 1st path delay is the power contribution corresponding to the first detected path in time. ● The channel response at the i-th path delay on a resource element corresponds to the response on the resource element of a shifted version of the channel where the i-th path delay is shifted to 0. | [24] R1- 2204943 [1] R1-2203099 | [N] Ok to discuss, but there is already a definition. | [vivo]: N [Huawei, HiSilicon]: H. We think some clarification such as proposal 6 from [1] is needed because the frequency domain average means the time domain response at t=0, not at any delay. [OPPO]: agree with the assessment of FL QC: We think we spent a lot of time for trying to write down with words what should have been a formula in the specification. From our side, we prefer to not really try to change it again, but if there is strong support, we could be open to it. Ericsson: H. we think there is a need to correct the definition now to avoid issues later in eg. RAN4. We agree with the comment from Huawei above that the current definition is missing the delay component. FL Final assessment: H |  |
| 3-9 |  | Reporting of relative time of arrival or path index: In previous meetings we discuss whether to indicate to the LMF that AOD measurements were done based on the same time of arrival. However we did not converge. 2 proposals are brought up in RAN1#109e: [24]Proposal 3 When reporting more than one PRS RSRPP for a TRP, the UE can optionally report the inter-arrival time for the first paths used in a pair of measurement. [1]Proposal 7: For each first path RSRPP measurement of DL-AoD positioning, UE indicates a path index. ● The path index may take a value from {0,1,2,3}. ● First path RSRPP measurements associated with the same path index for a TRP correspond to the same path. ● Determination of whether the first path RSRPP measurements belong to the same path or different paths is up to UE implementation. | [24] R1- 2204943 [1] R1-2203099 | [N] The topic was previously discussed but did not converge | [vivo]: N [Huawei, HiSilicon]: H. We think Proposal 7 from [1] should be discussed, which is also a new proposal as the compromise to resolve the deadlock in the previous meeting. [OPPO]: agree with the assessment of FL QC: Not a correction. Revisit in NR Rel-18 as/if needed. FL Final assessment: N, based on the lack of support. |  |
| 3-10 |  | DL PRS RSRPP and Rx Diversity: Two alternatives are proposed in [1] and [24] [24]Proposal 4 The following is added to the definition of DL PRS RSRPP: For frequency range 1 and 2, if receiver diversity is in use by the UE for DL PRS-RSRPP measurements: - The reported DL PRS-RSRPP value for the first and additional paths shall be provided for the same receiver branch(es) as applied for DL PRS-RSRP measurements, or - The reported DL PRS-RSRPP value for the first path shall not be lower than the corresponding DL PRS-RSRPP for the first path of any of the individual receiver branches and the reported DL PRS-RSRPP for the additional paths shall be provided for the same receiver branch(es) as applied DL PRS-RSRPP for the first path. [1] Proposal 4: Support the following Rx diversity scheme for RSRPP reporting for DL-TDOA and Multi-RTT ● For both frequency range 1 and frequency range 2, if receiver diversity is in use by the UE, the reported RSRPP may be associated with an Rx branch set ID. ● The RSRPP measurements associated with the same Rx branch set ID for a TRP correspond to the same set of Rx branches. | [24] R1- 2204943 [1] R1-2203099 | [H] We have not finished this discussion in the previous meetings. | [vivo]: N, based on the following RAN4 agreement, we think the issue is clear. - Same Rx branches as applied for PRS-RSRP measurement are used for path PRS-RSRP measurement [Huawei, HiSilicon]: H. According to feedback from RAN4, they were only intending to discuss DL-AoD, not DL-TDOA and Multi-RTT. [CATT]: H.Need further discuss. OPPO: ok to discuss QC: Agree with FL’s initial assessment FL Final assessment: H |  |
| 3-11 |  | Priorities of reported adjacent beams Proposal 5 The PRS resources order in the assistance data is used as a priority order for the PRS and PRS subset reports. | [24] R1- 2204943 | [N] The topic was previously discussed but did not converge | [vivo]: N OPPO: agree with assessment of FL QC: We support this feature, but we acknowledge it may not be as essential as other open issues. |  |
| 3-12 |  | Use of the same RX beam for different measurements The discussion on reporting based on the same Rx beam for different PRS measurements did not converge in previous meeting. Two proposals bring back the issue: Proposal 6 Path PRS-RSRP measurements of adjacent DL PRS Resources that the UE reports should be performed using the same Rx-beam. Proposal 1: When the UE reports DL PRS-RSRP measurements from DL PRS resource sets associated with the same positioning frequency layer, the DL PRS-RSRP measurements associated with the same DL PRS Rx beam index have been performed using the same spatial domain filter for reception. | [24] R1- 2204943 [9] R1-2203620 | [N] The topic was previously discussed but did not converge | [vivo]: even though we are supportive of the feature, maybe it is not an essential issue in this phase OPPO: agree with assessment of FL. QC: agree with assessment of FL. ZTE: If possible, we prefer to discuss this issue again, the motivation is to inform LMF the same Rx beam is used for two PRS resources from two sets rather than within one set. With this information being reported to LMF, LMF can determine that whether the two different DL PRS-RSRP measurements associated with the two DL PRS resources are due to different TRP transmission beams or different UE Rx beams when conducting angle calculation. FL Final assessment: N, based on the lack of support. |  |
| 3-13 |  | Measurement of PRS RSTD and PRS RSRPP based on different paths. Observation 1: The current RSRPP and RSTD definitions could lead to the LMF misunderstanding the reported values as the “paths” may not be defined the same way for both measurements. To address this problem, it should be ensured that the UE has the same understanding of paths when reporting to the LMF. As such we make the following proposal Proposal 3: Adopt the following TP to TS 38.214: 5.1.6.5 PRS reception procedure ------------------------------Text omitted-------------------------- The UE may be configured to optionally report a differential DL RSRPP for a PRS resource with reference to [nr-DL-PRS-FirstPathRSRP-Result] and/or a differential DL PRS RSRP with reference to [nr-DL-PRS-RSRP-Result] via higher layer parameter [NR-DL-AoD-AdditionalMeasurementElement]. -------------------------------Start TP------------------------------- If the UE is configured to report both PRS RSTD and PRS RSRPP measurements the same detected paths for both measurements should be used in the reporting. The UE may indicate via higher layer parameter [xxxx] if the same detected paths are not used for the PRS RSTD and PRS RSRPP measurements. ------------------------------End TP----------------------------------- | [27] R1- 2205095 | [H] | [vivo]: N It is unclear to us that any assumption can be made if RSTD measurement and RSRPP measurement form different method, especially considering AoD only support the first path RSRPP. And if RSTD measurement and RSRPP measurement are reported in an NR-DL-TDOA-AdditionalMeasurementElement in TDOA measurement, it should be from the same path based on RAN2 specification. OPPO: ok to discuss. But the second sentence in the proposed TP seems not needed. QC: N. ZTE: We think it is common understanding RSTD is always based on the first path. Hence, no ambiguity issues to be further discussed. FL Final assessment: N based on the lack of support. |  |
| 3-14 |  | Adding a parameter for the max number of requested RSRPPs: Proposal 10: Add a parameter for the number of DL PRS-RSRPP measurements per TRP requested by the LMF to the UE for DL-AoD. | [1] R1-2203099 | [N] | [vivo]: N OPPO: agree with assessment of FL QC: agree with assessment of FL FL Final assessment: N |  |
| 3-15 | TP on RX beam index - Proposal 8 Endorse the following TP to TS 38.214. - Reasons of change: The Rx beam index should consider the RSRPP reporting and the RSRP/RSRPP reporting across multiple resources, while the current restriction on condition when the Rx beam index is allowed to be indicated is too restrictive. - Summary of change: Relax the existing condition on Rx beam index reporting of two RSRP measurements contained within a single set, and add RSRPP as part of the condition to report the Rx beam index. - Consequences if not approved: Rx beam index reporting is not supported for RSRP reporting associated with two PRS resource sets and is not supported for RSRPP-based DL-AoD. ================== Start of TP ================== The UE may be configured to measure and report, subject to UE capability, up to 24 DL PRS-RSRP measurements on different DL PRS resources associated with the same dl-PRS-ID. When the UE reports DL PRS-RSRP measurements from one DL PRS resource set, the UE may indicate which DL PRS-RSRP measurements associated with the same higher layer parameter nr-DL-PRS-RxBeamIndex [17, TS 37.355] have been performed using the same spatial domain filter for reception if for each nr-DL-PRS-RxBeamIndex reported there are at least 2 DL PRS-RSRP measurements associated with it within the DL PRS resource set. The UE may be configured to measure and optionally report via higher layer signaling nr-DL-PRS-FirstPathRSRP-Result, subject to UE capability, up to 24 DL PRS RSRPP for the first detected path on different DL PRS resources associated with the same dl-PRS-ID. The UE may be configured to optionally report a differential DL RSRPP for a PRS resource with reference to [nr-DL-PRS-FirstPathRSRP-Result] and/or a differential DL PRS RSRP with reference to [nr-DL-PRS-RSRP-Result] via higher layer parameter [NR-DL-AoD-AdditionalMeasurementElement]. When the UE reports DL PRS-RSRP/DL PRS-RSRPP measurements, the UE may indicate which DL PRS-RSRP/DL PRS-RSRPP measurements associated with the same higher layer parameter nr-DL-PRS-RxBeamIndex [17, TS 37.355] have been performed using the same spatial domain filter for reception if for each nr-DL-PRS-RxBeamIndex reported there are at least 2 DL PRS-RSRP measurements or at least 2 DL PRS-RSRPP measurements associated with it within the DL PRS resource sets on a positioning frequency layer with the same dl-PRS-ID. ================== End of TP ================== Proposal 9: Send an LS to RAN2 informing that the following change to the field description of nr-DL-PRS-RxBeamIndex is preferred from RAN1 perspective. nr-DL-PRS-RxBeamIndex This field provides an index of the target device receive beam used for DL-PRS measurements. If the value of the receive beam index for two or more DL PRS measurements is the same, it indicates that the target device receive beam for the two or more DL PRS measurements were made with the same RX beam. The field is mandatory present if at least two DL-PRS RSRP measurements or at least two DL-PRS RSRPP measurements from the DL-PRS Resource Sets on a positioning frequency layer of the TRP have been made with the same RX beam by the target device; otherwise it is not present.. | [1] R1-2203099 | [H] |  | [vivo]: H OPPO: ok to discuss QC: ok to discuss ZTE: This issue is related to issue 3-12, we prefer to discuss them together. FL Final assessment:H |

Table 4 - Information reporting from UE and gNB for multipath/NLOS mitigation

| Issue# | Issue | References | FL initial assessment | Company inputs (if any) |
| --- | --- | --- | --- | --- |
| 4-1 | LOS/NLOS indicator component values for UE capability signaling. In [6]: Proposal 5 - For UE’s capability to support reporting LoS/NLoS indicator - Support component 1 candidate value {hard value, both hard value and soft value} In [15]: Proposal 4: A UE capability supporting both soft and hard value reporting for LoS/NLoS indication is not supported | [6] R1-2203515 [15] R1-2204127 | [H]* *unless discussed directly in UE features discussion | [vivo]: H OPPO: ok to discuss [InterDigital] : This can be “H”. We prefer to discuss this topic in 8.5. QC: It can be discussed in UE features. It’s a small proposal that doesn’t need to be discussed in main session. FL Final assessment: H |
| 4-2 | LOS/NLOS Indicator fallback type. In [6]: Proposal 6 - Support to differentiate the type of LOS/NLOS Indicator by the LoS-NLos-Indicator IE, where the hard value type should be supported as a default value | [6] R1-2203515 | [N] Having a default type seems non-essential for the spec to work | [vivo]: N OPPO: not essential issue QC: N FL Final assessment: N |

## Issues for agenda item “8.5.2 Others”

Table 5 - Latency improvements for both DL and DL+UL positioning methods

| Issue# | Issue | References | FL initial assessment | Company inputs (if any) |
| --- | --- | --- | --- | --- |
| 5-1 | PRS-data collision detection timeline | P4/P5, R1-2203176, Nokia/NSB, [3] P5, R1-2203516 vivo, [7] P1, R1-2203786, Xiaomi, [10] P3, R1-2203865, SS, [12] P1, R1-2204128, IDC, [16] P1, R1-2204276, CMCC, [18] P1/P2/P3, R1-2204903, Huawei/HiSi, [21] P3/P4, R1-2204986, Qualcomm, [25] | H | [vivo]: H QC: H FL Final assessment: H |
| 5-2 | Low latency PRS processing capability | P3, R1-2203516, vivo, [7] P2, R1-2203620, ZTE, [9] P6, R1-2203865, SS, [12] P2, R1-2204128, IDC, [16] P4, R1-2204903, Huawei/HiSi, [21] P1, R1-2204986, Qualcomm, [25] | H | [vivo]: H QC: H FL Final assessment: H |
| 5-3 | LMF indication of Rx beam sweeping factor FL: LS related | P1, R1-2203516, vivo, [7] P3, R1-2203620, ZTE, [9] P6, R1-2204986, Qualcomm, [25] | H | [vivo]: H QC: H FL Final assessment: H |
| 5-4 | Rx timing difference condition for measurement of PRS from non-serving cells FL: LS related | P2/P3, R1-2203176, Nokia/NSB, [3] P2, R1-2203516, vivo, [7] P5, R1-2203961, OPPO, [14] | H | [vivo]: H QC: H FL Final assessment: H |
| 5-5 | UL transmission in PPW | P6, R1-2203516, vivo, [7] P4/P5, R1-2203865, SS, [12] | H | [vivo]: H QC: We don’t see a need to clarify something more. The initial WA assumed that all UL channels/signals are transmitted as expected. We don’t consider the spec broken, nor there is a need to avoid doing UL inside a PPW; this is not a MG, and the UE is not required to retune. With regards to priority rules between PRS and UL channels, within a PPW, again, the UE will follow the DL/UL symbols as configured by serving cell, if it happens that a PRS appears within an UL symbol, the UE will not receive PRS; it will be serving cell responsibility to avoid this. Ericsson: We share similar view with QC. This may not be an essential issue. [SS]: H, to QC and E///, it’s not only the collsion with UL symbols, UL transmission can be in flexible symbos, UL transmission could be scheduled. Besides, UL transmission doesn’t equal the UL symbols in resource perspective, as the transmission is applying TA (which could be ms level) and it’s ahead the UL symbols, such collsion/detection line needs to be discussed. FL Final assessment: H |
| 5-6 | More than one positioning frequency layers to process within a PPW FL: LS related. | P5, R1-2204942, Ericsson, [22] P5, R1-2204986, Qualcomm, [25] | H | [vivo]: H FL Final assessment: H |
| 5-7 | TP to clarify the PRS duration calculation in relation to collision with other DL/UL signals | P7, R1-2203865, SS, [12] | H | [vivo]: merged in issue 5-1 QC: We don’t see the need of optimizing this and prefer not to change the definition of PRS duration. A PRS duration is something that should not depend on whether there were collision with other channels. [SS]: H, reply to QC, in RAN4 discussion on the measurement period,i.e., 9.9.1.2 in TS38.133, the discussed PRS are considering the collision with other channel with the priority. And current PRS duration calculation (although with potential) is only mentioning the expected RSTD related settings, we think it’s not enough considering PPW usage. FL Final assessment: H |
| 5-8 | TP to capture the priority states | P4, R1-2204942, Ericsson, [22] | H | [SS]: N. this should be discussed in RAN2 ASN.1 discussion. OPPO: no need to dicuss here Ericsson: Regarding SS/OPPO comments, note that field description in the latest 38.331 CR for priority states is as shown below. Clearly 38.331 does not provide definitions of the different priority states and simply references TS38.214. So, we think this should be captured in TS 38.214. priority Indicates the priority between PDCCH/PDSCH/CSI-RS and PRS as specified in TS38.214 [X]. [SS2]: H or E. thx E/// for pointing that out, we also find that, so we are ok to discuss it. Thx. FL Final assessment: H |
| 5-9 | TP to update “[PRSProcessingWindow]” with “PRS processing window by higher layer parameter [PRSProcessingWindow]” | P2, R1-2203437, CATT, [5] | E | [CATT]: We prefer to discuss this TP to make the specs clear. FL Final assessment: E |
| 5- 10 | Adding “receiving DL signals/channels except SSB” in parallel to “receiving DL PRS” in the PPW | P4, R1-2203516, vivo, [7] | E | FL Final assessment: E |
| 5-11 | PRS processing window activation request by UL MAC CE | P3, R1-2204128, IDC, [16] P2, R1-2204986, Qualcomm, [25] | N | [vivo]:N OPPO: support to discuss QC: H Ericsson: Agree with FL’s assessment. FL Final assessment: H |
| 5-12 | The priority between PRS and SSB | P1, R1-2203437, CATT, [5] P1/P2, R1-2203865, SS, [12] | N | [vivo]:N [SS:] H, RAN4 waits for RAN1 discussion output, although we understand that some companies understand it as RAN4 will continue discussion. But it should not be only dependent on RAN4. [CATT]: The priority between SSB and PRS is up to RAN4 to define. Then send an LS to RAN4 informing RAN1’s conclusion is needed. [OPPO:] This shall be dicussed as H since it is an essential issue. QC: RAN4 to discuss FL Final assessment: H (see if LS to RAN4 is needed) |
| 5-13 | UE PRS measurement both inside MG and inside PPW | P1, R1-2203176, Nokia/NSB, [3] | N | [vivo]:N OPPO: agree with the assessment of FL QC: N FL Final assessment: N |
| 5-14 | Implicit deactivation of MG or PPW for UE measures PRS in either MG or PPW | P8/P9, R1-2203176, Nokia/NSB, [3] | N | [vivo]:N QC: N FL Final assessment: N |
| 5-15 | TP to add non-serving cells on which the DL signals/channels are impacted FL: This TP is not correct because the text here in the spec is only addressing the data cell instead of PRS cells | P3, R1-2203437, CATT, [5] | N | [vivo]:N QC: N FL Final assessment: N |
| 5-16 | UE capability on simultaneously activated PRS processing window | P5, R1-2204903, Huawei/HiSi, [21] | N | [vivo]:H OPPO: agree with the assessment of FL QC: FL Final assessment: N (A way-out is see if it can be treated in the UE feature session) |
| 5-17 | UE indication of PRS dropping to the LMF | P6, R1-2204942, Ericsson, [22] | N | [vivo]:N OPPO: agree with the assessment of FL QC: N FL Final assessment: N |
| 5-18 | Maximum number of PPWs activated by a single MAC CE FL: Reverting RAN1 agreement. | P7, R1-2204986, Qualcomm, [25] | N | [vivo]:H In our view, the MAC CE design should be decided by RAN2. Our previous RAN1 agreement is different from the RAN2 specification, we can revert it and make RAN2 decide it. [SS]: H, this should be discussed. [OPPO:] Do not support to discuss. The number of PPWs in a single MAC CE is not essential issue to RAN1. Furthermore, the design of MAC CE shall be up to RAN2, not in RAN1. [QC]: H. To OPPO: RAN2 has made a different agreement than RAN1. If it is up to RAn2, then we should at least say that RAN1 agreement is reverted. FL Final assessment: H |

Table 6 - RRC_ INACTIVE positioning

| Issue# | Issue | References | FL initial assessment | Company inputs (if any) |
| --- | --- | --- | --- | --- |
| 6-1 | SRS switching time for transmission option 2 on FDD bands | P8, R1-2203516, vivo, [7] P1, R1-2204522, LGE, [20] P8, R1-2204903, Huawei/HiSi, [21] | H | [vivo]:H OPPO: Support to discuss it FL Final assessment: H |
| 6-2 | SUL support of SRS transmission option 2 | P7, R1-2203516, vivo, [7] P6/P7, R1-2204903, Huawei/HiSi, [21] | H | [vivo]:H ZTE: We think this issue has been handled by RAN2’s specification (offline endorsed), and no RAN1 spec impact. So we prefer not having repeating discussion. FL Final assessment: H |
| 6-3 | Handling of SRS transmission and valid RO in RRC_INACTIVE | P9, R1-2203865, SS, [12] | H | [SS]: H OPPO: Support to discuss it FL Final assessment: H |
| 6-4 | TP on clarifying that M-sample applies to DL PRS measurements including DL PRS-RSRPP | P4, R1-2203961, OPPO, [14] | H | [vivo]:H [Huawei, HiSilicon]: This should be under latency improvement. OPPO: Support to discuss it ZTE: M-sample can also be 2 based on RAN4’s latest agreements. We propose to fix the issues together. FL Final assessment: H (latency) |
| 6-5 | TP on terminology alignment to remove “mode” from “RRC_INACTIVE mode” in TS 38.214 FL: Editor was aware of that during the draft CR view, but chose to align the terminology within the spec between different releases by continuing using “RRC_INACTIVE mode”. | P1, R1-2203961, OPPO, [14] | E | OPPO: Thanks for FL’s clarification. We are ok not to discuss it. FL Final assessment: N |
| 6-6 | TP on terminology alignment on positioning SRS | P2, R1-2203961, OPPO, [14] | E | OPPO: The group can make agreement quickly and recommend it to the editor FL Final assessment: E |
| 6-7 | TP on clarifying that UE supporting PRS reception in RRC_INACTIVE is not mandatory | P3, R1-2203961, OPPO, [14] | E | OPPO: The group can make agreement quickly and recommend it to the editor FL Final assessment: E |
| 6-8 | Network indication of switching out of initial DL BWP for PRS measurement | P6, R1-2203176, Nokia/NSB, [3] | N | [vivo]:N QC: N FL Final assessment: N |
| 6-9 | PRS-data collision detection timeline for RRC_INACTIVE | P2/P3, R1-2203786, Xiaomi, [10] | N | [vivo]:N [SS]: H, this should be handled. FL Final assessment: H |
| 6-10 | Handling of SRS transmission and other DL reception and UL transmission in RRC_INACTIVE FL: This seems already covered by the previous working assumption. | P8, R1-2203865, SS, [12] | N | [vivo]:N [SS]: H, this is about the time line for SRS vs other scheduled DL/UL; the original working assumption is only caring the SRS vs collided/overlapped DL/UL, which are different from what want to do here. FL Final assessment: N |
| 6-11 | DRX cycle used for positioning only considers the ones indicated by PEI | P10, R1-2203865, SS, [12] | N | [vivo]:N [SS]: H, we think this should be clarified, otherwise, the Rel16 PS supported UE will have difficulty to operate measurement for these DRX cycle without being activated. FL Final assessment: N |
| 6-12 | Power control of SRS under unified TCI framework FL: This should rather be corrected by the feMIMO WI. | P6, R1-2203961, OPPO, [14] | N | [vivo]:N QC: N FL Final assessment: N |
| 6-13 | Priority between SRS and PUSCH/PUCCH in RRC_INACTIVE state FL: No conclusion reached on enhancing SRS priority handling of other UL in Rel-17, while the existing text in TS 38.214 suggest that SRS is lower priority than PUSCH. | P2/P3/P4, R1-2204522, LGE, [20] | N | [vivo]:N FL Final assessment: N |

Table 7 - On-demand DL PRS

| Issue# | Issue | References | FL initial assessment | Company inputs (if any) |
| --- | --- | --- | --- | --- |
| 7-1 | Update of agreement on parameter level for On-demand PRS Proposal 9: - Update the following agreement regarding on demand PRS. Agreement - From RAN1 perspective, for LMF-initiated request of on-demand DL PRS, the following group of on-demand DL PRS parameters is defined and signaled - per resource set per positioning frequency layer per FR - DL PRS Periodicity - DL PRS Resource Bandwidth - DL PRS Resource Repetition Factor - Number of DL PRS Resource Symbols per DL PRS Resource - DL-PRS CombSizeN - Two options for indication of DL PRS QCL-Info, either - Option 1: per resource set per positioning frequency layer per FR - LMF recommends a list of QCL sources - Option 2: per resource set per positioning frequency layer per FR - LMF requests to provide the QCL information in the assistance data in NRPPa - per FR - Number of DL PRS frequency layers - either per resource set per positioning frequency layer or per UETRP - Start/end time of DL PRS transmission - either per resource, or per resource set, or per UETRP - ON/OFF indicator (for LMF initiated request only). | [7] R1-2203516 | [H] Might be related to reply LS to RAN3 | [vivo]:H OPPO: There is the maintenance of R17 positioning. If RAN3 agreement has impact on RAN1 specifications, we can revise RAN1 spec accordingly. There is no need to revise previous RAN1 agreement directly. RAN2 also received the same LS, and they can revise RAN2 spec accordingly if necessary. QC: H FL Final assessment: H FL comment: Assume we can discuss as part of discussion if any LS or update is needed but will be good for RAN1 to clarify. |
| 7-2 | PRS fallback after on-demand PRS stop Proposal 11: After UE receiving the end time of DL PRS transmission, the UE will stop measuring on-demand PRS and the PRS configuration will fallback to normal PRS to perform subsequent PRS measurements. | [11] R1-2203864 | [N] Unclear what would be broken if we do nothing | [vivo]:N [SS]: H, the ambigious part is, what UE should do after the OD-PRS is ended? Maybe company did not agree to discuss this could clarify. [Huawei, HiSilicon]: N. Reply to SS: we do not think UE initiated on-demand PRS should have any impact on actual UE measurement, which should be only be determined based on actual assistance data and location request. OPPO: Agree with FL QC: N FL Final assessment: N |

## Conclusion

Based on the responses from participating companies during the preparation phase, the final FL recommendation is:

Table 4-1 FL recommendation

| Issue# | FL Final assessment | Comments |
| --- | --- | --- |
| 1-1 | Further discussion under the email thread for the response to RAN4 LS R1-2203024. |  |
| 1-2 | H |  |
| 1-3 | N |  |
| 1-4 | N |  |
| 1-5 | Further discussion under the email thread for the response to RAN3 LS R1-2203040. |  |
| 1-6 | H |  |
| 1-7 | N |  |
| 1-8 | N |  |
| 1-9 | E |  |
| 1-10 | N |  |
| 1-11 | N |  |
| 1-12 | N | FL： If additional discussion on time stamp is needed as a result of discussion of 1-2, we can have further a discussion in 1-2. |
| 1-13 | H |  |
| 1-14 | H |  |
| 1-15 | H |  |
| 1-16 | H |  |
| 2-1 | N |  |
| 2-2 | H |  |
| 3-1 | H |  |
| 3-2 | E |  |
| 3-3 | H |  |
| 3-4 | N |  |
| 3-5 | H |  |
| 3-6 | H |  |
| 3-7 | N |  |
| 3-8 | H |  |
| 3-9 | N |  |
| 3-10 | H |  |
| 3-11 | N |  |
| 3-12 | N |  |
| 3-13 | N |  |
| 3-14 | N |  |
| 3-15 | H |  |
| 4-1 | H |  |
| 4-2 | N |  |
| 5-1 | H |  |
| 5-2 | H |  |
| 5-3 | H |  |
| 5-4 | H |  |
| 5-5 | H |  |
| 5-6 | H |  |
| 5-7 | H |  |
| 5-8 | H |  |
| 5-9 | E |  |
| 5- 10 | E |  |
| 5-11 | H |  |
| 5-12 | H | FL: see if LS to RAN4 is needed. |
| 5-13 | N |  |
| 5-14 | N |  |
| 5-15 | N |  |
| 5-16 | N |  |
| 5-17 | N |  |
| 5-18 | H |  |
| 6-4 | H | FL: In the latency agenda item |
| 6-1 | H |  |
| 6-2 | H |  |
| 6-3 | H |  |
| 6-5 | N |  |
| 6-6 | E |  |
| 6-7 | E |  |
| 6-8 | N |  |
| 6-9 | H |  |
| 6-10 | N |  |
| 6-11 | N |  |
| 6-12 | N |  |
| 6-13 | N |  |
| 7-1 | H |  |
| 7-2 | N |  |

# References

- R1-2203099	Maintenance of Rel-17 positioning accuracy improvements	Huawei, HiSilicon

- R1- 2205095 Maintenance of NR Positioning Accuracy	Nokia, Nokia Shanghai Bell

- R1-2203176	Maintenance of Other NR positioning enhancements	Nokia, Nokia Shanghai Bell

- R1-2203436	Maintenance on enhancements of accuracy improvements for NR positioning	CATT

- R1-2203437	Maintenance on latency reduction for NR positioning	CATT

- R1-2203515	Maintenance on accuracy improvements for NR positioning enhancements	vivo

- R1-2203516	Discussion on other maintenance issues on NR positioning enhancements	vivo

- R1-2203619	Remaining issues on accuracy improvement for Rel-17 positioning	ZTE

- R1-2203620	Remaining issues other than accuracy improvement for Rel-17 Positioning	ZTE

- R1-2203786	Remaining issues on PRS collision detection	xiaomi

- R1-2203864	Maintenance on accuracy improvement related enhancement	Samsung

- R1-2203865	Maintenance on latency and efficiency improvement related enhancement	Samsung

- R1-2203960	Maintenance of Rel-17 P[ositioning Accuracy Enhancement	OPPO

- R1-2203961	Maintenance of Rel-17 Positioning enhancement other than accuracy enhancement	OPPO

- R1-2204127	Remaining issues for accuracy enhancements for NR positioning	InterDigital, Inc.

- R1-2204128	Remaining issues for NR positioning	InterDigital, Inc.

- R1-2204275	Remaining issues on accuracy improvements	CMCC

- R1-2204276	Remaining issues on latency improvements	CMCC

- R1-2204346	Remaining issue on accuracy improvements	NTT DOCOMO, INC.

- R1-2204522	Discussion on maintenance for NR positioning other enhancements	LG Electronics

- R1-2204903	Maintenance of Rel-17 positioning latency and efficiency improvements	Huawei, HiSilicon

- R1-2204942	Maintenance of accuracy improvements for NR positioning enhancements	Ericsson

- R1-2204943	Remaining issues for NR positioning enhancements	Ericsson

- R1-2204985	Maintenance on Accuracy Improvements	Qualcomm Incorporated

- R1-2204986	Maintenance on Other Issues in NR Positioning Enhancements	Qualcomm Incorporated

- R1-2203040 (R3-222721), “Questions concerning the implementation of RAN1 agreements in NRPPa,” RAN3 (Ericsson)

- R1- 2205095 (revision of R1- R1- 2205075)  Maintenance of NR Positioning Accuracy Nokia, Nokia Shanghai Bell