3GPP TSG RAN WG1 #109-e		R1-2205099

e-Meeting, May 9th – 20th, 2022

Agenda Item: 9.2.1

Source: MediaTek Inc.

Title: Overview to Support Artificial Intelligence over Air Interface

Document for: Discussion & Decision

# Introduction

One of the main objectives for Rel-18 NR_AIML_Air SI [1] is to study the 3GPP framework for AI/ML for air-interface, corresponding to each target use case regarding aspects, such as performance, complexity, and potential specification impact.

AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment, model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate

In this contribution, we discuss the general aspects of AI/ML frameworks and provide a high-level overview to support AI/ML over air interface.

# AI/ML Framework and Lifecycle

## Existing AI/ML framework for RAN Intelligence

RAN3 SI Rel-17 FS_NR_ENDC_data_collect to enable RAN intelligence through AI [2] has taken intensive study on the high-level principles, function framework, as well as benefits of AI enabled NG-RAN through the use cases including energy saving, load balancing, mobility management. The study done by RAN3 provides good reference for Rel-18 NR_AIML_Air SI. Some parts of the conclusions draw on the function framework, general principles and terminologies may also be applicable to AI/ML over air interface.

The function framework for RAN intelligence is illustrated in Fig 1 and the common terminologies are also defined in [2]. The framework is defined from function point of view, which considers the necessary functions to enable AI in RAN including data collection, model training, model inference and actor as well as the required flows among those functions.

Fig 1. Functional Framework for RAN Intelligence [2]

## AI/ML framework for Air interface

The framework defined in [2] is generic and potentially applicable to all the use cases, which can be optimized by AI/ML. The function framework can be considered as the baseline for Rel-18 AI/ML over air interface.

Proposal 1: The function framework defined to enable RAN intelligence in TR37. 817 is considered as the baseline for the AI/ML framework over air interface.

Note that this framework will include the basic functionalities of AI/ML model: (1) Data collection, (2) Model training, (3) Model inferencing and (4) Actor.

Proposal 2: The AI/ML framework for air interface includes the functions of data collection, model training, modeling inference and actor. FFS on the need of any other function.

In [2], the terms related to AI/ML are also defined as quoted below:

- Data collection: Data collected from the network nodes, management entity or UE, as a basis for AI/ML model training, data analytics and inference.

- AI/ML Model: A data driven algorithm by applying machine learning techniques that generates a set of outputs consisting of predicted information and/or decision parameters, based on a set of inputs

- AI/ML Training: An online or offline process to train an AI/ML model by learning features and patterns that best present data and get the trained AI/ML model for inference

- AI/ML Inference: A process of using a trained AI/ML model to make a prediction or guide the decision based on collected data and AI/ML model.

It is desired that the same definition is applied to the terminology all over the specifications. The definitions for data collection, AI/ML model, AI/ML training, and AI/ML inference can be considered as a starting point to define the terminology for AI/ML related functions in Rel-18 AI/ML over air interface. It is FFS whether any specific characteristic for AI/ML over air interface need to be considered for the definitions of those terminologies.

Proposal 3: The definitions for the terms “data collection”, “AI/ML model”, “AI/ML training” and AI/ML Inference, as mentioned in [2] are considered as the baseline.

For consistency across different Working Groups (WGs), it is required that the definitions of the terms, mentioned above should be kept aligned over all the specifications. Any specific characteristics for AI/ML over air interface can be captured in the definition only when it is essentially needed.

Proposal 4: The definition for the same term should be kept aligned among different RAN WGs. The specific characteristics for AI/ML over air interface can be captured in the definition only when it is essentially needed.

As shown in Fig. 1, the AI/ML framework will involve data flows between data collection, model training, model inferencing and actor. The major steps regarding these data flows are mentioned below:

- Training data needs to be sent from data collection to model training.

- Similarly, inference data (different from training data) needs to be sent from data collection to model inference,

- Model deployment and model updates need to be propagated from model training to model inference,

- Model performance feedback should be sent from model inference to model training,

- Output of the model needs to be sent from model inference to Actor.

- Finally, there will be feedback from Actor to data collection.

Proposal 5: The AI/ML framework for air interface includes the following data/information flows and FFS on the need of other flow:

- Training data from data collection to model training,

- Inference data from data collection to model inference,

- Model deployment/update from model training to model inference,

- Model performance feedback from model inference to model training,

- Output from model inference to Actor,

- Feedback from Actor to data collection.

# Management of AI/ML Model

The first step of AI/ML operation is model training. Model training is usually a difficult and expensive process and generally worth for big applications. For small applications, some existing, popular model is often used. Existing popular models also provide a basis for standard model development and aids the user to download and use it. However, if the data is unique or the existing solutions doesn’t satisfy the requirement, a complete model training is needed.

Proposal 6: RAN1 will discuss whether a common model can be used for individual use cases.

In the UE, training of AI/ML models could be performed offline or online. Typically, if the data needed for making a trained model is available on user’s device then there is an option to carry out the entire training locally on the UE itself. However, the corresponding power consumption also needs to be considered. Online learning is used where it is computationally infeasible to train over the entire dataset in the UE, where it is necessary for the UE to dynamically adapt to new patterns in the data, or when the data itself is generated as a function of time. Thus, online training is often more complicated comparing to its offline counterparts. Note that a model is “offline” trained even if it is being continuously refined on network side, as long as it is frozen on a UE once it is downloaded.

Proposal 7: RAN1 should study both offline and online model training in UE for individual use cases.

As many use cases (e.g., CSI-compression, beam prediction) involve both UE and gNB, some form of collaboration between UE and gNB is expected to facilitate the training and inferencing process. This collaboration could be of various levels.

- On the one extreme there is learning with “no collaboration”, i.e., implementation-based only AI/ML algorithms without information exchange. This can serve as the base case for comparison.

- On the other extreme there is completely distributed Federated Learning, which enables mobile devices to collaboratively learn a shared the model, while keeping the training data in the device.

Proposal 8: Various levels of collaboration between UE and gNB, pertinent to the selected use cases, should be identified, with “no collaboration”, i.e., implementation-based only AI/ML algorithms without information exchange, serving as the basis for comparison.

The model training is a computation and power consumed process, and usually takes relatively longer time. Generally, functions being learned or use cases being optimized by AI/ML should be stationary for a sufficiently long period time, so that the AI/ML model doesn’t need to be re-trained and updated frequently. The use cases studied in RAN intelligence e.g., load balance, mobility optimization and network energy saving, are more of non-real time or near-real time tasks. However, considering the fast channel variations over air interface, the validity time for the AI/ML models for air interface will be much shorter than the ones for RAN intelligence. It is challenging to update the AI/ML model adaptively to the instantaneous channel conditions.

The tradeoff between the performance and validity period should be considered for AI/ML over air interface. If model training and model inference are not performed at the same place, e.g., model training at network side and model inference at the UE side, the procedures for model deployment/update and performance feedback over air interface are required. In this case, the additional cost for AI/ML in terms of signaling overhead, radio resource consumptions, etc. also needs to be evaluated.

Proposal 9: The validity time and cost of model update for the AI/ML model over air interface should be considered for the performance evaluation.

# General Requirements for Datasets

The dataset is the basis of AI/ML technology. The quality of the dataset affects the AI/ML model/algorithm design and performance. Building data set is also critical to drive the development of AI/ML model/algorithm.

The dataset and the detailed AI/ML model and algorithm are specific for each use case. For each use case, the study on dataset should focus on the type of data as well as the data format required for model training and inference. For the use cases targeted in Rel-18, the evaluation of the AI/ML performance should first focus on the inputs with existing L1 measurements. Inputs with additional new measurement can be considered later when the performance is fully justified.

Considering different collaboration level between UE and gNB, dataset transfer and storage need to be considered. For some cases, gNB collects the data from UEs, stores the dataset and performs model training, the size of the dataset is not much concerned. If UE stores the dataset and perform model training itself, the size of dataset should be considered considering the limited size of memory and computation power at the UE side.

The consistency of the datasets for training, validation, testing, and inference should be guaranteed. In the ideal case, the AI/ML has optimum performance if datasets for training and inference are generated and collected in the same scenario. One way to guarantee the consistency is to identify the different scenarios and construct the dataset for each of the identified scenario.

Another important aspect mentioned for data collection is privacy and anonymization, which should also be considered in the study.

Proposal 10: High-level requirements of datasets for AI/ML over air interface:

- Focus on the existing L1 measurements,

- Restrict the dataset size if the dataset is stored as the UE side,

- Guarantee the consistency of the datasets for training, validation, testing, and inference,

- Respect the user data privacy and anonymization.

# Conclusion

In summary, based on the above discussion we have the following observations and proposals:

Proposal 1: The function framework defined to enable RAN intelligence in TR37. 817 is considered as the baseline for the AI/ML framework over air interface.

Proposal 2: The AI/ML framework for air interface includes the functions of data collection, model training, modeling inference and actor. FFS on the need of any other function.

Proposal 3: The definitions for the terms “data collection”, “AI/ML model”, “AI/ML training” and AI/ML Inference, as mentioned in [2] are considered as the baseline.

Proposal 4: The definition for the same term should be kept aligned over the entire specifications among different RAN WGs. The specific characteristics for AI/ML over air interface can be captured in the definition only when it is essentially needed.

Proposal 5: The AI/ML framework for air interface includes the following data/information flows and FFS on the need of other flow:

- Training data from data collection to model training,

- Inference data from data collection to model inference,

- Model deployment/update from model training to model inference,

- Model performance feedback from model inference to model training,

- Output from model inference to Actor,

- Feedback from Actor to data collection.

Proposal 6: RAN1 will discuss whether a common model can be used for individual use cases.

Proposal 7: RAN1 should study both offline and online model training in UE for individual use cases.

Proposal 8: Various levels of collaboration between UE and gNB, pertinent to the selected use cases, should be identified, with “no collaboration”, i.e., implementation-based only AI/ML algorithms without information exchange, serving as the basis for comparison.

Proposal 9: The validity time and cost of model update for the AI/ML model over air interface should be considered for the performance evaluation.

Proposal 10: High-level requirements of datasets for AI/ML over air interface:

- Focus on the existing L1 measurements,

- Restrict the dataset size if the dataset is stored as the UE side,

- Guarantee the consistency of the datasets for training, validation, testing, and inference,

- Respect the user data privacy and anonymization.

# References

- RP-213599, “Study on Aritificial Intelligence (AI) / Machine Learning (ML) for NR Air Interface,” Qualcomm.

- TR 37.817 V17.0.0 Study on enhancement for Data Collection for NR and EN-DC