3GPP TSG-RAN WG1 Meeting #109		R1-2205102

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.2.3.1

Source: 	MediaTek Inc.

Title:  	AI-assisted Target Cell Prediction for Inter-cell Beam Management

Document for:	Discussion and decision

Introduction

Until Rel-16, beam management is a procedure for intra-cell mobility. UE can be indicated to a TCI state QCLed with a different source RS from the same cell, and thus changes its serving beam. In Rel-17 feMIMO WI [1], we introduced inter-cell beam management (ICBM), where a UE may be configured with TCI states QCLed with source RSs from an additional cell. When UE is indicated to such inter-cell beams, its serving does not change, in other word, the UE acts as a guest to “borrow” resources from a neighbour cell. In Rel-18 Further Mobility Enhancement WI [2], one objective is to introduce L1/L2-based inter-cell mobility, which can be seen as an enhancement of Rel-17 ICBM that allows serving cell change indicated by L1/L2 signalling.

One major benefit of ICBM (with or without serving cell change), compared to legacy RRC-based handover procedure, is reduced latency and interruption during cell change. Ideally, ICBM allows UE to switch to a beam from different cell as fast as intra-cell case (around 5ms) and UE continues transmission with old beam before it switched to new beam (new TCI state). However, such seamless inter-cell beam switching is not achieved by simply replacing RRC-based handover command with L1/L2 signalling, as explained below.

Mobility procedure consists of three major parts: UE reconfiguration, DL synchronization, and UL synchronization. In ICBM, the actual mobility latency (between beam indication and UE using new TCI state) depends on when the beam indication is sent, as illustrated in Figure 1.

Figure 1.	Decomposition of mobility latency

Since ICBM is a L1/L2 procedure, we assume that there is some RRC pre-configuration for candidate cells, and related RRC procedures and UE processing are performed before inter-cell beam indication. Then we consider the mobility latency for different cases as below:

- If beam indication is sent at t1, latency = T1+T2+T3, UE needs to perform DL synchronization and RACH to obtain target cell timing advance, before using inter-cell TCI state (assuming inter-cell UL TCI state in the unified TCI framework for ICBM).

- If beam indication is sent at t2, meaning that UE has DL-synchronized with target cell beforehand, latency = T2+T3. UE needs to perform RACH before using inter-cell TCI state.

- If beam indication at t3, meaning that UE has DL- and UL-synchronized with target cell beforehand, latency = T3. UE can switch to inter-cell TCI state in around 5ms.

The above analysis shows that the latency of ICBM can be minimized only if UE obtains DL synchronization and timing advance for the target cell before the inter-cell beam indication. However, maintaining synchronization with another cell while communicating with current serving cell means higher UE complexity (e.g., requiring additional hardware), and UE is likely to be able to do this only for the most probable target cell, which needs to be predicted by UE or network. In other word, target cell prediction is the key enabler for seamless ICBM operation.

Observation 1:	Target cell prediction is the key enabler for seamless inter-cell beam management operation.

In this contribution, we discuss potential AI/ML assistance for target cell prediction for inter-cell BM. Notice that a cell is called “target cell” when UE is indicated to a beam from that cell, whether the serving cell is changed (Rel-18) or not (Rel-17).

Discussion

Target cell prediction and AI/ML assistance

The goal of target cell prediction for ICBM is to determine UE’s best cell (e.g., the cell with highest RSRP) after a predefined interval, even before UE measurements show that the cell is strong. This helps UE decide whether it will be indicated to an inter-cell beam in near future, and if yes, the target cell ID. This helps UE to perform early preparation works (e.g., synchronization) towards the predicted target, and then switch to an inter-cell beam immediately after receiving the beam indication.

An example is shown in Figure 2, where all three UEs are served by Cell#0. The AI/ML model predicts that UE0 will stay in Cell#0, while UE1 and UE2 will switch to Cell#1 and Cell#2, respectively.

Figure 2.	Target cell prediction for ICBM

For AI/ML-based target cell prediction, we consider beam measurements at a given duration as input, and the output is predicted cell ID after a predefined interval. In this way, we can model the target cell prediction problem as a classification problem, i.e., the cell IDs are classes to be determined by the AI/ML model. If the output class is the same as current serving cell, UE expects no inter-cell beam indication, otherwise UE may expect an inter-cell beam indication towards the output class (cell ID). The method is illustrated in Figure 3. Notice that the measurements from different beams and sampling points form a 2D “figure” as input for AI/ML model.

Figure 3.	Input & output of AI/ML model

Experiment settings

We consider a typical two-tier cellular configuration, with 7-BSs, 3-sector deployment (totally 21 cells), as shown in Figure 4. In each cell, beam sweeping of 24 beams (12 horizontal by 2 vertical directions) is assumed. The cell IDs are also marked in the figure.

Figure 4.	A 7-BS, 3-sector deployment

As mentioned before, in this work, we model the target cell prediction as a classification problem. Therefore, a convolutional neural network (CNN) is used in our AL/ML model. Moreover, we consider UEs served by Cell#0, and the input beam measurements are only taken from Cell#0 (i.e., the serving cell). Even if UE may perform measurements on neighbouring cells, those measurements are not considered. This is because our goal is to help UE identify potential target cell, even before that cell becomes strong. This also helps reduce the model size.

Results and discussions

The performance results of our proposed AI/ML-based target cell prediction are shown in Table 1.

Table 1.	Performance of AI/ML-based target cell prediction for ICBM

The performance metrics are defined as follows (TP = True Positive, TN = True Negative, FP = False Positive, FN = False Negative):

- Precision = TP/(TP+FP)

- Recall (sensitivity) = TP/(TP+FN)

- F1-score = 2*(Recall * Precision) / (Recall + Precision)

- Accuracy = (TP+TN)/(TP+FP+FN+TN), i.e., the overall percentage of correct predictions

Also, support means the number of occurrences of each class.

The experiment results show the following:

- We have a good overall accuracy of 77%, even though the target cell prediction is based only on serving cell beam measurements. This is possible because the variations of serving cell beam qualities imply the location and movement direction of UE.

- The precision (also recall, f1-score) is above 80% for non-handover case (i.e., target is Cell#0). In other words, for more than 80% cases, AI/ML correctly predicts that UE will remain in the same cell.

- For handover cases (i.e., target cell ID is not 0), precision is good (~70%) for adjacent cells (As shown in Figure 4, adjacent cells of Cell#0 include Cell#1, 2, 4, 5, 7, and 20). In other words, in 70% of the cases, AI/ML correctly predicts the target cell if the target is an adjacent cell.

We then have the following observation and proposal.

Observation 2:	AI/ML is beneficial for target cell prediction for inter-cell beam management.

Proposal 1:	RAN1 will discuss the usage of AI/ML-based target cell prediction for inter-cell beam management.

Conclusion

We have the following observations:

Observation 1:	Target cell prediction is the key enabler for seamless inter-cell beam management operation.

Observation 2:	AI/ML is beneficial for target cell prediction for inter-cell beam management.

Proposal 1:	RAN1 will discuss the usage of AI/ML-based target cell prediction for inter-cell beam management.

Reference

- RP-211586, Revised WID: Further enhancements on MIMO for NR

- RP-213565, New WID on Further NR mobility enhancements