3GPP TSG RAN WG1 #109-e			R1-2205115

e-Meeting, May 9th – 20th, 2022

Agenda item:	7.2.11

Source: 	Moderator (NTT DOCOMO, INC.)

Title: 	Summary on Rel-16 NR UE features related discussion

Document for:	Discussion and Decision

- Introduction

This contribution summarizes the discussions and proposals in AI 7.2.11 regarding Rel-16 UE features for eMIMO and URLLC.

Based on the discussions summarized in Section 2, following email discussions/approvals are suggested for AI 7.2.11.

FL proposal #1 of email discussion/approval:

[109-e-R16-UE-features-MIMO-01] Email discussion/approval on UE feature for maximum number of layers for multi-TRP

- Introduce a new FG for maximum number of layers applicable to both single-DCI based multi-TRP FDM scheme A/B and multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency for Rel-17, based on the discussion and proposals in R1-2202511, R1-2203630 and R1-2204703

  - Type is per band, and the FG is for FR2 only

  - Add a restriction to avoid conflict with the existing field maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9)

Companies are encouraged to check above FL proposal #1 and to provide feedback if any in below.

| Company | Comment |
| --- | --- |
| Apple | We are fine to introduce new UE capability. The detailed FG proposal can be for further discussion. |
| Intel | OK in principle – is it restricted to FR2 only |
| Moderator (NTT DOCOMO) | The proposal is updated to clarify that the new FG is for FR2 only as proposals in [1] and [2]. |
| MTK | We support the FL proposal. For the relation between the new UE FG and legacy UE FGs, we provided one comparison table in Observation 4 of R1-2204703 for companies’ reference. |
| Nokia, NSB | We are fine to continue discussion on this UE capability. |
| ZTE | Agree to discuss the issue. Technically, we think this new UE feature will be used to override the FG 16-2a-9. That is, if this new FG is reported by UE, then FG 16-2a-9 should not be used or reported. In such case, we think this new FG should cover the case that multi-DCI based multi-TRP overlapped in at least one RE. Therefore, the wording should be changed as “multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency for Rel-17”. |
| Qualcomm | Agree to discuss the issue and introduce the new FGs with the details to be further discussed. |
| Moderator (NTT DOCOMO) | Thanks for the feedbacks! It seems that companies are ok to discuss the proposal in this meeting. According to the comment from ZTE, the proposal is updated as below, but anyway further details should be discussed in the email discussion (not in preparation phase). FL proposal #1 of email discussion/approval: [109-e-R16-UE-features-MIMO-01] Email discussion/approval on UE feature for maximum number of layers for multi-TRP - Introduce a new FG for maximum number of layers applicable to both single-DCI based multi-TRP FDM scheme A/B and multi-DCI based multi-TRP overlapped in time [but non-overlapped in frequency] for Rel-17, based on the discussion and proposals in R1-2202511, R1-2203630 and R1-2204703 - Type is per band, and the FG is for FR2 only - Add a restriction to avoid conflict with the existing field maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9) |

Based on the discussions summarized in Section 3, following email discussions/approvals are suggested for AI 7.2.11.

FL proposal #2 of email discussion/approval:

[109-e-R16-UE-features-URLLC-01] Email discussion/approval on UE feature for Rel-16 span-based PDCCH monitoring

- Introduce a new FG for Rel-16 PDCCH monitoring capability with additional span patterns (2,1), (4,1) and (7,1), based on the proposal in R1-2204975

  - for Rel-16 or for Rel-17

  - whether or not to create a mirrored version of FG 11-2a, 11-2b, 11-2c, 11-2d, 11-2e, 11-2f and 11-2g which are directly or indirectly related to the new FG

Companies are encouraged to check above FL proposal #2 and to provide feedback if any in below.

| Company | Comment |
| --- | --- |
| MTK | Open to discuss. |
| Nokia, NSB | The proposal in R1-2204975 actually requires CRs to 38.213 to enable the new proposed FG(s). Given that we are not supposed to introduce new functionality to the specs that originate from UE capability AIs, we believe the correct approach here would be to carry out a discussion on the TPs in R1-2204975 within the scope of maintenance AIs first. After that we can have further discussion on how the FGs would be structured, in case the functionality is agreed in maintenance, naturally. |
| ZTE | We understand the intention of introducing new span patterns for UE complexity reduction, while on the other hand it would cause much complexity or restriction at NW side. For instance, for a UE reporting the new patterns, a CORESET with duration of 2 or 3 symbols would not be able to be configured. It would then complicate the implementation for coexistence with other normal UEs. In addition, except for the spec impacts listed in the contribution, it may potentially have more impacts if a UE is allowed to support both legacy and new span patterns. This would require discussion in AI 7.1 first as also commented by Nokia. |
| Moderator (NTT DOCOMO) | Thanks for the feedbacks! As above, there are comments that the proposal should be discussed in maintenance agenda first as the proposal includes not only the new FG but also TP(CR) for RAN1 specification. As the proponent intends Rel-16 (although the moderator thinks Rel-16 or Rel-17 should be the part of discussion given this timing), AI 7.2.5 would be the appropriate maintenance agenda to discuss this proposal. The moderator will contact to AI 7.2.5 moderator. |

- Discussion on UE features for eMIMO

  - Maximum number of layers for multi-TRP (a new FG) [1], [2]

Following proposals are made in contributions in AI7.2.11.

| [1] | Based on the elaboration in [1][2], in FR2, for single-DCI based MTRP schemes, the parameter maxNumberMIMO-LayersPDSCH is shared among the multi-TRP schemes and with the conventional single-TRP operations. A low-complexity UE implementation may support multi-TRP operations by using two panels, each with only one receiver unit (RxU). For single-TRP operations, UE can report maxNumberMIMO-LayersPDSCH = 2 by using both one-RxU panels for data reception. It also works for the single-DCI based SDM and TDM schemes. However, the single-DCI based FDM schemes cannot support 2 layers since the signals from the two TRPs arrive at the same time and in FR2 it is seldom that one panel can receive signals well from both TRPs simultaneously. Therefore, for single-DCI based FDM schemes, the maximum number of layers should be separately reported from single-TRP and the single-DCI based SDM and TDM schemes. Similarly, for multi-DCI based MTRP scheme, a low complexity UE equipped with two panels, each with only one RxU may only support 1 layer per PDSCH/TRP by reporting maxNumberMIMO-LayersPDSCH = 2, interpreted as the maximum number of layers across two PDSCHs if having at least one RE overlapped. However, when the two PDSCHs are overlapped in time but non-overlapped in frequency, the UE cannot support 2 layers per PDSCH in FR2 since generally 1 RxUs per panel is used per PDSCH. Therefore, a FL proposal was suggested as follows in [2]: Updated FL proposal #1: Introduce a new FG for maximum number of layers for single-DCI based multi-TRP FDM scheme A/B and multi-DCI based multi-TRP overlapped in time for Rel-17 Maximum number of layers for single-DCI based multi-TRP FDM scheme A and B and multi-DCI based multi-TRP overlapped in time; 1.For DL reception of single-DCI based multi-TRP FDM scheme A and scheme B, the maximum number of spatial multiplexing layer(s) supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH 2.For DL reception of multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency, the maximum number of spatial multiplexing layer(s) per PDSCH supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH; supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16; Per FSPC; FR2 only. From our view, the granularity should be per band rather than per FSPC to avoid complicated and large overhead capability signaling. Furthermore, for multi-DCI based MTRP, it is not good to split the same UE capability report into 2 cases where case 1 is two PDSCH overlapped in at least one RE as described in FG 16-2a-9, and case 2 is two PDSCHs overlapped in time but non-overlapped in frequency. Hence, we suggest using this new FG to override the FG 16-2a-9. Here is our suggestion: Proposal #1: Introduce a new FG for maximum number of layers for single-DCI based multi-TRP FDM scheme A/B and multi-DCI based multi-TRP overlapped in time for Rel-17 Maximum number of layers for single-DCI based multi-TRP FDM scheme A and B and multi-DCI based multi-TRP overlapped in time; 1. For DL reception of single-DCI based multi-TRP FDM scheme A and scheme B, the maximum number of spatial multiplexing layer(s) supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH 2. For DL reception of multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency, the maximum number of spatial multiplexing layer(s) per PDSCH supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH Note: if this feature is reported, FG 16-2a-9 should not be used.; supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16; Per band FSPC; FR2 only. |
| --- | --- |
| [2] | - Fundamental limitation of multi-TRP in FR2 Since the main benefit of multi-TRP is to have spatial diversity, the two TRPs are in general not co-located. In FR2, highly directional beams are required to combat pathloss and thus different Rx beams need to be formed for reception from different TRPs. It is less likely that one RxU can receive signals with good RSRP from both TRPs simultaneously. In short, in FR2 one RxU can only beamform to one TRP at a time. The above phenomenon is illustrated in Figure 1. Figure 1: Fundamental limitation of multi-TRP in FR2. Due to the above fact, the number of layers per TRP is limited by the number of associated RxUs. More specifically, for a UE equipped with N RxUs to be served by two TRPs simultaneously, the number of layers per TRPs is limited to N/2 RxUs assuming equal number of associated RxUs for each TRP. Observation 1: In FR2, the maximum number of layers per TRP should be upper bounded by the number of associated RxUs, instead of total number of RxUs. - Interpretation of maxNumberMIMO-LayersPDSCH Consider a UE with N RxUs. In the conventional single-TRP operation, all available RxUs can be used for reception from this TRP and thus UE can report maxNumberMIMO-LayersPDSCH = N. In the current NR, this field is shared among all multi-TRP schemes, possibly with a different interpretation. For the multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency and for the FDM schemes of single-DCI based multi-TRP, the maximum number of layers per PDSCH can only be N/2 in FR2, according to the discussion in Section 2. However, the current UE capability reporting assumes the same number of layers as single-TRP, which would force the UE to underreport its capability for single-TRP and other multi-TRP schemes, i.e., single-DCI based SDM and TDM schemes and multi-DCI based multi-TRP with overlap in time and frequency. Observation 2: For a UE intending to support the multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency and the single-DCI based FDM schemes, according to the current UE capability reporting, in FR2 the maximum number of layers needs to be underreported for single-TRP and other multi-TRP schemes, i.e., multi-DCI based multi-TRP with overlap in time and frequency, and single-DCI based SDM and TDM schemes. Observation 3: In the RAN1#108e meeting, we raised this issue of maximum number of layers for multi-TRP [1]. Although the issue was generally recognized, some companies commented that it would be too late for Rel-16 to introduce new FG and some further text check is needed. Thus, it was concluded “no consensus to introduce such a new FG for Rel-16” for the reported issue [2]. However, most companies seem fine to introduce the new FG to Rel-17, and we believe addressing this underreporting issue is important. One natural question is whether support of the mentioned schemes is needed for FR2. For multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency, it is the baseline of the multi-DCI based multi-TRP schemes. Thus, from the perspective of UE capability reporting, even if a UE only wants to support the case of full overlap in time and in frequency, the UE still needs to support the case of non-overlap in frequency. As for the single-DCI based FDM schemes, it has the following advantages for FR2: - More reliable than the single-TRP scheme. - The FDM scheme B is more robust than the SDM scheme when considering the blockage effect, as the current design cannot guarantee that the layers from the same TRP are self-decodable. - Higher system throughput and better power saving than the TDM schemes. For 3), the reason for higher system throughput is that it is challenging to serve multiple users simultaneously in FR2 as different analog beams are required in general. In this case, it is preferable to serve each UE using as less time as possible. Better power saving is achievable because UE can enter the sleeping mode earlier. Although the maximum UE throughput would be lower for the FDM schemes in FR2 as the number of layers is halved, the FDM schemes can still be beneficial for URLLC or low-power applications. Based on the above discussions, we find it necessary to introduce a new UE capability. Following the valuable comments in the RAN1#108e meeting, we propose the following unified UE feature applicable both for the multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency and for the FDM schemes of single-DCI based multi-TRP. In the meantime, we add a restriction to avoid conflict with the existing field maxMIMO-LayersForMulti-DCI-mTRP-r16 [3]. Proposal 1: Introduce the following new UE feature for Rel-17: Definitions for parameters; Per; M; FDD-TDD DIFF; FR1-FR2 DIFF. maxNumberMIMO-LayersPDSCH-For-mTRP-FDM-r17 If this field is included, for DL reception of single-DCI based multi-TRP FDM scheme A and scheme B, the maximum number of spatial multiplexing layer(s) supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH. If this field is included, for DL reception of multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency, the maximum number of spatial multiplexing layer(s) per PDSCH supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH. The UE that indicates support of this feature shall not indicate support maxMIMO-LayersForMulti-DCI-mTRP-r16.; Band; No; N/A; FR2 only. Observation 4: For multi-DCI based multi-TRP, the following table summarizes the UE behaviors considering maxMIMO-LayersForMulti-DCI-mTRP-r16 and maxNumberMIMO-LayersPDSCH-For-mTRP-FDM-r17. We note that if maxMIMO-LayersForMulti-DCI-mTRP-r16 is not included, the two FGs cover independent cases: maxMIMO-LayersForMulti-DCI-mTRP-r16 only covers the case of overlap in time AND in frequency; maxNumberMIMO-LayersPDSCH-For-mTRP-FDM-r17 only covers the case of overlap in time BUT NOT in frequency. It is noted that maxNumberMIMO-LayersPDSCH-For-mTRP-FDM-r17 is the newly proposed UE capability, while maxNumberMIMO-LayersPDSCH and maxMIMO-LayersForMulti-DCI-mTRP-r16 are existing 5G NR UE capabilities. maxMIMO-LayersForMulti-DCI-mTRP-r16; maxNumberMIMO-LayersPDSCH-For-mTRP-FDM-r17; Interpretation of maximum number of layers from maxNumberMIMO-LayersPDSCH (N). Overlap in time and in frequency; Overlap in time but not in frequency; Not overlap in time. Not include; Not include; N layers across two PDSCHs; N layers per PDSCH; N layers per PDSCH. Not include; Include; N layers across two PDSCHs; N/2 layers per PDSCH; N layers per PDSCH. Include; Not include; N layers per PDSCH; N layers per PDSCH; N layers per PDSCH. Include; Include; Error case. |

The issue discussed in [1] and [2] was discussed at the RAN1#108-e meeting as summarized in [4], and the proponent suggested to revisit this issue for Rel-17 in this meeting.

Based on the above, FL suggests discussing this issue in AI7.2.11 at the RAN1#109-e meeting.

Discussion point #1

- Introduce a new FG for maximum number of layers applicable to both single-DCI based multi-TRP FDM scheme A/B and multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency for Rel-17, based on the discussion and proposals in R1-2202511, R1-2203630 and R1-2204703

  - Type is per band, and the FG is for FR2 only

  - Add a restriction to avoid conflict with the existing field maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9)

- Discussion on UE features for URLLC

  - Rel-16 span-based PDCCH monitoring [3]

Following proposal is made in a contribution in AI7.2.11.

| [3] | In Rel. 16 URLLC, a new span-based PDCCH monitoring capability with limitations on the number of CCEs/BDs per span is specified. For this capability, each span is defined as follows [1]: # 10 UE procedure for receiving control information … A UE can indicate a capability to monitor PDCCH according to one or more of the combinations = (2, 2), (4, 3), and (7, 3) per SCS configuration of and . A span is a number of consecutive symbols in a slot where the UE is configured to monitor PDCCH. Each PDCCH monitoring occasion is within one span. If a UE monitors PDCCH on a cell according to combination , the UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of symbols between the first symbol of two consecutive spans, including across slots. A span starts at a first symbol where a PDCCH monitoring occasion starts and ends at a last symbol where a PDCCH monitoring occasion ends, where the number of symbols of the span is up to . …. Based on the span definition above, for the (2,2) pattern, a UE must be able to perform PDCCH decoding in every symbol. As a consequence, the control decoding overhead under the (2,2) pattern could be significant for a UE. Although PDCCH monitoring on every symbol of a slot is not required under (4,3) and (7,3) patterns, PDCCH processing across up to 3 consecutive symbols, in particular with timing capability 2 for DL and/or UL, is challenging. Given the mentioned issue, we propose to add additional patterns, in particular (2,1), (4,1) and (7,1), so that the required scheduling flexibility for supporting URLLC services can be maintained, while easing the UE complexity. This can be done by introducing a new FG as follows which includes the new span patterns: 11-2x; Rel-16 PDCCH monitoring capability; 1. Supported combination(s) of (X, Y, m). For each reported combination, the UE supports the limit C on the maximum number of non-overlapped CCEs for channel estimation per PDCCH monitoring span and the limit M on the maximum number of monitored PDCCH candidates per PDCCH monitoring span 2. Maximum number of DL and UL unicast DCI formats in a span For the set of monitoring occasions which are within the same span: - Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for FDD - Processing one unicast DCI scheduling DL and two unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD - Processing two unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD; pdcch-Monitoring-r16 { pdsch-ProcessingType1-r16{ scs-15kHz-r16 PDCCH-MonitoringOccasions-r16, scs-30kHz-r16 PDCCH-MonitoringOccasions-r16 }, pdsch-ProcessingType2-r16 { scs-15kHz-r16 PDCCH-MonitoringOccasions-r16, scs-30kHz-r16 PDCCH-MonitoringOccasions-r16 }; FeatureSetDownlink-v1610; n/a; n/a; This capability is signalled for SCS 15 kHz and 30 kHz. For μ =0 and 1, candidate value set for (X, Y, μ): {(7, 1, μ), (4, 1, μ), (2, 1, μ)} For component 1, a list of separate UE capabilities (X, Y, μ) for processing capability #1; For component 1, a list of separate UE capabilities (X, Y, μ) for processing capability #2;; Optional with capability signalling. Since, the (X,Y) patterns and their associated CCE/BD limits are included in 38.213, the following changes are then proposed: TP to TS38.213 Section 10 (Rel-16) # 10 UE procedure for receiving control information … A UE can indicate a capability to monitor PDCCH according to one or more of the combinations = (2,1), (2, 2), (4,1), (4, 3), (7,1) and (7, 3) per SCS configuration of and . A span is a number of consecutive symbols in a slot where the UE is configured to monitor PDCCH. Each PDCCH monitoring occasion is within one span. If a UE monitors PDCCH on a cell according to combination , the UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of symbols between the first symbol of two consecutive spans, including across slots. A span starts at a first symbol where a PDCCH monitoring occasion starts and ends at a last symbol where a PDCCH monitoring occasion ends, where the number of symbols of the span is up to . … ## 10.1 UE procedure for determining physical downlink control channel assignment … A PDCCH candidate with index for a search space set using a set of CCEs in a CORESET on the active DL BWP for serving cell is not counted for monitoring if there is a PDCCH candidate with index for a search space set , or if there is a PDCCH candidate with index and , in the CORESET on the active DL BWP for serving cell using a same set of CCEs, the PDCCH candidates have identical scrambling, and the corresponding DCI formats for the PDCCH candidates have a same size; otherwise, the PDCCH candidate with index is counted for monitoring. Table 10.1-2 provides the maximum number of monitored PDCCH candidates, , per slot for a UE in a DL BWP with SCS configuration for operation with a single serving cell. Table 10.1-2: Maximum number of monitored PDCCH candidates per slot for a DL BWP with SCS configuration for a single serving cell Maximum number of monitored PDCCH candidates per slot and per serving cell. 0; 44. 1; 36. 2; 22. 3; 20. Table 10.1-2A provides the maximum number of monitored PDCCH candidates, , per span for a UE in a DL BWP with SCS configuration for operation with a single serving cell. Table 10.1-2A: Maximum number of monitored PDCCH candidates in a span for combination (X, Y) for a DL BWP with SCS configuration for a single serving cell Maximum number of monitored PDCCH candidates per span for combination and per serving cell. (2,1), (2, 2); (4,1), (4, 3); (7,1), (7, 3). 0; 14; 28; 44. 1; 12; 24; 36. Table 10.1-3 provides the maximum number of non-overlapped CCEs, , for a DL BWP with SCS configuration that a UE is expected to monitor corresponding PDCCH candidates per slot for operation with a single serving cell. CCEs for PDCCH candidates are non-overlapped if they correspond to - different CORESET indexes, or - different first symbols for the reception of the respective PDCCH candidates. Table 10.1-3: Maximum number of non-overlapped CCEs per slot for a DL BWP with SCS configuration for a single serving cell Maximum number of non-overlapped CCEs per slot and per serving cell. 0; 56. 1; 56. 2; 48. 3; 32. Table 10.1-3A provides the maximum number of non-overlapped CCEs, , for a DL BWP with SCS configuration that a UE is expected to monitor corresponding PDCCH candidates per span for operation with a single serving cell. Table 10.1-3A: Maximum number of non-overlapped CCEs in a span for combination (X, Y) for a DL BWP with SCS configuration for a single serving cell Maximum number of non-overlapped CCEs per span for combination and per serving cell. (2,1), (2, 2); (4,1), (4, 3); (7,1), (7, 3). 0; 18; 36; 56. 1; 18; 36; 56. …. Based on the specified FGs for Rel. 16 URLLC, the following FGs have FG 11-2 as their prerequisite either directly or indirectly: FG 11-2a, 11-2b, 11-2c, 11-2d, 11-2e, 11-2f and 11-2g. With the addition of the proposed FG 11-2x, this FG can be added as a prerequisite to FG 11-2a, 11-2b, 11-2d and 11-2f. However, if the span-based feature is already implemented and there are concerns on backward compatibility, the alternative approach is to create a mirrored version of FG 11-2a, 11-2b, 11-2c, 11-2d, 11-2e, 11-2f and 11-2g which are directly or indirectly related to FG 11-2x. Proposal: For R16 span-based PDCCH monitoring capability, include (2,1), (4,1) and (7,1) as the new (X,Y) patterns and introduce the following FG: 11-2x; Rel-16 PDCCH monitoring capability; 1. Supported combination(s) of (X, Y, m). For each reported combination, the UE supports the limit C on the maximum number of non-overlapped CCEs for channel estimation per PDCCH monitoring span and the limit M on the maximum number of monitored PDCCH candidates per PDCCH monitoring span 2. Maximum number of DL and UL unicast DCI formats in a span For the set of monitoring occasions which are within the same span: - Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for FDD - Processing one unicast DCI scheduling DL and two unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD - Processing two unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD; pdcch-Monitoring-r16 { pdsch-ProcessingType1-r16{ scs-15kHz-r16 PDCCH-MonitoringOccasions-r16, scs-30kHz-r16 PDCCH-MonitoringOccasions-r16 }, pdsch-ProcessingType2-r16 { scs-15kHz-r16 PDCCH-MonitoringOccasions-r16, scs-30kHz-r16 PDCCH-MonitoringOccasions-r16 }; FeatureSetDownlink-v1610; n/a; n/a; This capability is signalled for SCS 15 kHz and 30 kHz. For μ =0 and 1, candidate value set for (X, Y, μ): {(7, 1, μ), (4, 1, μ), (2, 1, μ)} For component 1, a list of separate UE capabilities (X, Y, μ) for processing capability #1; For component 1, a list of separate UE capabilities (X, Y, μ) for processing capability #2;; Optional with capability signalling. Further, apply the following changes to TS38.213 (Rel. 16): TP to TS38.213 Section 10 (Rel-16): Reasons for change: Introducing (2,1), (4,1) and (7,1) as new PDCCH monitoring span patterns for R-16 PDCCH monitoring capability to ease UE complexity. Summary of change: Addition of the new patterns as possible candidates and determining their per-span CCE/BD limits. Consequence if not adopted: Supporting capability 2 with the new Rel.16 span-based PDCCH has significant UE complexity # 10 UE procedure for receiving control information … A UE can indicate a capability to monitor PDCCH according to one or more of the combinations = (2,1), (2, 2), (4,1), (4, 3), (7,1) and (7, 3) per SCS configuration of and . A span is a number of consecutive symbols in a slot where the UE is configured to monitor PDCCH. Each PDCCH monitoring occasion is within one span. If a UE monitors PDCCH on a cell according to combination , the UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of symbols between the first symbol of two consecutive spans, including across slots. A span starts at a first symbol where a PDCCH monitoring occasion starts and ends at a last symbol where a PDCCH monitoring occasion ends, where the number of symbols of the span is up to . … ## 10.1 UE procedure for determining physical downlink control channel assignment … A PDCCH candidate with index for a search space set using a set of CCEs in a CORESET on the active DL BWP for serving cell is not counted for monitoring if there is a PDCCH candidate with index for a search space set , or if there is a PDCCH candidate with index and , in the CORESET on the active DL BWP for serving cell using a same set of CCEs, the PDCCH candidates have identical scrambling, and the corresponding DCI formats for the PDCCH candidates have a same size; otherwise, the PDCCH candidate with index is counted for monitoring. Table 10.1-2 provides the maximum number of monitored PDCCH candidates, , per slot for a UE in a DL BWP with SCS configuration for operation with a single serving cell. Table 10.1-2: Maximum number of monitored PDCCH candidates per slot for a DL BWP with SCS configuration for a single serving cell Maximum number of monitored PDCCH candidates per slot and per serving cell. 0; 44. 1; 36. 2; 22. 3; 20. Table 10.1-2A provides the maximum number of monitored PDCCH candidates, , per span for a UE in a DL BWP with SCS configuration for operation with a single serving cell. Table 10.1-2A: Maximum number of monitored PDCCH candidates in a span for combination (X, Y) for a DL BWP with SCS configuration for a single serving cell Maximum number of monitored PDCCH candidates per span for combination and per serving cell. (2,1), (2, 2); (4,1), (4, 3); (7,1), (7, 3). 0; 14; 28; 44. 1; 12; 24; 36. Table 10.1-3 provides the maximum number of non-overlapped CCEs, , for a DL BWP with SCS configuration that a UE is expected to monitor corresponding PDCCH candidates per slot for operation with a single serving cell. CCEs for PDCCH candidates are non-overlapped if they correspond to - different CORESET indexes, or - different first symbols for the reception of the respective PDCCH candidates. Table 10.1-3: Maximum number of non-overlapped CCEs per slot for a DL BWP with SCS configuration for a single serving cell Maximum number of non-overlapped CCEs per slot and per serving cell. 0; 56. 1; 56. 2; 48. 3; 32. Table 10.1-3A provides the maximum number of non-overlapped CCEs, , for a DL BWP with SCS configuration that a UE is expected to monitor corresponding PDCCH candidates per span for operation with a single serving cell. Table 10.1-3A: Maximum number of non-overlapped CCEs in a span for combination (X, Y) for a DL BWP with SCS configuration for a single serving cell Maximum number of non-overlapped CCEs per span for combination and per serving cell. (2,1), (2, 2); (4,1), (4, 3); (7,1), (7, 3). 0; 18; 36; 56. 1; 18; 36; 56. …. |
| --- | --- |

Based on the above, FL suggests discussing this issue in AI7.2.11 at the RAN1#109-e meeting.

Discussion point #2

- Introduce a new FG for Rel-16 PDCCH monitoring capability with additional span patterns (2,1), (4,1) and (7,1), based on the proposals in R1-2204975

  - for Rel-16 or for Rel-17

  - whether or not to create a mirrored version of FG 11-2a, 11-2b, 11-2c, 11-2d, 11-2e, 11-2f and 11-2g which are directly or indirectly related to the new FG

- Conclusion

Based on the preparation phase email discussion in AI 7.2.11, RAN1 will take the following email discussion/approval for RAN1#109-e meeting.

[109-e-R16-UE-features-MIMO-01] Email discussion/approval on UE feature for maximum number of layers for multi-TRP by May 13 – Hiroki (DOCOMO)

- Introduce a new FG for maximum number of layers applicable to both single-DCI based multi-TRP FDM scheme A/B and multi-DCI based multi-TRP overlapped in time [but non-overlapped in frequency] for Rel-17, based on the discussion and proposals in R1-2202511, R1-2203630 and R1-2204703

  - Type is per band, and the FG is for FR2 only

  - Add a restriction to avoid conflict with the existing field maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9)

# Reference

[1]	R1-2203630	UE capability on maximum number of layers for MTRP	ZTE

[2]	R1-2204703	On UE capability of maximum number of layers for multi-TRP	MediaTek Inc.

[3]	R1-2204975	Discussion on Rel-16 Span-based PDCCH monitoring	Qualcomm Incorporated

[4]	R1-2202511	Summary on [108-e-R16-UE-features-MIMO-02]	Moderator (NTT DOCOMO, INC.)