3GPP TSG RAN WG1 Meeting #109e		                 	                 R1-2205118

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.4.1 - Support of sidelink on unlicensed spectrum

Source:	Continental Automotive GmbH

Title:	General considerations on SL-U

Document for:	Discussion

# Introduction

In RAN#95e [1], the need for supporting NR-sidelink operation in unlicensed spectrum was confirmed as follows:

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only.

In addition, the scope of this task was bounded mainly to the following aspects: channel access mechanisms and physical channel design. The task is limited to FR1 and is also expected to be completed by RAN#98. In this contribution, we provide our views about the important of supporting unlicensed spectrum operation for NR sidelink as well as the identification of major challenges to be addressed initially.

# SL-U for Advanced Automotive Use Cases

The need for supporting unlicensed-spectrum-capable sidelink should be realized from its potential to provide additional bandwidth to fulfil the requirements of advanced use cases from several industry verticals, particularly automotive. Indeed, according to the 5G Automotive Association (5GAA), up to 200 MHz could be needed to deploy the full range of V2X applications [2]. The feasibility of licensed-exempt spectrum has also been indicated in the so-called Use Cases Masterlist. With the spectrum currently allocated in several regions to Intelligent Transportation Systems (ITS), supporting the wide range of Advanced V2X use cases will be difficult, if not impossible. Thus, the need for leveraging the unlicensed spectrum adjacent to the ITS spectrum is pressing, and as a matter of fact, comes with additional benefit or possibility of reusing Radio Frequency implementations and components designed for the ITS spectrum in the band of 5.9 GHz.

However, it is clear that the open nature of the unlicensed spectrum also poses certain limitations and that some regulatory aspects, such as Listen-Before-Talk (LBT), transmission power, power spectral density, and channel occupancy time, must be respected. These requirements aim at providing fairness among several technologies and services utilizing the shared spectrum. Consequently, the 3GPP has defined channel access schemes, such as New Radio Unlicensed (NR-U) [3], to leverage this spectrum and fulfil these regulations. Thus, Rel. 18’s sidelink evolution targets the reuse of the NR-U framework to support operation in the unlicensed bands n46, n96, and n102.

However, it should also be noticed that unlicensed spectrum, particularly in FR2 (where larger bandwidths are available) can also be exploited to support many applications using NR sidelink. Larger bandwidths imply more system capacity, and shorter transmission time intervals, which can be translated in high data rates and reduced latency, as long as well-designed and fair co-existence and channel access schemes are in place [4]. Therefore, although the scope of this task in focused only on FR1, it would be convenient to keep-an-eye on the eventual need for adding unlicensed spectrum in FR2, and hence, provide a design that can serve, with minimal specification effort, to that end.

Proposal 1: RAN1 should consider the eventual need for supporting unlicensed bands in FR2 when specifying the solutions (channel access mechanisms and physical channel design) for SL-U in FR1. Following general principles of 3GPP, range a range-agnostic design would be desirable.

# Enhancements for SL-U

The adaptation of the existing NR-U framework to NR-sidelink could be a difficult task as the time for Rel. 18’s sidelink evolution is very limited. In very simple terms, the difficulty comes from the fact that the underlying physical layer (channel design) is substantially different. A remarkable example is the time granularity at which both channels can work, i.e., while sidelink operation is essentially slot-based, the Uu interface is provided with much more flexibility, e.g., mini-slots and symbol-level granularity.

In the context of SL-U, channel access schemes and physical layer design are not independent items; they affect each other significantly, and in the same line, it is well-known that LBT is one key element (mandated by regulation) for technologies using unlicensed spectrum. Ideally, the physical layer design should provide the flexibility to take the most from the medium, i.e., use it as efficiently as possible, which in terms of channel access means, take the channel as soon as it is available (obviously reducing the latency).  This fundamental capability, present in NR-U, is not possible with current design of NR-sidelink. In our view, this is a fundamental problem to be addressed by RAN1, bearing in mind that re-designing the physical layer structure won’t be feasible taking into consideration the limited time available to come-up with an effective solution to that.

Another aspect appears in the frequency domain with the resource allocation of NR-U. The interlace option, which provides some frequency diversity, and thus robustness/reliability, cannot be directly applied to SL-U due to the particular resource pool structure.

Proposal 2: RAN1 initial efforts should cover, at least, the following problems:

- adapting the channel access schemes of NR-U, in particular LBT, to the existing NR-SL physical layer (sensing and resource allocation, mode 2), such that channel access time can be made at symbol-level time scales, and

- adapting the resource allocation of NR-U, in particular the interlace option, to the existing SL resource pool structure.

# Summary

In this contribution, we propose our view regarding the importance of supporting operation in unlicensed bands, initially in FR1, but eventually in FR2 as well, and the implications that this could have on the proposed solutions. In addition, we pointed out some essential difficulties that should be addressed initially when defining an NR-U-based SL-U channel access and PHY. The following proposals have been made:

Proposal 1: RAN1 should consider the eventual need for supporting unlicensed bands in FR2 when specifying the solutions (channel access mechanisms and physical channel design) for SL-U in FR1. Following general principles of 3GPP, range a range-agnostic design would be desirable.

Proposal 2: RAN1 initial efforts should cover, at least, the following problems:

- adapting the channel access schemes of NR-U, in particular LBT, to the existing NR-SL physical layer (sensing and resource allocation, mode 2), such that channel access time can be made at symbol-level time scales, and

- adapting the resource allocation of NR-U, in particular the interlace option, to the existing SL resource pool structure.

# References

[1] RP-220320, “NR Sidelink Evolution Status Report to TSG”, OPPO, 3GPP RAN#95e, March 2022.

[2] 5GAA, “Study of spectrum needs for safety related intelligent transportation systems - day 1 and advanced use cases,” TR S-200137 v1.0, Jun. 2020.

[2] 3GPP, TSG RAN. TR 38.889: Study on NR-based access to unlicensed spectrum (v16.0.0), Dec. 2018.

[3] R1-2205119, “General considerations on coexistence between LTE- and NR-sidelink” Continental Automotive GmbH, 3GPP RAN1#109e, May 2022.