3GPP TSG RAN WG1 Meeting #109e		                 	                 R1-2205119

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.4.2 - Co-channel coexistence for LTE sidelink and NR sidelink

Source:	Continental Automotive GmbH

Title:	General considerations on coexistence between LTE- and NR-sidelink

Document for:	Discussion

# Introduction

In RAN#95e [1], the need for considering the co-channel coexistence between LTE- and NR-sidelink was confirmed, as follows:

- Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any

In this contribution, we provide our view on several aspects related to the previous tasks.

# LTE- and NR-sidelink co-channel coexistence

Co-channel coexistence allows two different RATs, in this case LTE-SL and NR-SL, to make use of the same frequency channel. This could certainly have impact not only in terms of specification, but also in terms of implementation.

Rel. 16 [2] defined in-vehicle coexistence and cross-RAT control. These mechanisms define schemes to manage the sidelink (SL) resources of both Radio Access Technologies (RATs) (particularly when they share common elements of the implementation, such as RF chains), as well as allow the Radio Access Network (RAN) to coordinate their operation, while in coverage conditions.

From our point of view, while the necessity of co-channel co-existence can be well-justified, for example in case of spectrum for Intelligent Transportation Systems (ITS), feasibility aspects need to carefully be studied considering the performance vs implementation-complexity trade-offs. Of particular interest are the aspects related to flexibly control the channel sharing between these two RATs to guarantee the QoS requirements of the different safety and non-safety V2X applications (assuming a channel can be used for both types of services) and different vehicular scenario conditions, e.g., load.

So far, several regional regulations allocate certain ITS channels for different technologies, such as DSRC and C-V2X, and the co-channel coexistence between these technologies has been already addressed and studied, e.g., in [3]. In addition, the priority-based framework defined in [4], where priorities and predefined RAT-level actions are defined could be a good starting point for the co-channel coexistence discussion in Rel. 18’s sidelink evolution. In our view, this approach provides the required implementation flexibility, regulation compatibility, and it is also compatible with the existing mechanisms for in-device co-existence defined in Rel. 16; particularly the long-term Time Division Multiplexing (TDM) solution.

Proposal 1: Consider the priority-based framework defined in [4] as baseline for the LTE- and NR-SL co-channel coexistence study.

# Summary

In this contribution, we have provided our view regarding the baseline assumptions to be considered for the LTE- and NR-SL co-channel coexistence study.  In particular, it is proposed to consider the priority-based framework defined in [4] as baseline for this discussion.

Proposal 1: Consider the priority-based framework defined in [4] as baseline for the LTE- and NR-SL co-channel coexistence study.

# References

[1] RP-220320, “NR Sidelink Evolution Status Report to TSG”, OPPO, 3GPP RAN#95e, March 2022.

[2] 3GPP, TSG RAN. TR 37.985: Overall description of radio access network (RAN) aspects for vehicle-to-everything (V2X) based on LTE and NR (v16.0.0, release 16), Sep. 2020.

[3] ETSI, Intelligent Transport Systems (ITS), TR 103 766: Pre-standardization study on co-channel co-existence between IEEE- and 3GPP- based ITS technologies in the 5855 MHz - 5925 MHz frequency band, V1.1.1, Sept. 2021.

[4] ETSI, Intelligent Transport Systems (ITS), TR 103 667: Study on Spectrum Sharing between ITS-G5 and LTE-V2X technologies in the 5855 MHz - 5925 MHz band, V1.1.1, Sept. 2021.