3GPP TSG RAN WG1 #109-e						R1-2205120

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.4

Source: 	Moderator (Thales)

Title: 	Moderator Summary for preparation phase on maintenance of Rel-17 WI on Solutions for NR to support non-terrestrial networks (NTN)

Document for:	Discussion and Decision

## Introduction

The issues in contributions submitted to RAN1#109e are summarized in the tables of section 2. An initial assessment on each of the maintenance issues is provided based on the following classification:

- High priority (H): high-priority item (essential, pending issues, broken spec components) and proposed editorial changes that either enhance the clarity of the specs or correct mistakes

- Non-essential (N): all other purposes such as spec optimization and low priority issues

- Editorial (E): editorial issues that will be handled as editorial CRs (to be communicated to the editors/chairs)

## Issues for agenda item “8.4 Maintenance on Solutions for NR to support non-terrestrial networks (NTN)”

## Issues on UL time/frequency sync and timing relationship

The issues related to UL time/frequency sync and timing relationship are summarized in the following table:

Table 1 – Issues on UL time/frequency sync and timing relationship

| Issue# | Issue | References | FL initial assessment | Company inputs (if any) |
| --- | --- | --- | --- | --- |
| 1-1 | NTA at initial access: The issue related to a possible initial TA overcompensation was discussed during past RAN1 meetings. Introducing a TA margin might be a potential solution but in RAN1#106-b-e meeting RAN1 it was concluded to not define a TA margin. The impact of initial TA overcompensation on NTA at initial access was extensively discussed. The following solutions were proposed: Option 1: Following working assumption made at RAN1#107-e: When TAC (TA) in msg2/msgB is received, UE receives the first adjustment and NTA is updated as: NTA=TA⋅16⋅642μ. Where, TA is the TAC field in msg2/msgB Option 2: support negative NTA values; NTA=-256. 16.642μ+TA. 16.642μ Moderator’s view: As long as the UE initial transmission error does not exceed Te_NTN specified by RAN4 (At FR1: 30% of CP in case of SCS of uplink signals is 30 kHz and only 18% of CP in case of 15 kHz) an over-estimation of Initial TA estimation is not a major issue, at least at FR1. For FR2, anyway the requirement of GNSS accuracy and serving-satellite position estimation error are still to be defined (to be tighter in FR2 than FR1 due to the shorter CP length). This is issue might be discussed as part of Release- 18 package; under sub-AI Other. | [1, 2, 3, 4, 12, 13, 14, 15, 18] | N | [Companies will fill input their views here] [Huawei]: Agree with FL initial assessment. We agree with moderator’s view that for FR2, the requirement are still to be defined. According to our analysis, if the same requirements on serving-satellite position estimation error and GNSS accuracy are reused for FR2, the overall transmission timing error will be too high, i.e. about 78% of the CP for PUSCH, and the UL performance may not be acceptable. Therefore, a tighter requirement on serving-satellite position estimation error and/or GNSS accuracy may anyway be needed. Overall, there is no obvious need to introduce a negative NTA since it does not solve the fundamental issue. [ZTE]: Agree with FL initial assessment. We support Option-1. For FR1, which is focused in Rel-17, the impact of overestimation is not significant. Therefore, introduction of negative offset is not needed at least in Rel-17. [Nokia, NSB]: Agree with FL initial assessment. As we have pointed out several times it would be the UE’s responsibility to ensure that correct timing is met. Hence no need to discuss further. [Samsung]: Agree with the FL assessment. [Ericsson]: Agree with the FL assessment. [Apple]: Agree with the FL assessment. We support Option 1 at least for FR1. It might be discussed in Rel-18 for FR2. [QC]: As Huawei pointed out, tighter requirements will be needed for FR2. Such a tighter requirement can be placed on regular transmission but not for PRACH. To avoid additional work, we should fix the issue now. [LG]: Agree with the FL assessment. If this issue is no longer handled in the Rel-17 NTN, it seems that the previous working assumption (Option 1) is automatically confirmed. [Lockheed Martin]: We agree with the FL’s initial assessment. We agree that a negative NTA does not resolve this issue. We would also like to see more progress on the question of position estimation accuracy for FR2. [NTT DOCOMO]: Same view as FL. Support option 1, we don’t see the necessity of option2. [Xiaomi]: Agree with the FL assessment. |
|  | UE behavior w.r.t Validity timer expiry: It is possible that UE re-acquires assistance information prior to validity timer expiry, but the new epoch time is after the expiry of the current validity timer. Subsequently, the UE behavior between the expiry of the current validity timer and the new epoch time is to be clarified. Moderator’s view: This is a valid issue that needs to be re-discussed and resolved in current RAN1 meeting. Although UE can suspend uplink transmission and wait to re-acquire new SIB19-r17, this is not an optimal solution as UE has already obtained fresh assistance information before expiry of validity timer. | [1, 2, 3, 4, 7, 8, 9, 10, 12, 13, 14, 15, 16, 17] | H | [Huawei]: We disagree with prioritizing the discussion on this issue. We suggest “N” for this issue. When new assistance information is available, it is up to UE implementation to maintain the UL synchronization no matter whether the new UL validity timer starts before or after the expiration of the old validity timer. [Mediatek]: This issue can be discussed in RAN2. [ZTE]: We suggest “N” for this issue. RAN2 has defined the behaviours that UE suspend UL transmission and re-acquire SI upon expiry of validity timer, which are enough to let the system work. The discussion is not essential since the case (UE re-acquires assistance information prior to expiry but the new epoch time is after the expiry) is not a general case and the discussed UE behaviours are just for optimization. Moreover, we think this issue can be handled via implementation by ensuring the obtained epoch time is before expiry. [Panasonic]: We agree with prioritizing the discussion of this issue. In RAN1, it should be clarified how validity periods are aligned in time, when SIB19 becomes valid and how often UE can expect SIB19 updates. [NEC]: it is ok to dicussion this issue in RAN1, as the Validity timer and epoch time are both defined by RAN1. [Nokia, NSB]: We are OK to discuss this aspect. Please note that this UE behavior covers two things: (a) The period between reading the new assistance information and the Epoch time, and (b) the aspect of how the UE regains its UL synchronization after validity timer expiry. [Samsung]: OK to discuss but the issue can be handled by UE implementation. [Ericsson]: Agree with the FL assessment. UE should be allowed to use acquired assistance information before the Epoch time to avoid unnecessary suspension of UL transmission. [Apple]: Agree with FL assessment. UE’s behavior needs to be specified for this case. Since UE already acquired assistance information whose epoch time is after the expiry of the current validity timer, UE does not need to re-acquire assistance information in this scenario. [QC]: We suggest to mark this issue as “N”. [LG]: Agree with the FL assessment. [Lockheed Martin]: We tend to feel this should be considered an H. We feel that issue should be resolved in RAN1. [NTT DOCOMO]: N, we think RAN2 agreement is clear. [Xiaomi]: Agree with the FL assessment, the UE behavior during this ambiguity period should be discussed in RAN1. |
| 1-3 | Ambiguity in the interpretation of SFN indicating Epoch time: The issue on a potential ambiguity in the interpretation of the SFN indicating Epoch time was discussed for the first time at previous RAN1 meeting [19]. The following two options were discussed: Option 1: If indicated explicitly by a SFN and subframe number, the UE considers this frame to be the frame which is nearest to the frame where the message is received. Option 2: Indicated SFN for Epoch time is current SFN or the next upcoming SFN after the frame where the message indicating the Epoch time is received. | [1, 2, 3, 5,8, 9, 11, 12, 13, 15, 16, 17] | H | [Huawei]: Agree with FL initial assessment. Support Option 1, for option2 if t_epoch = SFN 1023 and t = SFN 0 the network may needs to derive the ephemeris for a long time based on the current ephemeris information. Then, when UE receives the assistance information and t_epoch, at t, if the validity timer expires, the UE may need to propagate from SFN 1023 back to SFN 0. The derivation duration is long at both network and UE side, and the errors coming from both sides can be large. [Mediatek]: Agree on the priority and Huawei comment on Option 2. We proposed a 3rd Option, which is close to Option 1 in our TDoc R1-2203385. [ZTE]: Agree with FL initial assessment. We support Option-1 since it provides more flexibility on epoch time indication. Moreover, when fitting is applied in common TA indication, the backward propagation cannot be guaranteed so that the epoch time should not be set at far future. In this case, allowing UE to set epoch time at far future is not an advantage. [Nokia, NSB]: Support FL assessment of having this as high priority. The actual discussion should be deferred to the e-meeting. [Samsung]: Agree with the FL assessment. [Ericsson]: We agree with the FL assessment. We prefer option 2 and disagree with Huawei's comment. The network is free to select the epoch time and need not set it to t_epoch = SFN 1023 when t = SFN 0 if it does not want to. [Apple]: We agree with the FL assessment. We prefer Option 1, since Option 2 more likely leads to the scenario where the indicated epoch time is after the validity timer expiry (issue 1-2). [QC]: We don’t see there is an ambiguity issue but OK to discuss. [LG]: Agree with the FL assessment. [Lockheed Martin]: Agree with priority H [NTT DOCOMO]: Agree with FL. Option 2 is preferred, as Epoch time in the past will lead to invalid assistance information. [Xiaomi]: Agree with FL assessment. |
| 1-4 | Support of negative values of CommonDelayDriftVariation for GEO: The granularity and value ranges of common TA parameters were defined/agreed at RAN1#107-e. According to RAN1#107-e agreement, TACommonDriftVariation can only be positive. Such positive values and value range are appropriate in case of LEO based NTN. However, in case of GEO based NTN, NTACommonDriftVariation can be negative. The support of negative values for TACommonDriftVariation was discussed (for the first time) in previous RAN1 meeting. It was proposed [19] to add 1 bit for allowing support of negative TACommonDriftVariation values for GEO. But there was no consensus and the issue is still open [19]. | [2, 5, 9, 13, 15, 16, 17] | H | [Huawei]: Agree with FL initial assessment. We are open to discuss this further. [Mediatek]: We agree on the priority. As an alternative to adding 1 bit, a new granularity and range could be considered for CommonDelayDriftVariation for GEO as proposed in our TDoc R1-2203385. [ZTE]: Agree with FL initial assessment. We are supportive of including negative TACommonDriftVariation to enable long validity duration in GEO. [Nokia, NSB]: Disagree with FL assessment. As pointed out in our contribution, there is no need for indicating the 2nd order derivative for the relative stationary GEO case. Suggest to have this marked as “N”. [Samsung]: OK to discuss. [Ericsson]: We agree on the FL assessment and are supportive of negative drift variation to extend the validity time for GEO. [Apple]: We are open to discuss this issue. [QC]: We are fine to discuss this issue. [LG]: Ok to discuss [Lockheed Martin]: Agree with priority H [NTT DOCOMO]: Agree with FL. Ok with this proposal. |
| 1-5 | Neighbour cell’s epoch time: Assistance information (i.e. satellite ephemeris and common TA parameters) of neighbor cell would need to be indicated to UE. But there is still an ambiguity on associated Epoch time and related reference point. It would be necessary to clarify: Whether this epoch is based on serving cell’s timing or neighbor cell’s timing? Moderator’s view: This is a valid issue that needs to be discussed in current meeting. The epoch time and reference point related to neighbor cell’s ephemeris/common TA parameters should be provided based on serving cell’s timing. | [3, 11] | H | [Huawei]: Agree with FL initial assessment. We support that the epoch time of assistance information of neighbor cell is based on serving cell’s timing. Using serving cell’s timing can save the signaling overhead of epoch time as there may have several neighbors. [Mediatek]: This issue can be discussed in RAN2. [NEC]: support FL’s view. [Nokia, NSB]: OK to discuss this in the upcoming meeting, but RAN2/RAN3 would probably need to be consulted on the feasibility of the proposed solution, as it involves one gNB knowing the system timing of other gNBs (and potentially also across different NTN-GWs). [Samsung]: OK to discuss. [Ericsson]: Agree that this needs to be discussed. [Apple]: OK to discuss. [LG]: Ok to discuss [Lockheed Martin]: Agree to discuss at H, even if only to agree to have issue discussed in RAN2 [NTT DOCOMO]: N, we think discussion on this issue could be deprioritized. This epoch time could base on serving cell’s timing. [Xiaomi]: Ok to discuss. |
| 1-6 | CRs/TPs for 3GPP TS 38.213 on Common Delay formula and UE-specific TA: Reason for change: The proposal on the TP for TS 38.213 capturing the formula of one-way propagation delay (Delaycommont ) was extensively discussed during RAN1#108-e [19] but not endorsed. The formula of Delaycommont agreed in RAN1#107-e is essential because it provides how the UE interprets/uses the Common TA related parameters indicated by the Network. It is also used by the UE to compute/derive the NTA,adjcommon. Therefore, the agreement on Delaycommont made at RAN1#107e-meeting should be captured in the specifications. Consequences if not approved: Incomplete support for NTN operation in NR. | [4, 5, 6, 15, 16] | H | [Huawei]: Agree with FL initial assessment. Support CRs/TPs for 3GPP TS 38.213 on Common Delay formula and UE-specific TA [Nokia, NSB]: OK to discuss further at the upcoming meeting. [Samsung]: Agree with the FL assessment. [Ericsson]: Agree with FL assessment. [Apple]: OK to discuss. [QC]: Agree [LG]: Ok to discuss [Lockheed Martin]: Agree with priority H [NTT DOCOMO]: Agree with FL. |
| 1-7 | Correction of value ranges for TACommonDrift and TACommonDriftVariation The range for the TACommonDrift is - 261935… + 261935, however, it should be (–218-1 .. +218-1) which is -262143… +262143. The value range for the TACommonDrift should be (–218-1 .. +218-1)* 0.2 x 10-3 µs/s = -52.42 µs/s … + 52.42 µs/s. Similarly, the range for the TACommonDriftVariation should be (0 .. 215-1) which is 0… 32767. The value range for the TACommonDriftVariation should be (0 .. 215-1)* 0.2 x 10-4 µs/s2 = 0… 0.65 µs/s2. | [5] | E | [Huawei]: Agree with FL initial assessment. Support the correction [Nokia, NSB]: Change of parameter definitions and their range is not something that should be taken lightly. Suggest to have this marked as “H” such that we can have a thorough discussion of the situation. [Samsung]: OK to discuss. [Ericsson]: We support the correction as described here. We should provide an exact description of the value ranges to help the spec editor. Note also that issue 1-7 partly depends on the outcome of issue 1-4. [Apple]: OK to discuss. [LG]: Agree with FL initial assessment [Lockheed Martin]: Agree with FL [NTT DOCOMO]: Agree with FL. |
| 1-8 | Reference Frame for Ephemeris Set 2 – Orbital parameters: RAN1#104bis- agreed Support serving-satellite ephemeris broadcast based on Set1:PV state vectors or Set 2: orbital parameter ephemeris format. For set 1, RAN1 agreed position X,Y,Z in ECEF (m) and velocity VX, VY, Vz in ECEF(m/s). For set 2, the assumption for Reference Frame is ambiguous. | [5] | N | [Huawei]: We disagree with FL initial assessment. We support to further discuss ephemeris Set 2 – Orbital parameter. The assumption of the Reference Frame is ambiguous for Set2. The most frequently used reference frame for Set2 propagation is J2000 and needs absolute time at the UE side for rotate the reference frame. Without absolute time of ephemeris Set2, the assumption for reference frame needs further discussion. [Mediatek]: We think it should be high priority as discussed in our TDoc in R1-2203385. We also think RAN1 needs further discussion. [ZTE]: We suggest “H” for this issue. For orbital format, ECI should be the reference frame instead of ECEF. [Nokia, NSB]: support to have this as “N” [Samsung]: Agree with the FL assessment. [Ericsson]: If there is an ambiguity, we think this issue should be given priority "H". [LG] Agree with FL initial assessment. [Lockheed Martin]: We feel this should be a high priority H. [NTT DOCOMO]: Agree with FL. |
| 1-9 | Ambiguity of cell-specific K_offset in SIB modification period: For several meetings RAN1 has been discussing whether there is a need to resolve ambiguity of which cell-specific K_offset value to use during the SIB modification period and the majority of companies think that it can be handled by gNB implementation. RAN1 asks RAN2 to share their understanding on whether there is a need to address this potential ambiguity. Moderator’s view: Wait for RAN2's reply to the cell-specific K_offset ambiguity issue during the SIB modification period. | [5, 9] | N | [Huawei]: Agree with FL initial assessment. Support wait for RAN2's reply [Mediatek]: Agree. Wait for RAN2 reply. [ZTE]: Agree with FL initial assessment. Support wait RAN2 replay [NEC]: agree. Wait for RAN2’s reply. [Samsung]: Agree with the FL assessment. [Ericsson]: Agree with the FL assessment. [Apple]: Agree with the FL assessment. [LG] Agree with FL initial assessment. [Lockheed Martin]: Agree [NTT DOCOMO]: Agree with FL. [Xiaomi]: Agree with FL assessment. |
| 1-10 | CRs/TPs for 3GPP TS 38.213 on timing relationship in the uplink Power control on PUSCH and PUCCH: In the R1-2202984 CR 38.213, timing relationship in the uplink Power control on PUSCH and PUCCH should be considered in NTN specific scenario. But based on latest specification CR, the timing relationship in the uplink power control has not been modified. This issue should be fixed and the detailed description about timing relationship should be specified in 38.213. | [7] | E | [Huawei]: Agree with FL initial assessment. Fine to discuss the TP on power control for PUSCH and PUCCH further. [Samsung]: Agree with the FL assessment. [Ericsson]: Agree with the FL assessment. [LG] Agree with FL initial assessment. [Lockheed Martin]: OK with keeping E [NTT DOCOMO]: Agree with FL. [Xiaomi]: Agree with the FL assessment. |
| 1-11 | Joint operation of open and closed loop timing advance. In order to guarantee TA update loop stability, it is preferred to have well-defined UE behavior when updating the serving satellite ephemeris and common TA information [9]. Moderator’s view: RAN4 provided further updates on agreements about double-correction issue in [R1-2203020]. As per RAN4 agreement the framework of gradual timing adjustment requirement is used to alleviate the impact of double-correction in NTN. No further discussion is needed at RAN1. | [9, 12, 13, 15] | N | [Huawei]: Agree with FL initial assessment. We are open to look into this issue further to see whether it can be left to implementation. [ZTE]: Agree with FL initial assessment. Since RAN4 has agreed on how to handle double-correction issue, there is no need of discussion in RAN1. [Nokia, NSB]: If the general understanding is that this is a non-issue, we would suggest that the range of possible validity time values is significantly reduced (there is no sense in having larger values than 10 seconds for LEO case). Correspondingly we would like companies to refer to the results presented in [9]. Suggest to mark this as “H”. [Samsung]: Agree with the FL assessment. [Ericsson]: Agree with the FL assessment. We have provided comments to LS R1-2203020 in R1-2204658 under AI5. [Apple]: Agree with the FL assessment. [QC]: Agree [LG] Agree with FL initial assessment. [Lockheed Martin]: We agree with FL assessment. [NTT DOCOMO]: Same view as FL. [Xiaomi]: agree with FL assessment, this issue is handled by RAN4. |
| 1-12 | Extension of the range of explicit Epoch time for GEO: The limited range of the SFN (10.24 seconds) forces the network to frequently update the broadcast ephemeris data, which is an unnecessary burden on the network in e.g. GEO where the ephemeris can be valid for a significantly longer time [16]. | [3, 16] | N | [Huawei]: Agree with FL initial assessment. We don’t think this is an essential issue. [ZTE]: Agree with FL initial assessment. This is not a critical issue and no need to discuss in maintenance phase. [Panasonic]: We agree with FL’s initial assessment. To avoid frequent updates of assistance information, while being flexible about embedding SIB19, we have proposed to introduce an SFN-cycle counter in [3]. Depending on time availability, we propose to handle SFN-cycle counter aspects as well. [Nokia, NSB]: Agree with FL assessment. [Samsung]: Agree with the FL assessment. [Ericsson]: We think this issue deserves to be discussed further. [Apple]: Agree with the FL assessment. If needed, it can be discussed with issue 1-3. [LG] Agree with FL initial assessment. [NTT DOCOMO]: Agree with FL. |
| 1-13 | Clarification on Koffset for TAC: Koffset was introduced to cover the long RTD in NTN when a TAC is received on a DL slot and should be applied to an UL slot. If the TAC is received on an UL slot, the Koffset is not needed. However, current spec says for a TAC received on UL slot n, the corresponding adjustment of the UL transmission timing applies from the beginning of uplink slot n + m, where m is determined from the Koffset and the legacy processing time duration, which is not aligned with the motivation of introducing Koffset. Therefore, clarification on Koffset application for TAC is needed, i.e. Koffset should be applied when the TAC is received on DL slot n. | [11] | H | [Huawei]: We disagree with FL initial assessment. We suggest “N” for this issue. According to the current spec, “The uplink slot n is the last slot among uplink slot(s) overlapping with the slot(s) of PDSCH reception assuming TTA=0, where the PDSCH provides the timing advance command and TTA is defined in [4, TS 38.211].” Hence, there is no issue with the current spec since the UL slot n assume TA=0 hence K_offset should be there. [MediaTek] We think the priority can be “N”, as we do not see an issue. [ZTE]: We suggest “N” for this issue. Agree with Huawei that the uplink slot n is illustrated as the last slot overlapping with PDSCH reception assuming TTA=0. So Koffset is needed even if it is called “uplink”. [Nokia, NSB]: We would also suggest to mark this as “N”. [Samsung]: OK to discuss to establish a common understanding. [Ericsson]: We don't think there is an issue, as explained by Huawei. However, since the current text is easy to mis-interpret, we are ok to discuss to see if it is possible to improve the spec text. [Apple]: We think there is no issue here due to the assumption of TA=0. [LG] Suggest to “N” for this issue. [NTT DOCOMO]: We are in the same understanding with Huawei that uplink slot n is clearly specified, so that we may not need the discussion. [Xiaomi]: We don’t see there is an issue. |

[Lockheed]: We agree with the FL.

| 1-14 | Clarification on for MAC-CE Activation/Deactivation: According to RAN1 agreement, for a MAC-CE command in PDSCH, K_mac is needed for UE action and assumption on downlink configuration and is not needed for UE action and assumption on uplink configuration. In NTN, It might be understood for TCI states activation, UE would transmit HARQ-ACK in uplink slot n and apply the MAC-CE from the first downlink slot that is after downlink slot n+3Nslotsubframe,µ+2μ2μKmac∙kmac. For SP SRS activation, UE would transmit HARQ-ACK in uplink slot n and apply the MAC-CE from the first uplink slot that is after uplink slot n+3Nslotsubframe,µ. But the spec does not differentiate whether a downlink slot or an uplink slot should be assumed, the same descriptions apply for both downlink configuration and uplink configuration, and it would cause confusion for a reader without NTN context as a large TA gap exists between a downlink slot and an uplink slot with the same slot index. To clarify this issue, it is proposed to differentiate downlink/uplink slot for MAC-CE activation/deactivation for downlink configuration. | [11] | H | [Huawei]: Agree with FL initial assessment. Fine to have some clarification on for MAC-CE Activation/Deactivation [Ericsson]: Agree with FL assessment. [Lockheed]: We agree with the FL. [NTT DOCOMO]: Agree with FL. |
| --- | --- | --- | --- | --- |
| [1-15] | NTN SIB repetition across SI windows when the epoch time is implicitly known as the end of the SI window If the epoch time is not explicitly provided through SIB, it is implicit known as the end of the SI window. There is an implicit restriction that gNB has to update the assistance information in each SI window. To alleviate the burden to the gNB, it should be possible to allow repeating the same assistance information among different SI windows. | [1] | H | [ZTE]: We suggest “N” for this issue. When SI is repeated for multiple SI window, there will be ambiguity of epoch time if implicit indication method is applied. Even if we explicitly define that the epoch time is the end of first or last SI window, UE still cannot obtain the epoch time since it does not know the order of the SI window it detects. Hence, there is no need to discuss this issue. [Nokia, NSB]: Suggest to have this as “N”. If needed, it could be integrated into the discussion of issue 1-3 [Ericsson]: Explicit epoch time could be used to avoid this problem. We suggest "N" for this issue. [QC]: We don’t see this as an issue. Suggest to have this as “N” [LG] Suggest to “N” for this issue. [NTT DOCOMO]: Suggest ‘N’ for this issue. [Xiaomi]: Suggest to “N” for this issue. We don’t see why changing the content could be a gNB’s burden. |

## Recommendation from the moderator after 1st round of company comments

| Issue# | Issue | Recommendation from the moderator after 1st round of company comments |
| --- | --- | --- |
| 1-1 | NTA at initial access: The issue related to a possible initial TA overcompensation was discussed during past RAN1 meetings. Introducing a TA margin might be a potential solution but in RAN1#106-b-e meeting RAN1 it was concluded to not define a TA margin. The impact of initial TA overcompensation on NTA at initial access was extensively discussed. The following solutions were proposed: Option 1: Following working assumption made at RAN1#107-e: When TAC (TA) in msg2/msgB is received, UE receives the first adjustment and NTA is updated as: NTA=TA⋅16⋅642μ. Where, TA is the TAC field in msg2/msgB Option 2: support negative NTA values; NTA=-256. 16.642μ+TA. 16.642μ Moderator’s view: As long as the UE initial transmission error does not exceed Te_NTN specified by RAN4 (At FR1: 30% of CP in case of SCS of uplink signals is 30 kHz and only 18% of CP in case of 15 kHz) an over-estimation of Initial TA estimation is not a major issue, at least at FR1. For FR2, anyway the requirement of GNSS accuracy and serving-satellite position estimation error are still to be defined (to be tighter in FR2 than FR1 due to the shorter CP length). This is issue might be discussed as part of Release- 18 package; under sub-AI Other. | 11 companies (Huawei, ZTE, Nokia, NSB, Samsung, Ericsson, Apple, Thales, LG, Lockheed Martin, NTT DOCOMO, Xiaomi) think it is not essential issue. 1 company (QC) prefers to discuss it in this meeting and thinks we should fix the issue now to avoid additional work, Moderator’s view: From moderator perspective, as long as the UE initial transmission error does not exceed Te_NTN specified by RAN4 an over-estimation of initial TA estimation is not an issue, at least in FR1. The issue might be discussed in Rel-18 (under sub-AI Other) for FR2. Therefore, it is recommended not to discuss this issue in this meeting, unless the above logic is wrong. |
| 1-2 | UE behavior w.r.t Validity timer expiry: It is possible that UE re-acquires assistance information prior to validity timer expiry, but the new epoch time is after the expiry of the current validity timer. Subsequently, the UE behavior between the expiry of the current validity timer and the new epoch time is to be clarified. Moderator’s view: This is a valid issue that needs to be re-discussed and resolved in current RAN1 meeting. Although UE can suspend uplink transmission and wait to re-acquire new SIB19-r17, this is not an optimal solution as UE has already obtained fresh assistance information before expiry of validity timer. | Tentative: Discuss over email in RAN1#109-e. 5 companies (Huawei, Mediatek, ZTE, QC, NTT DOCOMO) think this it is Non-essential (N) issue. 11 Compnaies (Panasonic, NEC, Nokia, NSB, Samsung, Ericsson, Apple, Thales, LG, Lockheed Martin, Xiaomi) prefer or fine to discuss in this meeting. Moderator view: This issue needs to be re-discussed in current RAN1 meeting. Although UE can suspend uplink transmission and wait to re-acquire new SIB19-r17, this is not an optimal solution as UE has already obtained fresh assistance information before expiry of validity timer. |
| 1-3 | Ambiguity in the interpretation of SFN indicating Epoch time: The issue on a potential ambiguity in the interpretation of the SFN indicating Epoch time was discussed for the first time at previous RAN1 meeting [19]. The following two options were discussed: Option 1: If indicated explicitly by a SFN and subframe number, the UE considers this frame to be the frame which is nearest to the frame where the message is received. Option 2: Indicated SFN for Epoch time is current SFN or the next upcoming SFN after the frame where the message indicating the Epoch time is received. | Discuss over email in RAN1#109-e. All companies are ok to discuss this issue in RAN1#109-e. Moderator recommendation: discuss this issue with separate email thread in this meeting. |
| 1-4 | Support of negative values of CommonDelayDriftVariation for GEO: The granularity and value ranges of common TA parameters were defined/agreed at RAN1#107-e. According to RAN1#107-e agreement, TACommonDriftVariation can only be positive. Such positive values and value range are appropriate in case of LEO based NTN. However, in case of GEO based NTN, NTACommonDriftVariation can be negative. The support of negative values for TACommonDriftVariation was discussed (for the first time) in previous RAN1 meeting. It was proposed [19] to add 1 bit for allowing support of negative TACommonDriftVariation values for GEO. But there was no consensus and the issue is still open [19]. | Discuss over email in RAN1#109-e. 13 companies (Huawei, Mediatek, ZTE, Samsung, Ericsson, Apple, QC, Thales, LG, Lockheed Martin, NTT DOCOMO, Mavenir, Lenovo) are ok to discuss this issue in RAN1#109-e. 1 company (Nokia, NSB) suggests to have this marked as “Non-essential”. Moderator’s view: In case of GEO based NTN, NTACommonDriftVariation can be negative. If NTACommonDriftVariation is indicated in case of GEO with specified value range (only positive values are supported for now) there might be common TA prediction errors at the UE leading to out-of-sync issue. RAN1 should discuss if NTACommonDriftVariation is necessary for GEO based NTN deployment. If so, support of negative values of CommonDelayDriftVariation is needed. Moderator recommendation: discuss this issue with separate email thread in this meeting. |
| 1-5 | Neighbour cell’s epoch time: Assistance information (i.e. satellite ephemeris and common TA parameters) of neighbor cell would need to be indicated to UE. But there is still an ambiguity on associated Epoch time and related reference point. It would be necessary to clarify: Whether this epoch is based on serving cell’s timing or neighbor cell’s timing? Moderator’s view: This is a valid issue that needs to be discussed in current meeting. The epoch time and reference point related to neighbor cell’s ephemeris/common TA parameters should be provided based on serving cell’s timing. | Tentative: Discuss over email in RAN1#109-e. 10 Companies (Huawei, NEC, Nokia, NSB, Samsung, Ericsson, Thales, Apple, LG, Lockheed Martin, Xiaomi) are ok to discuss this issue in this meeting. 2 companies (Mediatek, NTT DOCOMO) think it is not essential. Moderator view: This is a valid issue that needs to be discussed in current meeting. The epoch time and reference point related to neighbor cell’s ephemeris/common TA parameters should be provided based on serving cell’s timing. |
| 1-6 | CRs/TPs for 3GPP TS 38.213 on Common Delay formula and UE-specific TA: Reason for change: The proposal on the TP for TS 38.213 capturing the formula of one-way propagation delay (Delaycommont ) was extensively discussed during RAN1#108-e [19] but not endorsed. The formula of Delaycommont agreed in RAN1#107-e is essential because it provides how the UE interprets/uses the Common TA related parameters indicated by the Network. It is also used by the UE to compute/derive the NTA,adjcommon. Therefore, the agreement on Delaycommont made at RAN1#107e-meeting should be captured in the specifications. Consequences if not approved: Incomplete support for NTN operation in NR. | Discuss over email in RAN1#109-e. All companies prefer or fine to discuss this issue in RAN1#109-e. Moderator recommendation: discuss this issue with separate email thread in this meeting. |
| 1-7 | Correction of value ranges for TACommonDrift and TACommonDriftVariation The range for the TACommonDrift is - 261935… + 261935, however, it should be (–218-1 .. +218-1) which is -262143… +262143. The value range for the TACommonDrift should be (–218-1 .. +218-1)* 0.2 x 10-3 µs/s = -52.42 µs/s … + 52.42 µs/s. Similarly, the range for the TACommonDriftVariation should be (0 .. 215-1) which is 0… 32767. The value range for the TACommonDriftVariation should be (0 .. 215-1)* 0.2 x 10-4 µs/s2 = 0… 0.65 µs/s2. | 11 Companies (Huawei, Nokia, NSB, Samsung, Ericsson, Apple, LG, Lockheed Martin, NTT DOCOMO, Thales) are ok to handle this issue as “E”. 1 company [Nokia, NSB] suggests to have this marked as “H” Moderator’s view: Discus this issue in the meeting. This is an editorial issue that will be handled as editorial CRs (to be communicated to the editors/chairs). |
| 1-8 | Reference Frame for Ephemeris Set 2 – Orbital parameters: RAN1#104bis- agreed Support serving-satellite ephemeris broadcast based on Set1:PV state vectors or Set 2: orbital parameter ephemeris format. For set 1, RAN1 agreed position X,Y,Z in ECEF (m) and velocity VX, VY, Vz in ECEF(m/s). For set 2, the assumption for Reference Frame is ambiguous. | Tentative: Discuss over email in RAN1#109-e. 6 Companies (Huawei, Mediatek, ZTE, Ericsson, Lockheed Martin, Thales) prefer or fine to discuss. 4 Companies (Nokia, NSB, Samsung, LG, NTT DOCOMO) think this is not essential. Moderator’s view: The assumption for reference frame needs further discussion for ephemeris Set 2 Keplerian/ Orbital parameters. It is moderator recommendation is to discuss this issue with separate email thread in this meeting. |
| 1-9 | Ambiguity of cell-specific K_offset in SIB modification period: For several meetings RAN1 has been discussing whether there is a need to resolve ambiguity of which cell-specific K_offset value to use during the SIB modification period and the majority of companies think that it can be handled by gNB implementation. RAN1 asks RAN2 to share their understanding on whether there is a need to address this potential ambiguity. Moderator’s view: Wait for RAN2's reply to the cell-specific K_offset ambiguity issue during the SIB modification period. | All companies are ok to wait for RAN2's reply to the cell-specific K_offset ambiguity issue during the SIB modification period. |
| 1-10 | CRs/TPs for 3GPP TS 38.213 on timing relationship in the uplink Power control on PUSCH and PUCCH: In the R1-2202984 CR 38.213, timing relationship in the uplink Power control on PUSCH and PUCCH should be considered in NTN specific scenario. But based on latest specification CR, the timing relationship in the uplink power control has not been modified. This issue should be fixed and the detailed description about timing relationship should be specified in 38.213. | Discuss over email in RAN1#109-e. All companies are ok to handle this issue as “E”. Moderator’s view: Discus this issue in the meeting. This is an editorial issue that will be handled as editorial CRs (to be communicated to the editors/chairs) |
| 1-11 | Joint operation of open and closed loop timing advance. In order to guarantee TA update loop stability, it is preferred to have well-defined UE behavior when updating the serving satellite ephemeris and common TA information [9]. Moderator’s view: RAN4 provided further updates on agreements about double-correction issue in [R1-2203020]. As per RAN4 agreement the framework of gradual timing adjustment requirement is used to alleviate the impact of double-correction in NTN. No further discussion is needed at RAN1. | 11 Companies (Huawei, ZTE, Samsung, Ericsson, Apple, QC, Thales, LG, Lockheed Martin, NTT DOCOMO, Xiaomi) think this is non-essential issue. 1 company (Nokia, NSB) suggests to mark this as “H”. Moderator’s view: No further discussion is needed at RAN1 as the issue was handled by RAN4: As per RAN4 agreement ( reply LS in [R1-2203020]) the framework of gradual timing adjustment requirement is used to alleviate the impact of double-correction in NTN. |
| 1-12 | Extension of the range of explicit Epoch time for GEO: The limited range of the SFN (10.24 seconds) forces the network to frequently update the broadcast ephemeris data, which is an unnecessary burden on the network in e.g. GEO where the ephemeris can be valid for a significantly longer time [16]. | 9 Companies (Huawei, ZTE, Panasonic, Nokia, NSB, Samsung, Apple, LG, NTT DOCOMO, Thales) think this is an non-essential issue 1 company (Ericsson) thinks this issue deserves to be discussed further. Moderator’s view: shares the same view as the majority. This is a low priority issue. No need to be discussed in this meeting. |
| 1-13 | Clarification on Koffset for TAC: Koffset was introduced to cover the long RTD in NTN when a TAC is received on a DL slot and should be applied to an UL slot. If the TAC is received on an UL slot, the Koffset is not needed. However, current spec says for a TAC received on UL slot n, the corresponding adjustment of the UL transmission timing applies from the beginning of uplink slot n + m, where m is determined from the Koffset and the legacy processing time duration, which is not aligned with the motivation of introducing Koffset. Therefore, clarification on Koffset application for TAC is needed, i.e. Koffset should be applied when the TAC is received on DL slot n. | 10 Companies (Huawei, MediaTek, ZTE, Nokia, NSB, Apple, LG, NTT DOCOMO, Thales, Xiaomi ) suggest “N” for this issue. 2 companies (Samsung, Ericsson) are ok to discuss. Moderator’s view: shares the same view as the majority. This is a low priority issue. |
| 1-14 | Clarification on for MAC-CE Activation/Deactivation: According to RAN1 agreement, for a MAC-CE command in PDSCH, K_mac is needed for UE action and assumption on downlink configuration and is not needed for UE action and assumption on uplink configuration. In NTN, It might be understood for TCI states activation, UE would transmit HARQ-ACK in uplink slot n and apply the MAC-CE from the first downlink slot that is after downlink slot n+3Nslotsubframe,µ+2μ2μKmac∙kmac. For SP SRS activation, UE would transmit HARQ-ACK in uplink slot n and apply the MAC-CE from the first uplink slot that is after uplink slot n+3Nslotsubframe,µ. But the spec does not differentiate whether a downlink slot or an uplink slot should be assumed, the same descriptions apply for both downlink configuration and uplink configuration, and it would cause confusion for a reader without NTN context as a large TA gap exists between a downlink slot and an uplink slot with the same slot index. To clarify this issue, it is proposed to differentiate downlink/uplink slot for MAC-CE activation/deactivation for downlink configuration. | Discuss over email in RAN1#109-e. 4 companies (Huawei, Ericsson, Lockheed, NTT DOCOMO) are ok to discuss this issue in current meeting. Moderator recommendation: discuss this issue with separate email thread in this meeting. |
| 1-15 | NTN SIB repetition across SI windows when the epoch time is implicitly known as the end of the SI window If the epoch time is not explicitly provided through SIB, it is implicit known as the end of the SI window. There is an implicit restriction that gNB has to update the assistance information in each SI window. To alleviate the burden to the gNB, it should be possible to allow repeating the same assistance information among different SI windows. | Companies (ZTE, Nokia, NSB, Ericsson, QC, LG, NTT DOCOMO, Thales, Xiaomi) think this is not an essential issue. Moderator’s view: When epoch time is implicitly known as the end of the SI window it is not possible to allow repeating the same assistance information among different SI windows. If repetition of the same assistance information is required, explicit indication using SFN can be used. This is non- essential issue. |

## Issues on HARQ

The issues related to HARQ enhancements are summarized in the following table:

Table 2 – Issues on HARQ

| Issue# | Issue | References | FL initial assessment | Company inputs (if any) |
| --- | --- | --- | --- | --- |
| 2-1 | Clarification on C-DAI and T-DAI definition: TP for TS 38.213 V17.1.0 to clarify C-DAI and T-DAI count for feedback-enabled HARQ processes. Indeed, as per RAN1 #105e agreement: For Type-2 HARQ codebook in NTN, for the DCI of PDSCH with feedback-enabled HARQ processes, the C-DAI and T-DAI are the count of only feedback-enabled processes. Thereby, the definition of the C-DAI and T-DAI is changed, however, the latest CR still uses the legacy C-DAI and T-DAI definition. | [11] | H | [Companies will fill input their views here] [Huawei]: Agree with FL initial assessment. Support clarification on C-DAI and T-DAI definition [ZTE]: Agree with FL initial assessment. Fine for further clarification. [Samsung]: Current specification text is OK – the value of the DAI in DCIs scheduling TBs with disabled HARQ is not used anywhere - the following is stated and the pseudo-code does not apply for such TBs. “If a UE is provided HARQ-feedbackEnabling-disablingperHARQprocess indicating disabled HARQ-ACK information for a HARQ process associated with a transport block for PDCCH monitoring occasion m or for SPS PDSCH receptions on serving cell c, the UE does not multiplex a HARQ-ACK information bit corresponding to the transport block in a Type-2 HARQ-ACK codebook and does not consider the transport block as received in the determination of Nm,creceived or of NSPS,c in clause 9.1.3.1.” [Ericsson]: Agree with FL assessment. [Apple]: We are fine for further clarification. [LG] Agree with FL initial assessment. [NTT DOCOMO]: We do not think this correction is necessary since text in section 9.1.3 of 213 seems to cover the intention of this correction already. Alternatively, one possible correction for clarification might be ‘… does not consider the transport block and the corresponding DCI format as received…’ in the section. [Xiaomi]: agree with the motivation, fine to further discuss how to reflect RAN1’s agreements on c-DAI/t-DAI in 38.213. |
| 2-2 | HARQ-ACK codebook SPS PDSCH: The HARQ-ACK codebook construction for SPS PDSCH has not been captured in the specifications. | [12] | H | [Huawei]: Agree with FL initial assessment. Support capture the HARQ-ACK codebook construction for SPS PDSCH. [ZTE]: Agree with FL initial assessment. Fine for further maintenance on specification. [Samsung]: OK to discuss but based on the agreement for the Type-1 CB that ‘NACK’ is provided for a TB with HARQ process associated with disabled HARQ-ACK, the TP seems incorrect. Also, the current specification captures the following in that section which would be contradicting. “If a UE is provided HARQ-feedbackEnabling-disablingperHARQprocess indicating disabled HARQ-ACK information for a HARQ process associated with a transport block in PDSCH reception occasion m on serving cell c, the UE reports a NACK value for a HARQ-ACK information bit corresponding to the transport block in a Type-1 HARQ-ACK codebook …” [Ericsson]: Ok to discuss further. [Apple]: Agree with FL initial assessment. Specification needs to be updated to capture the HARQ-ACK codebook construction for SPS PDSCH. [LG] Ok to discuss. [NTT DOCOMO]: We do not think this correction is necessary since text with HARQ-feedbackEnablingforSPSactive in section 9.1.3 of 213 seems to cover the intention of this correction. In the pseudo-code, any PDSCH receptions via HARQ process with feedback-disabling configuration are not considered based on the above text. [Xiaomi]: Ok to discuss |
| 2-3 | Application time of updated Koffset: It has been agreed that UE specific Koffset can be updated by MAC-CE and the application time of the new Koffset is defined as: If the UE is provided a KUE,offset value by a MAC CE command, the UE applies the MAC command in the first slot that is after slot k+3Nslotsubframe,μ where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the MAC CE command, μ is the SCS configuration for the PUCCH transmission that is determined in the slot when the MAC CE command is applied. When the scheduling PDCCH comes before the defined application time, k+3Nslotsubframe,μ, and the scheduled PUCCH/PUSCH is after the application time, it’s unclear if the new or old Koffset should be used. In fact, the transmit time of PUCCH and PUSCH depends on the value of the Koffset. This ambiguity exists in the following cases: The transmission timing of DCI scheduled PUSCH (including CSI on PUSCH). The transmission timing of HARQ-ACK on PUCCH (including PUCCH in response to MsgB). The transmission timing of aperiodic SRS. | [18] | H | [Huawei]: Agree with FL initial assessment. The ambiguity of new Koffset application exists and in fact this was discussed at the last meeting. We are fine to discuss it further. [ZTE]: We are OK for further discuss on this issue. [Nokia, NSB]: OK to discuss this further. [Samsung]: OK to further discuss. [Ericsson]: Ok to discuss further. [Apple]: Ok to discuss further. [QC]: Agree. [LG] Ok to discuss. [NTT DOCOMO]: We are fine to discuss, but the ambiguity can be handled by NW configurations or scheduling. [Xiaomi]: Ok to discuss. |

## Recommendation from the moderator after 1st round of company comments

| Issue# | Issue | Recommendation from the moderator after 1st round of company comments |
| --- | --- | --- |
| 2-1 | Clarification on C-DAI and T-DAI definition: TP for TS 38.213 V17.1.0 to clarify C-DAI and T-DAI count for feedback-enabled HARQ processes. Indeed, as per RAN1 #105e agreement: For Type-2 HARQ codebook in NTN, for the DCI of PDSCH with feedback-enabled HARQ processes, the C-DAI and T-DAI are the count of only feedback-enabled processes. Thereby, the definition of the C-DAI and T-DAI is changed, however, the latest CR still uses the legacy C-DAI and T-DAI definition. | Tentative: Discuss over email in RAN1#109-e 7 Companies (Huawei, ZTE, Ericsson, Apple, LG, Thales, Xiaomi) are ok to further discuss in current meeting. 2 companies (Samsung, NTT DOCOMO) think that current specification text is OK. No further discuss is needed. Moderator’ view: clarification on C-DAI and T-DAI definition is needed. The issue can be discussed in this meeting. |
| 2-2 | HARQ-ACK codebook SPS PDSCH: The HARQ-ACK codebook construction for SPS PDSCH has not been captured in the specifications. | Discuss over email in RAN1#109-e 7 companies (Huawei, ZTE, Samsung, Ericsson, Thales, LG, Xiaomi) companies are ok to discuss the issue in this meeting. 1 company (NTT DOCOMO) thinks this correction is necessary. Moderator’s view: The HARQ-ACK codebook construction for SPS PDSCH needs to be captured in the specifications. Further discussion is needed in current RAN1 meeting. |
| 2-3 | Application time of updated Koffset: It has been agreed that UE specific Koffset can be updated by MAC-CE and the application time of the new Koffset is defined as: If the UE is provided a KUE,offset value by a MAC CE command, the UE applies the MAC command in the first slot that is after slot k+3Nslotsubframe,μ where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the MAC CE command, μ is the SCS configuration for the PUCCH transmission that is determined in the slot when the MAC CE command is applied. When the scheduling PDCCH comes before the defined application time, k+3Nslotsubframe,μ, and the scheduled PUCCH/PUSCH is after the application time, it’s unclear if the new or old Koffset should be used. In fact, the transmit time of PUCCH and PUSCH depends on the value of the Koffset. This ambiguity exists in the following cases: The transmission timing of DCI scheduled PUSCH (including CSI on PUSCH). The transmission timing of HARQ-ACK on PUCCH (including PUCCH in response to MsgB). The transmission timing of aperiodic SRS. | Discuss over email in RAN1#109-e All companies are ok/fine to further discuss this is issue in this meeting. |

## Conclusion

Based on companies comments summarized in section 2 and Chairman’s guidance, the following issues will be discussed in RAN1#109e:

| For maintenance on timing relationship enhancements and UL time and frequency synchronization for NR NTN: |  |
| --- | --- |
| 1-02 | UE behavior w.r.t Validity timer expiry |
| 1-03 | Ambiguity in the interpretation of SFN indicating Epoch time |
| 1-04 | Support of negative values of CommonDelayDriftVariation for GEO |
| 1-05 | Neighbour cell’s epoch time |
| 1-06 | CRs/TPs for 3GPP TS 38.213 on Common Delay formula and UE-specific TA |
| 1-07 | Correction of value ranges for TACommonDrift and TACommonDriftVariation |
| 1-08 | Reference Frame for Ephemeris Set 2 – Orbital parameters |
| 1-10 | CRs/TPs for 3GPP TS 38.213 on timing relationship in the uplink Power control on PUSCH and PUCCH |
| 1-14 | Clarification on for MAC-CE Activation/Deactivation |
| For maintenance on scheduling and HARQ for NR NTN: |  |
| 2-01 | Clarification on C-DAI and T-DAI definition |
| 2-02 | HARQ-ACK codebook SPS PDSCH |
| 2-03 | Application time of updated Koffset |

These issues will be discussed over email in RAN1#109e using the following email discussions:

[109-e-R17-NR-NTN-01] Email discussion for maintenance on timing relationship enhancements and UL time and frequency synchronization for NR NTN, for issues 1-2, 1-3, 1-4, 1-5, 1-6, 1-7, 1-8, 1-10, 1-14 in R1-2205120, taking into account LS received in R1-2203019 and R1-2203020 – Mohamed (Thales)

- 1st check point: May 13 (any RRC impact by May 12)

- Final check point: May 18

[109-e-R17-NR-NTN-02] Email discussion for maintenance on scheduling and HARQ for NR NTN, for issues 2-1, 2-2 and 2-3 in R1-2205120 – Nan (ZTE)

- 1st check point: May 13 (any RRC impact by May 12)

- Final check point: May 18

# References

- R1-2203088	Maintenance on solutions for NR to support NTN	Huawei, HiSilicon

- R1-2203231	Remaining issues on NR-NTN	ZTE

- R1-2203289	Maintenance on Solutions for NR to support non-terrestrial networks (NTN)	PANASONIC R&D Center Germany

- R1-2203306	Maintenance on Solutions for NR to support non-terrestrial networks (NTN)	Spreadtrum Communications

- R1-2203385	Maintenance on Solutions for NR to support NTN	MediaTek Inc.

- R1-2203721	Discussion on ambiguity of common TA calculation	Sony

- R1-2203756	Maintenance on NR NTN	CATT

- R1-2203770	Discussion on maintenance issues in NR-NTN	xiaomi

- R1-2203843	Maintenance aspects af Rel-17 NR over NTN	Nokia, Nokia Shanghai Bell

- R1-2203935	Discussion on the remaining issues in R17 NR NTN	NEC

- R1-2203990	Discussion on remaining issue for NTN-NR	OPPO

- R1-2204207	On remaining issues of NR NTN	Apple

- R1-2204345	Remaining issues on NR NTN	NTT DOCOMO, INC.

- R1-2204519	Remaining issues on UL time and frequency synchronization enhancements in NTN	LG Electronics

- R1-2204556	Maintenance on Release-17 NR NTN	THALES

- R1-2204660	On NR NTN maintenance issues	Ericsson

- R1-2204933	Enhancements on UL time and frequency synchronization	Mavenir

- R1-2204984	Maintenance  on NR NTN	Qualcomm Incorporated

- FL Summary #4: Maintenance on UL time and frequency synchronization for NR NTN, Moderator (Thales), March 2022