3GPP TSG RAN WG1 #109-e			R1-2205124

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.2

Source: 	Moderator (Qualcomm)

Title: 	Moderator Summary for preparation phase on maintenance on Supporting NR from 52.6GHz to 71 GHz

Document for:	Discussion and Decision

## Introduction

The issues in contributions submitted to RAN1#109e are summarized in the tables of sections 2 and 3. An initial assessment on each of the maintenance issues is provided based on the following classification:

- High priority (H): high-priority item (essential, pending issues, broken spec components) and proposed editorial changes that either enhance the clarity of the specs or correct mistakes

- Non-essential (N): all other purposes such as spec optimization and low priority issues

- Editorial (E): editorial issues that will be handled as editorial CRs (to be communicated to the editors/chairs)

## Issues for agenda item “8.2.1	Initial access aspects”

Table 1 - Initial access aspects

| Issue# | Issue | References | FL initial assessment | Company inputs (if any) |
| --- | --- | --- | --- | --- |
| 1-1 | Additional RB offset for 96 PRB CORESET#0 | [1] [2] [4] [5] [6] [7] [8] [9] [10] [11] [13] | H | LGE: We prefer to defer this discussion until RAN4 finalize related issue, but open to discuss this issue. Nokia: Agree with FL assessment. Should be discussed to conclude the specification. vivo: Agree with FL assessment. Should discuss to make the specification complete. |
| 1-2 | Use of PBCH DMRS for partial indication of Q | [3] | N |  |
| 1-3 | Confirmation of WA on signaling for Q | [4] [7] [9] [11] | H | Huawei, HiSilicon: suggest to change to “N”, because the WA is already captured in the spec. it may not need to take time to confirm ZTE: We agree with confirming the WA but it seems to have been already captured into 38.213-h10. Nokia: Agree with FL assessment, hopefully brief. CATT: prefer to confirm the WA, this is nothing to do if current specification already |
| 1-4 | CD-SSB frequency indication using NCD-SSB | [6] | H | LGE: It seems optimization but open to discuss this issue. |
| 1-5 | Description update of longBitmap of SSB-ToMeasure | [6] | E (however for RAN2 spec, therefore suggest to treat as H |  |
| 1-6 | Removal of ‘-‘ sign from 24 RB offset for mux pattern 3 | [1] | E |  |

## Issues for agenda item “8.2.2	PDCCH monitoring enhancements”

Table 2 - PDCCH monitoring enhancements

| Issue# | Issue | References | FL initial assessment | Company inputs (if any) |
| --- | --- | --- | --- | --- |
| 2-1 | SS configuration for Type1 CSS without dedicated RRC and Type0/0A/2 CSS | [14] [15] [16] [17] [21] [22] [23] [24] [26] [28] [29] [31] [33] | H |  |
| 2-2 | default value of monitoringSlotsWithinSlotGroup-r17 | [29] | N | LGE: It may be not a necessary issue, but it should be clarified which value will be applied when the bitmap is absent if the bitmap is defined as optional field with “Need R” condition. |
| 2-3 | PDCCH repetition for Group (1) SS sets | [33] | N |  |
| 2-4 | Applicability of Xs,Ys PDCCH monitoring configuration to active or all SSSG in active BWP | [19] [21] [29] | H (related to 2-5) |  |
| 2-5 | SSSG switching between different Xs,Ys PDCCH monitoring combinations | [14] [15] [17] [19] [21] [22: FLNote1] [27] [28] [29] [30] [31] [33] | H (related to 2-4) |  |
| 2-6 | SSSG switching minimum time | [23] [27] | E (confirming WA) | Huawei, HiSilicon: the WA is already captured in the spec. No need to confirm ZTE: We agree with confirming the WA but it has already been captured into 38.213-h10. MTK: suggest to confirm the WA if issue 2-5 is resolved with same (Xs, Ys) |
| 2-7 | Support PDCCH monitoring before and after SSSG switching | [31] | N | Intel: This is same issue as 2-9, i.e., regarding potential dropping rule at the boundary of SSSG switching. We prefer to handle this issue together with issue 2-5 Sharp: We share same view with Intel. Issue #2-7 and #2-9 are highly related to issue #2-5 and therefore should be discussed together. |
| 2-8 | DCI processing limitations | [14] [33] | H | Qualcomm: We support this discussion and believe that this issue is related to 2-1. |
| 2-9 | Dropping rules in case of overbooking across different slot groups | [17] [25] [33] | N | Intel: This is same issue as 2-7, i.e., regarding potential dropping rule at the boundary of SSSG switching. We prefer to handle this issue together with issue 2-5 Sharp: We share same view with Intel. Issue #2-7 and #2-9 are highly related to issue #2-5 and therefore should be discussed together. |
| 2-10 | Scope of multi-slot PDCCH monitoring being mandatory capability for UE not supporting FR2-2 | [24] | H/E | DCM: As the proposing company, we generally believe this issue can be classified as E. CATT: E |
| 2-11 | BD/CCE budget determination for PDCCH monitoring (e.g. dependency on Ys) | [15] [16] [17] [21] [23] [28] [30] | H | Huawei, HiSilicon: suggest to change to non-essential (N). Five companies [15], [16], [21], [23], [28] suggest to confirm the WA. However, WA is already captured in the spec and no need to confirm. One company suggest to change the WA and provide dependency of the BD/CCE budget to Ys. However, this has been discussed extensively in the last two meetings and most companies have a different view. |
| 2-12 | UE capability signaling for CA/NR-DC operation | [14] [18] [22] [23] [30] | H |  |
| 2-13 | BD/CCE budget allocation over multiple serving cells (incl. multi-DCI multi-TRP) | [22] [30] | H | Ericsson: As proponent, we agree this needs to be completed as indicated by the spec editor after last meeting. |
| 2-14 | PDCCH monitoring pattern alignment across CCs | [30] | N | Intel: We prefer to discuss this issue for an conclusion, though we think the proposal is not necessary |
| 2-15 | Missing RRC parameter pdcch-BlindDetectionCA-CombIndicator-r17 | [22] | E | Ericsson: The parameter needs to be added to the RRC parameter spreadsheet to send to RAN2. |
| 2-16 | COT sharing conditions | [20] | treat in 8.2.4? | Qualcomm: This can be discussed in AI 8.2.4. Huawei, HiSilicon: Seem non-essential. Maybe discussed in 8.2.4 if deemed necessary. Nokia: Agree with FL initial assessment CATT: 8.2.4 |
| 2-17 | CSI-RS validation | [20] | treat in 8.2.4? | Qualcomm: This can be discussed in AI 8.2.4. Huawei, HiSilicon: Seem non-essential. Maybe discussed in 8.2.4 if deemed necessary. Nokia: Agree with FL initial assessment CATT: 8.2.4 |
| 2-18 | Cancellation of downlink reception | [20] | treat in 8.2.4? | Qualcomm: This can be discussed in AI 8.2.4. Huawei, HiSilicon: Seem non-essential. Maybe discussed in 8.2.4 if deemed necessary. Nokia: Agree with FL initial assessment CATT: 8.2.4 |
| 2-19 | PDCCH monitoring in a beam not covered by gNB sensing | [20] | treat in 8.2.4? | Qualcomm: This can be discussed in AI 8.2.4. Huawei, HiSilicon: Seem non-essential. Maybe discussed in 8.2.4 if deemed necessary. Nokia: Agree with FL initial assessment CATT: 8.2.4 |

FLNote1: For Issue 2-5, [22] suggests that the existing working assumption is sufficient.

## Issues for agenda item “8.2.3	PDSCH/PUSCH enhancements”

Table 3 – RS and timeline

| Issue# | Issue | References | FL initial assessment | Company inputs (if any) |
| --- | --- | --- | --- | --- |
| 3-1 | DMRS Bundling for same TB in FR2-2 with 120 kHz SCS across Multiple PUSCHs. FL note: the exact same issue had been discussed in RAN1#108-e where one company expressed concerns on lack of requirement study to apply this feature to FR2-2 in RAN4. FL is not aware of any change in RAN4 study status yet. | [36, 49] | N | Nokia: H It makes sense to conclude that from RAN1 point of view Rel-17 specs provide the necessary support for DMRS bundling for same TB. FL: this issue is low priority. Such conclusion can be discussed later in UE feature session. |
| 3-2 | Minimum applicable scheduling offset for 480/960 kHz SCS. Note: this issue is due to the newly agreed values from RAN2 on maxK0-SchedulingOffset-r17 and maxK2-SchedulingOffset-r17 for 480 kHz and 960 kHz SCS. | [50] | H | Huawei, HiSilicon: There are two proposals from [50]. The discussion should be focus on proposal #2 considering RAN2 already agreed on the proposal #1 FL: given RAN2 agreement states “Can come back to this in the next meeting if there are issues with RAN1 with these values”, it’s worth to confirm whether RAN1 has any issue on RAN2’s agreement. |
| 3-3 | Antenna port field when both DMRS mapping type A and B are indicated in the TDRA row of DCI. | [43] | H |  |
| 3-4 | PTRS-DMRS association field when both DMRS mapping type A and B are indicated in the TDRA row of DCI. | [43] | H | Huawei, HiSilicon: considering the PTRS-DMRS association field only indicate the relative sequence of scheduled DMRS instead of specific antenna port index, we think it is possible to unify the field for both mapping type by implementation. Thus, 3-4 is “N” from our perspective. Samsung: we are open to the conclusion such as “avoid by implementation”. Can discuss 3-3 and 3-4 jointly. FL: worth discussion to see if this issue requires specification change or can be avoid by implementation. |

Table 4 – Scheduling and HARQ

| Issue# | Issue | References | FL initial assessment | Company inputs (if any) |
| --- | --- | --- | --- | --- |
| 4-1 | Clarification on type-1 HARQ CB that time domain bundling is applied across PDSCHs scheduled by “the same DCI” | [34], [41] | H | Fujitsu: To FL, we have one question about the discussion plan. We understand 4-1 is about at least Issue 1 in our contribution. Is it the plan to also discuss Issue 2 and Issue 3 in our contribution under 4-1 here? Or should separate items for Issue 2 and Issue 3 be added? Thanks. Intel: Agree with FL. We think should be no confusion on the limitation of ‘same DCI’, but fine for discussion FL note: To Fujitsu, from my understanding, for Issue 3 in Fujitsu’s contribution, it is tied with Issue#4-2 (OOO), and for Issue 2 in Fujitsu’s contribution, I think UE does not receive SPS PDSCH overlapped with scheduled PDSCH. But if a clarification for Issue 2 is needed, we can include this issue in Issue#4-1. Nokia: N, in our view the RAN1#107bis-e agreement is clear. |
| 4-2 | DCI-to-PDSCH OOO (case 5) and timeline of NNK1 (case 6) are based on configured SLIV vs. valid SLIV. | [34], [35], [36], [39], [41], [44], [45], [47], [48], [49], [50], [51], [52], [53] | H |  |
| 4-3 | Type-1 HARQ CB for multiple PDSCHs scheduled by single DCI 1-1 and slot-aggregated PDSCH scheduled by DCI 1-2 | [34], [38], [43], [45], [49] | H |  |
| 4-4 | Confirm working assumption that type-2 CB is generated based on “configured SLIV” | [35], [47], [49] | H | Huawei, HiSilicon: suggest to change to “N” because the TP for the WA is already captured. It may not need take time to confirm |
| 4-5 | Collision handling between PUSCH and CORESET#0 | [37], [38], [47], [52] | N | Intel: This was discussed in many meetings, but was not concluded. Suggest to conclude in this meeting. CATT: agree with intel |
| 4-6 | Application of PDSCH-AggregationFactor /PUSCH-AggregationFactor for DCI format 1_1/0_1 “with CS-RNTI” FL note: As per previous agreement, PDSCH-AggregationFactor/PUSCH-AggregationFactor is not applied for DCI format 1_1/0_1 “with CS-RNTI” | [38], [41], [43], [46] | N |  |
| 4-7 | Clarification on UL DAI Indication in multi-PUSCH scheduling DCI | [38] | H | Intel: We do not think this is necessary. Current spec should be sufficient. Ericsson: Agree with Intel Samsung: Agree with Intel and Ericsson |
| 4-8 | TDRA information for a DCI indicating SCell dormancy without scheduling PDSCH | [39], [45], [48], [50] | H | Samsung: It was discussed in the last meeting. Nothing broken in the specification without this proposal. For maintenance phase, focus on an essential correction. CATT: agree with samsung |
| 4-9 | Clarification on type-2 CB generation for a disabled TB when time domain bundling is configured | [39] TP1 | H | Huawei, HiSilicon: We are fine to clarify the issue but do not see the necessity of the TP1. CATT: not necessary |
| 4-10 | Clarification on type-2 CB generation when both of spatial bundling and time domain bundling are configured | [39] | H |  |
| 4-11 | Introduce an independent nHARQ-ACK formula for type-2 HARQ CB when time domain is not configured but multi-PDSCH scheduling DCI is configured | [39] TP2 | N | CATT: It’s better to modify vivo: Suggest to discuss since the formula is not clear in spec |
| 4-12 | Clarification on nHARQ-ACK,TB when it is used for the first sub-codebook out of two sub-codebooks | [39] TP3 | E |  |
| 4-13 | Reflect the agreement that priority indicator indicated in a multi-PXSCH scheduling DCI is applied to all of scheduled PXSCHs | [40] | E |  |
| 4-14 | Clarification on HARQ ID skipping for invalidated PDSCH scheduled by multi-PDSCH scheduling DCI with ‘tdmSchemeA’ for single DCI based multi-TRP mechanism | [41] | N | Fujitsu: According to the specification as below, we see two possible interpretations for the case of multi-PDSCH scheduling with ‘tdmSchemeA’ - Interpretation 1: If at least one of the repetitions of a PDSCH collides with semi-static UL symbols, the PDSCH (i.e., both repetitions) is not allocated with HARQ process ID. - Interpretation 2: If the 1st repetition of a PDSCH collides with semi-static UL symbols, the PDSCH (i.e., both repetitions) is not allocated with HARQ process ID. HARQ process ID is not incremented for PDSCH(s) not received if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided.. As the proponent, if there is a common understanding among companies, we are okey to not discuss it in the next phase. But hopefully, FL or other companies can help to clarify the correct interpretation. Thanks. FL note: My understanding is Interpretation 1. |
| 4-15 | Relationship between COT duration and PDSCHs scheduled by a single DCI | [42], [47] | N |  |
| 4-16 | Restriction on the number of PDSCH receptions/PUSCH transmissions in a slot | [43] | N | Intel: Fine to make a conclusion on the proposal Samsung: clarification is needed as a proponent ZTE: we think that it is necessary to further clarify whether such restriction is also applied for CG-PUSCH/SPS PDSCH. |
| 4-17 | Combining HARQ-disabling feature with multi-PDSCH scheduling | [45] | N |  |
| 4-18 | Reflect the agreement that UE does not apply pusch-AggregationFactor to DCI format 0_1 if multi-PUSCH scheduling DCI is configured | [46] | E |  |
| 4-19 | Clarification on UE behavior if multi-PXSCH scheduling DCI indicates BWP switching | [47] | N |  |
| 4-20 | Applicability of time domain bundling for type-3 CB | [52] | N | Intel: We prefer to clarify the behavior on Type-3 CB generation |
| 4-21 | Clarification on the applicability of K1 set extension for K1 values for DCI 1_0 | [52] | N | Intel: As we analyze in our tdoc, the current spec is confusing on the utilization of K1 or extended K1. Suggest to discuss it CATT: agree with intel |
| 4-22 | If only one PDSCH is valid among PDSCHs scheduled by a DCI, HARQ-ACK bit for that PDSCH belongs to the first sub-codebook | [53] | N |  |
| 4-23 | Clarification of PDSCH mapping type of PDSCHs scheduled by multi-PDSCH scheduling DCI with ‘tdmSchemeA’ for single DCI based multi-TRP mechanism | [53] | N |  |
| 4-X | Application of TCI states within the span of multi-PDSCH | [34] | Treated in 8.2.5? | Intel: suggest treating together with 7-2 as it is the same issue. This should be “H” Huawei, HiSilicon: fine to treat in 8.2.5 if there email thread allocated. Nokia: Agree with FL initial assessment CATT: prefer no change |
| 4-Y | Channel access type indication for multiple PUSCHs in single DCI | [42] | Treated in 8.2.4? | Intel: Agree with FL. Nokia: Agree with FL initial assessment |

## Issues for agenda item “8.2.4	Channel access mechanism”

Table 5 - Channel access mechanism

| Issue# | Issue | References | FL initial assessment | Company inputs (if any) |
| --- | --- | --- | --- | --- |
| 5-1 | ED Threshold when LBT Bandwidth is larger than Active BWP, Upper limit on EDT Threshold | [71], [75], [56], [59], [63] | H |  |
| 5-2 | UL Contention Exempt Short Control Signaling: Duty Cycle Constraint | [72], [73], [75], [71], [55], [56], [58], [59], [62], [64], [66] | H |  |
| 5-3 | UL Contention Exempt Short Control Signaling: Signaling for Enabling CET for msg1/msgA together | [73], [75], [71], [55], [56], [59], [60], [65], [66] | H | Huawei, HiSilicon: Suggest to change it to N. It has been previously discussed in several meetings without consensus. |
| 5-4 | Multi-Beam Channel Access: Independent per beam sensing and LBT Procedure for UE Initiated COT | [71], [75], [63], [69] | H | Ericsson: suggest changing it to N. This can be left to implementation. |
| 5-5 | Multi-Beam Channel Access: Independent per beam sensing and LBT Procedure for UE Initiated COT: COT on a Subset of Beams | [71], [75], [56], [57], [69] | H | Ericsson: suggest changing it to N. This can be left to implementation. No regulations mandate any of these mechanisms. |
| 5-6 | Multi-Beam Channel Access: ED Threshold for independent per beam sensing | [71], [73], [75] [54], [55], [56], [59], [69] | H | Ericsson: suggest changing it to N. This can be left to implementation. There is only one EDT for a single channel access attempt with multiple beams according to the regulations. |
| 5-7 | LBT Upgrade in COT Sharing: RRC Configuration for Channel Access Type Change for UE from Type 1 to Type 2 or Type 3 LBT | [71], [75], [56], [58], [59], [60], [63], [64], [65], [66], [70] | H | Huawei, HiSilicon: Suggest to change it to N. It has been previously discussed in several meetings without consensus. Ericsson: suggest to change it to N. |
| 5-8 | COT resumption after a gap: RRC Configuration of Channel Access Type for resuming a UE initiated COT after a gap | [71], [75], [54], [57], [58], [60], [63], [64], [65] | H | Huawei, HiSilicon: Suggest to change it to N. It has been previously discussed in several meetings without consensus. Ericsson: suggest to change it to N. This can be left to implementation |
| 5-9 | Channel Access Indication within Fall-Back DCI | [55], [56], [58], [59], [63], [64], [65], [66], [70] | H | DCM: We believe this is one of the highest priority issue. Ericsson: Agree with DCM. |
| 5-10 | Cyclic prefix extension for CG UL transmissions | [73], [63], [70] | N | Intel: agree with the feature lead, and we are OK to conclude it. |
| 5-11 | UL To DL COT Sharing, clarification of gNB side LBT | [73], [64] | N | Intel: from our point of view the spec is still unclear on how the UL-to-DL COT sharing would be perform for CG UE, and it is not only a matter of clarifying the gNB’s side LBT but rather the UE’s behavior and the CG-UCI content. |
| 5-12 | UE Channel Access Type behavior before reporting of LBT Capability | [71], [55], [59], [70] | H |  |
| 5-13 | Clarification on UE Assumption on LBT mode at the gNB for the gNB-UE connection | [54], [59], [62] | H | Huawei, HiSilicon: Suggest to change it to N. It has been discussed for several meetings without consensus on whether the UE needs to make an assumption about the LBT mode of the gNB. Ericsson: suggest changing it to N. |
| 5-14 | SIB 1 indication of whether LBT is required for all UL Transmissions | [54], [58], [64] | H | Huawei, HiSilicon: Suggest to change it to N. Additional indication in SIB-1 has been discussed in the previous two meetings without consensus. Ericsson: suggest keeping it H. We have a proposal in our contribution to deal with only RACH and CG transmissions which could be a compromise. |
| 5-15 | Non-Fallback DCI : Extend the use of ChannelAccess-CPext-(CAPC) field to two other Non-Fallback DCI formats, namely 0_2 and 1_2 | [55] | N | Intel: agree with the feature lead, as this is not essential at this point. Nokia: this is non-essential, but a useful feature to have |
| 5-16 | Clarification on UE behavior when fallback DCI indicating Type 2 LBT when the UE does not have the capability | [63], [71], [55] | H | LGE: Since Proposal #9 in our contribution relates to this issue, we have added our document number to the reference. Huawei, HiSilicon: We have added our contribution ref [55] based on our Proposal 10 Nokia: in our view this is not needed, but this obviously depends on 5-9 |
| 5-17 | Beam Selection for consecutive PUSCH transmissions when CG-PUSCH and DG-PUSCH are multiplexed | [74] | N | Intel: agree with the feature lead, as this is not essential at this point. |
| 5-18 | RAN2 Correction for Value ranges for cg-COT-Sharing-r17 and cg-COT-SharingList-r17 | [55] | E | Huawei, HiSilicon: Since such a correction is not for related to any RAN1 TS, it is not clear to us how it would be applied to RAN2 CR on 38.331 if categorized as “E”. If applying the correction requires sending an LS to RAN2 as proposed in our contribution, we believe the issue should be categorized as “H” for RAN1 to agree on sending the LS to RAN2, and also due to its RRC impact. |
| 5-19 | Clarification on TCI state for inter-frequency RSSI measurements | [55] | H | Ericsson: suggest changing it to N. The status quo method must be allowed for now. Explicit TCI state can be studied if needed. |
| 5-20 | Alignment of RAN2 parameter ChannelAccessMode2-r17 with 37.213 | [55] | E |  |
| 5-21 | Editorial: Channel Access Procedure definition in Section 4.0 of 37.213 | [55] | E | Huawei, HiSilicon: We note that at least the proposed editorials on transmission bursts in FR2-2 would be required for the discussion on Issue#5-28 |
| 5-22 | Default Channel Access type in absence of channelAccessMode2-r17 | [56] | N | ZTE: we are fine to make a conclusion on the issue. |
| 5-23 | Beam Specific COT-SI, CO and SSGS for DCI 2_0 | [56], [57], [59], [62], [64], [66], [68] | N | Intel: we are OK to conclude it. ZTE: we are fine to make a conclusion on the issue. |
| 5-24 | RAN2 : Per Beam LBT failure indication under directional LBT | [56] | N | ZTE: we are fine to make a conclusion on the issue. |
| 5-25 | Editorial: Misalignment of higher-layer parameter name between TS 37.213 and TS 38.331. | [56] | E |  |
| 5-26 | Editorial: Misalignment of higher-layer parameter name between TS 38.212 and TS 38.331. | [56] | E |  |
| 5-27 | Editorial: Misalignment of higher-layer parameter name between TS 38.213 and TS 38.331 | [56] | E |  |
| 5-28 | Clarification on Channel access type indication for multiple PUSCHs in single DCI | [61], [64] | H |  |
| 5-29 | Clarification on Channel access Type determination when UE receives multiple channel access type indications | [61], [62], [68] | H | Huawei, HiSilicon: Suggest to focus the discussion on whether or not the dynamically indicated channel access type can be misaligned with the semi-statically indicated channelAccessMode2-r17. This is because for UE receiving multiple channelAccessMode2-r17 indications (cell-specific and UE-specific), RAN2 running CR on 38.331 in R2-2204126 already captures that the UE-specific indication overwrites the cell-specific ones. |
| 5-30 | Multi-Channel channel access clarification | [63] | N |  |
| 5-31 | Clarification on ED Threshold in COT Sharing | [63] | N | Intel See comments on 5-11, so generally our view is that this issue should be still discussed. Huawei, HiSilicon: Suggest to change it to H. The current spec in 37.213 and 38.212 do not capture the following conclusion and agreement in RAN1 as well as the new parameters (duration-r17, offset-r17) introduced in RAN2 running CR on 38.331 in R2-2204126. Conclusion UL to DL COT sharing is supported for FR2-2 unlicensed operation, including from dynamically scheduled UL and CG-PUSCH. Agreement For CG-PUSCH to DL COT sharing, extend the duration and offset range to {1, …, 319}. |
| 5-32 | Rx Assistance via PDCCH and PUCCH | [63] | N |  |
| 5-33 | Clarification/Editorial in 38.214 regarding use of beamCorrespondenceWithoutUL-BeamSweeping when used for directional sensing | [64] | E |  |
| 5-34 | LBT Downgrade from Type 2 to Type 1 on failure of Type 2 LBT | [67] | N |  |
| 5-35 | Misc. editorials of 37.213 from [73] | [73] | E | Intel: Since our proposal 12 was missing, we added it here, and would be good to capture it as editorial. |

## Issues for agenda item “8.2.5	Others”

Table 6 - Enhancements for PUCCH formats 0/1/4

| Issue# | Issue | References | FL initial assessment | Company inputs (if any) |
| --- | --- | --- | --- | --- |
| 6-1 | Determination of number of RBs for PF4 In [83] it is observed that 38.212 Section 6.3.1.4 contains the following sentence: , and NPRBPUCCH,4 are the number of PRBs that are determined by the UE for PUCCH formats 2/3/4 transmission respectively according to Clause 9.2 of [5, TS38.213] In [83], it is claimed that "there is no description in Clause 9.2 of TS 38.213 [2] to determine the actual number of PRBs used for enhanced PUCCH format 4" To the contrary, the FL observes that 38.213 Clause 9.2.1 does indeed specify the number of PRBs for enhanced PUCCH format 4 according to the following paragraph: If the format indicates PUCCH-format4, the PUCCH format configured for a PUCCH resource is PUCCH format 4, where the PUCCH resource also includes a number of symbols for a PUCCH transmission provided by nrofSymbols, an orthogonal cover code length by occ-Length, an orthogonal cover code index by occ-Index, and a first symbol for the PUCCH transmission provided by startingSymbolIndex. For PUCCH transmission in FR2-2, the PUCCH resource can also include a number of PRBs MRBPUCCH,4 provided by nrofPRBs; otherwise, MRBPUCCH,4=1. Hence, the FL's assessment is that it is not necessary to discuss this issue. | [83] | N | Huawei, HiSilicon: As the proponent company, we think the clarification provided in our TP is required. Please note a similar text to the second (lower) yellow text from 9.2.1 of 38.213 for PUCCH Fromat 4 that is mentioned by our FL is also provided for PUCCH Format 2 and 3 in clause 9.2.1 of 38.213: “If the format indicates PUCCH-format2 or PUCCH-format3, the PUCCH format configured for a PUCCH resource is PUCCH format 2 or PUCCH format 3, respectively, where the PUCCH resource also includes a number of PRBs provided by nrofPRBs,” This, however, does not mean that the actual PRBs for PUCCH transmission when PUCCH Format 2 or 3 is used is determined by nrofPRBs. See, for instance, the following text from clause 9.2.3: “If a UE transmits a PUCCH with OACK HARQ-ACK information bits and OCRC bits using PUCCH format 2 or PUCCH format 3 in a PUCCH resource that includes MRBPUCCH PRBs, the UE determines a number of PRBs MRB,minPUCCH for the PUCCH transmission to be the minimum number of PRBs, that is smaller than or equal to a number of PRBs MRBPUCCH provided respectively by nrofPRBs of PUCCH-format2 or nrofPRBs of PUCCH-format3 and start from the first PRB from the number of PRBs, that results to OACK+OCRC≤MRB,minPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r and, if MRBPUCCH>1, OACK+OCRC>MRB,minPUCCH-1⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r, where Nsc,ctrlRB, Nsymb-UCIPUCCH, Qm, and r are defined in clause 9.2.5.2. For PUCCH format 3, if MRB,minPUCCH is not equal 2α2⋅3α3⋅5α5 according to [4, TS 38.211], MRB,minPUCCH is increased to the nearest allowed value of nrofPRBs [12, TS 38.331]. If OACK+OCRC>MRBPUCCH-1⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r, the UE transmits the PUCCH over MRBPUCCH PRBs.” A similar clarification is not provided for PUCCH Format 4 in the specifications. Our TP clarifies that, unlike PUCCH format 2 and 3, the actual number of PRBs used for PUCCH Format 4 is the same as the number of Configured PRBs. In addition, according to TS38.212, the number of PRBs is determined by the UE for PUCCH format 4 according to Clause 9.2 of [5, TS38.213], which is UE’s behavior. However, the paragraph mentioned by our FL just describe the configuration for PUCCH format 4, which is gNB’s behavior. The UE behavior should be clarified in TS38.213 for PUCCH format 4 that the actual number of PRBs determined by the UE is equal to the configured number of PRBs. Moderator: Despite the explanation from the proponent, the FL's assessment is that the TP is still not needed. The current text for PUCCH format 2 and 3 describes a procedure for adjusting the actual number of PRBs based on the PUCCH payload. It is clearly stated that this applies only to PUCCH formats 2 and 3. Clearly for single RB PUCCH format 4 there is no legacy spec text about adjustment (since there is only 1 PRB). Now with multiple RBs, if there is no defined procedure for setting the actual number of RBs, then clearly there can be no other interpretation besides actual = configured, and it is already specified that the number of RBs is the configured value. |

Table 7 - Beam management for new SCSs

| Issue# | Issue | References | FL initial assessment | Company inputs (if any) |
| --- | --- | --- | --- | --- |
| 7-1 | Introduction of beam switching gap | [76], [80], [82], [83] | H | Huawei, HiSilicon: Added [83] to the references column. Suggest to wait for final decision of RAN4 regarding beam switching time. [76], [80], [82] propose to define UE capability for beam switch time. This has been discussed for multiple meetings already without agreement. We don’t think RAN1 should define a UE capability and decide on beam switching time values while RAN4 has a parallel discussion on UE beam switching time. If this has to be discussed in RAN1, RAN1 should make decision on the tentatively-agreed value of [200ns] in RAN4 which requires 1 symbol gap for both 480 and 960 kHz SCS (no need for UE capability). Nokia: N |
| 7-2 | Whether to update the applied TCI states within the span of multi-PDSCH | [76], [77], [78], [79], [80], [82], [34] | H | Qualcomm: We believe this is not essential. The current spec is clear already Ericsson: Agree with Qualcomm that no spec change is needed. Current spec already allows update within time span (see observations in our Tdoc) Huawei, HiSilicon: added [34] to the reference column. We submitted it to AI8.2.3. ZTE: Our 1st preference on this issue is to be handled by Rel-17 FeMIMO team together with the issue of Multi-slot scheduling + unified TCI framework. But we are open to discuss this issue in this agenda if majority agree. CATT: Agree with QC |
| 7-3 | Minimum guard period Y between two SRS resources of an SRS resource set for antenna switching | [77] | N (essential but RAN4 should discuss the issue based on the RAN1 LS) | Qualcomm: Prefer to wait for RAN4 response Nokia: Agree with FL assessment and comment regarding RAN4 Ericsson: Agree with Qualcomm; RAN1 cannot make progress until there is feedback from RAN4. ZTE: Agree with Qualcomm and Ericsson. CATT: Agree with QC |

Table 8 – Other issues

| Issue# | Issue | References | FL initial assessment | Company inputs (if any) |
| --- | --- | --- | --- | --- |
| 8-1 | Scaling of searchSpaceSwitchDelay for 480kHz and 960kHz | [81] |  | Nokia: The maximum range of the searchSpaceSwitchDelay IE is lower than the minimum value for Pswitch. To make signalling meaning full this should be addressed either under ag. 8.2 or 8.7. |

## Conclusion

Based on the responses from participating companies during the preparation phase, the moderator has the following observations:

- For Table 1 (initial access aspect): Most companies are fine with FL’s initial accessments

  - 1-3 is to confirm WA, which may not be needed given it is already captured

- For Table 2 (PDCCH monitoring enhancements): Most companies are fine with FL’s initial accessments

  - A few companies also propose to discussion 2-7 and 2-9 (currently N) together with 2-5 (currently H)

  - 2-10 can be categorized as “E”

  - 2-11 is mostly confirming WA, which may not be needed, given it is already captured

  - 2-16/17/18/19 can be discussed in AI 8.2.4

- For Table 3 (PDSCH/PUSCH enhancements): Most companies are fine with FL’s initial accessments

- For Table 4 (Scheduling and HARQ): Most companies are fine with FL’s initial accessments

  - 4-5: A few companies suggest to discuss this as well to conclude it

  - 4-7/8/9: Multiple companies believe these are “N”

  - 4-16: Multiple companies prefer to discuss this to conclude

  - 4-21: Multiple companies prefer to discuss to clarify

  - 4-X: Can discuss in 8.2.5 with Table 7

  - 4-Y: Discuss in 8.2.4

- For Table 5 (Channel access): Most companies are fine with FL’s initial accessments

  - HW proposes to change multiple items to N consider they have been discussed without conclusion. However the FL feels we should conclude one way or another for these. The spec may not be complete without these conclusions.

  - 5-11. Intel believes some clarification is still needed for both gNB and UE behavior for CG-PUSCH to gNB COT sharing

  - 5-31. There are proposals to change this to “H”

- For Table 6 (Enhancements to PUCCH formats 0/1/4): Most companies are fine with FL’s initial accessments

- For Table 7 (BM for new SCSs): Most companies are fine with FL’s initial accessments

  - 7-2. Multiple companies believe this is “N” as the spec is already clear enough.

- For Table 8 (others)

  - 8-1. Nokia brought up the issue searchSpaceSwitchDelay value range should be increase for high SCS as well. This can be discussed in 8.2.2

The Chair determined the following email threads for discussion:

[109-e-R17-FR2-2-01] Email discussion under 8.2.1 for maintenance on initial access aspects, for issues 1-1, 1-4, 1-5 and 1-6 in R1-2205124 – Daewon (Intel)

- 1st check point: May 13 (any RRC impact by May 12)

- Final check point: May 18

[109-e-R17-FR2-2-02] Email discussion under 8.2.2 for maintenance on PDCCH monitoring enhancements, for issues 2-1/2-8, 2-4, 2-5/2-7/2-9, 2-10, 2-12, 2-13, 2-15 (RRC) and issue 8-1 in R1-2205124 – Alex (Lenovo)

- 1st check point: May 13 (any RRC impact by May 12)

- Final check point: May 18

[109-e-R17-FR2-2-03] Email discussion under 8.2.3 for maintenance on RS and timeline, for issues 3-2, 3-3 and 3-4 in R1-2205124 – Huaming (vivo)

- 1st check point: May 13 (any RRC impact by May 12)

- Final check point: May 18

[109-e-R17-FR2-2-04] Email discussion under 8.2.3 for maintenance on scheduling and HARQ, for issues 4-1, 4-2, 4-3, 4-5, 4-10, 4-11, 4-12, 4-13, 4-16, 4-18, 4-21 in R1-2205124 – Seonwook (LGE)

- 1st check point: May 13 (any RRC impact by May 12)

- Final check point: May 18

[109-e-R17-FR2-2-05] Email discussion under 8.2.4 for maintenance on channel access mechanism – Jing (Qualcomm)

- Issues 5-1, 5-2, 5-3, 5-4, 5-5, 5-6, 5-7, 5-8, 5-9/5-16, 5-11, 5-12, 5-13, 5-14, 5-18, 5-19, 5-20, 5-21, 5-25, 5-26, 5-27, 5-28, 5-29, 5-31, 5-33, 5-35, as well as issues 2-16/17/18/19 and issue 4-Y in R1-2205124

- Aim to conclude on whether issues are essential or not by May 13

- Including discussion on LS in R1-2203027 if needed

- 1st check point: May 13 (any RRC impact by May 12)

- Final check point: May 18

[109-e-R17-FR2-2-06] Email discussion for maintenance on whether there is ambiguity on the determination of number of RBs for PUCCH Format 4, and if so any necessary correction, until May 13 – Steve (Ericsson)

# References

Contributions submitted to 8.2.1

[1]. R1-2203079, Remaining issue of initial access signals and channels for 52-71GHz spectrum, Huawei, HiSilicon, SIA

[2]. R1-2203290, Remaining issues on initial access aspects for 52.6 to 71GHz, ZTE, Sanechips

[3]. R1-2203369, Remaining issues for initial access operation in 52.6-71GHz, InterDigital, Inc.

[4]. R1-2203430, Remaining issues on Initial access aspects for up to 71GHz operation, CATT

[5]. R1-2203508, Maintenance on initial access for NR operation from 52.6GHz to 71GHz, vivo

[6]. R1-2203858, Maintenance on initial access aspects for NR from 52.6 GHz to 71 GHz, Samsung

[7]. R1-2203986, Discussion on remaining issue for initial access aspects, OPPO

[8]. R1-2204110, Initial Access Aspects, Ericsson

[9]. R1-2204201, Remaining issues for initial access aspects, Apple

[10]. R1-2204338, Remaining issues on initial access aspects for NR in FR2-2, NTT DOCOMO, INC.

[11]. R1-2204599, Initial access aspects, Nokia, Nokia Shanghai Bell

[12]. R1-2204611, Remaining issues of initial access aspects to support NR above 52.6 GHz, LG Electronics

[13]. R1-2204766, Discussion on initial access aspects for extending NR up to 71 GHz, Intel Corporation

Contributions submitted to 8.2.2

[14]. R1-2203080, Remaining issues of PDCCH monitoring enhancement for 52-71GHz spectrum, Huawei, HiSilicon, SIA

[15]. R1-2203291, Remaining issues on PDCCH monitoring enhancements for 52.6 to 71GHz, ZTE, Sanechips

[16]. R1-2203370, Remaining issues for PDCCH monitoring enhancements, InterDigital, Inc.

[17]. R1-2203431, Remaining issues on PDCCH monitoring enhancements for up to 71GHz operation, CATT

[18]. R1-2203509, Maintenance on PDCCH monitoring enhancements for NR operation from 52.6GHz to 71GHz, vivo

[19]. R1-2203859, Maintenance on PDCCH monitoring enhancements for NR from 52.6 GHz to 71 GHz, Samsung

[20]. R1-2203987, Discussion on remaining issue for PDCCH monitoring enhancement, OPPO

[21]. R1-2204075, Remaining issues on PDCCH monitoring , Panasonic

[22]. R1-2204111, PDCCH Monitoring Enhancements, Ericsson

[23]. R1-2204202, On remaining issues for PDCCH Monitoring, Apple

[24]. R1-2204339, Remaining issues on PDCCH monitoring enhancements for NR in FR2-2, NTT DOCOMO, INC.

[25]. R1-2204514, PDCCH monitoring enhancements, Sharp

[26]. R1-2204566, Remaining Issues on PDCCH Monitoring Enhancements in FR2-2, TCL Communication

[27]. R1-2204578, Remaining issues of PDCCH monitoring enhancements for above 52.6GHz, Transsion Holdings

[28]. R1-2204600, Remaining issues on PDCCH monitoring enhancements, Nokia, Nokia Shanghai Bell

[29]. R1-2204612, Remaining issues of PDCCH monitoring enhancements to support NR above 52.6 GHz, LG Electronics

[30]. R1-2204706, Remaining discussion on PDCCH monitoring enhancement for 52.6-71 GHz NR operation, MediaTek Inc.

[31]. R1-2204767, Discussion on PDCCH monitoring enhancements for extending NR up to 71 GHz, Intel Corporation

[32]. R1-2204825, Remaining issues on PDCCH for NR from 52.6 GHz to 71GHz, Lenovo

[33]. R1-2204979, PDCCH monitoring enhancements for NR in 52p6 to 71GHz band, Qualcomm Incorporated

Contributions submitted to 8.2.3

[34]. R1-2203081, Remaining issues of PDSCH/PUSCH enhancement for 52-71GHz spectrum, Huawei, HiSilicon, SIA

[35]. R1-2203292, Remaining issues on data channel enhancements for 52.6 to 71GHz, ZTE, Sanechips

[36]. R1-2203371, Remaining issues for PDSCH/PUSCH enhancements to supporting 52.6-71 GHz band in NR, InterDigital, Inc.

[37]. R1-2203401, Discussion on PDSCH/PUSCH enhancements for NR 52.6-71 GHz, Panasonic

[38]. R1-2203432, Remaining issues on PDSCH/PUSCH enhancements for up to 71GHz operation, CATT

[39]. R1-2203510, Maintenance on PDSCH/PUSCH enhancements for NR operation from 52.6GHz to 71GHz, vivo

[40]. R1-2203678, Remaining issues of PDSCH/PUSCH enhancements for 52.6 to 71GHz, NEC

[41]. R1-2203708, Remaining issues of multi-PDSCH/PUSCH scheduling via a single DCI, Fujitsu Limited

[42]. R1-2203784, Remaining issues on PDSCH and PUSCH enhancements  for NR 52.6-71GHz, xiaomi

[43]. R1-2203860, Maintenance on PDSCH/PUSCH enhancements for NR from 52.6 GHz to 71 GHz, Samsung

[44]. R1-2203988, Discussion on remaining issue for PDSCH/PUSCH enhancements, OPPO

[45]. R1-2204112, PDSCH-PUSCH Enhancements, Ericsson

[46]. R1-2204190, Discussion on multi-PXSCH scheduling, ASUSTeK

[47]. R1-2204203, On remaining issues for PDSCH PUSCH Enhancements, Apple

[48]. R1-2204340, Remaining issues on PDSCH/PUSCH enhancements for NR in FR2-2, NTT DOCOMO, INC.

[49]. R1-2204601, Remaining issues on PDSCH/PUSCH enhancements, Nokia, Nokia Shanghai Bell

[50]. R1-2204613, Remaining issues of PDSCH/PUSCH enhancements to support NR above 52.6 GHz, LG Electronics

[51]. R1-2204707, Remaining discussion on multi-PDSCH scheduling design for 52.6-71 GHz NR operation, MediaTek Inc.

[52]. R1-2204768, Discussion on PDSCH/PUSCH enhancements for extending NR up to 71 GHz, Intel Corporation

[53]. R1-2204980, PDSCH and PUSCH enhancements, Qualcomm Incorporated

Contributions submitted to 8.2.4

[54]. R1-2203056, Remaining Details in Channel access for Beyond 52.6 GHz, FUTUREWEI

[55]. R1-2203082, Remaining issues of channel access mechanism for 60 GHz unlicensed operation, Huawei, HiSilicon, SIA

[56]. R1-2203293, Remaining issues on channel access for 52.6 to 71GHz, ZTE, Sanechips

[57]. R1-2203372, Remaining issues for channel access mechanisms, InterDigital, Inc.

[58]. R1-2203433, Remaining issues on channel access mechanism for up to 71GHz operation, CATT

[59]. R1-2203511, Maintenance on channel access mechanism for NR operation from 52.6GHz to 71 GHz, vivo

[60]. R1-2203679, Remaining issues on channel access mechanism supporting NR from 52.6 to 71 GHz, NEC

[61]. R1-2203785, Remaining issues on channel access mechanism for NR on 52.6-71 GHz, xiaomi

[62]. R1-2203861, Maintenance on channel access mechanism for NR from 52.6 GHz to 71 GHz, Samsung

[63]. R1-2203989, Discussion on remaining issue for channel access mechanism, OPPO

[64]. R1-2204113, Channel Access Mechanisms, Ericsson

[65]. R1-2204204, Remaining details on channel access mechanisms for unlicensed access above, Apple

[66]. R1-2204341, Remaining issues on Channel access mechanism for NR in FR2-2, NTT DOCOMO, INC.

[67]. R1-2204546, Remaining issue on channel access for NR from 52.6GHz to 71GHz, WILUS Inc.

[68]. R1-2204567, Remaining Issues on Channel Access for NR in 60GHz Unlicensed Band, TCL Communication

[69]. R1-2204579, Remaining issues of channel access mechanism for above 52.6GHz, Transsion Holdings

[70]. R1-2204602, Remaining issues on channel access mechanism, Nokia, Nokia Shanghai Bell

[71]. R1-2204614, Remaining issues of channel access mechanism to support NR above 52.6 GHz, LG Electronics

[72]. R1-2204636, Remaining issue on channel access scheme for above 52.6GHz, ASUSTeK

[73]. R1-2204769, Discussion on Channel Access Mechanism for extending NR up to 71 GHz, Intel Corporation

[74]. R1-2204826, Remaining issues on channel access for NR from 52.6 GHz to 71GHz, Lenovo

[75]. R1-2204981, Channel access enhancements, Qualcomm Incorporated

Contributions submitted to 8.2.5

[76]. R1-2203294, Remaining issues on beam management for 52.6 to 71GHz, ZTE, Sanechips

[77]. R1-2203373, Remaining issues for beam management for new SCSs, InterDigital, Inc.

[78]. R1-2203512, Maintenance on beam management, vivo

[79]. R1-2204114, Remaining issues for beam management, Ericsson

[80]. R1-2204342, Remaining issues on beam based operation for new SCSs for NR from 52.6 to 71 GHz, NTT DOCOMO, INC.

[81]. R1-2204603, Supported values for SSSG switching delay for 480 kHz and 960 kHz SCS, Nokia, Nokia Shanghai Bell

[82]. R1-2204615, Remaining issues of beam management to support NR above 52.6 GHz, LG Electronics

[83]. R1-2204896, Remaining issues of PUCCH enhancement and beam management enhancement for 52-71GHz spectrum, Huawei, HiSilicon, SIA