3GPP TSG RAN WG1 #109-e						      R1-2205126

e-Meeting, May 09th – May 20th, 2022

Agenda item:	8.1.2

Source: 	Moderator (Nokia)

Title: 	Description of issues for Rel.17 NR FeMIMO maintenance: Item 2a DL/UL

Document for:	Discussion and Decision

- Introduction

The moderator summary of the maintenance-related issues raised in the submitted contributions for Rel.17 NR_FeMIMO maintenance is given below.

An initial assessment on each of the issues is given (but can be revised based on the outcome of the discussion during the preparation week). The assessment will be used as a basis to select four issues (per chairman instruction) for further discussion in the upcoming weeks.

- High priority (H): this includes high-priority item (essential, pending issues, broken spec components) and proposed editorial changes that either enhance the clarity of the specs or correct mistakes

- Non-essential (N): this includes all other purposes such as spec optimization and low priority issues

- Editorial (E): this includes editorial issues that will be handled as editorial CRs (to be communicated to the editors/chairs) and thereby not counted toward the four-thread quota

- Maintenance issues – mTRP DL

The issues are summarized in the following table: (FL note 2 indicates whether the issue has been discussed before or not)

Table 1 Summary

| # | Issue (summary of CR proposal) | Companies | FL assessment | Company inputs (if any) |
| --- | --- | --- | --- | --- |
| 1 | Define limit (X) for linked PDCCH candidates also be applicable for intra-span case. FL note 1: Majority of companies do not support defining limits related to memory requirements for intra-span case based on the previous discussions. FL note 2: Issue has been discussed before. | Huawei / HiSilicon | N (N: 10, H: 4) | Apple: Support to discuss it Samsung: Support FL’s assessment. Langbo: Open to discuss OPPO: Agree with FL Huawei, HiSilicon: We support to discuss it. Because when inter-span and intra-span can be configured simultaneously, a unified solution and framework for PDCCH repetition for both intra-span and inter-span is preferred to practically reduce UE complexity. LGE: Agree with FL’s assessment. NTT DOCOMO: Agree with FL QC: We are open to discuss it. vivo: From memory requirement perspective, it seems there is no difference between intra and inter span. However, this issue is non-essential, since UE can perform individual decoding within intra-span if the memory capability is not enough sometimes ZTE: Agree with FL’s assessment. Lenovo: Agree with FL’s assessment. Ericsson: Agree with FL. CATT: Agree with FL’s assessment. Intel: Agree with FL assessment |
| 2 | Processing time relaxation for PDCCH repetition assuming soft combining. FL note 1: Majority of companies do not support it based on previous discussions. FL note 2: Issue has been discussed before multiple times. | Huawei / HiSilicon | N (N: 10, H: 4) | Apple: Support to discuss it Samsung: Support FL’s assessment, but also okay to discuss. Langbo: Open to discuss OPPO: Agree with FL Huawei, HiSilicon: We support to discuss it. LGE: Support to discuss it NTT DOCOMO: agree with FL QC: Agree with FL vivo: For PDCCH repetition reception, total number of BD capability is same as legacy even though UE performs two individual decoding and one soft bit combining decoding. It seems there is no need to enhance the processing time. If the addition operation of soft bits combining consumes some processing time, could the proponent provide more details about the processing in chipset to deduce how much time is needed? Is the extended processing time related to the length of soft bits or the size of AL? ZTE: Agree with FL’s assessment. Lenovo: Agree with FL’s assessment. Ericsson: Agree with FL CATT: Agree with FL Intel: Agree with FL assessment |
| 3 | PDCCH repetition configured with SSGS. FL note 1: The suggested TP is not consistent with the previous conclusion. Further discussions may be needed regarding the justification for additional agreements / conclusions. Suggest discussing to align the understanding on whether there is a serious issue with the existing specification / conclusion. FL note 2: The specific issue (missing DCI) has not been discussed before. | vivo | N (N: 12, H: 2) | Apple: This is non-essential. Samsung: It seems not essential. Langbo: OK to discuss OPPO: It is not preferred to revert previous conclusion in the maintenance stage if the spec is not broken. Thus, we don’t support to discuss it. Huawei, HiSilicon: if there’s no problem with previous conclusion, don’t prefer to change it. LGE: It seems not essential. NTT DOCOMO: Not essential. We think the previous conclusion has no problem QC: Not essential. vivo: Based on the current spec 38.213 and 38.214, UE determining whether the PDCCH repetition follows the Rel-17 rule only rely on the configuration of searchSpaceLinking in RRC, even though one of PDCCH candidate is not monitored e.g. due to overlap with UL symbols indicated by DCI format 2-0 to avoid the misunderstanding if UE miss the detection of DCI format 2-0. We suggest same assumption is applied to SSGS switching too, otherwise, determination of PDCCH repetition depending on the condition of UE decoding will cause serious negative impact due to potential ambiguity, since some scheduling timeline or reference candidate rule for channels/RSs of DL and UL is different between legacy PDCCH and PDCCH repetition mechanism in Rel-17, if misunderstanding is unresolved. ZTE: Non-essential. Lenovo: It seems not essential but open for discussion since this has not been discussed before. Ericsson: “N” CATT: It seems not essential. Intel: Not essential |
| 4 | For accumulated TPC, only one TPC command value of the two repetitions is applied for PUSCH power control. FL note 1: The existing general description in 38.213 captures that “a PDCCH reception by a UE includes two PDCCH candidates”. Additional clarifications seem unnecessary. FL note 2: The proposal has been discussed before (which is also common understanding) | NEC | N (N: 14, H: 0) | Apple: This seems to be non-essential. Samsung: Support FL’s assessment. Langbo: Agree with FL. OPPO: Agree with FL Huawei, HiSilicon: agree with FL. LGE: Agree with FL NTT DOCOMO: agree with FL QC: Agree with FL vivo: Agree with FL ZTE: Agree with FL’s assessment. Lenovo: Agree with FL’s assessment. Ericsson “N” CATT: Agree with FL Intel: Agree with FL |
| 5 | PTRS Resource Mapping for Linked PDCCH candidates. FL note 1: In 38.211, it is mentioned “a detected PDCCH according to clause 5.1.4.1 of [6, TS38.214]”, which covers both cases of legacy and PDCCH repetition. FL note 2: The specific TP related to PTRS has not been discussed before. | Langbo | N (N: 12, H: 2) | Apple: This is non-essential Samsung: Support FL’s assessment. Langbo: If excluding linked PDCCHs for PTRS mapping is the common understanding, clarifications or editorial changes are needed for 38.211 or 38.214. According to the latest TS 38.214, a detected PDCCH means the REs occupied by a detected PDCCH candidate, NOT covering PDCCH repetition (highlighted as below). *************Texts in current TS38.214******************** “If a PDSCH scheduled by a PDCCH would overlap with resources in the CORESET containing the PDCCH, the resources corresponding to a union of the detected PDCCH that scheduled the PDSCH and associated PDCCH DM-RS are not available for the PDSCH. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], the resources corresponding to a union of the two PDCCH candidates scheduling the PDSCH and the associated PDCCH DM-RS are not available for the PDSCH.” OPPO: Agree with FL Huawei, HiSilicon: Open to discuss whether any clarification is needed. LGE: Agree with FL NTT DOCOMO: agree with FL QC: Agree with FL vivo: This is non-essential since the definition of ‘REs not available for PDSCH’ has been clarified in clause 5.1.4.1 of 38.214 in case of the ambiguity between AL8 and AL16 or in case of PDCCH detection only in one of PDCCH repetition candidate. ZTE: Agree with FL’s assessment. Lenovo: Agree with FL’s assessment. Ericsson “N” CATT: Agree with FL Intel: Agree with FL |
| 6 | Restrict same precoderGranularity for two linked CORESETs. FL note 1: Justification on the need for adding this restriction is not clear. FL note 2: The specific proposed restriction has not been discussed before. | Langbo | N (N: 12, H: 1) | Apple: This is non-essential Samsung: Support FL’s assessment, but also okay to discuss. Langbo: Two aspects are discussed in our contribution whereas FL only captured one. The other aspect is that all DMRS REs of two linked CORESETs should be rate matched around by PDSCH. In current TS38.214, it says “When precoderGranularity configured in a CORESET where the PDCCH was detected is set to 'allContiguousRBs', the associated PDCCH DM-RS are DM-RS in all REGs of the CORESET. Otherwise, the associated DM-RS are the DM-RS in REGs of the PDCCH”. When two CORESETs are linked and precoderGranularity is set to 'allContiguousRBs', whether to exclude all DMRS REs of the two CORESETs or exclude DMRS REs of only one CORESET is still open. We think this is essential as precoderGranularity = 'allContiguousRBs' is a very typical use case and is very important for PDCCH robustness for which the PDCCH repetition also targets. OPPO: It is non-essential LGE: Support FL’s assessment NTT DOCOMO: Support FL’s assessment QC: It is non-essential. vivo: restriction is unnecessary. In our opinion, 'allContiguousRBs' can bring the benefit to channel estimation only in lower SNR but with higher RS overhead, while 'sameAsREG-bundle ' can bring the diversity of transmitter with cyclic precoding for different REG-bundle in higher SNR. It is natural that gNB can select corresponding configuration flexibly to match with respective link of each TRP. Furthermore, regarding the clarification of rate matching in case 'allContiguousRBs' is configured, maybe moving the sentence as shown below is clearer. If a PDSCH scheduled by a PDCCH would overlap with resources in the CORESET containing the PDCCH, the resources corresponding to a union of the detected PDCCH that scheduled the PDSCH and associated PDCCH DM-RS are not available for the PDSCH. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], the resources corresponding to a union of the two PDCCH candidates scheduling the PDSCH and the associated PDCCH DM-RS are not available for the PDSCH. When precoderGranularity configured in a CORESET where the PDCCH was detected is set to 'allContiguousRBs', the associated PDCCH DM-RS are DM-RS in all REGs of the CORESET. Otherwise, the associated DM-RS are the DM-RS in REGs of the PDCCH. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], the resources corresponding to a union of the two PDCCH candidates scheduling the PDSCH and the associated PDCCH DM-RS are not available for the PDSCH.. ZTE: Non-essential. Lenovo: Agree with FL’s assessment. Ericsson “N” CATT: It is non-essential. Intel: Not essential |
| 7 | Editorial TP for 38.211 on PDSCH scheduled by DCI format 1_0 in two linked common search spaces. FL note: This is editorial. While it can make the description more accurate, it is already implied from the previous sentence (legacy) as it says “in a common search space”. | Lenovo | E | Apple: OK to discuss Samsung: Support FL’s assessment. Langbo: Agree with FL OPPO: Agree with FL that the spec is clear Huawei, HiSilicon: We prefer to discuss it. In RAN1#106bis, it was agreed that type0/0A/1/2 CSS is not allowed for PDCCH repetition. From our understanding, if a CSS is configured for PDCCH repetition, it must be type3 CSS. So to be more clear, the change can be Type3-PDCCH common search space set. Agreement The following SS sets cannot be linked with another SS set for PDCCH repetition: SS set 0, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace. LGE: Support FL’s assessment NTT DOCOMO: agree with FL QC: Ok with E, but it is also ok to not discuss it. vivo: Fine with the update if majority want to make the spec clearer. ZTE: Agree with E. Lenovo: Agree with E with updating to make the spec. clearer. Ericsson “E” CATT: It is non-essential. It is already implied from the previous sentence. Intel: Minor issue, okay with E |

Observation – mTRP DL

From the inputs shared by participating companies during the preparation phase, the following observation can be made:

- The following issue can be handled as E (a part of editorial CR): Issue #7

- The following issues can be designated as N (non-essential): Issues #1, #2, #3, #4, #5, #6

- Maintenance issues – mTRP UL

The issues are summarized in the following table:

Table 1 Summary

| # | Issue (summary of CR proposal) | Companies | FL assessment | Company inputs (if any) |
| --- | --- | --- | --- | --- |
| 1 | For mTRP PUSCH, 38.214 is not clear on whether only one NZP CSI-RS resource can be configured for each resource set or the kind of resource set with usage 'nonCodebook'. – (R1-2203106) FL note 1: TP is proposed to 38.214 to clarify the issue and FL views that this is an editorial issue. | HW/HiSi | E | QC: Ok with E. SS: Agree with FL. LGE: Agree with FL’s assessment. NTT DOCOMO: Agree with FL. OPPO: Ok to discuss ZTE: Agree with E. Apple: Agree with FL vivo: Agree with FL. Lenovo: Agree with FL. CATT: Agree with FL’s assessment. Intel: Agree with FL Ericsson: ok with FL’s assessment. |
| 2 | For mTRP PUSCH, if SRS resource set indicator field in DCI format 0_2 is present, when it equals to “01”, the SRS resource set corresponding to the SRS resource indicator field in DCI format 0_2 is composed of the first NSRS, 0_2SRS resources in the SRS resource set with higher srs-ResourceSetId in srs-ResourceSetToAddModList, and when SRS resource set indicator field in DCI format 0_2 equals to“00” or “10” or “11”, the SRS resource set corresponding to the SRS resource indicator field in DCI format 0_2 is composed of the first NSRS, 0_2SRS resources in the SRS resource set with lower srs-ResourceSetId in srs-ResourceSetToAddModList. – (R1-2203422) FL note 1: Issue seems valid, and a TP is proposed to 38.212 to correct the issue. FL views that further discussion may be needed on this. | CATT | H (N :1) | QC: Ok to discuss. SS: Issue is valid. Ok to discuss. ASUS: Ok to discuss LGE: Agree with FL’s assessment. NTT DOCOMO: OK to discuss OPPO: Ok to discuss ZTE: Agree with H. Apple: Agree with FL vivo: We fail to see the necessity to discuss this issue. Current specification is clear. When two SRS resource sets are configured for DCI format 0_2, the SRS resource set associated with SRS resource indicator is indicated by SRS resource set indicator, and the configuration condition of the SRS resource set is explicitly elaborated in the indication of SRS resource set indicator as given in Table 7.3.1.1.2-36. The sub-bullets only apply to “otherwise”. Therefore, it seems redundant to add repeated description. Lenovo: Agree with FL. CATT: Agree with FL’s assessment. Intel: OK to discuss Ericsson: Ok to discuss |
| 3 | For mTRP PUSCH, how to determine the association between PTRS port(s) and DMRS port(s) according to the PTRS-DMRS association field in DCI format 0_1/0_2 for the scenario that the SRS resource set indicator field is present and equals to "00" or "01" and maxRank=2, and the scenario that the SRS resource set indicator field is absent are not captured. – (R1-2203422, R1- 2204170) FL note 1: This issue has been discussed before. The current spec seems ok. | CATT, Lenovo | N (N : 7) | QC: Agree with FL. The issue was discussed in the previous meeting. SS: For clearer spec, we prefer to clarify the cases which are proposed by CATT and Lenovo. LGE: Agree with FL’s assessment. NTT DOCOMO: we are fine to clarify the cases proposed here. OPPO: Ok to discuss ZTE: Agree with FL’s assessment. Apple: Agree with FL vivo: We share similar views as QC. Lenovo: Agree with FL. CATT: We think this is an essential issue. We prefer to discuss the issue. Intel: Agree with FL Ericsson: Agree with FL |
| 4 | For mTRP PUSCH, it is specified that except when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', the maximum number of configured SRS resources for codebook based transmission is 2, which is not aligned with the design for M-TRP PUSCH transmission. – (R1-2203422) FL note 1: TP is proposed to 38.214 to correct the issue and FL views that this is an editorial issue. | CATT | E | QC: Ok with E. SS: Agree with FL. ASUS: Agree with FL. LGE: Agree with FL’s assessment. NTT DOCOMO: Agree with FL. OPPO: Agree with FL ZTE: Agree with E. Apple: Agree with FL vivo: OK with E. Lenovo: Agree with FL. CATT: Agree with FL’s assessment. Intel: Agree Ericsson: Ok |
| 5 | For mTRP PUSCH, in 38.212, NSRS/NSRS, 0_2 in formulas on determining the bitwidth of Second SRS resource indicator field in DCI format 0_1/0_2 is the number of configured SRS resources in the second SRS resource set. It is not clear the second SRS resource set is the second SRS resource set of which SRS resource sets. – (R1-2203422) FL note 1: To FL reading, even though the proponents have a clarification text is proposed for 38.212, the current spec seems ok. | CATT | N (N : 7) | QC: This can be editorial or discussed together with issue #2. SS: This can be E. ASUS: Agree with FL. LGE: This is editorial issue. NTT DOCOMO: Agree with FL. OPPO: The current spec is clear since in “Table 7.3.1.1.2-36: SRS resource set indication”, the second SRS resource set is defined. ZTE: Either N or E is fine. Apple: Agree with FL vivo: This is an editorial issue. Lenovo: Either N or E is fine. CATT: Clarification in the spec is needed. In current spec, the SRS resource set for DCI format 0_2 is composed of the first NSRS, 0_2 SRS resources, or in the SRS resource set with lower srs-ResourceSetId of two SRS resources sets configured by higher layer parameter srs-ResourceSetToAddModList. However, when SRS resource set indicator field equals to '01', the SRS resource set for DCI format 0_2 shall composed of the first NSRS, 0_2 SRS resources, or in the SRS resource set with higher srs-ResourceSetId of two SRS resources sets configured by higher layer parameter srs-ResourceSetToAddModList. Suggest to discussing the issue. Intel: Agree with FL Ericsson: Agree with FL’s assessment |
| 6 | For mTRP PUSCH, when the second PTRS-DMRS association field is configured, the association between UL PT-RS port(s) and DM-RS port(s) for PUSCH associated to two SRS resource sets should be signaled by the first and the second PTRS-DMRS association field in DCI format 0_1 and DCI format 0_2 respectively. This situation seems to be not included in current version. – (R1-2203506) FL note 1: Issue is valid, and TP is proposed to 38.214 to correct the issue. FL views that this is an editorial issue. | vivo | E | QC: Ok with E. SS: Ok with E. LGE: Agree with FL’s assessment. NTT DOCOMO: Agree with FL. OPPO: Ok with E ZTE: Agree with E. Apple: Agree with FL vivo: OK with E. Lenovo: Agree with FL. CATT: Ok with E. Intel: OK with E |
| 7 | For MTRP PUSCH, the coherence properties of the two indicated TPMIs may be different. So, the scaling factors for PT-RS transmitted on PUSCH repetitions towards 2TRPs should be determined according to the first and second TPMI indication respectively. Current spec does not capture this case. – (R1-2203506) FL note 1: This does not seem essential and more or less an optimization to FL reading. | vivo | N (N : 7) | QC: Agree with FL. Current spec seems clear. SS: We are fine to discuss this issue. LGE: Agree with FL’s assessment. NTT DOCOMO: OK to discuss this issue. OPPO: Agree with QC ZTE: Agree with FL’s assessment. Apple: Agree with FL vivo: This issue should be discussed because the coherence properties of the two indicated TPMIs may be different so the scaling factors for PT-RS transmitted on PUSCH repetitions towards 2TRPs should be determined according to the first and second TPMI indication respectively. Lenovo: Agree with FL. CATT: Ok to discuss. Intel: OK with FL Ericsson: Ok with FL assessment |
| 8 | For mTRP PUSCH, in current version of spec, two sets of power control parameter are described only when two TRP operation is indicated. Propose to further clarify default power control parameter acquisition for single TRP scheduling case. – (R1-2203506) FL note 1: This does not seem essential correction and current spec version seems clear on this. | vivo | H (N : 3) | QC: Ok to discuss this issue and make spec clearer. SS: Ok to discuss. ASUS: Ok to discuss. LGE: Agree with FL’s assessment. NTT DOCOMO: OK to discuss this issue. OPPO: Open to discuss ZTE: Agree with FL’s assessment. Apple: Agree with FL vivo: This issue should be discussed. Based on current description, one set of parameters determined as Rel-15/16 is shared for the first and second TRP when only the first or second TRP is scheduled which is unreasonable. Lenovo: OK to discuss. CATT: Ok to discuss. Intel: OK to discuss Ericsson: ok to discuss further. |
| 9 | For CB based PUSCH transmission, clarifying the spec text for determination of the actual number of UL PT-RS ports for mTRP CB based PUSCH. – (R1-2203856) FL note 1: This does not seem essential correction and current spec version seems clear on this. | Samsung | N (N : 6) | QC: Agree with FL. Current spec seems ok. SS: We think discussion or editorial change is needed because the field names for first TPMI ‘Precoding information and number of layers’ and second TPMI ‘Second Precoding information’ are different. LGE: Agree with Samsung. NTT DOCOMO: Agree with Samsung OPPO: Agree with FL ZTE: Non-essential. Apple: Agree with FL vivo: OK to discuss to make the spec clear. Lenovo: Agree with FL. CATT: Ok to discuss. Intel: Agree with FL |
| 10 | In case of PUSCH repetition and fullpowerMode1, update the TPMI index for the second TPMI field to capture correct available TPMIs for full power transmission. - (R1-2203674) FL note 1: This issue does not seem valid as the Table 7.3.1.1.2-2D in 38.212 is capturing all possible TPMIs that is supported in Table 7.3.1.1.2-2A for fullpowerMode1. The current spec seems ok. | NEC | H (H :11) | QC: The issue seems valid. We think this should be H. There is some indexing issues in the tables. SS: This issue seems valid. Ok to discuss. LGE: This issue seems valid. Ok to discuss. NTT DOCOMO: OK to discuss this issue. OPPO: Support to discuss ZTE: Agree to discuss as H since this is valid. Apple: OK to discuss vivo: OK to discuss. Lenovo: OK to discuss. CATT: Ok to discuss. Intel: OK to discuss Ericsson: Ok to discuss |
| 11 | For PUSCH repetition, RV sequence applied to second TRP is not a cyclic shifted version of legacy RV sequence (0 2 3 1). (R1-2203674) FL note 1: This issue does not seem valid, and this issue was discussed before. The current spec seems ok. | NEC | N (N : 11) | QC: The issue is not valid. In our understanding, the RV offset between the two TRPs is not wrt a cyclic shift of the “RV pattern” but is a regular offset. The table is same as Table 5.1.2.1-3 for DL from Rel-16. SS: Agree with FL. LGE: Agree with FL’s assessment. NTT DOCOMO: Agree with FL. OPPO: Agree with FL ZTE: Agree with FL’s assessment. Apple: Agree with FL vivo: Share similar views as QC. This issue is not valid. Lenovo: Agree with FL. CATT: Agree with FL’s assessment. Intel: Agree with FL |
| 12 | For Type 1 CG mTRP PUSCH, the ‘p0-PUSCH-Alpha’ and ‘powerControlLoopToUse’ association to SRS resource sets is not defined – (R1- 2204170) FL note 1: This issue does not seem valid, and this issue was discussed before. The current spec seems ok. | Lenovo | H (N : 1, H : 8) | QC: We see the issue as current spec does not cover the power control for Type 1 CG (it seems the spec only covers Type 2 CG and ReTx for Type1 or Type 2 CG, but not initial transmission of Type 1 CG). If other companies also agree with this, this should be H. NTT DOCOMO: agree with QC. OK to discuss this issue. OPPO: We are fine to discuss this issue. ZTE: Open to discuss. Apple: Agree with FL vivo: OK to discuss to make the spec clearer. Lenovo: According to the latest spec, the power control parameter determination of initial transmission of Type 1 CG PUSCH based M-TRP is not covered. While there is an agreement that two power control parameter sets can be configured for a Type 1 CG PUSCH. Therefore, this should be H. CATT: Ok to discuss. Intel: Ok to discuss Ericsson: ok to discuss |
| 13 | mTRP PHR reporting related TP is suggested mentioning that, - The default power control parameter set per TRP defined when the SRI field is absent (one SRS resource per SRS resource set) is applied for virtual PHR calculation when twoPHRMode is enabled. - The first and second virtual PHR are associated with the first and second SRS resource set respectively if two virtual PHRs are determined. – (R1- 2204170) FL note 1: There seems nothing wrong with the current version of 38.213 to support two PHR reporting. | Lenovo | H (H : 11) | QC: Ok to discuss the issue as in current spec, virtual PHR and default power control parameters are in different places and not linked to each other explicitly. SS: Ok to discuss. We have the similar view with Qualcomm. ASUS: We also provide this issue in our contribution (R1-2204193), and support to discuss this issue in this meeting. NTT DOCOMO: OK to discuss this issue. OPPO: Fine to discuss this issue. ZTE: Open to discuss. Apple: OK to discuss vivo: OK to discuss this issue, two virtual PHRs should be calculated by two sets of default power control parameter, which is not explicitly captured. Lenovo: The current spec is not clear about the determination of virtual PHR, and there is no agreement about the default power control parameter set for virtual PHR. Therefore, this issue should be to be discussed. CATT: Ok to discuss. Intel: OK to discuss Ericsson: OK to discuss |
| 14 | Type 1 and 3 PHR discussion – (R1-2204193) FL note 1: This was discussed in multiple meetings. No clear reasons to discuss further. | ASUSTeK | N (N : 11) | QC: Agree with FL. This was discussed before. SS: Agree with FL. ASUS: Current spec is not clear of which type-1 PHR is used for comparing with type-3 PHR as there are two type-1 PHRs. Could company or FL share the view regarding this? LGE: Agree with FL’s assessment. NTT DOCOMO: Agree with FL. OPPO: Agree with FL ZTE: Agree with FL’s assessment. Apple: Agree with FL vivo: Agree with FL’s assessment. Lenovo: Agree with FL CATT: Ok to discuss. Intel: Agree with FL Ericson: Agree with FL’s assessment |
| 15 | For mTRP PUSCH UCI multiplexing, when the number of PT-RS ports is different for different beams, the number of coded REs for UCI can be different, and this is not inline with the agreed mechanism. – (R1-2204200) FL note 1: A valid issue to FL reading. TP is proposed to 38.214 to correct the issue and FL views that further discussion may be needed on this. | Apple | H | QC: Ok with H. SS: This issue is valid. Ok with H. LGE: Agree with FL’s assessment. NTT DOCOMO: Agree with FL. OPPO: Open to discuss ZTE: Agree with H. Apple: Agree with FL vivo: OK to discuss. Lenovo: Agree with FL CATT: Agree with FL’s assessment. Intel: Agree with FL Ericsson: Agree with FL’s assessment |
| 16 | For m-TRP PUCCH, per-TRP power control in FR1 is supported.. However, in current specification, determination of closed-loop index based on the one or two power control parameter sets provided for a PUCCH resource for M-TRP PUCCH operation in FR1is missing. – (R1-2204336) FL note 1: A valid issue to FL reading. TP is proposed to 38.213 to correct the issue and FL views that further discussion may be needed on this. | DCM | H | QC: Ok with H. SS: Agree with FL. ASUS: Agree with FL. LGE: Agree with FL’s assessment. NTT DOCOMO: Agree with FL. OPPO: Support to discuss ZTE: Agree with H. Apple: Agree with FL vivo: Agree with FL’s assessment. Lenovo: Agree with FL CATT: Agree with FL’s assessment. Intel: Agree with FL Ericsson: Agree with FL’s assessment |
| 17 | For m-TRP PUCCH, in current specification, when PUCCH resource is activated with two spatial relations, it is not clear that PUCCH resource with lowest index in which BWP/carrier/serving cell is used. – (R1-2204336) FL note 1: TP is proposed to 38.213 to correct the issue and FL views that this is an editorial issue. | DCM | E | QC: Ok with E. SS: Agree with FL. ASUS: Agree with FL. LGE: Agree with FL’s assessment. NTT DOCOMO: Agree with FL. OPPO: If there is no explicit description on the BWP/Cell, it refers to the active BWP as a common understanding. ZTE: Agree with E. Apple: Some discussion may be needed vivo: OK with E. Lenovo: Agree with FL CATT: Agree with FL’s assessment. Intel: Agree with FL Ericsson: Agree with FL |
| 18 | Editorial TP to determine two RS resources for the case where pathlossReferenceLinking is provided for multi-TRP PUSCH. – (R1-2204536) FL note 1: This is editorial correction. | Nokia | E | QC: Ok with E. SS: Agree with FL. LGE: Agree with FL’s assessment. NTT DOCOMO: Agree with FL. OPPO: Ok with E ZTE: Agree with E. Apple: Agree with FL vivo: Agree with FL’s assessment. Lenovo: Agree with FL CATT: OK with E. Intel: Ok with E Ericsson: Ok with E |
| 19 | Additional UE capability to support non-consecutive slot-based M-TRP PUSCH repetition Type A should be supported, i.e., the repetition for M-TRP PUSCH repetition type A can be counted based on available slots. – (R1-2204764) FL note 1: This was discussed in the last meeting with no consensus. No clear reasons to discuss further. | Intel | N (N : 9) | QC: Given the large number of TPs, this extension does not seem to be high priority. SS: We can share the similar view with Qualcomm. LGE: Agree with FL’s assessment. NTT DOCOMO: OK to discuss this issue. We think M-TRP PUSCH repetition based on available slots can be considered. OPPO: Agree with FL. It is not preferred to introduce new functionality in the maintenance stage. ZTE: Agree with FL’s assessment. Apple: Agree with FL vivo: Agree with FL’s assessment. Lenovo: Agree with FL CATT: Similar view as QC. Intel: we think if a UE is capable of coverage extension and mTRP separately, the spec should allow it to support jointly – its an easy fix |

Observation – mTRP UL

From the inputs shared by participating companies during the preparation phase, the following observation can be made:

- The following issue can be handled as E (a part of editorial CR): Issues # 1, 4, 6, 17, 18

- The following issues can be designated as H (requiring discussion and additional agreements/conclusions): Issues #2, 8, 10, 12, 13, 15,16

- The following issues can be designated as N (non-essential): Issues #3, 5, 7, 9, 11, 14, 19

# References

| 1 | R1-2203106 | Remaining issues on enhancements for Multi-TRP deployment | Huawei, HiSilicon |
| --- | --- | --- | --- |
| 2 | R1-2203422 | Maintenance issues on Rel-17 M-TRP enhancements | CATT |
| 3 | R1-2203506 | Maintenance on enhancements for multi-TRP Deployment | vivo |
| 4 | R1-2203674 | Discussion on remaining issues for multi-TRP deployment | NEC |
| 5 | R1-2203856 | Maintenance on Rel-17 multi-TRP and HST-SFN | Samsung |
| 6 | R1-2204170 | Remaining issues on multi-TRP deployment | Lenovo |
| 7 | R1-2204193 | Remaining issues for mTRP PUSCH | ASUSTeK |
| 8 | R1-2204200 | Remaining Issues on Multi-TRP Enhancement | Apple |
| 9 | R1-2204336 | Remaining issues on enhancements for Multi-TRP Deployment | NTT DOCOMO, INC. |
| 10 | R1-2204536 | Maintenance of enhancements for Multi-TRP Deployment | Nokia, Nokia Shanghai Bell |
| 11 | R1-2204764 | MIMO Enhancements for Multi-TRP Deployment | Intel Corporation |