3GPP TSG-RAN WG1 Meeting #109-e 					R1-2205128

e-Meeting, May 9th – 20th, 2022

Source: 	Moderator (Ericsson)

Title: 	Summary of Preparation Phase for Rel.16 NR Positioning Maintenance [109-e-Prep-AI7.2.8 NR-pos]

Agenda item:	7.2.8

Document for: 	Discussion and Decision

# Introduction

This document summarizes the contribution submitted for Rel-16 NR positioning maintenance in [1] and provides a feature lead recommendation for  the following RAN1 e-mail discussion :

- [109-e-Prep-AI7.2.8 NR-pos] Preparation phase for Rel16  NR positioning maintenance

# Summary of submitted proposals

## Aspect #1: PRS reception without TDD configuration

In [1] it was observed that the existing specification text for handling PRS reception in dynamic/semi-static slot format configurations was intended for reception in the serving cell, for which the UE is aware of the slot format.  However, there are cases were there can be PRS configured in a frequency layers entirely outside of the serving cell.  [1] proposes to resolve the issue by specifying signalling of the TDD configuration for each TRP to the LMF.

The following observation are drawn:

- Observation 1: The existing PRS reception versus the slot format is intended for the case when serving cell is concerned, in which case the semi-static slot format and/or the dynamic SFI can be available at the UE.

Observation 2: It is possible that some positioning frequency layers does not contain any PRS from the serving cell.

Observation 3: The serving cell slot format may not be applied to the positioning frequency layers that do not contain the PRS from the serving cell.

The following proposals are given:

- Proposal 1: RAN1 to clarify that for Rel-16, on positioning frequency layers that do not contain the PRS from any serving cell, UE may assume the symbol as DL/FL that is configured for PRS reception.

- Proposal 2: RAN1 to discuss whether the following change is adopted in Rel-17:

- Introduce the TDD configuration in the assistance data - The TDD configuration is provided per TRP or per positioning frequency layer, which is common for all TRPs within a positioning frequency layer. - For the TDD configuration of the non-serving cell, UE may receive the PRS on DL/FL symbols. - UE capability for TDD configuration is introduced for backward compatibility. - Introduce the NRPPa signaling for LMF to obtain the TDD configuration from each TRP.

Considering only proposal 1 impacts rel-16, it is proposed to focus the preparation phase on proposal 1.  For the changes proposed for Rel-17 in proposal 2, we suggest to discuss the issue in agenda 8.5.

Preparation Phase Discussion

Round #1

Feature lead recommendation:

From the FL perspective, proposal 1 in [1] can be discussed as part of AI 7.2.8.  Companies are invited to express their views regarding which proposals in [1] to be further discussed by RAN1, and if so in which agenda item.

Comments from companies:

| Company Name | Comments |
| --- | --- |
| Company A | Example: P1 OK in 7.2.8, P2 OK in 8.5. |
| Huawei, HiSilicon | P1 OK in 7.2.8. P2 should be in 7.2.8. Note that in 7.1, the guideline is provided as below: Including CRs rejected for Rel-15, but can be considered for Rel-16 or Rel-17. |
| Qualcomm | OK to discuss P1 in 7.2.8 OK to discuss P2 in 8.5, but not in 7.2.8. To us, it appears that P2 is an enhancement, with high-layer impact that could be included within the discussion of all the remaining enhancements/open-issues in the 8.5. P2 doesn’t seem a correction to be considered within the “CR” bucket of Rel-17 |
| CATT | We prefer to discuss P1 in 7.2.8, and discuss P2 in 8.5. |

Conclusions

In this document, a summary of the proposals submitted to RAN1#109e for R16 NR positioning maintenance is provided. Two proposals have been brought forward for discussion.

References

- R1-2204922 PRS reception without TDD configuration, Huawei, HiSilicon