3GPP TSG RAN WG1 #109-e			R1-2205142

e-Meeting, May 9th – 20th, 2022

Agenda item:	7.2.11

Source: 	Moderator (NTT DOCOMO, INC.)

Title: 	Summary on [109-e-R16-UE-features-MIMO-01]

Document for:	Discussion and Decision

- Introduction

This contribution summarizes the following email discussion in AI 7.2.11 regarding Rel-16 UE features for eMIMO.

[109-e-R16-UE-features-MIMO-01] Email discussion/approval on UE feature for maximum number of layers for multi-TRP by May 13 – Hiroki (DOCOMO)

- Introduce a new FG for maximum number of layers applicable to both single-DCI based multi-TRP FDM scheme A/B and multi-DCI based multi-TRP overlapped in time [but non-overlapped in frequency] for Rel-17, based on the discussion and proposals in R1-2202511, R1-2203630 and R1-2204703

    - Type is per band, and the FG is for FR2 only

    - Add a restriction to avoid conflict with the existing field maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9)

- Discussion on UE features for eMIMO

  - Maximum number of layers for multi-TRP (a new FG)

Following proposals are made in contributions in AI7.2.11.

| [1] | Based on the elaboration in [1][2], in FR2, for single-DCI based MTRP schemes, the parameter maxNumberMIMO-LayersPDSCH is shared among the multi-TRP schemes and with the conventional single-TRP operations. A low-complexity UE implementation may support multi-TRP operations by using two panels, each with only one receiver unit (RxU). For single-TRP operations, UE can report maxNumberMIMO-LayersPDSCH = 2 by using both one-RxU panels for data reception. It also works for the single-DCI based SDM and TDM schemes. However, the single-DCI based FDM schemes cannot support 2 layers since the signals from the two TRPs arrive at the same time and in FR2 it is seldom that one panel can receive signals well from both TRPs simultaneously. Therefore, for single-DCI based FDM schemes, the maximum number of layers should be separately reported from single-TRP and the single-DCI based SDM and TDM schemes. Similarly, for multi-DCI based MTRP scheme, a low complexity UE equipped with two panels, each with only one RxU may only support 1 layer per PDSCH/TRP by reporting maxNumberMIMO-LayersPDSCH = 2, interpreted as the maximum number of layers across two PDSCHs if having at least one RE overlapped. However, when the two PDSCHs are overlapped in time but non-overlapped in frequency, the UE cannot support 2 layers per PDSCH in FR2 since generally 1 RxUs per panel is used per PDSCH. Therefore, a FL proposal was suggested as follows in [2]: Updated FL proposal #1: Introduce a new FG for maximum number of layers for single-DCI based multi-TRP FDM scheme A/B and multi-DCI based multi-TRP overlapped in time for Rel-17 Maximum number of layers for single-DCI based multi-TRP FDM scheme A and B and multi-DCI based multi-TRP overlapped in time; 1.For DL reception of single-DCI based multi-TRP FDM scheme A and scheme B, the maximum number of spatial multiplexing layer(s) supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH 2.For DL reception of multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency, the maximum number of spatial multiplexing layer(s) per PDSCH supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH; supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16; Per FSPC; FR2 only. From our view, the granularity should be per band rather than per FSPC to avoid complicated and large overhead capability signaling. Furthermore, for multi-DCI based MTRP, it is not good to split the same UE capability report into 2 cases where case 1 is two PDSCH overlapped in at least one RE as described in FG 16-2a-9, and case 2 is two PDSCHs overlapped in time but non-overlapped in frequency. Hence, we suggest using this new FG to override the FG 16-2a-9. Here is our suggestion: Proposal #1: Introduce a new FG for maximum number of layers for single-DCI based multi-TRP FDM scheme A/B and multi-DCI based multi-TRP overlapped in time for Rel-17 Maximum number of layers for single-DCI based multi-TRP FDM scheme A and B and multi-DCI based multi-TRP overlapped in time; 1. For DL reception of single-DCI based multi-TRP FDM scheme A and scheme B, the maximum number of spatial multiplexing layer(s) supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH 2. For DL reception of multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency, the maximum number of spatial multiplexing layer(s) per PDSCH supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH Note: if this feature is reported, FG 16-2a-9 should not be used.; supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16; Per band FSPC; FR2 only. |
| --- | --- |
| [2] | - Fundamental limitation of multi-TRP in FR2 Since the main benefit of multi-TRP is to have spatial diversity, the two TRPs are in general not co-located. In FR2, highly directional beams are required to combat pathloss and thus different Rx beams need to be formed for reception from different TRPs. It is less likely that one RxU can receive signals with good RSRP from both TRPs simultaneously. In short, in FR2 one RxU can only beamform to one TRP at a time. The above phenomenon is illustrated in Figure 1. Figure 1: Fundamental limitation of multi-TRP in FR2. Due to the above fact, the number of layers per TRP is limited by the number of associated RxUs. More specifically, for a UE equipped with N RxUs to be served by two TRPs simultaneously, the number of layers per TRPs is limited to N/2 RxUs assuming equal number of associated RxUs for each TRP. Observation 1: In FR2, the maximum number of layers per TRP should be upper bounded by the number of associated RxUs, instead of total number of RxUs. - Interpretation of maxNumberMIMO-LayersPDSCH Consider a UE with N RxUs. In the conventional single-TRP operation, all available RxUs can be used for reception from this TRP and thus UE can report maxNumberMIMO-LayersPDSCH = N. In the current NR, this field is shared among all multi-TRP schemes, possibly with a different interpretation. For the multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency and for the FDM schemes of single-DCI based multi-TRP, the maximum number of layers per PDSCH can only be N/2 in FR2, according to the discussion in Section 2. However, the current UE capability reporting assumes the same number of layers as single-TRP, which would force the UE to underreport its capability for single-TRP and other multi-TRP schemes, i.e., single-DCI based SDM and TDM schemes and multi-DCI based multi-TRP with overlap in time and frequency. Observation 2: For a UE intending to support the multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency and the single-DCI based FDM schemes, according to the current UE capability reporting, in FR2 the maximum number of layers needs to be underreported for single-TRP and other multi-TRP schemes, i.e., multi-DCI based multi-TRP with overlap in time and frequency, and single-DCI based SDM and TDM schemes. Observation 3: In the RAN1#108e meeting, we raised this issue of maximum number of layers for multi-TRP [1]. Although the issue was generally recognized, some companies commented that it would be too late for Rel-16 to introduce new FG and some further text check is needed. Thus, it was concluded “no consensus to introduce such a new FG for Rel-16” for the reported issue [2]. However, most companies seem fine to introduce the new FG to Rel-17, and we believe addressing this underreporting issue is important. One natural question is whether support of the mentioned schemes is needed for FR2. For multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency, it is the baseline of the multi-DCI based multi-TRP schemes. Thus, from the perspective of UE capability reporting, even if a UE only wants to support the case of full overlap in time and in frequency, the UE still needs to support the case of non-overlap in frequency. As for the single-DCI based FDM schemes, it has the following advantages for FR2: - More reliable than the single-TRP scheme. - The FDM scheme B is more robust than the SDM scheme when considering the blockage effect, as the current design cannot guarantee that the layers from the same TRP are self-decodable. - Higher system throughput and better power saving than the TDM schemes. For 3), the reason for higher system throughput is that it is challenging to serve multiple users simultaneously in FR2 as different analog beams are required in general. In this case, it is preferable to serve each UE using as less time as possible. Better power saving is achievable because UE can enter the sleeping mode earlier. Although the maximum UE throughput would be lower for the FDM schemes in FR2 as the number of layers is halved, the FDM schemes can still be beneficial for URLLC or low-power applications. Based on the above discussions, we find it necessary to introduce a new UE capability. Following the valuable comments in the RAN1#108e meeting, we propose the following unified UE feature applicable both for the multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency and for the FDM schemes of single-DCI based multi-TRP. In the meantime, we add a restriction to avoid conflict with the existing field maxMIMO-LayersForMulti-DCI-mTRP-r16 [3]. Proposal 1: Introduce the following new UE feature for Rel-17: Definitions for parameters; Per; M; FDD-TDD DIFF; FR1-FR2 DIFF. maxNumberMIMO-LayersPDSCH-For-mTRP-FDM-r17 If this field is included, for DL reception of single-DCI based multi-TRP FDM scheme A and scheme B, the maximum number of spatial multiplexing layer(s) supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH. If this field is included, for DL reception of multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency, the maximum number of spatial multiplexing layer(s) per PDSCH supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH. The UE that indicates support of this feature shall not indicate support maxMIMO-LayersForMulti-DCI-mTRP-r16.; Band; No; N/A; FR2 only. Observation 4: For multi-DCI based multi-TRP, the following table summarizes the UE behaviors considering maxMIMO-LayersForMulti-DCI-mTRP-r16 and maxNumberMIMO-LayersPDSCH-For-mTRP-FDM-r17. We note that if maxMIMO-LayersForMulti-DCI-mTRP-r16 is not included, the two FGs cover independent cases: maxMIMO-LayersForMulti-DCI-mTRP-r16 only covers the case of overlap in time AND in frequency; maxNumberMIMO-LayersPDSCH-For-mTRP-FDM-r17 only covers the case of overlap in time BUT NOT in frequency. It is noted that maxNumberMIMO-LayersPDSCH-For-mTRP-FDM-r17 is the newly proposed UE capability, while maxNumberMIMO-LayersPDSCH and maxMIMO-LayersForMulti-DCI-mTRP-r16 are existing 5G NR UE capabilities. maxMIMO-LayersForMulti-DCI-mTRP-r16; maxNumberMIMO-LayersPDSCH-For-mTRP-FDM-r17; Interpretation of maximum number of layers from maxNumberMIMO-LayersPDSCH (N). Overlap in time and in frequency; Overlap in time but not in frequency; Not overlap in time. Not include; Not include; N layers across two PDSCHs; N layers per PDSCH; N layers per PDSCH. Not include; Include; N layers across two PDSCHs; N/2 layers per PDSCH; N layers per PDSCH. Include; Not include; N layers per PDSCH; N layers per PDSCH; N layers per PDSCH. Include; Include; Error case. |

The issue discussed in [1] and [2] was discussed at the RAN1#108-e meeting as summarized in [3], and the proponent suggested to revisit this issue for Rel-17 in this meeting.

During the preparation phase email discussion, following comments were provided [4].

| Company | Comment |
| --- | --- |
| Apple | We are fine to introduce new UE capability. The detailed FG proposal can be for further discussion. |
| Intel | OK in principle – is it restricted to FR2 only |
| Moderator (NTT DOCOMO) | The proposal is updated to clarify that the new FG is for FR2 only as proposals in [1] and [2]. |
| MTK | We support the FL proposal. For the relation between the new UE FG and legacy UE FGs, we provided one comparison table in Observation 4 of R1-2204703 for companies’ reference. |
| Nokia, NSB | We are fine to continue discussion on this UE capability. |
| ZTE | Agree to discuss the issue. Technically, we think this new UE feature will be used to override the FG 16-2a-9. That is, if this new FG is reported by UE, then FG 16-2a-9 should not be used or reported. In such case, we think this new FG should cover the case that multi-DCI based multi-TRP overlapped in at least one RE. Therefore, the wording should be changed as “multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency for Rel-17”. |
| Qualcomm | Agree to discuss the issue and introduce the new FGs with the details to be further discussed. |
| Moderator (NTT DOCOMO) | Thanks for the feedbacks! It seems that companies are ok to discuss the proposal in this meeting. According to the comment from ZTE, the proposal is updated as below, but anyway further details should be discussed in the email discussion (not in preparation phase). FL proposal #1 of email discussion/approval: [109-e-R16-UE-features-MIMO-01] Email discussion/approval on UE feature for maximum number of layers for multi-TRP - Introduce a new FG for maximum number of layers applicable to both single-DCI based multi-TRP FDM scheme A/B and multi-DCI based multi-TRP overlapped in time [but non-overlapped in frequency] for Rel-17, based on the discussion and proposals in R1-2202511, R1-2203630 and R1-2204703 - Type is per band, and the FG is for FR2 only - Add a restriction to avoid conflict with the existing field maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9) |

Based on the above, the following FL proposal can be the starting point for the discussion.

FL proposal #1

- Introduce a new FG for maximum number of layers applicable to both single-DCI based multi-TRP FDM scheme A/B and multi-DCI based multi-TRP overlapped in time [but non-overlapped in frequency] for Rel-17, based on the discussion and proposals in R1-2202511, R1-2203630 and R1-2204703

  - Type is per band, and the FG is for FR2 only

  - Add a restriction to avoid conflict with the existing field maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9)

Companies are encouraged to check above FL proposal and to provide feedback if any in below.

| Company | Comment |
| --- | --- |
| ZTE | If the restriction is added as the shown in the second subbullet, ‘[but non-overlapped in frequency]’ should be removed. Here is our suggestion to make the proposal clear enough. - Introduce a new FG for maximum number of layers applicable to both single-DCI based multi-TRP FDM scheme A/B and multi-DCI based multi-TRP overlapped in time [but non-overlapped in frequency] for Rel-17, based on the discussion and proposals in R1-2202511, R1-2203630 and R1-2204703 - Type is per band, and the FG is for FR2 only - The UE that indicates support of this new feature shall not indicate support maxMIMO-LayersForMulti-DCI-mTRP-r16Add a restriction to avoid conflict with the existing field maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9) |
| Apple | We are fine For the last sub-bullet, do we need further restriction? - Add a restriction to avoid conflict with the existing field maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9) in the same band |
| QC | Support the revisions from ZTE as well as Apple. |
| LG | We are fine with ZTE and Apples’ revision. |
| MTK | We tend to think the sentence “[but non-overlapped in frequency]” in FL proposal needs to be kept. Following is the UE capability for FG 16-2a-9 copied from R16 38.306: Definitions for parameters; Per; M; FDD-TDD DIFF; FR1-FR2 DIFF. maxMIMO-LayersForMulti-DCI-mTRP-r16 Indicates the interpretation of maxNumberMIMO-LayersPDSCH for multi-DCI based mTRP. If this field is included, maxNumberMIMO-LayersPDSCH is interpreted as the maximum number of layers per PDSCH for multi-DCI multi-TRP operation. If this field is not included, maxNumberMIMO-LayersPDSCH is interpreted as the maximum number of layers across two PDSCHs if having at least one RE overlapped, for multi-DCI multi-TRP operation. The UE that indicates support of this feature shall support overlapPDSCHsFullyFreqTime-r16. … …; Band; No; N/A; N/A. Assume UE reports maxNumberMIMO-LayersPDSCH = N, it can be seen from the blue highlighted text above that, if the multi-TRP overlapped in both time and frequency (RE overlapping), the UE supported maximum number of layers across two PDSCHs should still be N (where FG 16-2a-9 is not supported/included). For the newly proposed UE FG, if the sentence “[but non-overlapped in frequency]” in FL proposal is removed, then the new FG may conflict with existing FG 16-2a-9 when the multi-TRP overlapped in both time and frequency (RE overlapping): - When UE reports support for the new FG, and not support FG 16-2a-9, and the multi-TRP overlapped in both time and frequency (RE overlapping), description in FG 16-2a-9 says the the UE supported maximum number of layers across two PDSCHs should be N, while the new UE FG says the UE supported maximum number of layers across two PDSCHs should be N/2 Conflict Therefore, we tend to think the sentence “[but non-overlapped in frequency]” in FL proposal needs to be kept. If the sentence is kept, to our understanding, the UE behavior would be as below and seems clear: maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG 16-2a-9); maxNumberMIMO-LayersPDSCH-For-mTRP-FDM-r17 (the new FG); Interpretation of maximum number of layers from maxNumberMIMO-LayersPDSCH (N). Overlap in time and in frequency (SDM); Overlap in time but NOT in frequency (FDM); Not overlap in time (TDM). Not include; Not include; N layers across two PDSCHs; N layers per PDSCH; N layers per PDSCH. Not include; Include; N layers across two PDSCHs; N/2 layers per PDSCH; N layers per PDSCH. Include; Not include; N layers per PDSCH; N layers per PDSCH; N layers per PDSCH. Include; Include; Error case. |
| Moderator (NTT DOCOMO) | Thank you very much for the feedbacks! Based on the feedbacks so far, we can further check companies’ views on following two alternatives. As pointed by MTK, the existing description for FG16-2a-9 “If this field is not included, maxNumberMIMO-LayersPDSCH is interpreted as the maximum number of layers across two PDSCHs if having at least one RE overlapped, for multi-DCI multi-TRP operation” should be considered. Alt.1 (ZTE, Apple, QC, LG): - Introduce a new FG for maximum number of layers applicable to both single-DCI based multi-TRP FDM scheme A/B and multi-DCI based multi-TRP overlapped in time for Rel-17 - Type is per band, and the FG is for FR2 only - The UE that indicates support of this new feature shall not indicate support maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9) in the same band Alt.2 (MTK): - Introduce a new FG for maximum number of layers applicable to both single-DCI based multi-TRP FDM scheme A/B and multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency for Rel-17 - Type is per band, and the FG is for FR2 only - The UE that indicates support of this new feature shall not indicate support maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9) in the same band The FG descriptions can be as below. Maximum number of layers applicable to both single-DCI based multi-TRP FDM scheme A and B and multi-DCI based multi-TRP overlapped in time [but non-overlapped in frequency]; 1. For DL reception of single-DCI based multi-TRP FDM scheme A and scheme B, the maximum number of spatial multiplexing layer(s) supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH 2. For DL reception of multi-DCI based multi-TRP overlapped in time [but non-overlapped in frequency], the maximum number of spatial multiplexing layer(s) per PDSCH supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH Note: The UE that indicates support of this new feature shall not indicate support maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9) in the same band.; supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16; Per band; FR2 only. |
| QC | We support Alt1. Based on the original motivation provided by MTK, we are not sure if Alt2 can address the issue. For example, if UE reports maxNumberMIMO-LayersPDSCH =4, Alt2 requires the UE to support rank 1+3 or 3+1 in multi-DCI based mTRP in case of overlap in both time and frequency, which may not be possible if the UE can only support half of the indicated value in FR2 (which is the case irrespective of freq. domain overlap or not as long as there is time domain overlap). Regarding the issue mentioned by MTK, we do not see a conflict. If this new FG is included, the behavior in this FG is followed (instead of Rel-16 behavior in the absence of indicating support of FG 16-2a-9). If needed, a note can be added to clarify this (we do not see the need, but we are ok if it can resolve some potential chance of confusion). |
| HW | For the difference Alt 1 and Alt 2, I would assume that single-DCI based multi-TRP FDM schemes and Multi-DCI based FDM schemes (i.e. the case of Overlap in time and in frequency) may share the common restriction for UE implementation, i.e. - If Bullet 1 prefers that “the maximum number of spatial multiplexing layer(s) supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH”, Bullet 2 for Multi-DCI based FDM schemes would prefer similarly, i.e. N/2 layers per PDSCH - Here is my understanding for Alt 1 after new FG. The blue part is something different from original understanding of MTK. - Also after changing, FG16-2a-9 and the new FG seems to be more symmetric and traceable, so that R17 new FG is an indication of weaker UE implementation (by half). Note the column of non-overlap in time (TDM) is irrelevant to R16 specification. maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG 16-2a-9); maxNumberMIMO-LayersPDSCH-For-mTRP-FDM-r17 (the new FG); Interpretation of maximum number of layers from maxNumberMIMO-LayersPDSCH (N). Overlap in time and in frequency (SDM); Overlap in time but NOT in frequency (FDM); Not overlap in time (TDM). Not include; Not include; N layers across two PDSCHs; N layers per PDSCH; N layers per PDSCH. Not include; Include; N/2 layers per PDSCH; N/2 layers per PDSCH; N layers per PDSCH. Include; Not include; N layers per PDSCH; N layers per PDSCH; N layers per PDSCH. |
| ZTE | After reviewing the comments, I still think Alt.1 is more reasonable which make the same UE behavior for the case of overlapping at least one RE and overlapping in time. On the other hand, in Alt. 2, 1+3 and 3+1 layers is possible for the case overlapping at least one RE, but only 1+1 and 2+2 layers are possible for the case overlapping in time but non-overlapping in frequency. This is not reasonable in FR2. The new FG is to override the legacy FG 16-2a-9, we can add a note as Mostafa mentioned, i.e. if the new FG is reported by the UE , UE will follow the new behavior and igore 16-2a-9 (the following sentence will not be valid) If this field is not included, maxNumberMIMO-LayersPDSCH is interpreted as the maximum number of layers across two PDSCHs if having at least one RE overlapped, for multi-DCI multi-TRP operation. The UE that indicates support of this feature shall support overlapPDSCHsFullyFreqTime-r16. |
| Moderator (NTT DOCOMO) | Thanks for the feedbacks! FL proposal can be updated as below based on the feedbacks and discussions. - Introduce a new FG for maximum number of layers applicable to both single-DCI based multi-TRP FDM scheme A/B and multi-DCI based multi-TRP overlapped in time for Rel-17 - Type is per band, and the FG is for FR2 only - The UE that indicates support of this new feature shall not indicate support maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9) in the same band - If the new FG is reported by the UE, UE will follow the new behavior and ignore 16-2a-9 (the following sentence will not be valid) - If this field is not included, maxNumberMIMO-LayersPDSCH is interpreted as the maximum number of layers across two PDSCHs if having at least one RE overlapped, for multi-DCI multi-TRP operation. Maximum number of layers applicable to both single-DCI based multi-TRP FDM scheme A and B and multi-DCI based multi-TRP overlapped in time; 1. For DL reception of single-DCI based multi-TRP FDM scheme A and scheme B, the maximum number of spatial multiplexing layer(s) supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH 2. For DL reception of multi-DCI based multi-TRP overlapped in time, the maximum number of spatial multiplexing layer(s) per PDSCH supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH Note: The UE that indicates support of this new feature shall not indicate support maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9) in the same band. Note: If this FG is reported by the UE, UE will follow the behavior of this FG and ignore following sentence of maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9). - “If this field is not included, maxNumberMIMO-LayersPDSCH is interpreted as the maximum number of layers across two PDSCHs if having at least one RE overlapped, for multi-DCI multi-TRP operation.”; supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16; Per band; FR2 only. |
| Nokia | Overall the proposal seems OK, but this part is problematic: - If the new FG is reported by the UE, UE will follow the new behavior and ignore 16-2a-9 (the following sentence will not be valid) If this field is not included, maxNumberMIMO-LayersPDSCH is interpreted as the maximum number of layers across two PDSCHs if having at least one RE overlapped, for multi-DCI multi-TRP operation. This creates a problem if the new FG is reported by the UE to a gNB that does not understand the new signaling, but which assumes the UE to behave according to Rel-16 mTRP, including 16-2a-9. Hence, this change creates a backward compatibility issue, and hence we are not fine with it, unfortunately. |
| Moderator (NTT DOCOMO) | It is true that the legacy gNB could not understand the new FG and the UE not reporting 16-2a-9 is considered according to following sentence i.e., cannot ignore it. If this field is not included, maxNumberMIMO-LayersPDSCH is interpreted as the maximum number of layers across two PDSCHs if having at least one RE overlapped, for multi-DCI multi-TRP operation. Then, we should take Alt.2 below for final checking. - Introduce a new FG for maximum number of layers applicable to both single-DCI based multi-TRP FDM scheme A/B and multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency for Rel-17 - Type is per band, and the FG is for FR2 only - The UE that indicates support of this new feature shall not indicate support maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9) in the same band Maximum number of layers applicable to both single-DCI based multi-TRP FDM scheme A and B and multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency; 1. For DL reception of single-DCI based multi-TRP FDM scheme A and scheme B, the maximum number of spatial multiplexing layer(s) supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH 2. For DL reception of multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency, the maximum number of spatial multiplexing layer(s) per PDSCH supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH Note: The UE that indicates support of this new feature shall not indicate support maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9) in the same band.; supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16; Per band; FR2 only. |
| Qualcomm | I am just wondering how Alt2 addresses the NBC issue? Consider the following using the scenario that Cassio mentioned: UE indicates the new FG as in Alt2 below to a gNB that does not understand the new signaling. Then, for the case that two PDSCHs “overlapped in time but non-overlapped in frequency”, UE assumes the behavior in Alt2 (N/2 layer per PDSCH) while gNB assumes Rel-16 behavior (N layers per PDSCH). Hence, we do not think Alt2 is addressing this issue. At the same time, Alt1 at least addresses the original motivation properly in FR2. |
| Moderator (NTT DOCOMO) | I thought that in Alt.2 the issue pointed by Cassio can be avoided as the following sentence in FG16-2a-9 is for the case of “RE overlapping”. If this field is not included, maxNumberMIMO-LayersPDSCH is interpreted as the maximum number of layers across two PDSCHs if having at least one RE overlapped, for multi-DCI multi-TRP operation. But I see the point from Mostafa, and other companies would have same understanding with Mostafa that gNB assumes N layers per PDSCH in such case as shown in the table used by MTK/HW. If such problem cannot be avoided in both Alt.1 and Alt.2, should we consider other alternative or give up to introduce this kind of new FG? Could someone explain whether/how the problem in case of the combination between legacy gNB + new UE reporting the new FG without FG16-2a-9 is significant? |
| MediaTek | For Alt. 1 v.s. Alt. 2, with the explanation and suggestion from Qualcomm/Huawei/ZTE, we now think Alt. 1 is also acceptable. As for the NBC issue mentioned by Nokia: “The problem in case of the combination between legacy gNB + new UE reporting the new FG” We found that it is indeed an issue and we would like to suggest another alternative below: to introduce another maxNumberMIMO-LayersPDSCH-r17 capability for R17 as shown below (the red text are diff part from the original IE of maxNumberMIMO-LayersPDSCH). - For UE side, it still can only under-report the maxNumberMIMO-LayersPDSCH value (Ex. 1), but can report a larger maxNumberMIMO-LayersPDSCH-r17 value (Ex. 2) - For gNB side, only R17 gNB would recognize maxNumberMIMO-LayersPDSCH-r17 and can do a proper arrangement of MIMO layer number - R16 gNB still only looks at the original IE of maxNumberMIMO-LayersPDSCH maxNumberMIMO-LayersPDSCH-r17 Defines the maximum number of spatial multiplexing layer(s) supported by the UE for DL reception. For single CC standalone NR, it is mandatory with capability signaling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. If absent, the UE does not support MIMO on this carrier.If included, maxNumberMIMO-LayersPDSCH-r17 overwrites maxNumberMIMO-LayersPDSCH. For DL reception of single-DCI based multi-TRP FDM scheme A and scheme B, the maximum number of spatial multiplexing layer(s) supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH-r17. For DL reception of multi-DCI based multi-TRP overlapped in time, the maximum number of spatial multiplexing layer(s) per PDSCH supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH-r17. The UE that indicates support of this feature shall not indicate support of maxMIMO-LayersForMulti-DCI-mTRP-r16.; FSPC; CY; N/A; FR2 only. |
| Moderator (NTT DOCOMO) | Let’s check companies’ views on the following alternative proposal from MTK (and please correct if I misunderstand something). - Introduce a new FG for maximum number of spatial multiplexing layer(s) supported by the UE for DL reception for Rel-17 - Type is per FSPC, and the FG is for FR2 only - Candidate values are same as maxNumberMIMO-LayersPDSCH - Prerequisite FGs are maxNumberMIMO-LayersPDSCH and {supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16} - If this FG is indicated, it overwrites maxNumberMIMO-LayersPDSCH - If this FG is indicated, for DL reception of single-DCI based multi-TRP FDM scheme A and scheme B, the maximum number of spatial multiplexing layer(s) supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH-r17 - If this FG is indicated, for DL reception of multi-DCI based multi-TRP overlapped in time, the maximum number of spatial multiplexing layer(s) per PDSCH supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH-r17 - The UE that indicates support of this new feature shall not indicate support maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9) in the same band |

Updated FL proposal #1

- Introduce a new FG for maximum number of spatial multiplexing layer(s) supported by the UE for DL reception for Rel-17

  - Type is per FSPC, and the FG is for FR2 only

  - Candidate values are same as maxNumberMIMO-LayersPDSCH

  - Prerequisite FGs are maxNumberMIMO-LayersPDSCH and {supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16}

  - If this FG is indicated, it overwrites maxNumberMIMO-LayersPDSCH

  - If this FG is indicated, for DL reception of single-DCI based multi-TRP FDM scheme A and scheme B, the maximum number of spatial multiplexing layer(s) supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH-r17

  - If this FG is indicated, for DL reception of multi-DCI based multi-TRP overlapped in time, the maximum number of spatial multiplexing layer(s) per PDSCH supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH-r17

  - The UE that indicates support of this new feature shall not indicate support maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9) in the same band

| ZTE | It seems the issue raised by Mostafa and Cassio still exists in the new proposal. That is, if the new FG is reported to Rel-16 gNB , then the interpretations between UE and gNB are still different for the case of overlapping in time but not in RE. Let's assume this kind of case is common and troublesome, we may not need to specially consider that here. In such case, I still think previous Alt.1 is better, at least the report granularity with per band is more recommendable. |
| --- | --- |
| Apple | In our view, the latest proposal from Hiroki/MTK solves NBC issue. The main issue is this new capability is Rel-17 capability, but all the other capabilities and mTRP feature itself is Rel-16 Therefore, for a UE that has the issue described by MTK - UE needs to under-report Rel-16 maxNumberMIMO-LayersPDSCH in case that UE operates in Rel-16 NW - UE can report the duplicate of maxNumberMIMO-LayersPDSCH as maxNumberMIMO-LayersPDSCH-r17 to work closer to its true capability in case that UE operates in Rel-17(+) NW The second bullet is the typical solution we had to solve NBC issue, i.e., by duplicating the impacted FG in earlier release. Hopefully, I did not mis-understand the problem, so in our view - Either we agree this new FG to be Rel-16, but it requires infra-vendor commitment - Or we agree this new FG to be Rel-17 and adopt the latest proposal from Hiroki/MTK. |
| Nokia | I have still an issue with the last sentence on Hiroki’s proposal: - The UE that indicates support of this new feature shall not indicate support maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9) in the same band In general this kind of statements create the backward compatibility issue. If we want to solve the original problem and keep the backward compatibility to R16, then we need to define a parallel suite of R17 FGs that override the R16 FGs. The UE will report *both* sets, in which case it will underreport the R16 capabilities and provide more aggressive R17 FGs. The R16 gNB will behave according to what it sees from R17, and the gNB that has implemented the R17 behavior, will be able to understand the more aggressive set of FGs, and it will know which R17 FGs are overridden by these ones. Of course we don’t need to repeat every single mTRP FG to achieve that. Hence, the job of the proponents here is to provide such a minimum set that we can then hopefully agree on. The current proposals are still creating more issues than they solve, unfortunately. |
| Qualcomm | We share the same understanding as MTK and Apple that the latest proposal does not have NBC issues. @ Cassio: If UE indicates support of this new FG, irrespective of whether gNB understands this new signaling or not, the UE will not indicate support of maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9). This is because if UE could support FG16-2a-9, there would be no reason for it to indicate the new FG discussed here. In other words, my understanding is that the statement below is not creating any issue for a gNB that does not understand this new signaling. Please let me know if I am missing something here. |
| Moderator (NTT DOCOMO) | I hope clarifications provided by Haitong and Mostafa could solve concerns from Chuangxin and Cassio. I would also like to provide some complementary comments based on my understanding. - As James explained, the latest proposal from MTK can solve the NBC issue pointed by Cassio as below. - UE reports maxNumberMIMO-LayersPDSCH with conservative value (e.g., N=1) and {supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16}, and the UE does not report maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9) - So, existing gNB (which cannot understand the proposed new FG) considers UE’s capability as below. Overlap in time and in frequency (SDM); Overlap in time but NOT in frequency (FDM); Not overlap in time (TDM). N(e.g., 1) layers across two PDSCHs; N(e.g., 1) layers per PDSCH; N(e.g., 1) layers per PDSCH. - The UE also reports the proposed new FG i.e., maxNumberMIMO-LayersPDSCH-r17 with larger value (e.g., N’=2) - So, gNB which can understand the proposed new FG can overwrite maxNumberMIMO-LayersPDSCH by maxNumberMIMO-LayersPDSCH-r17, and considers UE’s capability as below Overlap in time and in frequency (SDM); Overlap in time but NOT in frequency (FDM); Not overlap in time (TDM). N'/2 (e.g., 2/2=1) layers per PDSCH; N'/2 (e.g., 2/2=1) layers per PDSCH; N' (e.g., 2) layers per PDSCH. - As Mostafa clarified, not reporting maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9) by the UE is no problem, since the UE cannot support following and hence it under-reports maxNumberMIMO-LayersPDSCH with conservative value and reports the proposed new FG. Overlap in time and in frequency (SDM); Overlap in time but NOT in frequency (FDM); Not overlap in time (TDM). N (e.g., 2) layers per PDSCH; N (e.g., 2) layers per PDSCH; N (e.g., 2) layers per PDSCH. So, can we agree on the following latest proposal? l Introduce a new FG for maximum number of spatial multiplexing layer(s) supported by the UE for DL reception for Rel-17 Ø Type is per FSPC , and the FG is for FR2 only Ø Candidate values are same as maxNumberMIMO-LayersPDSCH Ø Prerequisite FGs are maxNumberMIMO-LayersPDSCH and {supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16} Ø If this FG is indicated, it overwrites maxNumberMIMO-LayersPDSCH Ø If this FG is indicated, for DL reception of single-DCI based multi-TRP FDM scheme A and scheme B, the maximum number of spatial multiplexing layer(s) supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH-r17 Ø If this FG is indicated, for DL reception of multi-DCI based multi-TRP overlapped in time, the maximum number of spatial multiplexing layer(s) per PDSCH supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH-r17 Ø The UE that indicates support of this new feature shall not indicate support maxMIMO-LayersForMulti-DCI-mTRP -r16 (FG 16-2a-9) in the same band |
| MediaTek | We support the latest moderator proposal and think there seems to be no further NBC issues. I try to provide another table similar to Hiroki’s version for companies’ reference. Hopefully it can resolve concerns from Chuangxin and Cassio. maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG 16-2a-9); maxNumberMIMO-LayersPDSCH-r17 (M) (New R17 IEUE reported capability); maxNumberMIMO-LayersPDSCH (N) (Original R16 IEUE reported capability). Overlap in time; Not overlap in time. Not include; Not include; N layers across two PDSCHs; N layers per PDSCH. Not include; Include; M/2 layers across two PDSCHs; M layers per PDSCH. Include; Not include; N layers per PDSCH; N layers per PDSCH. Include; Include; Error case. |
| ZTE | Maybe I misunderstood something. I am still not be convinced. One question is, how could UE know the gNB is Rel-16 or Rel-17 one? I think it cannot know. If so, let me take an example: The Rel-17 UE reports the new maximum layers which is 4, also reports the Rel-15 maximum layers is 4, so the UE assume maximum 2 layers per PDSCH for multi-DCI based MTRP for FDM or SDM case. However, the Rel-16 gNB cannot read the UE capability signaling, it also doesnot receive FG 16-2a-9, it only receives legacy maxNumberMIMO-LayersPDSCH = 4. then the Rel-16 gNB assumes 4 layers per PDSCH for multi-DCI based MTRP for FDM case, and assume 4 layers across two PDSCHs for SDM case. |
| Moderator (NTT DOCOMO) | In your example, we are assuming that the UE should not report value 4 for Rel-15 maximum layers. Since the UE reports value 4 for the proposed new Rel-17 maximum layers, the UE can support maximum 2 layers per PDSCH for multi-DCI based MTRP for FDM or SDM case. As you said, since Rel-16 gNB cannot understand the proposed new Rel-17 maximum layers, the UE should report value 2 instead of value 4 for Rel-15 maximum layers so that Rel-16 gNB assumes 2 layers per PDSCH for multi-DCI based MTRP for FDM case and 2 layers across two PDSCHs for SDM case. It is under-estimation of UE’s capability and hence does not cause NBC issue while if over-estimation of UE’s capability happens (such as reporting same value for Rel-15 maximum layers), it causes NBC issue. |
| ZTE | In my view, the Rel-15 parameter maxNumberMIMO-LayersPDSCH is shared for both single-TRP and multi-TRP case. If UE under report 2 layers rather than four layers, it will impact the capacity of basic STRP transmission as high rank cannot be supported for STRP scheduling anymore. |
| Nokia | My understanding is similar to that of ZTE. The proposals are not solving the issue. Since the UE cannot possibly know whether or not the gNB understands the new R17 FGs (which is different from simply being R16 or R17 gNB), it must *always* underreport its capability, thus making the new R17 signaling innocuous. Another alternative is to make an industry wide commitment to rewrite the R16 FGs and fix the issue directly. I am not sure this is feasible at this stage, and I would need to do several consultations within my own company before we can commit to that. I hope this explains our views, and why we think the issue is not solved with the current proposals. |
| Nokia | After reading the proposal again, it seems my concerns were a bit exaggerated. The new signaling of maximum number of layers would be understood by the R17 gNB, which then would be able to use the higher numbers. This means that this new FG would override the old number of MIMO layers across the board, otherwise it doesn’t work consistently. We need to do a very careful check in order to avoid bugs in the spec! One issue that still makes me uncomfortable is that this UE will report lower values for maxNumberMIMO-LayersPDSCH just for the eventual support of R16 mTRP. I find it very hard to believe there will be such UE in the market, and hence are we effectively making R16 mTRP obsolete by agreeing to this new indication? Shouldn’t we address the problem directly instead? |
| Apple | The issue raised by Cassio and Chuangxin that UE has to under-report Rel-15/16 capability, is exactly the reason why MTK needs a new FG, even though, we do not see the same problem. If you believe that there is some UE vendor who has the problem as MTK described, then that UE vendor has to report smaller number of maxNumberMIMO-LayersPDSCH to prepare for the worst case that gNB schedules the problematic mTRP case, in other scheduling in which there is no issue, that UE will under-perform. The fully resolve the issue, the right solution is to introduce additional Rel-16 UE feature. However, to address infra-vendor concern, MTK compromised to introduce Rel-17 UE feature. The compromise means that - The device under discussion will suffer worse performance in Rel-16 NW due to capability under-reporting - The device under discussion will perform up to its capability in Rel-17(+) NW This is a very normal practice we used many times during UE feature discussion. |
| Nokia | My point is just that the solution doesn’t seem to be resolving the problem. It is changing something so fundamental that we wonder if “the medicine is not worse than the disease” here, at least in its current form. Please note also that it is not enough to say R17+ networks will handle the UE to its full potential, you mean R17 networks implementing the new FG and capable of handling this new interpretation. Honestly, this impacts too fundamental aspects of MIMO operation to be considered as business as usual. We are not comfortable agreeing to this proposal now. |
| Apple | NW is not required to report its release, so infra-vendor has flexibility ignoring the new FG. This is our understanding. Therefore, strictly speaking, I should have rephrased as - The device under discussion will suffer worse performance in the NW that does not understand the new FG - The device under discussion will perform up to its capability in the NW that understands the new FG In the end, unfortunately, UE cannot determine whether we are the first bullet or the second bullet, it is up to the mercy of infra-vendor largely. But it is still better than nothing for the device company who has the issue, since otherwise, it is consistently first bullet without any chance to be the second bullet But we understand your concern, just to explain here. Maybe MTK can try to convince you since it is their problem largely. |
| Moderator (NTT DOCOMO) | I think it may be good to remind the original motivation of this proposal e.g., based on MTK’s contributions in this meeting and last meeting. The proponents want to support a low-complexity UE implementation for multi-TRP operation e.g., using two panels in FR2. In current Rel-16 specification, since the parameter maxNumbrMIMO-LayersPDSCH is shared among mTRP schemes and single TRP operations, the UE may need to choose either 1) under-report the parameter maxNumbrMIMO-LayersPDSCH so that it can operate with mTRP schemes or 2) not report mTRP schemes. The proposal is to solve this issue for such low complexity UE with mTRP operation, i.e., the problem of under-reporting (either MIMO layers or mTRP) may be anyway exist for such UE even without the proposal. Also please remember that we discussed at the last meeting on some alternatives, such as introducing new FG in Rel-16 and revising Rel-16 FG16-2a-9, but both were rejected by many companies including ZTE and Nokia. If there are still concerns to agree on the latest proposal, it would not be possible for the proposal to be included in the Rel-17 UE features list to be sent to RAN2 at the end of this week. But we can continue discussion since we are already discussing the solution to avoid potential NBC issue with network not implementing this new proposal. Your further feedback will be appreciated. |
| ZTE | For the low-complexity UE with two panels, I still see big problem for the fundamental STRP transmission. The example from MTK was one layer per panel, that means such UE can still support 2 layers for STRP . If the latest proposal is agreed, only rank 1 is supported for STRP . We should note that the probability of STRP is very high even the UE is in cell edge. I am sorry we cannot accept the latest proposal which even probably impacts Rel-15. Also sorry for the negative views in last meeting for revision of Rel-16. As you know, we should be very careful to change Rel-15/16 spec in such late stage of Rel-17. My concern is from performance perspective. In Rel-16, we actually did many simulation results to compare STRP and the schemes of multi-TRP , no rank limitation is assumed in the simulation assumption. Here, for MTRP , the performance of STRP may be sacrificed. I am not sure how can we get benefit for system capacity if the new proposal is agreed. I am afraid it is not easy to debate without any performance evaluation. |
| MediaTek | @Chuangxin: - Regarding your concern - “Only rank 1 is supported for STRP” That’s exactly the current UE behavior (for this kind of low-complexity UE, e.g., using two panels in FR2) in R16 since UE can only under-report maxNumberMIMO-LayersPDSCH for the preparation of m-TRP FDM case (if this UE supports both s-TRP and m-TRP), no matter introducing this new UE FG for R17 or not. We (MTK) are suggesting this new FG in R17 so that UE reports another max MIMO layer number (which UE would not under-report) and hence R17 gNB can properly utilize the MIMO layers. @Cassio: - Regarding your concern - Since the UE cannot possibly know whether or not the gNB understands the new R17 FGs (which is different from simply being R16 or R17 gNB ), it must *always* underreport its capability, thus making the new R17 signaling innocuous. Yes, that’s the issue in R16 for this kind of low-complexity UE if it supports both s-TRP and m-TRP. We were trying to solve this issue by modifying UE FG in R16, but was objected by Huawei/Ericsson/… due to the fear to impact legacy UE in last meeting. Hence, we then try to solve this issue in R17, with the aforementioned FG in this email thread. From our perspective, there may be more UEs supporting m-TRP from R17 and afterwards, while to our understanding, not many UEs support m-TRP in R16. As a result, introducing this FG to R17 actually helps a lot for the ecosystem. Hopefully this explanation also makes sense to you. |
| Huawei | It is not my expectation with more changes, like FSPC, over-writing maxNumberMIMO-LayersPDSCH (which is Rel15 signalling), giving rise to more confusions. We can be neutral only in condition that new FG is simple, straightforward and no NBC, similar with the design of FG 16-2a-9. - I don’t understand the meaning of “If this FG is indicated, it overwrites maxNumberMIMO-LayersPDSCH”. This FG is Rel-17 MTRP with pre-requisite of supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16. Rel-17 gNB will still use maxNumberMIMO-LayersPDSCH for Rel-15/16/17 features unless features have own rules to overwrite maxNumberMIMO-LayersPDSCH. This overwriting clause is not self-explainable. - We don’t support FSPC granularity. - For NBC issue with respect to Rel-16 gNB understanding Rel-17 FG, I don’t think that it is a big issue. We have quite some Rel15 late CRs with potential impact stating the gNB will upgrade/already implement new changes. If gNB NBC is a concern, as new Rel17 FG is moving backward with lower capability and conflict with the motivation of FG 16-2a-9 (even for the case without FG 16-2a-9), it seems to me that a clean solution is to duplicate everything of Rel-16 FGs into R17 so that a low cost UE will not report anything in Rel-16 but in Rel-17. It may be too extreme. I also expect that more problems will show up for other Rel-16 features to support this kind of low-cost UE implementation. - Therefore I think that the note of “ignoring following sentence of maxMIMO-LayersForMulti-DCI-mTRP-r16” seems to cause more troubles. Perhaps we can remove that note. Removing above note does not fully solve NBC issue for Rel-16 gNB but at least it does not look odd in RAN2 specification, i.e. ignoring a field statement from a UE capability signal that is not indicated. - gNB still have to update to Rel-17 in order to support Rel-17 UE signalling/new FG, as we cannot have two different interpretations in Rel16 and Rel-17 respectively, for the case of “FG 16-2a-9” not include and “new FG” not included. Some IoT among gNB and UE vendors may ensure joint support of a low cost UE and MTRP features at given FR2 deployment. |
| Moderator (NTT DOCOMO) | Thank you very much for the feedbacks and discussions! Let me try to summarize the discussion so far. ・Following Alt.1 proposal seems acceptable for ZTE, Apple, Qualcomm, LG, Huawei and MediaTek, while Nokia raised concern on the potential NBC issue for the Alt.1 proposal. Alt.1 - Introduce a new FG for maximum number of layers applicable to both single-DCI based multi-TRP FDM scheme A/B and multi-DCI based multi-TRP overlapped in time for Rel-17 - Type is per band, and the FG is for FR2 only - The UE that indicates support of this new feature shall not indicate support maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9) in the same band ・Following Alt.3 proposal is provided by MTK to solve the potential NBC issue raised by Nokia, but it seems not acceptable for ZTE, Nokia and Huawei while MTK, Apple and Qualcomm are fine with the Alt.3 proposal. Alt.3 - Introduce a new FG for maximum number of spatial multiplexing layer(s) supported by the UE for DL reception for Rel-17 - Type is per FSPC, and the FG is for FR2 only - Candidate values are same as maxNumberMIMO-LayersPDSCH - Prerequisite FGs are maxNumberMIMO-LayersPDSCH and {supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16} - If this FG is indicated, it overwrites maxNumberMIMO-LayersPDSCH - If this FG is indicated, for DL reception of single-DCI based multi-TRP FDM scheme A and scheme B, the maximum number of spatial multiplexing layer(s) supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH-r17 - If this FG is indicated, for DL reception of multi-DCI based multi-TRP overlapped in time, the maximum number of spatial multiplexing layer(s) per PDSCH supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH-r17 - The UE that indicates support of this new feature shall not indicate support maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9) in the same band ・Some company argued that the potential NBC issue raised by Nokia is not a big issue. According to above situation, the moderator would like to ask companies again for following questions. Q1: Is Alt.1 proposal above acceptable for you? Q2: If answer to Q1 is No, is Alt.3 proposal above acceptable for you? Q3: If answers to both Q1 and Q2 are No, do you have any good suggestion to update Alt.1 or Alt.3 so that it can be acceptable to you (and others)? |

Updated FL proposal #1

Alt.1

- Introduce a new FG for maximum number of layers applicable to both single-DCI based multi-TRP FDM scheme A/B and multi-DCI based multi-TRP overlapped in time for Rel-17

  - Type is per band, and the FG is for FR2 only

  - The UE that indicates support of this new feature shall not indicate support maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9) in the same band

Alt.3

- Introduce a new FG for maximum number of spatial multiplexing layer(s) supported by the UE for DL reception for Rel-17

  - Type is per FSPC, and the FG is for FR2 only

  - Candidate values are same as maxNumberMIMO-LayersPDSCH

  - Prerequisite FGs are maxNumberMIMO-LayersPDSCH and {supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16}

  - If this FG is indicated, it overwrites maxNumberMIMO-LayersPDSCH

  - If this FG is indicated, for DL reception of single-DCI based multi-TRP FDM scheme A and scheme B, the maximum number of spatial multiplexing layer(s) supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH-r17

  - If this FG is indicated, for DL reception of multi-DCI based multi-TRP overlapped in time, the maximum number of spatial multiplexing layer(s) per PDSCH supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH-r17

  - The UE that indicates support of this new feature shall not indicate support maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9) in the same band

Q1: Is Alt.1 proposal above acceptable for you?

Q2: If answer to Q1 is No, is Alt.3 proposal above acceptable for you?

Q3: If answers to both Q1 and Q2 are No, do you have any good suggestion to update Alt.1 or Alt.3 so that it can be acceptable to you (and others)?

Companies are encouraged to check above FL proposal and to provide feedback if any in below.

| Company | Comment |
| --- | --- |
| MTK | Q1: Is Alt.1 proposal above acceptable for you? - Yes. - Also, considering Min (Huawei) mentioned that maxNumberMIMO-LayersPDSCH is an R15 basic UE capability (for STRP), and infra vendors would not prefer UE to under report an R15 basic UE capability for R16 new features (supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16), which seems similar view from ZTE, we tend to think Alt. 1 is the way to go. As for Nokia’s concern on Rel-16 gNB does not know this Rel-17 UE capability, since this kind of low-cost UE would not under report maxNumberMIMO-LayersPDSCH (R15 basic UE capability) for R16 UE features, there seems no easy way to solve the issue of R16 gNB to support R17 UE. This low-cost UE can only wait for gNB upgrade to operate correctly with R17 gNB (on the features supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16). Q2: If answer to Q1 is No, is Alt.3 proposal above acceptable for you? - We can accept Alt. 3, but it seems Alt. 1 may have more potential to achieve consensus. Q3: If answers to both Q1 and Q2 are No, do you have any good suggestion to update Alt.1 or Alt.3 so that it can be acceptable to you (and others)? - (None) |
| Nokia/NSB | Q1: [Nokia/NSB] No. The problem is not only UE waiting for the network to be upgraded to make it operate correctly, but that the network will have a wrong assumption of the UE’s capabilities. It is OK if the UE waits for the upgrade without disturbing R16 operation, of course. Q2: [Nokia/NSNB] No. It is still unclear how this will operate and it has potential to create even more confusion by touching a very critical capability, which is the number of MIMO layers. Q3: [Nokia/NSB] Define a R17 version of supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16, and potentially maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9). In this way the UE signals the new behavior to the gNB supporting R17, while under-reporting or not supporting R16. That is clean and avoids NBC issues. Of course, the R17 FGs need to be properly defined to ensure there is no confusion. |
| Moderator (NTT DOCOMO) | >Nokia/NSB Regarding Alt.1 proposal, you said it is not acceptable while you also said “It is OK if the UE waits for the upgrade without disturbing R16 operation, of course”. James clarified that “This low-cost UE can only wait for gNB upgrade to operate correctly with R17 gNB (on the features supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16)”. So, if other companies supporting Alt.1 also share the same understanding with MTK, can Nokia/NSB accept Alt.1 proposal? >MediaTek and all other companies supporting Alt.1 proposal Please check the comments from Nokia/NSB and provide your feedback if any, especially on following points. - As above, whether you also share the same understanding with MTK that “This low-cost UE can only wait for gNB upgrade to operate correctly with R17 gNB (on the features supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16)” - Regarding Nokia’s alternative proposal that “Define a R17 version of supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16, and potentially maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9)”, whether this is acceptable for you and you can provide detailed FG structures to ensure there is no confusion within this week Please provide your feedback on above questions and proposals by 23:59 17th May. |
| Nokia | This part is still problematic in alt.1, as it disturbs R16 operation: - The UE that indicates support of this new feature shall not indicate support maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9) in the same band |
| MediaTek | The reason to include - The UE that indicates support of this new feature shall not indicate support maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9) in the same band in the new proposed FG is that FG16-2a-9 is an feature for advanced UE that UE can process maxNumberMIMO-LayersPDSCH per PDSCH, which is equivalent to doubled supportable MIMO layers for a 2-TRP case. For the proposed new FG, it is for low-cost UE which can only process half of the layers indicated by maxNumberMIMO-LayersPDSCH across PDSCHs in FDM case, and this kind of low-cost UE does not have the ability to support FG16-2a-9. If a UE can support FG16-2a-9, it is already more advanced than the basic case, and would not need this new FG. We (MTK) are willing to try to draft the UE feature structure as suggested by Nokia - “Define a R17 version of supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16, and potentially maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9)” But we think we first want to check whether Nokia also thinks the “R17 version of supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16” also has to support FG 16-2a-9? |
| Nokia | Regarding this question: But we think we first want to check whether Nokia also thinks the “R17 version of supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16” also has to support FG 16-2a-9? The answer is ‘no’- My reasoning is that this simpler UE should not support the R16 mTRP at all, and hence not indicating FG 16-2a-9 either. |
| MediaTek | In that case, we tried to draft the following new R17 UE features. The assumption is similar to Nokia’s previous comment that - UE signals the new behavior to the gNB supporting R17, while not supporting R16. (Hence, the signaling overhead is not increased.) We identified the following 5 features may need a R17 version to serve the intention: supportFDM-SchemeA-r167 Indicates whether UE supports single DCI based FDMSchemeA. The maximum number of spatial multiplexing layer(s) supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH. The UE that indicates support of this new feature shall not indicate support of supportFDM-SchemeA-r16 for any band.; Band; No; N/A; N/A. supportFDM-SchemeB-r167 Indicates whether UE supports single DCI based FDMSchemeB. The maximum number of spatial multiplexing layer(s) supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH. The UE that indicates support of this new feature shall not indicate support of supportFDM-SchemeB-r16 for any FSPC.; FSPC; No; N/A; N/A. multiDCI-MultiTRP-r167 Indicates whether the UE supports multi-DCI based multi-TRP and support of fully/partially overlapping PDSCHs in time and non-overlapping in frequency. This capability applies only to BWPs where two values of coresetPoolIndex are configured. The capability signalling contains the following: - maxNumberCORESET-r16 indicates maximum number of CORESETs configured per BWP per cell in addition to CORESET 0. - maxNumberCORESETPerPoolIndex-r16 indicates maximum number of CORESETs configured per coresetPoolIndex per BWP per cell in addition to CORESET 0. - maxNumberUnicastPDSCH-PerPool-r16 indicates maximum number of unicast PDSCHs per coresetPoolIndex per slot. For DL reception of multi-DCI based multi-TRP of fully/partially overlapping PDSCHs in time and non-overlapping in frequency, the maximum number of spatial multiplexing layer(s) per PDSCH supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH. The UE that indicates support of this new feature shall not indicate support of multiDCI-MultiTRP-r16 for any FSPC. The UE that indicates support of this new feature shall not indicate support of maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9) for any FSPC. NOTE 1: A UE may assume that its maximum receive timing difference between the DL transmissions from two TRPs is within a Cyclic Prefix. NOTE 2: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of coresetPoolIndex. NOTE 3: If UE reports value N1 for maxNumberCORESET-r16, that means UE supports up to min (N1+1, 5) CORESETs in total (including CORESET#0) if there is CORESET#0, and supports maximal N1 CORESETs if there is no CORESET#0. NOTE 4: If UE reports value N2 for maxNumberCORESETPerPoolIndex-r16, that means UE supports up to min (N2+1, 3) CORESETs in total (including CORESET#0) for a TRP if there is CORESET#0, and supports maximal N2 CORESETs for another TRP if there is no CORESET#0.; FSPC; No; N/A; N/A. overlapPDSCHsFullyFreqTime-r167 Indicates the maximal number of PDSCH scrambling sequences per serving cell when the UE supports PDSCHs with fully overlapping Resource Elements. The UE that indicates support of this feature shall support multiDCI-MultiTRP-r167. For DL reception of multi-DCI based multi-TRP of fully overlapping Resource Elements, the maximum number of spatial multiplexing layer(s) per PDSCH supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH. Note: A UE may assume that its maximum receive timing difference between the DL transmissions from two TRPs is within a Cyclic Prefix; Band; No; N/A; N/A. overlapPDSCHsInTimePartiallyFreq-r167 Indicates whether the UE support PDSCHs with partially overlapping Resource Elements. The UE that indicates support of this feature shall support multiDCI-MultiTRP-r167. For DL reception of multi-DCI based multi-TRP with partially overlapping Resource Elements, the maximum number of spatial multiplexing layer(s) per PDSCH supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH.; Band; No; N/A; N/A. |
| Moderator (NTT DOCOMO) | I’m afraid that I may be a bit confused about below concern from Cassio. --- This part is still problematic in alt.1, as it disturbs R16 operation: - The UE that indicates support of this new feature shall not indicate support maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9) in the same band --- As explained by James, I think that the target UE for the proposed new FG is a lower complexity UE which cannot support FG16-2a-9. If a UE can support FG16-2a-9, there is no issue and no new FG is necessary for the UE. Since the target lower complexity UE cannot support FG16-2a-9 but the UE is possible to support mTRP SDM/FDM operation only with reduced number of layers, we are trying to introduce the new FG like Alt.1. While James’s proposed FGs according to Nokia’s proposal also have similar (even stronger) conditions like below. The UE that indicates support of this new feature shall not indicate support of supportFDM-SchemeA-r16 for any band. The UE that indicates support of this new feature shall not indicate support of supportFDM-SchemeB-r16 for any FSPC. The UE that indicates support of this new feature shall not indicate support of multiDCI-MultiTRP-r16 for any FSPC. The UE that indicates support of this new feature shall not indicate support of maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9) for any FSPC. Could you please explain why the concerned note for Alt.1 is not acceptable while we can have similar notes for the Nokia’s proposed FGs? |
| Apple | In our view, if we introduce Rel-17 UE capability, now there are two proposals, both of which work without NBC with different level of under-reporting cost for the problematic UE. - Alt. 3 from FL proposal, which is essentially to allow UE to report Rel-17 version of maxNumberMIMO-LayersPDSCH to resolve NBC - The cost to the problematic UE is that UE has to under-report Rel-15/16 maxNumberMIMO-LayersPDSCH for Multi-TRP in case the UE works in a pre-Rel-17 NW - But at least, UE can still support Multi-TRP in Rel-16 NW - The new proposal from MTK, which is to replicate the basic feature group on support of FDMSchemeA/FDMSchemeB/Multi-DCI Multi-TRP. This proposal does resolves NBC - The price to pay for problematic UE is even higher, since this time, the cost is that UE cannot even support the corresponding Multi-TRP in Rel-16 NW including some case that is not even problematic - This also applies to both FR1 and FR2 based on the latest proposal from MTK. I think there is consensus that, for the problematic UE described by MTK, there is UE capability under-reporting issue given the current Rel-15/16 UE capability. It is preferable that we can introduce new Rel-17 FG so that the problematic UE can work more properly in the NW that understands the new Rel-17 FG. With that in mind, since Nokia is the major company who has concern, (or maybe ZTE as well), would it be possible for Nokia to propose some modification on top of Alt. 3 from the moderator, and then we can work on finalizing the TP to address Nokia concern - For example, on top of Alt 3, what other FG should also be duplicated to address Nokia concern? Of course, we can also go with the new proposal from MTK. |
| Ericsson | Our answer to Question 1 is Yes. We think a UE supporting the capability of Alt 1 will do some form of capability under-reporting (i.e., under reporting Rel-15/16 maxNumberMIMO-LayersPDSCH so that the UE is still operable in Rel-15/Rel-16 gNBs). In a Rel-17 gNB, the UE can be scheduled with the multi-TRP FDM A/B and multi-DCI based multi-TRP (with overlapping in time) schemes with the maximum number of layers indicated by the newly introduced capability. We prefer not to introduce a capability with reporting type FSPC. To accommodate MediaTek request, we can accept Alt 1. Also, duplicating Rel-16 multi-TRP features into Rel-17 just for the sake of supporting this type of low-cost UE implementation sounds overly complicated to us. So, we prefer not to duplicate Rel-16 multi-TRP features into Rel-17. |
| Moderator (NTT DOCOMO) | It seems that only Alt.1 has a chance to be agreed in this meeting (I assume Haitong is also fine with Alt.1). <Alt.1> - Introduce a new FG for maximum number of layers applicable to both single-DCI based multi-TRP FDM scheme A/B and multi-DCI based multi-TRP overlapped in time for Rel-17 - Type is per band, and the FG is for FR2 only - [The UE that indicates support of this new feature shall not indicate support maxMIMO-LayersForMulti-DCI-mTRP-r16 (FG16-2a-9) in the same band] So, I’d like to ask companies again that is Alt.1 above acceptable given the extensive discussion so far? In addition, is it problematic if we remove the second sub-bullet concerned by Nokia? Even if we don’t have the second sub-bullet, it is natural that UE not capable of FG16-2a-9 will not report it. If UE reports FG16-2a-9, gNB can assume the UE is capable of FG16-2a-9 irrespective of the new FG. Is this acceptable to all? |
| Apple | Thank you for the discussion. We are fine with Alt 1. On the side note, don’t we need similar sub-bullets as Alt.3 for clear definition of the new FG? I mean the following Ø Prerequisite FGs are maxNumberMIMO-LayersPDSCH and {supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16} Ø If this FG is indicated, it overwrites maxNumberMIMO-LayersPDSCH and maxMIMO-LayersForMulti-DCI-mTRP-r16, if reported, for single-DCI based multi-TRP FDM scheme A/B and multi-DCI based multi-TRP overlapped in time for the gNB that understands this FG Ø If this FG is indicated, for DL reception of single-DCI based multi-TRP FDM scheme A and scheme B, the maximum number of spatial multiplexing layer(s) supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH-r17 Ø If this FG is indicated, for DL reception of multi-DCI based multi-TRP overlapped in time, the maximum number of spatial multiplexing layer(s) per PDSCH supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH-r17 |
| MediaTek | Regarding the latest queries, our (MTK) input is: - Is Alt.1 above acceptable given the extensive discussion so far? - Alt. 1 is acceptable to us. To us it is kind of like a condensed version of the aforementioned - “R17 version of supportFDM-SchemeA-r16 / supportFDM-SchemeB-r16 / multiDCI-MultiTRP-r16 / overlapPDSCHsInTimePartiallyFreq-r16 / overlapPDSCHsFullyFreqTime-r16” (proposed by Nokia/MTK) - We are fine to go with the condensed version (Alt. 1) or the detailed R17 version (supportFDM-SchemeA-r17 …) - In addition, is it problematic if we remove the second sub-bullet concerned by Nokia? - We are fine with removing the second sub-bullet. As mentioned by moderator, it is natural that UE not capable of FG16-2a-9 will not report it. If UE reports FG16-2a-9, gNB can assume the UE is capable of FG16-2a-9 irrespective of the new FG. |
| Moderator (NTT DOCOMO) | As suggested by Haitong, let me try to clarify the proposed FG in Alt.1 as below. - Introduce a new FG for maximum number of layers applicable to both single-DCI based multi-TRP FDM scheme A/B and multi-DCI based multi-TRP overlapped in time for Rel-17 - Type is per band, and the FG is for FR2 only - Prerequisite FGs are maxNumberMIMO-LayersPDSCH and {supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16} - If this FG is indicated, for DL reception of single-DCI based multi-TRP FDM scheme A and scheme B, the maximum number of spatial multiplexing layer(s) supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH - If this FG is indicated, for DL reception of multi-DCI based multi-TRP overlapped in time, the maximum number of spatial multiplexing layer(s) per PDSCH supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH Maximum number of layers applicable to both single-DCI based multi-TRP FDM scheme A and B and multi-DCI based multi-TRP overlapped in time; 1. For DL reception of single-DCI based multi-TRP FDM scheme A and scheme B, the maximum number of spatial multiplexing layer(s) supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH 2. For DL reception of multi-DCI based multi-TRP overlapped in time, the maximum number of spatial multiplexing layer(s) per PDSCH supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH; maxNumberMIMO-LayersPDSCH, {supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16}; Per band; FR2 only. |
| Nokia | My earlier proposal was just an attempt to allow this simplified UE to realize its full potential without disturbing R16 networks. However it seems Alt.1 is indeed the only option with more traction for now, so let's keep elaborating on that one. Quoting Siva below: "We think a UE supporting the capability of Alt 1 will do some form of capability under-reporting (i.e., under reporting Rel-15/16 maxNumberMIMO-LayersPDSCH so that the UE is still operable in Rel-15/Rel-16 gNBs). In a Rel-17 gNB, the UE can be scheduled with the multi-TRP FDM A/B and multi-DCI based multi-TRP (with overlapping in time) schemes with the maximum number of layers indicated by the newly introduced capability." This is also our understanding on the intention of Alt.1 and what it should achieve. But I wonder if the most recent version of Alt.1 is achieving this intention: Ø If this FG is indicated, for DL reception of single-DCI based multi-TRP FDM scheme A and scheme B, the maximum number of spatial multiplexing layer(s) supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH Ø If this FG is indicated, for DL reception of multi-DCI based multi-TRP overlapped in time, the maximum number of spatial multiplexing layer(s) per PDSCH supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH So is it correct understanding that a UE that is already under-reporting maxNumberMIMO-LayersPDSCH will support mTRP with half the number of the layers indicted in the under-reported capability maxNumberMIMO-LayersPDSCH? This is not matching the intention explained by Siva above, i.e. that this UE would be able to realize its full potential in R17. Or do I miss something here? |
| Moderator (NTT DOCOMO) | As Cassio pointed, my latest proposal did not assume the under-reporting of maxNumberMIMO-LayersPDSCH (as in original Alt.1 intension). If we assume that the UE under-reports maxNumberMIMO-LayersPDSCH for Alt.1, it seems the concern on Alt.3 is caused as well for Alt.1, i.e., as ZTE commented, the performance of sTRP operation degrades due to the under-reporting, right? The intension of original Alt.1 would be to avoid such under-reporting of maxNumberMIMO-LayersPDSCH (for sTRP operation) while enabling the report of reduced capability (compared with FG16-2a-9) for mTRP operations. So, the target lower complexity UE we are discussing will have two possibilities of under-reporting if no new FG is introduced. - The UE under-reports maxNumberMIMO-LayersPDSCH so that UE can report the support of mTRP operations even without new FG - In this case, the performance of sTRP operation degrades and we would like to avoid this case. - The UE does not report the support of mTRP while UE reports its actual capability of maxNumberMIMO-LayersPDSCH (for sTRP operation) - For this case, if we introduce new FG (such as Alt.1 or Nokia’s proposal such as Rel-17 versions of mTRP FGs), gNB which can understand the new FG can appropriately configure mTRP operations for the UE without causing any issue on sTRP operation (since there is no under-reporting of maxNumberMIMO-LayersPDSCH). I thought all companies have same preference i.e., case 2) is better than case 1). |
| Nokia | Thank you for the explanation, so indeed there is some mismatch of assumptions from different companies. I agree that case 2 is the desirable outcome from our point of view, and I assume also for the proponents. However there is an inconsistency between case 2 and the latest proposal for the Rel-17 FG: - The UE does not report the support of mTRP while UE reports its actual capability of maxNumberMIMO-LayersPDSCH (for sTRP operation) - For this case, if we introduce new FG (such as Alt.1 or Nokia’s proposal such as Rel-17 versions of mTRP FGs), gNB which can understand the new FG can appropriately configure mTRP operations for the UE without causing any issue on sTRP operation (since there is no under-reporting of maxNumberMIMO-LayersPDSCH). However, the proposal says the following: Prerequisite FGs are maxNumberMIMO-LayersPDSCH and {supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16} The highlighted parts are contradicting each other, either R16 mTRP is a pre-requisite or it is not reported. This is the fundamental flaw in the proposal that I have been trying to point out. |
| Moderator (NTT DOCOMO) | The following description of case 2) is assuming there is no new FG. Sorry for the confusion. - The UE does not report the support of mTRP while UE reports its actual capability of maxNumberMIMO-LayersPDSCH (for sTRP operation) In case that the Alt.1 FG is introduced, UE can report the support of mTRP together with the new FG so that gNB which can understand the new FG can appropriately configure mTRP operations for the UE without causing any issue on sTRP operation. However, for gNB which cannot understand the Alt.1 FG, there may be NBC issue (over-estimation of UE’s capability for mTRP operations) as Cassio originally concerned while other companies said “This low-cost UE can only wait for gNB upgrade to operate correctly with R17 gNB (on the features supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16)”. On the other hand, in case that Nokia’s proposed FGs (Rel-17 versions of mTRP FGs) are introduced, such potential issue can also be avoided as UE does not need to report existing mTRP FGs. However, Ericsson prefers not to duplicate Rel-16 mTRP FGs into Rel-17 as it sounds overly complicated just for lower complexity UE implementation. |
| Huawei | I think that Case 2 means to report the support MTRP with actual capability of maxNumberMIMO-LayersPDSCH for sTRP operation. Otherwise the sub bullet of 2) cannot be explained. We can accept Alt 1 at most since Alt 3 has too many issues which cannot be agreed easily. Also both Alt 1 and Alt 3 cannot solve the issue of Rel-16 gNB understanding Rel-17 signalling. Please note that there is no maxNumberMIMO-LayersPDSCH for Rel16. We have only re-interpret such Rel15 signalling for Rel-16 MTRP operation if there is any ambiguity , so as for Rel-17 MTRP operation (if agree with New FG for the low-cost UE). To have a clean solution, duplicating Rel16 FGs into Rel-17 MTPRP operation seems to be only solution only for the low-cost UE, but too costly. It is similar to define a new UE cap in LTE with associated UE capabilities. For NW perspective, I don’t foresee that the low-cost UE will be rush to support Rel-16 MTRP operation since such kind of UE implementation may have other unresolved issues for Rel-15/16 legacy feature in FR2 and potential use cases of joint the low-cost UE and MTRP operation need to be tested in the field firstly. Therefore, we have relatively less concern for gNB implementation. Alt 1 is our compromise for MTK’ preferred implementation, at least it is better than nothing. |
| Moderator (NTT DOCOMO) | Again, I’m sorry for the confusion. As I explained, case 1 and case 2 descriptions are how the lower complexity UE under-reports its capability(es) assuming no new FG. On the other hand, the sub-bullet of case 2 is to explain what we can achieve if new FG(s) is introduced. I should describe that as separate case e.g., case 2A as below. Case 1) The UE under-reports maxNumberMIMO-LayersPDSCH so that UE can report the support of mTRP operations even without new FG - In this case, the performance of sTRP operation degrades and we would like to avoid this case. Case 2) The UE does not report the support of mTRP while UE reports its actual capability of maxNumberMIMO-LayersPDSCH (for sTRP operation) - In this case, the UE cannot be configured with mTRP operation. Case 2A) The UE reports the support of mTRP, its actual capability of maxNumberMIMO-LayersPDSCH (for sTRP operation) and new FG(s) - In this case, gNB which can understand the new FG can appropriately configure mTRP operations for the UE without causing any issue on sTRP operation (since there is no under-reporting of maxNumberMIMO-LayersPDSCH). Anyway, to achieve the case 2A) above, proposed FG of Alt.1 or duplicating Rel-16 mTRP FGs into Rel-17 is necessary, and I see Huawei also prefers Alt.1 compared with duplicating Rel-16 mTRP FGs into Rel-17 with same understanding as MediaTek that “This low-cost UE can only wait for gNB upgrade to operate correctly with R17 gNB (on the features supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16)”. |
| Nokia | Another point of view to this whole discussion is that this new simplified UE architecture was not considered for R16 design of mTRP, and not R17 either, and hence it can be considered for R18 TEI, if proposed eventually. Then we can have a proper discussion on a solution that doesn’t cause harm to R16 networks and allows introduction of this new feature. What we have been trying to achieve with the current discussion is to allow those UEs to at least support mTRP with gNBs supporting the R17 capability. But we are also OK to continue is as R18 TEI, in which case it will only be available in R18 RRC though. In any case, we don’t see a strong motivation to compromise stability and performance of R16 mTRP for the sake of introducing an ad-hoc support to a new kind of simplified UE that has not been properly considered and analyzed before. I hope we can still find a compromise that allows the UE to operate in R17 in a reasonable manner though, as it is not our intention to prevent any particular UE implementation, of course. |
| Moderator (NTT DOCOMO) | I see that Nokia is not ok with Alt.1 as it would cause harm to Rel-16 network such as potential NBC issue Cassio identified. In my understanding, the alternative proposal of duplicating mTRP FGs can solve Nokia’s concern, but several companies such as Huawei and Ericsson do not prefer such approach as it may be “too costly” and “overly complicated”. Although we have strived for finding a solution which can solve the issue with reasonable manner, it seems we are running out of time to discuss further in this meeting. Unless some company(es) can be further flexible or can provide a miracle proposal, we should stop the discussion in this meeting and may continue discussion in future meeting. |
| MediaTek | We (MTK) would want to have a last try by using one table to collect UE behaviors of Alt. 1 as shown below. Could Cassio assist to provide further information on which case (row) in the table below may cause harm to R16 (or R17) networks? If identified then we (MTK) may have a last chance to amend it with some revision. Define - {supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16} as “R16 mTRP” - Alt. 1 as “R17 new FG” - 0 as “not support”, 1 as “support” - N as “maxNumberMIMO-LayersPDSCH” - X as “Not defined” * SM = spatial multiplexing R16 mTRP; FG 16-2a-9; R17 new FG; UE behavior. 0; 0; 0; UE only supports R15 sTRP and would not under report N.. 0; 0; 1; X (R16 mTRP is pre-requisite of R17 new FG). 0; 1; 0; X (multiDCI-MultiTRP-r16 is pre-requisite of FG 16-2a-9). 0; 1; 1; X (multiDCI-MultiTRP-r16 is pre-requisite of FG 16-2a-9). 1; 0; 0; UE supports R16 mTRP and would not under report N. UE supported maximum number of SM layers under FDM is N.. 1; 0; 1; UE supports R16 mTRP and would not under report N. UE supported maximum number of SM layers under FDM is N/2 (low-complexity UE).. 1; 1; 0; UE supports R16 mTRP and would not under report N. UE supported maximum number of SM layers under FDM is 2*N (advanced UE).. 1; 1; 1; UE supports R16 mTRP and would not under report N. UE supported maximum number of SM layers under FDM is 2*N (advanced UE). (gNB assumes the UE is capable of FG16-2a-9 irrespective of the new FG). |
| Moderator (NTT DOCOMO) | RAN1 chair closed this email discussion. |

- Conclusion

There is no consensus to introduce new FG(s) for lower complexity UE implementation of mTRP operations in Rel-17 at RAN1#109-e meeting.

# Reference

[1]	R1-2203630	UE capability on maximum number of layers for MTRP	ZTE

[2]	R1-2204703	On UE capability of maximum number of layers for multi-TRP	MediaTek Inc.

[3]	R1-2202511	Summary on [108-e-R16-UE-features-MIMO-02]	Moderator (NTT DOCOMO, INC.)

[4]	R1-2205115	Summary on Rel-16 NR UE features related discussion	Moderator (NTT DOCOMO, INC)