3GPP TSG RAN WG1 #109-e						          R1-2205145

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.1.2

Source: 	Moderator (Intel)

Title: 	Moderator Summary for Rel.17 NR FeMIMO maintenance: Item 2d: HST

Document for:	Discussion and Decision

- Introduction

The moderator summary of the maintenance-related issues raised in the submitted contributions for Rel.17 NR_FeMIMO maintenance Item 2d: HST-SFN is given below.

An initial assessment on each of the issues is given (but can be revised based on the outcome of the discussion during the preparation week). The assessment will be used as a basis to select four issues (per chairman instruction) for further discussion in the upcoming weeks.

- High priority (H): this includes high-priority item (essential, pending issues, broken spec components) and proposed editorial changes that either enhance the clarity of the specs or correct mistakes

- Non-essential (N): this includes all other purposes such as spec optimization and low priority issues

- Editorial (E): this includes editorial issues that will be handled as editorial CRs

- Maintenance issues

The issues are summarized in Table 1. Please provide company inputs in the last column, along with preferences for alternatives (if listed). In some cases, the alternatives are listed to identify sub-cases for the main issue and multiple alternatives can be considered for email discussion if the issue is designated as “H” after company inputs. Note also that some issues have been extensively discussed in previous RAN1 meetings and may need to be concluded in this meeting.

Table 1 Summary

| # | Issue (summary of CR proposal) | Companies | FL assessment | Company inputs (if any) |
| --- | --- | --- | --- | --- |
|  | In 38.214 Section 5.1, capture agreements stating combination of SFN PDCCH scheme 1 and single-TRP PDSCH is supported, and the combination of SFN PDCCH TRP-based pre-compensation and single-TRP PDSCH is not supported - Alt-1: TP to capture related behavior in R1-2203506 (vivo), R1-2203302 (Spreadtrum) - If a UE is configured with sfnSchemePdcch set to 'sfnSchemeA' for a DL BWP and activated with two TCI states by MAC CE, and the UE does not report its capability of [nonSfnPdsch-sfnPdcch], the UE does not expect to be indicated with one TCI state in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2. - If a UE is configured with sfnSchemePdcch set to 'sfnSchemeB' for a DL BWP and activated with two TCI states by MAC CE, the UE does not expect to be indicated with one TCI state in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2. - Alt-2: TP to capture related behavior in R1-2204977 (Qualcomm) - If a UE is configured with sfnSchemePdcch set to 'sfnSchemeA' for a DL BWP and CORESET is activated with two TCI states by MAC CE, and the UE does not report its capability of [nonSfnPdsch-sfnPdcch], the UE shall be configured with sfnSchemePdsch set to 'sfnSchemeA'. - If a UE is configured with sfnSchemePdcch set to 'sfnSchemeB' for a DL BWP and CORESET is activated with two TCI states by MAC CE, the UE shall be configured with sfnSchemePdsch set to 'sfnSchemeB'. FL Note: This issue was extensively discussed in the last meeting. Since the agreements do not seem to be reflected in current specification, it seems to be a valid issue. Companies can provide further inputs along with preference of TPs proposed in the alternatives. | Vivo, Qualcomm, Spreadtrum | H | Alt-1: - Support: vivo, Spreadtrum, Apple, OPPO, Huawei/HiSi, Ericsson, Xiaomi, ZTE, CATT, Samsung, DOCOMO, Lenovo - Not Support: Alt-2: - Support: Qualcomm - Not Support: Company Views: Apple: Alt-1 OPPO: Alt-1 Huawei, HiSilicon: Alt-1 Ericsson: Alt-1 QC: Alt-2 Xiaomi: Alt-1 ZTE: Alt-1 CATT: Alt-1 vivo: Alt-1 Samsung: Alt-1. Docomo: Alt-1. Lenovo: Alt-1 |
|  | UE not capable of sTRP / SFN dynamic switching - Alt-1: When SFN is configured for PDSCH and not configured for PDCCH, dynamic switching between single TRP and SFN should be supported - Alt-2: When SFN is configured for PDSCH and not configured for PDCCH, TCI field may be present in DCI formats 1_1 and 1_2 for a UE that is not capable of dynamic switching FL Note: This issue was extensively discussed in the last meeting with no conclusion. Initial FL assessment is that this may be an essential issue for Rel-17. Companies can provide input if they think otherwise. | ZTE, Lenovo | H | Alt-1 - Support: ZTE, Huawei/HiSi, Ericsson, ZTE, DOCOMO - Not Support: QC, Samsung Alt-2 - Support: Lenovo, Apple, OPPO, Xiaomi, LGE, CATT, vivo, Spreadtrum - Not Support: QC, Samsung Company Views: Apple: Alt-2 OPPO: Alt-2 Huawei, HiSilicon: Alt-1 Ericsson: Alt-1 QC: Don’t support either alternative. Dynamic switching of PDSCH is independent whether PDCCH is SFN or not. Xiaomi: Alt-2 ZTE: Alt-1 LGE: Alt-2 CATT: We support Alt-2. But we don’t see the difference from Alt 1-2 of issue#4. [Mod: We can discuss in the next round based on issues finalized as H] vivo: Alt-2 Spreadtrum: Alt-2 Samsung: Similar view with Qualcomm. Docomo: Alt.1 Lenovo: Alt-2 |
|  | Default TCI state for PDSCH when the threshold is less than timeDurationForQCL for the following cases: - Alt-1: SFN-PDSCH is configured, and enableTwoDefaultTCI-States is configured and there is no TCI codepoint with two TCI states activated by MAC CE, and if dynamic switching between STRP and SFN is supported, - If enhanced SFN PDCCH transmission scheme 1 or TRP-based pre-compensation is configured and the lowest CORESET ID in the latest slot is indicated with two TCI states, UE applies both TCI states of the CORESET - Otherwise, UE applies the one active TCI state of the CORESET with the lowest ID in the latest slot - Alt-2: SFN-PDSCH is configured and SFN-PDCCH is not configured, enableTwoDefaultTCI-States is configured and if dynamic switching between single TRP and SFN is not supported, - UE should apply the TCI states of the first and the second lowest CORESET as the default TCI states - Alt-3: SFN-PDSCH is configured, enableTwoDefaultTCI-States is not configured - Alt-3-1: Error case - Alt-3-2: for both cases with and without TCI state field in DCI - If enhanced SFN PDCCH transmission scheme 1 or TRP-based pre-compensation is configured and the lowest CORESET ID in the latest slot is indicated with two TCI states, UE applies both TCI states of the CORESET - Otherwise, UE applies the one active TCI state of the CORESET with the lowest ID in the latest slot - Alt-3-3: UE applies the activated TCI state of the CORESET with the lowest CORESET ID in the latest slot when receiving the PDSCH. - Alt-4: SFN-PDSCH and SFN-PDCCH are configured, enableTwoDefaultTCI-States is not configured FL Note: This issue was discussed in previous RAN1 meetings, and no consensus was reached. FL assessment is that this may not be an important issue for Rel-17. Companies can provide input if they think otherwise. The alternatives list different sub-cases. One or more alternatives can be selected if the issue is discussed. Please also provide you support (or not) of the listed alternatives FL Note 2: The objective in the preparation phase is to decide if this issue should be discussed in RAN1#109-e. The detailed sub-cases and alternatives can then be further discussed as sub-issues if this issue is selected. | Vivo, ZTE, Samsung, CATT | N (N:7; H:5) | Alt-1: - Support: vivo - Not Support: Alt-2: - Support: ZTE, DOCOMO - Not Support: Alt-3-1: - Support: vivo - Not Support: Alt-3-2: - Support: Samsung, Ericsson - Not Support: Alt-3-3: - Support: CATT - Not Support: Alt-4: - Support: CATT - Not Support: Company Views: Apple: We agree with “N”. gNB can ensure the timing offset between DCI and PDSCH is greater than or equal to timeDurationForQCL for FR2. OPPO: Agree with Apple. We have discussed these default TCI state issues for meetings. We should not pay much time on them in maintenance. For these cases, UE can expect that the scheduling offset is larger than timeDurationForQCL. Huawei, HiSilicon: agree with “N”. Ericsson: Alt-3-2 QC: Agree with “N”. ZTE: Discuss as “H” and support Alt-2, since we think this issue in the problem is existing and valid. That is, if dynamic switching is not supported, UE should apply two TCI states as the default TCI states as long as SFN based PDSCH is configured. However, if only one TCI state can be applied, how to understand the UE’s behavior of applying one TCI state in the case of not supporting dynamic switching? LGE: We agree with FL’s assessment. CATT: To be consistent with Rel-16 SDM1a, we think the condition of enableTwoDefaultTCI-States should be further associated with the default beams of SFN-ed PDSC H when the scheduling offset is less than timeDurationForQCL. vivo: Ok with FL’s assessment, if can’t converge. Spreadtrum: Agree with FL’s assessment. Samsung: This issue has been discussed many times so we need to make any decision. Support Alt3-2. Docomo: Agree to discuss. We don’t agree with Apple/OPPO’s comment. In practical gNB operation in FR2, most of (or, all) cases is scheduling offset < timeDurationForQCL. We support Alt.2. Lenovo: In our opinion Alt 3 is an important case. If enableTwoDefaultTCI-States is not configured, one TCI state should be determined. Our preference is Alt 3-4, not sure why it was omitted in the middle of the email discussion [Mod] I’m sure which Alt 3-4 is being referenced. There was no Alt-3-4 in the original FL summary. |
|  | Default TCI state for PDSCH when the threshold is greater than timeDurationForQCL for the following cases: - Alt-1: SFN-PDSCH is configured and SFN-PDCCH is not configured, and if dynamic switching between single TRP and SFN is not supported - Alt-1-1: UE applies the TCI states activated for PDSCH as the default TCI states - Alt-1-2: PDSCH is not expected to be scheduled by a DCI with TCI state field absent - Alt-1-3: UE expects SFN PDCCH is configured by RRC and the CORESET that schedules PDSCH by DCI format 1_0 should be activated with two TCI states - Alt-2: SFN-PDSCH is configured and SFN-PDCCH is not configured, and if dynamic switching between single TRP and SFN is supported - Alt-2-1: UE applies QCL assumptions of the CORESET that schedules the PDSCH when receiving the PDSCH FL Note: This issue was discussed in previous RAN1 meetings, and no consensus was reached. FL assessment is that this may not be an important issue for Rel-17. Companies can provide input if they think otherwise. FL Note 2: The objective in the preparation phase is to decide if this issue should be discussed in RAN1#109-e. The detailed sub-cases and alternatives can then be further discussed as sub-issues if this issue is selected | ZTE, CATT | N (N:7; H6) | Alt-1-1: - Support: ZTE - Not Support: Alt-1-2: - Support: CATT - Not Support: Alt-1-3: - Support: vivo - Not Support: Alt-2-1: - Support: CATT - Not Support: Company Views: Apple: We agree with “N”. In this case, gNB can configure TCI field for non-fallback DCI OPPO: For Alt-1, if dynamic switching between single TRP and SFN is not supported, UE is not expected to be configured with SFN-PDSCH but not SFN-PDCCH. For Alt-2, the UE should follow the TCI state indicated in DCI. Huawei, HiSilicon：Agree with “N” Ericsson: Alt-1 QC: Agree with “N”. The TCI states should always be present in DCI for nonSFN PDCCH + SFN PDSCH. ZTE: Discuss as “H” and we support Alt-1-1. Basically, it should be clarified at first that whether to support only one codepoint containing two TCI states activated by MAC CE? It should be noticed that no TCI field is needed in such case in Rel-16, we think the backward compatibility should be considered. LGE: We agree with FL’s assessment. CATT: We think the Alt-2 in issue 2 is equivalent to Alt1-2 in this issue, that is, a PDSCH is to not expected to be scheduled by a DCI with TCI state field absent when dynamic switching between single TRP and SFN is not supported. vivo: Support to discuss it as “H”, and we should handle the remaining case with DCI 1_0. Therefore, Alt 1-3 can be added as “UE expects SFN PDCCH is configured by RRC and the CORESET that schedules PDSCH by DCI format 1_0 should be activated with two TCI states” Spreadtrum: Agree with FL’s assessment. Samsung: Agree with FL’s assessment. Docomo: OK to discuss including Alt.3 from vivo. Lenovo: Agree with FL assessment “N” |
|  | Selection of multiple BFD RS for implicit BFD-RS configuration - Alt-1: Support to keep the number of monitored BFD-RS up to 2 - Alt-1-1: When configured with two CORESETs with one or two active TCI States and the number of monitored BFD-RSs is 2, UE selects one RS from one CORESET and one RS from another CORESET. - Alt-2: Support increasing the maximum number of monitored BFD-RSs to up to 4 based on UE capability. - If all CORESETs are SFN-CORESETs, one BFD RS pair for one SFN CORESET is counted as two BFD RSs and remaining one or two BFD-RSs are selected from the first TCI state (for X = 3) or both TCI states (for X = 4) of another SFN CORESET, respectively - If CORESETs are a mix of SFN-CORESET(s) and non-SFN CORESET(s) - if X = 3, one BFD RS pair for an SFN CORESET is counted as two BFD RSs and the remaining one BFD-RS is selected from the TCI state of a non-SFN CORESET. - if X = 4, one BFD RS pair for an SFN CORESET is counted as two BFD RSs and the remaining two BFD-RSs are selected from the TCI state of a non-SFN CORESET and one TCI state from the remaining CORESET - Alt-3: Support increasing the maximum number of monitored BFD-RSs to up to 4 - The detailed rules for selecting multiple BFD RSs for implicit BFD RS configuration should be specified. - CORESET activated with two TCI states can be detected with higher priority - Alt-4: A CORESET activated with two TCI states can be detected with higher priority FL Note: This issue has been discussed in previous RAN1 meetings with no consensus. Initial FL assessment is that this may be non-essential. Companies can provide input if they think otherwise. FL Note 2: The objective in the preparation phase is to decide if this issue should be discussed in RAN1#109-e. The detailed sub-cases and alternatives can then be further discussed as sub-issues if this issue is selected | Vivo, Nokia/NSB, Lenovo, CATT, Xiaomi | N (N:8; H:6) | Alt-1: - Support: vivo, Nokia/NSB, Lenovo, OPPO, Xiaomi - Not Support: Alt-1-1: - Support: Nokia/NSB - Not Support: Alt-2: - Support: Nokia/NSB, DOCOMO - Not Support: Alt-3: - Support: CATT, DOCOMO - Not Support: Alt-4: - Support: Xiaomi, DOCOMO - Not Support: Company Views: Apple: We agree with “N”. By default, we already support Alt-1 in the current specification (38.213) OPPO: Support Alt-1. Huawei, HiSilicon：Agree with “N”. we don’t support to increase the maximum number of monitored BFD-RSs. Ericsson: N QC: Agree with “N”, no enhancement needed. Xiaomi: prefer Alt- 4 with keeping the number of monitored BFD-RS up to 2 (Alt-1). ZTE: Open to discuss. LGE: Support to discuss this issue as ‘H’. The total number of TCI states across all CORESETs in a BWP is doubled by SFN-PDCCH, so that it is critical to increase the number of BFD-RSs and/or clarify the BFD-RS selection criteria. CATT: Support to discuss this issue as ‘H’. In order to ensure that UE can monitor two or more CORESETs as much as possible, our suggestion is to extend the number of monitored BFD RSs in a BFR RS set to 4. vivo: Agree with “N”. Alt-1 is a legacy solution without any spec modification Spreadtrum: Agree with FL’s assessment. Samsung: Agree with FL’s assessment. Similar with the current specification, BFD RS selection is up to UE’s decision. Docomo: OK to discuss, and support Alt.2/3/4. Lenovo: Agree with FL assessment “N” |
|  | Explicit BFD-RS configuration - Alt-1: Reuse the existing Rel-15/Rel-16 approach for BFD RS configuration - Alt-2: Support defining CSI-RS resource or SSB pairs for BFD explicit configuration FL Note: This issue has been discussed in previous RAN1 meetings with no consensus. Initial FL assessment is that this may be non-essential i.e., Alt-1 will apply unless new agreement is achieved. Companies can provide input if they think otherwise. | Nokia/NSB, Lenovo, CATT, Xiaomi | N (N:10; H:4) | Alt-1: - Support: Nokia/NSB - Not Support: Alt-2: - Support: Lenovo, CATT, Xiaomi - Not Support: Company Views: Apple: We agree with “N”. OPPO: Agree with FL. Huawei, HiSilicon：Agree with “N” Ericsson: N QC: Agree with “N”, Alt-1 is default. Xiaomi: to align with implicit BFD-RS configuration which considers RSs in two TCI states of one CORESET, Alt-2 should be supported. ZTE: Discuss as H since this issue is valid. Support Alt-2. LGE: We agree with FL’s assessment. CATT: Agree with “N” vivo: Agree with “N” Spreadtrum: Agree with FL’s assessment. Samsung: Agree with FL’s assessment. Docomo: OK to discuss. Lenovo: Support Alt-2 for enhancement |
|  | NBI RS configuration when two TCI states are configured for a CORESET - Alt-1: reusing the existing NBI-RS configuration without NBI-RS pair when SFN transmission is configured in Rel-17 - Alt-2: two new beam identification CSI-RS resource sets / new beam identification CSI-RS resource pairs or SSB pairs - (Nokia/NSB): The UE can be configured with additional information that indicates a candidate beam or a candidate beam pair (pairs) that support SFN operation. If the UE indicates candidate beam or candidate beam pair for beam failure that supports SFN operation, the UE can continue with SFN operation after BFR. FL Note: This issue has been discussed in previous RAN1 meeting with no consensus. Initial FL assessment is that this may be non-essential. Companies can provide input if they think otherwise. | Vivo, Nokia/NSB, Lenovo | N (N:10; H:3) | Alt-1: - Support: vivo - Not Support: Alt-2: - Support: Nokia/NSB, Lenovo - Not Support: Company Views: Apple: We agree with “N”. OPPO: Agree with FL. Huawei, HiSilicon：Agree with “N” Ericsson: N QC: Agree with “N”. Xiaomi: prefer Alt-2. Thus candidate beam pairs can be found with high probability. ZTE: Discuss as H and support Alt-2. As the infra vendor, we do believe this issue in the problem is valid and will avoid unnecessary scheduling latency of re-configuration. LGE: We agree with FL’s assessment. CATT: N vivo: Agree with “N” Spreadtrum: Agree with FL’s assessment. Docomo: Agree with N. Lenovo: Support Alt-2 for enhancement |
|  | SFN for CORESETS associated with CSS Type 0/0A/1/2 - Alt-1: UE doesn’t expect PDCCH candidates in CSS type 0/0A/1/2 to be associated with CORESET activated with two TCI states. - Alt-2: Apply only the first TCI state for PDCCH reception - Alt-3: Apply both TCI states for PDCCH reception FL Note: This issue was extensively discussed in previous meetings with no consensus. Companies can provide further inputs. Please also provide inputs on alternatives listed | Vivo, Qualcomm, Nokia/NSB, Lenovo | H (N:3;H:10) | Alt-1: - Support: Qualcomm, OPPO - Not Support: Alt-2: - Support: vivo, Nokia/NSB, Lenovo, OPPO, Huawei/HiSi, LGE, Samsung, DOCOMO - Not Support: Alt-3: - Support: ZTE - Not Support: Company Views: Apple: We prefer not to discuss. We do not think we need any specification change. i.e., we following whatever gNB configures subject to our capability. We do not need to introduce search space based TCI configuration. OPPO：We are fine with either Alt-1 or Alt-2. Huawei, HiSilicon：Support Alt-2 Ericsson: N QC: Alt-1 ZTE: Agree with FL’s assessment, support Alt-3. LGE: Support Alt-2. CATT: N vivo: Agree with ‘H’ and support Alt-2. When CSS (especially SS#0) associated with SFN CORESET#0, the UE behavior for reception of SI, paging information should be specified. Spreadtrum: Fine to discuss. Samsung: Agree with ‘H’, support Alt-2. Docomo: Support Alt.2. Lenovo: Support Alt-2 |
|  | CORESET#0 configured with two TCI states - Alt-1: Only one TCI state should be used for CSS reception - Alt-2: Both TCI states should be used for CSS reception FL Note: This issue is based on reply-LS to RAN2 in the last meeting. Further inputs from companies are needed. | ZTE | H (N:3; H:8) | Alt-1: - Support: OPPO, QC, ZTE, DOCOMO, vivo, Lenovo - Not Support: Alt-2: - Support:, LGE - Not Support: Company Views: Apple: We prefer not to discuss. We do not think we need any specification change. i.e., we following whatever gNB configures subject to our capability. We do not need to introduce search space based TCI configuration. OPPO: Alt-1 is preferred. Ericsson: N QC: Alt-1 based on UE capability. ZTE: Agree with FL’s assessment and support Alt-1. LGE: Support Alt-2. CATT: N vivo: Agree with ‘H’, and support Alt-1. Spreadtrum: Fine to discuss. Docomo: Support Alt.1. Lenovo: Alt-1 is preferred |
|  | Specification based RLM RS selection rule for CORESETs activated with two TCI states - Alt-1: CORESET with two activated TCI states have higher priority - Alt-2: Apply the same selection rule for RLM-RS and BFD-RS if the number of configured RS exceeds the respective maximum value FL Note: Specification based RLM RS selection rule was briefly discussed in the last meeting, and the discussion was concluded without any conclusion. Initial FL assessment is that it may not be an essential issue. Companies can provide further inputs if they think otherwise. | Samsung, Nokia/NSB | N (N:9; H:4) | Alt-1: - Support: Samsung - Not Support: Alt-2: - Support: Nokia/NSB, ZTE - Not Support: Company Views: Apple: We agree with “N”. OPPO: We agree with “N”. Huawei, HiSilicon：Agree with “N” Ericsson: N QC: agree with “N”. ZTE: Discuss as H and support Alt-2. We think this should be discussed with issue#5 together. LGE: Support to discuss this issue as ‘H’. We think that this issue can be combined with issue #5. CATT: N vivo: Agree with “N” Spreadtrum: Agree with FL’s assessment. Samsung: RLM RS selection rule is defined in current specification. Hence, it should be defined considering CORESETs with two TCI states. Docomo: Support to discuss. Either Alt.1/2 is fine. Lenovo: Agree with “N” |
|  | In clause 5.2.1.5.1, the text description for specifying the TCI state for the reception of the aperiodic CSI-RS when the CORESET with lowest ID is indicated with two TCSI states has extra “if” wording that should be deleted. FL Note: This seems to be an editorial update. | Qualcomm | E | Company Views: Apple: We agree with “E”. OPPO: We agree with “E”. Huawei, HiSilicon：Agree with “E” Ericsson: E QC: agree with “E”. ZTE: Agree with “E”. LGE: Fine with the TP CATT: E vivo: Agree with “E”. Spreadtrum: Agree with E Samsung: Agree with “E” Docomo: Agree with E. Lenovo: Agree with “E” |
|  | SFN CORESET before reception of MAC-CE: - For a CORESET that is indicated with SFN mode by higher layer signalling and RRC-configured with only two TCI states, the UE assumes that the DM-RS antenna port associated with PDCCH receptions in the CORESET are QCLed with the DL RSs in the two TCI states FL Note: This issue was discussed and not agreed in the last meeting. Companies can provide input to check if the situation has changed. Initial FL assessment based on discussion in the last meeting is that this issue may not be essential. | Qualcomm | N (N:8; H:3; E:1) | Company Views: OPPO: Agree with FL. Huawei, HiSilicon：Agree with “N” Ericsson: E QC: This should be “Essential topic. ZTE: Agree with FL’s assessment. LGE: We agree with FL’s assessment. CATT: Agree with FL. vivo: Agree with ‘N’ Spreadtrum: Fine to have the clarification. Samsung: Agree with FL’s assessment. Docomo: Support the proposal. Lenovo: Agree with FL’s assessment |
|  | Default TCI for single-TRP PDSCH reception TP for 38.214 Section 5.1.5 to capture the case when CORESET which is activated one or two TCI states overlaps with a scheduled single-TRP PDSCH and is located in same or different component carrier with the scheduled single-TRP PDSCH - If the overlapped CORESET has two TCI states, the UE is expected to prioritize the reception of PDCCH associated with the TCI states of that CORESET. FL Note: This issue has not been discussed before. This can benefit from further discussion and hence initial FL assessment is H. Please provide inputs and the assessment will be revised based on inputs. | Samsung | N (N:10; H:4) | Apple: We do not think it is needed. Don’t we have the similar sentence before, i.e., " the UE is expected to prioritize the reception of PDCCH associated with that CORESET” OPPO: We also think overlapping of CORESET and PDSCH exists in Rel-15/16. Similar rule as Rel-15/16 can be reused regardless of SFN transmission. Huawei, HiSilicon: We also think the legacy rule can be reused here. Ericsson: H QC: non needed. Reuse legacy rules is enough. Xiaomi: Legacy rule can be reused. ZTE: Open to discuss. Further, one case is needed additionally, that is: if the TCI state of PDSCH is the same with one of the two TCI states of the overlapped CORESET, the PDSCH should have the same priority. LGE: We don’t think further discussion is needed. CATT: N. vivo: N. Legacy rule can be reused. Spreadtrum: we also think reuse legacy rule is enough Samsung: We think that this issue should be at least “E”. Docomo: Agree with H. Lenovo: We think legacy rule can be reused |
|  | Default TCI for Rel-17 SFN PDSCH reception TP for 38.214 Section 5.1.5 to capture the case of reception for Rel-17 SFN PDSCH with a CORESET, which is activated one or two TCI states, is overlapped and located in same or different component carrier with the scheduled Rel-17 SFN PDSCH - In this case, if the qcl-Type is set to 'typeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. If the overlapped CORESET has two TCI states, the UE is expected to prioritize the reception of PDCCH associated with the first TCI state of that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers). FL Note: This issue has not been discussed before. This can benefit from further discussion and hence initial FL assessment is H. Please provide inputs and the assessment will be revised based on inputs. | Samsung | N (N:10; H:4) | Apple: We do not think it is needed. When gNB configures UE to monitor a CORESET with two TCI states, it means UE is capable of SFN PDCCH. When we prioritize PDCCH TCI over PDSCH TCI, we do not need to drop one of the TCI of PDCCH OPPO: We think overlapping of CORESET and PDSCH exists in Rel-15/16. Similar rule as Rel-15/16 can be reused regardless of SFN transmission. Huawei, HiSilicon: we also think the legacy rule can be reused here. Ericsson: H QC: non needed. Reuse legacy rules is enough Xiaomi: Legacy rule can be reused. ZTE: Open to discuss. In addition, we think the ‘difference’ between PDSCH and PDCCH should be clarified. For simplicity, if any one TCI state of the PDSCH is different from the TCI state of PDCCH, the PDCCH should have higher priority. LGE: We don’t think further discussion is needed. CATT: Agree with Huawei. vivo: N. Legacy rule can be reused Spreadtrum: we also think reuse legacy rule is enough Samsung: We think that this issue should be at least “E”. Docomo: Agree with H. Lenovo: We also think legacy rule can be reused |

- Observation

From the inputs shared by participating companies during the preparation phase, the following observation can be made:

- The following issue can be handled as E (a part of editorial CR): Issue 11

- The following issues can be designated as H (requiring discussion and additional agreements/conclusions): 1, 2, 8 and 9

- The following issues can be designated as N (non-essential) but can be discussed again in future meetings: 13, 14 (these were new issues for this meeting)

- The following issues can be designated as N (non-essential) and have been discussed in previous meeting(s): 3, 4, 5, 6, 7, 8, 9, 10, 12 (all issues have been discussed previously without consensus and the situation has not changed from more than one meeting)

# References

|  | R1-2203106 | Remaining issues on enhancements for Multi-TRP deployment | Huawei, HiSilicon |
| --- | --- | --- | --- |
|  | R1-2203259 | Remaining issues on multi-TRP deployment | ZTE |
|  | R1-2203302 | Remaining issues on multi-TRP deployment | Spreadtrum Communications |
|  | R1-2203422 | Maintenance issues on Rel-17 M-TRP enhancements | CATT |
|  | R1-2203506 | Maintenance on enhancements for multi-TRP Deployment | vivo |
|  | R1-2203644 | Remaining issues for Multi-TRP Deployment | Ericsson |
|  | R1-2203674 | Discussion on remaining issues for multi-TRP deployment | NEC |
|  | R1-2203765 | Maintenance of Enhancements for Multi-TRP Deployment | Langbo |
|  | R1-2203772 | Remaining issues on HST-SFN deployment enhancement | xiaomi |
|  | R1-2203856 | Maintenance on Rel-17 multi-TRP and HST-SFN | Samsung |
|  | R1-2203949 | Maintenance on enhancements for mTRP deployment | OPPO |
|  | R1-2204138 | Text proposal on multi-TRP deployment | LG Electronics |
|  | R1-2204170 | Remaining issues on multi-TRP deployment | Lenovo |
|  | R1-2204193 | Remaining issues for mTRP PUSCH | ASUSTeK |
|  | R1-2204200 | Remaining Issues on Multi-TRP Enhancement | Apple |
|  | R1-2204336 | Remaining issues on enhancements for Multi-TRP Deployment | NTT DOCOMO, INC. |
|  | R1-2204536 | Maintenance of enhancements for Multi-TRP Deployment | Nokia, Nokia Shanghai Bell |
|  | R1-2204683 | Maintenance of Rel-17 beam management for multi-TRP | MediaTek Inc. |
|  | R1-2204764 | MIMO Enhancements for Multi-TRP Deployment | Intel Corporation |
|  | R1-2204977 | Remaining Details for Multi-TRP Operation | Qualcomm Incorporated |