3GPP TSG RAN WG1 #109-e		                                                      R1-2205146

e-Meeting, May 9th – 20th, 2022

Source:	Moderator (CATT)

Title:	Preparation phase FL summary for Rel-17 URLLC/IIoT intra-UE MUX A

Agenda Item:	8.3.2

Document for:	Discussion and Decision

- Introduction

This document summarizes the remaining issues on Rel-17 URLLC/IIoT intra-UE MUX A based on the input contributions in section 2. Companies’ views on whether the issues in section 2 should be discussed in RAN1#109-e are provided in section 3. The final RAN1#109-e scope based on Mr. Chair’s guidance can be found in section 4.

- Issue list based on input contributions

  - Issue 1: power allocation order

Huawei [1] and vivo [8] discussed to update power allocation order considering PUSCH multiplexed with UCI of a different priority with essentially the same proposal.

Huawei’s proposal is as follows.

Proposal 6: Update the transmission power allocation order for Rel-17 by considering inter-priority UCI-on-PUSCH cases: - LP PUSCH with HP HARQ-ACK should be of the same priority as HP PUSCH with HP HARQ-ACK, i.e., higher than HP PUSCH with CSI, as well as HP PUSCH only. - LP HARQ-ACK on HP PUSCH should of the same priority as HP PUSCH only, i.e., lower than HP PUSCH with HP HARQ-ACK, as well as HP PUSCH with CSI.

Vivo’s proposal is as follows.

Proposal 2: For HP HARQ-ACK on a LP PUSCH, power allocation order should be the same as HP PUSCH with HP HARQ-ACK. For LP HARQ-ACK on HP PUSCH, power allocation order should be the same as HP PUSCH only.

  - Issue 2: Joint operation of R17 intra-UE multiplexing and UL CI

InterDigital [11], ETRI [17] and Qualcomm [19] discussed joint operation of Rel-17 intra-UE multiplexing and UL CI.

InterDigital [11] proposed that Rel-17 intra-UE multiplexing and UL CI can be configured simultaneously and LP PUSCH multiplexed with HP HARQ-ACK should not be cancelled with a proposed TP.

Proposal 1: A UE can be configured with both UCI-MuxWithDifferentPriority and UplinkCancellation. Proposal 2: A UE that detects CI and is provided uplinkCancellationPriority cancels a PUSCH if the PUSCH is with priority 0 and if no HARQ-ACK information of priority 1 is multiplexed in the PUSCH.

ETRI [17] also proposed that PUSCH carrying HP UCI is not cancelled by UL CI with a proposed TP.

Proposal 4:The PUSCH carrying HP UCI may not be cancelled by the ULCI.

Qualcomm [19] proposed that PUSCH cancellation due to UL CI is performed after step 2 of Rel-17 intra-UE multiplexing. In addition, similar as InterDigital and ETRI, Qualcomm proposed that a LP PUSCH with HP UCI should be exempt from UL-CI.

Proposal 2: For Rel-17 intra-UE multiplexing, PUSCH cancellation due to UL-CI are performed after step 2 of Rel-17 intra-UE multiplexing. - A LP PUSCH has HP UCI multiplexing on it is exempt from UL-CI, if RRC parameter uplinkCancellationPriority is configured.

  - Issue 3: TP for simultaneous PUCCH/PUSCH transmission

Huawei [1] proposed the following text proposal for simultaneous PUCCH/PUSCH transmission to avoid ‘adding’ and ‘excluding’ behavior. To be more specific, Huawei thinks that given that the capability of simultaneous PUCCH/PUSCH transmission is reported per band combination, it is straightforward for a UE to perform the inter-priority multiplexing/prioritization only within a group of bands that does not support simultaneous transmission and ignore the band(s) that support the simultaneous transmission from the multiplexing/prioritization procedure. However, the current spec requires the UE to first add all PUSCHs from all bands, then exclude the PUSCH on the band that does not support simultaneous transmission with the PUCCH.

Proposal 7: Adopt the following TP for TS38.213 ------------------ Text Proposal for 38.213 v17.1.0 Clause 9 ------------------ 9 UE procedure for reporting control information *** Unchanged text is omitted *** If a UE - is provided simultaneousPUCCH-PUSCH and would transmit a PUCCH with a first priority index and PUSCHs with a second priority index that is different than the first priority index, where the PUCCH and the PUSCHs overlap in time - can simultaneously transmit the PUCCH on the first cell and the PUSCHs on a set of second cells [16, TS 38.306], the UE excludes the PUSCHs for resolvesing the time overlapping between the PUCCH and PUSCHs on cells that do not belong to the set of second cells if any in the remaining of this clause, where the timeline conditions are not required for the PUCCH on the first cell and excluded PUSCHs on the set of second cells. *** Unchanged text is omitted ***.

  - Issue 4: TP for intra-UE multiplexing

Huawei [1] proposed the following text proposals for intra-UE multiplexing for both step 2.1 and 2.2.

## 4.2 Multiplexing for Step 2.1 Issue 1. It has been agreed that the LP PUCCH is associated with the first overlapped subslot of HP PUCCH, and if the LP PUCCH is not multiplexed/dropped after the association with the first overlapped subslot, LP PUCCH would associate to the next HP PUCCH subslot. The current text separates this procedure into disconnected sub-bullets, which impacts its readability. Hence some changes are given by adjusting the order of the sub-bullets to make the procedure more clearly addressed. Issue 2. If the UE does not multiplex or drop the LP PUCCH at the current HP time unit, it will be associated to the next HP time unit. In the current text, it says ‘the UCI of the first PUCCH transmission is not multiplexed in a second PUCCH transmission of larger priority index’, which is not clear what type of UCI it is. E.g., if the LP PUCCH includes HARQ-ACK and CSI, and LP HARQ-ACK is multiplexed into the current HP time unit while the CSI is dropped, then it is ambiguous whether the UCI is multiplexed or not. The correction is to replace this part with ‘no UCI of the first PUCCH transmission is multiplexed…’. Issue 3. For the second but last sub-bullet, it is not clear whether ‘when the second priority index is larger than the first priority index’ is applied to only the latter part of the sentence (after ‘, or’) or both parts. If it is applied to only the latter part, then the two parts would redundantly include the case of overlapping with LP PUCCH with HARQ-ACK. The editorial suggestion is to change ‘first’/‘second’ priority index to ‘smaller’/‘larger’ priority index so that the former part and the latter part can describe orthogonal cases. So a text proposal is given as follows Proposal 8: Adopt the following TP for TS38.213 ------------------ Text Proposal for 38.213 v17.1.0 Clause 9 ------------------ # 9 UE procedure for reporting control information *** Unchanged text is omitted *** - second, the UE resolves the overlapping for PUCCH transmissions of different priority indexes, and - if the UE is provided subslotLengthForPUCCH in the second PUCCH-Config, a first PUCCH transmission of smaller priority index is associated with the first overlapping slot with subslotLengthForPUCCH symbols of larger priority index; otherwise, the PUCCH transmission of smaller priority index is associated with the overlapping slot with Nsymslot symbols [4, TS 38.211] of larger priority index. - if the UE determines that a the first PUCCH transmission of the smaller priority index is not dropped and the no UCI of the first PUCCH transmission is not multiplexed in a second PUCCH transmission of larger priority index in an overlapping slot with subslotLengthForPUCCH symbols, the first PUCCH transmission is associated with the next overlapping slot with subslotLengthForPUCCH symbols for PUCCH transmissions with the larger priority index - for each overlapping slot after the association of the first PUCCH transmission, - the UE first resolves the overlapping for PUCCH transmissions, where at least one of the PUCCH transmissions is with NPUCCHrepeat>1 repetitions, within a slot of larger priority index as is subsequently described in this clause, if any, and then the UE resolves the overlapping for PUCCH transmissions without repetitions within the slot using the pseudo-code in clause 9.2.5 - if the UE determines that a first PUCCH transmission of the smaller priority index is not dropped and the UCI of the first PUCCH transmission is not multiplexed in a second PUCCH transmission of larger priority index in an overlapping slot with subslotLengthForPUCCH symbols, the first PUCCH transmission is associated with the next overlapping slot with subslotLengthForPUCCH symbols for PUCCH transmissions with the larger priority index - the UE does not expect a PUCCH transmission that includes UCI of different priority indexes to overlap with a PUCCH transmission with NPUCCHrepeat>1 repetitions after resolving the overlapping for PUCCH transmissions without repetitions within a slot - the UE does not expect a PUCCH transmission with UCI of first smaller and second larger priority indexes to overlap with a PUCCH transmission with HARQ-ACK information of the first smaller priority index, or with a PUCCH transmission or with a PUSCH transmission of the second larger priority index when the second priority index is larger than the first priority index - the UE does not expect a PUCCH transmission with HARQ-ACK information of larger priority index to overlap with more than one PUCCH transmissions with HARQ-ACK information of smaller priority index *** Unchanged text is omitted ***. ## 4.3 Multiplexing for Step 2.2 In Step 2.2, the text “the UE drops PUSCH transmissions of smaller priority index that overlap with a PUCCH transmission with NPUCCHrepeat>1 repetitions of larger priority index prior to multiplexing UCI in a PUSCH transmission of smaller priority index, if any” addresses the behavior of dropping of repetition, which is redundant with a later paragraph as below. - else - if the UE would transmit the following channels that would overlap in time where, if a channel transmission is with repetitions, the following are applicable per repetition - a first PUCCH transmission of larger priority index and a second PUCCH transmission of smaller priority index - a first PUCCH transmission of larger priority index and a second PUSCH transmission of smaller priority index when the UE cannot simultaneously transmit the first PUCCH and second PUSCH - a first PUCCH transmission of smaller priority index and a second PUSCH transmission of larger priority index when the UE cannot simultaneously transmit the first PUCCH and second PUSCH the UE - transmits the PUCCH or the PUSCH of the larger priority index, and - does not transmit a PUCCH or a PUSCH of smaller priority index. Thus a text proposal is given as follows. Proposal 9: Adopt the following TP for TS38.213 ------------------ Text Proposal for 38.213 v17.1.0 Clause 9 ------------------ # 9 UE procedure for reporting control information *** Unchanged text is omitted *** - third, the UE resolves the overlapping for PUCCH and PUSCH transmissions of different priority indexes - the UE drops PUSCH transmissions of smaller priority index that overlap with a PUCCH transmission with positive SR of larger priority index prior to multiplexing UCI in a PUSCH transmission of smaller priority index, if any - the UE drops PUSCH transmissions of smaller priority index that overlap with a PUCCH transmission with NPUCCHrepeat>1 repetitions of larger priority index as subsequently described in this clause prior to multiplexing UCI in a PUSCH transmission of smaller priority index, if any - the UE multiplexes HARQ-ACK information in a PUSCH transmission, as is subsequently described in this clause for multiplexing HARQ-ACK information from a PUCCH transmission in a PUSCH transmission of a same priority index, if a PUCCH transmission with HARQ-ACK information of a first priority index overlaps with one or more PUSCH transmissions of a second priority index that is different than the first priority index *** Unchanged text is omitted ***.

  - Issue 5: Processing order of intra-UE multiplexing and PUSCH dropping

Samsung [9] proposed the following proposal for processing order of intra-UE multiplexing and PUSCH dropping.

Proposal 3: For resolving collision of overlapping PUCCHs/PUSCHs with different priorities in R17, drop PUSCHs colliding with semi-static DL symbols before intra-UE multiplexing/prioritization.

It is moderator’s understanding that this issue was discussed in RAN1#108-e with the following agreement so there is no need for additional discussion.

Agreement For Rel-17 intra-UE multiplexing, PUCCH and PUSCH cancellation due to dynamic SFI, semi-static DL symbols and SSB symbols are performed after step 2 of Rel-17 intra-UE multiplexing.

  - Issue 6: Priority of resultant PUCCH/PUSCH

OPPO [10] proposed to specify priority of resultant PUCCH/PUSCH for Rel-17 intra-UE multiplexing.

It seems to moderator that the proposal is related to issue 1, issue 2 and issue 9 and can be discussed with respective issue if agreed by the group.

In Rel-16, if PHY priority is configured for a UE, a PUSCH or a PUCCH is either of priority index 0 or of priority index 1. The priority of uplink channel is not only used for Rel-16 intra-UE prioritization, but also used for: - Inter-UE uplink cancellation; - Prioritizations for transmission power reductions; and - Prioritization between SL HARQ-ACK information in a PUCCH and DL HARQ-ACK or SR or CSI in a PUCCH. In order to keep minimize impact on other procedures, we support that the priority of the resultant PUCCH/PUSCH for Rel-17 intra-UE multiplexing should be specified. Proposal 1: The priority of the resultant PUCCH/PUSCH for Rel-17 intra-UE multiplexing should be specified.

  - Issue 7: Number of PUCCH resources after UCI multiplexing

Apple [12] discussed the issue that it is not clear when PUCCHs should be trimmed to ensure up to 2 PUCCHs per PHY priority in a slot/sub-slot and proposed to clarify.

In TS 38.213 release 15, it is prescribed there can be up to 2 PUCCHs in a slot, e.g. two short PUCCHs, one short PUCCH + one long PUCCH, etc. With the introduction of sub-slot based feedback, then such a prescription also applies to sub-slot based transmissions. It is noted that on the outcome of PUCCH multiplexing, i.e. resources Z, there is no restriction on the number of PUCCHs generated. Actually, it is not clear where in the PUCCH multiplexing flow, PUCCHs should be trimmed to satisfy the requirement of allowing up to 2 PUCCHs in a slot/sub-slot, e.g. prior to UCI multiplexing over PUSCH or after UCI multiplexing over PUSCH. In Rel-15, such a question was raised. The situation may become more severe as in Rel-17 PUCCH multiplexing is executed three times: two times in Step 2.1 (one for LP PUCCHs, another for HP PUCCHs), and one time in Step 2.2 (for LP PUCCH(s) and HP PUCCH(s)). And UCI multiplexing over PUSCH is also executed three times: two times in Step 1.2 and one time in Step 2.2. Any different processing/assumption between gNB and UE in those procedures concerning the two PUCCH restriction could lead to misunderstanding between gNB and UE. From that, we feel it is important to clarify the trimming procedure to satisfy the two PUCCHs per slot/sub-slot requirement. We have Proposal 2: Clarify the number of PUCCH resources Z after PUCCH UCI multiplexing.

  - Issue 8: Dynamic enabling/disabling of Rel-17 intra-UE multiplexing

ITRI [14] proposed a couple of proposals on dynamic enabling/disabling of Rel-17 intra-UE multiplexing. It is moderator’s understanding that dynamic enabling/disabling of Rel-17 intra-UE multiplexing is not supported so that no discussion is needed.

Proposal 1: Dynamic indication in multiple DCIs associated with a group of overlapping channels should be consistent. Proposal 2: UE should not expect to receive a dynamic indication resulting in demultiplexing of previously multiplexed PUCCHs/PUSCHs channels. Proposal 3: An UL channel associated with a DCI disabling multiplexing can collide with another UL channel of a same priority if dynamic indication in multiple DCIs associated with a group of overlapping channels are consistent. Proposal 4: The configuration of prioritization/multiplexing for channels without dynamic indication can be determined according to the dynamic indication of validation DCI, if any. Otherwise, the UE should always assume the Rel-17 multiplexing is enabled for the channels, the multiplexing timeline should be satisfied and handled by gNB configuration. Proposal 5: If a set of UL channels without dynamic indication are decided to be multiplexed, UE should not expect the set of UL channel would overlap with other UL channel which is dynamically indicated to disable the multiplexing. Proposal 6: Confirm the working assumption of UE Capability #3 to support dynamically enable/disable the Rel-17 multiplexing in step 2.

  - Issue 9: Joint operation of R17 URLLC/IIoT and sidelink

ETRI [17] has the following proposals on joint operation with sidelink.

Proposal 1: Simultaneous transmission of both PUSCH and PUCCH can be supported in the joint operation of sidelink and IIoT. Proposal 2:The previous conclusion is still valid in Rel-17 in the same priority: No support of multiplexing of SL HARQ and Uu UCI on PUCCH or PUSCH in Rel-16. Proposal 3: It is allowed that the joint configuration between two features and the serving gNB guarantee to no multiplex sidelink HARQ and Uu UCI.

  - Issue 10: Step 2 considers resultant PUCCH/PUSCH after step 1 only

In addition to the case of a HP PUCCH with PF 0/1 overlapping with LP HARQ-ACK, Samsung [9] discussed another case shown in Figure 2 below and has a proposal that for resolving collision of PUCCHs and/or PUSCHs of different priorities, Step 2 only considers the resulting PUCCHs and/or PUSCHs in Step 1.

In RAN1#108-e, overlapping of a HP PUCCH PF 0/1 with HP HARQ-ACK and overlapping of HP SR with LP PUCCH with HARQ-ACK were discussed. That case may happen in Step 2 of intra-UE multiplexing. For example, in step 1, a HP PUCCH with PF 0/1 with HARQ-ACK overlaps with at least one HP PUCCH with SR, and HARQ-ACK and SR are multiplexed in a same HP PUCCH. In step 2, the HP PUCCH with HARQ-ACK and SR overlaps with a LP PUCCH with HARQ-ACK. There are two candidate options. Option 0: LP HARQ-ACK is dropped when HP SR is positive. Option 2: 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1 Option 2 requires a UE to differentiate the following cases shown in Figure 1. Case a) Case b) Figure 1 For negative SR, the result is same for intra-UE multiplexing of same priority in Step 1. However, UE behaviour is different for both cases in Step 2. After multiplexing in Step 2, the multiplexed UCI only includes HP HARQ-ACK and LP HARQ-ACK for case a), but for Case b) the multiplexed UCI includes 1 more bit for SR. A similar case is given in Figure 2. In Step 1, HP SR is dropped regardless of positive or negative SR. In Step 2, consideration of the HP SR only complicates UE implementation without any gain. Figure 2 In previous meetings discussion for the interaction between Step 1 and Step 2, to ease UE implementation one principle is that UE does not go back to Step 1 when resolving collisions in Step 2. Similarly, to facilitate UE implementation, using the information of PUCCHs/PUSCHs before multiplexing in Step 1 should be avoided when resolving collisions in Step 2. Only considering the PUCCHs and/or PUSCHs in Step 1 when resolving collision of PUCCHs and/or PUSCHs of different priorities can reduce spec impact. Proposal 1: For resolving collision of PUCCHs and/or PUSCHs of different priorities, Step 2 only considers the resulting PUCCHs and/or PUSCHs in Step 1.

- Companies views on whether to discuss the issues in section 2

Companies please indicate whether you think the issues listed in section 2 should be discussed in RAN1#109-e by indicating ‘Y’ or ‘N’ in the following table.

| Company | Issue 1 | Issue 2 | Issue 3 | Issue 4 | Issue 5 | Issue 6 | Issue 7 | Issue 8 | Issue 9 | Issue 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Samsung | Y | N | N | N | Y | N | N | N | N | Y |
| HW | Y | N | Y | Y | N | N | N | N | N | Y |
| vivo | Y | N | N | N | N | N | N | N | N | Y |
| Intel | N | N | N | N | N | N | N | N | N | Y |
| QC | Y | Y | N | N | N | Y | N | N | N | Y |
| OPPO | Y | Y |  |  | N | Y |  | N | Y |  |
| DOCOMO | Y | N | Y | N | N | Y | N | N | N | Y |
| LG | Y | N | N | N | N | N |  | N | N | Y |
| New H3C | Y | N | N | N | N | N | N | N | N | Y |
| Nokia/NSB | Y | Y | N | N | N | N | N | N | N | Y |
| Spreadtrum | Y | Y | Y | N | N | Y | N | N | N | Y |
| CATT | Y | N | N | N | N | N | N | N | N | Y |
| InterDigital | Y | Y | N | N | N | Y | N | N | N | Y |
| ZTE | Y | Y | N | N | N | N | N | N | N | Y |
| Ericsson | Y | N | N | N | N | N | N | N | N | Y |
| Sony | Y | Y | N | N | N | Y | N | N | N | Y |

Please provide your comments if any in the following table.

| Company | Comments |
| --- | --- |
| Samsung | For Issue 2, It is an optimization and can be handled by the gNB through the indication of the time-frequency resources by UL CI (i.e. exclude the LP PUSCH with the HP HARQ-ACK – the proposal is anyway to not cancel it). For Issue 3, the clarification is not necessary, the current spec is clear. For Issue 4, Issue 1) the current spec is clear, it first clarifies the time unit and then clarifies the order of multiplexing with/without repetitions. After that it further clarifies UE behaviour if the collision is not addressed. Issue 2) the current spec is clear, regarding the example given by Huawei, clearly LP HARQ-ACK is multiplexed and the condition is not satisfied, no deferral. Issue 3) the current spec is clear, this is one sentence the condition applies to both. For Issue 6, not necessary, UE behaviour is clear. For Issue 7, it is clear that there are up to 2 PUCCHs per priority according to the spec below. In the remaining of this clause, a UE multiplexes UCIs with same priority index in a PUCCH or a PUSCH before considering limitations for UE transmission as described in clause 11.1, clause 11.1.1, and clause 17.2. A PUCCH or a PUSCH is assumed to have a same priority index as a priority index of UCIs a UE multiplexes in the PUCCH or the PUSCH.. For Issue 8 and 9, not essential For Issue 10, there is a typo in Figure 2, updated above. |
| Huawei/Hisi | Issue1: R17 intra-UE multiplexing should adopt the consistent rule with R16 prioritization for UL power prioritization, otherwise the HP UCI/PUSCH would be unnecessarily dropped with wrong priority order. Issue2: No need for joint operation similar to the situation where prioritization and intra-UE multiplexing are not jointly operated. Issue3: As also discussed in the previous meeting, the UE is not supposed to first ‘add’ then ‘exclude’, but not consider the simultaneous PUSCH at the first place. Issue4: Support the changes to make the specified procedure more clear. Issue5: Follow the agreement of 108-e Issue6: Can be discussed after resolving Issue1 and Issue2 Issue7: Seems the same rule of legacy can be reused. How to guarantee the number of rest PUCCHs can be gNB implementation. Issue8: No need to discuss this issue at maintenance phase. Issue9: Basically we do not see a need for joint operation. Can be discussed in R17 Sidelink topic if necessary. Issue10: The issue needs to be discussed, and we also raised this issue in our contribution. |
| vivo | For Issue 2, gNB scheduling can avoid the issue by time-frequency resource indication in UL CI. For issue 3, it is not essential. For issue 4 Issue 1): no need change the existing spec. we are fine with the current text description. Issue 2) it seems no difference between huawei’s version and the current text Issue 3) the current spec is clear For Issue 5, agree with moderator’s understanding For issue 6, not essential. For Issue 7, the existing spec is clear. Agree with Sumsang’s understanding. For Issue 8, dynamic enabling/disabling of Rel-17 intra-UE multiplexing is not supported in Rel-17 For Issue 9, not essential. |
| Intel | We think only Issue 10 needs to be discussed. Issue 1, 2, 7, 9 are unessential optimization. For issue 7, it is Rel-15 issue, we think gNB can ensure up to 2 PUCCHs per priority by implementation. Issue 3,4 and 6 are unessential editorial revision. The current wording in spec is clear. Issue 5 was already excluded in previous meeting. Issue 8 was already excluded in previous meeting. |
| QC | Issue 2: gNB may want to cancel LP PUSCH while avoid cancel HP PUSCH. We are not sure if gNB can always achieve this via time-frequency resource indication in UL-CI. If not, then at least the following can be considered to avoid drop HP UCI. - A LP PUSCH has HP UCI multiplexing on it is exempt from UL-CI, if RRC parameter uplinkCancellationPriority is configured |
| OPPO | Issue 1,2,6 and 9 are relevant and a basic common issue is that the priority of the resultant PUCCH/PUSCH for Rel-17 intra-UE multiplexing is not clear. So we prefer to discuss issue 1,2,6,9 as a group to find a unified solution firstly. If a unified solution can not be found, then we discuss it case by case. Issue 5 and 8, we share with moderator’s understanding. Issue 10 overlaps with Issue 1 in intra-UE multiplexing B. Replicated discussion should be avoided. |
| DOCOMO | Issue 2: it is an optimization and gNB can avoid the issue. Issue 3: we are open to discuss as the spec is not very clear from our perspective. Issue 4: it is not essential. Issue 5: we agree with moderator’s understanding. Issue 6: it can be discussed with Issue 1. Issue 7: the existing spec is clear. Agree with Sumsang’s understanding. Issue 8: dynamic enabling/disabling of Rel-17 intra-UE multiplexing is not supported in Rel-17. Issue 9: it is not essential. |
| LG | We share similar view with other companies. Issue 2/5/6/8/9 doesn’t seems essential to be discussed. |
| New H3C | OK to discuss issue 1&issue 10 during this meeting. Other issues can be discussed with low priority. |
| Nokia/NSB | Issues 3 and 4: non-essential editorial revisions. Issue 5: already addressed in RAN1#108-e. Issue 6: no need to change the priority of the resultant channel. Issue 7: as commented by other companies, this would be guaranteed by gNB implementation. Issue 8: too late for this to be discussed. Issue 9: not essential. |
| InterDigital | Agree with moderator suggestion to treat 1, 2, 6 jointly (not sure if 9 is related). Addressing issue 6 could address issues 1 and 2 at the same time. For Issue 2, not sure how the situation is different from a HP PUSCH? The same argument could be used to say we do not need uplinkCancellationPriority at all. For consistency it should be treated the same as HP PUSCH. |
| ZTE | Issue 1: Fine to discuss. Issue 2: Open to the discussion. Issue 3: It depends on UE implementation. Issue 4: Seems not essential. Issue 5: No need to revert the agreement in RAN1#108-e. Issue 6: Can be merged to issue 1/2 as moderator suggestion. Issue 7: Seems not serious, as Rel-15 also has the same question. Issue 8: Out of date. Issue 9: Can be discussed in Sidelink session. Issue 10: May be discussed together with the issue#1 in Mux-B. |

- Outcome

Based on the email discussion preparation, Mr. Chair set the following scope of the RAN1#109-e email discussions:

[109-e-R17-IIoT-URLLC-02] Email discussion on intra-UE multiplexing/prioritization A by May 18– Yanping (CATT)

- Issue 1: power allocation order

- Issue 2: Joint operation of R17 intra-UE multiplexing and UL CI

- Issue 10: Step 2 considers resultant PUCCH/PUSCH after step 1 only

- 1st check point on May 12 and final check point on May 18

- Reference

- R1-2203073		Intra-UE multiplexing enhancements	Huawei, HiSilicon

- R1-2203189		Discussion on enhanced intra-UE multiplexing	ZTE

- R1-2203213		Remaining issue on intra-UE multiplexing and prioritization	New H3C Technologies Co., Ltd.

- R1-2203274		Maintenance of Rel-17 URLLC / IIoT Intra-UE Multiplexing/Prioritization	Nokia, Nokia Shanghai Bell

- R1-2203305		Discussion on intra-UE multiplexing/prioritization	Spreadtrum Communications

- R1-2203398		Remaining Issues of Intra-UE Multiplexing/Prioritization for IIoT/URLLC	Ericsson

- R1-2203435		Maintenance on intra-UE multiplexing and prioritization	CATT

- R1-2203514		Remaining issues on intra-UE Multiplexing/Prioritization for Rel-17 URLLC	vivo

- R1-2203863		Maintenance on Intra-UE Multiplexing/Prioritization	Samsung

- R1-2204003		Remaining issues on intra-UE multiplexing/prioritization	OPPO

- R1-2204091		Remaining issues of Intra-UE Multiplexing/Prioritization	InterDigital, Inc.

- R1-2204206		Remaining issues in intra-UE multiplexing/prioritization	Apple

- R1-2204344		Remaining issues on intra-UE multiplexing/prioritization for Rel.17 URLLC	NTT DOCOMO, INC.

- R1-2204439		Remaining details on intra-UE multiplexing and prioritization	ITRI

- R1-2204547		Remaining issues on intra-UE multiplexing/prioritization for URLLC/IIoT	WILUS Inc.

- R1-2204617		Remaining issues on Intra-UE multiplexing enhancements	LG Electronics

- R1-2204647		Intra-UE Multiplexing/Prioritization	ETRI

- R1-2204770		Remaining issue for Intra-UE Uplink Channel Multiplexing and Prioritization	Intel Corporation

- R1-2204983		Remaining issues on Intra-UE multiplexing and prioritization for IOT and URLLC	Qualcomm Incorporated