3GPP TSG RAN WG1 #109-e		                                                         R1-2205147

e-Meeting, May 9th – 20th, 2022

Source:	Moderator (CATT)

Title:	Summary #1 of email thread [109-e-R17-IIoT-URLLC-02]

Agenda Item:	8.3.2

Document for:	Discussion and Decision

- Introduction

According to the preparation phase discussion [20], it was agreed to discuss the following three issues for Rel-17 URLLC/IIoT intra-UE multiplexing/prioritization A.

This document summarizes the discussions during RAN1#109-e under the following email thread.

[109-e-R17-IIoT-URLLC-02] Email discussion on intra-UE multiplexing/prioritization A by May 18– Yanping (CATT)

- Issue 1: power allocation order

- Issue 2: Joint operation of R17 intra-UE multiplexing and UL CI

- Issue 10: Step 2 considers resultant PUCCH/PUSCH after step 1 only

- 1st check point on May 12 and final check point on May 18

- Issue 1: power allocation order

  - 1st round discussion

In TS 38.213 v17.1.0 Clause 7.5, the prioritizations for transmission power reductions are defined as follows with priority order highlighted in green.

## 7.5 Prioritizations for transmission power reductions For single cell operation with two uplink carriers or for operation with carrier aggregation, if a total UE transmit power for PUSCH or PUCCH or PRACH or SRS transmissions on serving cells in a frequency range in a respective transmission occasion would exceed , where is the linear value of in transmission occasion as defined in [8-1, TS 38.101-1] for FR1 and [8-2, TS38.101-2] for FR2, the UE allocates power to PUSCH/PUCCH/PRACH/SRS transmissions according to the following priority order (in descending order) so that the total UE transmit power for transmissions on serving cells in the frequency range is smaller than or equal to for that frequency range in every symbol of transmission occasion . When determining a total transmit power for serving cells in a frequency range in a symbol of transmission occasion , the UE does not include power for transmissions starting after the symbol of transmission occasion . The total UE transmit power in a symbol of a slot is defined as the sum of the linear values of UE transmit powers for PUSCH, PUCCH, PRACH, and SRS in the symbol of the slot. - PRACH transmission on the PCell - PUCCH or PUSCH transmissions with higher priority index according to clause 9 - For PUCCH or PUSCH transmissions with same priority index - PUCCH transmission with HARQ-ACK information, and/or SR, and/or LRR, or PUSCH transmission with HARQ-ACK information - PUCCH transmission with CSI or PUSCH transmission with CSI - PUSCH transmission without HARQ-ACK information or CSI and, for Type-2 random access procedure, PUSCH transmission on the PCell - SRS transmission, with aperiodic SRS having higher priority than semi-persistent and/or periodic SRS, or PRACH transmission on a serving cell other than the PCell In case of same priority order and for operation with carrier aggregation, the UE prioritizes power allocation for transmissions on the primary cell of the MCG or the SCG over transmissions on a secondary cell. In case of same priority order and for operation with two UL carriers, the UE prioritizes power allocation for transmissions on the carrier where the UE is configured to transmit PUCCH. If PUCCH is not configured for any of the two UL carriers, the UE prioritizes power allocation for transmissions on the non-supplementary UL carrier.

Huawei [1] and vivo [8] proposed to update power allocation order considering PUSCH multiplexed with UCI of a different priority with the following proposals. The proposals are essentially the same despite different wordings in moderator’s view.

Huawei’s proposal in [1]:

Proposal 6: Update the transmission power allocation order for Rel-17 by considering inter-priority UCI-on-PUSCH cases: - LP PUSCH with HP HARQ-ACK should be of the same priority as HP PUSCH with HP HARQ-ACK, i.e., higher than HP PUSCH with CSI, as well as HP PUSCH only. - LP HARQ-ACK on HP PUSCH should of the same priority as HP PUSCH only, i.e., lower than HP PUSCH with HP HARQ-ACK, as well as HP PUSCH with CSI.

Vivo’s proposal in [8]:

Proposal 2: For HP HARQ-ACK on a LP PUSCH, power allocation order should be the same as HP PUSCH with HP HARQ-ACK. For LP HARQ-ACK on HP PUSCH, power allocation order should be the same as HP PUSCH only.

Companies please share your views on whether you support the proposal.

Proposal 2.1:

Update the transmission power allocation order for Rel-17 by considering inter-priority UCI-on-PUSCH cases:

- LP PUSCH with HP HARQ-ACK should be of the same priority as HP PUSCH with HP HARQ-ACK, i.e., higher than HP PUSCH with CSI, as well as HP PUSCH only.

- LP HARQ-ACK on HP PUSCH should of the same priority as HP PUSCH only, i.e., lower than HP PUSCH with HP HARQ-ACK, as well as HP PUSCH with CSI.

|  | Company |
| --- | --- |
| Support | Sony, Huawei/Hisi, Samsung, InterDigital, DOCOMO,vivo, ITRI, New H3C,OPPO, CATT, ZTE, QC, Ericsson |
| Not support | Nokia/NSB, Intel, LG |

| Company | Comments |
| --- | --- |
| Nokia/NSB | We see this as a non-essential optimization. There had been the option to support dynamic indication for such cases (if to multiplex or not) but this was not adopted. If one is worrying about the related tx-power allocation prioritization, then the multiplexing should not be configured in the first place. On the 2nd bullet: is this the case that there is only LP HARQ (and not HP HARQ) included? |
| Huawei/Hisi | The power reduction rule should be consistent with R16, where HP HARQ-ACK >HP PUSCH with HP CSI> HP PUSCH only. When extended to R17, these two cases should follow the same principle - HP HARQ-ACK HP HARQ-ACK on LP PUSCH: it should not be lower than HP PUSCH only. - HP PUSCH HP PUSCH with LP HARQ-ACK: it should not be higher than HP PUSCH with HP CSI. Otherwise there will be unnecessary dropping of HP HARQ-ACK due to wrong dropping order in case of limited power. @ Nokia/NSB: 1) One of the major reason of not introducing dynamic indication is it complicates the multiplexing procedure. However, the power reduction happens after the multiplexing, i.e., it does not impact the multiplexing procedure. 2) As per our understanding, Yes, on the 2nd bullet there is only LP HARQ (and not HP HARQ) included. |
| Samsung | An editorial suggestion as following, Proposal 2.1: Update the transmission power allocation order for Rel-17 by considering inter-priority UCI-on-PUSCH cases: - LP PUSCH with HP HARQ-ACK should be of the same priority as HP PUSCH with HP HARQ-ACK, i.e., higher than HP PUSCH with CSI, as well as HP PUSCH only. - LP HARQ-ACK on HP PUSCH with LP HARQ-ACK should be of the same priority as HP PUSCH without UCI only, i.e., lower than HP PUSCH with HP HARQ-ACK, as well as HP PUSCH with CSI. |
| InterDigital | In case of cross-priority transmissions, only the high-priority components (PUSCH and/or UCI) should be considered for this prioritization. |
| Intel | We agree with NOKIA that this is just optimization. Degradation of HP performance can be avoided by gNB proper scheduling. |
| DOCOMO | Support the proposal to avoid performance degradation of HP HARQ-ACK on LP PUSCH. |
| vivo | We share the same view with HW. According to the current spec, for PUCCH or PUSCH transmissions with same priority index, the first order is. - PUCCH transmission with HARQ-ACK information, and/or SR, and/or LRR, or PUSCH transmission with HARQ-ACK information. LP PUSCH with HP HARQ-ACK and LP PUSCH with LP HARQ-ACK should has different priority to avoid dropping of HP HARQ-ACK. |
| ITRI | Support the proposal. Performance degradation of HP HARQ-ACK on LP PUSCH should be avoided. |
| LG | Similar view with Nokia and Intel, it seems to be non-essential optimization. It could be handled by gNB implementation, for example, by properly scheduling HP PUSCH so that HP HARQ-ACK would be multiplexed on the HP PUSCH rather than LP PUSCH. |
| QC | This is a quite reasonable proposal to avoid dropping HP HARQ-ACK. To Nokia: we don’t gNB knows UE Tx power precisely, due to PHR aging issue and UE missing closed loop PC command. Plus, we don’t think gNB mimic/track power control loop for each UE. So, in practice, gNB only knows UE Tx power roughly. It cannot be assumed gNB can track UE Tx power precisely. With the above, relying on gNB to disable intra-UE mux to avoid this power control issue may not work. To Intel: if we can just rely on gNB scheduler to handle everything, we probably don’t need Rel-17 intra-UE mux. |
| Ericsson | We agree that the new cases of PUCCH and PUSCH with mixed priority payload should be addressed in the transmit power priority list. Other than the PUSCH in the proposal, suggest to add that: “A PUCCH carrying multiplexed HP UCI and LP UCI is treated as a PUCCH with higher priority index”. |

  - Summary of 1st round discussion

A clear majority of companies support proposal 2.1. The proposal is updated with some editorial modifications as below based on the comments from companies.

Ericsson proposed to add “A PUCCH carrying multiplexed HP UCI and LP UCI is treated as a PUCCH with higher priority index”. But according to moderator’s understanding, HP and LP UCI are always multiplexed in HP PUCCH so there is no need to have the addition.

Updated Proposal 2.1:

Update the transmission power allocation order for Rel-17 if UE is configured with UCI-MuxWithDifferentPriorityby considering inter-priority UCI-on-PUSCH cases:

- LP PUSCH with HP HARQ-ACK should be of the same priority as HP PUSCH with HP HARQ-ACK, i.e., higher than HP PUSCH with CSI, as well as HP PUSCH only.

- LP HARQ-ACK on HP PUSCH with LP HARQ-ACK only should haveof the same priority as HP PUSCH without UCIonly, i.e., lower than HP PUSCH with HP HARQ-ACK, as well as HP PUSCH with CSI.

The proposal will be discussed in GTW session on May 12th if time allows. Otherwise, the proposal is for email approval if no comments are received until UTC 23:59 May 12th (Thu).

| Company | Comments |
| --- | --- |
| LG | As Intel commented in the reflector, since all overlapped channels should meet Rel-15 mux timeline, the gNB could schedule properly so that the PUSCH with HP HARQ-ACK would avoid degradation due to power limitation. But if we are the only company objecting the proposal (and the majority companies have concern without it), we can live with it. |
| Nokia/NSB | Similar as LG & Intel, we don’t see a real reason to change this. But looking at the strong majority input from the first round, we could not block progress here either. |
| vivo | Support the proposal. |
| Ericsson | Support. Regarding our suggestion “A PUCCH carrying multiplexed HP UCI and LP UCI is treated as a PUCCH with higher priority index”: the concern is that “HP” “LP” is sometimes used to refer to the content (e.g., HP/LP HARQ-ACK), sometimes used to refer to the PUCCH/PUSCH resources. This was no problem if no mixed priority. When mixed priority, it’s good to clarify that for power control section text below, “priority index” refers to the PUCCH/PUSCH resources, unless otherwise stated. - PUCCH or PUSCH transmissions with higher priority index according to clause 9 - For PUCCH or PUSCH transmissions with same priority index Assuming the understanding is “priority index” refers to the PUCCH/PUSCH resources, then 1st bullet in Proposal 2.1 is needed, but 2nd bullet is not needed. If this understanding is not agreed, then our suggestion on PUCCH need to be captured as well. |
| Samsung | Support. |

- Issue 2: Joint operation of R17 intra-UE multiplexing and UL CI

  - 1st round discussion

InterDigital [11], ETRI [17] and Qualcomm [19] discussed joint operation of Rel-17 intra-UE multiplexing and UL CI.

InterDigital [11] proposed that Rel-17 intra-UE multiplexing and UL CI can be configured simultaneously and LP PUSCH multiplexed with HP HARQ-ACK should not be cancelled with a proposed TP.

Proposal 1: A UE can be configured with both UCI-MuxWithDifferentPriority and UplinkCancellation. Proposal 2: A UE that detects CI and is provided uplinkCancellationPriority cancels a PUSCH if the PUSCH is with priority 0 and if no HARQ-ACK information of priority 1 is multiplexed in the PUSCH.

ETRI [17] also proposed that PUSCH carrying HP UCI is not cancelled by UL CI with a proposed TP.

Proposal 4:The PUSCH carrying HP UCI may not be cancelled by the ULCI.

Qualcomm [19] proposed that PUSCH cancellation due to UL CI is performed after step 2 of Rel-17 intra-UE multiplexing. In addition, similar as InterDigital and ETRI, Qualcomm proposed that a LP PUSCH with HP UCI should be exempt from UL-CI.

Proposal 2: For Rel-17 intra-UE multiplexing, PUSCH cancellation due to UL-CI are performed after step 2 of Rel-17 intra-UE multiplexing. - A LP PUSCH has HP UCI multiplexing on it is exempt from UL-CI, if RRC parameter uplinkCancellationPriority is configured.

During the discussions in last meeting and the preparation phase for this meeting, companies have different views on whether joint operation of R17 intra-UE multiplexing and UL CI is supported or not. In addition, for companies who support joint operation, some companies think that no optimization is needed and gNB can avoid the issue via proper time-frequency resource indication in UL CI.

Companies please share your views on which option below you support.

Proposal 3.1:

For joint operation of Rel-17 intra-UE multiplexing and UL CI:

- Option 1: joint operation of Rel-17 intra-UE multiplexing and UL CI is not supported.

- Option 2: joint operation of Rel-17 intra-UE multiplexing and UL CI is supported without special handling of LP PUSCH multiplexed with HP UCI.

- Option 3: joint operation of Rel-17 intra-UE multiplexing and UL CI is supported and LP PUSCH multiplexed with HP UCI is exempt from UL CI.

|  | Company |
| --- | --- |
| Option 1 | Nokia/NSB (2nd) |
| Option 2 | Sony, Nokia/NSB (1st), Samsung, Intel, DOCOMO (2nd),vivo, ITRI, New H3C, Spreadtrum, CATT, ZTE, LG, Ericsson |
| Option 3 | InterDigital (when uplinkCancellationPriority is configured), DOCOMO (1st), OPPO, QC(when uplinkCancellationPriority is configured) |

| Company | Comments |
| --- | --- |
| Nokia/NSB | No need for optimization here (i.e. Option 1 or Option 2 should be adopted) |
| Samsung | Option 2 is preferred. If gNB would like to exclude the LP PUSCH with HP UCI, gNB can indicate not cancelling the PUSCH, no need to specify the case. |
| InterDigital | Considering that HP UCI has higher priority than HP PUSCH, it would be a little strange that the HP PUSCH would be protected but not HP UCI? Option 3 should only be in case uplinkCancellationPriority is configured. |
| Intel | Protection of HP UCI can be achieved by proper gNB scheduling. |
| DOCOMO | Option 3 is preferred to avoid cancellation of HP UCI on LP PUSCH but we would be fine with Option 2 as well since gNB can avoid it by proper scheduling. Similar to InterDigital, we also think the clarification for the case is needed. It is only the case where uplinkCancellationPriority is configured. |
| vivo | We share the same view with Samsung and Intel |
| ITRI | HP UCI dropping could be avoided by gNB scheduling. |
| OPPO | Option 3 is preferred to avoid HP UCI cancellation. Due to resource granularity indicated by UL CI is coarser than the granularity of PUSCH/PUCCH resource allocation, even HP PUSCH/PUCCH from UE 1 and LP PUSCH with HP UCI from UE 2 can be scheduled in different time-frequency resource, but LP PUSCH with HP UCI may still overlap with region indicated by UL CI. So it is not easy to avoid HP UCI cancellation by gNB scheduling. |
| ZTE | Option 2 doesn't change the procedure of Rel-16. Also it is flexible for gNB to schedule a high priority PUSCH to cancel the LP PUSCH with HP UCI. |
| LG | Similar view with other companies, Option 2 is preferred. |
| QC | Same view as Docomo and InterDigital. |
| Ericsson | Option 2: No new procedure is needed here. |

  - Summary of 1st round discussion

A clear majority of companies support option 2 of proposal 3.1. Company commented that it is better to have a conclusion to make it clear that joint operation of UL CI and Rel-17 intra-UE multiplexing is supported.

Proposed conclusion:

A UE can be configured with both UCI-MuxWithDifferentPriority and UplinkCancellation.

- No special handling for LP PUSCH multiplexed with HP UCI

The proposal will be discussed in GTW session on May 12th if time allows. Otherwise, the proposal is for email approval if no comments are received until UTC 23:59 May 12th (Thu).

| Company | Comments |
| --- | --- |
| LG | We are fine with the proposal. |
| Nokia/NSB | Support the proposed conclusion. |
| ZTE | Support |
| vivo | support |
| Ericsson | Support |
| Samsung | Support |

- Issue 10: Step 2 considers resultant PUCCH/PUSCH after step 1 only

  - 1st round discussion

In previous meetings, the case of a HP PUCCH with PF 0/1 overlapping with LP HARQ-ACK was discussed without conclusion. There were two candidate options.

- Option 0: LP HARQ-ACK is dropped when HP SR is positive.

- Option 2: 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1

Samsung [9] discussed that the result is same for intra-UE multiplexing of same priority in Step 1 for negative SR for the following two cases but Option 2 above requires a UE to differentiate the two cases.

| Case A | Case B |
| --- | --- |

Figure 1: HP PUCCH with PF0/1 overlapping with LP HARQ-ACK

In addition, Samsung [9] provided another case shown in Figure 2, where HP SR is dropped in Step 1.

Figure 2: overlapping of HP HARQ-ACK with PF1, HP SR with PF0 and LP PUSCH

The proposal from Samsung is as follows.

Proposal 1: For resolving collision of PUCCHs and/or PUSCHs of different priorities, Step 2 only considers the resulting PUCCHs and/or PUSCHs in Step 1.

Let’s first hear companies’ views for the two cases provided by Samsung.

Proposal 4.1:

Moderator’s note: please check proposal 4.3 and 4.4 directly.

For a HP SR overlapping with HP HARQ-ACK with PF 0/1, if the HP SR is not multiplexed with HP HARQ-ACK in Step 1 (i.e. SR is negative or SR is dropped) and the HP HARQ-ACK with PF 0/1 overlaps with LP HARQ-ACK, down-select from:

- Option 1: HP SR is multiplexed with LP HARQ-ACK

- Option 2: HP SR is not multiplexed with LP HARQ-ACK

|  | Company |
| --- | --- |
| Option 1 |  |
| Option 2 | Samsung,vivo, New H3C, LG |

| Company | Comments |
| --- | --- |
| Nokia/NSB | Need for clarification (unclear target of the proposal) First, the claim that “For a HP SR overlapping with HP HARQ-ACK with PF 0/1, if the HP SR is not multiplexed with HP HARQ-ACK in Step 1 (i.e. SR is negative or SR is dropped) …” may not be accurate as SR is multiplexed with HARQ-ACK (explicitly or implicitly) even if SR is negative. The only case where SR is dropped is when SR is with F0 and HARQ-ACK is with F1. In our view, the HP SR and HP HARQ-ACK should be first resolved based on Rel-15/Rel-16 rules, and if the resulting HP channel/content overlaps with LP HARQ-ACK, then the resulting HP content would be multiplexed with the LP HARQ-ACK. Specifically, we propose (at least for HP SR with F1 and HP HARQ-ACK with F0/F1, and HP SR with F0 and HP HARQ-ACK with F0): - In Step 1, 1-bit HP SR is multiplexed to HP HARQ-ACK bits (based on Rel-15 rules) for determining the resultant PUCCH resource. - In Step 2, - if the HP HARQ-ACK is using a PUCCH resource provided by PUCCH-ResourceSet or SPS-PUCCH-AN-List, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1 - The resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is either of PF2, PF3, or PF4. - Reuse the (agreed) procedures for multiplexing of LP HARQ-ACK and HP HARQ-ACK on PUCCH resource with PF 2/3/4, i.e., PUCCH resource selection, separate coding, PRB determination, rate matching, power control, etc. - if HP HARQ-ACK is using a PUCCH resource provided by n1PUCCH-AN, the LP HARQ-ACK is dropped. |
| Huawei/Hisi | In Rel-15, when HARQ-ACK of PF1 collides with SR of PF0, the SR should be dropped, and only HARQ-ACK is transmitted on PUCCH. When that case occurs in Step 1 of Rel-17, the UE has no information of the status of SR which has been discarded from the buffer at Step 1. So the HP PUCCH under this case should be regarded as the PUCCH with HP HARQ-ACK only, and consequently the handling of the HP PUCCH including HARQ-ACK and SR described in the above proposal should not be applied to this case. As a comparison case of channel selection between HP HARQ-ACK PF1 and HP SR PF1 or HP SR is negative at Step 1, the HP SR status is still stored in buffer before Step 2, so the above proposal should be applied to this case which is regarded as ‘PUCCH carrying implicit SR’. Proposal: For the case of HP SR with PF0 dropped due to collision with HP HARQ-ACK PF1 in Step 1, the resultant HP PUCCH after Step 1 is regarded as HP PUCCH with HARQ-ACK only when resolving the overlapping with LP HARQ-ACK in Step 2. |
| Samsung | The 2-step procedure is to simplify the issue of resolving overlapping PUCCHs/PUSCHs with different priorities. If intermediate channels of Step 1 are considered in Step 2, the issue will become very complicated and lots of remaining cases need to be discussed in the maintenance phase. We suggest to have a simple and unified solution for all the cases. DO NOT consider the intermediate channels in Step 1 when resolving overlapping PUCCHs/PUSCHs in step 2. For this case, considering the intermediate HP UCI in step 1 only complicates the spec and implementation with no gain. Without the LP HARQ-ACK, the HP SR is not multiplexed with HP HARQ-ACK, why should the HP SR be multiplexed with HP HARQ-ACK if there is overlapping LP HARQ-ACK? Whether multiplexing the HP SR should not depend on the existence of LP HARQ-ACK. |
| InterDigital | Agree with Samsung that having to go back to step 1 when resolving step 2 is not nice. However, when HP HARQ-ACK is PF0 and there is no LP PUCCH (as in R15), the HP SR information is not lost. Dropping HP SR due to the presence of LP PUCCH is not a desirable outcome. A better way would be that LP PUCCH is dropped in this scenario. |
| Intel | We’d like to clarify, (1) what is the relation between Proposal 4.1 and proposal 4.2? Proposal 4.1 also includes the case SR is dropped (i.e., SR is negative or SR is dropped). (2) for option 2 ‘HP SR is not multiplexed with LP HARQ-ACK’, does it mean, HP SR is not considered in step 2, or HP SR is considered in step 2 and LP HARQ-ACK is dropped? |
| vivo | HP SR is dropped in step 1, it should not be reconsidered in step 2. |
| Moderator | You can check proposal 4.3 directly. |
| LG | Similar view with Samsung and other companies, HP SR is dropped already in Step 1, so it is not to be considered in Step 2. |
| QC | We are not sure what is the interaction between this proposal and the proposal being discussed in UCI mux B. If the intention is to move the discussion of the following proposal (discussed in last meeting in UCI mux B) to UCI MUX A. We then support option 2 in the following proposal. By the way, we have similar confusion about “SR is negative or SR is dropped” in proposal 4.1 as Nokia. We think the formulation of the problem is clearer in the following proposal and suggest continuing that formulation and down selection option 0 and option 2. Proposal discussed in RAN1 108e: When a PUCCH carrying explicit/implicit HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs which are all negative or include at least one positive, - In Step 1, 1-bit HP SR is multiplexed to HP HARQ-ACK bits (based on Rel-15 rules) for determining the resultant PUCCH resource. - Note: The description of Step 1 here is only for information purpose, which has no spec impact. - In Step 2, down-select from the two options: - Option 0: LP HARQ-ACK is dropped. - Option 2: 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1 - The resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is either of PF2, PF3, or PF4. |

For the case shown in Figure 2, we had the following agreement that LP PUSCH is only dropped by positive HP SR. We therefore only need to discuss the case that the HP SR is positive.

Agreement Confirm the following working assumption in RAN1#107bis-e with the following update (in RED): For resolving collision of PUCCHs and PUSCHs of different priorities in step 2.2, LP PUSCH(s) overlapping with HP PUCCH resource which carries at least positive HP SR are dropped before UCI multiplexing. - Step 1.2 behavior is not affected by the above

Proposal 4.2:

Moderator’s note: please check proposal 4.3 and 4.4 directly.

For a HP SR with PF0 overlapping with HP HARQ-ACK with PF1 and the HP SR is positive, if the HP SR overlaps with a LP PUSCH, down-select from:

- Option 1: LP PUSCH is not dropped due to positive HP SR

- Option 2: LP PUSCH is dropped due to positive HP SR

|  | Company |
| --- | --- |
| Option 1 | Sony Huawei/Hisi, Samsung, InterDigital, Intel, DOCOMO,vivo, ITRI, New H3C, LG, Ericsson |
| Option 2 |  |

| Company | Comments |
| --- | --- |
| Nokia/NSB | Needed clarification also on this one: According to our understanding, based on the outcome of Step 1 the SR overlapping with HP HARQ-ACK is to be dropped already. And the interaction with LP channel is then handled only in step 2. So unclear why we need to define some behavior for this (should be clear that after step 1 the HP HARQ & LP PUSCH are remaining, and step 2 is executed accordingly). |
| Huawei/Hisi | As there is no valid SR to collide with PUSCH, Option 1 should be preferred. |
| Samsung | For this case, considering the intermediate channel in Step 1 has worse performance. |
| InterDigital | As per cited agreement above, since the HP PUCCH resources does not carry a positive HP SR, the LP PUSCH is not dropped. A small benefit of doing the opposite would be that the network could now detect positive SR implicitly from the dropping of LP PUSCH, but it may be preferable to stick to earlier agreement to keep things simple. |
| DOCOMO | Share the same view as Huawei/HiSi. |
| vivo | HP SR is dropped in step1. Then there is no overlapping between HP SR and LP PUSCH in step 2. |
| ITRI | HP SR chould not be considered in step 2 since it has been dropped in step 1. |
| LG | Similar view with other companies, HP SR is dropped already in Step 1, so it is not to be considered in Step 2. |
| QC | Same view as Nokia. We are not sure why this case need to be handled/specified specifically? Isn’t the general two steps procedure already handling this case? So far, we don’t see the need to agree on the proposal. |

Based on the comments above, the new proposals are provided by separating the case that HP SR is dropped and HP SR is negative.

Proposal 4.3:

For the case of HP SR with PF0 dropped due to collision with HP HARQ-ACK PF1 in Step 1, the HP SR is not considered in Step 2.

|  | Company |
| --- | --- |
| Support | New H3C Huawei/Hisi, OPPO, Spreadtrum, CATT, Nokia/NSB, ZTE, LG, InterDigital, ITRI |
| Not support | Samsung |

| Company | Comments |
| --- | --- |
| Huawei/Hisi | The SR is dropped from the buffer and is absent for Step 2. |
| Samsung | We suggest to consider a more general issue without the restriction of PUCCH formats. HP SR can be dropped if a HP HARQ-ACK overlaps with multiple HP SRs. |
| QC | To us, the two-step approach already implies this: whatever dropped in step 1 is not involved in step 2. But if majority want to clarify this particular case, we are fine to take this as a conclusion, instead of an agreement, as it seems there is no spec impact. |

Proposal 4.4:

For a HP SR overlapping with HP HARQ-ACK with PF 0 and a HP SR with PF1 overlapping with HP HARQ-ACK with PF1, if the HP SR is negative and the HP HARQ-ACK with PF 0/1 overlaps with LP HARQ-ACK, down-select from:

- Option 1: HP SR is multiplexed with LP HARQ-ACK

- Option 2: HP SR is not multiplexed with LP HARQ-ACK

|  | Company |
| --- | --- |
| Support | Huawei/Hisi, OPPO, CATT, Nokia/NSB (with comments), ZTE, ITRI |
| Not support | Samsung, LG (Option 1 or 2 according to UCI payload size as in Rel-15/16) |

| Company | Comments |
| --- | --- |
| Huawei/Hisi | Option 1. For the HP SR PF0 vs HP HARQ-ACK PF1 case, the HP HARQ-ACK PUCCH cannot accommodate 1 more bit SR, thus the SR information is dropped from the buffer. Different from that case, for HP SR PF1 vs HP HARQ-ACK PF1, the SR is not dropped, and the 2 bits HARQ-ACK + SR information is transmitted with channel selection manner. Similar to the negative SR case, where the 1 bit negative SR still exists. Therefore, for these two cases, HP SR (and HP HARQ-ACK) should still be multiplexed with LP HARQ-ACK at Step 2. |
| OPPO | Option 1 to avoid blind decoding in gNB due to SR status is unknown. To be specific, - In Step 2, - if the HP HARQ-ACK is using a PUCCH resource provided by PUCCH-ResourceSet or SPS-PUCCH-AN-List, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1 |
| Nokia/NSB | We are fine with the proposal and Option 1. But we wonder, why is the proposal just for negative SR? Wouldn’t we still need to decide on the case of positive SR well? As we commented on Proposal 4.1, the following handling could be considered (at least for HP SR with F1 and HP HARQ-ACK with F0/F1, and HP SR with F0 and HP HARQ-ACK with F0): - In Step 1, 1-bit HP SR is multiplexed to HP HARQ-ACK bits (based on Rel-15 rules) for determining the resultant PUCCH resource. - In Step 2, - if the HP HARQ-ACK is using a PUCCH resource provided by PUCCH-ResourceSet or SPS-PUCCH-AN-List, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1 - The resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is either of PF2, PF3, or PF4. - Reuse the (agreed) procedures for multiplexing of LP HARQ-ACK and HP HARQ-ACK on PUCCH resource with PF 2/3/4, i.e., PUCCH resource selection, separate coding, PRB determination, rate matching, power control, etc. - if HP HARQ-ACK is using a PUCCH resource provided by n1PUCCH-AN, the LP HARQ-ACK is dropped. |
| Samsung | We don’t think there should be any difference between a HP HARQ-ACK with or without negative SR, multiplexing negative HP SR with LP HARQ-ACK only complicates the spec and implementation without any gain. |
| LG | The above proposal seems to be directly connected to the Issue #1 of the email thread “Intra-UE mux B”. As we already commented in previous meeting and also in our contribution to this meeting, applying Option 1 without considering UCI payload size would cause significant change/impact to Rel-15/16 PUCCH multiplexing framework (and relevant UE/gNB implementations) as well as increase of PUCCH resource overhead. Therefore, to minimize the impacts to UE/gNB implementations and PUCCH resource overhead as well as to guarantee the performance/reliability of HP UCI transmission, it is reasonable to adopt following behaviour. - If there is 1-bit HP HARQ-ACK and 1 bit LP HARQ-ACK, LP HARQ-ACK is dropped. - If at least one of the HP HARQ-ACK payload size or LP HARQ-ACK payload size is greater than or equal to 2, - If HP HARQ-ACK is dynamic HARQ-ACK or SPS HARQ-ACK with sps-PUCCH-AN-List, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1. - If HP HARQ-ACK is SPS HARQ-ACK with n1PUCCH-AN, LP HARQ-ACK is dropped. |
| QC | We are not sure what is the interaction between this proposal and the proposal being discussed in UCI mux B. If the intention is to move the discussion of the following proposal (discussed in last meeting in UCI mux B) to UCI MUX A. We then support option 2 in the following proposal. By the way, we have similar confusion about “SR is negative or SR is dropped” in proposal 4.1 as Nokia. We think the formulation of the problem is clearer in the following proposal and suggest continuing that formulation and down selection option 0 and option 2. Proposal discussed in RAN1 108e: When a PUCCH carrying explicit/implicit HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs which are all negative or include at least one positive, - In Step 1, 1-bit HP SR is multiplexed to HP HARQ-ACK bits (based on Rel-15 rules) for determining the resultant PUCCH resource. - Note: The description of Step 1 here is only for information purpose, which has no spec impact. - In Step 2, down-select from the two options: - Option 0: LP HARQ-ACK is dropped. - Option 2: 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1 The resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is either of PF2, PF3, or PF4. |
| InterDigital | Multiplexing a negative SR bit every time there is overlap with LP HARQ-ACK + HP HARQ-ACK PF0 is wasteful considering that HP SR is typically configured with low periodicity. It would seem more reasonable to multiplex HP SR+HARQ-ACK based on R15 rule and then drop the LP HARQ-ACK in step 2. |

  - 2nd round discussion

Proposal 4.3 is supported by majority companies and Samsung suggested to consider a more general issue without the restriction of PF.

Updated Proposal 4.3 (for conclusion):

For the case of HP SR with PF0 dropped due to collision with HP HARQ-ACK PF1 in Step 1, the HP SR is not considered in Step 2.

|  | Company |
| --- | --- |
| Support | LG, Nokia/NSB,vivo |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | Except the case of HP SR with PF0 collides with HP HARQ ACK PF1, is there any other case can be regarded the SR is dropped? If there is no any case, the updated proposal is same with original proposal. Is this understanding correct? |
| Sony | Similar comment with ZTE. Can we clarify what “dropped” means here. Technically, only case SR is dropped is SR in PF0 + HARQ-ACK in PF1. Does “dropped” also means negative SR? For the case where SR in PF1 + HARQ-ACK in PF1, do we consider SR is dropped in this case (because it isn’t dropped)? |
| Apple | Agree with ZTE and Sony, it is good to clarify, enumerating the underlying case(s) is better. |
| Ericsson | We have similar understanding as ZTE/Sony that the only case SR is dropped is: SR in PF0 + HARQ-ACK in PF1. For the case of multiple SR resources overlapping with another UCI, UE (MAC) is responsible to select one SR to trigger. The non-triggered ones are not considered as ‘dropped’ in our understanding. With the above, we prefer the previous version (i.e., limit to: SR in PF0 + HARQ-ACK in PF1). |
| Samsung | Support. Regarding E///’s comment on multiple positive SR resources, we have different understanding, MAC can trigger two non-overlapping SRs before receiving a DL grant indicating HARQ-ACK. The issue of multiple positive SRs was discussed in Rel-15, if there is only one positive SR, there is no need to discuss such issue in RAN1. With E///’s understanding, we need to revert Rel-15 agreement. |

For proposal 4.4, it is related to issue #1 in [109-e-R17-IIoT-URLLC-03]. The reason why it is discussed in this email thread is because the proposal from Samsung in [9] covers this case.

To moderator’s understanding, the question is whether negative HP SR multiplexed in Step 1 is considered in Step 2 if the resultant PUCCH for HP SR and HP HARQ-ACK is the same as PUCCH for HARQ-ACK only.

Question 1: Other than the pending issue of HP PUCCH with HARQ-ACK and SR with PF 0/1 overlapping with LP HARQ-ACK, do you see any other scenario which requires discussion of whether negative HP SR multiplexed in Step 1 is considered in Step 2 if the resultant PUCCH for HP SR and HP HARQ-ACK is the same as PUCCH for HARQ-ACK only?

|  | Company |
| --- | --- |
| Yes | Samsung |
| No | LG, Nokia/NSB, ZTE, Sony,vivo |

| Company | Comments |
| --- | --- |
| Ericsson | If only considering negative SR, it’s true that the resultant PUCCH is the same as HARQ-ACK PUCCH. On the other hand, we think it is good to have a unified solution to cover all cases of SR (PF0/1) + HARQ-ACK (PF0/1), positive and negative SR. For the case of SR (PF1) + HARQ-ACK (PF1), if positive SR, HARQ-ACK is transmitted on the SR PUCCH, according to Rel-15. This should be covered by the unified solution. |
| Samsung | When colliding with LP PUSCH, for similar reason, we should consider how to handle negative SR if we consider multiplexing negative SR with LP HARQ-ACK. We prefer a unified solution. |

Question 2: If your answer to Question 1 is no, do you agree to continue discussing the pending issue of HP PUCCH with HARQ-ACK and SR with PF 0/1 overlapping with LP HARQ-ACK in Rel-17 URLLC/IIoT intra-UE multiplexing/prioritization B ([109-e-R17-IIoT-URLLC-03] in this meeting) as in previous meetings?

|  | Company |
| --- | --- |
| Yes | LG, Nokia/NSB, ZTE, Sony,vivo, Ericsson |
| No |  |

| Company | Comments |
| --- | --- |
| Nokia/NSB | Important to move on, and being able to start discussions in Jia’s / MUX B thread on this one. |
| Ericsson | We are fine to continue the discussion in Mux B thread. |

- Reference

- R1-2203073		Intra-UE multiplexing enhancements	Huawei, HiSilicon

- R1-2203189		Discussion on enhanced intra-UE multiplexing	ZTE

- R1-2203213		Remaining issue on intra-UE multiplexing and prioritization	New H3C Technologies Co., Ltd.

- R1-2203274		Maintenance of Rel-17 URLLC / IIoT Intra-UE Multiplexing/Prioritization	Nokia, Nokia Shanghai Bell

- R1-2203305		Discussion on intra-UE multiplexing/prioritization	Spreadtrum Communications

- R1-2203398		Remaining Issues of Intra-UE Multiplexing/Prioritization for IIoT/URLLC	Ericsson

- R1-2203435		Maintenance on intra-UE multiplexing and prioritization	CATT

- R1-2203514		Remaining issues on intra-UE Multiplexing/Prioritization for Rel-17 URLLC	vivo

- R1-2203863		Maintenance on Intra-UE Multiplexing/Prioritization		Samsung

- R1-2204003		Remaining issues on intra-UE multiplexing/prioritization	OPPO

- R1-2204091		Remaining issues of Intra-UE Multiplexing/Prioritization	InterDigital, Inc.

- R1-2204206		Remaining issues in intra-UE multiplexing/prioritization	Apple

- R1-2204344		Remaining issues on intra-UE multiplexing/prioritization for Rel.17 URLLC	NTT DOCOMO, INC.

- R1-2204439		Remaining details on intra-UE multiplexing and prioritization	ITRI

- R1-2204547		Remaining issues on intra-UE multiplexing/prioritization for URLLC/IIoT	WILUS Inc.

- R1-2204617		Remaining issues on Intra-UE multiplexing enhancements	LG Electronics

- R1-2204647		Intra-UE Multiplexing/Prioritization	ETRI

- R1-2204770		Remaining issue for Intra-UE Uplink Channel Multiplexing and Prioritization	Intel Corporation

- R1-2204983		Remaining issues on Intra-UE multiplexing and prioritization for IOT and URLLC	Qualcomm Incorporated

- R1-2205146		Preparation phase FL summary for Rel-17 URLLC/IIoT intra-UE MUX A	Moderator (CATT)