3GPP TSG RAN WG1 Meeting #109-e	R1-2205152

e-Meeting, May 9 - 20, 2022

Agenda Item:	8.9

Source:	Moderator (Huawei)

Title:	Preparation phase discussion on 109-e-Prep-AI8.9 NB-IoT-eMTC

Document for:	Discussion and Decision

Introduction

This documents summarizes the preparation phase discussion of contributions submitted to AI 8.9 for Rel-17 WI NB-IoT and eMTC enhancements [2-5].

Issues

The issues are summarized in the following table.

| Issues | Summary of issues | Related contributions | Proposals |
| --- | --- | --- | --- |
| #1 | On whether and how to use the DwPTS in special subframes for NPDSCH with 16QAM. | [2] | Text proposal to TS 36.211: - On an NB-IoT carrier for which higher-layer parameter operationModeInfo indicates inband-SamePCI or inband-DifferentPCI, or higher-layer parameter inbandCarrierInfo is present, or on an NB-IoT carrier for SystemInformationBlockType1-NB for which sib1-carrierInfo indicates non-anchor and the value of the higher layer parameter sib-GuardbandInfo is set to sib-GuardbandInbandSamePCI or sib-GuardbandinbandDiffPCI, DwPTS in special subframe configuration 0 and 5 for normal cyclic prefix is not used for NPDCCH and NPDSCH transmission. DwPTS in special subframe configuration 9 for normal cyclic prefix is not used for NPDSCH transmission with 16QAM, when NPDSCH-16QAM-Config-NB is configured. |
|  |  | [4] | Text proposal to TS 36.211: On an NB-IoT carrier for which higher-layer parameter operationModeInfo indicates inband-SamePCI or inband-DifferentPCI, or higher-layer parameter inbandCarrierInfo is present, or on an NB-IoT carrier for SystemInformationBlockType1-NB for which sib1-carrierInfo indicates non-anchor and the value of the higher layer parameter sib-GuardbandInfo is set to sib-GuardbandInbandSamePCI or sib-GuardbandinbandDiffPCI, DwPTS in special subframe configuration 0 and 5 for normal cyclic prefix is not used for NPDCCH and NPDSCH transmission, and when npdsch-16QAM-Config-r17 is configured then DwPTS in special subframe configuration 9 for normal cyclic prefix is also not used for NPDSCH transmission. |
|  |  | [5] | Proposal 1: It is up to the eNB to ensure that NPDSCH transmission on DwPTS using 16-QAM is self-decodable (e.g. coding rate lower than 0.932) by the UE after rate matching. There is no need for specification change. |
| #2 | The power allocation for NPDSCH with 16QAM is missed for PDSCH in PUR procedure. | [3] | Text proposal to TS 36.213: If a UE is configured with higher layer parameters npdsch-16QAM-Config or pur-DL-16QAM-Config and nrs-PowerRatio, - the ratio of NPDSCH EPRE to NRS EPRE among NPDSCH REs in symbols with NRS is given by for a cell with one NRS antenna port and for a cell with two NRS antenna ports, where is given by the parameter nrs-PowerRatio. |

Discussion

It is proposed to have an email thread to address the two issues:

- One email thread to address the following issues for NB-IoT 16QAM

  - Issue #1: On whether and how to use the DwPTS in special subframes for NPDSCH with 16QAM.

    - Discussed in R1-2203223, R1-2204082 and R1-2204878

  - Issue #2: On the power allocation for NPDSCH with 16QAM in PUR procedure

    - Discussed in R1-2203631

Please input your comments for the proposed email discussion:

| Companies | Comments |
| --- | --- |
| Lenovo | For issue 1), we are fine to have a discussion/clarification in spec on NBIoT DL 16QAM in DwPTS in specifical subframe configuration 9. For issue 2), the current spec of TS36.213 includes - case of npdsch-16QAM-Config configured in PhysicalConfigDedicated-NB (e.g., connected mode) - case of npdsch-16QAM-Config configured in pur-PhysicalConfig (e.g., idle mode). So, we prefer not to have the CR. TS36.213 If a UE is configured with higher layer parameters npdsch-16QAM-Config and nrs-PowerRatio, TS36.331 PhysicalConfigDedicated-NB-r13 ::= SEQUENCE { …… [[ npusch-ConfigDedicated-v1700 NPUSCH-ConfigDedicated-NB-v1700 OPTIONAL, -- Need ON npdsch-ConfigDedicated-v1700 SetupRelease {NPDSCH-16QAM-Config-NB-r17} OPTIONAL, -- Need ON uplinkPowerControlDedicated-v1700 UplinkPowerControlDedicated-NB-v1700 OPTIONAL -- Cond npusch-16QAM ]] } pur-PhysicalConfig-v1700 SEQUENCE { pur-UL-16QAM-Config-r17 SetupRelease {PUR-UL-16QAM-Config-NB-r17} OPTIONAL, -- Need ON pur-DL-16QAM-Config-r17 SetupRelease {NPDSCH-16QAM-Config-NB-r17} OPTIONAL -- Need ON NPDSCH-16QAM-Config-NB-r17 ::=SEQUENCE{ nrs-PowerRatio-r17 ENUMERATED {dB-6, dB-4dot77, dB-3, dB-1dot77, dB0, dB1, dB2, dB3} OPTIONAL, nrs-PowerRatioWithCRS-r17 ENUMERATED {dB-6, dB-4dot77, dB-3, dB-1dot77, dB0, dB1, dB2, dB3} OPTIONAL -- Cond InBand } |
| Ericsson | We are ok with the FL’s proposal of having a single e-mail thread including both “Issue #1” and “Issue #2”. |
| Nokia, NSB | We are OK to discuss Issue#1. On Issue#2, we agree with Lenovo’s comment and think that there is no need for a CR. |
| ZTE, Sanechips | For issue1, we are OK to have the discussion. For issue2, we understand the comments from Lenovo. However, there would be some clairfications. - npdsch-16QAM-Config is not equal to NPDSCH-16QAM-Config. As seen, capitalized parameter is the correct name. - In RAN2, we usually use the lowercase for the starting of parameter name and uppercase for the starting of a IE structure parameter, e.g., NPDSCH-16QAM-Config. So, in RAN1, the correct parameter name should be referred to lowercase for the starting of parameter name. - In TS36.213, npdsch-16QAM-Config also shows in another place. We also need to check this parameter just contains one case or two cases mentioned by Lenovo. So, at least, in current spec, parameter name npdsch-16QAM-Config is not correct. How to address this issue can be further discussed. Ericsson’s suggestion, including both “Issue #1” and “Issue #2” in one eamil thread, is also OK for us |
| Moderator (Huawei) | For issue#1, all commenting companies agree to discuss it. For issue#2, companies have different understanding on the RRC parameters names in 36.331, I kind of agree that npdsch-16QAM-Config is different with NPDSCH-16QAM-Config-NB-r17 as the latter one has a suffix -NB, so it seems worthwhile to have some discussion whether some clarification in spec is needed. Therefore, the following is proposed for email discussion: - One email thread to address the following issues for NB-IoT 16QAM - Issue #1: On whether and how to use the DwPTS in special subframes for NPDSCH with 16QAM. - Discussed in R1-2203223, R1-2204082 and R1-2204878 - Issue #2: On clarification of the power allocation for NPDSCH with 16QAM in PUR procedure - Discussed in R1-2203631 |

Summary

The following is proposed for email discussion:

- One email thread to address the following issues for NB-IoT 16QAM

  - Issue #1: On whether and how to use the DwPTS in special subframes for NPDSCH with 16QAM.

    - Discussed in R1-2203223, R1-2204082 and R1-2204878

  - Issue #2: On clarification of the power allocation for NPDSCH with 16QAM in PUR procedure

    - Discussed in R1-2203631

- References

- RP-211340, “WID revision: Additional enhancements for NB-IoT and LTE-MTC”, Huawei, HiSilicon, RAN#92e, E-meeting, June 2021.

- R1-2203223	On use of DwPTS for 16QAM NPDSCH in NB-IoT	Huawei, HiSilicon

- R1-2203631	Clarifications for DL power allocation for 16-QAM	ZTE, Sanechips

- R1-2204082	Support of 16-QAM for unicast in UL and DL in NB-IoT	Ericsson

- R1-2204878	Support of 16-QAM in NB-IoT TDD	Nokia, Nokia Shanghai Bell