3GPP TSG-RAN WG1 #109-e	R1-2205156

e-Meeting, May 9 – 20, 2022

Agenda Item:	8.12.1

Source:	Moderator (Huawei)

Title:	FL summary#2 on improving reliability for MBS for RRC_CONNECTED UEs

Document for:	Discussion and Decision

# Introduction

Based on the preparation phase discussion, the following email discussion is organized for discussion maintenance issues for HARQ-ACK for multicast:

[109-e-R17-MBS-03] Email discussion/approval for maintenance on mechanisms to improve reliability for RRC_CONNECTED UEs, for issues #1-1, 1-2, 1-3, 1-4, 1-11, 1-12, 1-13, 1-14, 1-15 in R1-2205129 – Jinhuan (Huawei)

- 1st check point: May 13 (any RRC impact by May 12)

- Final check point: May 18

The issues to be discussed are summarized in the following table, where

- Issues to be discussed for agreements and up to editor for the spec impact include 1-1, 1-2, 1-3, 1-4, 1-11, 1-15.

- Issues to be discussed/approved in TP format include 1-12, 1-13, 1-14.

| Issue# | Issue | References |
| --- | --- | --- |
| 1-1 | Multicast HARQ-ACK on PUSCH: UL-DAI issues for multicast for Type1 and Type 2 codebooks, including the case for multiple configured G-RNTIs | Huawei, ZTE, TD-Tech, Nokia, Spreadtrum, CATT, vivo, Lenovo, Samsung, Apple, CMCC, DOCOMO, Google, Ericsson, Qualcomm |
| 1-2 | PUCCH resource determination when multiplexing HARQ-ACK for unicast and multicast (including the case of Alt1 for >1 TB where NACK-only is converted to ACK/NACK) | Huawei, ZTE, Nokia, CATT, vivo, Lenovo, Samsung, OPPO, CMCC, DOCOMO, MediaTek |
| 1-3 | NACK-only issues for Alt4, including: - The mapping between PUCCH resources and TB combinations - Codebook for NACK-only - PDSCH processing timeline - Multiplexing NACK-only with other UCI/PUSCH except issue 1-6 - power determination of a PUCCH transmission for the “NACK-only” reporting mode. | Huawei, ZTE, Nokia, CATT, vivo, NEC, Lenovo, Langbo, Samsung, OPPO, Apple, CMCC, DOCOMO, MediaTek, Ericsson |
| 1-4 | Type-1 HARQ codebook generation in cases where UE is configured with a mixture of ACK/NACK based HARQ and disabled HARQ for different G-RNTIs/G-CS-RNTIs | Huawei, ZTE, TD-Tech, Nokia, Spreadtrum, CATT, vivo, ETRI, Lenovo, OPPO, CMCC, DOCOMO, MediaTek, Ericsson, Qualcomm |
| 1-11 | TP/proposal for Type2 sub-codebook generation for G-CS-RNTI | CATT, Samsung |
| 1-12 | TP for reflecting PUCCH for NACK-only overlapping with CSI | Samsung |
| 1-13 | - TP#1 to correct TS38.213 that Alt1 can support more than 4 HARQ-ACK information bits; - TP#2 to clarify “other UCI” in clause 18 of TS38.213 does not include the other NACK-only HARQ-ACK nor CG-UCI | LGE |
| 1-14 | TP for removing the “PCell” condition for fallback operation with Type-1 HARQ-ACK codebook for multicast | Samsung |
| 1-15 | How to feedback for the SPS PDSCH associated with reactivation PDCCH, as described in section 4.1 of R1-2203194 | ZTE |

# Open issues

Issues to be discussed for agreements and up to editor for handling the spec impact include 1-1, 1-2, 1-3, 1-4, 1-11.

## (ACTIVE)Issue1-1: UL-DAI

Regarding UL-DAI for multicast, the following agreements were reached in the last meeting:

Agreement If Type-1 codebook is configured for both multicast and unicast, at least for single cell case for both unicast and multicast: - If the UE is configured to construct the HARQ-ACK codebooks for unicast and multicast jointly, a single UL DAI bit applies for unicast and multicast - Otherwise, 1 additional bit UL DAI is included for multicast in DCI format 0_1/0_2, in addition to the UL DAI for unicast. The 1-bit UL DAI for multicast is applied to all configured G-RNTIs. - FFS: additional restrictions Agreement If Type-2 codebook is configured for unicast and multicast, the following option2-2 (from the previous agreement) is adopted: - Option2-2: 2-bit UL DAI(s) are included in DCI for multicast, in addition to the 2-bit UL DAI for unicast. - The 2-bit UL DAI for multicast is applied for all configured G-RNTIs. - FFS: how to count the total number of HARQ-ACK bits for all configured G-RNTIs. - Alt1-1: UL-DAI indicates the sum of DL-DAIs - Alt1-2: DL-DAIs have the same value. - Alt1-3: largest DL DAI value among the configured G-RNTIs. - Other alternatives are not precluded Agreement When Type-1 codebook is configured for unicast and Type-2 codebook is configured for multicast, or when Type-2 codebook is configured for unicast and Type-1 codebook is configured for multicast, the UL-DAI for multicast is included in DCI format 0_1/0_2, in addition to the UL-DAI field for unicast. - The UL-DAI for multicast is 1-bit for Type-1 codebook - The 1-bit UL-DAI for multicast is applied to all configured G-RNTIs. - The UL-DAI for multicast is 2-bit for Type-2 codebook applied for all configured G-RNTIs. - FFS: how to count the total number of HARQ-ACK bits for all configured G-RNTIs. - Alt1-1: UL-DAI indicates the sum of DL-DAIs - Alt1-2: DL-DAIs have the same value. - Alt1-3: largest DL DAI value among the configured G-RNTIs. - Other alternatives are not precluded - FFS: details of the UL DAI field

This meeting continues to discuss the remaining FFS.  The proposals submitted are summarized as follows:

| R1-2203070 Huawei | Proposal 1: For Type-2 codebook configured for multicast, UL-DAI field for multicast indicates the largest value of DL-DAI in the scheduling DCI, and the number of HARQ-ACK bits on PUSCH is the product of the UL-DAI value with the number of G-RNTIs configured with HARQ-ACK feedback enabled. Proposal 2: The UL-DAI field for multicast is absent in DCI format 0_1/0_2 when disabling HARQ-ACK is configured by RRC signaling for all configured G-RNTIs. |
| --- | --- |
| R1-2203194 ZTE | Proposal 7: If a type2 codebook is configured for unicast and multicast, and if a type2 codebook is transmitted in a PUSCH scheduled by a UL grant, then the 2 bit-UL DAI in the UL grant is for all configured G-RNTIs. Either Alt1-2 or Alt1-3 is workable, and further downselection can be made between them. - Alt1-2: DL-DAIs have the same value. For example, the T-DAI value in the last DCI corresponding to each G-RNTI is the same in a type2 codebook. - Alt1-3: DL-DAIs can have different values, and the largest DL DAI value among the configured G-RNTIs is used for determining the size of sub-codebook corresponding to each G-RNTI in a type2 codebook. Proposal 8: When Type-1 codebook is configured for unicast and Type-2 codebook is configured for multicast, or when Type-2 codebook is configured for unicast and Type-1 codebook is configured for multicast, the UL-DAI for multicast is included in DCI format 0_1/0_2, in addition to the UL-DAI field for unicast. The UL-DAI for multicast is 2-bit for Type-2 codebook applied for all configured G-RNTIs. Alt1-2 should be adopted - Alt1-2: DL-DAIs have the same value. For example, the T-DAI value corresponding to each G-RNTI is the same in a type2 codebook. |
| R1-2203227 TD-Tech | Proposal: Alt1-1: UL-DAI indicates the sum of DL-DAIs is adopted to count the total number of HARQ-ACK bits for all configured G-RNTIs |
| R1-2203287 Nokia | Proposal 8: If Type-2 codebook is configured for unicast and multicast, 2-bit UL DAI(s) are included in DCI for multicast, in addition to the 2-bit UL DAI for unicast: - The 2-bit UL DAI for multicast that is set to largest DL DAI value among the configured G-RNTIs is applied for all configured G-RNTIs. - UE creates sub-codebooks per each G-RNTI with the size indicated as the UL DAI. - In case the largest received DL DAI is less than the indicated UL DAI value for some G-RNTIs, the UE could pad the corresponding sub-codebook, accordingly. Proposal 9: When Type-1 codebook is configured for unicast and Type-2 codebook is configured for multicast, the 2-bit UL DAI for multicast that is set to largest DL DAI value among the configured G-RNTIs is applied for all configured G-RNTIs. - UE creates sub-codebooks per each G-RNTI with the size indicated as the UL DAI. - In case the largest received DL DAI is less than the indicated UL DAI value for some G-RNTIs, the UE could pad the corresponding sub-codebook, accordingly Proposal 10: When NACK-only feedback is to be multiplexed in PUSCH (with and without unicast HARQ-ACK codebook), the same UL DAI mechanisms that were previously agreed for the case of ACK/NACK based feedback with Type-2 codebook are followed by the UE. |
| R1-2203314 Speadtrum | Proposal 3: If Type-1 codebook is configured for both multicast and unicast, for multiple cells case for both unicast and multicast: - If the UE is configured to construct the HARQ-ACK codebooks for unicast and multicast jointly, a single UL DAI bit applies for unicast and multicast - Otherwise, 1 additional bit UL DAI is included for multicast in DCI format 0_1/0_2, in addition to the UL DAI for unicast. The 1-bit UL DAI for multicast is applied to all configured G-RNTIs. Proposal 4: If Type-2 codebook is configured for unicast and multicast, the 2-bit UL DAI for multicast is applied for each configured G-RNTI. Proposal 5: If different codebook type is configured for unicast and multicast, for Type-2 codebook for multicast, the 2-bit UL DAI for multicast is applied for each configured G-RNTI. |
| R1-2203427 CATT | Proposal 6: UL DAI for multicast should be applied to each configured G-RNTI/G-CS-RNTI and the setting of the value of UL DAI can be up to the gNB implementation regardless of the value of DL DAI indicated for each G-RNTI/G-CS-RNTI. |
| R1-2203526 vivo | Proposal 1: For multiplexing unicast and multicast HARQ-ACK onto the same PUSCH with the same priority for the case multicast HARQ-ACK is configured with type 2 codebook: - The 2-bit UL-DAI is applied for all configured G-RNTIs and indicates the sum of DL-DAIs. |
| R1-2203699 Lenovo | Proposal 2: For Type-2 HARQ-ACK codebook for both unicast and multicast, UL DAI for multicast indicates the largest DL DAI values among the configured G-RNTIs, i.e., Alt 1-3 is supported. Proposal 3: For Type-2 HARQ-ACK codebook configured for multicast and Type-1 HARQ-ACK codebook configured for unicast, UL DAI for multicast indicates the largest DL DAI values among the configured G-RNTIs, i.e., Alt 1-3 is supported. |
| R1-2203874 Samsung | Proposal 8: For Type-2 HARQ-ACK codebook, a UE applies a DAI value in an UL grant per G-RNTI and applies the Rel-16 procedure to determine a multicast Type-2 HARQ-ACK codebook based on the DAI. |
| R1-2204216 Apple | Proposal 5: If multiple G-RNTIs are configured, 2bit total DAI in DCI format 4_2 is to indicate the all the scheduled PDSCH for a G-RNTI and the total DAIs in all the DCIs have the same value. |
| R1-2204282 CMCC | Proposal 6. When Type-2 codebook is configured for multicast, the 2-bit UL-DAI for multicast applied for all configured G-RNTIs indicates the sum of DL-DAIs for all configured G-RNTIs |
| R1-2204354 DOCOMO | Proposal 2 For how to count the total number of HARQ-ACK bits for all configured G-RNTIs, support Alt 1-3. - No spec change is necessary. |
| R1-2204502 Google | Proposal 1: Update Alt-1 with following. UE includes 2 bits of the last received C-DAI value (VC-DAI) of a G-RNTI at the beginning of the associated sub-codebook. |
| R1-2204945 Ericsson | If Type-2 codebook is configured for unicast and multicast the total number of HARQ-ACK bits is signalled with UL DAI indicating the sum of DL DAIs. If Type-2 codebook is configured for multicast and type-1 codebook is configured for unicast the total number of multicast HARQ-ACK bits is signalled with the multicast UL DAI indicating the sum of multicast DL DAIs. |
| R1-2204994 Qualcomm | Proposal 4: If the 2-bit UL DAI included in UL grant is for multicast feedback with Type-2 HARQ-ACK codebook, the UE assume same value of UL DAI per G-RNTI. - If the 2-bit DAI field for multicast feedback with dynamic HARQ-ACK codebook has the value of VT-DAIUL=4 and the UE has not received any PDCCH within the monitoring occasions with DCI format 4_1/4_2, the UE does not multiplex HARQ-ACK information for multicast feedback in the PUSCH transmission. - Endorse TP#2 for TS 38.213 |

### (CLOSED)Round-1

For Type-2 codebook, UL-DAI for multicast was agreed to be 2-bit applied to all configured G-RNTIs. Regarding how to count the total number of HARQ-ACK bits for all configured G-RNTIs, from the submitted papers:

- Support Alt1-1 (UL-DAI indicates the sum of DL-DAIs): TD-Tech, vivo, CMCC, Ericsson

- Support Alt1-2 (DL-DAIs have the same value): ZTE, Apple.

- Support Alt1-3 (largest DL DAI value among the configured G-RNTIs): Huawei, ZTE, Nokia, Lenovo, DOCOMO

- Support UL-DAI per G-RNTI: Spreadtrum, CATT, Samsung,

- UE assume same UL-DAI per G-RNTI: Qualcomm

- UE includes 2 bits of the last received C-DAI value (VC-DAI) of a G-RNTI at the beginning of the associated sub-codebook: Google

As analyzed in companies’ papers, Alt1-1 could not differentiate e.g., last two DCIs are missing from one G-RNTI or one DCI is missing from each of two G-RNTIs. Alt1-2 is apparently too restrictive to network scheduling. Alt1-3 is supported by slightly majority. It is also noted that four companies support to include an UL-DAI bit in UL grant for each G-RNTI, but that this is not consistent with the earlier agreement to use 2-bit UL DAI field for all configured G-RNTIs. FL understands the UL-DAI in Alt1-3 is also applied for each G-RNTI, so the only difference is whether to specify the UL-DAI is the largest DL DAI value among the configured G-RNTIs.

#### Proposal 2.1.1-1

For the 2-bit UL-DAI for multicast indicating the total number of Type-2 HARQ-ACK bits multiplexed in PUSCH, UE generates the Type-2 HARQ-ACK sub-codebook for each G-RNTI per the UL-DAI and concatenates all the sub-books in the ascending order of G-RNTI value.

- If the 2-bit UL-DAI field for multicast has the value of VT-DAIUL=4 and the UE has not received any PDCCH within the monitoring occasions with DCI format 4_1/4_2, the UE does not multiplex HARQ-ACK information for multicast feedback in the PUSCH transmission.

- The UL-DAI is the largest DL DAI value among the configured G-RNTIs

#### Company views

| Company | Comments |
| --- | --- |
| Nokia, NSB | We support the proposal with further clarification. #### Proposal 2.1.1-1 For the 2-bit UL-DAI for multicast indicating the total number of Type-2 HARQ-ACK bits multiplexed in PUSCH, UE generates the Type-2 HARQ-ACK sub-codebook for each G-RNTI per the UL-DAI and concatenates all the sub-codebooks in the ascending order of G-RNTI value. - If the 2-bit UL-DAI field for multicast has the value of VT-DAIUL=4 and the UE has not received any PDCCH within the monitoring occasions with DCI format 4_1/4_2, the UE does not multiplex HARQ-ACK information for multicast feedback in the PUSCH transmission. - The UL-DAI is the largest DL DAI value among the configured G-RNTIs - In case the largest received DL DAI is less than the indicated UL DAI value for some G-RNTIs, the UE pads the corresponding sub-codebook, accordingly. |
| Qualcomm | Our view is not to have separate UL DAI per G-RNTI, but one UL DAI field applied to all G-RNTIs. We think it is up to network to configure whether Alt1-2 or Alt1-3. But from UE perspective, UE just assume same UL-DAI per G-RNTI. If this is the intention of the main bullet of Proposal 2.1.1-1, we are fine with it. But we don’t think the second subbullet should be specified. #### Proposal 2.1.1-1 For the 2-bit UL-DAI for multicast indicating the total number of Type-2 HARQ-ACK bits multiplexed in PUSCH, UE generates the Type-2 HARQ-ACK sub-codebook for each G-RNTI per the UL-DAI and concatenates all the sub-books in the ascending order of G-RNTI value. - If the 2-bit UL-DAI field for multicast has the value of VT-DAIUL=4 and the UE has not received any PDCCH within the monitoring occasions with DCI format 4_1/4_2, the UE does not multiplex HARQ-ACK information for multicast feedback in the PUSCH transmission. |
| Samsung | Do not support the last sub-bullet. Our interpretation for the rest of the proposal is same as suggested by Qualcomm. It is a gNB issue how to set the value of the UL DAI – gNB behavior is not specified. Basically, the HARQ-ACK codebook construction in the PUSCH can directly follow the Rel-16 procedure for unicast. |
| LG Electronics | We support one UL DAI field applied to all G-RNTIs. Regarding the last sub-bullet, we are fine to keep this point to understand the expected gNB behavior. Whether to specify this gNB behavior can be up to the spec editor. |
| Lenovo | We support FL proposal and Nokia’s addition. |
| vivo | We still prefer Alt 1-1. It can guarantee the total size of the codebook is understood correctly between UE and gNB and at least UL-SCH can be decoded. Alt1-3 will potential increase the codebook size specifically when the number of G-RNTI is large. In addition, in NR Rel-16 NRU WI, when HARQ-ACK of two PDSCH groups are multiplexed on one PUSCH with 2 bit UL-DAI(the UL-DAI is configurable between 2 bit and 4bit), the UL-DAI is only applied to the first PDSCH group, and for the other PDSCH group, HARQ-ACK generation is same as that in PUCCH(with no UL-DAI). |
| OPPO | We are generally OK with FL’s proposal. |
| ZTE | Ok in principle. We would like to add one more bullet to further clarify. For the 2-bit UL-DAI for multicast indicating the total number of Type-2 HARQ-ACK bits multiplexed in PUSCH, UE generates the Type-2 HARQ-ACK sub-codebook for each G-RNTI per the UL-DAI and concatenates all the sub-books in the ascending order of G-RNTI value. - If the 2-bit UL-DAI field for multicast has the value of VT-DAIUL=4 and the UE has not received any PDCCH within the monitoring occasions with DCI format 4_1/4_2, the UE does not multiplex HARQ-ACK information for multicast feedback in the PUSCH transmission. - If the 2-bit UL-DAI field for multicast has the value of VT-DAIUL≠4 and the UE has not received any PDCCH within the monitoring occasions with DCI format 4_1/4_2, the UE multiplexes HARQ-ACK information for multicast feedback in the PUSCH transmission. - The UL-DAI is the largest DL DAI value among the configured G-RNTIs |
| NTT DOCOMO | We are fine with the proposal. |
| Spreadtrum | Agree with Qualcomm. It is up to gNB’s implementation to configure UL DAI same or different across G-RNTIs. But from UE side, it can be taken as the same UL DAI across G-RNTIs. |
| MediaTek | We share the similar view with QC, Samsung and Spreadtrum. |
| Ericsson | Support the proposal Question to FL: Should the proposal also mention that the intention is that the HARQ feedback multiplexed in PUSCH has a size of UL_DAI*nrof_GRNTIs? We haven’t captured how the UL DAI is intended to be used in any agreement. |
| Apple | One question for clarification for this proposal, if only one PDSCH is scheduled for a G-RNTI, and UE doesn’t detect the related PDCCH, how UE report the HARQ-ACK for the G-RNTI. The codebook doesn’t include the bits of this G-RNTI or report the bits as indicated by G-RNTI. With the current proposal, UL DAI could not check whether multicast PDSCH is missed or not. |
| CATT | Share same view with Qualcomm. We support the 2-bit UL-DAI of multicast is applied to all configured G-RNTIs. From UE perspective, there is no difference between Alt1-2 and Alt1-3, the UE only generates HARQ CB for each G-RNTI with legacy mechanism based on the indicated UL DAI. The value of UL DAI can be up to the gNB implementation regardless of the value of DL DAI indicated for each G-RNTI/G-CS-RNTI. |
| Google | We still prefer our proposal, which is a compromise between reliability and HARQ codebook size. We are also OK with FL’s proposal. It is true that how to set the UL-DAI is based on gNB behavior, but how a UE generates HARQ codebook according to the UL-DAI should be specified. Thus, Nokia’s modification should be added to the proposal. |

#### Summary

Based on the discussion: - QC, SS, Spreadtrum, MediaTek don’t agree the last bullet and view it should be up to gNB. - ZTE’s addition may not be needed because it is not even stated for legacy. To respond Ericsson’s comment: How to react to the UL-DAI is from UE perspective, “UE generates the Type-2 HARQ-ACK sub-codebook for each G-RNTI per the UL-DAI and concatenates all the sub-codebooks in the ascending order of G-RNTI value.” Was intended to address the total number of harq-ack bits are UL_DAI*nrof_GRNTIs To respond Apple’s comment: it is the same issue for legacy, UL-DAI indicates 1 PDSCH but UE does not detect DCI so UE still reports NACK because it means UE missed the DCI. I suppose the addition from Nokia is still the legacy procedure, i.e., normal UE behavior to react to UL-DAI indication, so no need to add to my understanding. Accordingly, the proposal is updated as: Proposal 2.1.1-1 For the 2-bit UL-DAI for multicast indicating the total number of Type-2 HARQ-ACK bits multiplexed in PUSCH, UE generates the Type-2 HARQ-ACK sub-codebook for each G-RNTI per the UL-DAI and concatenates all the sub-codebooks in the ascending order of G-RNTI value. - If the 2-bit UL-DAI field for multicast has the value of VT-DAIUL=4 and the UE has not received any PDCCH within the monitoring occasions with DCI format 4_1/4_2, the UE does not multiplex HARQ-ACK information for multicast feedback in the PUSCH transmission.

Likewise, when NACK-only feedback transmission overlaps with PUSCH transmission and then NACK-only is multiplexed in PUSCH by transforming into ACK/NACK based feedback, the UL-DAI can adopt the same approach.

#### Proposal 2.1.1-2

When NACK-only feedback is to be multiplexed in PUSCH (with and without unicast HARQ-ACK codebook), the 2-bit UL-DAI for multicast is applied for all configured G-RNTIs for Type-2 codebook.

  - Note: the case for Type-1 codebook for NACK-only (if supported) will be separately discussed.

#### Company views

| Company | Comments |
| --- | --- |
| Nokia, NSB | Ok in principle, however, we prefer to avoid using the Type-2 codebook terminology and either: 1. Wait until a conclusion on the NACK-only DL DCI fields is made, since that should resolve the terminology we should use 2. Rephrase this agreement to remove the Type-2 codebook terminology. |
| Qualcomm | Understand the intention of the proposal. NACK-only will be treated in a similar way as ACK/NACK, when it is transformed to ACK/NACK. #### Proposal 2.1.1-2 When NACK-only feedback is to be transformed to ACK/NACK and multiplexed in PUSCH, the 2-bit UL-DAI for multicast is applied for all configured G-RNTIs for Type-2 codebook. - Note: the case for Type-1 codebook for NACK-only (if supported) will be separately discussed. |
| Samsung | Support. Also OK with the revision by Qualcomm. |
| LG Electronics | We support Qualcomm’s revision. |
| Lenovo | OK with Qualcomm’s update. |
| vivo | We support Qualcomm’s revision. |
| OPPO | We are OK with the update by Qualcomm. |
| ZTE | OK with Qualcomm’s update. |
| NTT DOCOMO | OK with Qualcomm’s update. |
| Spreadtrum | OK with Qualcomm’s update. |
| MediaTek | OK with Qualcomm’s update. |
| Ericsson | Support |
| Apple | In general, Ok with this proposal. Just for clarification, with this proposal, it is assumed the DL DAI field is applied to NACK-only feedback, right? - Downlink assignment index – number of bits as defined in the following - 2 bits if the higher layer parameter pdsch-HARQ-ACK-Codebook-Multicast=dynamic, where the 2 bits are the counter DAI; - 0 bits otherwise. |
| CATT | We are OK with the update by Qualcomm. |

#### Summary

To respond Apple: yes, DL-DAI is applied to NACK-only. As majority suggested, the proposal is updated as: Proposal 2.1.1-2 When NACK-only feedback is to be transformed to ACK/NACK and multiplexed in PUSCH, the 2-bit UL-DAI for multicast is applied for all configured G-RNTIs.

UL-DAI in UL grant for multicast was agreed to be one additional bit or to share the existing bit for Type-1 codebook for the single cell case. How to set the UL-DAI in UL grant for the case of UE being configured with multiple cells for unicast (but a single cell for multicast) is missing. Proposal 2.1.1-3 is to solve the UL-DAI for the multiple cell case.

#### Proposal 2.1.1-3

When UE is configured with multiple cells for unicast and Type-1 codebook is configured for both multicast and unicast,

- if the UE is configured to construct the HARQ-ACK codebooks for unicast and multicast jointly, a single UL DAI bit applies for unicast and multicast;

- Otherwise, 1 additional bit UL DAI is included for multicast in DCI format 0_1/0_2, in addition to the UL DAI for unicast. The 1-bit UL DAI for multicast is applied to all configured G-RNTIs.

#### Company views

| Company | Comments |
| --- | --- |
| Nokia, NSB | Ok. |
| Qualcomm | Thanks for FL to raise this issue. It seems RAN1 hasn’t discussed the codebook generation for PUCCH for this case yet. If CA is for unicast and single CC is for multicast, does UE generate concatenated sub-codebook for unicast and multicast? |
| Samsung | Support |
| LG Electronics | OK |
| Lenovo | OK |
| vivo | OK |
| OPPO | OK |
| ZTE | OK |
| NTT DOCOMO | OK |
| Spreadtrum | OK |
| Ericsson | Support |
| Apple | OK. |
| CATT | OK |

#### Summary

To respond QC’s comment: With the agreement that multicast is only configured in a single cell, the earlier agreement of generating Type-1 codebook is still workable by only counting one cell for multicast. Please correct me if others have different view. With this clarified, the proposal does not need change.

### (ACTIVE)Round-2

After the 1st GTW session, the pending proposal is:

Proposal 2.1.1-2-r1 When NACK-only feedback is to be transformed to ACK/NACK and multiplexed in PUSCH, the 2-bit UL-DAI for multicast is applied for all configured G-RNTIs with HARQ feedback enabled by RRC configuration when Type-2 codebook is configured for multicast.

Solving the comments from GTW:

- It is essentially counting the number of HARQ-ACK bits to be multiplexed to PUSCH. Whether it is always 2 bits if Type-1-like codebook is supported for NACK-only

- What if some G-RNTIs are configured with ACK/NACK but others are configured with NACK-only?

#### Proposal 2.1.2

TBD (After solving how to count the HARQ-ACK bits for NACK-only in section 2.3.2)

## (ACTIVE)Issue1-2: PUCCH resource for Alt1

The issue to be discussed is PUCCH resource determination when multiplexing HARQ-ACK for unicast and multicast (including the case of Alt1 for >1 TB where NACK-only is converted to ACK/NACK) and the other related text proposal to be discussed in TP#5 in section 3.5.

Alt1 UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits was agreed in the last meeting with FFS how to determine PUCCH resource.

Agreement For supporting more than one NACK-only feedback in the same PUCCH transmission, define RRC configuration to configure between Alt1 and Alt4 (from previous agreements): - Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits. - FFS: how to determine PUCCH resource - Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission. - define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback, - FFS: The PUCCH slot for the transmission is based on the K1 in the “last DCI” scheduling multicast. - FFS: The PUCCH resource for the transmission is from PUCCH-config configured for NACK-only based feedback according to the mapping between number of TBs with PUCCH resource ID. - FFS mapping details. - How to determine the number of TBs is discussed separately, e.g., Type-1-like and/or Type-2-like codebook. - FFS: whether this applies to a single G-RNTI or multiple G-RNTIs - Alt4 is not supported for more than 4 TBs - FFS: whether RRC configuration between Alt1 and Alt4 is per G-RNTI or per CFR - FFS: UE capability

The following submitted proposals are related to determining the PUCCH resources for Alt1.

| R1-2203070 Huawei | Proposal 4: Adopt the following text proposal to TS38.213 v17.1.0 - Reason for change: - The current TS38.213 v17.1.0 does not clarify the determined PUCCH resource for multiplexed HARQ-ACK of unicast and multicast is provided by PUCCH-config for unicast or PUCCH-config-Multicast. - Summary of change: - Clarify the determined PUCCH resource for multiplexed HARQ-ACK of unicast and multicast is provided by PUCCH-config for unicast. - Consequence if not approved: - It is not clear that the determined PUCCH resource is provided by PUCCH-config for unicast or PUCCH-config-Multicast, especially the names of the parameters included in PUCCH-config-Multicast are exactly the same as those included in PUCCH-config for unicast. |  |
| --- | --- | --- |
| R1-2203194 ZTE | Proposal 1: In order to support NACK only feedback of more than one MBS PDSCHs by converting NACK only to HARQ-ACK, dedicated PUCCH resources are defined to transmit multiplexed UCIs from multiple NACK only. |
| R1-2203287 Nokia | Proposal 1: For Alt. 1, to transmit the multiplexed HARQ-ACK feedback, the UE is configured by RRC signalling with specific dedicated PUCCH resource(s) that the UE utilizes to transmit multiplexed NACK-only bits, rather than selecting the PUCCH resource based on the PRI field of the last DCI. - Dedicated PUCCH resource(s) can be configured within the NACK-only or unicast PUCCH-config. Proposal 2: For Alt. 1., to transmit the multiplexed HARQ-ACK feedback, concatenation of the converted NACK-only bits is based on the ascending order of PUCCH resource IDs that are originally scheduled for NACK-only feedback. |
| R1-2203427 CATT | Proposal 1: When more than one NACK-only feedback is transmitted in the same PUCCH transmission and Alt 1 is applied, the PUCCH resource can be determined by one of the following options: - Option 1: using PUCCH resource configured for unicast If UE is indicated to provide the HARQ-ACK information bits in a PUCCH according to Alt.1, regardless whether separate PUCCH-Config/PUCCH-ConfigurationList for NACK-only is configured, PUCCH-Config/PUCCH-ConfigurationList for unicast applies. - Option 3: using PUCCH resource configured for NACK-only with PUCCH format 2, format 3 and format 4 If UE is indicated to provide the HARQ-ACK information bits in a PUCCH according to Alt.1, and a number of HARQ-ACK information bits is more than 2 bits, the UE can be configured with either one of PUCCH format 2, format 3 or format 4 in PUCCH-Config-Multicast2/PUCCH-ConfigurationList-Multicasts when separate PUCCH-Config/PUCCH-ConfigurationList for NACK-only is configured. |
| R1-2203526 vivo | Proposal 2: When more than one NACK-only based feedback is available for transmission in the same PUCCH slot, - If Alt 1 is used, the PUCCH for transmission is determined in the same way as ACK/NACK mode. - If Alt 4 is used, - The PUCCH slot is determined based on the K1 indication in each corresponding scheduling DCI - The mapping between TBs and PUCCH resources are predefined. - This applies to a single G-RNTI. - Two separate UE capabilities should be defined for alt 1 and alt 4 correspondingly. |
| R1-2203874 Samsung | for specification simplicity but also for offering some flexibility in PUCCH resource indication, PRI-based resource indication can apply for PDSCHs scheduled by DCI formats (and the SPS-PUCCH-AN-List can be used for SPS PDSCHs). |
| R1-2203974 OPPO | - For more than one NACK-only feedback in the same PUCCH transmission with Alt 1, the PUCCH resource can be up to gNB implementation with proper scheduling. |
| R1-2204282 CMCC | Proposal 1. For supporting more than one NACK-only feedback in the same PUCCH transmission, when Alt 1 is configured by RRC signalling, the PUCCH resource is determined by the PUCCH resource configured for unicast. - the PUCCH slot and PUCCH resource index for the transmission is based on the K1 and PRI in the “last DCI” scheduling multicast. |
| R1-2204354 DOCOMO | When Alt 1 (i.e., moreThanOneNackOnly mode 1) is applied, the PUCCH resource should be determined by the same method as ACK/NACK based feedback. |
| R1-2204717 MediaTek | Proposal 3: The PUCCH resource of NACK-only based feedback configured via RRC signalling is reused for the Alt 1, and the “last DCI” scheduling multicast is used to determine the PUCCH resource ID. |

### (CLOSED)Round-1

Based on the submitted proposals, the supportive options are as follows:

Opt1: dedicated PUCCH resources configured for multiplexing more than one NACK-only.

- ZTE, Nokia

Opt2: PUCCH resources configured for unicast based on k1/PRI in the last DCI scheduling multicast

- CATT, vivo, Samsung, OPPO, CMCC, DOCOMO, MediaTek

Opt3: PUCCH resources configured for NACK-only with PF2/3/4

- CATT

The argument for Opt1 is that different UEs may be receiving different MBS services, and unicast PUCCH configurations would be different for different UEs, this would increase the complexity of the gNB scheduling orthogonal resources for the multiplexed HARQ feedback, i.e., the last PRI in the DCIs cannot be used for all the UEs to select an optimal PUCCH resource to transmit a multiplexed HARQ-ACK feedback (which can include HARQ feedback of different MBSs at each UE). Some other companies (e.g., vivo, DOCOMO) also pointed out that it is similar to ACK/NACK-based feedback for multicast which is based on k1 and PRI included in the last DCI to determine the PUCCH transmission resource. The difference is that the last DCI for ACK/NACK based is the DCI scheduling unicast but for NACK-only based the last DCI is the DCI scheduling multicast. In addition, even though the PRI included in the last DCI scheduling multicast is the common value for all UEs receiving the group-common PDCCH/PDSCH, the PRI can still be mapped to UE-specific PUCCH resources that are orthogonal as if the PRI is indicated in the DCI scheduling unicast. Given there are vast majority support for Opt2, FL suggests taking Opt2.

#### Proposal 2.2.1

For a UE configured with NACK-only HARQ-ACK feedback, when NACK-only HARQ-ACK bits are transformed into ACK/NACK HARQ-ACK bits, the PUCCH resource used for transmitting the multiplexed HARQ-ACK bits is determined from PUCCH-Config/PUCCH-ConfigurationList configured for unicast based on the k1 and the PRI indication in the last DCI scheduling multicast.

#### Company views

| Company | Comments |
| --- | --- |
| Nokia, NSB | Not support. This alternative would require synchronization of unicast PUCCH-config for each UE, to provide orthogonality using the same k1 and PRI of the multicast DCI. In addition, not all the UEs will receive the same services, i.e., last DCI scheduling multicast would be different for different UEs. Hence, scheduling decisions to ensure orthogonality in unicast PUCCH-config are expected to be quite complex. Therefore, we would like to adopt Opt. 1, which would be a simple solution from the network perspective. |
| Samsung | Support |
| LG Electronics | Support |
| Lenovo | OK |
| vivo | We think when NACK only is transformed into ACK/NACK, it can follow the same way as that of ACK/NACK mode to determine PUCCH, multiplexing with other UCI(e.g., with CSI as discussed in the following section), i.e., from the PUCCH/PUCCH-configList for ACK/NACK for multicast, based on the last DCI. |
| OPPO | Support |
| ZTE | Support |
| NTT DOCOMO | Support |
| MediaTek | Support |
| Ericsson | Support. Our understanding is that this applies when there is no multiplexing of other feedback. |
| Apple | OK |
| CATT | Support |

#### Summary

To respond Nokia’s comment: For ACK/NACK based feedback for multicast, k1 and PRI included in DCI is the same for all UEs, but UE can still be managed to use the UE-specific PUCCH for the HARQ-ACK feedback, so this reasoning is applied to NACK-only based as well. In addition, when NACK-only transformed into ACK/NACK based, it is then UE specific feedback, the last DCI is the last from UE perspective. I don’t see the proposal for the case that last DCI scheduling multicast would be different for different UEs. To respond vivo’s comment: ACK/NACK based and NACK-only based is configured by RRC. If NACK-only is configured, UE is probably not configured with PUCCH/PUCCH-configList for ACK/NACK for multicast. This is why taking resources from unicast is proposed to my understanding. With these responses, the proposal is kept the same.

### (CLOSED)Round-2

The following proposal from 1st GTW is still pending:

Proposal 2.2.1 For a UE configured with NACK-only HARQ-ACK feedback, when NACK-only HARQ-ACK bits are transformed into ACK/NACK HARQ-ACK bits, the PUCCH resource used for transmitting the multiplexed HARQ-ACK bits is determined from PUCCH-Config/PUCCH-ConfigurationList configured for unicast based on the k1 and the PRI indication in the last DCI scheduling multicast. - FFS in the case where the UE is also configured with ACK/NACK-based feedback for a different G-RNTI - This also applies in the case where the UE is also configured with ACK/NACK-based feedback for a different G-RNTI - FFS in the case where the UE is not configured with PUCCH-Config/PUCCH-ConfigurationList for unicast

Firstly, FL wants to clarify as the following agreement states ACK/NACK or NACK-only is configured per G-RNTI and if the PUCCH resources is not configured for multicast, then the PUCCH resource configured for unicast applies:

Agreement: For UE supporting both ACK/NACK based and NACK-only based feedback for multicast, for the same G-RNTI, support the following - UE can be configured with either ACK/NACK based or NACK-only feedback for a single G-RNTI. - Note: Case1-1: if configured with ACK/NACK based feedback, UE can be optionally configured a separate PUCCH-Config/PUCCH-ConfigurationList for multicast. Otherwise, PUCCH-Config/PUCCH-ConfigurationList for unicast applies (This has been agreed.) - Case 1-2: if configured with NACK-only based feedback, when separate PUCCH-Config/PUCCH-ConfigurationList for NACK-only is not configured, PUCCH-Config/PUCCH-ConfigurationList for unicast applies.

With this agreement, the last bullet FFS is not valid which means when NW does not configure the PUCCH resources for NACK-only, NW is supposed to configure PUCCH resource for unicast.

In addition, we earlier also agreed that when NACK-only for one G-RNTI collides with PUCCH transmission for ACK/NACK feedback for another G-RNTI with the same priority, UE will do multiplexing as follows with unclear point being which PUCCH resource is used for this multiplexing:

Agreement When PUCCH transmission for the NACK-only based feedback for one G-RNTI collides with PUCCH transmission for ACK/NACK feedback for another G-RNTI with the same priority, support UE multiplexing the NACK-only based feedback with the ACK/NACK feedback onto the same PUCCH by transforming NACK-only into the ACK/NACK based HARQ-ACK bit. - Note: When the TB configured with NACK-only feedback is correctly decoded, the ACK will be transmitted and multiplexed with others.

These two issues could be discussed together as in the following FL proposal. The first bullet is about NACK-only feedback for one G-RNTI multiplexed with ACK/NACK with the other G-RNTI. The second bullet is purely for G-RNTI(s) configured with NACK-only feedback.

#### Proposal 2.2.2

- For the case when the PUCCH transmission for the NACK-only based feedback for one G-RNTI collides with the PUCCH transmission for ACK/NACK feedback for another G-RNTI with the same priority and then UE multiplexes the NACK-only based feedback with the ACK/NACK feedback onto the same PUCCH by transforming NACK-only into the ACK/NACK based HARQ-ACK bit, the PUCCH resources for transmitting the multiplexed HARQ-ACK bits is from the PUCCH-Config/PUCCH-ConfigurationList configured for multicast with ACK/NACK based feedback.

- For a UE configured with G-RNTI(s) with NACK-only HARQ-ACK feedback, when NACK-only HARQ-ACK bits are transformed into ACK/NACK HARQ-ACK bits, the PUCCH resource used for transmitting the multiplexed HARQ-ACK bits is determined from PUCCH-Config/PUCCH-ConfigurationList configured for unicast based on the k1 and the PRI indication in the last DCI scheduling multicast.

#### Company views

| Company | Comments |
| --- | --- |
| Apple | OK with this proposal. |
| Nokia, NSB. | We are ok with the first bullet. Regarding second bullet, the issue is different than the case where ACK/NACK feedback is configured for the UEs and same PRI and k1 schedule orthogonal resources. In our view, multicast PUCCH-config for ACK/NACK feedback can be configured in an appropriate way for this purpose. However, now, we are discussing that the unicast PUCCH-config of the UE should also be configured with such restriction, so that the orthogonality is achieved. In our view, no such restriction should be introduced to the configuration of unicast PUCCH-config. This is the reason why we propose dedicated/configured resources for transmitting multiple NACK-only bits converted to the ACK/NACK, in case of Alt.1. In addition, if we were to achieve orthogonal resources scheduled in unicast PUCCH-config with the same PRI and k1, it could be argued that one PUCCH resource set in the unicast PUCCH-config can be configured in a way that one or several PRIs can be used to achieve orthogonality. However, what if there are some UEs receiving some services, and some other UEs receiving other services, and some UEs receiving several services. In our view, this would even create some capacity issues in existing unicast PUCCH resource set when trying to maintain orthogonality, that could be avoided by using a separate dedicated PUCCH resource(s). |
| Lenovo | OK with 1st bullet. On 2nd bullet, we share same concern with Nokia. |
| Samsung | OK with first bullet. OK with the second bullet to progress and close the issue (although we don’t think the agreement cited by the moderator as a justification is relevant to this case). We don’t consider the case of the second bullet of practical relevance – the simplest mechanism that completes the specifications is acceptable. |
| NTT DOCOMO | OK |
| vivo | OK with the 1st bullet. For the second bullet, actually, we prefer to use the PUCCH-config/PUCCHconfigList for ACK/NACK mode for multicast. If it is not configured, using the PUCCH-config/PUCCHconfigList for unicast. If most companies prefer to use the PUCCH-config/PUCCHconfigList for unicast, we are ok. In addition, it is possible that the NACK-only is for SPS PDSCH only, there is no PRI and PUCCH resource list for unicast SPS should be used in that case. |
| LG Electronics | We have the same view with Vivo. We prefer to use the PUCCH-config/PUCCHconfigList for ACK/NACK mode for multicast, if configured. |
| ZTE | We are fine with the proposal. |
| CATT | We are ok with the proposal. |

#### Summary

Based on the comments: For 1st bullet NACK-only multiplexed with ACK/NACK - Support proposal: Apple, Nokia, Samsung, DOCOMO, vivo, ZTE, CATT For the 2nd bullet NACK-only transformed into ACK/NACK - Support proposal: Apple, DOCOMO, ZTE, CATT - Dedicated configured resources: Nokia, Lenovo - Use the PUCCH-config/PUCCHconfigList for ACK/NACK mode for multicast. If it is not configured, using the PUCCH-config/PUCCHconfigList for unicast: vivo, LGE

### (ACTIVE)Round-3

Nokia/Lenovo argued that: if the same PRI and k1can achieve orthogonal resources scheduled in unicast PUCCH-config, it could be argued that one PUCCH resource set in the unicast PUCCH-config can be configured in a way that one or several PRIs can be used to achieve orthogonality.

Vivo and LGE suggested using the PUCCH-config/PUCCHconfigList for ACK/NACK mode for multicast. If it is not configured, using the PUCCH-config/PUCCHconfigList for unicast. Since Nokia commented that multicast PUCCH-config for ACK/NACK feedback can be configured in an appropriate way to schedule orthogonal resources by the same PRI and k1. Maybe this can be a compromised solution.

#### Proposal 2.2.3

- For the case when the PUCCH transmission for the NACK-only based feedback for one G-RNTI collides with the PUCCH transmission for ACK/NACK feedback for another G-RNTI with the same priority and then UE multiplexes the NACK-only based feedback with the ACK/NACK feedback onto the same PUCCH by transforming NACK-only into the ACK/NACK based HARQ-ACK bit, the PUCCH resources for transmitting the multiplexed HARQ-ACK bits is from the PUCCH-Config/PUCCH-ConfigurationList configured for multicast with ACK/NACK based feedback.

- For a UE configured with G-RNTI(s) with NACK-only HARQ-ACK feedback, when NACK-only HARQ-ACK bits are transformed into ACK/NACK HARQ-ACK bits, the PUCCH resource used for transmitting the multiplexed HARQ-ACK bits is determined from PUCCH-Config/PUCCH-ConfigurationList configured for multicast based on the k1 and the PRI indication in the last DCI scheduling multicast. If PUCCH-Config/PUCCH-ConfigurationList configured for multicast is not configured, the PUCCH-Config/PUCCH-ConfigurationList configured for unicast is used.

- FFS: the case NACK-only is for multicast SPS PDSCH only.

#### Company views

| Company | Comments |
| --- | --- |
| Samsung | OK |
| Lenovo | OK with the first two bullets. Regarding the FFS, one thing may need be clear is NACK-only for multicast SPS in this FFS is referring to the first bullet or the second bullet or both? |
| vivo | OK. Regarding the FFS, we think it is for the second bullet, since there is no DCI in this case. |
| NTT DOCOMO | OK |
| LG Electronics | OK with the proposal. |
| CATT | OK |
| MediaTek | Ok for the first bullet. For the second bullet, from my understating, bases on the previous agreements, either the PUCCH for NACK-only feedback or the PUCCH for ACK/NACK is configured by RRC, and the NW will not configure the PUCCH for ACK/NACK and NACK-only feedback simultaneously. Please correct me if my understanding is incorrect. So, at the current stage, we still prefer the previous version. |
| ZTE | OK |

## (ACTIVE)Issue1-3: NACK-only issues for Alt4

As discussed in preparation phase, the issues to be discussed include:

- The mapping between PUCCH resources and TB combinations

- Codebook for NACK-only

- PDSCH processing timeline

- Multiplexing NACK-only with other UCI/PUSCH except issue 1-6

- power determination of a PUCCH transmission for the “NACK-only” reporting mode.

### (CLOSED)TB vs PUCCH mapping

Alt4 is agreed and details of define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback need to be fixed:

Agreement For supporting more than one NACK-only feedback in the same PUCCH transmission, define RRC configuration to configure between Alt1 and Alt4 (from previous agreements): - Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits. - FFS: how to determine PUCCH resource - Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission. - define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback, - FFS: The PUCCH slot for the transmission is based on the K1 in the “last DCI” scheduling multicast. - FFS: The PUCCH resource for the transmission is from PUCCH-config configured for NACK-only based feedback according to the mapping between number of TBs with PUCCH resource ID. - FFS mapping details. - How to determine the number of TBs is discussed separately, e.g., Type-1-like and/or Type-2-like codebook. - FFS: whether this applies to a single G-RNTI or multiple G-RNTIs - Alt4 is not supported for more than 4 TBs - FFS: whether RRC configuration between Alt1 and Alt4 is per G-RNTI or per CFR - FFS: UE capability

The proposals presented to this meeting:

| R1-2203070 Huawei | Proposal 6: For Alt 4 if more than one NACK-only feedback are available for transmission in the same PUCCH slot, support the following mapping between PUCCH resource IDs and decoding results of TBs combinations, where the TBs are ordered according to the sequence in which they are received: TB combination PUCCH resource IDs; TB#1; TB#1-TB#2; TB#1-TB#2-TB#3; TB#1-TB#2-TB#3-TB#4. PUCCH resource ID#0; N; N-N/DTX; N-N-N; N-N-N-N. PUCCH resource ID#1; A-N/DTX; A-N-N; A-N-N-N. PUCCH resource ID#2; N-A; N-A-N; N-A-N-N. PUCCH resource ID#3; A-A-N; A-A-N-N. PUCCH resource ID#4; N-N-A; N-N-A-N. PUCCH resource ID#5; A-N-A; A-N-A-N. PUCCH resource ID#6; N-A-A; N-A-A-N. PUCCH resource ID#7; A-A-A-N. PUCCH resource ID#8; N-N-N-A. PUCCH resource ID#9; A-N-N-A. PUCCH resource ID#10; N-A-N-A. PUCCH resource ID#11; A-A-N-A. PUCCH resource ID#12; N-N-A-A. PUCCH resource ID#13; A-N-A-A. PUCCH resource ID#14; N-A-A-A. Proposal 7: When more than one NACK-only feedbacks are available for transmission in the same PUCCH slot, Alt 4 can be applied to both single/multiple G-RNTIs case. |
| --- | --- |
| R1-2203194 ZTE | Proposal 2: In order to support NACK only feedback of no more than 4 MBS PDSCHs in the same PUCCH, the number and HARQ-ACK feedback order of the MBS PDSCHs are determined based on the DAI mechanism. Proposal 3: In order to support NACK only feedback of no more than 4 MBS PDSCHs in the same PUCCH, Table 1 for the mapping relationship between up to 15 PUCCH resources and MBS PDSCHs decoded as NACK should be adopted. - The MBS PDSCHs can correspond to a single G-RTNI or multiple G-RTNIs based on base station implementation. One or more MBS PDSCH decoded as NACK; PUCCH resources. PDSCH0; PUCCH 0. PDSCH1; PUCCH 1. PDSCH2; PUCCH 2. PDSCH3; PUCCH 3. PDSCH0 and PDSCH1; PUCCH 4. PDSCH0 and PDSCH2; PUCCH 5. PDSCH0 and PDSCH3; PUCCH 6. PDSCH1 and PDSCH2; PUCCH 7. PDSCH1 and PDSCH3; PUCCH 8. PDSCH2 and PDSCH3; PUCCH 9. PDSCH0, PDSCH1 and PDSCH2; PUCCH 10. PDSCH 0, PDSCH1 and PDSCH3; PUCCH 11. PDSCH0, PDSCH2 and PDSCH3; PUCCH 12. PDSCH1, PDSCH2 and PDSCH3; PUCCH 13. PDSCH0, PDSCH1, PDSCH2 and PDSCH3; PUCCH 14. |
| R1-2203287 Nokia | Proposal 3: In Alt. 4, the UE is configured with a specific mapping between the NACK-only sequences of the decoding results of the scheduled TBs and PUCCH resource IDs to utilize per G-RNTI using RRC signalling. |
| R1-2203427 CATT | Table 1. Mapping of HARQ-ACK value to a PUCCH resource in the first PUCCH resource set with maximum 16 PUCCH resources HARQ-ACK Value; PUCCH resource. {0}; {0,0}; {0,0,0}; {0,0,0,0}; 1st PUCCH resource provided by pucch-ResourceId obtained from the 1st value of resourceList. {1,0}; {1,0,0}; {1,0,0,0}; 2nd PUCCH resource provided by pucch-ResourceId obtained from the 2nd value of resourceList. {0,1}; {0,1,0}; {0,1,0,0}; 3rd PUCCH resource provided by pucch-ResourceId obtained from the 3rd value of resourceList. {1,1,0}; {1,1,0,0}; 4th PUCCH resource provided by pucch-ResourceId obtained from the 4th value of resourceList. {0,0,1}; {0,0,1,0}; 5th PUCCH resource provided by pucch-ResourceId obtained from the 5th value of resourceList. {1,0,1}; {1,0,1,0}; 6th PUCCH resource provided by pucch-ResourceId obtained from the 6th value of resourceList. {0,1,1}; {0,1,1,0}; 7th PUCCH resource provided by pucch-ResourceId obtained from the 7th value of resourceList. {1,1,1,0}; 8th PUCCH resource provided by pucch-ResourceId obtained from the 8th value of resourceList. {0,0,0,1}; 9th PUCCH resource provided by pucch-ResourceId obtained from the 9th value of resourceList. {1,0,0,1}; 10th PUCCH resource provided by pucch-ResourceId obtained from the 10th value of resourceList. {0,1,0,1}; 11th PUCCH resource provided by pucch-ResourceId obtained from the 11th value of resourceList. {1,1,0,1}; 12th PUCCH resource provided by pucch-ResourceId obtained from the 12th value of resourceList. {0,0,1,1}; 13th PUCCH resource provided by pucch-ResourceId obtained from the 13th value of resourceList. {1,0,1,1}; 14th PUCCH resource provided by pucch-ResourceId obtained from the 14th value of resourceList. {0,1,1,1}; 15th PUCCH resource provided by pucch-ResourceId obtained from the 15th value of resourceList. Proposal 2: When more than one NACK-only feedback is transmitted in the same PUCCH transmission and Alt 4 is applied, the PUCCH slot is based on the K1 indication of the scheduling DCI and the end slot of the corresponding PDSCH. |
| R1-2203526 vivo | The mapping between of TBs with PUCCH resource ID can be predefined. For example, one TB is mapped to the first PUCCH resource configured in the PUCCH-config for NACK-only. 4TBs is mapped to the 16th PUCCH resource configured in the PUCCH-config for NACK-only. As discussed in previous meetings, it would be questionable for alt 4 in multiple G-RNTIs case. Since there would be more than 16 combinations for up to 4 TBs. For example, if UE is configured with two G-RNTIs, there are 32 TB and G-RNTI combinations for up to 4TBs. Then, the 15 PUCCH resources are not enough to cover all combinations. The TB and G-RNTI combinations will increase explosively when UE is configured with more G-RNTIs. Thus, we prefer to apply alt4 to a single G-RNTI case. Proposal 4: When more than one NACK-only based feedback is available for transmission in the same PUCCH slot, - If Alt 1 is used, the PUCCH for transmission is determined in the same way as ACK/NACK mode. - If Alt 4 is used, - The PUCCH slot is determined based on the K1 indication in each corresponding scheduling DCI - The mapping between TBs and PUCCH resources are predefined. - This applies to a single G-RNTI. - Two separate UE capabilities should be defined for alt 1 and alt 4 correspondingly. |
| R1-2203677 NEC | Table 1 Relationship between different TB combination and shift cyclic NACK-only for; m0+mcs; NACK-only for; m0+mcs. TB1; 0; TB1 and TB3; 6. TB2; 1; TB2 and TB3; 8. TB3; 2; TB1, TB2 and TB3; 10. TB1 and TB2; 4. Table 2 Relationship between different TB combination and shift cyclic NACK-only for; m0+mcs; timeDomainOCC. TB1; 0; 0. …; …; …. TB6; 0; 5. TB1 and TB2; 0; 6. …; …; …. TB5 and TB6; 1; 6. …; …; …. All TBs; 11; 6. |
| R1-2203699 Lenovo | Proposal 10: The PUCCH slot is determined based on K1 value indicated by the last multicast DCI. Proposal 11: A set of PUCCH resources for mapping multiple NACK-only feedback multiplexing is configured by RRC signaling. Proposal 13: The PUCCH resource for transmission is determined according to the HARQ-ACK feedback combination and the PUCCH mapping table. Proposal 14: The PUCCH mapping table is defined based on the number of TBs and the PUCCH format. Proposal 15: The PUCCH mapping table is applied to both single G-RNTI case and multiple G-RNTI case. |
| R1-2203874 Samsung | Another FFS is the PUCCH resource mapping. The FFS says “between number of TBs with PUCCH resource ID” but it is HARQ-ACK values, not TBs, that are mapped to PUCCH resource ID. Any mapping works as long as corresponding PUCCH resources are orthogonal. For example, the mapping in Table 1 as discussed in previous meetings can apply. Table 1: Mapping of values of HARQ-ACK information bits to PUCCH resources HARQ-ACK Values; PUCCH resource. {0}; {0, 0}; {0, 0, 0}; {0, 0, 0, 0}; 1st PUCCH resource in resourceList. {1, 0}; {1, 0, 0}; {1, 0, 0, 0}; 2nd PUCCH resource in resourceList. {0, 1}; {0, 1, 0}; {0, 1, 0, 0}; 3rd PUCCH resource in resourceList. {1, 1, 0}; {1, 1, 0, 0}; 4th PUCCH resource in resourceList. {0, 0, 1}; {0, 0, 1, 0}; 5th PUCCH resource in resourceList. {1, 0, 1}; {1, 0, 1, 0}; 6th PUCCH resource in resourceList. {0, 1, 1}; {0, 1, 1, 0}; 7th PUCCH resource in resourceList. {1, 1, 1, 0}; 8th PUCCH resource in resourceList. {0, 0, 0, 1}; 9th PUCCH resource in resourceList. {1, 0, 0, 1}; 10th PUCCH resource in resourceList. {0, 1, 0, 1}; 11th PUCCH resource in resourceList. {1, 1, 0, 1}; 12th PUCCH resource in resourceList. {0, 0, 1, 1}; 13th PUCCH resource in resourceList. {1, 0, 1, 1}; 14th PUCCH resource in resourceList. {0, 1, 1, 1}; 15th PUCCH resource in resourceList. Proposal 1: HARQ-ACK codebook construction is same for both HARQ-ACK reporting modes. For the “NACK-only” mode and reporting in a PUCCH, the 1/2/3/4 HARQ-ACK information bits are mapped to 1/3/7/15 orthogonal PUCCH resources (e.g. as in Table 1). |
| R1-2204216 Apple | Table 1: HARQ-ACK bit mapping to PUCCH resource HARQ-ACK feedback; PUCCH resource. TB1; TB1,TB2; TB1,TB2,TB3; TB1,TB2,TB3,TB4. N; N,N; N,N,N; N,N,N,N; 1st PUCCH resource. N,A; N,A,N; N,A,N,N; 2nd PUCCH resource. A,N; A,N,N; A,N,N,N; 3rd PUCCH resource. N,N,A; N,N,A,N; 4th PUCCH resource. N,A,A; N,A,A,N; 5th PUCCH resource. A,N,A; A,N,A,N; 6th PUCCH resource. A,A,N; A,A,N,N; 7th PUCCH resource. N,N,N,A; 8th PUCCH resource. N,N,A,A; 9th PUCCH resource. N,A,N,A; 10th PUCCH resource. A,N,N,A; 11th PUCCH resource. N,A,A,A; 12th PUCCH resource. A,N,A,A; 13th PUCCH resource. A,A,N,A; 14th PUCCH resource. A,A,A,N; 15th PUCCH resource. Proposal 4: Adopt the above HARQ-ACK bit mapping to PUCCH resource table for NACK-only feedback mode Alt4 |
| R1-2204282 CMCC | Proposal 2. For supporting more than one NACK-only feedback in the same PUCCH transmission, when Alt 4 is configured by RRC signalling, define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback, - The PUCCH slot for the transmission is based on the K1 in the last DCI scheduling multicast. - The PUCCH resource for the transmission is from PUCCH-Config configured for NACK-only based feedback according to the mapping between number of TBs with PUCCH resource ID. - This applies to a G-RNTI/G-CS-RNTI or multiple RNTIs. Table 1. Mapping of HARQ-ACK value to a PUCCH resource in the first PUCCH resource set HARQ-ACK Value; PUCCH resource. {0}; {0,0}; {0,0,0}; {0,0,0,0}; 1st PUCCH resource provided by pucch-ResourceId obtained from the 1st value of resourceList. {1,0}; {1,0,0}; {1,0,0,0}; 2nd PUCCH resource provided by pucch-ResourceId obtained from the 2nd value of resourceList. {0,1}; {0,1,0}; {0,1,0,0}; 3rd PUCCH resource provided by pucch-ResourceId obtained from the 3rd value of resourceList. {1,1,0}; {1,1,0,0}; 4th PUCCH resource provided by pucch-ResourceId obtained from the 4th value of resourceList. {0,0,1}; {0,0,1,0}; 5th PUCCH resource provided by pucch-ResourceId obtained from the 5th value of resourceList. {1,0,1}; {1,0,1,0}; 6th PUCCH resource provided by pucch-ResourceId obtained from the 6th value of resourceList. {0,1,1}; {0,1,1,0}; 7th PUCCH resource provided by pucch-ResourceId obtained from the 7th value of resourceList. {1,1,1,0}; 8th PUCCH resource provided by pucch-ResourceId obtained from the 8th value of resourceList. {0,0,0,1}; 9th PUCCH resource provided by pucch-ResourceId obtained from the 9th value of resourceList. {1,0,0,1}; 10th PUCCH resource provided by pucch-ResourceId obtained from the 10th value of resourceList. {0,1,0,1}; 11th PUCCH resource provided by pucch-ResourceId obtained from the 11th value of resourceList. {1,1,0,1}; 12th PUCCH resource provided by pucch-ResourceId obtained from the 12th value of resourceList. {0,0,1,1}; 13th PUCCH resource provided by pucch-ResourceId obtained from the 13th value of resourceList. {1,0,1,1}; 14th PUCCH resource provided by pucch-ResourceId obtained from the 14th value of resourceList. {0,1,1,1}; 15th PUCCH resource provided by pucch-ResourceId obtained from the 15th value of resourceList. |
| R1-2204354 DOCOMO | Proposal 8: For NACK-only based HARQ-ACK feedback for multicast, use the following table to determine the PUCCH resource. HARQ-ACK information; PUCCH resource. 1 bit; 2 bits; 3 bits; 4 bits. ‘0’; '00'; '000'; ‘0000’; 1st PUCCH resource in the PUCCH resource set. ‘1’; '10'; '100'; ‘1000’; 2nd PUCCH resource in the PUCCH resource set. '01'; '010'; ‘0100’; 3rd PUCCH resource in the PUCCH resource set. '11'; '110'; ‘1100’; 4th PUCCH resource in the PUCCH resource set. '001'; ‘0010’; 5th PUCCH resource in the PUCCH resource set. '101'; ‘1010’; 6th PUCCH resource in the PUCCH resource set. '011'; ‘0110’; 7th PUCCH resource in the PUCCH resource set. '111'; ‘1110’; 8th PUCCH resource in the PUCCH resource set. ‘0001’; 9th PUCCH resource in the PUCCH resource set. ‘1001’; 10th PUCCH resource in the PUCCH resource set. ‘0101’; 11th PUCCH resource in the PUCCH resource set. ‘1101’; 12th PUCCH resource in the PUCCH resource set. ‘0011’; 13th PUCCH resource in the PUCCH resource set. ‘1011’; 14th PUCCH resource in the PUCCH resource set. ‘0111’; 15th PUCCH resource in the PUCCH resource set. ‘1111’; 16th PUCCH resource in the PUCCH resource set. Proposal 9: moreThanOneNackOnly mode 2 applies to multiple G-RNTIs. - When a UE reports NACK-only based feedback for multiple G-RNTIs, the UE generates a sub-codebook for each G-RNTI and concatenates them in the order of G-RNTIs. |
| R1-2204717 MediaTek | Proposal 5: For Alt4, the PUCCH resource for the transmission is from PUCCH-config configured for NACK-only based feedback according to the pre-defined mapping relationship between number of TBs and PUCCH resource ID. Proposal 6: Alt4 applies to the all G-RNTIs for supporting more than one NACK-only feedback in the same PUCCH transmission. |
| R1-2204945 Ericsson | A UE configured with NACK only is always configured with a separate PUCCH configuration for NACK only if there is more than one NACK-only feedback bit. The PRI in the DCI is interpreted by UEs in NACK-only mode to select one of the PUCCH resource pools configurations for NACK-only. Multiple PUCCH resources pools could be configured for dynamic selection via PRI. This has the same advantage as the PRI method has for legacy ACK/NACK feedback, i.e. it provides flexibility to the gNB to more dynamically dedicate a PUCCH resource to a legacy ACK/NACK feedback configured for a particular UE or for the NACK-only feedback configured for a particular G-RNTI. |

#### Round-1

Regarding the mapping between HARQ-ACK values and the PUCCH resources for the TBs configured with NACK-only, Huawei, CATT, Samsung, Apple, CMCC, DOCOMO proposed the same mapping table. With the table proposed, companies also propose the table can be applied to a single G-RNTI and multiple configured G-RNTIs.

#### Proposal 2.3.1.1

For UE configured with NACK-only HARQ-ACK feedback, for Alt4 (from previous agreement), define the mapping between HARQ-ACK values and the PUCCH resources for the TBs configured with NACK-only as follows:

- The mapping is applied to both a single G-RNTI and multiple configured G-RNTIs cases.

| HARQ-ACK Value |  |  |  | PUCCH resource |
| --- | --- | --- | --- | --- |
| {0} | {0,0} | {0,0,0} | {0,0,0,0} | 1st PUCCH resource provided by pucch-ResourceId obtained from the 1st value of resourceList |
|  | {1,0} | {1,0,0} | {1,0,0,0} | 2nd PUCCH resource provided by pucch-ResourceId obtained from the 2nd value of resourceList |
|  | {0,1} | {0,1,0} | {0,1,0,0} | 3rd PUCCH resource provided by pucch-ResourceId obtained from the 3rd value of resourceList |
|  |  | {1,1,0} | {1,1,0,0} | 4th PUCCH resource provided by pucch-ResourceId obtained from the 4th value of resourceList |
|  |  | {0,0,1} | {0,0,1,0} | 5th PUCCH resource provided by pucch-ResourceId obtained from the 5th value of resourceList |
|  |  | {1,0,1} | {1,0,1,0} | 6th PUCCH resource provided by pucch-ResourceId obtained from the 6th value of resourceList |
|  |  | {0,1,1} | {0,1,1,0} | 7th PUCCH resource provided by pucch-ResourceId obtained from the 7th value of resourceList |
|  |  |  | {1,1,1,0} | 8th PUCCH resource provided by pucch-ResourceId obtained from the 8th value of resourceList |
|  |  |  | {0,0,0,1} | 9th PUCCH resource provided by pucch-ResourceId obtained from the 9th value of resourceList |
|  |  |  | {1,0,0,1} | 10th PUCCH resource provided by pucch-ResourceId obtained from the 10th value of resourceList |
|  |  |  | {0,1,0,1} | 11th PUCCH resource provided by pucch-ResourceId obtained from the 11th value of resourceList |
|  |  |  | {1,1,0,1} | 12th PUCCH resource provided by pucch-ResourceId obtained from the 12th value of resourceList |
|  |  |  | {0,0,1,1} | 13th PUCCH resource provided by pucch-ResourceId obtained from the 13th value of resourceList |
|  |  |  | {1,0,1,1} | 14th PUCCH resource provided by pucch-ResourceId obtained from the 14th value of resourceList |
|  |  |  | {0,1,1,1} | 15th PUCCH resource provided by pucch-ResourceId obtained from the 15th value of resourceList |

#### Company views

| Company | Comments |
| --- | --- |
| Nokia, NSB | Ok. |
| Qualcomm | Ok in principle |
| Samsung | Support The exact mapping does not matter (as long as different bits are mapped to orthogonal resources). Should be generally agreeable to pick any “reasonable” mapping. |
| NEC | Agree. The mapping between the HARQ-ACK bit and the specific PDSCH should be further identified. In detail, it should be aligned between UE and NW that the HARQ-ACK bit represent a same PDSCH feedback. Simply, for the signal G-RNTI, the HARQ-ACK bit position corresponds to the receiving time of PDSCH (or scheduled DCI). For multiple G-RNTIs, the HARQ-ACK feedbacks of PDSCHs with the same G-RNTI can be constructed in the adjacent location within the codebook. |
| LG Electronics | OK |
| Lenovo | OK in principle We think the detailed mapping table can be up to editor. |
| OPPO | OK |
| NTT DOCOMO | Support |
| MediaTek | Ok |
| Ericsson | Not support. We remain with our proposal that the PRI plays a role in the mapping from HARQ-ACK values and the PUCCH resources. We don't see how Proposal 2.3.1.1 can be agreed without having a agreement on the use of PRI first. Therefore, we do not agree to Proposal 2.3.1.1. We suggest to first discuss on the use of PRI. Additionally, a mapping as in Proposal 2.3.1. needs to be complemented with the clarification that the number of TBs in the table refers to the number of TBs in the codebook (CB) and not to the number of TBs received per UE. The size of the CB may be 1-4 and the number of TBs er UE may also be 1-4 but the CB is always equal to or larger than the number of TBs per UE. Consider e.g. four different G-RNTIs: G-RNTI1-4 with TB1-4 sharing the same NACK-only Codebook. If UE1 is configured with only G-RNTI2, even if the UE is only configured with one G-RNTI (1 TB) it shall not use the PUCCH resource corresponding to TB1 but the PUCCH resource corresponding to NACK for TB2, if relevant, or no feedback (if ACK). |
| Apple | Ok with this proposal for signal G-RNTI case. For multiple G-RNTIs case, how to split the TBs is not clear, such as TB1 and TB2 is for G-RNTI 1, TB3 and TB4 is for G-RNTI2 or other spliting. So multiple G-RNTI mapping can be FFS. |
| CATT | Support |

#### Summary

To respond NEC’s comment: I guess the comment is regarding the order of the HARQ-ACK bits because the HARQ-ACK is bounded to a TB, one bullet “the TBs are ordered according to the sequence in which they are received” is added above the table. To respond Apple: the TBs should be naturally ordered according to the sequence in which they are received. To respond Ericsson: I assume the majority support this proposal are assuming PRI will not be used. I don’t mind adding a FFS whether/how PRI is used for indication. Proposal 2.3.1.1 For UE configured with NACK-only HARQ-ACK feedback, for Alt4 (from previous agreement), define the mapping between HARQ-ACK values and the PUCCH resources for the TBs configured with NACK-only as follows: - The mapping is applied to both a single G-RNTI and multiple configured G-RNTIs cases. - The TBs are ordered according to the sequence in which they are received - FFS whether/how PRI is used for indication

### (ACTIVE)Codebook for NACK-only

The following FL proposal was discussed in the last meeting but was not concluded:

Proposal 3.4.2 For determining the HARQ-ACK feedback for NACK-only to be transmitted in the same PUCCH (for either Alt1 or Alt4), - based on C-DAI included in the scheduling DL DCI, the total number of HARQ-ACK bits for all G-RNTIs configured with NACK-only is the sum of C-DAI in the last DCI from each G-RNTI. - Note1: It is similar to Type-2 codebook size determination for ACK/NACK based feedback. - Note2: C-DAI is independently counted per G-RNTI configured with NACK-only. - Note3: Whether Alt4 is applied to a single G-RNTI or all configured G-RNTI can still be discussed further. - FFS: based on the k1 set and TDRA table configured by higher layer signaling, - Note4: It is similar to Type-1 codebook size determination for ACK/NACK based feedback. - The number of HARQ-ACK bits consists of all G-RNTIs configured with NACK-only based feedback

The proposals submitted are summarized as follows:

| R1-2203070 Huawei | Proposal 8: For determining the HARQ-ACK feedback for NACK-only to be transmitted in the same PUCCH (for either Alt1 or Alt4), support the following: - based on C-DAI included in the scheduling DL DCI, the total number of HARQ-ACK bits for all G-RNTIs configured with NACK-only is the product of the largest value of DL-DAI in the scheduling DCI with the number of G-RNTIs configured. - Note1: It is similar to Type-2 codebook size determination for ACK/NACK based feedback. - Note2: C-DAI is independently counted per G-RNTI configured with NACK-only. |
| --- | --- |
| R1-2203194 ZTE | Proposal 2: In order to support NACK only feedback of no more than 4 MBS PDSCHs in the same PUCCH, the number and HARQ-ACK feedback order of the MBS PDSCHs are determined based on the DAI mechanism. |
| R1-2203287 Nokia | Proposal 4: For determining the HARQ-ACK feedback for NACK-only to be transmitted in the same PUCCH (for either Alt. 1 or Alt. 4), - The number of NACK-only bits to be transmitted is based on C-DAI included in the scheduling DL DCI. - Note1: It is similar to Type-2 codebook size determination for ACK/NACK based feedback. - Note2: C-DAI is independently counted per G-RNTI configured with NACK-only. - Note3: For Alt. 1, the total number of HARQ-ACK bits for all G-RNTIs configured with NACK-only is the sum of C-DAI in the last DCI from each G-RNTI, as for ACK/NACK feedback. |
| R1-2203427 CATT | Proposal 3: When more than one NACK-only feedback is transmitted in the same PUCCH transmission and Type-2-like codebook is configured for Alt 4, the number of TBs is determined by the maximum number of PUCCH resource indicated for the UE. |
| R1-2203699 Lenovo | Proposal 12: The HARQ-ACK feedback combination is determined based on Type-2 HARQ-ACK codebook. |
| R1-2203874 Samsung | For the Type-1 HARQ-ACK CB, as the TDRA entries and the K1 values are same for all G-RNTIs, there is no issue with supporting “NACK-only” for one G-RNTI or for multiple G-RNTIs. Although Type-1 HARQ-ACK CB may not be meaningful for “NACK-only” reporting mode (e.g. if there are non-overlapping TDRA entries for the typical use case of one PDSCH/slot), it is trivial to directly support by existing specifications and there is no reason to preclude. Therefore, current specifications are essentially complete with the only thing missing being a mapping table for combinations of {ACK, NACK} values to a PUCCH resource. |
| R1-2203974 OPPO | - For more than one NACK-only feedback in the same PUCCH transmission with Alt 4, Type-1-like codebook design can be considered to determine the number of TBs. - Alt 4 is applied to a single G-RNTI. |
| R1-2204216 Apple | Proposal 3: 2bit DAI field in DCI format 4_2 is applied for NACK-only feedback as counter DAI |
| R1-2204282 CMCC | Proposal 3. For determining the HARQ-ACK feedback for NACK-only to be transmitted in the same PUCCH (for either Alt1 or Alt4), only Type-2 codebook like size determination for ACK/NACK based feedback is supported. |
| R1-2204354 DOCOMO | Proposal 11: For NACK-only based HARQ-ACK feedback for multicast, support only dynamic HARQ-ACK codebook. |
| R1-2204945 Ericsson | For NACK-only transmission of HARQ feedback for group scheduling, a dedicated NACK-only CB type is supported, which is different from both semistatic and dynamic ACK/NACK codebooks A NACK-only CB codebook can be configured for single G-RNTI or for multiple G-RNTIs. For single G-RNTI configuration of NACK-only CB, the DCI includes the 2-bit cDAI field, which is incremented for each TB of the G-RNTI within the CB. For multiple G-RNTI configuration of NACK-only CB, the DCI includes a 2-bit cDAI* field, which is counted across all G-RNTI TBs of the NACK-only CB. The DCI may in addition be configured with the cDAI field. |

#### (CLOSED)Round-1

Based on the submitted proposals:

Type-1-Like codebook for NACK-only

- Samsung, OPPO

Type-2-Like codebook for NACK-only

- ZTE, Nokia, CATT, Lenovo, Samsung, Apple, CMCC, DOCOMO, Huawei,

different from Type-1/2:

- Ericsson: include C-DAI* in DCI counting PDSCH across multiple G-RNTIs which all UEs will receive

Type-1 codebook is not so meaningful for “NACK-only” reporting but it is stated in [12] it is trivial to directly support by existing specifications and there is no reason to preclude and the UE procedures and specifications can be same as for the “ACK/NACK” reporting mode with the only difference being the use of the “NACK-only” reporting mode (for the mapping of HARQ-ACK values to PUCCH resources and for the PUCCH transmission only in case of at least one incorrect TB reception). In addition, it is also stated that it is also useful when “NACK-only” is converted to “ACK/NACK” as a UE does not need to apply any additional procedure other than either selecting a resource to report HARQ-ACK values for “NACK-only” or directly reporting them by individual “ACK” or “NACK” values. Therefore, current specifications are essentially complete with the only thing missing being a mapping table for combinations of {ACK, NACK} values to a PUCCH resource.

To solve the issue of counting the number of HARQ-ACK bits from more than one G-RNTI for which C-DAI is separately configured, one company in [20] proposes to include C-DAI* counting PDSCH across multiple G-RNTIs which all UEs will receive.

Given the vast majority supports Type-2-like codebook due to the benefits in PUCCH overhead reduction, FL suggests taking Type-2-like codebook for NACK-only based feedback.

#### Proposal 2.3.2.1

For determining the HARQ-ACK feedback for NACK-only to be transmitted in the same PUCCH (for either Alt1 or Alt4),

- The total number of HARQ-ACK bits for all G-RNTIs configured with NACK-only is the sum of the C-DAI values in each of the last scheduling DCI for each G-RNTI.

- The order of HARQ-ACK bits is in the ascending order of G-RNTI values.

#### Company views

| Company | Comments |
| --- | --- |
| Nokia, NSB | Ok. |
| Qualcomm | It’s not clear to us how to let UE understand a ‘C-DAI’ counting PDSCH across all G-RNTIs for all UEs. Does it mean we need to let a UE know the number of G-RNTIs for other UEs? |
| Samsung | The proposal is somewhat confusing and seems unnecessary. RRC configuration allows separate configurations for the HARQ-ACK codebook and for ACK/NACK or NACK-only. As long as no restriction is agreed here (e.g. to limit only to Type-2), it is unclear what else needs to be said. The only difference between ACK/NACK and NACK-only will be in the PUCCH resource selection and the specifications are already complete. |
| Lenovo | Support. We understand this proposal is to generate the NACK-only bit combination following already agreed Type-2 HARQ-ACK codebook for Multicast. Although this generation is not perfect since it has HARQ-ACK codebook ambiguity, we think it is an accept way. |
| vivo | If type 1 codebook is configured for multicast, there would be no C-DAI in the scheduling DCI. then how to determine the codebook in that case? |
| ZTE | OK |
| NTT DOCOMO | Support |
| Ericsson | Not support. The proposal does not work unless all UEs are configured to receive the same set of G-RNTIs and is also unnecessarily sensitive to missed PDCCHs for multiple G-RNTIs. Assume e.g. that HARQ feedback for G-RNTI1 (TB1, TB2) and G-RNTI2 (TB3 and TB4) share the same NACK-only Codebook, and UE1 is configured to receive both G-RNTI1 and G-RNTI2 whereas UE2 is configured to only receive G-RNTI2. If both UEs receive all PDCCHs UE1 can count to 4 HARQ-ACK bits using the sum of the cDAIs. However, UE2 will only count to 2 bits. This means that UE2 will transmit its 2 bits for TB3 and TB4 in the same CB positions as UE1 puts TB1 and TB2. In addition, even if the two UEs would be configured with the same G-RNTIs, but e.g. UE2 would miss e.g. TB1, UE2 will then infer that HARQ-ACK is for 3 bits and will use CB positions 1, 2 and 3 for TB2, TB3 and TB4, which will destroy the whole CB. To solve this, there needs to be a cDAI (or cDAI*, as we call it in our contribution) to be counted over all G-RNTIs. With the use of cDAI* any received PDCCH (of any G-RNTI) can immediately be mapped to the right CB position, which allows for UEs to be configured with different sets of G-RNTIs (all sharing the same NACK-only CB) and also for some PDCCHs to be missed without destroying the whole CB. Regarding the second bullet point, we disagree. With a cDAI* (counted across all G-RNTIs of the codebook) in the DCI, the order of the HARQ-ACK bits is rather the same order as the cDAI* increments, so the G-RNTI/TB with cDAI*=’00’ will map to TB1 in the CB etc. |
| Apple | OK with this proposal. is it common understanding that DL DAI field is applied for NACK-only feedback as counter DAI? |
| CATT | Not support If the total number of HARQ-ACK feedback for NACK-ACK is determined by the sum of the C-DAI values in each of the last scheduling DCI for each G-RNTI, the issue on the generation of CB for NACK-only will happen when the last DCI is missed. Assuming one case in the following table that the C-DAI carried in the last DCI transmitted by gNB is 4, and the actual C-DAI of the last DCI received by UE is 3. When the first three TBs are correctly decoded by the UE and the forth TB is not received, the UE will feedback nothing because the HARQ-ACK value corresponds to {1,1,1} instead of {1,1,1,0}. In this case, the gNB will mistakenly regard that all 4 TBs have been decoded correctly by the UE. gNB(The last DCI transmitted); UE(The last DCI received ); HARQ-ACK value; PUCCH resource. Case-1: last DCI missed; C-DAI=4; C-DAI=3,; {1,1,1,×}; none. Case-2: last DCI received; C-DAI=4; C-DAI=4; {1,1,1,0}; 8th PUCCH resource. To avoid the above issue, one potential method is that the total number of HARQ-ACK bits for all G-RNTIs configured with NACK-only is determined by the number of PUCCH resources configured for NACK-only feedback. For example, if the number of PUCCH resources configured for NACK-only feedback is 16, the HARQ-bits will be 4 bits. Furthermore, we think the type-1-like codebook should be studied considering integrity of the spec. |

#### Summary

To respond QC’s comment: - The intention was not to count PDSCHs across all G-RNTIs for all UEs. It is from a given UE perspective, meaning the counting is only across the G-RNTIs configured to the UE. To respond SS’s comment: - The intention is to preclude Type-1 as majority commented that there is no benefit for the support. To respond vivo’s comment: - If we preclude Type-1 for NACK-only, then the Type-1 can only be configured for ACK/NACK based for multicast, which means C-DAI will be included DCI. To respond Ericsson’s comment - Thanks for the comments. I think at least the agreed mapping TB at least is applicable to the case of different G-RNTIs that are common to all UEs in the same group. Regarding the case when UEs are configured with different G-RNTIs, your first point seems valid to me. - You second point of the missing DCI issue, if not considering UEs are configured with different G-RNTIs, the issue can be solved by the C-DAI as legacy, please refer to my response to CATT’s comment for the issue. - As to solution, UE counts the number of HARQ-ACK bits based on C-DAI* counting across all G-RNTIs even for G-RNTI not configured the UE technically works to me. It can be an open solution to be discussed. To respond CATT’s comment - For the issue, I acknowledge it happens but it is just from a single UE perspective. As long as NOT all UEs from the same group received that TB correctly but was missed by a certain UE, gNB will retransmit that TB per the table agreed. In addition, the last DCI missing issue also happens to Type2 CB for legacy and even worse than NACK-only because no chance for others reporting N for that TB. - For the solution, the number of resources are configured by RRC semi-statically, but the number of TBs scheduled is dynamic and I agree it can be a solution to count the number of HARQ-ACK bits for NACK-only.

#### (CLOSED)Round-2

From the 1st GTW session discussion, the mapping between HARQ-ACK values and the PUCCH resources for the TBs configured with NACK-only with the following FFSs:

Agreement For UE configured with NACK-only HARQ-ACK feedback, for Alt4 (from previous agreement), define the mapping between HARQ-ACK values and the PUCCH resources for the TBs configured with NACK-only as follows: - The mapping is applied to both a single G-RNTI - FFS: multiple configured G-RNTIs cases. - FFS: how to order the TBs, whether/how to account for missed or non-scheduled TBs - FFS whether/how PRI is used for indication.

At least we need to work out how to count the number of HARQ-ACK bits for Alt4 for a single G-RNTI case. The DCI missing case could happen, but as what I responded to CATT that as long as NOT all UEs from the same group received that TB correctly but was missed by a certain UE, gNB will retransmit that TB per the table agreed. In addition, the last DCI missing issue also happens to Type2 CB for legacy and even worse than NACK-only because no chance for others reporting N for that TB. Then the question is on top of this whether we need to introduce one more solution to address the DCI missing issue specifically. At least Type-1-like codebook is not exactly the Type-1 codebook for ACK/NACK based, because it is based on RRC configured K1 set and TDRA to determine the number of HARQ-ACK bits and it would be very likely to exceed 4 TBs and it is unclear how to control the TBs to be lower than 4TB. CATT suggested one solution that the number of HARQ-bits is based on the number of PUCCH resource configured, i.e., 4TBs correspond to 15 resources, 3TB correspond to 7 resources, etc.

Regarding whether the mapping table agreed is applicable to more than one G-RNTI, Ericsson has a point that whether all UEs are configured with the same set of G-RNTIs matters. Ericsson also suggested one solution that C-DAI* across all configured G-RNTIs.

To progress further, I summarized all the cases and possible solutions as follows:

#### Proposal 2.3.2.2

For addreessing how to count and order the HARQ-ACK bits for NACK-only for Alt4: down-select the supported cases and options from the following:

Case 1: when the HARQ-ACK-to-PUCCH mapping table for Alt4 (from previous agreement) is applied to a single G-RNTI,

  - Opt1-1: based on C-DAI included in DL DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits.

  - Opt1-2: based on the number of PUCCH resources in the PUCCH resource set configured for NACK-only to count the number of HARQ-ACK bits: the number of HARQ-ACK bits=log2(the number of PUCCH resource+1), i.e., 15 PUCCH resources correspond to 4 TBs, and based on the C-DAI included in DL DCI for ordering the HARQ-ACK bits.

Case 2: for the case of all UEs configured with the same set of G-RNTIs

- Opt2-1: support Alt4 for this case

  - Opt2-1-1: based on the number of PUCCH resources in the PUCCH resource set configured for NACK-only to count the number of HARQ-ACK bits, i.e., the number of HARQ-ACK bits=log2(the number of PUCCH resource+1), 15 PUCCH resources correspond to 4 TBs, and based on Opt2-1-2 or Opt2-1-3 for ordering the HARQ-ACK bits

  - Opt2-1-2: based on the sum of C-DAI included in the last scheduling DCI from each G-RNTI for counting the total number of HARQ-ACK bits. The ordering of HARQ-ACK bits is per C-DAI from each G-RNTI and in the ascending order of G-RNTI values.

  - Opt2-1-3: based on C-DAI* included in the last scheduling DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits. The C-DAI* is accumulating the scheduled TBs across different G-RNTIs

- Opt2-2: does not support Alt4 for this case.

Case 3: for the case of different UEs configured with different G-RNTIs

- Opt3-1: support Alt4 for this case

  - Opt3-1-1: based on the number of PUCCH resources in the PUCCH resource set configured for NACK-only to count the number of HARQ-ACK bits, i.e., the number of HARQ-ACK bits=log2(the number of PUCCH resource+1), 15 PUCCH resources correspond to 4 TBs, and based on Opt3-1-2 for ordering the HARQ-ACK bits

  - Opt3-1-2: based on C-DAI* included in the last scheduling DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits. The C-DAI* is accumulating the scheduled TBs across different G-RNTIs

- Opt3-2: does not support Alt4 for this case.

Please indicate which case and which option you support. Note case 1 has been agreed but need to solve the FFS how to count and order the TBs.

#### Company views

| Company | Comments |
| --- | --- |
| Moderator | Please indicate which case and which option you support. Note case 1 has been agreed but need to solve the FFS how to count and order the TBs. |
| Apple | Our preference: Case 1: Opt1-1 Case 2: Opt2-1-2. Second preference is Opt2-2 as this use case is not typical. Case 3: Opt3-1-2 with updates. For case3, “The C-DAI* is accumulating the scheduled TBs across different G-RNTIs” seems not work. Even different UEs configured with different G-RNTIs, for a G-RNTI, it is still allocated for a group of users, so C-DAI should be counting per G-RNTI. It could be better to limit the G-RNTI number to 2. each G-RNTI associate with two TBs, such as G-RNTI 1 associates TB1(first HARQ bit), TB2 (second HARQ bit); G-RNTI 2 associates TB3(third HARQ bit), TB4 (forth HARQ bit). the hard splitting of the TBs is to avoid the issue of missing the last TB for a G-RNTI, due to no UL DAI to check the missed last DCI. |
| Nokia, NSB. | For Case 1, we prefer Opt1-1. There is no need to address the last DCI missing problem in this release further. Enhancements could be made in next releases. For Case 2, we prefer Opt2-1-2. It is perfectly aligned with ACK/NACK feedback operation with no additional problems. For case 3, Opt3-1-2 cannot work, as not all the UEs will receive the same G-RNTI, accumulating C-DAI among G-RNTIs is not feasible. In addition, we do not believe that there is a problem when using the mapping table discussed earlier. In our view, a similar solution to Case 2’s Opt2-1-2 shall be introduced. |
| Lenovo | Case 1: generally support opt1-1. For opt1-2, I don’t know how it works in case there are 31 PUCCH resources configured in the set. Case 2: We support Opt2-1-3 which can solve the last DCI missing for each G-RNTI. Case 3: We support Opt3-1-2 which can solve the last DCI missing for each G-RNTI. |
| Samsung | Case 1: Opt. 1-1 Reason: It is simple and essentially the same approach as implemented in LTE for PUCCH with channel selection Case 2: Opt. 2-1-3 Reason: It is same as for “Case 1, Opt. 1-1”. The set of G-RNTIs can be viewed as a single “set G-RNTI” and C-DAI is for the “set G-RNTI”. Basically, same specifications/procedure as for “Case 1, Opt. 1-1”. Case 3: Opt. 3-1-2 Reason: Case 3 is basically same as Case 2. The “all UEs” in Case 2 are “all UEs with same G-RNTIs” in Case 3. We think that Case 3 should be for “Group of UEs with multiple G-RNTIs that are same for the group UEs”. In our opinion, there is generally no meaningful solution for “different UEs with different G-RNTIs”. |
| NTT DOCOMO | Case 1: We prefer Opt1-1. Case 2: We prefer Opt2-1-2. It is aligned with ACK/NACK based feedback. Case 3: In Opt3-1-2, when the DAI of the first DCI received by the UE with a G-RNTI is 2, the UE cannot distinguish whether the DCI with DAI=1 was a DCI of another G-RNTI or a missed DCI with the same G-RNTI. If it was a DCI of another G-RNTI, the UE does not need to send NACK-only feedback, but should send NACK-only feedback if it was a missed DCI. |
| ZTE | Case 1: Opt 1-1 Case 2: Opt2-1-2 Case 3: Opt3-1-2 |
| CATT | Case 1：we prefer Opt1-2. The motivation of Option1-2 is to solve the ‘last DCI missing issue’ for NACK-only feedback for Alt 4, especially for the case that a set of UEs within the MBS group miss the last DCI and another set of UE within the same group decode the TB scheduled by this DCI successfully. The legacy Type-2 CB has the ‘last DCI missing issue’ as well, so the ‘UL DAI’ mechanism was intruded to solve this issue when the Type-2 CB is multiplexed in the PUSCH. For the same reason, it is also a feasible solution to introduce the number of NACK PUCCH resources to calculate the number based on TB in order to solve the last DCI missing issue for NACK-only feedback for Alt.4. Case 2：we prefer Opt2-1. The ‘last DCI missing issue’ can be solved by this way. Case 3：we prefer Opt3-1. The ‘last DCI missing issue’ can be solved by this way. |
| Qualcomm | In general, we prefer to use DAI as legacy way instead of introducing new scheme based on counting PUCCH resources, since the last DCI missing issue is not new even for legacy unicast feedback. Case 1: Opt 1-1 Case 2: Opt2-1-2 Case 3: Opt3-2 We are not sure how to make it work since UE only monitors the configured G-RNTI. For example, the G-RNTI1 is for UE1 and G-RNTI2 is for UE2. If the DCI2 with G-RNTI2 is the last DCI, the UE1 will not detect DCI2 and cannot know the *c-DAI in DCI2. Eventually, UE1 and UE2 cannot map to the same PUCCH due to different understanding of *c-DAI. |

#### Summary

To respond the question about C-DAI* as some companies questioned how it works. Ericsson is the only proponent. From my reading Ericsson’s paper, this is my understanding how it works (companies are also encouraged to check Ericsson’s paper): - The C-DAI* is additionally configured in the DL-DCI scheduling GC-PDSCH. The C-DAI* is only configured for G-RNTIs configured with NACK-only based feedback with Alt4. - From NW perspective, C-DAI* is accumulating across all G-RNTIs. - From UE perspective, the C-DAI* is used by UE for counting the number of HARQ-ACK bits and for the ordering, the UE reports ACK (instead of NACK) to react to the DL-DCL scheduling G-RNTIs not configured to the UEs. - The legacy C-DAI is used by the UE for counting the number of HARQ-ACK bits and for ordering for a G-RNTI. Based on the comments, companies’ views are summarized (Ericsson’s view is added though no input for Round-2): Case 1: when the HARQ-ACK-to-PUCCH mapping table for Alt4 (from previous agreement) is applied to a single G-RNTI, - Opt1-1: based on C-DAI included in DL DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits. (Apple, Nokia, NSB, Lenovo, Samsung, DOCOMO, ZTE, Qualcomm) - Opt1-2: based on the number of PUCCH resources in the PUCCH resource set configured for NACK-only to count the number of HARQ-ACK bits: the number of HARQ-ACK bits=log2(the number of PUCCH resource+1), i.e., 15 PUCCH resources correspond to 4 TBs, and based on the C-DAI included in DL DCI for ordering the HARQ-ACK bits. (CATT) Case 2: for the case of all UEs configured with the same set of G-RNTIs - Opt2-1: support Alt4 for this case - Opt2-1-1: based on the number of PUCCH resources in the PUCCH resource set configured for NACK-only to count the number of HARQ-ACK bits, i.e., the number of HARQ-ACK bits=log2(the number of PUCCH resource+1), 15 PUCCH resources correspond to 4 TBs, and based on Opt2-1-2 or Opt2-1-3 for ordering the HARQ-ACK bits (CATT) - Opt2-1-2: based on the sum of C-DAI included in the last scheduling DCI from each G-RNTI for counting the total number of HARQ-ACK bits. The ordering of HARQ-ACK bits is per C-DAI from each G-RNTI and in the ascending order of G-RNTI values. (Apple, Nokia, NSB, DOCOMO, ZTE, Qualcomm) - Opt2-1-3: based on C-DAI* included in the last scheduling DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits. The C-DAI* is accumulating the scheduled TBs across different G-RNTIs (Lenovo, Ericsson, Samsung) - Opt2-2: does not support Alt4 for this case. (Apple) Case 3: for the case of different UEs configured with different G-RNTIs - Opt3-1: support Alt4 for this case - Opt3-1-1: based on the number of PUCCH resources in the PUCCH resource set configured for NACK-only to count the number of HARQ-ACK bits, i.e., the number of HARQ-ACK bits=log2(the number of PUCCH resource+1), 15 PUCCH resources correspond to 4 TBs, and based on Opt3-1-2 or Opt2-1-3 for ordering the HARQ-ACK bits (CATT) - Opt3-1-2(New): based on the sum of C-DAI included in the last scheduling DCI from each G-RNTI for counting the total number of HARQ-ACK bits. The ordering of HARQ-ACK bits is per C-DAI from each G-RNTI and in the ascending order of G-RNTI values. (Apple with 2 G-RNTIs, Nokia, NSB, DOCOMO, ZTE) - Opt3-1-3: based on C-DAI* included in the last scheduling DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits. The C-DAI* is accumulating the scheduled TBs across different G-RNTIs. (Lenovo, , Ericsson) - Opt3-2: does not support Alt4 for this case. (Samsung, Qualcomm) Please companies check whether companies’ views are correctly reflected: - To Samsung, case 3 is exactly for different UEs with different G-RNTIs. “Group of UEs with multiple G-RNTIs that are same for the group UEs” is case 2

#### (ACTIVE)Round-3

Not sufficient input in Round-2 for FL to judge and suggest a FL proposal though this issue has been discussed a few meetings.

One approach is FFS on all options and all cases since the proposal of C-DAI* is still unclear to some companies how it works. The other approach is agreeing on the cases and options based on the current majority views.

Two alternative proposals (Alt1 and Alt2) are suggested as follows. Companies are encouraged to provide your view of supporting which and also further check others’ views for continuing the discussion:

#### Proposal 2.3.2.3-1 (Alt1)

For addressing how to count and order the HARQ-ACK bits for NACK-only for Alt4: down-select the supported cases and options from the following:

- Case 1: when the HARQ-ACK-to-PUCCH mapping table for Alt4 (from previous agreement) is applied to a single G-RNTI,

    - Opt1-1: based on C-DAI included in DL DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits. (Apple, Nokia, NSB, Lenovo, Samsung, DOCOMO, ZTE, Qualcomm)

    - Opt1-2: based on the number of PUCCH resources in the PUCCH resource set configured for NACK-only to count the number of HARQ-ACK bits: the number of HARQ-ACK bits=log2(the number of PUCCH resource+1), i.e., 15 PUCCH resources correspond to 4 TBs, and based on the C-DAI included in DL DCI for ordering the HARQ-ACK bits. (CATT)

- Case 2: for the case of all UEs configured with the same set of G-RNTIs

- Opt2-1: support Alt4 for this case

  - Opt2-1-1: based on the number of PUCCH resources in the PUCCH resource set configured for NACK-only to count the number of HARQ-ACK bits, i.e., the number of HARQ-ACK bits=log2(the number of PUCCH resource+1), 15 PUCCH resources correspond to 4 TBs, and based on Opt2-1-2 or Opt2-1-3 for ordering the HARQ-ACK bits (CATT)

  - Opt2-1-2: based on the sum of C-DAI included in the last scheduling DCI from each G-RNTI for counting the total number of HARQ-ACK bits. The ordering of HARQ-ACK bits is per C-DAI from each G-RNTI and in the ascending order of G-RNTI values. (Apple, Nokia, NSB, DOCOMO, ZTE, Qualcomm)

  - Opt2-1-3: based on C-DAI* included in the last scheduling DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits. The C-DAI* is accumulating the scheduled TBs across different G-RNTIs (Lenovo, Ericsson, Samsung)

- Opt2-2: does not support Alt4 for this case. (Apple)

- Case 3: for the case of different UEs configured with different G-RNTIs

- Opt3-1: support Alt4 for this case

  - Opt3-1-1: based on the number of PUCCH resources in the PUCCH resource set configured for NACK-only to count the number of HARQ-ACK bits, i.e., the number of HARQ-ACK bits=log2(the number of PUCCH resource+1), 15 PUCCH resources correspond to 4 TBs, and based on Opt3-1-2 or Opt2-1-3 for ordering the HARQ-ACK bits (CATT)

  - Opt3-1-2(New): based on the sum of C-DAI included in the last scheduling DCI from each G-RNTI for counting the total number of HARQ-ACK bits. The ordering of HARQ-ACK bits is per C-DAI from each G-RNTI and in the ascending order of G-RNTI values. (Apple with 2 G-RNTIs, Nokia, NSB, DOCOMO, ZTE)

  - Opt3-1-3: based on C-DAI* included in the last scheduling DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits. The C-DAI* is accumulating the scheduled TBs across different G-RNTIs. (Lenovo, , Ericsson)

- Opt3-2: does not support Alt4 for this case. (Samsung, Qualcomm)

#### Proposal 2.3.2.3-2 (Alt2)

For addressing how to count and order the HARQ-ACK bits for NACK-only for Alt4, support the following cases and options:

- Case 1: when the HARQ-ACK-to-PUCCH mapping table for Alt4 (from previous agreement) is applied to a single G-RNTI,

  - Opt1-1: based on C-DAI included in DL DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits.

- Case 2: for the case of all UEs configured with the same set of G-RNTIs

  - Opt2-1-2: based on the sum of C-DAI included in the last scheduling DCI from each G-RNTI for counting the total number of HARQ-ACK bits. The ordering of HARQ-ACK bits is per C-DAI from each G-RNTI and in the ascending order of G-RNTI values.

- Case 3: for the case of different UEs configured with different G-RNTIs

  - Opt3-2: does not support Alt4 for this case

#### Company views

| Company | Comments |
| --- | --- |
| Moderator | Which alternative proposal do you support, Alt1 or Alt2? Note：Opt3-2 is suggested for Proposal 2.3.2.3-2 (Alt2) because some companies commented Opt3-1-2(New) does not work. |
| Samsung | We support Alt. 1. In particular, we continue to support: Opt. 1-1 (same in Alt. 2) Opt2-1-3 because it clearly provides better protection to errors than Opt2-1-2 since, if all UEs have same G-RNTIs then, for DAI counting purposes there is practically a single G-RNTI and Case 2 is same as Case 1. Case 3 can be supported with Opt3-1-3. There is no additional specification support required and there is no apparent reason to preclude Case 3 just because Opt3-1-2 may be problematic for it. |
| Lenovo | Alt 1 is supported. Within Alt 1, we support option 1-1, Opt2-1-3 and Opt3-1-3. We see the problems of last DCI missing if based on the sum of C-DAI included in the last scheduling DCI from each G-RNTI. As long as a last DCI of any G-RNTI is missed, UE and gNB have different understanding on the number of HARQ-ACK information bits. This error case should be avoided. |
| vivo | For case 1, we are fine with Opt1-1. It is not clear to us how the Opt1-2 work well. The number of PUCCH resources in the PUCCH resource set configured for NACK-only can be used to determined the number of HARQ-ACK bits, but we are talking about the order. It does not solve the issue. In addition, one question for confirm, is C-DAI present when a UE is configured with only one G-RNTI with NACK-only feedback? For case 2, we think itself is too restrictive and can be corner case. Can be treated same as case 3. i.e., Opt2-2. For case 3, we support Opt3-2. We don’t think the other options can work well. |
| NTT DOCOMO | We support Alt 1. For Case 1, we prefer Opt1-1. For Case 3, Opt3-1-3 is not the best, but seems to be a better method considering the complexity of PUCCH resource configuration and the wider applicability of Alt 4. |
| LG Electronics | We support Alt 1 for which we prefer Opt1-1, Opt2-2, and Opt3-2. Case 2 seems a rare case. If a same set of services is configured for multiple UEs, we think that gNB can configure a single G-RNTI for the set of services. Hence, Case 2 can be avoided. |
| CATT | For Opt1-2 of case 1, the number of HARQ bits is determined by the number of configured PUCCH resources for NACK-only, and the order of HARQ-ACK bits is generated in the ascending order of C-DAI of DCI. This method can solve last mission issue. But if majority of companies have consensus on the last DCI missing issue doesn’t require to solve, we can compromise to Opt1-2. For the reason that a UE can’t know of configured G-RNTIs of other UEs in the same MBS group, case 2 and case 3 are the same from the perspective of the UE. Thus, the same mechanism should be defined for case 2 and case3. We prefer Opt2-1-1 and Opt3-1-1, and can compromise to Opt2-1-2 and Opt3-1-2. It unclear for us how does the Opt3-1-3 of case 3 work? If different UEs are configured with different G-RNTIs, how to ensure that the value of C-DAI* in the scheduled DCI for different UEs configured with different G-RNTIs is the same? |
| ZTE | We support Alt 1. In addition, our position is wrong in the summary. For Case 3, we support Opt3-1-3. |

### (ACTIVE)PDSCH processing timeline for NACK-only

#### (CLOSED)Reference PUCCH for defining timeline

In R15/R16, the UE processing time Tproc1 is defined as the minimum required gap between the end of the PDSCH reception and the earliest possible start of the PUCCH carrying the corresponding HARQ-ACK.

In addition to the PDSCH processing time, another timeline related to PUCCH resource is N3 used in HARQ-ACK codebook construction. If a UE determines a first PUCCH resource for HARQ-ACK in a slot, and later receives a second DCI indicating another HARQ-ACK also in the slot, the time gap between the second DCI and the first PUCCH resource is required to be larger than N3. The details about N3 can be found in the follow spec.

If a UE determines a first resource for a PUCCH transmission with HARQ-ACK information corresponding only to a PDSCH reception without a corresponding PDCCH or detects a first DCI format indicating a first resource for a PUCCH transmission with corresponding HARQ-ACK information in a slot and also detects at a later time a second DCI format indicating a second resource for a PUCCH transmission with corresponding HARQ-ACK information in the slot, the UE does not expect to multiplex HARQ-ACK information corresponding to the second DCI format in a PUCCH resource in the slot if the PDCCH reception that includes the second DCI format is not earlier than from the beginning of a first symbol of the first resource for PUCCH transmission in the slot where, κ and Tc are defined in clause 4.1 of [4, TS 38.211] and corresponds to the smallest SCS configuration among the SCS configurations of the PDCCHs providing the DCI formats and the SCS configuration of the PUCCH. If processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell with the second DCI format and for all serving cells with corresponding HARQ-ACK information multiplexed in the PUCCH transmission in the slot, for , for , for ; otherwise , for , for , for , for .

For a UE configured in NACK-only mode, the PUCCH resource depends on the decoding of PDSCH, which means gNB has no knowledge about which PUCCH is used by UE to report NACK-only HARQ-ACK feedback and blind decoding through the whole PUCCH resource set is required. Thus, it is hard for gNB to guarantee the time gap between the end of the PDSCH reception and the earliest possible start of the chosen PUCCH is always larger than TProc1. Therefore, a reference PUCCH is needed for defining the timeline.

##### Round-1

Regarding how to define the reference PUCCH for defining PDSCH processing timeline in terms of  TProc1 and N3, one possible solution is to use the starting symbol of the NACK-only PUCCH resource set as the reference point for NACK-only feedback, where the starting symbol of the PUCCH resource set means the earliest starting symbols of all the PUCCH resources in the set. With the earliest starting symbol as the reference PUCCH, gNB can always guarantee the PDSCH processing timeline is satisfied.

#### Proposal 2.3.3.1.1

The PUCCH with the earliest symbol in the resource set configured for NACK-only feedback is used as the PUCCH for determining the PDSCH processing timeline in terms of  TProc1 and the timeline of UE reporting HARQ-ACK in terms of N3.

#### Company views

| Company | Comments |
| --- | --- |
| Nokia, NSB | Support |
| Qualcomm | Not sure it is necessary to define a new reference point. Our understanding of PUCCH resources for more than one NACK-only bits is to use different PUCCH sequences but same time/frequency resources. |
| Samsung | The consideration motivating the proposal is valid but the proposal requires further discussion. The overall issue is similar to that for proposal 2.3.4.1. For example, it is possible to address this proposal, and the next one, and proposal 2.3.4.1, and any related timeline issue with a single solution – i.e. all PUCCH resources have same symbols. Also, it would be preferable to discuss timing issues in general. For example, currently it is not allowed (error case) for a UE to receive DCI scheduling PUSCH in a slot after receiving a DCI triggering multicast PUCCH in the slot – but that situation cannot be avoided both in practice and by default in some scenarios such as when a UE multiplexes CSI in the PUSCH. |
| vivo | We are not sure it is necessary to define a new reference point, either. Our understanding is that the PUCCH resources actually used for more than one NACK-only bits should meet the timeline. From gNB perspective, for safety, earliest symbol from PUCCH resources for more than one bit NACK-only feedback can be used. However, from spec perspective, no define a new reference point |
| ZTE | Not sure whether we need to define this in the spec. Even if this is not specified in the spec, network may have to do something to guarantee this timeline. |
| NTT DOCOMO | OK |
| FL response | As Samsung commented, we may need to look at proposal 2.3.3.1.1, 2.3.3.1.2, and 2.3.4.1 together because the consideration motivating the proposal seems common, i.e., the PUCCH resource can only be determined after all TBs are decoded, which is the significant difference from the legacy. For example, the processing time currently defined is not applicable to NACK-only as illustrated in the figure, so when defining the processing time, a reference PUCCH needs to fixed, and the PUCCH with the earliest symbol is more sensible to be the reference. When NACK-only is overlapped with other UCI/PUSCH, NACK-only is converted to ACK/NACK and multiplexed, then how to adjust whether there is overlapping because the PUCCH resource depends on decoding results but this multiplexing procedure should not depend on decoding results to avoid NW blind decoding. The underlying reason this issue is to be considered is NW cannot guarantee the up to 32 PUCCH resources for NACK-only always having the same time and frequency location, especially because it is semi-static configured by RRC. |
| Ericsson | Support |

#### (CLOSED)PDSCH processing timeline

One company discussed the PDSCH processing timeline for NACK-only based feedback and proposed to extend Tproc1:

| R1-2203070 Huawei | Proposal 10：Extend Tproc1 by adding d3 in NACK-only reporting mode and set the value of d3 as, SPDSCH - SPDCCH + N1 where SPDSCH is the ending symbol of PDSCH, SPDCCH is the ending symbol of PDCCH, and N1 is the same as the N1 in Tproc1. Proposal 11: Define the reference point as the starting symbol of PUCCH resource set instead of the chosen PUCCH for NACK-only feedback, and use the new reference point for PDSCH processing time, where the starting symbol of the PUCCH resource set means the earliest starting symbols of all the PUCCH resources in the set. - Proposal 12: Define the reference point as the starting symbol of PUCCH resource set instead of the chosen PUCCH for NACK-only feedback, and use the new reference point for N3, where the starting symbol of the PUCCH resource set means the earliest starting symbols of all the PUCCH resources in the set. |
| --- | --- |

Background:

In R15/R16, the UE processing time Tproc1 is defined as the minimum required gap between the end of the PDSCH reception and the earliest possible start of the PUCCH carrying the corresponding HARQ-ACK. UE is required to finish PDSCH reception and PUCCH preparation during the Tproc1 time.

If the first uplink symbol of the PUCCH which carries the HARQ-ACK information, as defined by the assigned HARQ-ACK timing K1 and Koffset, if configured, and the PUCCH resource to be used and including the effect of the timing advance, starts no earlier than at symbol L1, where L1 is defined as the next uplink symbol with its CP starting after after the end of the last symbol of the PDSCH carrying the TB being acknowledged, then the UE shall provide a valid HARQ-ACK message. - N1 is based on µ of table 5.3-1 and table 5.3-2 for UE processing capability 1 and 2 respectively, where µ corresponds to the one of (µPDCCH, µPDSCH, µUL) resulting with the largest Tproc,1, where the µPDCCH corresponds to the subcarrier spacing of the PDCCH scheduling the PDSCH, the µPDSCH corresponds to the subcarrier spacing of the scheduled PDSCH, and µUL corresponds to the subcarrier spacing of the uplink channel with which the HARQ-ACK is assumed to be transmitted regardless of whether or not the PDSCH reception provides a transport block for a HARQ process with disabled HARQ-ACK information as indicated by HARQ-feedbackEnabling-disablingperHARQprocess, if provided, and κ is defined in clause 4.1 of [4, TS 38.211].

For a R15/R16 UE, PUCCH resource is determined by PDSCH slot, K1 value and PRI. All the information can be determined based on the DCI scheduling the corresponding PDSCH, which means UE can do the preparation of PUCCH resource right after the decoding of DCI.

However, for a UE configured with Alt4 for more than one TB with NACK-only reported in the same slot, the PUCCH resource depends on the decoding of PDSCHs. Thus, UE can only start the preparation of PUCCH resource after the decoding of PDSCHs. Compared to the UE in ACK/NACK reporting mode, UE in NACK-only mode requires more time to do the PDSCH reception and PUCCH preparation. Therefore, Tproc1 as specified needs to be extended to include the case for NACK-only based feedback, e.g., by adding d3 in the formula.

##### Round-1

Regarding the value of d3, two options are discussed:

- Option 1: The value of d3 equals to the time gap between PDCCH ending symbol and PDSCH ending symbol plus N1.

- Option 2: The value of d3 depends on the UE capability and can be reported by UE.

With option 1, given the PUCCH determination is postponed as shown in the figure, the value of d3 can be set to the postponed time. According to the figure, the PUCCH determination is postponed as

SPDSCH - SPDCCH + TPDSCH_decod - TPDCCH_decod

where SPDSCH is the ending symbol of PDSCH, SPDCCH is the ending symbol of PDCCH, TPDSCH_decod is the decoding time for PDSCH, and TPDCCH_decod is the decoding time for PDCCH. As the decoding time for PDCCH is always shorter than the one for PDSCH, TPDSCH_decod - TPDCCH_decod can be set to N1 for simplicity. Thus the value of d3 equals to

SPDSCH - SPDCCH + N1

where N1 is the same as the N1 in Tproc1.

With option 2, UE can choose one value from the pre-configured candidate value set and report it as d3. However, if UE has different value for d3, from network perspective, the scheduling for the PUCCH transmission will turn out considering the worst case with the largest value of d3. Therefore, it is eventually equivalent to taking a reasonable value as a fixed value for d3 as in option 1.

#### Proposal 2.3.3.2.1

When UE is configured with NACK-only based feedback, extend UE PDSCH processing procedure timeline Tproc1 by adding d3 and d3=SPDSCH - SPDCCH + N1, where SPDSCH is the ending symbol of PDSCH, SPDCCH is the ending symbol of PDCCH, and N1 is the same as the N1 in Tproc1.

#### Company views

| Company | Comments |
| --- | --- |
| Nokia, NSB | We support the proposal |
| Qualcomm | Thanks for raising this issue. Some questions for clarification: Does this ‘SPDSCH - SPDCCH’ consider PDSCH repetitions? It may extensively increase the required processing timeline if large number of repetitions is configured. In case of NACK-only, how likely this issue will happen. In worst cases, even if the processing time is not sufficient for the last PDSCH, UE can just send NACK for the last PDSCH, rather than discard all the previous PDSCHs. |
| Samsung | Similar comment as for 2.3.3.1.1. |
| vivo | Similar comment as for 2.3.3.1.1. |
| ZTE | We are not sure about the necessity of d3 here. For legacy ACK/NACK based feedback, UE also needs to first decode PDSCH and then generate feedback, choose PUCCH sequence, etc. This is similar to NACK-only feedback. Once UE decodes PDSCH, UE can determine the PUCCH resource for NACK-only. |
| MediaTek | Thanks for pointing the issue. We think it is a valid issue. As FL indicated, for the more than one TB with NACK only case, the UE needs to prepare the PUCCH processing after decoding the PDSCH instead of after DCI parsing used by legacy unicast. So, it needs to extend the Tproc1 processing time. Regarding the specific value, we have the similar question with QC, and it needs further clarification. |
| Ericsson | Support |

#### (ACTIVE)Processing timeline for NACK-only

##### (CLOSED)Round-1

This is the first meeting discussing this essential issue, so maybe too early to go far. Let’s take steps back.

Firstly, we should get everybody on the same page that the following PDSCH processing is defined for ACK/NACK based and it is assumed UE implementation should be based on this timeline definition.

From TS38.214 ## 5.3 UE PDSCH processing procedure time If the first uplink symbol of the PUCCH which carries the HARQ-ACK information, as defined by the assigned HARQ-ACK timing K1 and Koffset, if configured, and the PUCCH resource to be used and including the effect of the timing advance, starts no earlier than at symbol L1, where L1 is defined as the next uplink symbol with its CP starting after after the end of the last symbol of the PDSCH carrying the TB being acknowledged, then the UE shall provide a valid HARQ-ACK message. - N1 is based on µ of table 5.3-1 and table 5.3-2 for UE processing capability 1 and 2 respectively, where µ corresponds to the one of (µPDCCH, µPDSCH, µUL) resulting with the largest Tproc,1, where the µPDCCH corresponds to the subcarrier spacing of the PDCCH scheduling the PDSCH, the µPDSCH corresponds to the subcarrier spacing of the scheduled PDSCH, and µUL corresponds to the subcarrier spacing of the uplink channel with which the HARQ-ACK is assumed to be transmitted regardless of whether or not the PDSCH reception provides a transport block for a HARQ process with disabled HARQ-ACK information as indicated by HARQ-feedbackEnabling-disablingperHARQprocess, if provided, and κ is defined in clause 4.1 of [4, TS 38.211].

Secondly, it should be also the common understanding that PUCCH resource for NACK-only can only be determined after PDSCH decoding, which is different from for legacy as shown in the following figure.

Observation 1: The current PDSCH processing procedure timeline defined in clause 5.3 TS38.214 is for ACK/NACK based feedback.

Observation 2: For NACK-only based feedback, PUCCH  resource for NACK-only can only be determined after PDSCH decoding. The current TProc1 is not sufficient for NACK-only based feedback.

#### Proposal 2.3.3.3.1

Extending the PDSCH processing procedure timeline for NACK-only based feedback as:

TProc1_NACK=TProc1+Toffset

- Note: TProc1 is the same as defined in clause 5.3 TS 38.214 where N1 is based on the UE processing capability 1.

- FFS: value for Toffset.

- FFS: which PUCCH resource from the resource set configured for NACK-only is used for the reference PUCCH for defining TProc1.

#### Company views

| Company | Comments |
| --- | --- |
| Nokia, NSB. | Ok. |
| Samsung | Do not support. As previously mentioned, a joined discussion with 2.3.4.2 is needed. Please see comment for 2.3.4.2. |
| NTT DOCOMO | OK |
| ZTE | We don’t see the need to prolong the processing timeline for NACK-only. For NACK-only feedback, the UE transmits PUCCH when it does not decode the transport block correctly. It is the same as that the UE transmits PUCCH to carry NACK for ACK/NACK feedback when it does not decode the transport block correctly. Extending the PDSCH processing timeline is not needed. |
| CATT | Don’t support. It is not necessary to introduce a new parameter of T_offset. The simplest method is to define the first uplink symbol of the PUCCH resource configured for NACK-only HARQ-ACK feedback as the referent symbol for the PDSCH processing procedure time. |
| HiSilicon | MediaTek and Qualcomm asked question regarding the specific value in Round-1. The extended time in Proposal 2.3.3.2.1 does not need to count PDSCH repetition because the PUCCH resources is determined after decoding all PDSCH and TProc1 is also defined from after the last symbol of PDSCH. Even though there is no PDSCH repetition, the current processing timeline needs to be extended because PUCCH can only be determined after decoding PDSCH. This is not an issue NW vendor says it is not necessary, then it is not necessary for UE implementation. At least no other UE vendor says it is not necessary so far. #### Regarding Samsung’s comment, even though all the PUCCHs for NACK-only configured with the same starting symbol and the same number of symbols or even though it is a single TB for NACK-only, the current PDSCH processing timeline is not sufficient. The difference is the time for determining the PUCCH resource (including whether send it or not) can only be possible after decoding the PDSCH. For legacy ACK/NACK based, PUCCH resource can be determined after decoding PDCCH. |

#### Summary

Regarding whether extending PDSCH processing timeline, the following is observed from the last two rounds discussion: NW/operator: - Nokia, Ericsson, DOCOMO, Huawei supports - Samsung suggests discussing together with proposal 2.3.4.2 - ZTE, CATT don’t support. UE: - MediaTek views issue is valid - Qualcomm asked question about the value for the case of PDSCH repetition. - HiSilicon supports.

##### (ACTIVE)Round-2

More companies may need to further check since this is the first meeting to discuss this issue. FL suggests further discussing this issue in next meeting with some guidance for further study as the following proposal:

#### Proposal 2.3.3.3.2

Regarding the PDSCH processing procedure timeline for NACK-only based feedback, further discuss on whether/how to extending the PDSCH processing procedure timeline.

- Note1: The current PDSCH processing procedure timeline defined in clause 5.3 TS38.214 is for ACK/NACK based feedback.

- Note2: For NACK-only based feedback, PUCCH  resource for NACK-only can only be determined after decoding PDSCH.

#### Company views

| Company | Comments |
| --- | --- |
| Samsung | Timeline issues and issues related to multiplexing/resolving overlapping can be discussed once a decision is made whether to have PUCCH resources with different symbols. If the resources have same symbols, no other discussion is needed and the specifications are complete from every aspect. Even though a UE may not know the exact resource prior to decoding, that does not matter for timelines or for multiplexing/resolving overlapping. |
| vivo | Agree with Samsung. |
| MediaTek | Support the proposal. The issue is valid, and we agree with FL’s proposal for further study as it is proposed firstly and needs more time to check the value. |
| ZTE | Agree with Samsung. |

### (ACTIVE)Multiplexing NACK-only with others

If PUCCH transmission for NACK-only based feedback overlaps in time with other PUCCH or PUSCH, the UE will multiplex the NACK-only based feedback with other UCI in PUCCHs or PUSCHs by transforming NACK-only into the ACK/NACK information, as described in section 18 of TS 38.213.

When more than one NACK-only feedback is transmitted in the same PUCCH transmission and Alt 4 is applied, the UE will select one PUCCH resource from the configured PUCCH resources for NACK-only feedback according to the decoding results of each PDSCH.

However, gNB doesn’t know the decoding result of each PDSCH before receiving HARQ-ACK report. Then the issue is how to determine whether a PUCCH for NACK-only overlaps in time with other PUCCHs or PUSCH.

Some companies submitted proposals as follows:

| R1-2203070 Huawei | Observation: When UE multiplexes PUCCH transmission for Atl4 with other UCI/PUSCH for the same priorities, how to determine the resource overlapping needs to be defined. Proposal 9: Define the nominal NACK-only PUCCH as a PUCCH consisting of all the symbols from PUCCHs of the NACK-only PUCCH resource set and the nominal NACK-only PUCCH is used for determining whether PUCCH for NACK-only overlaps with other UCI/PUSCH for multiplexing for the same priority. |
| --- | --- |
| R1-2203427 CATT | Proposal 4: When more than one NACK-only feedback is transmitted in the same PUCCH transmission and Alt 4 is applied, use all PUCCH resources configured/indicated for NACK-only to determine if a PUCCH transmission for NACK-only overlaps in time with other PUCCHs or PUSCH, i.e. overlap occurs if any one of PUCCH resource configured/indicated for NACK-only transmission overlaps in time with other PUCCHs/PUSCHs |
| R1-2203838 Langbo | It is still open how to resolve overlapping among PUCCH resources in the set of PUCCH resources and other PUCCHs or PUSCHs. If PUCCH resources in the set of PUCCH resources are considered independently, i.e., only the overlapping among the selected PUCCH resource and other PUCCHs or PUSCHs is resolved, the multiplexing result depends on the value of the HARQ-ACK information bits. This may lead to chick-and-egg situation at gNB side as gNB may not be able to know the multiplexing result prior to successfully decoding the HARQ-ACK information bits. Note that blind decoding by gNB may not work effectively especially when the HARQ-ACK bits are multiplexed on a non-sequence based PUCCH or PUSCH. A simple way to address this problem is to consider the PUCCH resources in the set of PUCCH resources as a whole when multiplexing with other PUCCHs or PUSCHs. |
| R1-2203874 Samsung | Proposal 4: For resolving collisions with other PUCCHs or PUSCHs of same priority for a UE in case of “NACK-only” reporting mode, specify one of: - all PUCCH resources have same symbols - the UE assumes the PUCCH resource for all-NACK values for any HARQ-ACK values |

#### (CLOSED)Round-1

As analyzed in [1], to solve this issue, the nominal NACK-only PUCCH can be used for the determination based on the pre-configured NACK-only PUCCH resource set. Some options can be considered as listed as follows:

- Option 1: The nominal NACK-only PUCCH consists of all the symbols from PUCCHs of the NACK-only PUCCH resource set, as the yellow block in the following figure (the NACK-only PUCCH resource set includes PUCCH1~PUCCH3).

- Option 2: The nominal NACK-only PUCCH starts from the earliest starting symbol of PUCCHs in the NACK-only PUCCH resource set, and ends at the latest ending symbol of PUCCHs in the set, as the green block in the figure.

- Option 3: The nominal NACK-only PUCCH consists of all the symbols of the PUCCH slot, as the blue block in the figure.

Among all three options, option 1 is suggested as it only consists of the symbols belonging to the PUCCHs of the NACK-only PUCCH resource set. Option 2 and option 3 may consist of symbols not belonging to the PUCCHs in the set, and multiplexing may happen more frequently especially for option 3.

The other two companies in [6] [11] recommends to use all PUCCH resources indicated for NACK-only to determine overlapping or consider the PUCCH resources in the set of PUCCH resources as a whole when multiplexing with other PUCCHs or PUSCHs, i.e. overlap occurs if any one of PUCCH resource configured/indicated for NACK-only transmission overlaps in time with other PUCCHs/PUSCHs, which is understood option 1.

One company in [12] propose to specify one of two schemes: a) all PUCCH resources have same symbols; b) the UE assumes the PUCCH resource for all-NACK values for any HARQ-ACK values. Apparently, a) is too restrictive for network scheduling and b) does not really reflect whether the resources are overlapped because all-NACK is of low probability in general.

With this analysed, FL suggests taking option 1.

#### Proposal 2.3.4.1

Define the nominal NACK-only PUCCH as a PUCCH consisting of all the symbols from PUCCHs of the NACK-only PUCCH resource set and the nominal NACK-only PUCCH is used for determining whether PUCCH for NACK-only overlaps with other UCI/PUSCH for multiplexing for the same priority.

#### Company views

| Company | Comments |
| --- | --- |
| Nokia, NSB | Not support. Based on the current specs, the UEs are not capable of transmitting at more than one PUCCH resource including HARQ-ACK in the same slot for the same priority. Therefore, if more than one NACK-only feedback are to be transmitted in the same slot using Alt. 4, and if the corresponding PUCCH resource does not overlap with the PUCCH resource of other, e.g., unicast PUCCH with HARQ-ACK, UE is not able to transmit both PUCCHs. Due to this reason, we prefer Option 3. In that case, being scheduled in the same slot would be enough for multiplexing, which would solve the above ambiguity. |
| Qualcomm | If this is only for the case of multiplexing more than one NACK-only bits in PUCCH, we can use different PUCCH sequences but same time/frequency resources. The overlapping of this PUCCH resource and others can be treated using existing rules. |
| Samsung | Do not support because the proposal does not solve the problem. The issue is not only to define a PUCCH resource for the “all-NACK” case. If the number of symbols for PUCCH resources are different for other HARQ-ACK values, a gNB cannot know in general if there is overlapping of PUCCH and PUSCH and cannot know whether a UE multiplexes HARQ-ACK in the PUSCH (and, due to UE-common PUCCH resources, PUCCH DTX cannot work as there can be transmissions from other UEs in any resource). The all-NACK resource does not relate to how likely or not is for a UE to have all-NACK values – it is only for a general reference resource and it is the only choice for the 1-bit case. If the all-NACK resource is not used as reference, the alternative is for all PUCCH resources to have same symbols. That is the simplest solution for the specifications as it also solves the issues in the previous two proposals and removes any related timeline issues and is not restrictive to the NW. For example, a likely configuration would be to have different cyclic shifts for the resources. Also, there is no reason to have different coverage/BLER (due to different number of symbols) depending on what the HARQ-ACK values are and that may have further specification impact if it is to be avoided. |
| Lenovo | Agree with Qualcomm and Samsung. |
| vivo | Same view with QC and Samsung, we prefer to use different PUCCHs with the same time domain configuration to simplify the issue. |
| OPPO | Not support this proposal with the similar view of Nokia and Samsung. |
| ZTE | We don’t support this proposal. From our perspective, as long as any configured NACK-only PUCCH resource is overlapped with UCI/PUSCH for multiplexing, the multiplexing can be performed. |
| NTT DOCOMO | For multiplexing of NACK-only feedback and ACK/NACK-based feedback, we agree with Nokia. Multiplexing should be applied when both are reported in the same slot. For multiplexing conditions of NACK-only feedback and CSI/PUSCH, we support proposal 2.3.4.1. It seems a bit restrictive to use the same symbol for all PUCCH resources. |
| Langbo | We support this proposal. We are also fine with Option 3. We think it is too restrictive if all PUCCH resources can be only configured to use a same set of symbols. |
| Ericsson | Not support. We have same view as Nokia. |
| CATT | We don’t support this proposal. Share same view with ZTE. It is recommended to use all PUCCH resources indicated for NACK-only to determine overlapping, i.e. overlap occurs if any one of PUCCH resource configured/indicated for NACK-only transmission overlaps in time with other PUCCHs/PUSCHs. |
| LG Electronics | We agree with Qualcomm and Samsung |

#### (CLOSED)Round-2

This is the first meeting discussing this essential issue, so maybe too early to go far. Let’s take steps back.

Firstly, the following are what PUCCH-resources are configured from TS38.331. Note that for NACK-only both PF0 and PF1 are agreed.

PUCCH-Resource ::=                      SEQUENCE {

pucch-ResourceId                        PUCCH-ResourceId,

startingPRB                             PRB-Id,

intraSlotFrequencyHopping               ENUMERATED { enabled }                                                OPTIONAL, -- Need R

secondHopPRB                            PRB-Id                                                                OPTIONAL, -- Need R

format                                  CHOICE {

format0                                 PUCCH-format0,

format1                                 PUCCH-format1,

format2                                 PUCCH-format2,

format3                                 PUCCH-format3,

format4                                 PUCCH-format4

}

}

PUCCH-format0 ::=                               SEQUENCE {

initialCyclicShift                              INTEGER(0..11),

nrofSymbols                                     INTEGER (1..2),

startingSymbolIndex                             INTEGER(0..13)

}

PUCCH-format1 ::=                               SEQUENCE {

initialCyclicShift                              INTEGER(0..11),

nrofSymbols                                     INTEGER (4..14),

startingSymbolIndex                             INTEGER(0..10),

timeDomainOCC                                   INTEGER(0..6)

}

Given PF0 and PF1 are both supported, it does not make sense to configure the same starting symbol and the same number of symbols for all the PUCCH resources (especially to PF0 there is even not enough initialCyclicShift to configure 15 PUCCH orthogonal PUCCH resources for 4TBs.) and it is too restrictive to network as well. In addition, when NACK-only for Alt4 overlaps with other UCI/PUSCH, NACK-only is transformed into ACK/NACK and operate multiplexing. However, the multiplexing should not depend on UE decoding results because otherwise NW will do blind decoding.

Observation 1: For the PUCCH resources in the PUCCH resource set configured for NACK-only feedback, the PUCCH resources may have different starting symbol and/or number of symbols.

Observation 2: When UE multiplexes HARQ-ACK bits transformed from NACK-only for Alt4 with other PUCCH/PUSCH transmission, how to determine whether the resources are overlapped needs to be defined.

#### Proposal 2.3.4.2

When the nominal NACK-only PUCCH overlaps with other PUCCH/PUSCH transmission, NACK-only is transformed into ACK/NACK and multiplexed with other PUCCH/PUSCH transmission. Down-select from the following options for the nominal NACK-only PUCCH:

- Option 1: The nominal NACK-only PUCCH consists of all the symbols from PUCCHs of the NACK-only PUCCH resource set.

- Option 2: The nominal NACK-only PUCCH starts from the earliest starting symbol of PUCCHs in the NACK-only PUCCH resource set, and ends at the latest ending symbol of PUCCHs in the set.

- Option 3: The nominal NACK-only PUCCH consists of all the symbols of the PUCCH slot.

- Note: The terminology of the “nominal NACK-only PUCCH” is used for facilitate describing the issue. Up to editor for the specification impact.

#### Company views

| Company | Comments |
| --- | --- |
| Moderator | Please indicate which option you support. |
| Nokia, NSB. | Prefer option 3. Based on the current specs, the UEs are not capable of transmitting at more than one PUCCH resource including HARQ-ACK in the same slot for the same priority. Therefore, if more than one NACK-only feedback are to be transmitted in the same slot (e.g., using Alt. 4), and if the corresponding PUCCH resource does not overlap with the PUCCH resource of other, e.g., unicast PUCCH with HARQ-ACK, UE is not able to transmit both PUCCHs. Due to this reason, we prefer Option 3. In that case, being scheduled in the same slot would be enough for multiplexing, which would solve the above ambiguity. All other options would create the above mentioned ambiguity, if resources do not overlap in time. |
| Lenovo | We support opt 2. |
| Langbo | Our first preference is Option 1, while we can also accept Option 2 or Option 3 if it is majority view. |
| Samsung | We do not support the above options (Option 1 is OK as default reference resource in case of one bit). The simplest and complete solution for all timeline and resource overlapping issues is: “a UE does not expect the resources to have different symbols”. We do not agree with the argument that resources cannot have same symbols (or that is difficult to support) – that is already the case in LTE deployments for channel/resource selection and under much less flexibility than what NR allows. For example, it is trivial for a NW to use different CS for the resources. Also, it is not reasonable to have different coverage depending on the values of HARQ-ACK bits (due to using different symbols). Another problem with all above options is that they will require more maintenance at least for adjustments to the definition of timelines and the resolution of overlapping. At least one new UE capability will also be required for a UE to resolve overlapping and perform multiplexing when PUXCH transmissions do not actually overlap (which will also affect UE hardware which is not allowed by the WID). |
| NTT DOCOMO | For multiplexing NACK-only based feedback with ACK/NACK based feedback, we prefer Option 3. For multiplexing NACK-only based feedback with CSI/PUSCH, we prefer Option 2. |
| ZTE | We prefer Option 1. We also agree Samsung that all the NACK-only resource configured with the same symbol is a simple solution. |
| CATT | We share same view with Samsung. If symbols of all the PUCCH resources are same, then the issues on multiplexing NACK-only with others and PDSCH processing timeline for NACK-only can be solved. |
| Qualcomm | Opt1. Same understanding as Samsung. We should use the different sequences in the PUCCH with the same time/freq resources for Alt4. |
| Huawei, HiSilicon | The PDSCH processing timeline has no relevance to the PUCCHs configuration. Even though all PUCCHs have the same starting symbol and the same time duration, the current PDSCH scheduling timeline is not sufficient. The two issues should be discussed separately. |

#### Summary

Based on the comments, summary of views: Opt1: The nominal NACK-only PUCCH consists of all the symbols from PUCCHs of the NACK-only PUCCH resource set. - Langbo (ok with others), ZTE, Qualcomm, Huawei, HiSilicon Opt2: The nominal NACK-only PUCCH starts from the earliest starting symbol of PUCCHs in the NACK-only PUCCH resource set, and ends at the latest ending symbol of PUCCHs in the set. - Lenovo Opt3: The nominal NACK-only PUCCH consists of all the symbols of the PUCCH slot. - Nokia, DOCOMO Opt4(new): All PUCCHs have the same starting symbol and the same time duration - Samsung, - ZTE/Qualcomm/CATT also share the same view as Samsung

#### (ACTIVE)Round-3

Little chance to converge in the first meeting discussing this issue but deriving guidances for further discussion in the next meeting would be helpful.

#### Proposal 2.3.4.3

When the nominal NACK-only PUCCH overlaps with other PUCCH/PUSCH transmission, NACK-only is transformed into ACK/NACK and multiplexed with other PUCCH/PUSCH transmission. Down-select from the following options for the nominal NACK-only PUCCH:

- Opt1: The nominal NACK-only PUCCH consists of all the symbols from PUCCHs of the NACK-only PUCCH resource set.

- Opt2: The nominal NACK-only PUCCH starts from the earliest starting symbol of PUCCHs in the NACK-only PUCCH resource set, and ends at the latest ending symbol of PUCCHs in the set.

- Opt3: The nominal NACK-only PUCCH consists of all the symbols of the PUCCH slot.

- Opt4: All PUCCHs have the same starting symbol and the same time duration

- Note1: The terminology of the “nominal NACK-only PUCCH” is used for facilitate describing the issue. Up to editor for the specification impact.

- Note2: The following factors can be considered for down-selection:

  - Up to 12 initial cyclic shifts can be configured to PF0. Up to 12 initial cyclic shifts and up to 7 time domain OCC can be configured to PF1.

  - Up to 32 PUCCHs resources in the resource set are configured for NACK-only.

  - The PUCCH configuration for unicast will be used when the PUCCH is not configured for NACK-only.

#### Company views

| Company | Comments |
| --- | --- |
| Samsung | We support Opt. 4 (and support Opt.1 to determine a reference resource for “all-ACK”). There is no issue with the number of CS – if not enough, another PRB can be configured for remaining resources. Most trivial for a NW to do. Having different number of symbols does not make sense for a number of reasons as previously discussed, will have a large maintenance/specification impact, require definition of at least one new UE capability, and may have implications that are not according to the WID. It is also noted that the requirement for same number of symbols does not need to be specified – it can be left to the NW. A UE does not need to support any procedures related to different resources having different symbols (i.e. no additional specifications). The only missing part is a reference resource for “all ACK” in order to determine multiplexing in another PUCCH or PUSCH. For that Opt. 1 is fine/enough. If not possible to conclude at this meeting, it can be postponed to next meeting so that all implications for corresponding alternatives are identified in Tdocs. |
| Lenovo | OK to list all the options and down-select one in next meeting. |
| vivo | We support Opt. 4. We share similar view with Samsung. Different PUCCH resources can be configured with different CSs and/or PRBs. This option has minimum impact on spec and UE implementation. |
| NTT DOCOMO | OK |
| CATT | We are open to discuss Opt.1 and Opt.4. But, for Opt.2 and Opt.2, it may increase the possibility of NACK-only multiplexing with other PUCCH/PUSCH. |
| MediaTek | Ok for the further study. |
| ZTE | We support Opt1 and Opt4. |

### (CLOSED)Power determination for NACK-only

This issue is regarding the PUCCH transmission power for the “NACK-only” reporting mode.

| R1-2203874 Samsung | Proposal 2: For power determination of a PUCCH transmission for the “NACK-only” reporting mode, ∆UCIi=0. Proposal 3: For power determination of a PUCCH transmission with multicast HARQ-ACK, a UE with two CLPC adjustment states applies the one corresponding to l=0. |
| --- | --- |

#### Round-1

The PUCCH transmission power can be determined using ∆UCIi=0 (as for PUCCH format 0 in Rel-15) since, due to the orthogonal resource selection, there is practically no dependence of the BLER on the number of HARQ-ACK bits.

#### Proposal 2.3.5.1-1

For power determination of a PUCCH transmission for the “NACK-only” reporting mode in clause 7.2.1 in TS 38.213, ∆UCIi=0.

#### Company views

| Company | Comments |
| --- | --- |
| Nokia, NSB | We support the proposal. |
| Qualcomm | ok |
| Samsung | Support |
| Lenovo | OK |
| vivo | ok |
| OPPO | OK |
| ZTE | OK |
| NTT DOCOMO | OK |
| Langbo | OK |
| Ericsson | Support |
| Apple | OK |
| CATT | OK |

Also, regardless of HARQ-ACK reporting mode, the power for a PUCCH transmission with multicast HARQ-ACK follows the one for PUCCH with unicast HARQ-ACK. One remaining aspect is the CLPC adjustment state to apply when unicast PUCCH has two CLPC adjustment states. The CLPC adjustment state should be the one for l=0 (as when the UE is not provided twoPUCCH-PC-AdjustmentStates or PUCCH-SpatialRelationInfo for PUCCH with unicast HARQ-ACK).

#### Proposal 2.3.5.1-2

For power determination of a PUCCH transmission with multicast HARQ-ACK mode in clause 7.2.1 in TS 38.213, a UE with two CLPC adjustment states applies the one corresponding to l=0.

#### Company views

| Company | Comments |
| --- | --- |
| Nokia, NSB | We support the proposal. |
| Qualcomm | ok |
| Samsung | Support |
| Lenovo | OK |
| OPPO | OK |
| ZTE | OK |
| NTT DOCOMO | OK |
| Langbo | OK |
| Ericsson | Support |
| Apple | OK |
| CATT | OK |

## (ACTIVE)Issue1-4: CB for disabled HARQ-ACK

The issue to be discussed is Type-1 HARQ codebook generation in cases where UE is configured with a mixture of ACK/NACK based HARQ and disabled HARQ for different G-RNTIs/G-CS-RNTIs to solve the FFSs from the following agreements:

Agreement For Type-1 codebook generation, - if all configured G-RNTIs are with HARQ-ACK disabled by RRC signalling, UE does not report any HARQ-ACK information in the PUCCH slot; - For other cases, FFS between the 3 alternatives below: - Alt1: if at least one configured G-RNTI is with HARQ-ACK enabled, UE will report NACK for the G-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH. - Alt2: if at least one configured G-RNTI is with HARQ-ACK enabled, UE reports actual HARQ-ACK result for all G-RNTIs. - Alt3: UE is not expected to be configured with some G-RNTI with HARQ-ACK disabled by RRC signalling and some other G-RNTI with HARQ-ACK enabled by RRC signalling for all configured G-RNTI - Other alternatives are not precluded Agreement If a UE supports ACK/NACK based feedback for dynamic multicast scheduling and for multicast SPS, and if the UE is configured with ACK/NACK based feedback for multicast dynamic scheduling and is configured with disabled HARQ feedback for multicast SPS scheduling, for Type-1 codebook generation, - FFS between the 4 alternatives below: - Alt1: UE will report NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH. - Alt2: UE will report ACK/NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH. - Alt3: UE is not expected to be configured with G-RNTI with HARQ-ACK enabled by RRC signalling and G-CS-RNTI with HARQ-ACK enabled by RRC signalling. - Alt4: UE does not report any HARQ-ACK information for G-CS-RNTI with HARQ-ACK feedback disabled by RRC. - Other alternatives are not precluded.

The submitted proposals are summarized as follows:

| R1-2203070 Huawei | Proposal 13: For Type-1 codebook generation of dynamic multicast scheduling with multiple G-RNTIs, - if at least one configured G-RNTI is with HARQ-ACK enabled, UE will report NACK for the G-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH. Proposal 14: For Type-1 codebook generation of dynamic multicast scheduling configured with ACK/NACK based feedback and multicast SPS configured with disabled HARQ feedback, - UE will report NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH. Proposal 15: UE is not expected to be configured with Type-1 HARQ-ACK codebook and with presence of enabled/disabled HARQ-ACK feedback indicator in multicast DCI simultaneously. - Note: this restriction is per UE. |
| --- | --- |
| R1-2203194 ZTE | Proposal 5: For Type-1 codebook generation, Alt2 should be adopted. - Alt2: .if at least one configured G-RNTI is with HARQ-ACK enabled, UE reports actual HARQ-ACK result for all G-RNTIs. Proposal 6: If a UE supports ACK/NACK based feedback for dynamic multicast scheduling and for multicast SPS, and if the UE is configured with ACK/NACK based feedback for multicast dynamic scheduling and is configured with disabled HARQ feedback for multicast SPS scheduling, for Type-1 codebook generation, Alt2 should be adopted. - Alt2: UE will report ACK/NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH. |
| R1-2203227 TD-Tech | Proposal 1: For Type-1 codebook generation, the following solution is suggested. - if all configured G-RNTIs are with HARQ-ACK disabled by RRC signalling, UE does not report any HARQ-ACK information in the PUCCH slot; - For other cases, Alt 1 is used: - Alt1: if at least one configured G-RNTI is with HARQ-ACK enabled, UE will report NACK for the G-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH. - Alt2: if at least one configured G-RNTI is with HARQ-ACK enabled, UE reports actual HARQ-ACK result for all G-RNTIs. - Alt3: UE is not expected to be configured with some G-RNTI with HARQ-ACK disabled by RRC signalling and some other G-RNTI with HARQ-ACK enabled by RRC signalling for all configured G-RNTI |
| R1-2203287 Nokia | Proposal 13: For Type-1 codebook generation if at least one configured G-RNTI of the UE is with HARQ-ACK enabled, UE reports NACK for the G-RNTI(s) with HARQ-ACK disabled regardless of decoding results for that latter G-RNTI(s). |
| R1-2203314 Speadtrum | Proposal 1: For Type-1 codebook generation for G-RNTI, support Alt1, i.e., if at least one configured G-RNTI is with HARQ-ACK enabled, UE will report NACK for the G-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH. |
| R1-2203427 CATT | Proposal 5: For Type-1 codebook generation, the UE will not report any HARQ-ACK information for the configured G-RNTI with HARQ-ACK disabled by RRC signaling. |
| R1-2203526 vivo | Proposal 7: For Type-1 codebook generation, - If at least one configured G-RNTI is with HARQ-ACK enabled, UE will report NACK for the G-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH. - If the UE is configured with disabled HARQ feedback for multicast SPS scheduling, UE will report NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH. |
| R1-2203613 ETRI | Proposal1: For Type-1 codebook generation, Alt3: UE is not expected to be configured with some G-RNTI with HARQ-ACK disabled by RRC signalling and some other G-RNTI with HARQ-ACK enabled by RRC signalling for all configured G-RNTI is supported. Proposal2: If a UE supports ACK/NACK based feedback for dynamic multicast scheduling and for multicast SPS, and if the UE is configured with ACK/NACK based feedback for multicast dynamic scheduling and is configured with disabled HARQ feedback for multicast SPS scheduling, for Type-1 codebook generation, Alt3: UE is not expected to be configured with G-RNTI with HARQ-ACK enabled by RRC signalling and G-CS-RNTI with HARQ-ACK enabled disabled by RRC signalling. |
| R1-2203699 Lenovo | Proposal 5: For Type-1 HARQ-ACK codebook determination, UE does not report any HARQ-ACK information for a G-RNTI configured with disabled HARQ-ACK feedback. Proposal 6: Type-1 HARQ-ACK codebook and presence of 1-bit HEI in DCI should not be configured simultaneously for a UE. |
| R1-2203974 OPPO | Proposal 5: For Type-1 codebook generation, Alt 4 is supported that UE does not report any HARQ-ACK information for G-CS-RNTI with HARQ-ACK feedback disabled by RRC. |
| R1-2204282 CMCC | Proposal 7. For Type-1 codebook generation, - if all configured G-RNTIs are with HARQ-ACK disabled by RRC signalling, UE does not report any HARQ-ACK information in the PUCCH slot; - For other cases, Alt2 or Alt3 can be adopted. - Alt2: if at least one configured G-RNTI is with HARQ-ACK enabled, UE reports actual HARQ-ACK result for all G-RNTIs. - Alt3: UE is not expected to be configured with some G-RNTI with HARQ-ACK disabled by RRC signalling and some other G-RNTI with HARQ-ACK enabled by RRC signalling for all configured G-RNTI Proposal 8. If a UE supports ACK/NACK based feedback for dynamic multicast scheduling and for multicast SPS, and if the UE is configured with ACK/NACK based feedback for multicast dynamic scheduling and is configured with disabled HARQ feedback for multicast SPS scheduling, for Type-1 codebook generation, - Alt2 or Alt3 can be adopted. - Alt2: UE will report ACK/NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH. - Alt3: UE is not expected to be configured with G-RNTI with HARQ-ACK enabled by RRC signalling and G-CS-RNTI with HARQ-ACK disabled by RRC signalling. |
| R1-2204354 DOCOMO | Proposal 1: For Type-1 HARQ-ACK codebook generation for multicast - When multiple G-RNTIs are configured, support Alt 1. - When HARQ-ACK feedback for dynamic multicast scheduling is enabled and HARQ-ACK feedback for multicast SPS is disabled, support Alt 1. |
| R1-2204717 MediaTek | Proposal 8: For Type-1 codebook generation, all the configured G-RNTIs are with the same HARQ enabling/disabling behaviour via RRC signalling configuration, i.e., UE is not expected to be configured with some G-RNTI with HARQ-ACK disabled by RRC signalling and some other G-RNTI with HARQ-ACK enabled by RRC signalling for all configured G-RNTI. Proposal 9: If a UE supports ACK/NACK based feedback for dynamic multicast scheduling and for multicast SPS, UE is not expected to be configured with G-RNTI with HARQ-ACK enabled (or disabled) by RRC signalling and G-CS-RNTI with HARQ-ACK disabled (or enabled) by RRC signalling. |
| R1-2204945 Ericsson | Support Alt2 Proposal 24 We propose the same handling as for multiple G-RNTIs in dynamic scheduling, i.e. support Alt2, with a slight clarification: UE will report ACK/NACK for the G-CS-RNTI with HARQ-ACK disabled depending on the decoding results of corresponding PDSCH. |
| R1-2204994 Qualcomm | Proposal 1: If UE does not detect any DCI with enabled feedback (including unicast and multicast), UE does not report any HARQ-ACK feedback even if UE detects multicast DCI with disabled feedback; If UE detects multicast DCIs with enabled and disabled feedback, - If UE is configured with Type-1 CB for multicast, - UE reports multicast HARQ-ACK feedback on the PUCCH resource/slot based on last DCI with enabled or disabled feedback - For a multicast DCI with disabled feedback, UE reports NACK. - If UE is configured with Type-2 CB for multicast, - UE reports multicast HARQ-ACK feedback on the PUCCH resource/slot based on last DCI with enabled feedback. Proposal 2: For a UE configured with Type-1 CB for multicast, if UE detects PDSCH scheduled by dynamic grant multicast and SPS multicast with at least one enabled feedback, - UE reports multicast HARQ-ACK feedback on the PUCCH resource/slot based on the last dynamic grant multicast DCI with enabled or disabled feedback. For multicast G-CS-RNTI or G-RNTI with disabled feedback, UE will report NACK regardless of decoding results of corresponding PDSCH. |

### (CLOSED)Round-1

Based on the submitted proposals, observations are summarized as follows:

Alt1: UE will report NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH

- TD-Tech, Nokia, Spreadtrum, DOCOMO, Huawei

Alt2: UE will report ACK/NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

- ZTE, CMCC, Ericsson, Samsung, vivo

Alt3: UE is not expected to be configured with G-RNTI with HARQ-ACK enabled by RRC signalling and G-CS-RNTI with HARQ-ACK enabled by RRC signalling.

- ETRI, CMCC, MediaTek, Samsung

Alt4: UE does not report any HARQ-ACK information for G-CS-RNTI with HARQ-ACK feedback disabled by RRC.

- CATT, Lenovo, OPPO, Samsung

The argument for Alt1 is that if the HARQ-ACK information for a TB with disabled HARQ-ACK feedback is fixed to NACK, gNB can decode the HARQ-ACK feedback using the known information.

Companies proposing Alt.2 argued that since the UE needs to send HARQ feedback if at least one TB position is received with an enabled G-RNTI, and with Type I codebook the full codebook is anyway sent, it does not make sense to send NACK for disabled G-RNTIs that are correctly received. However, others argued that as the UE anyway produces a Type-1 HARQ-ACK codebook, however, this will add more complexity to UE when constructing the codebook and there would be nearly zero gain on the network side.

One company also pointed out that there is a conclusion about ‘PUCCH resource Indicator’ and ‘PDSCH-to-HARQ_feedback timing indicator’ fields of DCI format 4_1/4_2 for HARQ feedback disabled as follows:

Conclusion (RAN1#108e) When HARQ feedback is disabled, the following fields (if present) of DCI format 4_1/4_2 can be assumed to be reserved and UE ignores them: - PUCCH resource Indicator - PDSCH-to-HARQ_feedback timing indicator

As long as one G-RNTI is configured with HARQ-ACK enabled, UE needs to report something to keep the Type-1 codebook size fixed. Given UE needs to decode the TB and generate the Type-1 codebook anyway, from UE side, it does not make sense seemingly to send NACK for disabled G-RNTIs that are correctly received but network does not expect to take that HARQ-ACK feedback either if it is disabled. Since reporting NACK has been agreed for the disabled HARQ for NR NTN, keeping the same approach may simplify specification change. FL suggests taking Alt1.

#### Proposal 2.4.1-1

For Type-1 codebook generation, if at least one configured G-RNTI is with HARQ-ACK enabled, UE will report NACK for the G-RNTI with HARQ-ACK disabled regardless of decoding results of the corresponding PDSCH.

#### Company views

| Company | Comments |
| --- | --- |
| Nokia, NSB | We support the proposal. |
| Qualcomm | Our views in R1-2202994 are missing. We think if UE does not detect any DCI with enabled feedback (including unicast and multicast), the UE does not report any HARQ-ACK feedback even if UE detects multicast DCI with disabled feedback. If UE detects multicast DCIs with enabled and disabled feedback, UE reports NACK for the G-RNTI with HARQ-ACK disabled. To report ACK/NACK means UE ignores the gNB configuration of disabled HARQ-ACK feedback and send information which is not preferred to be reported. Therefore, we suggest changing the proposal as #### Proposal 2.4.1-1 For Type-1 codebook generation, if at least one configured G-RNTI is with HARQ-ACK enabled and at least one G-RNTI with HARQ-ACK enabled is detected, UE will report NACK for the G-RNTI with HARQ-ACK disabled regardless of decoding results of the corresponding PDSCH. Besides, we think it is not clear how to let UE determine PUCCH resource/slot in case of Type-1 codebook with enabled/disabled feedback. RAN1 to decide Alt1 or Alt2: Alt1: PUCCH resource/slot is based on last DCI with enabled or disabled feedback Alt2: PUCCH resource/slot is based on last DCI with enabled feedback |
| Samsung | NACK-only for NTN was agreed in order to reduce PUCCH transmission power in case of RM coding (due to known bit values). However, such reduction was not subsequently agreed. If that is now agreed for MBS (as it should as, otherwise, Alt. 1 is fully detrimental), there will not be commonality with NTN. There are also a few other occasions where MBS differs from NTN (e.g. on whether other fields, like the TPC or the PRI, from a DCI format associated with disabled HARQ-ACK are used or not). If a UE sends a bit, there is no other reason for that bit to not have any information. Alt. 4 would be closer to the spirit of disabling HARQ-ACK for a G-RNTI but either Alt. 2 or Alt. 3 are also fine to minimize specification impact (there is no added UE complexity). Alt. 1 is not preferred. |
| LG Electronics | OK |
| Lenovo | Not support. Since the HARQ-ACK feedback enabled or disabled is configured by RRC signaling, there should be no misunderstanding between UE and gNB on the Type-1 HARQ-ACK codebook size. For a G-RNTI configured with disabled HARQ-ACK feedback, it does make sense that UE shall not report any HARQ-ACK information for the G-RNTI. |
| Vivo | There is an editorial mistake in our proposal in our contribution. But in the text, we prefer to use Alt 2. As Samsung points out, NACK-only for NTN was agreed in order to reduce PUCCH transmission power in case of RM coding (due to known bit values). However, such reduction was not subsequently agreed. In addition, RM codeing is only used for UCI with no more than 11bits, for type 1 codebook, there may be more than 11 bits, typically. |
| OPPO | Not support. Similar view with Lenovo that it should be aligned between gNB and UE on whether to report HARQ-ACK information by enabling/disabling. |
| ZTE | Similar view as Samsung, Alt.2 is more reasonable from our perspective. |
| NTT DOCOMO | Support. As per FL’s summary, Alt 1 would be useful to improve transmission performance of Type-1 codebook. |
| Spreadtrum | Ok |
| MediaTek | We still think the Alt 3 is simpler and it can be avoided by gNB implementation, and it will not be configured with G-RNTI with HARQ-ACK enabled by RRC signalling and G-CS-RNTI with HARQ-ACK enabled by RRC signalling. However, considering the meeting progress, we can live with the FL’s proposal if majority views are ok with it. |
| Ericsson | Not support. The discussion can be divided into aspects related to complexity and aspects related to performance. For complexity we are open to consider arguments why there may be issues, but since the UE anyway supports correct HARQ ACK/NACK feedback for Type I codebook, it is difficult to see any significant complexity issues by supporting this also when a subset of PDSCHs have HARQ-ACK feedback disabled (via RRC or DCI). After all, the UE must anyway be prepared to send HARQ-ACK feedback for all G-RNTIs of the Type I codebook. Regarding performance, this is affected in two different ways if (1) NACK is always sent in such cases or if (2) the correct ACK/NACK feedback is always sent (which we prefer). - If NACK is always sent for a G-RNTI with disabled HARQ-ACK feedback, the Type I CB feedback does not carry any information for such G-RNTIs, so the transmitted HARQ-ACK info is a priori-known by the gNB. This allows the gNB to use this for a slightly more robust detection (i.e. advantage) of the HARQ feedback relevant for the enabled G-RNTIs, which do transmit HARQ-ACK feedback in the same codebook. - If correct HARQ-ACK feedback is always sent, this allows a failed PDSCH reception to trigger a retransmission (i.e. advantage). The difference between the two cases is thus that whereas in (1) the UL robustness increases, the support for HARQ retransmission is disabled for some G-RNTIs, which has obvious negative DL robustness impact. In the second case the HARQ-ACK feedback has normal robustness but lacks the additional robustness of the first case. Instead, it can maintain full HARQ retransmission support. When comparing the importance of increased UL robustness of the HARQ-ACK feedback with the benefit of DL robustness by having HARQ retransmission support, it is clear that maintaining HARQ retransmission support is far more important than further increasing the UL robustness, which is anyway maintained at normal level. As an example: With (1) the absence of HARQ-ACK feedback of G-RNTI1 increases the HARQ-ACK feedback robustness for G-RNTI1, at the expense of lost HARQ retransmission support for G-RNTI2. With (2) both G-RNTI1 and G-RNTI2 maintain their HARQ retransmission functionality and also their normal HARQ-ACK feedback robustness. |
| CATT | Not support. Per our understanding, PDSCH scheduled by DCI format 4_1/4_2 has no corresponding PRI and K1 indication when HARQ feedback is disabled, which is similar to the PDSCH scheduled by DCI format 1_0 scrambled by SI-RNTI. As we known the UE will not report any HARQ-ACK information of the PDSCH scheduled by DCI format 1_0 scrambled by SI-RNTI when the UE is configured with Type-1 codebook. Based on the same principle, Alt.4 is more reasonable for disable HARQ-ACK. |
| Qualcomm2 | Comment to CATT: We think the case of SI-RNTI is different from multicast feedback for different G-RNTIs. SI-RNTI are configured with periodicity and offset and even no codebook is applied to this case. For Type1-CB for multicast, G-RNTIs are sharing same TDRA table and k1 with common PDSCH candidate occasions. Let’s say G-RNTI1 with ACK/NACK and G-RNTI2 without ACK-NACK. G-RNTI1 is sent in PDSCH occasion1 and G-RNTI2 is sent in PDSCH occasion2. When UE generates the Type-1 CB with fixed size, what UE should do for PDSCH occasion2? Based on your preference, UE will send NACK since UE does not detect G-RNTI1? If so, we think it is same as Alt1. |

#### Summary

Based on the comments: Alt1: UE will report NACK for the G-RNTI/G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH - TD-Tech, Nokia, Spreadtrum, DOCOMO, Huawei, [MediaTek], Qualcomm, LGE, Alt2: UE will report ACK/NACK for the G-RNTI/G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH. - ZTE, CMCC, Ericsson, Samsung, vivo Alt3: UE is not expected to be configured with G-RNTI with HARQ-ACK enabled by RRC signalling and G-CS-RNTI with HARQ-ACK enabled by RRC signalling. - ETRI, CMCC, MediaTek, Samsung Alt4: UE does not report any HARQ-ACK information for G- RNTI/G-CS-RNTI with HARQ-ACK feedback disabled by RRC. - CATT, Lenovo, OPPO, Samsung UE does not detect enabled DCI but detect disabled DCI, UE does not report: Qualcomm There are companies constantly commenting that UE should not report anything. Based on the discussion, it should be obviously clear that Type-1 codebook size is not aligned between NW and UE. Keep in mind that UE is not aware which PDSCH occasion the G-RNTI will be scheduled even though both UE and NW know which G-RNTI is configured with enabled/disabled HARQ-ACK

Likewise, the same approach can be taken for the case of UE with ACK/NACK based feedback for multicast dynamic scheduling and is configured with disabled HARQ feedback for multicast SPS scheduling.

#### Proposal 2.4.1-2

If a UE supports ACK/NACK based feedback for dynamic multicast scheduling and for multicast SPS, and if the UE is configured with ACK/NACK based feedback for multicast dynamic scheduling and is configured with disabled HARQ feedback for multicast SPS scheduling, for Type-1 codebook generation,

- UE will report NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of the corresponding PDSCH.

#### Company views

| Company | Comments |
| --- | --- |
| Nokia, NSB | We support the proposal. |
| Qualcomm | Our views in R1-2202994 are missing. We think if UE detects PDSCH scheduled by dynamic grant multicast and SPS multicast with at least one enabled feedback, UE will report NACK for G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH; otherwise, UE does not report. #### Proposal 2.4.1-2 If a UE supports ACK/NACK based feedback for dynamic multicast scheduling and for multicast SPS, and if the UE is configured with ACK/NACK based feedback for multicast dynamic scheduling and is configured with disabled HARQ feedback for multicast SPS scheduling and detects at least one PDSCH with HARQ-ACK feedback, for Type-1 codebook generation, - UE will report NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of the corresponding PDSCH. Although RAN1 agreed to use dynamic multicast DCI for PUCCH based on the following agreement. However, it is not clear how to let UE determine PUCCH resource/slot with enabled/disabled feedback. Agreement When HARQ-ACK for multicast dynamic grant PDSCHs and multicast SPS PDSCHs with ACK/NACK based feedback are multiplexed on the same PUCCH for the same priority case, the PUCCH carrying the multiplexed HARQ-ACK is determined from PUCCH-Config/PUCCH-ConfigurationList configured for multicast RAN1 to decide Alt1 or Alt2: Alt1: PUCCH resource/slot is based on last dynamic grant multicast DCI with enabled or disabled feedback Alt2: PUCCH resource/slot is based on last dynamic grant multicast DCI with enabled feedback |
| Samsung | Same as for proposal 2.4.1-1. |
| LG Electronics | OK |
| Lenovo | Same as for proposal 2.4.1-1. |
| Vivo | Same as for proposal 2.4.1-1 |
| OPPO | Same as for proposal 2.4.1-1 |
| ZTE | Same as for proposal 2.4.1-1. |
| NTT DOCOMO | Support |
| Spreadtrum | Support |
| MediaTek | Share the similar view as for proposal 2.4.1-1. |
| Ericsson | Not support. Same comment as for Proposal 2.4.1-1. |
| CATT | Same as for proposal 2.4.1-1 |

#### Summary

This issue can be discussed after the dynamic scheduling case is fiexed.

The following proposal has been discussed and converged in the last meeting but was not agreed in the end due to limited time, the same proposal is kept for approval in this meeting:

#### Proposal 2.4.1-3

UE is not expected to be configured with Type-1 HARQ-ACK codebook and with presence of enabled/disabled HARQ-ACK feedback indicator in multicast DCI simultaneously.

- Note: this restriction is per UE.

#### Company views

| Company | Comments |
| --- | --- |
| Nokia, NSB | We support the proposal. |
| Qualcomm | We think DCI-based enabled/disabled HARQ-ACK feedback can be supported for Type-1 CB and treated in the same way as reporting NACK for disabled HARQ-ACK feedback. |
| Samsung | Support |
| LG Electronics | OK |
| Lenovo | OK |
| vivo | OK |
| OPPO | OK |
| ZTE | OK |
| NTT DOCOMO | Support |
| Spreadtrum | Support |
| MediaTek | Support |
| Ericsson | Not support. Consider the following use case: G-RNTI1 and G-RNTI2 are transmitted to a set of UEs. Only G-RNTI2 is transmitted to another set of UEs. Due to UL congestion the gNB needs to release PUCCH resources by disabling HARQ feedback for the second group, which only receives DCI with disabled G-RNTI2 HARQ feedback. The first group UEs will receive G-RNTI1 DCIs with enabled and G-RNTI2 DCIs with disabled HARQ feedback and can then transmit correct HARQ feedback on both G-RNTI1 and G-RNTI2 (with our proposed UE behavior in this case). With Proposal 2.4.1-3, both G-RNTIs could not share CB in this way for the first set of UEs. Proposal 2.4.1-3 therefore unnecessarily restricts flexibility. |

#### Summary

Qualcomm and Ericsson have different views from the majority. In general Type-1 codebook size should not be affected by dynamic scheduling. Qualcomm views the DCI can disable the HARQ-ACK but UE just reports NACK or the actual decoding result. Ericsson bring up a case that for the HARQ-ACK can be disabled by DCI for one group of UE but enabled by RRC for another group of UEs. Essentially, the question is when UE is configured with Type-1 codebook and whether DCI can disable the HARQ-ACK feedback and if the DCI field indicates disabling HARQ-ACK, what’s UE behaviour to generate the HARQ-ACK codebook? Clearly, the majority view is DCI is not expected to indicate disabling HARQ-ACK. If this proposal is not agreed, FL’s understanding is that UE behaviour is unclear when this case happen then NW is likely not to make this case happen.

### (ACTIVE)Round-2

Based on the Round-1 discussion, the first two proposals are relevant so can focus on solving the first one how to report HARQ-ACK for the G-RNTI configured with disabled HARQ-ACK.

Alt1: UE will report NACK for the G-RNTI/G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH - TD-Tech, Nokia, Spreadtrum, DOCOMO, Huawei, [MediaTek], Qualcomm, LGE, Alt2: UE will report ACK/NACK for the G-RNTI/G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH. - ZTE, CMCC, Ericsson, Samsung, vivo Alt3: UE is not expected to be configured with G-RNTI with HARQ-ACK enabled by RRC signalling and G-CS-RNTI with HARQ-ACK enabled by RRC signalling. - ETRI, CMCC, MediaTek, Samsung Alt4: UE does not report any HARQ-ACK information for G- RNTI/G-CS-RNTI with HARQ-ACK feedback disabled by RRC. - CATT, Lenovo, OPPO, Samsung UE does not detect enabled DCI but detect disabled DCI, UE does not report: Qualcomm

There are companies constantly commenting that UE should not report anything. Based on the discussion, it should be obviously clear that Type-1 codebook size is not aligned between NW and UE. Keep in mind that UE is not aware which PDSCH occasion the G-RNTI will be scheduled even though both UE and NW know which G-RNTI is configured with enabled/disabled HARQ-ACK.

Based on the feedback, the main debate is whether UE reports NACK or reports the actual decoding result. Samsung argues that reporting NACK is not subsequently agreed for NTN and then there will not be commonality with NTN. Companies are encouraged to further check.

Moreover, Qualcomm commented further differentiating whether the G-RNTI with HARQ-ACK enabled is detected or not is needed, i.e., UE report HARQ-ACK for G-RNTI with HARQ-ACK disabled when the G-RNTI with HARQ-ACK enabled is detected and UE does not report HARQ-ACK for G-RNTI with HARQ-ACK disabled when UE does not detect any DCI for G-RNTI with HARQ-ACK enabled. In addition, when UE reports HARQ-ACK for G-RNTI with HARQ-ACK disabled, which PUCCH resources is used, PUCCH resource/slot is based on last DCI with enabled or disabled feedback or only based on last DCI with enabled feedback. Two FFS are added for further discussion.

#### Proposal 2.4.2

For Type-1 codebook generation, if at least one configured G-RNTI is with HARQ-ACK enabled, down-select between the following:

- Alt1: UE will report NACK for the G-RNTI with HARQ-ACK disabled regardless of decoding results of the corresponding PDSCH.

- Alt2: UE will report ACK/NACK for the G-RNTI with HARQ-ACK disabled per the decoding result of corresponding PDSCH.

- FFS whether to differentiate whether the G-RNTI with HARQ-ACK enabled is detected or not.

- FFS the PUCCH resource for the codebook transmission

  - Opt1: PUCCH resource/slot is based on last DCI with enabled or disabled feedback

  - Opt2: PUCCH resource/slot is based on last DCI with enabled feedback

#### Company views

| Company | Comments |
| --- | --- |
| Moderator | Alt1 and Alt2 may be decided on GTW. Companies can check the views on the newly added FFS. |
| Samsung | Alt.2. It is also fine to skip the above scenario without any agreement (clearly not a case worth specifying support for). To clarify the NTN situation. It was agreed for a UE to report NACK – the only reason was to exploit known bit values in case of RM code and reduce Tx power/interference. But those were not agreed in a same package. Then, after NACK reporting was agreed, power reduction was not agreed. If ‘NACK’ report happens for MBS, the only reason again would be to reduce Tx power – but that would then be different from the (bad) NTN design. As previously mentioned, there are additional differences in the MBS and NTN designs on same functionalities (e.g. for HARQ-ACK disabling). |
| Lenovo | Not support. If a first G-RNTI is configured with disable HARQ-ACK feedback while a second G-RNTI is configured with enabled HRQ-ACK feedback, based on current proposal, either Alt 1 or Alt 2 reports feedback for the first G-RNTI. What’s the substantial meaning for gNB to configure different HARQ-ACK feedback options for different G-RNTIs? why not configure same option for all the configured G-RNTIs? FL’s response: All G-RNTI should be enabled/disabled all together or can be separately configured has been debated long and we ended up this flexibility. I hope we don’t go back to the beginning to reopen the discussion again. |
| vivo | Alt 2. We think alt 2 is only applicable when UE transmit type 1 codebook, when UE does not detect any DCI with enabled HARRQ-ACK, UE does not need to transmit HARQ-ACK, thus, UE would not feedback ACK/NACK for the PDSCH with disabled HARQ-ACK. |
| NTT DOCOMO | We prefer Alt 1. For the 2nd FFS, we prefer Opt2. |
| CATT | Not support. Alt 4 is preferred. We have achieve the conclusion in RAN1#108e meeting that UE will ignore PRI and K1 when HARQ feedback is disabled. Based on this conclusion, UE can’t report any HARQ-ACK information (ACK/NACK or NACK for the G-RNTI with HARQ feedback disabled). Conclusion (RAN1#108e) When HARQ feedback is disabled, the following fields (if present) of DCI format 4_1/4_2 can be assumed to be reserved and UE ignores them: - PUCCH resource Indicator - PDSCH-to-HARQ_feedback timing indicator. Responds to Qualcomm. We don’t think Alt4 is same as Alt 1. For Alt.4, the K1 and PRI is not required in the DCI which aligns with the above conclusion achieved in RAN1#108e. For Alt.1, the K1 and PRI is required in the DCI, then UE can determine the PUCCH resource/slot to feedback NACK for G-RNTI with HARQ-ACK disabled. FL’s response: at least for Type-2 codebook, when HARQ-ACK is disabled and UE does not report, this conclusion can still hold. When it is type-1 codebook, if all G-RNTI are disabled HARQ-ACK, this conclusion can also hold. However, when it is type-1 for both enabled and disabled HARQ-ACK, I hope you have the common understanding that Type-1 CB size needs to be fixed between NW and UE |
| MediaTek | Our first preference is that the issue can be avoided by NW scheduling. Considering the meeting progress, we can live with Alt 1 |
| ZTE | We prefer Alt 2. |

## (ACTIVE)Issue1-11: Type2 CB for multicast SPS

- RAN1 agreed DAI is separately counted per G-RNTI and Type-2 codebook is constructed by concatenating Type-2 sub-codebook of each RNTI following the ascending order of the G-RNTI value as follows:

Agreement: When UE supports and is configured with more than one G-RNTI, - for Type-2 codebook construction, DAI is separately counted per G-RNTI. - Type-2 codebook is constructed by concatenating Type-2 sub-codebook of each RNTI following the ascending order of the G-RNTI value.

How Type-2 CB is generated for G-CS-RNTI is missing since G-RNTI and G-CS-RNTI are separately configured to UEs and they are not associated, which is different from unicast SPS and dynamic scheduling, because for unicast C-RNTI and CS-RNTI are not differentiated when generating Type-2 HARQ-ACK CB and DAI is counted regardless of the RNTI.

Two companies proposed solutions for this issue:

| R1-2203427 CATT | Proposal 7: If Type-2 HARQ-ACK is configured for one or more G-CS-RNTI, the C-DAI counted and sub-codebook constructed for each G-CS-RNTI separately. |
| --- | --- |
| R1-2203874 Samsung | Proposal 9: For Type-2 HARQ-ACK CB construction, down-select between the following two options and append the Type-2 HARQ-ACK CB for G-CS-RNTIs to the Type-2 HARQ-ACK for G-RNTIs Option 1) DAI is counted separately per G-CS-RNTI – the HARQ-ACK sub-codebooks associated with G-CS-RNTIs are in ascending order of the corresponding G-CS-RNTI values Option 2) DAI is counted jointly across all G-CS-RNTIs |

### (CLOSED)Round-1

Considering what has been agreed for G-RNTI and two companies proposed the same option, FL suggests the following proposal:

#### Proposal 2.5.1

When UE supports and is configured with more than one G-CS-RNTI,

  - for Type-2 codebook construction, DAI is separately counted per G-CS-RNTI,

  - Type-2 codebook is constructed by concatenating Type-2 sub-codebook of each G-CS-RNTI following the ascending order of the G-CS-RNTI value.

  - Type-2 HARQ-ACK sub-book for G-CS-RNTIs is appended to the one for G-RNTI.

#### Company views

| Company | Comments |
| --- | --- |
| Nokia, NSB | Support. |
| Qualcomm | Ok in principle |
| Samsung | Support |
| LG Electronics | ‘sub-book’ should be changed to ‘sub-codebook’ in the last sub-bullet. |
| Lenovo | OK with first two bullets. For the last bullet, we are not clear about it. What’s intention to append CB for G-CS-RNTIs to the one for G-RNTI? Does it imply there is association between G-CS-RNTI and G-RNTI? |
| Vivo | Ok in principle |
| OPPO | Generally OK. |
| ZTE | For group-common SPS, we are not sure why there would be DAI for them? Can the proponents clarify a little bit? |
| Spreadtrum | Ok. @ZTE, here DAI is UL DAI, not DL DAI. |
| Ericsson | Support |
| CATT | Support. |

#### Summary

To respond Lenovo/ZTE comment: - The last bullet is to solve how to generate the TYPE-2 codebook for G-CS-RNTI and G-RNTI since DL-DAI is independently counting, which is different from legacy for which C-RNTI and CS-RNTI are jointly counting. To spreadtrum and all, - I think we are talking about DL-DAI, right? The reason is the legacy procedure does not apply. For legacy, the DL-DAI for C-RNTI and CS-RNTI is jointly counting.

### (CLOSED)Round-2

The proposal is updated based on comments/suggestions from Round-1.

#### Proposal 2.5.2

When UE supports and is configured with more than one G-CS-RNTI,

  - for Type-2 codebook construction, DL-DAI is separately counted per G-CS-RNTI,

  - Type-2 codebook is constructed by concatenating Type-2 sub-codebook of each G-CS-RNTI following the ascending order of the G-CS-RNTI value.

  - Type-2 HARQ-ACK sub-codebook for G-CS-RNTIs is appended to the one for G-RNTI.

#### Company views

| Company | Comments |
| --- | --- |
| Apple | Not sure the proposal is really needed, maybe the last bullet can be considered. For legacy SPS PDSCH, there is no scheduling DCI, thus no DL-DAI. Checking the legacy SPS HARQ-ACK bit generation for type 2 CB in 9.1.3.1 of 38.213, If a UE is configured to receive SPS PDSCH and the UE multiplexes HARQ-ACK information for multiple activated SPS PDSCH receptions, including the ones associated with the corresponding activation DCI, in the PUCCH in slot n, the UE generates the HARQ-ACK information as described in clause 9.1.2 and appends it to the OACK HARQ-ACK information bits. In section 9.1.2, it gives how to count the HARQ-ACK bits of SPS. |
| Nokia, NSB. | Ok. |
| Lenovo | OK |
| Samsung | OK @Apple: That is for TB retransmissions that are scheduled by G-CS-RNTI (are considered DG-PDSCHs), not for initial TB transmissions (SPS PDSCHs). |
| NTT DOCOMO | OK |
| vivo | OK |
| LG Electronics | OK |
| ZTE | We are fine with this proposal. |
| CATT | OK |
| Qualcomm | ok |

#### Summary

Based on comments, the view seems converging with clarification from Samsung. Not sure if it needs to be clarified in the proposal…

### (ACTIVE)Round-3

Based on Apple’s comment and Samsung’s clarification, the proposal is updated to solve the case of Type-2 codebook generation for the TB retransmissions that are scheduled by G-CS-RNTI.

#### Proposal 2.5.3

When UE supports and is configured with more than one G-CS-RNTI, for TB retransmissions that are scheduled by G-CS-RNTI,

  - for Type-2 codebook construction, DL-DAI is separately counted per G-CS-RNTI,

  - Type-2 codebook is constructed by concatenating Type-2 sub-codebook of each G-CS-RNTI following the ascending order of the G-CS-RNTI value.

  - Type-2 HARQ-ACK sub-codebook for G-CS-RNTIs is appended to the one for G-RNTI.

#### Company views

| Company | Comments |
| --- | --- |
| Samsung | OK |
| Lenovo | OK |
| vivo | We think for the first multicast SPS PDSCH after activation, it is also scheduled by G-CS-RNTI, same rule should be applied. We suggest to update “for TB retransmissions” to “for TB transmissions”. |
| NTT DOCOMO | OK |
| LG Electronics | OK |
| CATT | OK |
| MediaTek | Ok |
| ZTE | OK |

## (ACTIVE)Issue1-15: HARQ-ACK feedback for SPS reactivation

As discussed in [2], if a UE fails to receive a GC-PDCCH for activating a SPS transmission, gNB would resend the activation GC-PDCCH for this UE. Then, some UEs which had already received the initial transmission of the activation may also receive this retransmission of the activation. What’s UE behavior for HARQ-ACK feedback for this reactivation. The proposal from [2] is as follows:

| R1-2203194 ZTE | Proposal 10: A GC-PDCCH activating a SPS transmission which has already been activated can be considered as a reactivation GC-PDCCH. - For the PDSCH associated with the reactivation GC-PDCCH, feedback information are generated according to feedback mode of SPS PDSCH without PDCCH. |
| --- | --- |

### (CLOSED)Round-1

Since SPS PDSCH transmission occasions need to follow the configuration from higher layer and the timing of k1 included in activation command, the PDSCH transmission occasions are not expected to be changed even though gNB may retransmit the activation DCI for UEs which missed the DCI. From this sense, it is FL’s understanding that, for UEs has already been activated, when UE receives the reactivation DCI, the UE is not expected to generate the HARQ-ACK feedback.

#### Proposal 2.6.1 (for conclusion)

If UEs has received a first DCI activating a PDSCH, when UE receives a second DCI activating the same PDSCH, UE is not expected to generate HARQ-ACK feedback for the PDSCH activated by the second DCI.

#### Company views

| Company | Comments |
| --- | --- |
| Spreadtrum | Ok |
| ZTE | Thanks FL for including the discussion for this issue. The conclusion itself may be not totally correct since UE still needs to generate feedback for the PDSCH activated by the second DCI, it is just that UE generates the feedback by considering the first SPS PDSCH after the second DCI as SPS PDSCH without PDCCH. Since different feedback modes may be configured for SPS activation and SPS PDSCH without PDCCH. Thus, we prefer to have the following wording. If UE has received a first DCI activating a SPS, when UE receives a second DCI activating the same SPS, UE considers the first SPS PDSCH after the second DCI as SPS PDSCH without PDCCH. |
| Nokia, NSB. | Ok and agree with ZTE’s understanding. |
| Samsung | Agree with ZTE. |
| NTT DOCOMO | If a UE receives a SPS activation correctly but the HARQ-ACK feedback does not reach gNB correctly, there will be a state recognition mismatch between gNB and the UE. If NACK-only based feedback is configured for SPS PDSCH without PDCCH, when gNB retransmits the activation, gNB will expect ACK/NACK based feedback, but the UE will send NACK-only based feedback. We are not sure if the mismatch can be easily resolved. |
| vivo | Not agree. Currently, for a unicast SPS PDSCH, gNB can reactive the SPS PDSCH with a second DCI when the SPS PDSCH is already activated with a first PDCCH, in this case, UE just follow the current spec to feedback HARQ-ACK for the activation. i.e., UE feedback HARQ-ACK for the first SPS PDSCH after the second DCI as SPS PDSCH with PDCCH. We are not clear why new behavior is needed. |
| LG Electronics | We are fine with ZTE’s proposal. |
| ZTE | We would like to further clarify the scenario. In our contribution, the scenarios that a first UE receives a second DCI activating the same SPS PDSCH include the network actually transmits the second activation DCI for a second UE due to the fact that the second UE is newly added to receive the MBS service or the second UE feeds back a NACK for the previous activation DCI. In each case, the network can be aware of the UE has already detected the first activation DCI. For the scenario mentioned by DOCOMO, we agree that this UE may also receives the second activation DCI if the network misunderstands the ACK from the UE. From understanding, the possibility of PUCCH detection failure is quite low. We can hear more views for this case. |
| Qualcomm | For the wording from ZTE, there may be misalignment between gNB and UE. - Similar concern as DCM, if the UE received the first DCI activating a SPS but gNB didn’t receive the PUCCH, gNB cannot know whether the SPS is activated to the UE or not. - If the UE received the first DCI activating a SPS and send NACK, the second DCI for the retransmission of activating SPS cannot be considered as SPS PDSCH without PDCCH. |

#### Summary

FL’s clarification to the proposal: - The following figure shows if reactivation DCI is transmitted, the PDSCH transmission occasions are not expected to be changed because some other UEs have been receiving PDSCH that has been activated. - When UE receives this re-activation DCI, the SPS PDSCH2 HARQ-ACK feedback is still generated following the procedure UE received the activation DCI command. UE does not need to additionally report the HARQ-ACK for SPS PDSCH2 following the the procedure UE receives the re-activation DCI command. - ZTE’s suggestion may not consider the case in the following figure: - Vivo commented that for SPS unicast, gNB can reactive the SPS PDSCH with a second DCI when the SPS PDSCH is already activated with a first PDCCH, in this case, UE just follow the current spec to feedback HARQ-ACK for the activation. i.e., UE feedback HARQ-ACK for the first SPS PDSCH after the second DCI as SPS PDSCH with PDCCH.

### (ACTIVE)Round-2

Based on the comments in Round-1, companies have different options to solve this issue. Basically, there are two views, UE follows the HARQ-ACK generation procedures according to the first activation command, or UE follows the HARQ-ACK generation procedures according to the re-activation command.

#### Proposal 2.6.2

If UE has received a first DCI activating a SPS PDSCH configuration, when the UE receives a second DCI re-activating the same SPS PDSCH configuration, down-select from the following two options:

- Opt1: the UE generates HARQ-ACK feedback following the procedure according to the first DCI activating the SPS PDSCH configuration. The UE is not expected to generate additional HARQ-ACK feedback for the SPS PDSCH re-activated by the second DCI.

- Opt2: the UE generates HARQ-ACK feedback only following the procedure according to the second DCI re-activating the SPS PDSCH configuration.

#### Company views

| Company | Comments |
| --- | --- |
| Moderator | Companies can share the view which option is suggested. |
| Samsung | Support Opt2. Opt2 ensures a same understanding between UE and gNB – Opt1 does not. |
| Lenovo | Generally, we support option 2 and would like to make it more generic as following the latest received DCI. |
| vivo | Support Opt2 |
| NTT DOCOMO | We prefer Opt2. |
| LG Electronics | We prefer Opt2 |
| CATT | Support Opt2. |
| ZTE | First, what is the procedure according to the second DCI re-activating the SPS PDSCH configuration? Our preference keeps unchanged, which is that the UE considers the first PDSCH after the second DCI as a SPS PDSCH without PDCCH. |

# Text proposals to TS38.213

Issues to be discussed/approved in TP format include issues 1-12, 1-13, 1-14.

## (ACTIVE)TP#1 (issue1-12)

This is a text proposal from [12] regarding multiplexing of “NACK-only” HARQ-ACK with CSI which has not yet been captured in TS38.213.

### (CLOSED)Round-1

#### Proposal 3.1.1

Adopt the following text proposal to TS38.213 v17.1.0:

- Reason for change

  - How UE multiplexes NACK-only HARQ-ACK feedback for multicast with only CSI reports is undefined from the latest version of TS 38.213.

- Summary of change

  - Adding a sentence in clause 18 that “If the first PUCCH includes only CSI reports, the UE multiplexes the HARQ-ACK information as described in clause 9.2.5.2 for HARQ-ACK information that is in response to PDSCH reception without corresponding PDCCH”.

- Consequences if not approved

  - UE behavior about multiplexing NACK-only HARQ-ACK feedback for multicast with CSI for unicast is unclear.

------------------------------Text proposal to TS38.213 v17.1.0 Starts--------------------------------- # 18 Multicast Broadcast Services < Unchanged parts are omitted > If a UE would multiplex HARQ-ACK information that the UE would provide according to the second HARQ-ACK reporting mode with other UCI in a first PUCCH, or in a PUSCH, as described in clauses 9 and 9.2.5, the UE provides the HARQ-ACK information according to the first HARQ-ACK reporting mode. For resolving an overlapping among a second PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode and other PUCCHs or PUSCHs prior to multiplexing the HARQ-ACK information in a PUCCH or PUSCH, the UE considers that the UE would transmit the second PUCCH when all values of the HARQ-ACK information are 'ACK'. If the first PUCCH includes only CSI reports, the UE multiplexes the HARQ-ACK information as described in clause 9.2.5.2 for HARQ-ACK information that is in response to PDSCH reception without corresponding PDCCH. If a UE is provided multiple G-RNTIs or G-CS-RNTIs, a configuration for a HARQ-ACK codebook type applies to all G-RNTIs or G-CS-RNTIs. < Unchanged parts are omitted > --------------------------------Text proposal to TS38.213 v17.1.0 Ends--------------------------------

#### Company views

Companies can comment on the summary of the TP and the TP itself.

| Company | Comments |
| --- | --- |
| Qualcomm | It is not clear to us why NACK-only+CSI should be same as PDSCH without PDCCH. If NACK-only is transformed to ACK/NACK, can we just follow the way of ACK/NACK+CSI? |
| Samsung | Support. |
| LG Electronics | We have the same comment with Qualcomm. |
| Lenovo | Same concern with Qualcomm. |
| vivo | We have the same view with Qualcomm and LG. |
| Ericsson | Agree to the proposed change of the text proposal. However, in the original text proposal we think the statement "the UE considers that the UE would transmit the second PUCCH when all values of the HARQ-ACK information are 'ACK' " is not clear. The statement should be updated after agreement on Proposal 2.3.4.1. |
| Samsung2 | To clarify, for multiplexing CSI with HARQ-ACK associated with PDSCH without PDCCH, the PUCCH resource of the CSI is used – same as for NACK-only+CSI (but if the HARQ-ACK was for DG-PUSCH, the PUCCH resource for HARQ-ACK would be used – not possible for NACK-only). The proposal was motivated for maintenance (moderator’s assumption below is correct). @Ericsson: Agree that the description is not complete and update should be expected after relevant agreements. |
| vivo | Based on Samsung’s clarification, we are fine with the TP. For NACK-only+CSI, CSI PUCCH should be used. |
| CATT | We are fine with the TP. |

#### Summary

This is proposal from Samsung and I assumed it was motivated from spec maintenance perspective. Perhaps Samsung can clarify the reasoning of this change? After one more thinking, I guess the reason is: the resources for multiplexing ACK/NACK with CSI as legacy is based on indicated PF3/4 resource. However, there is on PRI indicating PUCCH resources for NACK-only, so when transformed into ACK/NACK multiplexing with CSI, then the multiplexing can follow the same approach of multiplexing CSI and HARQ-ACK information that is in response to PDSCH reception without corresponding PDCCH, i.e., based on resources for CSI.

### (ACTIVE)Round-2

The proposal is not changed and is assumed to be agreeable possibly after Samsung’s clarification.

#### Proposal 3.1.2

Adopt the following text proposal to TS38.213 v17.1.0:

- Reason for change

  - How UE multiplexes NACK-only HARQ-ACK feedback for multicast with only CSI reports is undefined from the latest version of TS 38.213.

- Summary of change

  - Adding a sentence in clause 18 that “If the first PUCCH includes only CSI reports, the UE multiplexes the HARQ-ACK information as described in clause 9.2.5.2 for HARQ-ACK information that is in response to PDSCH reception without corresponding PDCCH”.

- Consequences if not approved

  - UE behavior about multiplexing NACK-only HARQ-ACK feedback for multicast with CSI for unicast is unclear.

------------------------------Text proposal to TS38.213 v17.1.0 Starts--------------------------------- # 18 Multicast Broadcast Services < Unchanged parts are omitted > If a UE would multiplex HARQ-ACK information that the UE would provide according to the second HARQ-ACK reporting mode with other UCI in a first PUCCH, or in a PUSCH, as described in clauses 9 and 9.2.5, the UE provides the HARQ-ACK information according to the first HARQ-ACK reporting mode. For resolving an overlapping among a second PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode and other PUCCHs or PUSCHs prior to multiplexing the HARQ-ACK information in a PUCCH or PUSCH, the UE considers that the UE would transmit the second PUCCH when all values of the HARQ-ACK information are 'ACK'. If the first PUCCH includes only CSI reports, the UE multiplexes the HARQ-ACK information as described in clause 9.2.5.2 for HARQ-ACK information that is in response to PDSCH reception without corresponding PDCCH. If a UE is provided multiple G-RNTIs or G-CS-RNTIs, a configuration for a HARQ-ACK codebook type applies to all G-RNTIs or G-CS-RNTIs. < Unchanged parts are omitted > --------------------------------Text proposal to TS38.213 v17.1.0 Ends--------------------------------

#### Company views

Companies can comment on the summary of the TP and the TP itself.

| Company | Comments |
| --- | --- |
| Moderator | Companies are encouraged to check proponent’s clarification in Round-1 and further confirm whether this TP is agreeable. |
| Samsung | OK |
| vivo | OK |

## (ACTIVE)TP#2 (issue1-13)

Original text proposal from [18] is about the corrections of supporting more than four TBs with NACK-only feedback on the same PUCCH.

### (CLOSED)Round-1

#### Proposal 3.2.1

Adopt the following text proposal to TS38.213 v17.1.0:

- Reason for change

  - The latest version of TS38.213 V17.1.0 does not reflect the agreement correctly regarding how to support more than one TB with NACK-only feedback for multicast. In particular, when UE is configured with moreThanOneNackOnly-Mode indicating the HARQ-ACK information is transmitted according to the first HARQ-ACK report mode, the number of TBs with NACK-only can be any number not smaller than one and otherwise the number should be no more than four.

- Summary of change

  - Clarifying that when a number of HARQ-ACK information bits is more than four, the UE does not transmit the PUCCH that would include HARQ-ACK information unless UE is configured with moreThanOneNackOnly-Mode indicating the HARQ-ACK information is transmitted according to the first HARQ-ACK report mode.

- Consequences if not approved

  - NACK-only HARQ-ACK feedback for more than four TBs is not supported when UE is configured to transmit the PUCCH according to the first HARQ-ACK report mode.

--------------------------------Text proposal to TS38.213 v17.1.0 Starts---------------------------------------- # 18 Multicast Broadcast Services < Unchanged parts are omitted > For the second HARQ-ACK reporting mode, the UE does not transmit a PUCCH that would include only HARQ-ACK information with ACK values. The second HARQ-ACK reporting mode is not applicable for the first SPS PDSCH reception after activation of SPS PDSCH receptions for a SPS configuration, or for DCI formats having associated HARQ-ACK information without scheduling a PDSCH reception. For the second HARQ-ACK reporting mode, when a number of HARQ-ACK information bits is one, a UE transmits a PUCCH only when the HARQ-ACK information bit has NACK value. For a PUCCH resource associated with PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining m0 as described for HARQ-ACK information in clause 9.2.3 and by setting mcs=0. For a PUCCH resource associated with PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting b0=0. For the second HARQ-ACK reporting mode, when a number of HARQ-ACK information bits is more than one, the UE can be indicated by moreThanOneNackOnly-Mode to provide the HARQ-ACK information bits in a PUCCH either according to the first HARQ-ACK reporting mode or by selecting a resource from a set of resources for the PUCCH transmission based on the values of the HARQ-ACK information bits as described in Table 18-1. When a number of HARQ-ACK information bits is more than four, the UE does not transmit the PUCCH that would include HARQ-ACK information unless UE is configured with moreThanOneNackOnly-Mode indicating the HARQ-ACK information is transmitted according to the first HARQ-ACK report mode. If a UE is provided pucch-Config-Multicast1 or pucch-Config-Multicast2 for PUCCH transmissions with a priority value, the UE transmits a PUCCH with the priority value according to pucch-Config-Multicast1 or pucch-Config-Multicast2 for each G-RNTI or G-CS-RNTI that the UE provides associated HARQ-ACK < Unchanged parts are omitted > --------------------------------Text proposal to TS38.213 v17.1.0 Ends-----------------------------------------

#### Company views

Companies can comment on the summary of the TP and the TP itself.

| Company | Comments |
| --- | --- |
| Qualcomm | In RAN2 spec, moreThanOneNackOnlyMode-r17 ENUMERATED {mode1,mode2} OPTIONAL -- Need S moreThanOneNackOnlyMode-r17 Indicates the mode of supporting more than one NACK-only feedback in the same PUCCH transmission. Mode 1 is UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits. Mode 2 is UE transmitting a specific sequence or a PUCCH transmission corresponding the combination more than one NACK-only HARQ feedback. If absent, UE uses the mode2 for mulicast CFR if configured. Here, in RAN1 spec, ‘when a number of HARQ-ACK information is 2, 3, or 4’ is not needed, based on RAN2 specification. Also, in case of more than 4 for mode2, we think it can be an error case and avoided by gNB. For the second HARQ-ACK reporting mode, the UE can be indicated by moreThanOneNackOnlyMode to provide the HARQ-ACK information bits in a PUCCH either according to the first HARQ-ACK reporting mode or by selecting a resource from a set of resources for the PUCCH transmission based on the values of the HARQ-ACK information bits as described in Table 18-1. When the number of HARQ-ACK information bits is more than four, the UE does not expect to be configured with moreThanOneNackOnlyMode as ‘mode2’. |
| Samsung | Support with a simplification as below. When a number of HARQ-ACK information bits is more than four, the UE does not transmit the PUCCH that would include HARQ-ACK information if the unless UE is not provided configured with moreThanOneNackOnly-Mode = ‘mode1’indicating the HARQ-ACK information is transmitted according to the first HARQ-ACK report mode. |
| LG Electronics | We are fine with the TP. We are also fine with Samsung’s revision. |
| Lenovo | Agree with Samsung’s update. |
| vivo | For the case of more than 4 for mode2, we think it is error case, and can be avoided by gNB. But with this TP, gNB can have such scheduling and UE would not feedback HARQ-ACK information. |
| NTT DOCOMO | We are fine with Samsung’s revision. |
| Ericsson | OK. It would be clearer if we could use the two values taken by the moreThanOneNackOnly-Mode IE in RRC. |
| CATT | OK |

#### Summary

Based on the comments, for UE configured with mode2 (PUCCH selection), how to handle the case of more than 4 TBs, companies all think it is an error case and is supposed to not to happen, but how to reflect this point in spec, there are two views: - Approach1: the UE does not transmit the PUCCH when it happens - Samsung, LGE, Lenovo, DOCOMO - Approach2: if this case happen, UE is not expected to be configured with mode2. - Qualcomm, vivo Approach1 is more friendly to NW and UE behavior is clear if the case happens. Suggest taking Approach1

### (CLOSED)Round-2

#### Proposal 3.2.2

Adopt the following text proposal to TS38.213 v17.1.0:

- Reason for change

  - The latest version of TS38.213 V17.1.0 does not reflect the agreement correctly regarding how to support more than one TB with NACK-only feedback for multicast. In particular, when UE is configured with moreThanOneNackOnly-Mode indicating the HARQ-ACK information is transmitted according to the first HARQ-ACK report mode, the number of TBs with NACK-only can be any number not smaller than one and otherwise the number should be no more than four.

- Summary of change

  - Clarifying that when a number of HARQ-ACK information bits is more than four, the UE does not transmit the PUCCH if UE is not provided moreThanOneNackOnly-Mode = ‘mode1’.

- Consequences if not approved

  - NACK-only HARQ-ACK feedback for more than four TBs is not supported when UE is configured to transmit the PUCCH according to the first HARQ-ACK report mode.

--------------------------------Text proposal to TS38.213 v17.1.0 Starts---------------------------- # 18 Multicast Broadcast Services < Unchanged parts are omitted > For the second HARQ-ACK reporting mode, the UE does not transmit a PUCCH that would include only HARQ-ACK information with ACK values. The second HARQ-ACK reporting mode is not applicable for the first SPS PDSCH reception after activation of SPS PDSCH receptions for a SPS configuration, or for DCI formats having associated HARQ-ACK information without scheduling a PDSCH reception. For the second HARQ-ACK reporting mode, when a number of HARQ-ACK information bits is one, a UE transmits a PUCCH only when the HARQ-ACK information bit has NACK value. For a PUCCH resource associated with PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining m0 as described for HARQ-ACK information in clause 9.2.3 and by setting mcs=0. For a PUCCH resource associated with PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting b0=0. For the second HARQ-ACK reporting mode, the UE can be indicated by moreThanOneNackOnly-Mode to provide the HARQ-ACK information bits in a PUCCH either according to the first HARQ-ACK reporting mode or by selecting a resource from a set of resources for the PUCCH transmission based on the values of the HARQ-ACK information bits as described in Table 18-1. When a number of HARQ-ACK information bits is more than four, the UE does not transmit the PUCCH if UE is not provided moreThanOneNackOnly-Mode = ‘mode1’. If a UE is provided pucch-Config-Multicast1 or pucch-Config-Multicast2 for PUCCH transmissions with a priority value, the UE transmits a PUCCH with the priority value according to pucch-Config-Multicast1 or pucch-Config-Multicast2 for each G-RNTI or G-CS-RNTI that the UE provides associated HARQ-ACK < Unchanged parts are omitted > --------------------------------Text proposal to TS38.213 v17.1.0 Ends-----------------------------

#### Company views

Companies can comment on the summary of the TP and the TP itself.

| Company | Comments |
| --- | --- |
| Samsung | OK |
| vivo | We prefer to treat the case that a number of HARQ-ACK information bits is more than four for mode 2 as error case. QC’s version in the first round is fine to us. |
| LG Electronics | OK |
| CATT | OK |
| Qualcomm | It does not make sense to us that gNB configure mode 2 when there are more than 4 TBs, which is an error case obviously. |

#### Summary

The sentence “When a number of HARQ-ACK information bits is more than four, the UE does not transmit the PUCCH if UE is not provided moreThanOneNackOnly-Mode = ‘mode1’.” is controversial. Except this sentence, the other two places of changes seem still valid.

### (ACTIVE)Round-3

#### Proposal 3.2.3

Adopt the following text proposal to TS38.213 v17.1.0:

- Reason for change

  - The latest version of TS38.213 V17.1.0 does not reflect the agreement correctly regarding how to support more than one TB with NACK-only feedback for multicast. In particular, when UE is configured with moreThanOneNackOnly-Mode indicating the HARQ-ACK information is transmitted according to the first HARQ-ACK report mode, the number of TBs with NACK-only can be any number not smaller than one and otherwise the number should be no more than four.

- Summary of change

  - Deleting “when a number of HARQ-ACK information bits is more than four, or” and “when a number of HARQ-ACK information bits is 2, 3, or 4 more than one,”.

- Consequences if not approved

  - NACK-only HARQ-ACK feedback for more than four TBs is not supported when UE is configured to transmit the PUCCH according to the first HARQ-ACK report mode.

--------------------------------Text proposal to TS38.213 v17.1.0 Starts---------------------------- # 18 Multicast Broadcast Services < Unchanged parts are omitted > For the second HARQ-ACK reporting mode, the UE does not transmit a PUCCH that would include only HARQ-ACK information with ACK values. The second HARQ-ACK reporting mode is not applicable for the first SPS PDSCH reception after activation of SPS PDSCH receptions for a SPS configuration, or for DCI formats having associated HARQ-ACK information without scheduling a PDSCH reception. For the second HARQ-ACK reporting mode, when a number of HARQ-ACK information bits is one, a UE transmits a PUCCH only when the HARQ-ACK information bit has NACK value. For a PUCCH resource associated with PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining m0 as described for HARQ-ACK information in clause 9.2.3 and by setting mcs=0. For a PUCCH resource associated with PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting b0=0. For the second HARQ-ACK reporting mode, the UE can be indicated by moreThanOneNackOnly-Mode to provide the HARQ-ACK information bits in a PUCCH either according to the first HARQ-ACK reporting mode or by selecting a resource from a set of resources for the PUCCH transmission based on the values of the HARQ-ACK information bits as described in Table 18-1. If a UE is provided pucch-Config-Multicast1 or pucch-Config-Multicast2 for PUCCH transmissions with a priority value, the UE transmits a PUCCH with the priority value according to pucch-Config-Multicast1 or pucch-Config-Multicast2 for each G-RNTI or G-CS-RNTI that the UE provides associated HARQ-ACK < Unchanged parts are omitted > --------------------------------Text proposal to TS38.213 v17.1.0 Ends-----------------------------

#### Company views

Companies can comment on the summary of the TP and the TP itself.

| Company | Comments |
| --- | --- |
| Samsung | OK |
| vivo | One question for clarification, with this TP, does it mean that for the second HARQ-ACK reporting mode, when a number of HARQ-ACK information bits is more than 4, UE behaviour is not specified? |
| LG Electronics | We have the same question with Vivo. With this change, we wonder if the second HARQ-ACK reporting mode is applicable for more than four NACK-only. |

## (Stable for Set#2)TP#3 (issue1-13)

Original text proposal from [18] is to clarify what “other UCI” means in current specification for the operation of multiplexing NACK-only. This TP can be compatible with TP#1 with the understanding that TP#3 is including multiplexing CSI in general but TP#1 further clarifies how to multiplex NACK-only with CSI only reports in details.

### (CLOSED)Round-1

#### Proposal 3.3.1

Adopt the following text proposal to TS38.213 v17.1.0:

- Reason for change

  - Specification should be clear and accurate regarding the meaning of “other UCI” for UE multiplexing with NACK-only for multicast.

- Summary of change

  - Clarify that “other UCI” includes HARQ-ACK information associated with multicast configured with the first HARQACK reporting mode, HARQ-ACK information associated with unicast DCI formats or CSI reports.

- Consequences if not approved

  - UE behavior may not be determinable for multiplexing NACK-only if the meaning of “other UCI” is unclear from specification.

--------------------------------Text proposal to TS38.213 v17.1.0 Starts--------------------------- # 18 Multicast Broadcast Services < Unchanged parts are omitted > If a UE would multiplex HARQ-ACK information that the UE would provide according to the second HARQ-ACK reporting mode with HARQ-ACK information associated with multicast configured with the first HARQ-ACK reporting mode, HARQ-ACK information associated with unicast DCI formats or CSI reports in a first PUCCH, or in a PUSCH, as described in clauses 9 and 9.2.5, the UE provides the HARQ-ACK information according to the first HARQ-ACK reporting mode. For resolving an overlapping among a second PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode and other PUCCHs or PUSCHs prior to multiplexing the HARQ-ACK information in a PUCCH or PUSCH, the UE considers that the UE would transmit the second PUCCH when all values of the HARQ-ACK information are 'ACK'. If a UE is provided multiple G-RNTIs or G-CS-RNTIs, a configuration for a HARQ-ACK codebook type applies to all G-RNTIs or G-CS-RNTIs. < Unchanged parts are omitted > --------------------------------Text proposal to TS38.213 v17.1.0 Ends-----------------------------

#### Company views

Companies can comment on the summary of the TP and the TP itself.

| Company | Comments |
| --- | --- |
| Qualcomm | Not necessary, but it’s ok if majority companies prefer the clarification. |
| Samsung | The TP is not necessary as the applicable multiplexing procedures are described. However, OK with the TP if a majority prefers. |
| LG Electronics | We support the TP in order to avoid misunderstanding about multiplexing. |
| vivo | Agree with the principle. Other UCI may also include SR which should be excluded since this case has not been agreed. |
| NTT DOCOMO | Support. The current description could be interpreted to include multiplexing with SR, but that has not been agreed yet. We think this clarification is needed. |
| Ericsson | Support |
| CATT | OK |

## (CLOSED)TP#4 (issue1-14)

This is a proposal (though not directly text proposal) to correct the fallback operation for Type-1 HARQ-ACK codebook should not only be constrained to the case of PCell because multicast can be transmitted on SCell as well although not simultaneously transmitted on PCell.

### (CLOSED)Round-1

#### Proposal 3.4.1

Adopt the following text proposal to TS38.213 v17.1.0:

- Reason for change

  - There is no difference between the PCell and an SCell for multicast PDSCH in terms of the fallback operation for Type-1 codebook. When multicast is configured on SCell only, the fallback operation for Type-1 codebook should be supported as well.

- Summary of change

  - Deleting “on the PCell” from the conditions for fallback operation for Type-1 codebook for multicast scheduled by DCI format 4_1.

- Consequences if not approved

  - Fallback operation for Type-1 codebook for multicast when multicast is configured on SCell only is not supported.

--------------------------------Text proposal to TS38.213 v17.1.0 Starts---------------------------- ### 9.1.2 Type-1 HARQ-ACK codebook determination < Unchanged parts are omitted > If a UE reports HARQ-ACK information in a PUCCH only for - a SPS PDSCH release indicated by DCI format 1_0 or by DCI format 4_1, with counter DAI field value of 1, or - a PDSCH reception scheduled by DCI format 1_0 or by DCI format 4_1 having enabled associated HARQ-ACK information report as described in clause 18, with counter DAI field value of 1, or - SPS PDSCH receptions associated with a CS-RNTI or with G-CS-RNTIs having enabled associated HARQ-ACK information reports as described in clause 18 < Unchanged parts are omitted > --------------------------------Text proposal to TS38.213 v17.1.0 Ends----------------------------

#### Company views

Companies can comment on the summary of the TP and the TP itself.

| Company | Comments |
| --- | --- |
| Qualcomm | We don’t support the TP. The legacy fallback behavior for DCI format 1_0 is only applied to PCell, similar for multicast DCI format 4_1. |
| Samsung | Support. There is no legacy fallback behavior when receptions are only on an SCell (never on PCell) as it is possible for multicast. The initial agreement was to “extend fallback operation to multicast”. Without the TP, there is no fallback for multicast when it is on an SCell – a UE will always report the full Type-1 CB. The reason of “PCell” for unicast is because it is not likely to have a reception only on an SCell without receiving anything on the PCell. For multicast, that can be by default via configuration for receptions only on an SCell. |
| Lenovo | OK |
| vivo | ok |
| NTT DOCOMO | Conditions for application of legacy fallback operation should not be changed. We suggest the following updates. - a PDSCH reception scheduled by DCI format 1_0 or by DCI format 4_1 having enabled associated HARQ-ACK information report as described in clause 18, with counter DAI field value of 1, or on the PCell, or - a PDSCH reception scheduled by DCI format 4_1 having enabled associated HARQ-ACK information report as described in clause 18, with counter DAI field value of 1, or |
| Ericsson | Support |
| CATT | OK |
| Qualcomm | For multicast on SCell, we think it is not likely to have a reception only multicast on an SCell without receiving anything for unicast on the PCell. So, we prefer to keep similar as Type-1 codebook fallback on PCell only. |

#### Summary

There is no consensus on this TP. Qualcomm views that a reception only multicast on an SCell without receiving anything for unicast on the PCell is not likely to happen. The legacy procesure should be kept. FL suggests closing this issue with no consensus on the TP.

## (Need more input)TP#5 (issue1-2)

This is a text proposal related to PUCCH resource determination when HARQ-ACK for multicast multiplexed with the HARQ-ACK for unicast.

### (CLOSED)Round-1

#### Proposal 3.5.1

Adopt the following text proposal to TS38.213 v17.1.0:

    - Reason for change:

      - The current TS38.213 v17.1.0 does not clarify the determined PUCCH resource for multiplexed HARQ-ACK of unicast and multicast is provided by PUCCH-config for unicast or PUCCH-config-Multicast.

    - Summary of change:

      - Clarify the determined PUCCH resource for multiplexed HARQ-ACK of unicast and multicast is provided by PUCCH-config for unicast.

    - Consequence if not approved:

      - It is not clear that the determined PUCCH resource is provided by PUCCH-config for unicast or PUCCH-config-Multicast, especially the names of the parameters included in PUCCH-config-Multicast are exactly the same as those included in PUCCH-config for unicast.

--------------------------------Text proposal to TS38.213 v17.1.0 Starts------------------------------------------ 18 Multicast Broadcast Services < Unchanged parts are omitted > A UE determines a PUCCH resource for a PUCCH transmission with HARQ-ACK information as described in clauses 9.2 and 9.2.1 through 9.2.5. If the UE multiplexes HARQ-ACK information associated with unicast DCI formats and HARQ-ACK information associated with multicast DCI formats in a same PUCCH, the last DCI format that the UE uses to determine the PUCCH resource, as described in clause 9.2.3, is a last unicast DCI format. Values of a set of PUCCH resource indexes that the PUCCH resource indicator field are mapped to, as defined in Table 9.2.3-2 for a PUCCH resource indicator field of 3 bits, are provided by resourceList in PUCCH-ResourceSet configured in PUCCH-config for unicast. Values of a set of PUCCH resource indexes that the PUCCH resource indicator field are mapped to, as defined in Table 9.2.3-2 for a PUCCH resource indicator field of 3 bits, are provided by resourceList in PUCCH-ResourceSet configured in PUCCH-config for unicast. < Unchanged parts are omitted > --------------------------------Text proposal to TS38.213 v17.1.0 Ends-------------------------------------------

#### Company views

Companies can comment on the summary of the TP and the TP itself.

| Company | Comments |
| --- | --- |
| Qualcomm | ok |
| Samsung | The TP is not necessary and it seems incorrect as the PRI in the unicast DCI is not mandated to have 3 bits. |
| Lenovo | Not needed. |
| vivo | Ok in principle. But the TP seems with duplicated sentences of “Values of a set of PUCCH resource indexes that the PUCCH resource indicator field are mapped to, as defined in Table 9.2.3-2 for a PUCCH resource indicator field of 3 bits, are provided by resourceList in PUCCH-ResourceSet configured in PUCCH-config for unicast.” |
| Ericsson | Support |

#### Summary

To respond Samsung’s comment: - The 3 bits is saying the table 9.2.3-2 Mapping of PUCCH resource indication field values to a PUCCH resource in a PUCCH resource set with maximum 8 PUCCH resources. Actually, the similar sentence has been used in TS38.213. Agree with vivo, the change needed is mistakenly duplicated, updated in Round-2.

### (ACTIVE)Round-2

#### Proposal 3.5.2

Adopt the following text proposal to TS38.213 v17.1.0:

    - Reason for change:

      - The current TS38.213 v17.1.0 does not clarify the determined PUCCH resource for multiplexed HARQ-ACK of unicast and multicast is provided by PUCCH-config for unicast or PUCCH-config-Multicast.

    - Summary of change:

      - Clarify the determined PUCCH resource for multiplexed HARQ-ACK of unicast and multicast is provided by PUCCH-config for unicast.

    - Consequence if not approved:

      - It is not clear that the determined PUCCH resource is provided by PUCCH-config for unicast or PUCCH-config-Multicast, especially the names of the parameters included in PUCCH-config-Multicast are exactly the same as those included in PUCCH-config for unicast.

--------------------------------Text proposal to TS38.213 v17.1.0 Starts-------------------------------- 18 Multicast Broadcast Services < Unchanged parts are omitted > A UE determines a PUCCH resource for a PUCCH transmission with HARQ-ACK information as described in clauses 9.2 and 9.2.1 through 9.2.5. If the UE multiplexes HARQ-ACK information associated with unicast DCI formats and HARQ-ACK information associated with multicast DCI formats in a same PUCCH, the last DCI format that the UE uses to determine the PUCCH resource, as described in clause 9.2.3, is a last unicast DCI format. Values of a set of PUCCH resource indexes that the PUCCH resource indicator field are mapped to, as defined in Table 9.2.3-2 for a PUCCH resource indicator field of 3 bits, are provided by resourceList in PUCCH-ResourceSet configured in PUCCH-config for unicast. < Unchanged parts are omitted > --------------------------------Text proposal to TS38.213 v17.1.0 Ends---------------------------------

#### Company views

Companies can comment on the summary of the TP and the TP itself.

| Company | Comments |
| --- | --- |
| Samsung | We think the TP is unnecessary. It is clear that the resource indicated by a unicast DCI format is from PUCCH-config for unicast. |
| vivo | We agree with the intension of the TP to clarify the determined PUCCH resource for multiplexed HARQ-ACK of unicast and multicast is provided by PUCCH-config for unicast. It would be clear. We are fine with the TP in general. |
| CATT | Share same view with Samsung. |
| LG Electronics | We are fine with the TP in general. |

# (CLOSED)Set#1 proposal for GTW on May.11

#### Proposal 2.1.1-1-r1

For the 2-bit UL-DAI for multicast indicating the total number of Type-2 HARQ-ACK bits multiplexed in PUSCH, UE generates the Type-2 HARQ-ACK sub-codebook for each G-RNTI per the UL-DAI and concatenates all the sub-codebooks in the ascending order of G-RNTI value.

- If the 2-bit UL-DAI field for multicast has the value of VT-DAIUL=4 and the UE has not received any PDCCH within the monitoring occasions with DCI format 4_1/4_2, the UE does not multiplex HARQ-ACK information for multicast feedback in the PUSCH transmission.

#### Proposal 2.1.1-2-r1

When NACK-only feedback is to be transformed to ACK/NACK and multiplexed in PUSCH, the 2-bit UL-DAI for multicast is applied for all configured G-RNTIs.

#### Proposal 2.1.1-3

When UE is configured with multiple cells for unicast and Type-1 codebook is configured for both multicast and unicast,

- if the UE is configured to construct the HARQ-ACK codebooks for unicast and multicast jointly, a single UL DAI bit applies for unicast and multicast;

- Otherwise, 1 additional bit UL DAI is included for multicast in DCI format 0_1/0_2, in addition to the UL DAI for unicast. The 1-bit UL DAI for multicast is applied to all configured G-RNTIs.

#### Proposal 2.2.1

For a UE configured with NACK-only HARQ-ACK feedback, when NACK-only HARQ-ACK bits are transformed into ACK/NACK HARQ-ACK bits, the PUCCH resource used for transmitting the multiplexed HARQ-ACK bits is determined from PUCCH-Config/PUCCH-ConfigurationList configured for unicast based on the k1 and the PRI indication in the last DCI scheduling multicast.

#### Proposal 2.3.1.1-r1

For UE configured with NACK-only HARQ-ACK feedback, for Alt4 (from previous agreement), define the mapping between HARQ-ACK values and the PUCCH resources for the TBs configured with NACK-only as follows:

- The mapping is applied to both a single G-RNTI and multiple configured G-RNTIs cases.

- The TBs are ordered according to the sequence in which they are received.

- FFS whether/how PRI is used for indication.

| HARQ-ACK Value |  |  |  | PUCCH resource |
| --- | --- | --- | --- | --- |
| {0} | {0,0} | {0,0,0} | {0,0,0,0} | 1st PUCCH resource provided by pucch-ResourceId obtained from the 1st value of resourceList |
|  | {1,0} | {1,0,0} | {1,0,0,0} | 2nd PUCCH resource provided by pucch-ResourceId obtained from the 2nd value of resourceList |
|  | {0,1} | {0,1,0} | {0,1,0,0} | 3rd PUCCH resource provided by pucch-ResourceId obtained from the 3rd value of resourceList |
|  |  | {1,1,0} | {1,1,0,0} | 4th PUCCH resource provided by pucch-ResourceId obtained from the 4th value of resourceList |
|  |  | {0,0,1} | {0,0,1,0} | 5th PUCCH resource provided by pucch-ResourceId obtained from the 5th value of resourceList |
|  |  | {1,0,1} | {1,0,1,0} | 6th PUCCH resource provided by pucch-ResourceId obtained from the 6th value of resourceList |
|  |  | {0,1,1} | {0,1,1,0} | 7th PUCCH resource provided by pucch-ResourceId obtained from the 7th value of resourceList |
|  |  |  | {1,1,1,0} | 8th PUCCH resource provided by pucch-ResourceId obtained from the 8th value of resourceList |
|  |  |  | {0,0,0,1} | 9th PUCCH resource provided by pucch-ResourceId obtained from the 9th value of resourceList |
|  |  |  | {1,0,0,1} | 10th PUCCH resource provided by pucch-ResourceId obtained from the 10th value of resourceList |
|  |  |  | {0,1,0,1} | 11th PUCCH resource provided by pucch-ResourceId obtained from the 11th value of resourceList |
|  |  |  | {1,1,0,1} | 12th PUCCH resource provided by pucch-ResourceId obtained from the 12th value of resourceList |
|  |  |  | {0,0,1,1} | 13th PUCCH resource provided by pucch-ResourceId obtained from the 13th value of resourceList |
|  |  |  | {1,0,1,1} | 14th PUCCH resource provided by pucch-ResourceId obtained from the 14th value of resourceList |
|  |  |  | {0,1,1,1} | 15th PUCCH resource provided by pucch-ResourceId obtained from the 15th value of resourceList |

#### Proposal 2.3.5.1-1

For power determination of a PUCCH transmission for the “NACK-only” reporting mode in clause 7.2.1 in TS 38.213, ∆UCIi=0.

#### Proposal 2.3.5.1-2

For power determination of a PUCCH transmission with multicast HARQ-ACK mode in clause 7.2.1 in TS 38.213, a UE with two CLPC adjustment states applies the one corresponding to l=0.

# (ACTIVE)Set#2 stable proposal for email approval

#### Proposal 3.3.1

Adopt the following text proposal to TS38.213 v17.1.0:

- Reason for change

  - Specification should be clear and accurate regarding the meaning of “other UCI” for UE multiplexing with NACK-only for multicast.

- Summary of change

  - Clarify that “other UCI” includes HARQ-ACK information associated with multicast configured with the first HARQACK reporting mode, HARQ-ACK information associated with unicast DCI formats or CSI reports.

- Consequences if not approved

  - UE behavior may not be determinable for multiplexing NACK-only if the meaning of “other UCI” is unclear from specification.

--------------------------------Text proposal to TS38.213 v17.1.0 Starts--------------------------- # 18 Multicast Broadcast Services < Unchanged parts are omitted > If a UE would multiplex HARQ-ACK information that the UE would provide according to the second HARQ-ACK reporting mode with HARQ-ACK information associated with multicast configured with the first HARQ-ACK reporting mode, HARQ-ACK information associated with unicast DCI formats or CSI reports in a first PUCCH, or in a PUSCH, as described in clauses 9 and 9.2.5, the UE provides the HARQ-ACK information according to the first HARQ-ACK reporting mode. For resolving an overlapping among a second PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode and other PUCCHs or PUSCHs prior to multiplexing the HARQ-ACK information in a PUCCH or PUSCH, the UE considers that the UE would transmit the second PUCCH when all values of the HARQ-ACK information are 'ACK'. If a UE is provided multiple G-RNTIs or G-CS-RNTIs, a configuration for a HARQ-ACK codebook type applies to all G-RNTIs or G-CS-RNTIs. < Unchanged parts are omitted > --------------------------------Text proposal to TS38.213 v17.1.0 Ends-----------------------------

# Set#3 proposal for GTW on May.16

#### Proposal 2.4.2

For Type-1 codebook generation, if at least one configured G-RNTI is with HARQ-ACK enabled, down-select between the following:

- Alt1: UE will report NACK for the G-RNTI with HARQ-ACK disabled regardless of decoding results of the corresponding PDSCH.

- Alt2: UE will report ACK/NACK for the G-RNTI with HARQ-ACK disabled per the decoding result of corresponding PDSCH.

- FFS whether to differentiate whether the G-RNTI with HARQ-ACK enabled is detected or not.

- FFS the PUCCH resource for the codebook transmission

  - Opt1: PUCCH resource/slot is based on last DCI with enabled or disabled feedback

  - Opt2: PUCCH resource/slot is based on last DCI with enabled feedback

#### Proposal 2.3.2.3-1 (Alt1)

For addressing how to count and order the HARQ-ACK bits for NACK-only for Alt4: down-select the supported cases and options from the following:

- Case 1: when the HARQ-ACK-to-PUCCH mapping table for Alt4 (from previous agreement) is applied to a single G-RNTI,

    - Opt1-1: based on C-DAI included in DL DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits. (Apple, Nokia, NSB, Lenovo, Samsung, DOCOMO, ZTE, Qualcomm)

    - Opt1-2: based on the number of PUCCH resources in the PUCCH resource set configured for NACK-only to count the number of HARQ-ACK bits: the number of HARQ-ACK bits=log2(the number of PUCCH resource+1), i.e., 15 PUCCH resources correspond to 4 TBs, and based on the C-DAI included in DL DCI for ordering the HARQ-ACK bits. (CATT)

- Case 2: for the case of all UEs configured with the same set of G-RNTIs

- Opt2-1: support Alt4 for this case

  - Opt2-1-1: based on the number of PUCCH resources in the PUCCH resource set configured for NACK-only to count the number of HARQ-ACK bits, i.e., the number of HARQ-ACK bits=log2(the number of PUCCH resource+1), 15 PUCCH resources correspond to 4 TBs, and based on Opt2-1-2 or Opt2-1-3 for ordering the HARQ-ACK bits (CATT)

  - Opt2-1-2: based on the sum of C-DAI included in the last scheduling DCI from each G-RNTI for counting the total number of HARQ-ACK bits. The ordering of HARQ-ACK bits is per C-DAI from each G-RNTI and in the ascending order of G-RNTI values. (Apple, Nokia, NSB, DOCOMO, ZTE, Qualcomm)

  - Opt2-1-3: based on C-DAI* included in the last scheduling DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits. The C-DAI* is accumulating the scheduled TBs across different G-RNTIs (Lenovo, Ericsson, Samsung)

- Opt2-2: does not support Alt4 for this case. (Apple)

- Case 3: for the case of different UEs configured with different G-RNTIs

- Opt3-1: support Alt4 for this case

  - Opt3-1-1: based on the number of PUCCH resources in the PUCCH resource set configured for NACK-only to count the number of HARQ-ACK bits, i.e., the number of HARQ-ACK bits=log2(the number of PUCCH resource+1), 15 PUCCH resources correspond to 4 TBs, and based on Opt3-1-2 or Opt2-1-3 for ordering the HARQ-ACK bits (CATT)

  - Opt3-1-2: based on the sum of C-DAI included in the last scheduling DCI from each G-RNTI for counting the total number of HARQ-ACK bits. The ordering of HARQ-ACK bits is per C-DAI from each G-RNTI and in the ascending order of G-RNTI values. (Apple with 2 G-RNTIs, Nokia, NSB, DOCOMO,)

  - Opt3-1-3: based on C-DAI* included in the last scheduling DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits. The C-DAI* is accumulating the scheduled TBs across different G-RNTIs. (Lenovo, , Ericsson, ZTE)

- Opt3-2: does not support Alt4 for this case. (Samsung, Qualcomm)

#### Proposal 2.2.3-r1

- For the case when the PUCCH transmission for the NACK-only based feedback for one G-RNTI collides with the PUCCH transmission for ACK/NACK feedback for another G-RNTI with the same priority and then UE multiplexes the NACK-only based feedback with the ACK/NACK feedback onto the same PUCCH by transforming NACK-only into the ACK/NACK based HARQ-ACK bit, the PUCCH resources for transmitting the multiplexed HARQ-ACK bits is from the PUCCH-Config/PUCCH-ConfigurationList configured for multicast with ACK/NACK based feedback.

- For a UE configured with G-RNTI(s) with NACK-only HARQ-ACK feedback, when NACK-only HARQ-ACK bits are transformed into ACK/NACK HARQ-ACK bits, the PUCCH resource used for transmitting the multiplexed HARQ-ACK bits is determined from PUCCH-Config/PUCCH-ConfigurationList configured for multicast based on the k1 and the PRI indication in the last DCI scheduling multicast. If PUCCH-Config/PUCCH-ConfigurationList configured for multicast is not configured, the PUCCH-Config/PUCCH-ConfigurationList configured for unicast is used.

- FFS: the case NACK-only is for multicast SPS PDSCH only.

# References

R1-2203070	Mechanisms to improve reliability for RRC_CONNECTED UEs	Huawei, HiSilicon, CBN

R1-2203194	Maintenance of reliability improvement for MBS	ZTE

R1-2203227	Open issues on reliability for NR MBS	TD Tech Ltd

R1-2203287	Remaining Issues on Reliability Improvements for RRC_CONNECTED UEs supporting MBS	Nokia, Nokia Shanghai Bell

R1-2203314	Discussion on the remaining issues on mechanisms to improve MBS reliability for RRC_CONNECTED UEs	Spreadtrum Communications

R1-2203427	Remaining issues on reliability improvement mechanism for RRC_CONNECTED UEs in MBS	CATT

R1-2203526	Remaining issues on mechanisms to improve reliability for RRC_CONNECTED UEs	vivo

R1-2203613	Remaining issues on mechanisms to improve reliability for MBS	ETRI

R1-2203677	Remaining Issues on Reliability Improvements for RRC_CONNECTED UEs	NEC

R1-2203699	Remaining issues on reliability improvement for RRC-CONNECTED UEs	Lenovo

R1-2203838	Remaining issues on mechanisms to improve reliability for RRC_CONNECTED UEs	Langbo

R1-2203874	Maintenance on mechanisms to improve reliability	Samsung

R1-2203974	Discussion on remaining issues of mechanism to improve reliability for RRC_CONNECTED UEs	OPPO

R1-2204216	Remaining issues on MBS reliability improvement for RRC_connected UEs	Apple

R1-2204282	Maintenance on mechanisms to improve reliability for RRC_CONNECTED UEs	CMCC

R1-2204354	Remaining issues on HARQ-ACK feedback procedure for multicast	NTT DOCOMO, INC.

R1-2204502	Remaining reliability issues of MBS for RRC_CONNECTED UEs	Google Inc.

R1-2204622	Mechanisms to improve reliability of Broadcast/Multicast service	LG Electronics

R1-2204717	Remaining issues on improve multicast reliability for RRC_CONNECTED UEs	MediaTek Inc.

R1-2204945	Remaining issues for improvement of reliability of NR MBS	Ericsson

R1-2204994	Mechanisms to improve reliability for RRC_CONNECTED UEs	Qualcomm Incorporated