3GPP TSG RAN WG1 #109-e                                                                             R1-2205160

e-Meeting, May 9th – 20th, 2022

# Source:             CATT

Title:                  Evaluation results of network energy saving

Agenda Item:    9.7.3

Document for:  Discussion and Decision

Introduction

In RAN#95 e-meeting, network energy saving study item was agreed [1].

1. Definition of a network energy consumption model [RAN1] - Adapt the framework of the power consumption modelling and evaluation methodology of TR38.840 to the network side, including relative energy consumption for DL and UL (considering factors like PA efficiency, number of TxRU, network load, etc), sleep states and the associated transition times, and one or more reference parameters/configurations. 2. Definition of an evaluation methodology and KPIs [RAN1] - The evaluation methodology should target for evaluating system-level network energy consumption and energy savings gains, as well as assessing/balancing impact to network and user performance (e.g. spectral efficiency, capacity, UPT, latency), energy efficiency, and UE power consumption/complexity. The evaluation methodology should not focus on a single KPI, and should reuse existing KPIs whenever applicable; where existing KPIs are found to be insufficient new KPIs may be developed as needed. 3. Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception [RAN1, RAN2, RAN3, [RAN4]]. Note: Other techniques are not precluded. The study should prioritize idle/empty and low/medium load scenarios (the exact definition of such loads is left to the study), and different loads among carriers and neighbor cells are allowed. The following example scenarios (mapping between scenarios and network loads is left to the study) including single-carrier and multi-carrier deployments are used as the starting point for discussion on prioritized scenarios for the study. The following example scenarios are listed in no particular order. - Urban micro in FR1, including TDD massive MIMO (note: this scenario can also model small cells) - FR2 beam-based scenarios (note: this scenario can also model small cells) - Urban/Rural macro in FR1 with/without DSS (no impact to LTE expected in case of DSS) - EN-DC/NR-DC macro with FDD PCell and TDD/Massive MIMO on higher FR1/FR2 frequency Note 1: legacy UEs should be able to continue accessing a network implementing Rel-18 network energy savings techniques, with the possible exception of techniques developed specifically for greenfield deployments. Note 2: the study of energy savings specifically for IAB is not part of the scope.

In this contribution, the evaluation methodology, system configuration and power model are proposed with details in [2].    Some network energy saving schemes are introduced in section 2. The evaluation results are showed in the section 3.

Evaluation Assumptions

The evaluation methodology of network energy saving includes simulation methodology, the deployment simulation scenario, simulation assumption of system configuration, traffic model and general network operation procedure. System-level simulation should be considered for the network energy saving performance evaluation to evaluate network energy consumption and energy saving gains. The evaluation methodology should target for the evaluation of network energy consumption and energy saving gains of network energy saving techniques comparing to the baseline network configuration and operation. The simulation assumptions are included in Table 1. The evaluation methodology and KPIs are discussed with details in [2].

Table 1:  Simulation assumptions for network energy saving schemes

| Parameter | Configuration |
| --- | --- |
| Deployment | Urban Macro cell with 46dBm Tx power. |
| Numerology | 30 kHz |
| Carrier frequency | 3.5GHz |
| Duplex mode | TDD |
| System Bandwidth | 100 MHz |
| gNB antenna configuration | 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) (dH, dV) = (0.5λ, 0.5λ) |
| UE antenna configuration | 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |
| Channel estimation | Non-ideal |
| Transmission scheme | MU-MIMO |
| Traffic model | FTP3, 0.5Mbytes per packet |
| System Load | Zero system load: RU<5%; Light system load: RU <20% Medium system load: RU <50% High system load: RU >50% |

Furthermore, the proposed network energy saving schemes are described in Table 2.

Table 2:  Network energy saving schemes for evaluation

| Network energy saving schemes | Explanation |
| --- | --- |
| Baseline (No power saving scheme, micro sleep is the only sleep state) | - Network sleeping state: Only SSB transmission for UE detection and network access. |
|  | - Network active state: DL broadcast and control channels, and CSI-RS transmission. |
| Case1: SSB/RMSI transmission reduction | SSB with longer cycle, e.g.{20,40,80,160,320,640}ms SSB with less number of beams L=2,4,8,16,32,64 |
| Case2: gNB DTX scheme | gNB transmits PDCCH/PDSCH and reference signals, e.g. SSB, CSI-RS during DTX-ON; gNB goes to sleep during DTX-OFF [3]. |

Evaluation Results

We have proposed some network energy saving schemes with preliminary results for initial evaluation of network energy study, which includes SSB/RMSI transmission reduction and gNB DTX scheme.

- 3.1 Evaluation Baseline

We could classify the network energy consumption into two categories, which are network sleeping state and network active state as shown in Figures 1 and 2. Only SSB and RMSI are transmitted in the same period e.g. 20ms. SSB is transmitted for UE detection and network access, RMSI is transmitted for delivering the essential system information.

Figure 1:  Network in sleeping state

The normal network operation is when network in active state to transmit reference signals, control and data channels.   SSB and RMSI would be transmitted for UE detection and network access. CSI-RS is configured for CSI measurements and report for link adaptation. SSB and CSI-RS are assumed to be transmitted with the same period, e.g. 20ms. PDCCH and PDSCH are transmitted when the data is scheduled to transmit.

Figure 2:  Network in active state

The detailed gNB energy consumption model is shown in [2]. The procedure of network in sleeping and active state are considered as the baseline to evaluate the energy saving gains of network energy saving schemes in the followed section 3.2~3.3.  The KPIs of UPT and latency are also considered to evaluate the impact to user performance.

- 3.2 SSB transmission reduction scheme

The network in sleeping state can turn OFF some units in gNB (light/deep sleep) or set in standby (micro sleep) to reduce energy consumption. Network only transmits SSB and RMSI at the transmit cycle. In this section, the network energy saving gain with different SSB periods and different number of beams for SSB beam sweeping under FR1 and FR2 are evaluated respectively.  The detailed configuration of FR1 and FR2 is shown in Table 3.

Table 3:  Configuration of SSB transmission reduction scheme

| Case | Parameters |
| --- | --- |
| 1-1 | SSB with period varied 20ms, 40ms, 80ms, 160ms 320ms, 640ms L=2,4,8 SCS = 30KHz, FR1 |
| 1-2 | SSB with period varied 20ms, 40ms, 80ms, 160ms 320ms, 640ms L=8,16,32,64 SCS = 120KHz, FR2 |

The energy saving gain under different SSB periods and number of SSB beams for FR1 and FR2 are shown in Figure 3~ Figure 7.

Figure 3:  Energy saving gain under different SSB periods, L=2

Figure 4: Energy saving gain under different SSB periods, L=4

Figure 5: Energy saving gain under different SSB periods, L=8

The energy saving gain of SSB with L = 2, 4 ,8 under different SSB periods are illustrated in Figure 3 ~  Figure 5,  respectively. It can be observed that:

- 2/4/8 beams of SSB transmission with longer period can obtain up to about 80% energy saving gain.

- With the period of SSB up to 640ms, the energy saving gain from the longer SSB period reaches saturation state.

Observation 1: The longer SSB period can bring higher network energy saving gain with the SSB period up to 640ms.

Figure 6: Relative energy saving gain with different number of SSB beams（baseline:2beams）,  FR1

Figure 7: Relative energy saving gain with different number of SSB beams（baseline:8beams）,  FR2

The relative energy saving of SSB beams reduction from 8 beams to 2 beams (FR1) , 64 beams to 8 beams(FR2) under different SSB periods are illustrated in Figure 6 and  Figure 7,  respectively. It can be observed that:

- The larger number of SSB beams, the larger network energy saving gain can be obtained by longer SSB period. It could be up to about 4% for FR1.

- The larger number of SSB beams, the larger network energy saving gain can be obtained by longer SSB period. It could be up to about 35% for FR2.

Observation 2: The larger number of SSB beams (FR1&FR2), the more network energy saving gain can be obtained by longer SSB period, especially for FR2.

- 3.3 gNB DTX scheme

Similar to DRX mechanism for UE power saving, the network operation in active state can also reduce energy consumption based on discontinuous transmission by allocated the SSB, CSI-RS, scheduled PDCCH/PDSCH  transmission within a range of slots and no transmission at the remaining of slots within the cycle when the system load is low.  gNB transmits PDCCH/PDSCH and reference signals, e.g. SSB, CSI-RS only during DTX-ON and goes to sleep during DTX-OFF.  Furthermore, the ON period of DTX of network aligns with that of the DRX of the serving UEs [3]. The network is in DTX-ON as long as one of the serving UE is under DRX-ON, the network turns to DTX-OFF only when all serving UEs are in DRX-OFF. The detail evaluation cases of DTX are shown in Table 4.

Table 4: Evaluation cases of DTX

| Case | Parameters |
| --- | --- |
| 2-1 | RU<10% gNB is configured by DTX=(320,10,80) ,(100,10,10), (80,10,10)ms FTP3:0.5Mbytes Packet，200ms inter-arrival time |
| 2-2 | 10%<RU<20% gNB is configured by DTX=(160,10,10) ,(100,10,10), (80,10,10)ms FTP3: 0.5Mbytes Packet，100ms inter-arrival time |

The energy saving gain and the user performance for case 2-1 and case 2-2 are shown in Table 5.

Table 5: Simulation results for case 2-1 and case 2-2 under Urban Macro

| DTX config | （320,10,80） | （160,10,10） | (100,10,10) |  | (80,10,10) |  |
| --- | --- | --- | --- | --- | --- | --- |
| Case | 2-1 | 2-2 | 2-2 | 2-1 | 2-2 | 2-1 |
| RU level | 7% | 15% | 14% | 7% | 15% | 9% |
| Average energy saving gain | 46.1% | 39.3% | 47.0% | 53.2% | 44.1% | 48.8% |
| 5% UPT loss | 0.56% | 1.8% | 1.1% | 1.1% | 2.8% | 2.1% |
| 50% UPT loss | 12.3% | 1.3% | 2.4% | 2.2% | 9.1% | 5.7% |
| 95% UPT loss | 0% | 0% | 0% | 0% | 0% | 0% |
| 5% Latency loss | 0% | 0% | 0% | 0% | 0% | 0% |
| 50% Latency loss | 13.3% | 0% | 2.4% | 2.3% | 10.0% | 5.4% |
| 95% Latency loss | 0.37% | 0.72% | 1.1% | 1.1% | 2.9% | 2.7% |

Based on the above results, it could be observed that:

- When the system RU is lower than 10%, 46.1 %, 53.2% and 48.8% network energy saving gain can be achieved under DTX configured with (320,10,80) ms, (100,10,10) ms and (80,10,10) ms, respectively. The UPT and latency loss are about 10% at most.

- When the system RU is about 15%, 39.3 %, 47% and 44.1% network energy saving gain can be achieved under DTX configured with (160,10,10) ms, (100,10,10) ms and (80,10,10) ms, respectively. The UPT and latency loss are about 10% at most.

Observation 3: For network in active state, time domain energy saving mechanism of gNB DTX could reduce network power consumption.

Conclusion

In this contribution, it discussed the evaluation assumption and provided the preliminary results.  We have the following observations.

Observation 1: The longer SSB period can bring higher network energy saving gain with the SSB period up to 640ms.

Observation 2: The larger number of SSB beams (FR1&FR2), the more network energy saving gain can be obtained by longer SSB period, especially for FR2.

Observation 3: For network in active state, time domain energy saving mechanism of gNB DTX could reduce network energy consumption.

References

- RP-220297, “Revised SI: Study on network energy savings for NR”, Huawei.

- R1-2203481, “Evaluation Methodology and Power Model for Network Energy Saving”, CATT.

- R1-2203482, “Network Energy Saving techniques in time, frequency, and spatial domain”, CATT