3GPP TSG RAN WG1 #109-e							    R1-2205181

e-Meeting, 09 – 20 May, 2022

Source:	Moderator (OPPO)

Title:	FL summary #2 for AI 9.4.1.1: SL-U channel access mechanism

Agenda item:	9.4.1.1

Document for:	Discussion and Decision

Introduction

In the latest version of Rel-18 WID for NR sidelink evolution, the objective for supporting NR sidelink operation in unlicensed spectrum is captured as followed.

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

This contribution provides a summary of submitted contributions, email discussion topics and outcomes that are related to the channel access mechanisms for SL-U (blue text part of objective) during this RAN1 meeting. Note that, all past outcomes including agreements, conclusions and working assumptions reached during this WI are captured in Section 5 (Appendix) of this document.

Collection of all agreements / outcomes of RAN1#109-e

To be collected once agreement is reached.

Contact information

Since this meeting is conducted electronically, it has been requested to collect contact information of the delegate(s) who handle this topic from each company during this meeting. You are kindly requested to provide the following contact information.

| Company | Delegate name(s) | Email address(es) |
| --- | --- | --- |
| InterDigital | Aata El Hamss | aata.elhamss@interdigital.com |
| Intel | Salvatore Talarico | salvatore.talarico@intel.com |
| LG Electronics | Daesung Hwang Seungmin Lee | daesung.hwang@lge.com edison.lee@lge.com |
| OPPO | Kevin Lin Zhenshan ZHAO | Kevin.lin@oppo.com zhaozhenshan@oppo.com |
| Futurewei | Guosen Yue | gyue@futurewei.com |
| Qualcomm | Giovanni Chisci Stelios Stefanatos | gchisci@qti.qualcomm.com sstefana@qti.qualcomm.com |
| Panasonic | Ayako Iwata | iwata.ayako@jp.panasonic.com |
| CMCC | Pengyu JI Jingwen ZHANG | jipengyu@chinamobile.com zhangjingwen@chinamobile.com |
| ZTE,Sanechips | Yuzhou Hu | hu.yuzhou@zte.com.cn |
| Sharp | Luochao | chao.luo@cn.sharp-world.com |
| CableLabs | Dorin Viorel | d.viorel@cablelabs.com |
| xiaomi | Wensu Zhao Qun Zhao | zhaowensu@xiaomi.com zhaoqun1@xiaomi.com |
| Lenovo | Karthikeyan Ganesan Alexander Golitschek Haipeng Lei | kganesan@lenovo.com aelbwart@lenovo.com leihp1@lenovo.com |
| NTT DOCOMO | Shohei Yoshioka | shohei.yoshioka@docomo-lab.com |
| Spreadtrum | Mimi Chen | mimi.chen@unisoc.com |
| Vivo | Huan Wang Zichao Ji | wanghuan@vivo.com jizichao@vivo.com |
| Fujitsu | Jian Zhang | zhangjian1288@fujitsu.com |
| CATT, GOHIGH | Rui Zhao Shupeng Li Xiaoran Wen | zhaorui@goghigh.com.cn lsp@catt.cn wenxiaoran@gohigh.com.cn |
| Sony | Kazuyuki Shimezawa | kazuyuki.shimezawa@sony.com |
| Nokia | Timo Lunttila Torsten Wildschek | timo.lunttila@nokia.com Torsten.wildschek@nokia.com |
| Nokia Shanghai Bell | Naizheng Zheng | naizheng.zheng@nokia-sbell.com |
| Fraunhofer | Sarun Selvanesan | sarun.selvanesan@hhi.fraunhofer.de |
| Transsion | Xingya Shen | xingya.shen@transsion.com |
| Ericsson | Ricardo Blasco Jose Leon Calvo | name.surname at company . com jose.leon.calvo@ericsson.com |
| NEC | Jin Yang Zhaobang Miao | yangjin@labs.nec.cn miao_zhaobang@nec.cn |
| MediaTek | Tao CHEN Junqiang CHENG | Tao.chen@mediatek.com Junqiang.cheng@mediatek.com |
| HiSilicon | Fan Yang | james.yangfan@huawei.com |
| Huawei | Xiang Mi | shawn.mixiang@huawei.com |
| Apple | Huaning Niu Chunxuan Ye | Huaning_niu@apple.com Chunxuan_ye@apple.com |
| WILUS | Minseok Noh | minseok.noh@wilusgroup.com |
| Bosch | Khaled Hassan | Khaled.hassan@de.bosch.com |

Topics for email discussion

[109-e-R18-SL-01] Email discussion on channel access mechanism by May 20 – Kevin (OPPO)

- Check points: May 16, May 20

## [ACTIVE] Topic #1: Evaluation methodology

Background: According to the objective for SL-U in the WID [36], the work on evaluating performance of NR sidelink operation in unlicensed band is not defined. However, according to existing evaluation methodology defined so far has been primarily focus on V2X applications/use cases with some updates for commercial use cases in Rel-17 for power saving and inter-UE coordination RA.

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

Since the current SL-U work mainly target for NR sidelink operation in the unlicensed spectrum with large available bandwidth for high data rate applications and uncertain access to the unlicensed channel due to interference, further updates to the existing evaluation methodology would seem necessary to evaluate NR sidelink performance in the unlicensed spectrum. But we should keep a principle of updating only the essential parameters and try to leave other details based on existing definitions and company choice.

Question 1: The followings are proposed as basic set of evaluation methodology updates for evaluating NR sidelink performance in the unlicensed spectrum for this WI. Please indicate your preferences and comments (agree, choice of one or more options per topic, different option(s), suggestions).

- Evaluation methodology baseline

  - Option 1: NR-U TR 38.889

  - Option 2: NR sidelink TR 37.885

- Evaluation scenarios / layouts

  - Baseline: Indoor layouts

    - Option 1: NR-U based (R1-2205033)

      - a = 20m, b = 60m, c = 20m, d = 80 m

      - There are two operators to model two RATs at a time. The red one is SL-U, the blue one can be NR-U or Wi-Fi

      - The topology of SL-U is pair topology (1) or optional star topology (2), and the SL-U UEs are dropped uniformly at random in the area

      - For NR-U/Wi-Fi, 5 UEs/STAs associated per each gNB/AP per 20 MHz

      - For SL-U pairs: 3, 5 or 10 pairs of UEs per 20MHz

      - For SL-U stars: 3 Hub UEs with 5 associated peripheral UEs per 20MHz

    - Option 2: SL UE clusters (R1-2203146)

      - Indoor layout and UE dropping model with N = 6 or 12 clusters and each with M=5 UEs

      - Each cluster is a circle, with a central point and radius Rmax = 15 or 10m and Rmin = 5 or 1m

      - No overlapping among the N clusters

  - Optional: Outdoor Macro deployment with ISD = 200m (from TR 38.889)

- Channel model

  - Reuse NR-U channel model for evaluating commercial use cases

  - Reuse channel model from NR sidelink for V2X use cases

- Traffic model

  - Option 1: R17 sidelink commercial traffic model

  - Option 2: XR traffic model TR38.838

  - Option 3: Mixed traffic between XR and FTP model 3

- Interference

  - Option 1: Explicit modelling of WiFi transmissions

  - Option 2: Use statistical interference model

- Performance metrics

  - Baseline: UPT and latency (NR-U)

  - Optional: PRR and PIR (V2X)

| Company | Comments |
| --- | --- |
| InterDigital | For the evaluation methodology baseline, our preference is urban and highway deployment scenarios specified in Option 2. Because our preference is outdoor V2X based deployment scenario. We think outdoor deployment should be prioritized over indoor. For channel mode, both NR-U model and V2X model can be used. For traffic model, Option 1 is preferred. We think that the evaluation should focus on the existing sidelink traffic. For interference, Option 2 is preferred. For performance metric, we think PRR/PIR to be considered as baseline and UPT as optional. |
| Intel | Given that the WID does not include any co-existence study, first of all, we would like to clarify what the intention is in this matter. If the intention is to converge on evaluation methodologies so that companies can provide evaluations at their will, we are fine to discuss this topic. Otherwise, we believe it is out of scope. Also, given that we expect much of the channel access procedure defined from NR-U (e.g, channel access type and modes) to be reused with minor changes, as indicated by the WID, and given that UEs are quite power limited compared to gNBs, we do not think any evaluations toward co-existence are needed. With that said, we believe that evaluation methodologies in line with SL use cases should be considered, including V2X deployments, channel and traffic models. As for interference modelling, and as mentioned above, we do not think any evaluation toward co-existence is really needed, and therefore we do not see the need to explicitly model a wi-fi module as in NR-U. As for performance metrics, this may depend on what should be evaluated and PRR, PIR and UPT could be all considered. |
| LGE | According to NR-U indoor layout, the locations of red points are fixed. So, the distance between different APs or gNB is also fixed. On the other hand, in case of the modified one, we may need to further consider whether or how to define minimum distance between AP/gNB and SL UE. Without this restriction, the performance degradation would be overestimated. Regarding the traffic model, mixed traffic needs to be deprioritized. As we know, evaluating different situations in the same time will make analysis more difficult. |
| OPPO | For the evaluation methodology baseline, Option 1 is preferred. To introduce another RAT as the interference model, it is the straightforward way to reuse NR-U evaluation methodology as the baseline. For evaluation layout, indoor layout Option 1 is preferred. For channel mode, both NR-U model and V2X model can be used. For traffic model, Option 1 is preferred. SL-U work is aim to expand the applicability of NR sidelink to commercial use cases, so we think SL-U evaluation should focus on the R17 commercial traffic model. For interference model, Option 1 is preferred. Although this WID does not include any co-existence evaluation study, it is necessary to introduce the interference model to simulate a realistic scenario where other RATs compete with SL-U for access to the channel. For performance metric, we think PRR/PIR and UPT/latency can be used for V2X periodic/aperiodic traffic model and FTP model respectively, this should depend on which traffic model is selected in simulation. |
| Futurewei | The WID on SL-evo does not include the study of the evaluation methodology for SL-U. Also per WID, channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation. The NR-U channel access mechanisms have been evaluated and studied. We do not see the need of any new mechanisms which require simulation evaluations. Therefore, we are not clear on the necessity of studying the evaluation methodology. If the WG agrees to study evaluation methodology for SL-U, we suggest include one of V2x scenarios (urban or highway) from TR 37.885 for outdoor and/or only one indoor scenario from TR 38.889. To avoid exponentially increasing number of simulation settings, we prefer to limit only one option on traffic model and interference model. |
| Qualcomm | - Evaluation methodology baseline: We do prefer Option 1 (NR-U TR 38.889). TR 37.885 was introduced to provide an evaluation methodology for NR-based V2X use cases. Use cases like, platooning, extended sensors, advanced and remote driving, have stringent requirements in terms of reliability, latency, range, and speed to support different degrees of V2X automation (TR 22.886). SL-U should be focusing (per the WID) on high throughput commercial applications over unlicensed spectrum (e.g. peer-to-peer gaming and infotainment), where the uncertainty of LBT, the presence of other RATs, and potentially lower-grade devices are defining factors. We believe that TR 38.889 is the natural baseline for the objective since it was defined for taking all these factors into account. - Evaluation scenarios/layouts: We agree in having indoor as baseline and optional outdoor. For indoor, we do prefer Option 1 (NR-U based, R1-2205033). Firstly, we think that evaluating SL-U in the presence of another RAT will be fundamental to discover its limitations and design/specify enhancements, therefore we advocate for a two operators configuration. Secondly, for the dropping model, we believe that the differences between the proposed options are subtle but important. We think, in fact, that “pairs” could be a good compromise between simplicity and modelling capability. Nevertheless, it would be interesting keeping around more complex scenarios (like stars) to explore more coordination. In that regard, we think that the differences between the “stars” in R1-2205033 and “clusters” in R1-2203146 are mainly: a) allowing overlaps vs. not allowing overlaps, and b) associating UEs via RSRP thresholding after random drop vs. being random distance. For (a) we think that there is no reason to impose non-overlapping clusters, since there should be no restriction on the location of UEs location in the room (e.g. think of a gaming scenario where students are playing in separate groups from different rooms in a dorm); moreover imposing non-overlapping clusters may neglect important interference and collision challenges. For (b), while we believe that both techniques may be acceptable, we do prefer random UE scattering with RSRP association, which reuses the technique provided in TR 36.843 (LTE D2D proximity services). - Channel model: We believe that the choices made for NR-U evaluation methodology better represent SL-U use cases, therefore we stand by Option 1 for NR-U channel models used in TR 38.889. - Traffic model: We prefer Option 3, and we propose to focus on FTP3, with keeping XR as optional, to better reflect the high throughput eMBB-like use cases as peer-to-peer gaming, IoE-based social networking, and in-car infotainment, as well as keeping the evaluation methodology as lean as possible. - Interference: We prefer Option 1, where we suggest to focus on the essential aspects of WiFi modelling, since we advocate for the WiFi system to be included as an “interferer to SL-U”, and not studied per-se. This would help focusing on the relevant aspects for SL-U design/specification. Some aspects to be modelled in WiFi would be: access for frame duration, Cat4 LBT, asynchronous (μs level) access, transmission and reception, feedbacks. - Performance metrics: We believe that UPT and latency will be fundamental to evaluate SL-U (follow NR-U). We believe that PRR and PIR will not be necessary to evaluate SL-U, since they are defined based on the requirements for V2X applications over licensed, which are far apart from the objectives of SL-U. While we see that reworking those metrics to better match the objectives of SL-U could, in principle, be a possibility, we propose to avoid this step for the sake of a leaner evaluation methodology, also considering that UPT and latency would be good metrics to evaluate potential issues like starvation and collisions. |
| CMCC | Agree |
| Vivo | For indoor layout, option 1 is preferred. For the outdoor layout, the urban street model is a typical scenario for sidelink commercial use case, e.g., the support of extended sensors for exchanging data among the vehicle or pedestrian, which needs higher data rate or bigger bandwidth. Therefore, the urban street layout defined in TR37.885 should be contained in the evaluation in R18 Sidelink. For channel model, we support the NR-U channel model for evaluation of commercial use case mainly including the indoor scenario, and the channel model of NR sidelink for V2X use case. For traffic model, option1 of R17 sidelink commercial traffic model is supported. For the interference model, option 1 is supported since the effect from sidelink to WIFI system should be evaluated and the explicit modelling of WIFI transmission should be modelled. Regarding the performance metrics, UPT/latency and PRR/PIR are all desirable. |
| ZTE,Sanechips | It is suggested to add the following discussion of simulation use cases. - Simulation use case - Option 1:Evaluate commercial use case only - Option 2: Evaluate both commercial and V2X use cases For simulation use cases, we support the above Option 1. For channel mode, NR-U channel model is reused with the modification of antenna height. For traffic model, we support Option 1. For interference modeling, we prefer Option 2. For performance metrics, it is recommended to evaluate both the UPT and packet loss rate, for example, ftp files that do not complete transmission for a long time will lead to the loss of whole package. |
| CableLabs | Before proceeding with any details concerning the Evaluation Methodology, a couple of preliminary steps should be under-taken: - A Co-existence Study targeting i) 5GHz coexistence between SL LTE, LAA LTE and 802.11ac and ii) 6GHz coexistence study between SL NR, NR-U and 802.11ax - Define a proper Fairness Coexistence Criteria (e.g. One SL-U node shall not degrade the performance of another coexistent node (e.g.6GHz: NR-U or 802.11ax) more than any other coexistent node employing a similar access technology (e.g. 6GHz: NR-U or 802.11ax) Without addressing these steps, we do not believe an evaluation methodology could deliver meaningful results. |
| Lenovo | We think there is no need to evaluate channel access itself, as the WID requires the re-use of NR-U mechanisms which have been fully evaluated for coexistence. We understand the WID such that the following may require evaluation: - "Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation". For such evaluations, we do not think that detailed interference models are required, so Option 2 (statistical interference model) seems sufficient; anyway the current Option 1 only talks about modelling Wi-Fi in detail, even though the premise of the layout seems to allow NR-U (i.e. Uu) as well. We don't see a need to evaluate Wi-Fi interferers with a higher priority than NR-U interferers, so a statistical model looks like a good approach. |
| NTT DOCOMO | Motivation of SL-U would be commercial use case mainly and V2X optionally. From this reason, basically NR-U-based evaluations would be more appropriate than V2X-based evaluations. - Evaluation methodology baseline: Option 1 - Evaluation scenarios/layouts: Option 1 - Channel model: OK with both - Traffic model: Option 1 - Interference: Option 1 (we are not sure option 2 can evaluate sufficiently) - Performance metrics: OK with both |
| CATT/GOHIGH | First of all, we think the target use case of SL-U should be clarified. Considering the traffic characteristics of V2X, we don’t think V2X service ca be supported in SL-U and prefer to focus on infotainment services. Therefore, we think the evaluation methodology of NR-U (Option 1) can be a starting point. For the evaluation layout, both indoor layout Option 1 and outdoor macro deployment with ISD = 200m can be supported. For channel model, NR InH mixed Office model defined in TR38.901 and NR-U channel model for evaluating commercial use cases should be used for indoor scenario and outdoor scenario, respectively. For traffic model, an option should be added for FTP model 3. And we support option 2 and the newly added option 4. - Option 4: FTP model 3 For the interference modelling, we think the key motivation is to better evaluate the performance of SL-U instead of deploying a new system. In this case, a statistical model for other RAT interference is preferred. Meanwhile, if it is difficult to implement the statistical model, single-system interference modelling can also be considered. For performance metrics, UPT and latency used in NR-U can be reused for FTP model traffic. For XR traffic, successful UE ratio is the preferred performance metric. |
| Samsung | As study on evaluation methodology is not explicitly captured by WID, we prefer not spending much time on discussing evaluation methodology. A basic model set with limited number of options is preferred by us, and we think the evaluation work in NR-U should be reused as much as possible. For evaluation methodology baseline and indoor layouts, we prefer option 1. For channel model, since the two models are for different use cases, we’re fine with both. For traffic model, option 1 is preferred. For interference, option 1 is more accurate, we be can also accept option 2 for simplicity. For performance metrics, all metrics are beneficial, and we prefer to use PRR/PIR as basic metric and UPT/latency as optional. |
| Nokia, NSB | Regarding possible co-existence studies, we agree with Intel that those are out of scope. Companies are of course free to provide results at their own will, but since we’ve agreed to re-use NR-U channel access principles, fair co-existence will naturally be guaranteed. We think it is important also to consider NR sidelink V2x scenario (TR 37.885), since information sharing between vehicles can still take advantage of unlicensed spectrum. V2x evaluation can be based on selected scenarios such as Highway, where it is not expected much interference issues with other RATs. Nevertheless, we are also ok with NR-U based assumptions (TR 38.889) for indoor Option 1. NR-U channel access should be in place in either scenario to ensure coexistence. Regarding traffic model we prefer Option 1 and Option 2 optionally. WiFi interference should be optional and, if present, should follow TR 38.889. |
| Ericsson | Regarding this proposal, we have the following comments: - Regarding the baseline methodology: - Our preference is to take the SL TR (37.885) and make any additions that are necessary. - What in 38.889 would we consider if that is “the baseline” ? - Indoor layout option 1 is an example of something to take from 38.889: - Interference can be SL-U or WiFi. - Outdoor deployment is not necessary. - Channel model for indoor (NR InH Mixed Office model) from TR 38.889. - Traffic mode. Our preference is Option 1, but a scaling per 20 MHz is necessary. A factor of 5 is good as explained in R1-2204741 - Interference - Option 1 is good, but it depends on the choice of technology (SL-U or WiFi) - Performance metrics: proposal is fine |
| Mediatek | - For evaluation methodology baseline, Option 1 is preferred because we do think for SL operated on the unlicensed spectrum, the commercial use cases and some industry use cases rather than V2X use cases can be prioritized for study. - For evaluation scenarios / layouts, the indoor scenario is preferred, and the outdoor Macro deployment with ISD = 200m can be optional. For indoor scenario, we do prefer Option 1 with a pair topology of SL-U. For the star topology of SL-U, the traffic direction should be clarified first. - For channel model, the NR-U channel model can be used. - For traffic model, Option 3 is preferred but the term of “mixed” should be clarified first. Does it mean simulating both FTP3 and XR traffic at the same time? We think the best effort traffic type such as FTP3 traffic can be used for evaluation of the basic service. XR traffic as the advanced IoT service characterized by the large/varied packet size and the short latency is also recommended for evaluation to verify the upper bound capability of SL-U, which is important for the commercial use cases and the industry use cases. - For the interference, Option 2 is preferred (or introduce NR-U system as the second system considering inter-RAT/Inter-operator operation). We think the study of the coexistence between SL-U and WIFI is not necessary since NR-U inherited by SL-U has been proved for the performance of the coexistence with WIFI based on the extensive study. Thus, for performance evaluation we can focus on the target scenarios to just evaluate SL-U performance and feasibility. There is also no need to compare the performance between SL-U and WIFI since it is out of the scope of this WID. - For the performance metrics, the UPT and latency (NR-U) is preferred because we think the high throughput and low latency offered by SL-U may provide the possibility to support some advanced IoT traffic (e.g., XR traffic). The PRR used for V2X evaluation may not be suitable for SL-U evaluation anymore since it is not for the evaluation of V2X scenario. |
| Huawei, HiSilicon | - Evaluation methodology baseline: Option 1 is preferred, given that 37.885 mainly targets for V2X uses cases, although Rel-17 updates to support commercial use cases, the modification is minor, and is still mainly targeted for outdoor scenario, such as urban and highways. - Evaluation scenarios / layouts: for Indoor layouts, support Option 2, the simulation of coexistence with other RATs is not required in the WID, and NR-U has already simulated coexistence, given that SL-U channel access reusing that of NR-U, e.g. Type 1 and Type 2, no need to additional assess co-existence. For UE dropping model, the cluster model seems to be more appropriate, and close to practical deployment, e.g. devices are connected to a smart phone. Outdoor Macro deployment can be considered, as optional. - Channel model: Both models can be supported - Traffic model: We support to use XR traffic model for SL-U evaluation, since the XR service is critical for Rel-18 SL-U. FTP3 is the one used for NR-U, and it can be reused in Rel-18 SL-U. In addition, a UE may consider to support multiple types of traffic, and a mixed traffic model can be also supported. So we suggest to have following changes (in red) on the proposal : Traffic model - Option 1: R17 sidelink commercial traffic model - Option 2: XR traffic model TR38.838 - Option 3: Mixed traffic between XR and FTP model 3 - Mixed traffic of above is allowed, companies can report the ratio of each traffic type. - Interference: support Option2, there is no need to simulate the coexistence with WiFi, and in order to focus more on the enhancement of SL-U, statistical interference model is enough. - Performance metrics: we think PRR is the most important performance metric for sidelink, which should be included for SL-U EM. However, the legacy PRR in Rel-16 is specified for V2X service in ITS band, and cannot reflect traffic in unlicensed band. For traffic in SL-U, due to the failure of LBT, the data packet may not be sent due to exceeding PDB. Thus, the definition of PRR needs to be modified, considering the packet may be not sent when exceeding PDB. PRR is enough to measure the reliability performance, so the PIR may be not needed to save evaluation efforts. From evaluation perspective, it is not necessary to make a specific metric as baseline. Because all metrics can be calculated in one simulation, i.e., no additional efforts to consider multiple metrics. So we suggest to have following changes on the performance metrics part (in red): Performance metrics - Baseline: UPT and latency (NR-U) - Optional: PRR and PIR (V2X) - For a packet of which delay exceeding a given PDB, the packet is regarded as transmission failure. |
| Broadcom | We support the position of CableLabs as stated above. As the current proposals clearly have an impact on other technologies operating in unlicensed spectrum we believe that before proceeding with any details concerning the Evaluation Methodology, a couple of preliminary steps should be under-taken: - A Co-existence Study targeting i) 5GHz coexistence between SL LTE, LAA LTE and 802.11ac and ii) 6GHz coexistence study between SL NR, NR-U and 802.11ax - Define a proper Fairness Coexistence Criteria (e.g. One SL-U node shall not degrade the performance of another coexistent node (e.g.6GHz: NR-U or 802.11ax) more than any other coexistent node employing a similar access technology (e.g. 6GHz: NR-U or 802.11ax) Without addressing these steps, we do not believe an evaluation methodology could deliver meaningful results. |
| Apple | Prefer option 1. The design target of the SL-U, as described in WID, is to enable commercial use case with high throughput requirement. Many of those use cases are indoor, where reuse NR-U baseline can be the starting point. |
| WILUS | We prefer option 1 as evaluation methodology baseline for SL-U from the perspective that SL-U would be mainly commercial use case rather than V2X use case. For evaluation scenarios/layouts, we prefer option 1 for indoor layouts. For channel model, we are ok with both. For traffic model and interference modeling, we prefer option 1. For performance metrics, we are ok with both. |
| Bosch | Evaluation baseline: We prefer to keep SL TR (37.885) and make necessary change based on NR-U. We still prefer to consider SL-U for V2X as well as for commercial use cases. For the layout, we prefer Indoor (option 1) and outdoor (option 2, for V2X application) For traffic model, we are fine with all option and we would like to consider Traffic model from TR 37.885 (at least periodic). Performance metrics: PRR and PIR (V2X), UPT and latency (NR-U) |

### Proposals for round 2

Summary of inputs from round 1 inputs:

- Evaluation methodology baseline

  - Option 1: NR-U TR 38.889

    - OPPO (including interference), Qualcomm, ZTE/SC, DOCOMO, CATT/GH, Samsung, MediaTek, HW/HiSi, Apple, WILUS

  - Option 2: NR sidelink TR 37.885

    - InterDigital, Futurewei, Nokia/NSB, Ericsson, Bosch

- Evaluation scenarios / layouts

  - Baseline: Indoor layouts

    - Option 1 (NR-U based)

      - OPPO, Futurewei, Qualcomm (pairs for simplicity), vivo, DOCOMO, CATT/GH, Samsung, Nokia/NSB, Ericsson, MediaTek (pairs), WILUS, Bosch

      - LGE: define minimum distance between AP/gNB and SL UE

    - Option 2 (SL UE clusters)

      - HW/HiSi

  - Optional: Outdoor Macro with ISD = 200m (from TR 38.889)

    - CATT/GH, MediaTek, HW/HiSi

  - Others

    - V2X urban / highway (TR 37.885)

      - InterDigital (urban + highway), Futurewei (urban or highway), vivo (urban), Nokia/NSB (highway, no interference)

- Channel model

  - Reuse NR-U channel model for evaluating commercial use cases

    - OPPO, Qualcomm, vivo, ZTE/SC (update ant. height), DOCOMO, CATT/GH, Samsung, MediaTek, HW/HiSi, WILUS

  - Reuse channel model from NR sidelink for V2X use cases

    - OPPO, vivo, DOCOMO, Samsung, HW/HiSi, WILUS

  - NR InH mixed Office model

    - CATT/GH (TR38.901)

    - Ericsson (TR38.889)

- Traffic model

  - Option 1: R17 sidelink commercial traffic model

    - InterDigital, OPPO, vivo, ZTE/SC, DOCOMO, CATT/GH, Samsung, Nokia/NSB, Ericsson (scaling factor = 5 per 20MHz), HW/HiSi, WILUS

  - Option 2: XR traffic model TR38.838

    - CATT/GH, Nokia/NSB (optional)

  - Option 3: Mixed traffic between XR and FTP model 3

    - Qualcomm/MediaTek (FTP3 based, XR optional),

    - LGE: deprioritized

  - Limit to only one option

    - Futurewei

- Interference

  - Option 1: Explicit modelling of WiFi transmissions (as per TR38.889)

    - OPPO, Qualcomm, vivo, DOCOMO, Samsung, Nokia/NSB, Ericsson, WILUS

  - Option 2: Use statistical interference model

    - ZTE/SC, Lenovo, CATT/GH, MediaTek, HW/HiSi

  - Limit to only one option

    - Futurewei

- Performance metrics

  - UPT and latency (NR-U)

    - InterDigital (UPT optional), Intel (UPT), OPPO (FTP), Qualcomm, vivo, ZTE/SC (packet loss rate), DOCOMO, CATT/GH (FTP3), Samsung (optional), Ericsson (baseline), MediaTek, HW/HiSi, WILUS, Bosch

  - PRR and PIR (V2X)

    - InterDigital (baseline), Intel, OPPO (V2X), vivo, DOCOMO, Samsung (baseline), Ericsson (optional), HW/HiSi (PRR only, modify to consider PDB), WILUS, Bosch

  - Others

    - Packet loss rate: ZTE/SC

    - Successful UE ratio: CATT/GH (XR traffic)

- Coexistence study and define coexistence fairness criteria

  - Define / reuse existing coexistence fairness criteria (e.g., from LAA):

    - CableLabs, Broadcom

  - Evaluation methodology / coexistence study is not mandated (not part of WID scope):

    - Intel, OPPO, Futurewei, Lenovo, Samsung, Nokia/NSB, MediaTek, HW/HiSi

FL comments and proposal:

- Based on the inputs, it is observed that most companies consider the main use case for SL-U is commercial related applications, while some think V2X use cases are still relevant or even more important. And hence, the choice of the evaluation methodology baseline. To clarify, some of the main differences between EM for NR-U and V2X are carrier frequency, BW, channel access schemes (missing in V2X), antenna model, channel model, etc. Since different company have different interests in the use cases, we can try a different approach where company can choose a scenario they wish to simulate according to one of the followings for evaluating SL-U performance.

  - Scenario 1 (commercial use cases)

  - Scenario 2 (V2X use cases)

- Note that, there are as many as 10 companies mentioned that the evaluation methodology work is not part of the objective in the WID. But as mentioned in the background section that it is still necessary to update the existing evaluation methodology / simulation assumptions (either from NR-U or sidelink V2X) in order to evaluation the performance of SL-U due to different UE dropping, use cases and traffic model used in the past. So, as long as the essential/critical assumptions are updated, we should not spend too much time on discussing minor details or precise updates. In additional, we should leave some flexibility in the choice of the scenarios and some parameters to each company to decide. A lot of times, we evaluate the relative performance from one scheme to another within the same setting.

- Although the evaluation methodology work is not included as part of the SL-U objective, there are also proposals from 2 companies to evaluate coexistence performance between SL-U and another RAT (e.g., NR-U, Wi-Fi, LAA). In addition, it is also proposed to define a Fairness Coexistence Criterion to evaluate whether SL-U will degrade the performance of other RAT in the unlicensed band. Regarding these points, some company pointed out that since it is stated in the WID that the existing NR-U channel access mechanisms should be reused, fair coexistence will naturally be guaranteed. Hence, this coexistence evaluation work is not needed. Since the request is made from the two companies, FL proposes for the evaluation of at least the commercial scenario (indoor layout) the interference should be explicitly modelled. For evaluation of the V2X scenario, as commented during the 1st round, it is not expected much interference issues with other RATs, the interference is not modelled.

  - Regarding coexistence performance between SL-U mode 2 and WiFi transmission in a shared carrier, some evaluation results are submitted in R1-2203985, where the performance of WiFi per station is improved when they coexist with SL-U UEs. Details of simulation assumptions and reasonings are explained in the Tdoc.

Proposal 1 (I): The followings, two evaluation scenarios can be used for evaluating performance of SL-U designs, resource allocation schemes, and coexistence study with another RAT (commercial scenario only) in a shared channel.

- Scenario 1 (commercial use cases):

  - Evaluation methodology baseline is NR-U from TR 38.889 with the following updates.

  - Indoor layout is a pairs topology for SL-U from R1-2205033

    - a = 20m, b = 60m, c = 20m, d = 80 m

    - There are two operators to model two RATs at a time. The red one is SL-U UE, the blue one can be NR-U or Wi-Fi

    - The topology of SL-U is pair topology and the SL-U UEs are dropped uniformly at random in the area

    - For NR-U/Wi-Fi, 5 UEs/STAs associated per each gNB/AP per 20 MHz

    - For SL-U pairs: 3, 5 or 10 pairs of UEs per 20MHz

  - Channel model follows NR InH Mixed Office model used in NR-U (TR38.889)

  - Traffic model baseline is R17 sidelink commercial traffic model with periodic model 3 reduced by a factor of 5. Optionally FTP model 3.

  - Interference model: Explicit modelling of WiFi transmissions (as per TR38.889)

  - Performance metric: UPT, latency and fair coexistence criterion used in NR-U TR38.889

- Scenario 2 (V2X use cases):

  - Evaluation methodology baseline is NR sidelink from TR 37.885.

  - Layout: Highway (baseline), urban (optional)

  - Channel model follows NR sidelink TR 37.885

  - Traffic model baseline is R17 sidelink commercial traffic model

  - Interference is not modelled for highway and urban

  - Performance metric: PRR and PIR (V2X)

| Company | Comments |
| --- | --- |
| ZTE,Sanechips | For traffic model, we would like to emphasize FTP model 3 with larger overall packet size should be more appropriate. We can live with both options being listed but we cannot accept having this as optional. We are also fine to have Rel-17 sidelink commercial traffic model with or without periodic model 3 as optional to reduce the simulation burden. The V2X use cases can be accepted but marked with optional. |
| LGE | Since Mode 2 RA can work to avoid other UE’s reserved resource, it is unclear whether or not to consider the evaluation methodology without other RAT. We are OK to put Scenario 2 as optional. |
| vivo | Similar as LGE’s comment, we wonder whether we need a baseline methodology and the other as optional (e.g., scenario 2) |

### Proposal for Week 2 Monday GTW session

Proposal 1 (II): The followings, two evaluation scenarios can be used for evaluating performance of SL-U designs, resource allocation schemes, and coexistence study with another RAT (commercial scenario only) in a shared channel.

- Scenario 1 (commercial use cases) – baseline:

  - Evaluation methodology baseline is NR-U from TR 38.889 with the following updates.

  - Indoor layout is a pairs topology for SL-U from R1-2205033

    - a = 20m, b = 60m, c = 20m, d = 80 m

    - There are two operators to model two RATs at a time. The red one is SL-U UE, the blue one can be NR-U or Wi-Fi

    - The topology of SL-U is pair topology and the SL-U UEs are dropped uniformly at random in the area

    - For NR-U/Wi-Fi, 5 UEs/STAs associated per each gNB/AP per 20 MHz

    - For SL-U pairs: 3, 5 or 10 pairs of UEs per 20MHz

  - Channel model follows NR InH Mixed Office model used in NR-U (TR38.889)

  - Traffic model

    - Option 1: R17 sidelink commercial traffic model with periodic model 3 reduced by a factor of 5

    - Option 2: FTP model 3

    - It is up to each company to use either Option 1 or 2

  - Interference model: Explicit modelling of WiFi transmissions (as per TR38.889)

  - Performance metric: UPT, latency and fair coexistence criterion used in NR-U TR38.889

- Scenario 2 (V2X use cases) – optional:

  - Evaluation methodology baseline is NR sidelink from TR 37.885.

  - Layout: Highway (baseline), urban (optional)

  - Channel model follows NR sidelink TR 37.885

  - Traffic model baseline is R17 sidelink commercial traffic model

  - Interference is not modelled for highway and urban

  - Performance metric: PRR and PIR (V2X)

## [ACTIVE] Topic #2: Channel access mechanisms for SL-U (including SCSt)

Background:

According to TS37.213, as summarized in some contributions, LBE-based dynamic channel access procedures for DL and UL can be summarized as followed (mostly from [2] with thanks).

- DL Type 1 channel access is applicable to any DL transmission initiated by a gNB. DL Type 2 is used for certain transmissions, e.g., transmission initiated by a gNB with discovery burst, or transmissions by a gNB following transmission by a UE after a gap of 25 us/16us in a shared channel occupancy.

- UL Type 1 channel access is applicable to PUSCH/PUCCH/SRS transmissions scheduled or configured by gNB, or any transmission related to random access procedure. UL Type 2 is applicable in the case where a UE receives a signal indicated by a gNB to perform Type 2 LBT.

| Type 1 | Time duration that are sensed to be idle before transmissions is random. A gNB or a UE determines an initial counter N which is randomly selected between 0 and CWp, where CWmin,p≤CWp≤CWmax,p, CWmin,p and CWmax,p are subject to CAPC. N can be decreased when channel is sensed to be idle for a certain period of time. Transmission can only take place when N reaches 0. |
| --- | --- |
| Type 2A | Time duration that are sensed to be idle before transmissions is deterministic. The channel needs to be idle for a sensing interval of 25 µs. |
| Type 2B | Time duration that are sensed to be idle before transmissions is deterministic. The channel needs to be idle equal to or longer than a sensing interval of 16 µs but less than 25 µs. |
| Type 2C | Does not sense the channel before transmission. Time gap to the previous transmission is less than 16 µs. Duration of the corresponding transmission is at most 584 µs. |

For NR sidelink operation in the unlicensed spectrum (SL-U), as stipulated by the WID “Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation”, the LBE-based dynamic channel access schemes (Type 1 and Type 2A/2B/2C) are the most discussed and supported LBT schemes for SL-U to ensure fair coexistence with other RATs operating in the same unlicensed bands and to behave like a “good neighbour”.

According to the summary captured in Section 5.3, just about all companies support dynamic channel access schemes to be used for SL-U, while most of the companies additionally expressed that the FBE-based semi-static channel access scheme is also to be supported (with 5 companies expressed that further study and discussion is needed). From reviewing the Tdocs submitted in this meeting, in addition to which and the timing of Type 1 or Type 2 LBT should be performed for certain SL channels, signals and scenarios, many also expressed that the exact details of CAPC for SL (based on PQI), contention window adjustment based on HARQ feedback (no HARQ, ACK/NACK, NACK-only) and the energy detection threshold adaptation should be further discussed and decided for SL-U.

As brought up by several contributions and during the first GTW session for SL-U, SL UE performing Type 1 LBT in unlicensed channel to obtain channel occupancy time (COT) can often experience the inter-UE blocking issue, where one SL UE’s transmission is blocking other SL UE’s LBT sensing and losing the benefit of FDM multiple UE’s transmission in the same slot in SL. So, it is worth to look into this aspect / issue to find a solution to mitigate this, which is the key advantage and benefit of using SL mode 2 RA.

As pointed out by several contributions, short control signalling transmissions (SCSt) is allowed according to European regulations in the unlicensed band without needing to perform LBT channel access scheme when certain transmission limitations are met. Please refer to Section 5.5 for the summary. While some companies have already indicated their support to introduce this feature for SL-U, some expressed further study is needed on which SL channel(s) and signal(s) are applicable (e.g., PSFCH and S-SSB transmissions).

Proposal 2 (I): Type 1 and Type 2 (2A/2B/2C) dynamic channel access schemes for LBT from NR-U are supported for NR sidelink operation in unlicensed spectrum.

- FFS the timing to perform them based on the type of SL channels and signals transmitted and based on COT sharing conditions (if supported).

  - Including the inter-UE/mutual blocking issue

- FFS any necessary update or enhancement to CAPC table, CW adjustment, energy detection threshold adaptation, etc.

- FFS whether semi-static channel access mode for FBE can be also supported

- FFS whether short control signalling transmission (SCSt) can be supported and for which channel(s)/signal(s), including the feasibility, reliability and benefits

| Company | Comments |
| --- | --- |
| InterDigital | In general, we are fine with the proposal. For the first bullet, what is the meaning of “timing to perform them”? Our understanding for LBE is that there is no timing restriction on when to perform LBT. Is the intention here to study the starting time of transmission of SL channels/signals? If so, we can say: “FFS the transmission start of SL channels and signals based on the LBT type and COT sharing (if supported)” |
| Intel | We are generally fine with the proposal, but we have several comments: - Similarly as dynamic channel access mode, semi-static channel access mode should be also supported for SL-U. While this may not guarantee same performance as dynamic channel access mode, it still ensures fair co-existence and may actually overperform dynamic channel mode in low traffic scenarios or scenarios where incumbent may be ither absent or have less impact. - We have same question/comment as InterDigital regarding the first sub-bullet and the meaning of “timing”. - Also definition of where the measurement windows is applied, which was left up to implementation in NR-U, within an LBT procedure should be further studied considering propagation delays, sync errors and possible other issues that may cause inter-UE collision and mutual blocking, and in this matter an additional FFS should be included. - We do not believe the CAPC tables in 37.213 should be enhanced or updated, since these were defined strictly following the ETSI BRAN, but perhaps one aspect that should be discussed in whether the UL CAPC or DL CAPC or both should be used, and in the later case how to indicate and choose which one to use. - As detailed in our tdoc, another FFS is regarding the relationship between CAPC and PPPPs. In our view, a UE must indicate which CAPC has been used to acquire a COT, especially if it intends to share its COT, and therefore together with PPPP also CAPC is needed. - For the short control signalling exception, another aspect that we believe should be further clarified is on the understanding of the ETSI BRAN text, and how this should be interpreted in terms of the type of LBT to apply, and the requirements needed for a channel to qualify for such exception, for instance whether this exception may apply per device. Therefore we would like to propose the following changes to proposal 1: Both dynamic and semi-static channel access mode are supported. Type 1 and Type 2 (2A/2B/2C) dynamic channel access schemes for LBT from NR-U are supported for NR sidelink operation in unlicensed spectrum. - FFS the timing to perform them based on the type of SL channels and signals transmitted and based on COT sharing conditions (if supported). - Including the inter-UE/mutual blocking issue - FFS whether the timing of the measurement windows within an observation window in the LBT procedure should be changed compared to NR-U procedure. - FFS any necessary update or enhancement to CAPC table, CW adjustment, energy detection threshold adaptation, etc. - FFS whether UL CAPC or DL CAPC or both should be used, and in the later case how to indicate and choose which one to use. - FFS whether semi-static channel access mode for FBE can be also supported - FFS whether short control signalling transmissions (SCSt) can be supported, the requirements to qualify and LBT type needed for this exception, and for which channel(s)/signal(s), including the feasibility, reliability and benefits - FFS whether a relationship between PPPPs and CAPC is needed |
| LGE | We are fine with this direction. On 1st sub-bullet, does “timing to perform them” means applicable scenario of each channel access type or when UE can perform a certain channel access type? Next, for FBE discussion, we think that it would be necessary to discuss when the FBE can be used and this scenario will be feasible. To be specific, since the SL transmission can be done in distributed manner (e.g., not controlled by gNB), it would be necessary that any other links (e.g., DL or UL) does not share the same channel as well as other RAT. Currently, we are not so sure this scenario is feasible or not. One possible case would be that IIoT is realized with SL-U instead of DL or UL in NR-U. In those points of view, we are fine with putting it FFS. According to “Uu operation for mode 1 is limited to licensed spectrum only” in WID, it does not say that gNB cannot perform NR-U for Uu link. The wording itself says that DCI format 3_0 or RRC signalling for mode 1 will be on licensed spectrum only. In this case, if gNB want, gNB can transmit DL transmission and UL reception for Uu link, but not for SL scheduling. This clarification is important, because if gNB cannot know the status of unlicensed band, gNB cannot schedule accurate SL resources. In this case, there is no need to support Mode 1 RA for SL-U. |
| OPPO | We are generally fine with the proposal. For the 1st bullet: we have same view as IDC and Intel. “timing” needs to be clarified. For Type 1 LBT, the timing for start performing LBT is not specified, it is up to UE implementation. For Type 2A/2B LBT, UE should sense at least 25us/16us idle immediately before transmission. When to perform LBT is not specified either (maybe considering different capability of RX/TX switching time). For the 3rd bullet: we think FBE should also be supported for SL-U. It is useful in some scenarios, such as IIoT, that the existing of other RAT is controllable. While we think the study of FBE can be low priority than LBE. |
| Futurewei | We support the proposal. |
| Qualcomm | - We support dynamic channel access to be the priority, and semi-static channel access to be potentially supported as a lower priority. - We support studying further the timing of different types of LBT based on type of SL channels and signals and based on COT sharing conditions. - We support study further enhancements to CAPC table, CW adjustment, EDT adaptation. - We support study further SCSt to determine if/for which signals can be supported. |
| Panasonic | For PSCCH/PSSCH, our original view is that only Type 1 channel access procedure is used. It can work with shorter length of PSSCH is (pre-)configured by sl-LengthSymbols in the resource pool in unlicensed band to use consecutive slots. It has minimum specification impact considering all SL topics to be finalized in R18. As the majority support Type 2 (2A/2B/2C), we are ok to support Type 2 but the specification impact should be minimized. |
| CMCC | We are generally fine with proposal. We also think the meaning of “timing” should be further clarified especially for Type 1 LBT, which is not specified in NR-U; We propose to reuse the design of CAPC table, CW adjustment, energy detection threshold adaptation, etc, in NR-U as much as possible to avoid too much workload, enhancements can only be done when essential issues have been found. |
| Vivo | Regarding FBE vs LBE, we agree with Intel. in the first meeting, there is no need to prioritize either FBE or LBE, let us keep them in the same level, either support both or consider both - Both semi-static channel access and Type 1 and Type 2 (2A/2B/2C) dynamic channel access schemes for LBT from NR-U are supported for NR sidelink operation in unlicensed spectrum. Regarding “FFS the timing to perform them based on… “, in our understanding, we just further study the applicable scenario/case for each of the channel access type. We prefer to revise the wording as following FFS the timing when/how to perform them based on the type of SL channels and signals transmitted and based on COT sharing conditions (if supported). |
| ZTE, Sanechips | Support the proposal in general For the first bullet, if the intention is to discuss the conditions to perform the listed dynamic channel access schemes, we are OK with it. For FBE, we are open to discuss it, but we should focus on LBE with higher priority first. For CAPC table and energy detection threshold adaptation, we think it is better to reuse current definitions or schemes, but whether to use DL or UL parameters in NR-U should be discussed. For SCSt, we understand that the intention is for S-SSB/PSFCH transmission, but we think at least type 2A for the discovery burst of NR-U(including SSB) can be supported for S-SSB/PSFCH transmission, FFS whether SCSt can additionally be supported. For the relationship between PPPPs and CAPC, we are OK to discuss it, but it seems a RAN2 issue. |
| Sharp | We are in general fine with the proposal. We propose to change the first sub-bullet to “FFS the actual channel access type(s) used for each SL channel/signal and whether it is based on COT sharing conditions (if supported)”. And we also support semi-static channel access mode for FBE considering SL-U would be deployed for some use cases for example IIoT where other technologies would be guaranteed to be absent. |
| CableLabs | We support the use of LBT Type 2A, 2B and 2C, CAPC DL per Table 4.1.1-1, UL CAPC (if warranted) per Table 4.2.1-1, DL Transmission Contention Window Adjustment per #4.1.4.2, UL Contention Window adjustment (if justified) per #4.2.2.1, energy detection threshold adaptation, per #4.1.4.3, as specified by NR-U TS37.213 v17.1.0 (2022-03). Any Short Control Signaling should be assimilated with similar NR-U signaling and comply with the related NR-U specifications. We support SL-U FBE operation for environments where Wi-Fi coexistence absence could be guaranteed (e.g. by level of regulation). Any possible deviation from the NR-U specification (TS37.213) should be further identified and discussed, if this will be the case. |
| xiaomi | We are generally fine with the proposal. For the 1st bullet: we have same view as IDC, Intel and oppo. The exact meaning of “timing” needs to be clarified. For the 2nd bullet, we think the UL CAPC tables in 37.213 can be reused, however, how to map sidelink channel and signals to the CAPC value shall be studied. So we make the following revision: - FFS the timing to perform them based on the type of SL channels and signals transmitted and based on COT sharing conditions (if supported). - Including the inter-UE/mutual blocking issue - FFS any necessary update or enhancement to CAPC table, CW adjustment, energy detection threshold adaptation, etc. - FFS how to map sidelink channel/signal to CAPC values. - FFS whether semi-static channel access mode for FBE can be also supported - FFS whether short control signalling transmission (SCSt) can be supported and for which channel(s)/signal(s), including the feasibility, reliability and benefits |
| Lenovo | We support the modified proposal from Intel |
| NTT DOCOMO | Basically fine with the proposal. For CAPC table, condition to use each CAPC would be additional discussion point. For FBE, although we are fine to discuss FBE, LBE should be prioritized in consideration of time limitation. |
| Spreadtrum | We are fine with the proposal. For “timing to perform them” in the 1st bullet, we also think further clarification is needed or we can change it to - FFS which type should be performed for every SL channels and signals transmitted and COT sharing conditions (if supported). For the 2nd bullet, we are fine to further study the enhancement to CAPC table, CW adjustment, energy detection threshold adaptation, etc. For the 3rd bullet, semi-static channel access mode for FBE is also important in SL-U to support IIoT, where the absence of any other technology can be guaranteed. For the 4th bullet, we support the proposal. |
| Fujitsu | We share a similar view with InterDigital. “FFS the timing to perform them…” is not clear in that the timing to perform LBT may not be specified. We support the other parts of the Proposal. |
| CATT/GOHIGH | We are generally fine with the proposal. We think the supportive of semi-static channel access needs further study and should be kept as FFS. |
| Sony | We generally support the proposal. For second sub-bullet, we have similar view with Intel on CAPC enhancement, and we are OK with Intel’s fourth sub-bullet. For third sub-bullet, we are OK with support of the semi-statice channel access, while we think the dynamic channel access should be prioritised. |
| Samsung | We are generally fine with the proposal. We also think the wording “timing” in 1st bullet needs to be clarified. For the FFS bullets, we support to further study the feasibility and details. |
| Nokia, NSB | We support the proposal 2. The FFS points are not strictly speaking necessary, but can be a starting point for the discussion. Naturally the further discussion will not be limited to those points only. |
| Fraunhofer | We are supportive of the proposal, and are fine with further studying the timing aspects of when LBT has to be performed, and whether any enhancements need to be done to the CAPC table and CW adjustments. |
| Transsion | We are fine with the proposal in general. Regarding the first sub-bullet, we share the same concern as some companies that the actual time to perform a type 1 LBT is left to the implementation. Regarding the third sub-bullet, FBE is beneficial for some use cases, such as factory, where other technologies would be guaranteed to be absent. Therefore, we prefer to support semi-static channel access mode for FBE in SL-U. Regarding the last sub-bullet, although in some regions, short control signalling is allowed to be transmitted without channel access, the existing DRS for NR-U/LAA should be transmitted after a successful type 2 LBT. In order to be a good neighbour, it is worth studying whether type 2A LBT is supported for short control signaling of SL-U. |
| Ericsson | For this proposal, we have the following comments/questions: - We agree on supporting the dynamic channel access schemes for LBT from NR-U (Type 1 and Type 2), however, we think that not all of them are suitable for each type of transmission, e.g., PSCCH/PSSCH, PSFCH and S-SSB. Therefore, we propose to include an FFS bullet regarding the applicability of each of the channel access schemes depending on the type of channel. - Include the following text: “FFS on applicability of each channel access scheme for the different channels.” - For the first bullet and sub-bullet we have the following comments: - What does exactly mean “timing to perform” the channel access scheme? - We think that we do not need to only focus on one particular issue at this time - In general, we propose to avoid getting to details now. Study is needed Proposal 2 (I): Type 1 and Type 2 (2A/2B/2C) dynamic channel access schemes for LBT from NR-U are supported for NR sidelink operation in unlicensed spectrum. - FFS on applicability of each channel access scheme for the different channels - FFS details and enhancements, if necessary - FFS whether semi-static channel access mode for FBE can be also supported - FFS whether short control signalling transmission (SCSt) can be supported and for which channel(s)/signal(s), including the feasibility, reliability and benefits |
| NEC | In general, we agree with the proposal, some further details as below: - prefer using UL channel access schemes (type 1 and type 2A/2B/2C) as baseline; - prefer using UL CAPC table as baseline; - FBE with low priority; |
| Mediatek | We are generally fine with the proposal but with some comments/updates as below: - For the 2nd bullet, we agree with Intel that the CAPC tables in 37.213 do not need to be enhanced. But whether the UL CAPC or DL CAPC to be re-used should be discussed. - For the 3rd bullet, we think the semi-static channel access can considered for SL-U because it is useful in some scenarios like factory. - For the last FFS, Very-low-power (VLP) operation should also be considered in addition to short control signalling mechanism. The very lower power (VLP) operation has been supported in some regions for 5/6GHz spectrum, which can reduce the mutual interference and improve the spectrum efficiency. Besides, the VLP operation may also simplify the implementation for short signalling transmission since LBT operation may not be necessary. Thus we think VLP operation for SL-U should be listed for FFS. |
| Huawei, HiSilicon | We support the proposal in principle. For the first FFS, we are not clear about the meaning of “timing to perform them…”. If it means to study what time the UE performs LBT, we think it is up to UE implementation and not necessary to have further discussion. Which type of LBT is performed is irrelevant to the specific channel/signal, it only relates to the gap between transmissions. So we think the first FFS is not needed. For the second FFS, based on the WID, Rel-16 NR-U LBT procedure shall be reused, so neither enhancements on the energy detection threshold adaptation nor CAPC table are allowed. For CAPC table, the FFS point should be whether to use UL or DL CAPC table, instead of changing the values inside the table. The FFS should be clear about this. The procedure of CW adjustment can be reused for HARQ ACK/NACK based procedure. The FFS point should only address SL blind retransmission case. We see scenarios that FBE can be applied, for example in a factory, other RATs can be precluded by regulations. So we think FBE can be also supported. Therefore, we suggest to have following changes on the proposal in red: Proposal 2 (I): Type 1 and Type 2 (2A/2B/2C) dynamic channel access schemes for LBT from NR-U and semi-static channel access mode for FBE are supported for NR sidelink operation in unlicensed spectrum. - FFS the timing to perform them based on the type of SL channels and signals transmitted and based on COT sharing conditions (if supported). - Including FFS How to address the inter-UE/mutual blocking issue - FFS any necessary update or enhancement to CAPC table, CW adjustment, energy detection threshold adaptation, etc. - CW adjustment in Rel-16 is reused for SL-U in the case of SL HARQ is enabled. - FFS the case of SL HARQ is disabled - FFS using which CAPC table(s) for SL-U - FFS whether semi-static channel access mode for FBE can be also supported - FFS whether short control signalling transmission (SCSt) can be supported and for which channel(s)/signal(s), including the feasibility, reliability and benefits |
| Broadcom | The DL and UL priority classes had been subject to long discussions followed by heavily debated trade-offs, due to the NR-U/Wi-Fi coexistence ramifications. We support the position stated above by CableLabs with regard to the priority classes, contention window adjustment, energy detection threshold adaptation, etc.. |
| Apple | Do not understand the 1st FFS. Do not see the need to specify when UE should start type 1 CCA. Suggest adding another FFS: whether DL CAPC or UL CAPC should be used for sidelink transmission. Do not support the update or enhance the table itself. These two tables are copied from EN 302 567. For LBE versus FBE, we support LBE has higher priority. |
| WILUS | We are generally fine with the proposal. The FFS points are fine as starting point for further discussion. It should not be restricted as only FFS points for SL-U channel access. |
| Bosch | We agree with the proposal; we also support FBE and LBE with equal priority. |

### Proposal for round 2

Summary of inputs from round 1 inputs:

- Main bullet:

  - Support semi-static channel occupancy from NR-U:

    - Yes: Intel, vivo, Sharp, CableLabs, Spreadtrum, Transsion, HW/HiSi, Bosch

    - Lower priority / FFS: LGE, OPPO, Qualcomm, ZTE/SC, DOCOMO, CATT/GH, Sony, NEC, Apple

- First FFS:

  - What is the meaning of the timing to perform?

- Second FFS:

  - UL or DL CAPC table? PQI mapping (PPPP) to priority classes?

  - Why update to CAPC, CW adjustment, and energy detection threshold?

- Fourth FFS:

  - Short control signalling transmission (SCSt) exception is per device according to ETSI BRAN regulation?

- Other issues:

  - To clarify the WID scope/scenario, in SL-U mode 1, whether gNB is capable of performing LBT on a shared channel and indicate the type of channel access to the scheduled SL UE?

FL comments and proposal:

- To clarify the meaning / intention behind “FFS the timing to perform” channel access modes can be related to a few issues as brought up by a few contributions. E.g.,

  - Although the timing in which Type 1 LBT is performed by gNB or UE in NR-U is not specified (e.g., gNB is a central node that initiate the COT for shares with UEs or UE sometimes initiates COT for UL transmission), but since sidelink is a distributed system where the UE could be performing Type 1 LBT (long LBT) quite often and blocked by other SL UE’s transmissions. There are proposals to perform Type 1 LBT at a specific timing in a slot to mitigate this issue.

  - When COT is shared by a UE, it is understood that the COT may not be usable/applicable to all other surrounding UEs. As such, for some UEs’ transmissions that are FDM’ed in a slot, some of them may be perform Type 2 LBT and others Type 1 LBT, and also CPE may be applied. Therefore, we should look into exactly the timing in which these LBTs / channel measurement are performed to see whether there is sufficient time for performing LBT, Tx-Rx switching, CPE length and maybe even take into consideration of inter-UE blocking and mutual blocking mentioned by Intel. In the end, likely we don’t need to specify anything related to the timing. But at least these aspects should be studied and discussed.

  - As some companies are supportive of this FFS bullet, maybe they have other timing related issues in mind that they would like to bring to and discussed in the next meeting. So, I suggest that we keep this FFS bullet.

  - @Intel, as explained in the above. I hope it is OK for you that we cover your mutual blocking issue using this FFS bullet, since so far you are the only company that has brought up this issue in RAN1. And I think it is worthwhile to discuss whether the sync error, propagation delay and transient period will affect the LBT timing and procedure. If necessary, we may even need to send an LS to RAN4 to look into these issues. Also this FFS bullet can also cover the timing of the measurement windows within an observation window in the LBT procedure should be changed compared to NR-U procedure, as suggested in your input.

- The second most commented topic is on the CAPC table. As mentioned in the background section, we need to address the PQI values used in the sidelink transmission / PC5 interface instead of 5QI values used currently in the CAPC tables. In PHY layer transmission, L1 priority (transmitted in SCI) is determined (based on priority information from UE higher layer), used for each transmission of SL channel / signals and encoded in SCI (PSCCH) for sensing purpose. There are currently 8 priority values provided the higher layer.  According to contributions submitted in this meeting, at least we have to update the CAPC table according to either PQI (which is more related to RAN2/SA domain) or L1 priority levels (referred as the PPPP levels in LTE sidelink). The other point that needs to be decided is either DL and/or UL CAPC tables from NR-U should be used as the baseline for the further update. Based on these points, I create a new FFS capturing the CAPC table related issues.

- Regarding the support of semi-static channel occupancy in Rel-18 for SL operation in a shared channel, as summarized in the above, some companies are in favour of supporting it while others think the priority should be on supporting the dynamic channel occupancy first. It is recommended to keep this as FFS.

- Other modifications:

  - Incorporate some suggested changes based on the inputs and reasonings.

  - Alignment of terminologies and use only the terminologies described in TS 37.213 (i.e., shared spectrum channel or shared channel, no FBE, no dynamic, channel occupancy).

Proposal 2 (II): Type 1 and Type 2 (2A/2B/2C) dynamic channel access procedures schemes from NR-U are supported for NR sidelink operation in a shared channel.

- FFS the timing and conditions for the actual channel access type(s) used for each SL channels and signals transmitted and based on COT sharing conditions (if supported)

  - Including the inter-UE/mutual blocking issue and the timing of the channel measurement

- FFS any necessary update or enhancement to CAPC table, CW adjustment, energy detection threshold adaptation e.g., due to no and groupcast type 1 SL-HARQ feedback

- FFS whether UL CAPC or DL CAPC or both should be used as the baseline, and how sidelink priority levels (PQI or L1 priority) should be mapped to the 4 channel access priority classes

- FFS whether semi-static channel occupancy access mode for FBE can be also supported

- FFS whether short control signalling transmissions (SCSt) can be supported, the requirements to qualify and LBT type needed for this exception, and for which channel(s)/signal(s), including the feasibility, reliability and benefits

| Company | Comments |
| --- | --- |
| ZTE,Sanechips | We would prefer to drop the type 1 in the second bullet given we believe type 2 also needs further optimization. - FFS any necessary update or enhancement to CAPC table, CW adjustment, energy detection threshold adaptation e.g., due to no and groupcast type 1 SL-HARQ feedback |
| LGE | On the main bullet, since companies have different understanding on whether gNB can perform LBT or not, we need to add “at least for SL Mode 2 UE”. To be specific, as we know, Type 2 channel access procedure can be used after COT is initiated. So, if gNB cannot perform LBT, it is necessary to have feasibility study first for type 2 channel access for Mode 1 UE. On 2nd bullet, we may need to add groupcast type 2 SL-HARQ feedback as well, or remove whole examples. To be specific, in groupcast type 2 SL HARQ feedback, the TX UE determines ACK only if it receives ACK from all the RX UEs. On the other hand, in NR-U, if a gNB or UE receives at least one ACK for TB transmission(s), it can reset the CWS. If we reuse similar approach for groupcast HARQ feedback Option 2, we may need to change it as well. On 5th bullet, according to TS37.213, short control signalling seems a part of FR2-2 at least for NR-U. In this point of view, since the study will focus on FR1, we’d better to deprioritize this issue in this stage. |

### Proposal for Week 2 Monday GTW session

FL comments/responses:

- @LGE, in regards to adding “at least for SL Mode 2 UE” in the main bullet, it is not really necessary at the moment, as we are just supporting these channel access procedures at a high-level. Also, the first FFS bullet could cover your intention as well. Furthermore, the UE may still perform Type 1 LBT to acquire a COT for transmission in mode 1 (e.g., when gNB’s COT has finished or transmitting other SL channels and signals). An FFS is added in the Proposal 2 to study COT sharing in mode 1.

For the SCSt, since there are quite some number of companies who would like to support it in Round 1, I think it is OK to have it here since it is just an FFS.

- @ALL, based to CableLab’s suggestion over the RAN1 email reflector, a sentence is added (with track changes ON) I also did some clear up in the proposal for easier reading.

Proposal 2 (III): Type 1 and Type 2 (2A/2B/2C) dynamic channel access procedures, transmission gap and LBT sensing idle time requirements specified in TS37.213 for NR-U are supported for NR sidelink operation in a shared channel.

- FFS the timing and conditions for the actual channel access type(s) used for each SL channels and signals transmitted and based on COT sharing conditions (if supported)

  - Including the inter-UE/mutual blocking issue and the timing of the channel measurement

- FFS any necessary update or enhancement to CAPC table, CW adjustment, energy detection threshold adaptation e.g., due to no and groupcast type 1 SL-HARQ feedback

- FFS whether UL CAPC or DL CAPC or both should be used as the baseline, and how sidelink priority levels (PQI or L1 priority) should be mapped to the 4 channel access priority classes

- FFS whether semi-static channel occupancy can be also supported

- FFS whether short control signalling transmissions (SCSt) can be supported, the requirements to qualify and LBT type needed for this exception, and for which channel(s)/signal(s), including the feasibility, reliability and benefits

## [ACTIVE] Topic #3: Shared channel occupancy (COT sharing) and CP extension (CPE)

Background:

NR-U supports COT sharing between UEs and gNB, i.e., a gNB can share a COT initiated by a UE for downlink transmission, and a UE can share a COT initiated by a gNB for UL transmission. When a gNB shares a COT initiated by a UE, the gNB may transmit a transmission that follows UL transmission after performing Type 2 LBT. When a UE shares a COT initiated by a gNB, once gNB has informed UE the resources for UL transmission within the COT, and UE will perform Type 2 LBT ahead of UL transmission. In general, such COT sharing is beneficial for reducing channel access delay and makes utilization of a COT more efficient, thus improving system performance.

The above same advantages of NR-U COT sharing also apply to sidelink scenario. In NR-U, a gNB can share a COT and indicate the COT structure to a number of UEs. Similarly, a sidelink UE, after initiating a COT, can also share the COT and indicate the COT structure to a number of sidelink UEs, thus enjoying the benefits of reduced channel access delay and improved resource utilization.

Based on contribution review in this meeting, most of companies (except 2) expressed their preference to support COT sharing in SL-U operation in Rel-18 and the performance of PSCCH/PSSCH transmissions in the unlicensed band will be greatly limited by performing only Type 1 LBT for every transmission if COT sharing is not supported. Since the COT sharing mechanism is already supported in NR-U and for future compatibility of operating mode 1 under NR-U gNB control, it is in our best interests to support COT sharing in SL-U. Further details, such as any restriction on the usage scenarios, cast type, whether a shared COT can be used by both mode 1 and mode 2 UEs, the COT forwarding effect, inter-UE coordination, COT structure, signalling contents and container when sharing COT to other UEs and gNB, etc can be further studied and discussed.

In NR-U, COT sharing is further aided by utilizing a CP extension (CPE), where it is used by the transmitter node to utilize the COT from being idle for too long and losing the access to the unlicensed channel to another RAT. In the current SL slot structure, there is always a gap symbol at the end of a slot for Tx/Rx switching (and switching to UL transmission with TA when UE is RRC connected to the network). For SCS = 15kHz and 30kHz which are supported in FR1, their symbol length is always greater than 25µs. For these cases, CPE is very useful in retaining the COT.  Also, in some contributions, it is mentioned that CPE can be also used to resolve inter-UE blocking. Therefore, it is recommended that CPE is supported in SL-U operation.

Proposal 3 (I):

- UE-to-UE COT sharing is supported in NR sidelink operation in unlicensed spectrum (SL-U).

  - FFS applicable SL channels and signals (e.g., PSCCH/PSSCH, PSFCH, S-SSB) for shared COT access

  - FFS all other details in compliance to regulation requirement including any restriction on the usage scenarios, cast type, whether a shared COT can be used by both mode 1 and mode 2 UEs, the COT forwarding effect, inter-UE coordination, COT structure, signalling contents and container when sharing COT to other UEs and gNB, etc

  - FFS whether gNB-to-UE COT sharing is supported in mode 1 for SL-U

- The principle of CP extension (CPE) from NR-U is supported for NR sidelink operation in unlicensed spectrum.

  - FFS all remaining details including applicable scenarios, usage, PHY structure, etc.

| Company | Comments |
| --- | --- |
| InterDigital | On the second sub-bullet, “COT forwarding effect” need to be clarified. For the last sub-bullet on gNB-to-UE COT sharing, we think it is out of the scope of the WID. WID scope limited to Uu being in licensed, so there is no guarantee that the gNB can operate in unlicensed spectrum. |
| Intel | We have a few comments regarding the proposal: - As a general comment, we prefer to decouple the discussion of the CP extension and UE-to-UE COT sharing. - For the CP extension - we think that more clarifications are needed before we can agree that the principle of the NR-U procedure could be supported. For instance what “principle of the CP extension” means? In NR-U, the CP to apply is provided by the gNB, based on the knowledge of the TA and channel type to be used. But in SL, it is unclear how this “principle” would be applied, considering that transmissions in SL legacy design are based on DL timing. Furthermore, it is unclear whether the gNB can perform sensing in the unlicensed spectrum and determine what CP extension should be used, and furthermore serve as a coordinator. In addition, before agreeing on the CP extension, we believe that we should first discussion and converge on when and scenario where this is needed. - For the UE-to-UE COT sharing, we are generally OK to enable this feature. However, we have a few comments: - In the second bullet, we are not very clear on the meaning of some of the elements listed (i.e, the COT forwarding effect and inter-UE coordination) and it may be great if the FL could please comment and provide more descriptions/context. - Also in the list of restrictions, we believe that restrictions based on CAPC type should be also included since a UE initiating a COT with a given CAPC, may not share the COT with a UEs that have different QoS/traffic requirements. - Also the list should also include whether a UE may be allowed to operate as either initiating or responding device at the same time, and the types of UEs for which sharing is allowed: for instance whether the COT sharing should be only limited to responding UEs within which the initiating UE is communicating, or this should be extended to any UEs. - As InterDigital, we think that last FFS is out of scope. |
| LGE | We are fine with this direction. Again, according to “Uu operation for mode 1 is limited to licensed spectrum only” in WID, it does not say that gNB cannot perform NR-U for Uu link. The wording itself says that DCI format 3_0 or RRC signalling for mode 1 will be on licensed spectrum only. In this case, if gNB want, gNB can transmit DL transmission and UL reception for Uu link, but not for SL scheduling. This clarification is important, because if gNB cannot know the status of unlicensed band, gNB cannot schedule accurate SL resources. In this case, there is no need to support Mode 1 RA for SL-U. Considering coexistence between NR-U and SL-U, gNB-to-UE COT sharing could be efficiently used. Currently, we are fine with putting it as FFS. |
| OPPO | - For COT sharing: - For the last sub-bullet, we agree with InterDigital and Intel, it is out of scope - For CPE: - It was used in NR-U. while the key difference between NR-U and SL-U is that all UL resource is controlled/scheduled by gNB. gNB can avoid inter-UE blocking by suitable scheduling and/or indicating corresponding CPE length to different UEs. While SL-U is distributed system. In mode 2, UE selects resource based on sensing and there is no central scheduler. FDM among UEs should be supported. If different UE in the same slot transmits CPE at different time, that will cause inter-UE blocking. Early CPE transmission will block other UE’s LBT procedure. Then we proposal to FFS whether and how to use CPE mechanism in SL-U. - FFS whether/how to apply The principle of CP extension (CPE) from NR-U is supported for NR sidelink operation in unlicensed spectrum. - FFS all remaining details including applicable scenarios, usage, PHY structure, etc. |
| Futurewei | We support the first bullet on UE-to-UE COT sharing in general. However, for the last subbullet, as we pointed out on GTW, according to WID, the mode 1 Uu link is on licensed band and gNB cannot transmit in unlicensed. Therefore, gNB cannot start a COT and share it. We propose to remove it. For the second bullet, there are other proposals to reduce the gap, e.g., using the guard symbol for PSSCH transmission (either repeat or rate matching with additional IR modulated symbols). We suggest discuss it in an independent proposal to include various methods for reducing the time gap between consecutive transmissions. Proposal 3a (I): - Support one or more of the following methods to reduce the time gap between consecutive transmissions. - Transmitting on the gap symbol - The principle of CP extension (CPE) from NR-U is supported for NR sidelink operation in unlicensed spectrum. - FFS all remaining details including applicable scenarios, usage, PHY structure, etc. |
| Qualcomm | - Support COT sharing between UEs, at least for PSFCH and PSCCH/PSSCH from responder to initiator, in both mode 1 and mode 2 - Support at least for unicast to the initiator, support also for groupcast/broadcast to a group of UEs including the initiator - Support COT structure signaling, FFS its content and container - FFS whether and how COT sharing between gNB and UE can be considered in SL-U, and what means “gNB-to-UE COT sharing is supported in mode 1 for SL-U” - Support CPE in SL-U - Support CPE to close gaps in symbols #10 and #13 in slot structure for enabling long transmission bursts - Support CPE to close the Tx gap during COT-sharing between UEs, where the responder can transmit after a Type 2 LBT - Support CPE to enable micro-second transmission starting points to provision SL-U with the possibility of TDM operation of UEs, also providing intra-RAT collision protection. This is especially useful in two cases for first transmission (the protection of resource reservation is absent): a) when UEs allocate the full LBT BW and, b) in congested networks. Notably, case (b) is the case where many UEs try to select a few subchannels (even a small portion of the available LBT BW) and try to access with FDM, but they may collide anyway due to being in large number. In this case a form of prioritization via CPE (high priority class takes longer CPE and earlier starting point) may be beneficial to reduce collisions within SL-U. - FFS using CPE to start transmissions early after completing a Type 1 LBT, as a filler to occupy the COT at symbol-level granularity to avoid other devices to jump-in and causing the loss of the COT. |
| Panasonic | For the 1st bullet, we are not yet clear which UE triggers a COT and the amount of the length to be informed when initiating a COT sharing among UEs. We are ok to further study COT sharing among UE including usage scenarios, cast type, and so on as described in FFS. So instead of agreeing "to supported", "to study" or "working assumption" would be more proper. We support CP extension proposal. |
| CMCC | Acceptable except the 3rd FFS. We are still not quite clear about how to support gNB-to-UE COT sharing because gNB can only work on licensed band in Rel-18, and only UE can initiate a COT in sidelink from our point of view, so we think more clarifications are needed. |
| vivo | Regarding the 2nd bullet, is the intention to include all aspects mentioned by papers? In our understanding, it is more constructive to list some clear action points for further study, e.g., - FFS all other details in compliance to regulation requirement including - Condition for a UE to use a shared COT - UE behaviour for a UE to use a shared COT - Signalling contents and container when sharing COT to other UEs Regarding 3rd bullet, for mode 1, we assume Uu is operating on licensed carrier, so we prefer to remove the following - FFS whether gNB-to-UE COT sharing is supported in mode 1 for SL-U Regarding the CPE, we think the motivation to use CPE in NR-U is resolve the resource collision when a resource is scheduled to multiple UEs. We think this motivation is not suitable for SL resource selection. if CPE is used to fill the gap, we prefer to discuss this topic in 9.8.4.2, we hope FL re-coordinates this topic. By the way, we agree the proposal of CPE |
| ZTE, Sanechips | Support in general. We share similar view with InterDigital that ‘gNB-to-UE COT sharing’ is not in the scope the WI. For COT sharing, since all the details are FFS, it is hard to list all the topics/issues here, to make less confusing, we prefer to remove the list, - FFS all other details in compliance to regulation requirement including any restriction on the usage scenarios, cast type, whether a shared COT can be used by both mode 1 and mode 2 UEs, the COT forwarding effect, inter-UE coordination, COT structure, signalling contents and container when sharing COT to other UEs and gNB, etc |
| Sharp | We are in general fine with the main bullet. For the second sub-bullet, it is unclear what it actually means by “inter-UE coordination”; if it is referring to the Rel-17 feature with the same name, we don’t think it should be listed there. In fact we think at this stage it suffices to simply say “FFS details”. For the last sub-bullet we share LGE’s understanding. Moreover, we have one clarification for UE-to-UE COT sharing, i.e. whether the UE herein includes all transmitting UEs and/or responding receiving UEs. |
| CableLabs | We believe the Mode1 applicability to SL-U may require further clarifications. Until these clarifications will be provided, we do not see necessary to discuss COT sharing at this time. Also, we consider COT sharing and CP extension as different topics, which should be discussed separately. Any UE to UE COT sharing should follow a discussion concerning how this type of COT sharing could be assimilated to NR-U COT sharing mechanisms. Otherwise, this discussion may be premature. |
| xiaomi | We are generally fine with the proposal. 1.For the COT sharing, We share similar view with other companies that gNB-to-UE COT sharing shall not be supported. 2．For the CP extension (CPE), We also think CPE is useful for sidleink-U. However, we are not sure what is the exact meaning of “principle of CP extension (CPE) from NR-U”. We suggest to revise the bullet as: - CPE is supported for NR sidelink operation in unlicensed spectrum. - FFS details |
| Lenovo | Overall, we think that UE-UE COT sharing should not be coupled with CP extension. For the first sub-bullet, if the intention is to define which SL signals/channels are allowed to be transmitted in a shared COT, we think the SL channels/signals as such may not be the important criterion, but rather if the content is data or control and whether the (or at least) one recipient is the COT initiating device. Since the Rel18 WID prohibits any specific optimization for Rel17 RA, the inter-UE coordination in the second sub-bullet can be removed as it is implicitly covered by the first sub-bullet. The third sub-bullet on the usage of gNB to UE COT sharing in mode 1 RA requires more clarification as the current WID prohibits transmitting SL grant using DCI format 3_0 using the unlicensed spectrum. The FFS part should capture the COT structure signalling, cast type of the COT sharing indicator, cast type transmission allowed within the shared COT, mechanism to transmit COT sharing to multiple destinations/Rx UEs, mechanism to select a COT donor UE by a COT recipient (if there are multiple COT donors) |
| NTT DOCOMO | We support the two main bullets. Regarding the 2nd FFS of COT sharing, if details are controversial, let’s remove the details (i.e. from ‘including ...’) since this is FFS. No need to consume time to discuss details of FFS. For the 3rd FFS of COT sharing, the definition of ‘gNB-to-UE COT sharing’ is unclear for us. Before deciding remove or not, meaning should be clarified sufficiently. |
| Spreadtrum | We are fine with the proposal except the 3rd FFS about gNB-to-UE COT sharing. We also think it is not in the scope. |
| Fujitsu | We support Proposal 3 in principle. As for “FFS all other details in compliance to regulation requirement including any restriction on”, we think “condition for a UE to use a shared COT” and “which COT can be shared if there are more than one shared COT” should be also included. Also, “the COT forwarding effect, inter-UE coordination” should be clarified. |
| CATT/GOHIGH | For the 2nd FFS of UE-to-UE COT sharing, as mentioned by other companies, we think “COT forwarding effect” and “inter-UE coordination” are not clear and need to be clarified. Meanwhile, considering that only licensed gNB is involved in SL-U, sharing COT to the gNB is invalid and should be removed from the FFS. For the 3rd FFS of UE-to-UE COT sharing, from our perspective, mode-1 related issues would better be postponed at this stage considering there are still many key topics that need to be discussed. Furthermore, the first bullet is about supporting UE-to-UE COT sharing, which has nothing to do with gNB-to-UE COT sharing. So, we suggest removing the 3rd FFS of the first bullet. |
| Sony | We support both bullets on UE-to-UE COT sharing and CPE. The UE-to-UE COT sharing should be supported at least for PSFCH from receiver UE to transmitter UE. We are open to keep FFS for gNB-to-UE COT sharing in the last sub-bullet at this stage. |
| Samsung | We are generally fine with the principle. For the third sub-bullet “FFS whether gNB-to-UE COT sharing is supported in mode 1 for SL-U”, our understanding is that gNB can acquire the information of COT from initiating UE and use the information for mode 1 scheduling, but there is no guarantee gNB can initiate a COT and share it to UE. So we think the current wording is inaccurate and suggest to modify it as: “FFS whether to support gNB acquiring information of a UE initiated COT gNB-to-UE COT sharing is supported in mode 1 for SL-U”. In our understanding such issue is within WID scope. We generally support the other FFS sub-bullets under 1st bullet regarding COT sharing. For the second bullet regarding CPE, we support this direction but also think it can be discussed in a separate proposal. |
| Nokia, NSB | We support the initial proposal by the FL. |
| Fraunhofer | We are supportive of both the main bullets. For the 2nd sub-bullet, it could just state “FFS all other details” and remove all the text after. For the 3rd sub-bullet, our understanding is that the gNB can configure a resource pool in the unlicensed band, and use a DCI over the Uu on a licensed band to provide COT sharing details. If this is not possible, we are unclear on how Mode 1 operations can be supported for SL-U. |
| Transsion | Regarding gNB-to-UE COT sharing, we prefer to remove this sub-bullet. The WID restricts Mode 1 Uu operation to occur only on licensed bands. This limits the SL UE to receiving DCI 3-0 or RRC signaling and transmitting PUCCH carrying SL HARQ feedback on the licensed spectrum. If gNB-to-UE COT sharing is supported, the SL UE need to receive DCI formats on unlicensed band. Receiving PDCCH on both licensed and unlicensed bands at the same time will significantly increase the implementation complexity of the SL UE. Regarding CPE, CPE is not only applicable to COT sharing, but also beneficial to guarantee continuous transmission in SL-U. This is because CPE can reduce the gap between two consecutive transmissions. In addition, CPE can be applied to CG sidelink transmission to avoid collision. |
| Ericsson | For this proposal we have the following comments: - We are supportive of the main bullet and propose to study the UE-to-UE COT sharing. - We think that the applicability and details must be left FFS at this point - CP extension can be studied as a detail too, although it is not clear to us whether it is necessary. Proposal 3 (I): - UE-to-UE COT sharing is supported in NR sidelink operation in unlicensed spectrum (SL-U). - FFS applicable SL channels and signals (e.g., PSCCH/PSSCH, PSFCH, S-SSB) for shared COT access - FFS all other details in compliance to regulation requirements |
| NEC | We generally agree with the proposal, and hold the same view as mentioned above, the item“whether gNB-to-UE COT sharing is supported in mode 1 for SL-U” is out of the study scope. |
| Mediatek | We are generally fine with the proposal but with the following comments: COT sharing - For the 2nd sub-bullet, the actual meaning of “the COT forwarding” and “inter-UE coordination” should be clarified first. - For the 3rd sub-bullet, we have the same opinion with InterDigital, Intel and OPPO, etc. It is out of the scope. |
| Huawei, HiSilicon | Support in general. COT sharing and CPE are both of the important enhancements in SL-U, and they are supported in NR-U and should be reused. - For the first FFS of UE-to-UE COT sharing bullet, we do not see any justifications that certain channels/signals need to be excluded from COT sharing, and COT sharing is explained/verified by many companies that would be beneficial. So we think just a simple conclusion can be made to clarify all channels/signals can be applied for COT sharing, no further FFS. - For the second FFS of UE-to-UE COT sharing bullet, since the main bullet is define the principle of COT sharing between UEs, it is not related to gNB, so “and gNB” should be deleted. “the COT forwarding effect” and “inter-UE coordination” have not been justified how to operate with COT sharing schemes and will complicate the whole design. So we prefer not to discuss them. - For the third FFS of UE-to-UE COT sharing bullet, as per WID, Uu signalling is only applicable on licensed band, and thus a COT cannot be initiated by gNB in unlicensed band. Considering sharing the unlicensed band with NR-U is not in the scope, so we suggest to delete this bullet Therefore, we suggest to have following changes on the proposal in red. Proposal 3 (I): - UE-to-UE COT sharing is supported in NR sidelink operation in unlicensed spectrum (SL-U). - FFS applicable SL channels and signals (e.g., PSCCH/PSSCH, PSFCH, S-SSB) for shared COT access - Shared COT access is applicable for all SL channels and signals (i.e., PSCCH/PSSCH, PSFCH, S-SSB). - FFS all other details in compliance to regulation requirement including any restriction on the usage scenarios, cast type, whether a shared COT can be used by both mode 1 and mode 2 UEs, the COT forwarding effect, inter-UE coordination, COT structure, signalling contents and container when sharing COT to other UEs and gNB, etc - FFS whether gNB-to-UE COT sharing is supported in mode 1 for SL-U - The principle of CP extension (CPE) from NR-U is supported for NR sidelink operation in unlicensed spectrum. - FFS all remaining details including applicable scenarios, usage, PHY structure, etc. |
| Broadcom | The UE to UE COT sharing is a use case which was not covered by NR-U. If such a use case is to be considered by SL-U, then we request a related Study Item in order to understand the benefits and impact upon the coexistent traffic. We object considering this scenario without a proper study. |
| Apple | Support UE to UE COT sharing and CPE. For COT sharing, - Second bullet needs definition for “COT forwarding effect”. Also regarding “signalling contents and container when sharing COT to other UEs and gNB”, we do not see why sidelink COT can be shared with gNB. Third bullet is out of scope. “FFS whether gNB-to-UE COT sharing is supported in mode 1 for SL-U”. Even Uu link work in the same band, sidelink transmission sharing gNB acquired COT is not allowed in the regulation, as sidelink transmissions are not between initiator and responders within gNB acquired COT. |
| WILUS | We are supportive of two main bullets such as UE-to-UE COT sharing and CP extension. However, we are not sure whether or not the 3rd FFS sub-bullet in the 1st main bullet is within a scope of this WI considering Uu is limited in licensed. |
| Bosch | We are fine with the proposal but we need to clarify: - What is exactly meant by COT forwarding? - Whether 3rd FFS is withing the scope of this release. |

### Proposal for round 2

Summary of inputs from round 1 inputs:

- UE-to-UE COT sharing

  - Main bullet (UE-to-UE COT sharing):

    - CableLabs: UE to UE COT sharing should follow a discussion concerning how this type of COT sharing could be assimilated to NR-U COT sharing mechanisms

    - Broadcom: UE-to-UE COT sharing should be a study item in order to understand the benefits and impact upon the coexistent traffic

  - First FFS (applicable channels/signals):

    - HW/HiSi: to make all SL channels and signals are applicable in UE-to-UE COT sharing

  - Second FFS (all other details and restrictions):

    - The meaning of “COT forwarding effect” and “inter-UE coordination” are unclear

    - restrictions based on CAPC type should be also included since a UE initiating a COT with a given CAPC, may not share the COT with a UEs that have different QoS/traffic requirements

    - whether a UE may be allowed to operate as either initiating or responding device at the same time, and the types of UEs for which sharing is allowed

  - Third FFS (gNB-to-UE COT sharing):

    - Support (gNB is capable of LBT / performing NR-U in the shared channel)

      - LGE, Fraunhofer, [Samsung], [Sony]

    - Not support / out of WID scope

      - InterDigital, Intel, OPPO, Futurewei, CMCC, vivo, ZTE/SC, xiaomi, Lenovo, Spreadtrum, CATT/GH, Transsion, NEC, MediaTek, HW/HiSi, Apple, WILUS, Bosch

- CP extension

  - Main bullet:

    - what “principle of the CP extension” means?

    - we should first discussion and converge on when and scenario where this is needed

    - COT sharing and CP extension as different topics, which should be discussed separately

- Other issues:

  - Mode1 applicability to SL-U may require further clarifications. Until these clarifications will be provided, we do not see necessary to discuss COT sharing at this time.

  - Any UE to UE COT sharing should follow a discussion concerning how this type of COT sharing could be assimilated to NR-U COT sharing mechanisms.

  - The UE to UE COT sharing is a use case which was not covered by NR-U. If such a use case is to be considered by SL-U, then we request a related Study Item in order to understand the benefits and impact upon the coexistent traffic.

FL comments and proposal:

- @Sharp, on the question that you ask about “UE-to-UE” and whether the UE herein includes all transmitting UEs and/or responding receiving UEs, it is exactly why the long list of aspects is provided in the 2nd FFS.

- @vivo, thanks for some wording suggestion. It seems like many preferred to have a simple FFS rather than listing all potential aspects that we need to discuss about UE-to-UE COT sharing. In addition, the discussion on CPE is very closely tied together with channel access and resource allocation discussions as shown by Qualcomm and Transsion responses. This topic has been coordinated between FLs and decided it is better to be treated under this agenda, including all future structure design to avoid split and duplicated discussions in two AIs.

- @CableLabs, NR-U COT sharing is between gNB and UEs. If gNB is assumed not operating NR-U for sidelink mode 1 operation in Rel-18, then the UE would need to perform Type 1 LBT to obtain a COT for transmitting SL. Then in this case there is no NR-U COT sharing. UE-to-UE COT sharing can still apply to sidelink mode 1 and mode 2 operation without any dependency on NR-U COT sharing.

- @Broadcom, the concept of COT sharing between different transmitting nodes in NR-U is supported. The benefits and impacts studied during NR-U are already clear. As the current WID has captured that “Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation”, the UE-to-UE COT sharing for SL-U is bounded by the same regulation requirements.

- @All, to address common comments:

  - As commented / explained by Qualcomm and Transsion, CPE can be used in conjunction with Type 1 and Type 2 LBT, and the usage can be very closely tied together with COT sharing. When discussing COT sharing operation, it is good to also discuss how CPE is used. For now, I have kept them in separate bullets.

  - On the long list of aspects that should be considered during COT sharing discussion, the intention was bring to everyone’s attention of the list and the reasons behind them. Some may seem straight forward and some may be not obvious at first look. It seems like many preferred to have a simple FFS bullet rather than listing all potential aspects that we need to discuss about UE-to-UE COT sharing. I am fine to delete them. To answer some common questions, let me clarify the meaning / reasons why “COT forwarding effect” and “inter-UE coordination” were included:

    - When a UE1 uses a COT from UE2, whether UE1 can indicate the same COT for communicating with UE3 when UE3 is not within the range of UE2 and UE1/UE2/UE3 are within or not within the same group?

    - In inter-UE coordination, UE-A can be explicitly or implicitly trigger to respond to UE-B’s request with a set of resources during the resource selection process in UE-A. Another scenario is when UE-A responds in PSFCH to UE-B indicating a conflict. Are all these can be considered as part of COT sharing such that UE-A only needs to perform Type 2 LBT?

  - Lastly, as it was discussed during Tdoc presentation in the first GTW session, the topic on whether gNB is capable of performing LBT (or having NR-U operation in the same shared channel as the SL operation) and indicate the channel access procedure to be performed by the UE should be also clarified in RAN1 to have a common understanding. So far, only a handful of companies have expressed their views. It is worthwhile to further discuss this aspect, and hence, a new FFS bullet is added.

Proposal 3 (II):

- UE-to-UE COT sharing is supported in NR sidelink operation in a shared channel (SL-U).

  - FFS applicable SL channels and signals (e.g., PSCCH/PSSCH, PSFCH, S-SSB) for shared COT access

  - FFS all other details in compliance to regulation requirement including any restriction on the usage scenarios, cast type, whether a shared COT can be used by both mode 1 and mode 2 UEs, the COT forwarding effect, inter-UE coordination, COT structure, signalling contents and container when sharing COT to other UEs and gNB, etc

- The principle of CP extension (CPE) from NR-U is supported for NR sidelink operation in a shared channel.

  - FFS all remaining details including applicable scenarios, usage, PHY structure, etc.

- FFS whether and how COT sharing between gNB and UE can be considered in SL-U, including

  - what is required on the gNB side to start a COT (e.g. perform LBT on the scheduling shared channel, DL data transmission, SSB, etc.) in order to share it with an SL-U UE to perform UE-to-UE communications

  - whether to support gNB acquiring information of a UE initiated COT in mode 1 for SL-U

| Company | Comments |
| --- | --- |
| ZTE,Sanechips | We would prefer the last FFS deleted given it's questionable whether gNB and UE COT sharing is in the scope. One may say the scope does not preclude the gNB operating in unlicensed band as well as licensed, yet this point can be further investigated when the functional part is completed for the WI. |
| LGE | On 1st bullet, since companies have different understanding on whether gNB can perform LBT procedure or not, which type of UE can initiate COT need to be further discuss. So, we’d like to add “FFS: whether Mode 1 UE or Mode 2 UE can initiate COT”. On the last bullet, I’d like to share our views further. The motivation and benefit of Mode 1 is that all the resources are under gNB’s control and gNB can coordinate all the transmissions in the carrier including DL, UL, an SL. Meanwhile, if gNB cannot perform LBE procedure or cannot indicate COT, resources are no longer under gNB’s control, so gNB cannot coordinate the resources. In this case, we do not need to support Mode 1 RA for unlicensed band. Meanwhile, in the previous email discussion, I found that some companies to suggest to consider COT duration for Mode 1 RA or to support DCI format 3_0 indicating channel access type. However, when gNB cannot perform LBT procedure on the unlicensed carrier, gNB does not know COT duration, and cannot decide which channel access type will be used by UE (actually, this indication is part of implicit COT sharing). Moreover, considering inter-operator scenario, we cannot say that it is always ensured that SL-U does not coexist with NR-U. |
| vivo | We agree with ZTE, the last FFS on COT sharing between gNB and UE should be removed. The WID restrict that “where Uu operation for mode 1 is limited to licensed spectrum only”, Uu operation on licensed band means no LBT by gNB. |

### Proposal for Week 2 Monday GTW session

FL comments/responses:

- @ZTE/SC, vivo, since not all company have the same understanding, further study and discussion is needed. By removing it, it does not mean everyone’s understanding is aligned, until an agreement or conclusion is reached on this aspect.

- @LGE, ALL, the suggested new FFS bullet is not really needed in my view. Even if gNB is capable of performing LBT and share the COT with UE in mode 1, the UE may still perform Type 1 LBT to acquire a COT for transmission (e.g., when gNB’s COT has finished or transmitting other SL channels and signals). I have included it for now and if others are fine with it. I also did some clear up in the proposal for easier reading.

Proposal 3 (III):

- UE-to-UE COT sharing is supported in NR sidelink operation in a shared channel (SL-U).

  - FFS applicable SL channels and signals (e.g., PSCCH/PSSCH, PSFCH, S-SSB) for shared COT access

  - FFS: whether Mode 1 UE or Mode 2 UE can initiate COT

  - FFS all other details in compliance to regulation requirement

- CP extension (CPE) is supported for NR sidelink operation in a shared channel.

  - FFS all remaining details including applicable scenarios, usage, PHY structure, etc.

- FFS whether and how COT sharing between gNB and UE can be considered in SL-U, including

  - what is required on the gNB side to start a COT (e.g. perform LBT on the scheduling shared channel, DL data transmission, SSB, etc.) in order to share it with an SL-U UE to perform UE-to-UE communications

  - whether to support gNB acquiring information of a UE initiated COT in mode 1 for SL-U

## [ACTIVE] Topic #4: Multiple channel access

Background:

In NR-U downlink,

- Type A requires gNB to perform Type 1 LBT on each channel.

- Type B requires gNB to perform Type 1 LBT on one of the multiple channels and sense other channels for at least a sensing interval 25µs before transmission, but with constraint that selection of channels should be at random or selection of the same channel should be no more frequent than once every 1 second.

In NR-U uplink,

- UE is either scheduled to start transmissions on channels initiated by gNB or perform uplink transmissions on configured resources, and it will only perform single-channel sensing procedure (Type 1 or Type 2) on a certain channel as indicated by gNB.

Similar to NR-U, simultaneous access to multiple unlicensed channels for SL transmission in more than one channels in the same slot from a UE is one of the main motivations of supporting SL operation in the unlicensed spectrum to support high data rate applications. Therefore, on the top-level principle, multi-channel access should be supported in SL-U from the beginning to take advantage of the full bandwidth of the unlicensed spectrum in 5GHz and 6GHz. The details on how to support this operation can be further studied.

Proposal 4 (I):

- Sidelink transmission in multiple LBT channels / BWs in a slot is supported for NR sidelink operation in the unlicensed spectrum

  - FFS whether the downlink (Type A and Type B) and uplink multi-channel access mechanisms from NR-U should be supported and how they are applied in SL mode 1 and mode 2 operation

| Company | Comments |
| --- | --- |
| InterDigital | We are fine with Proposal 4. |
| Intel | We are fine with Proposal 4. |
| LGE | We are fine with this direction. One more thing to clarify is that which mechanism is used for multi-channel transmission could be different depending on SL channel type. To be specific, in case of PSCCH/PSSCH, UE can transmit only one PSCCH/PSSCH in a time. It seem like UL transmission. On the other hand, in case of PSFCH, UE can transmit multiple PSFCH with different SL priority or CAPC in a time. It seems like DL transmission. |
| OPPO | We are fine with Proposal 4. |
| Futurewei | We are ok with the proposal. |
| Qualcomm | We support SL transmissions in multiple LBT channels, and we support further studying multi-channel LBT, both Type A and Type B, and their potential application for mode 1 and mode 2. |
| Panasonic | We are fine with Proposal 4. |
| CMCC | Comments. We think we should first define UE resource allocation/selection behaviour, if UE can only be allocated/select resources within a single RB set, there is no need to discuss the multiple LBT BWs operation. BTW, we think even for NR-U uplink, the transmission can be performed only when all the LBT BWs are successfully accessed, not only a certain one among them. |
| vivo | We support the proposal [vivo2] For now, the time domain granularity can be pending, since slot structure may be changed based on outcome of 9.8.2. also, the DL/UL to be baseline can be down selected. - Sidelink transmission in multiple LBT channels / BWs in a slot is supported for NR sidelink operation in the unlicensed spectrum - FFS whether the downlink (Type A and Type B) and/or uplink multi-channel access mechanisms from NR-U should be supported and how they are applied in SL mode 1 and mode 2 operation |
| ZTE, Sanechips | Support in general |
| Sharp | We are in general fine with the proposal. As a general comment, we propose to reuse the exact terms established in NR-U agreements, e.g. “LBT channels / BWs” => “LBT bandwidths”. And we propose to remove “in a slot” to avoid unnecessary debates not directly related to multi-channel transmission. |
| xiaomi | We are fine with the proposal. |
| Lenovo | We support the proposal 4 |
| NTT DOCOMO | Support this proposal. |
| Spreadtrum | We support the proposal. |
| Fujitsu | We are fine with Proposal 4. |
| CATT/GOHIGH | We are fine with Proposal 4. |
| Sony | We are fine with Proposal 4. |
| Samsung | We are fine with proposal 4. |
| Nokia, NSB | We support the proposal. |
| Fraunhofer | We are supportive of the FL’s proposal. |
| Transsion | We are fine with Proposal 4. |
| Ericsson | We are supportive of sidelink transmission in multiple LBT channel/BWs in one slot to be considered in this release. However, we think that the FFS bullet restraints too much the scope of the discussion at this point of the release. We propose to modify the FFS bullet to give a more general FFS description, e.g., FFS details on SL transmissions in multiple LBT channel/BWs in a slot. Given the restrictions on CA in the WID, we think it is clear that this corresponds to wideband mode. Proposal 4 (I): - Sidelink transmission spanning multiple LBT channels / BWs (i.e., wideband mode) in a slot is supported for NR sidelink operation in the unlicensed spectrum - FFS details |
| NEC | We are fine with the proposal. |
| Mediatek | We are fine with the Proposal. |
| Huawei, HiSilicon | We are generally fine with Proposal. |
| Apple | OK with the proposal |
| WILUS | We support the Proposal 4. |
| Bosch | We are fine with the proposal. We also prefer to have FFS for details (only), i.e., FFS details. |

### Proposal for email endorsement

Summary of inputs from round 1 inputs:

- Support / OK with Proposal 4 (I): InterDigital, Intel, LGE, OPPO, Futurewei, Qualcomm, Panasonic, vivo, ZTE/SC, Sharp, xiaomi, Lenovo, DOCOMO, Spreadtrum, Fujitsu, CATT/GH, Sony, Samsung, Nokia/NSB, Fraunhofer, Transsion, Ericsson, NEC, MediaTek, HW/HiSi, Apple, WILUS, Bosch [32]

- Dependent on whether resource allocation/selection is allowed across the RB_set: CMCC

FL comments/responses:

- @LGE, thank you for bring up multi-channel access could be different / possible depending on SL channels / signals. This aspect can be further discussed as part of the FFS bullet.

- @CMCC, resource allocation/selection behaviour could be also dependent on whether multi-channel access is supported or not. If multi-channel access is allowed, then resource allocation/selection may be updated to enable multiple RB_set. If not allowed, then no resource allocation/selection update is needed. Based on majority of inputs in the last round, it seems like we can first agree on supporting multi-channel access. The next step is we can further determine whether and how to update the resource allocation / selection across the RB_set. For the second point, please refer to Ericsson’s comment to just use a simple “FFS details”. I believe this covered everything including your point on SL transmission only when all LBT BWs are successful.

- @vivo, I understand your point due to multiple starting positions in a slot is not yet clear, but if we remove “in a slot” then others can interpret the proposal to be in different slots. In this interpretation, naturally it should be supported. To address your concern, how about adding the word “concurrently” in the proposal to ensure we are referring to the case when the UE is transmitting at the same time in multiple LBT channels / BWs? For the second point, please refer to Ericsson’s comment to just use a simple “FFS details”.

- @Sharp, please refer to my reply to vivo just in the above regarding your second point. On the first point, it is updated in the updated proposal.

- @Ericsson, done. Note that, the term “wideband mode” is not used in NR-U spec (38.213) although it may be a common used term during discussion.

- @ALL, please further checking the following updated proposal addressing all comments from the last round. Please comment ONLY IF YOU HAVE CONCERN/OBJECTION to save you from typing and to make this document shorter.

Proposal 4 (II):

- Sidelink transmission in multiple LBT channels / BWs bandwidths (e.g., wideband mode) concurrently in a slot is supported for NR sidelink operation in the unlicensed spectrum

  - FFS details whether the downlink (Type A and Type B) and/or uplink multi-channel access mechanisms from NR-U should be supported and how they are applied in SL mode 1 and mode 2 operation

| Company | Comments |
| --- | --- |
| Samsung | We are ok with the spirit of the proposal, but the wording needs improvement in our understanding. The key of the discussion is on whether to support multi-channel access procedure for SL-U, and we support so. The channel access procedure aspect is not reflected in the main bullet, and purely talking about SL transmission over multiple channels seems not within the scope of this AI (but in PHY frame). Also “in a slot” is not needed. The key is how to initiating channel occupancy on multi-channel, so “in a slot” seems not accurate. Lastly, we are not sure what type of enhancement is considered, but it should be emphasized that existing multi-channel access procedures are the baseline to study from (which is aligned with the WID), and any enhancement should consider complying with regulation. Based on above, we have the following suggestion as a revised proposal, trying to use spec language (not tracking changes for a clear view): Proposal 4 (II) – Revised by Samsung: - Channel access procedures for transmission(s) on multiple channels are supported for NR sidelink operation on the unlicensed spectrum - Note: Type A and/or Type B DL multi-channel access procedures and/or UL multi-channel access procedures are considered as baseline for study, and enhancement for further study shall comply with regulation |
| Intel | While we are OK with the spirit of the proposal, we have a few concerns: - since this would be our first agreement, and we have not yet agreed on which LBT types will be supported in NR-U SL, and whether the same definition of LBT BW from NR-U will be re-used, it would be better to define what LBT BW means. Perhaps, if we all have the same understanding, a simple note could be added below the initial proposal indicating that the “the definition of LBT bandwidth is the same as that in NR-U for FR-1”; - We agree with Samsung’s comments, and that “in a slot” is not needed since here we are discussing how to acquire a COT in a multi-carrier case, but we are OK with keeping the FFS since this is our first discussion and we have not yet had time to identify and discuss possible issues. Also, we are generally OK with Samsung’s revised proposal, but we prefer to translate the note into a more general FFS, given that; i) once again type A /type B imply that type 1 and type 2A LBTs are used in NR-U SL, while we have not yet agreed on it; ii) the note seems to imply only dynamic channel access mode is supported; and iii) as indicated above this is our first discussion and we have not yet had time to identify and discuss possible issues. |
| LGE | Regarding Samsung’s comments for WID, we do not think that SL transmission on multi-channel violate the current WID. In this point of view, we prefer to keep FL’s original proposal. It seem necessary to support SL transmission on multi LBT BW to increase data rate for commercial use cases as per WID. Regarding Intel’s comment on the definition of LBT BW, we think the same definition of NR-U is reused. If we define new LBT BW for SL, WI needs to be updated to include relevant RAN4 work. However, currently, I cannot find the relevant RAN4 work, so I think reusing those of NR-U is a common understanding. According to NR-U, Type A or Type B targets the case when gNB transmits DL transmission(s) in the same time. So, we need to keep "in a slot". For SL transmission(s) in different time, UE will access each channel independently. In our understanding, that is not a target of this proposal. I think minor change is needed for main bullet. That is to add “(s)” after SL transmission to cover the case when UE transmits multiple PSFCHs simultaneously. According to WID, simultaneous multiple PSSCH transmissions in the same time will not be allowed. |
| Ericsson | We are fine with the proposal and would prefer avoiding discussion regarding “in a slot”. All we need to say at this point is that the transmission is simultaneous on those multiple channels. Proposal 4 (II): - Sidelink transmission in multiple LBT bandwidths (e.g., wideband mode) simultaneously is supported for NR sidelink operation in the unlicensed spectrum - FFS details To conclude, a comment on priorities. It is great to have agreements on any topic and we should move quickly on the topics that are not controversial. But it is a bit strange to start R18 with an agreement on transmission over multiple channels when there is nothing else in place 😊 |
| MediaTek | We support the proposal in principle. Considering the comments above and the WID description, we may have the following updates on the proposal: (To be noted, except for the yellow part, the other words are actually from the WID with common understanding.) Proposal: Support sidelink channel access mechanism to enable sidelink transmission over the multiple LBT sub-bands simultaneously for a UE in the unlicensed spectrum based on regional regulation requirement and use the existing channel success schemes from NR-U as a starting point - FFS: details For the details, we may list them during the discussion for information but may not list them for the agreement in this meeting. For details, there could be some issues for further discussion later based on our understanding: - Whether one TB or multiple TBs are transmitted over multiple LBT sub-bands for a UE. - Whether the number of LBT sub-bands can be non-contiguous. - Whether to have the single LBT counter or multiple LBT counters for access over multiple LBT sub-bands. - Etc. |
| vivo | We support wording Ericsson, where “in a slot” is removed. We are favor of the simple proposal by FL, high level agreement is beneficial for quick progress. |
| Sharp | We support the version from Ericsson. “sidelink transmission” is a term well specified since Rel-16, so we don’t see any chance of ambiguity in removing “in a slot”. In our view it is actually more accurate to say “A sidelink transmission spanning multiple LBT bandwidths …”, but we fine to follow Ericsson’s version. |
| CATT/GOHIGH | We are supportive of the proposal by the FL. Also, we can accept the main bullet proposed by Samsung. However, considering that we haven’t reached any agreement regarding channel access mechanism, a general FFS may be more appropriate at this stage, as mentioned by Intel. Furthermore, we also propose to focus on the detail of other essential issues such as dynamic channel access mechanism and COT sharing in the following few meetings after we reach this early agreement. The details of multi-channel access mechanism can be discussed after the overall procedure of single-channel access mechanism is clear. |
| InterDigital | We support the revised proposal from Samsung. For now we should focus on channel access/transmitting on a single LBT sub-band. Simultaneous SL transmissions in multiple available LBT sub-bands can be discussed later. |
| Huawei, HiSilicon | We are fine with the direction of the proposal. Multi-channel access can be applied to achieve the broader bandwidth to adapt to the high date rate service, e.g. the XR service. On the main-bullet, we prefer the change from Samsung, since it more clearly claims the motivation to have the proposal, i.e. support channel access on multiple bandwidths for SL-U. On the sub-bullet, we also share the views with companies that it should contain more specific guidance to help the further study. Based on the description in the WID, channel access procedure from Rel-16 NR-U shall be reused. So no matter DL or UL multi-channel access is reused, no further enhancement is needed. The only remaining issue would be which procedure, DL or UL, should be reused for Rel-18 SL-U. Therefore, we suggest following changes on the revised version from Samsung. Proposal 4 (II) – Revised based on Samsung’s version: - Channel access procedures for transmission(s) on multiple channels are supported for NR sidelink operation on the unlicensed spectrum - Note: Type A and/or Type B DL multi-channel access procedures and/or UL multi-channel access procedures are considered as baseline for study, and enhancement for further study shall comply with regulation - FFS which multi-channel access procedure specified in Rel-16 for NR-U, DL or UL, is reused for SL-U |
| Transsion | We support the revised proposal from Ericsson in general. However, considering that the proposal mainly applies to wideband operation and that "LBT bandwidth" is not explicitly defined in the specification, we propose to change "LBT bandwidth" to "RB set". Proposal 4 (II): - Sidelink transmission in multiple RB sets (e.g., wideband mode) simultaneously is supported for NR sidelink operation in the unlicensed spectrum - FFS details |
| Futurewei | We support the updated proposal 4(II) from FL which is general enough. We are also ok with the minor update of removing “in a slot” suggested by other companies. |
| Qualcomm | We are in general ok with the spirit of the proposal, though we propose the following modifications to the FL updated version: - Channel access procedures for sidelink transmission(s) in multiple LBT bandwidths are supported for NR sidelink operation in the unlicensed spectrum - FFS details Specifically, we motivate our suggestions in the following: - “Channel access procedures for…” is proposed to be added, optionally. By following the line of thoughts of Samsung and Huawei, we believe that this bullet should serve the purpose of indicating procedures to be used to clear multiple LBT bandwidths. - In “transmission(s)”, the “(s)” is proposed to be added for two reasons: - Following the line of thinking of LGE: In the case of PSFCH, the UE can transmit multiple PSFCH in a given time - Following the line of thinking of Mediatek: We should not preclude in this phase the possibility of adopting the Rel-16 NR-U gNB behavior to multi-channel access, that is, the gNB after clearing the multi-channel LBT can transmit multiple TBs via multiple PDSCHs to different destination UEs. We believe it would be beneficial to study further the possibility for an SL-U UE to transmit multiple PSSCHs to multiple destination UEs. We understand that this would be contingent to UE implementation (which may not support it), still, we believe it should be FFS. - We are ok in leaving studies for this sub-topic generically included in the “FFS details” to not block early agreements - We propose the removal of “e.g. wideband mode” or at least a clarification of what it does mean, since a “wideband mode” is neither defined nor agreed. As per our comment 2.b above, we believe we should not preclude the SL-U UE to perform (potentially narrowband) separate transmissions to different UEs after performing a multi-channel access. - For the removal of “concurrently in a slot”, we believe that it unnecessarily restricts the scope of transmission actions, which we would like to keep open for FFS as we mention in our comment 2.b above. - Furthermore, we do not support proposals for adding the word “simultaneously”, since there is no reason to polarize the proposals towards UE all-or-nothing versions of multi-channel access. |
| DCM | We support the FL’s proposal. At the same time, updates by Ericsson or Transsion seem to be OK. We do not prefer Samsung’s/HW’s wording since whether the proposal means wideband operation or not is unclear. For example, TX is performed in LBT channel A at slot n, and another TX is performed in LBT channel B at slot n+1; this case is also included, which is not wideband operation. We prefer concrete proposal (i.e. proposals based on FL version). |

### Proposal for round 2

Main comments:

- Wording improvement on the meaning/intention of “in a slot” and it is not needed

- Considering that we haven’t reached any agreement regarding channel access mechanism, a general FFS may be more appropriate at this stage

FL comments/responses:

- For the main bullet, regarding “in a slot”, after double checking 37.213, I tend to agree with Samsung, Intel and others that it is better to reuse the language from the spec “channel access procedures for transmission(s) on multiple channels”.

  - It is FL’s understanding that as long the UE acquires COT in multiple channels and COT duration overlaps in time, the UE is allowed to transmit simultaneously on multiple channels.

- A note is added to clarify the definition of a channel, instead of using terms like LBT channel / bandwidths or others which cannot be found in 37.213.

- Not tracking changes in Proposal 4 (III) for easier reading as many terminologies have been updated.

Proposal 4 (III):

- Channel access procedures for transmission(s) on multiple channels are supported for NR sidelink operation

  - Note, the definition of a channel follows TS 37.213.

  - FFS whether the downlink, uplink and/or semi-static multiple channel access procedure(s) (if supported) from NR-U should be used as a baseline and how they are applied in SL mode 1 and mode 2 operation

| Company | Comments |
| --- | --- |
| ZTE,Sanechips | Support in general |
| LGE | We are generally OK with this proposal. One thing to change is that “whether/how they are applied in SL mode 1 and mode 2 operation”. In our understanding, if gNB cannot perform LBT procedure, it would be better to focus on single channel transmission at least for SL Mode 1 operation. |
| vivo | Support in general Why “semi-static multiple channel access procedure(s)” is emphasized, we just further study whether DL/UL as baseline. - FFS whether the downlink and/or uplink and/or semi-static multiple channel access procedure(s) (if supported) from NR-U should be used as a baseline and how they are applied in SL mode 1 and mode 2 operation |

### Proposal for Week 2 Monday GTW session

FL comments/responses:

- @vivo, In TS37.213, multiple channel access procedure for semi-static channel occupancy is supported in NR-U.

Proposal 4 (IV):

- Channel access procedures for transmission(s) on multiple channels are supported for NR sidelink operation

  - Note, the definition of a channel follows TS 37.213.

  - FFS whether the downlink, uplink and/or semi-static multiple channel access procedure(s) (if supported) from NR-U should be used as a baseline and whether/how they are applied in SL mode 1 and mode 2 operation

## [ACTIVE] Topic #5: Sidelink resource allocation Mode 1 and Mode 2

Background: According to the objective for SL-U in the WID [36], RAN1 should study and assess the applicability of both existing SL mode 1 and mode 2 resource allocation Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation. To this end, numerous papers have been submitted to this meeting discussing how the existing RA modes can be used in conjunction with especially LBE-based Type 1 and Type 2 LBT (some suggested enhancements are captured in Section 5.8), where mode 1 is referred as gNB provides DG and CG sidelink grants to UEs for SL transmissions (much like UL scheduling) and mode 2 is referred as UE performs resource selection and reservation based on a sidelink full/partial sensing procedure (not LBT sensing), random selection or combined with inter-UE coordination (preferred/non-preferred/colliding resources).

In summary, since mode 1 RA mechanism behaves much like UL scheduling, where gNB provides a transmission resource grant and UE transmits on the carrier using the scheduled resources, it is observed no input submitted in this meeting has questioned the applicability of reusing the existing SL mode 1 RA in the unlicensed spectrum.

- Although the WID objective limits the Uu operation for mode 1 to be on the licensed spectrum only, the scope of this work is unclear whether the gNB is capable of performing LBT on the unlicensed carrier for which the SL transmission grant is provided. If gNB is capable of performing LBT on the unlicensed carrier (much like Rel-16 NR-U operation in 5GHz and 6GHz bands), the gNB could directly indicate in the SL scheduling DCI the LBT scheme for which the UE should perform before its scheduled SL transmission. Otherwise, the UE should perform Type 1 LBT to obtain channel access and a COT for its transmissions or use a shared COT from another UE. Therefore, the scope of mode 1 RA should be clarified in terms of assumed capability of the gNB.

For sidelink RA mode 2, many companies expressed a view that the current SL sensing and reservation scheme is a transmission mechanism to avoid conflicts among SL users (intra-RAT collision avoidance) and LBT is a mechanism to avoid inter-RAT collisions (e.g., between SL and WiFi transmissions). The SL sensing (long sensing) decodes SCI and measures RSRP for selecting future transmission resources and LBT (short sensing) based on energy detection is performed just before the transmission using the selected resources to gain access to the unlicensed carrier. Since they are designed and used for different purposes (intra- and inter-RAT collision avoidance), they are not competing sensing schemes for which RAN1 should only adopt one of them as such. In fact, many said they can be used together in a complimentary manner. nevertheless, the following options of jointly using mode 2 RA and LBT are considered based on review of contributions.

- Option 1: SL resource selection is performed based on existing R16/R17 sidelink mode 2 RA, and LBT is performed before SL transmission on the selected/reserved resources

- Option 2: Type 1 LBT is performed by the UE and SL resource(s) can be selected within the COT

- Option 3: UE performs joint LBT sensing results and resource selection procedure

Due to a regulation set out by FCC in the US, two companies/institutes expressed that SL communication using mode 2 RA is not applicable / allowed in the 6GHz unlicensed spectrum (band n96 in 5.925–7.125 GHz) without the control of a standard power access point, indoor access point or subordinate devices and client devices are prohibited from connecting directly to another client device. However, one of the papers also pointed out this restriction in the 6GHz band does not apply to Europe. Other regions/parts of the world may not have the same restriction as the US. In this understanding, equipment manufacturers should always ensure that they comply to all applicable regulations in the region/country they are operating. And 3GPP RAN1 should continue to specify SL mode 2 RA scheme for regions / countries that do not have the same restriction. Furthermore, it should be clarified that a SL UE can operate in the mode 2 RA scheme and still fully under the control of its connecting / serving gNB or eNB according to existing R16/R17 specifications. In this case, the operating sidelink bandwidth part and Tx/Rx resource pools are fully configured by the base station.

In the existing mode 2 resource sensing and selection, the procedure is carried out on a per resource pool basis. In Rel-18, depending on how a TX resource pool is defined / configured for the unlicensed band (e.g., only within a 20MHz LBT bandwidth / channel or across multiple LBT BWs/channels), UE selection of resources may be different. Therefore, it is worthwhile to study further whether the mode 2 resource selection procedure needs to be updated / enhanced due to multi-channel access.

Finally, as summarized in Section 5.8, there is a lot of interests in studying multi-consecutive slots transmission (also often referred as burst, back-to-back or just multi-slot transmission) in Rel-18 for sidelink operation in the unlicensed spectrum due to the following two main motivations:

- COT retaining to avoid long gap (at slot level) between SL transmissions

- Improve channel access efficiency by reducing number of channel accesses for transmitting a TB or multiple TBs

In order to enable / introduce “multi-consecutive slots TX” in sidelink, the work will involve multiple aspects including channel access (at slot boundary), resource allocation (sensing, selection and reservation), and PHY channel design (PSCCH/PSSCH mapping, rate matching), and hence, spanning across both channel access and PHY design framework AIs. As coordinated between the FLs for these two AIs, it is decided for now this topic will be firstly handled in the channel access agenda. If this enhancement feature is to be supported, the FLs will further discuss whether it should be handled sorely within one AI (to avoid duplicated discussions) or it can be treated under both AIs (if remaining design issues are mutually independent). Since this feature has not been investigated by all companies, it is proposed to study this further including the above-mentioned aspects.

Proposal 5 (I):

- The existing sidelink mode 1 RA including dynamic grant, Type 1 and Type 2 configured grants are supported for sidelink operation in unlicensed spectrum. SL UE performs Type 1 or Type 2 LBT before SL transmission using the allocated resource(s).

  - FFS whether gNB is assumed to be capable of performing LBT on the scheduling unlicensed carrier in Rel-18 such that the LBT scheme to be performed by UE is directly indicated in the SL scheduling DCI.

- The existing sidelink mode 2 RA schemes (including full sensing, partial sensing, random resource selection and inter-UE coordination) are supported for sidelink operation in unlicensed spectrum, subject to applicable regional regulations. SL UE performs Type 1 or Type 2 LBT before SL transmission using the selected and reserved resources.

  - FFS whether one or more or combination of the following options should be supported

    - Option 1: SL resource selection is performed based on existing R16/R17 sidelink mode 2 RA, and LBT is performed before SL transmission on the selected/reserved resources

    - Option 2: Type 1 LBT is performed by the UE and SL resource(s) can be (re-)selected within the COT

    - Option 3: UE performs joint LBT sensing results and resource selection procedure

  - FFS whether mode 2 resource selection procedure needs to be updated / enhanced due to multi-channel access

- FFS whether/how multi-consecutive slots transmission can be supported for NR sidelink operation in unlicensed spectrum, including the following aspects

  - channel access, resource allocation and PHY channel design

| Company | Comments |
| --- | --- |
| InterDigital | We think first FFS should be removed. As we mentioned above, we do not assume that the gNB can operate in unlicensed spectrum. We can add an FFS on LBT failure impact on SL Mode 1 resource allocation. We think other enhancements should not be precluded at this stage. |
| Intel | At this stage of this WI, we would prefer not to preclude other enhancements (e.g., multiple starting positions within a slot, etc..) and to agree on the existing SL RAs as a baseline. Furthermore, the sentence related to when LBT is applied compared to the SL transmission could be interpreted as if the LBT is always applied at the symbol level, and should be further studied whether this is the case or whether CP extension or other methods are applied. Therefore, the following edits are suggested: - The existing sidelink mode 1 RA including dynamic grant, Type 1 and Type 2 configured grants are supported as a baseline for sidelink operation in unlicensed spectrum. In dynamic channel access mode, SL UE performs Type 1 or any of the Type 2 LBTs before any SL transmission using the allocated resource(s) FFS: whether additional transmission is needed between the end of the LBT procedure and the start of the SL transmission. - The existing sidelink mode 2 RA schemes (including full sensing, partial sensing, random resource selection and inter-UE coordination) are supported as a baseline for sidelink operation in unlicensed spectrum, subject to applicable regional regulations. In dynamic channel access mode, SL UE performs Type 1 or any of the Type 2 LBTs before any SL transmission using the allocated resource(s) FFS: whether additional transmission is needed between the end of the LBT procedure and the start of the SL transmission. |
| LGE | The benefit of Mode 1 RA is that all the resources are under gNB’s control. Meanwhile, if gNB cannot know the status of unlicensed band, gNB’s SL scheduling on unlicensed band would be inaccurate. From our side, at least gNB can still perform NR-U operation or Uu link, and can consider it to schedule SL resources. Without this knowledge, there is no need to support Mode 1 RA for SL-U. It is unclear how gNB can decide SL resources without the knowledge of the unlicensed band. In summary, whether or not to support gNB-to-UE COT sharing for SL is different issue even though we allow that gNB performs LBT on unlicensed band. There is no reason to restrict gNB operation or capability. We think that additional FFS is not needed in this stage. |
| OPPO | We are fine with the proposal. For the first FFS in first bullet, we agree with InterDigital to remove it since it is out of scope. |
| Futurewei | We are ok with this proposal in general. However, we are not clear on option 3 how UE performs joint LBT sensing results and resource selection. Some clarification or elaboration is needed before we can include it as an option Also we would like to include an FFS on how to handle LBT failure for resource re-selection for the second bullet. - The existing sidelink mode 2 RA schemes (including full sensing, partial sensing, random resource selection and inter-UE coordination) are supported for sidelink operation in unlicensed spectrum, subject to applicable regional regulations. SL UE performs Type 1 or Type 2 LBT before SL transmission using the selected and reserved resources. - FFS whether one or more or combination of the following options should be supported - Option 1: SL resource selection is performed based on existing R16/R17 sidelink mode 2 RA, and LBT is performed before SL transmission on the selected/reserved resources - Option 2: Type 1 LBT is performed by the UE and SL resource(s) can be (re-)selected within the COT - Option 3: UE performs joint LBT sensing results and resource selection procedure - FFS whether mode 2 resource selection procedure needs to be updated / enhanced due to multi-channel access - FFS whether and how to handle LBT failure for resource (re)selection. |
| Qualcomm | - For mode 1 RA, we support DG and CG (both types) with the UE accessing with either Type 1 or Type 2 LBT. We also support studying further whether the gNB can perform LBT over the unlicensed carrier, schedule the UE via signaling over licensed carrier, and share its COT with that UE over the scheduled unlicensed carrier. Specifically, it should be studied what is required on the gNB side to start a COT (e.g. DL data transmission, SSB, etc.) in order to share it in a following slot with an SL-U UE to perform UE-to-UE communications. - For mode 2 RA, we support further studying solutions that can facilitate meeting the targets of efficient channel access and high throughput in the unlicensed spectrum. Specifically, while we support operations including full/partial sensing, random resource selection, inter UE coordination, and reservation, we deem as important to revisit the mode 2 RA at least in some aspects, including but not restricted to a) the possibility of reserving a higher number of contiguous resources in time/frequency domain, b) the possibility of reclaiming resources previously reserved by other UEs. Those considerations would be of high importance in regions with uncontrolled interference due to the presence of other RATs, to ensure 1) that a UE can access the channel as soon as possible after completing LBT (to avoid other nodes to jump-in), 2) potentially have multiple LBT opportunities per each resource-selection (to avoid delay due to re-selection procedure when LBT fails), and 3) access for as long as possible (multiple contiguous resources) to reduce the frequency of LBT attempts and the possibility of failures. If RAN 1 decides to support mode 2 resource allocation as it is in Rel-16/Rel-17, it would be desirable, as an alternative to mode 2 enhancements, to have another mode of operation controlled by a higher layer parameter to disable resource selection and reservation, so that SL-U can operate unlicensed access based on LBT only in harsh interference scenarios. This would be contingent to other enhancements for channel access as those described in our comment to Topic #3 of the FL summary (access with random CPE for micro-second level Tx start points to enable intra-RAT collision resolution via LBT). Regarding the options proposed by the FL: - Option 1: We support studying and specifying mode 2 RA with enhancements as described above, to be performed before LBT, which is then performed based on the selected/reserved resources. - Option 2: We do not support this option, as we think it would be difficult to select resources after completing LBT, due to the need to occupy the medium as early as possible to not lose the COT to another device that may jumps-in. - Option 3: We do not support this option, as it is unclear how resource allocation and LBT can be performed jointly. - We do support studying further mode 2 resource selection for multi-channel access, which in our view lies in the space of study jointly LBT and resource selection. We do support consecutive transmissions over multiple slots, which in our view lies in the space of study of joint LBT, resource selection, and phy. design |
| Panasonic | For the 1st bullet, how gNB can detect/know the status of unlicensed band should be concluded before to agree this. For the 2nd bullet, we support the proposal. |
| CMCC | Comments. For mode 1, similar to proposal 3, we are not quite clear whether gNB can share a COT to UE because gNB cannot transmit and occupy the unlicensed channel on sidelink, then we think only UE can initiate a COT in this case, so we think more clarifications are needed. For mode 2, we are OK about option 1; Besides, does option 2 mean that UE should first do LBT and then perform resource selection? For option 3, we think this is a more general option in which option 1 and 2 can be covered by it, so maybe clarifications are needed for why we need such an option. We are OK about the remaining parts. |
| vivo | For mode 1, we agree the main bullet. For the subbullet, based on the WID, we assume gNB does not perform LBT for UE, we suggest to remove the subbullet. - FFS whether gNB is assumed to be capable of performing LBT on the scheduling unlicensed carrier in Rel-18 such that the LBT scheme to be performed by UE is directly indicated in the SL scheduling DCI. For mode 2, we assume some enhancement can be supported based on based on LBT procedure, multi-consecutive slots, based on COT sharing. Our preference to list the high level aspects. - The existing sidelink mode 2 RA schemes (including full sensing, partial sensing, random resource selection and inter-UE coordination) are supported for sidelink operation in unlicensed spectrum, subject to applicable regional regulations. SL UE performs Type 1 or Type 2 LBT before SL transmission using the selected and reserved resources. - FFS whether one or more or combination of the following options should be supported - Option 1: SL resource selection is performed based on existing R16/R17 sidelink mode 2 RA, and LBT is performed before SL transmission on the selected/reserved resources - Option 2: Type 1 LBT is performed by the UE and SL resource(s) can be (re-)selected within the COT - Option 3: UE performs joint LBT sensing results and resource selection procedure - FFS whether mode 2 resource selection procedure needs to be updated / enhanced due to channel access procedure, including multi-channel access - FFS whether/how multi-consecutive slots transmission can be supported for NR sidelink operation in unlicensed spectrum, including the following aspects - channel access, resource allocation and PHY channel design The COT sharing feature is applicable to both mode 1 and mode 2, we prefer to add the following bullet - FFS resource allocation enhancement based on shared COT |
| ZTE, Sanechips | Support in general. We share similar view with InterDigital that the first FFS is not needed. For the relationship b/w Mode 2 RA and LBT, for Option 1, it follows the similar procedure as NR-U and Mode 1 RA, i.e., get a grant resource for transmission, and then perform LBT before the resource. But for option 2/3, the procedure and UE behavior is unclear to us, and we prefer to support Option 1 as a baseline, and FFS to other options. In addition, we would prefer to have an FFS bullet on whether to use the same resource allocation mechanism for those resources within or outside COT. |
| Sharp | Same understanding as LGE regarding Mode 1 resource allocation. We think it is very important to reach consensus in RAN1#109-e on interpretation of the sentence “Uu operation for mode 1 is limited to licensed spectrum only” in the WID. |
| CableLabs | We generally do not support LBT Cat 1 for coexistent access in unlicensed spectra, due to the aggressive behavior of this type of LBT towards coexistent access technologies. However, we agree with LBT Cat1 exceptions provided by NR-U TS37.213 Type 2C (e.g. #4.1.2.3, 4.2.1.2.3) |
| xiaomi | We are generally fine with the proposal. - For first bullet, The FFS subbullet shall be removed. - For second bullet, The description of option 3 is not very clear. Generally speaking, both option 1 and 2 can be deemed as special option 3 solution. If the FL intention is to not preclude other potential options, we suggest to use “other options are not precluded.” |
| Lenovo | We are generally fine with the proposal and ask to update the first FFS bullet and support adding LBT failure and consistent LBT failure handling in the FFS as it may impact the resource selection mechanism - FFS whether gNB is assumed to be capable of obtaining LBT result on the scheduling unlicensed carrier in Rel-18 such that the LBT scheme to be performed by UE is directly indicated in the SL scheduling DCI. - FFS whether and how to handle LBT failure for resource (re)selection. - FFS whether and how to handle consistent LBT failure |
| NTT DOCOMO | Support the 1st bullet. Support the 2nd bullet, but we do not support Option 2/3. In option 2, even LBT is successful, the UE does not start TX at that time. This means COT is not obtained; thus selection at the virtual COT would be meaningless. Option 3 is quite unclear. It seems that option 1 is only feasible option. We do not support the last bullet. If UE would like to transmit in consecutive slots, the UE can indicate those slots and use without any spec impact on resource allocation procedure. Further modification like TBoMS without gap/AGC symbols is not preferable from spec impact perspective. |
| Spreadtrum | We are fine with the proposal except the 1st FFS According to the WID, Uu operation for mode 1 is limited to licensed spectrum only. So we prefer to remove it. |
| Fujitsu | We support Proposal 5 in general. As for mode 2 RA, 3 options are listed as FFS. For Option 1, it may not be necessary to highlight “based on existing R16/R17 mode 2 RA”. This is because mode 2 RA may also need enhancements due to multi-channel access or multi-consecutive slots transmission. For Option 3, “joint LBT sensing results and resource selection procedure” may need more explanations. - Option 1: SL resource selection is performed based on existing R16/R17 sidelink mode 2 RA, and LBT is performed before SL transmission on the selected/reserved resources - Option 2: Type 1 LBT is performed by the UE and SL resource(s) can be (re-)selected within the COT Option 3: UE performs joint LBT sensing results and resource selection procedure |
| CATT/GOHIGH | For the first bullet, as we mentioned before, mode-1 related issues would better be postponed at this stage. For the second main bullet, we think at least full sensing and random selection can be supported in SL-U. Regarding partial sensing and inter-UE coordination, further study is needed for the applicability in SL-U. For the 1st FFS of the second bullet, three options are listed. From our understanding, Option 1 corresponds to the procedure that resource selection is performed before LBT, while Option 2 corresponds to LBT performed before resource selection. Option 3 seems to say that LBT and resource selection are carried out simultaneously. Considering the support of these three options needs further discussed, the last sentence of the main bullet, which already determines that resource selection is performed before Type 1 or Type 2 LBT, should be deleted. For the 2nd FFS, “multi-channel access” should be revised to “channel access mechanism”, since mode 2 resource selection procedure may be enhanced not only due to multi-channel access but also due to dynamic/semi-static channel access mechanisms. The revised second bullet is proposed as follows: - The existing sidelink mode 2 RA schemes (at least including full sensing, partial sensing, and random resource selection and inter-UE coordination) are supported for sidelink operation in unlicensed spectrum, subject to applicable regional regulations. SL UE performs Type 1 or Type 2 LBT before SL transmission using the selected and reserved resources. - FFS whether one or more or combination of the following options should be supported - Option 1: SL resource selection is performed based on existing R16/R17 sidelink mode 2 RA, and LBT is performed before SL transmission on the selected/reserved resources - Option 2: Type 1 LBT is performed by the UE and SL resource(s) can be (re-)selected within the COT - Option 3: UE performs joint LBT sensing results and resource selection procedure - FFS whether mode 2 resource selection procedure needs to be updated / enhanced due to multi-channel access mechanisms. We are fine with the third bullet. |
| Sony | For the first bullet, we also would like to add to consider LBT failure impact on SL Mode 1 resource allocation as mentioned by InterDigital. For the second bullet, we support to keep first and second options in the proposal at this stage. We may need to clarify third option. |
| Samsung | For 1st FFS sub-bullet for mode 1, we have similar comment as in topic #3 that gNB may not be able to initiate and share a COT to SL UEs, but it may acquire information of a UE initiated/shared COT and performs mode 1 scheduling accordingly. We prefer at least to discuss the latter case. For feasibility of the former case, we can follow majority understanding. For the 2nd and 3rd bullets, we agree with the principle and support to study the FFS points. In addition, we support the sub-bullet added by Futurewei: “FFS whether and how to handle LBT failure for resource (re)selection.” |
| Nokia, NSB | In our view, the FFS on whether a gNB is assumed to be capable of performing LBT or not should not be included in the proposal. Since even if the gNB is able to perform LBT (e.g. LBT Type 1) it will not be allowed (per WID restriction) to perform a transmission in unlicensed spectrum and a acquire the COT. Therefore, the decision of the LBT Type to apply should be left to the UE to decide. The remaining points of the proposal are ok to be left for FFS. |
| Fraunhofer | We are supportive of the main bullets, and have the following comments: For Mode 1, we share the same concern as LG, where it is unclear how the gNB can provide grants on the unlicensed band if the gNB is not aware of the prevalent conditions of the said unlicensed band. We agree with Sharp that the interpretation of the WID has to be clarified. For Mode 2, we support Option 1. For Option 2, it is unclear how the UE can perform LBT first and then carry out resource (re-)selection without unnecessarily occupying/holding the channel. It is also unclear how Option 3 intends to work. |
| Transsion | Regarding the first sub-bullet, we prefer to decouple gNB sensing and the indication of channel access type. This is because we believe that there is a time gap between the LBT sensing of the gNB and the indication of the channel access type. When the SL UE receives the channel access type indication from the gNB, the channel state may change and the channel access type indication other than type 1 is not suitable. However, the joint indication of the channel access type and CPE is beneficial for consecutive transmission. Therefore, we do not think it is necessary to bind LBT sensing of the gNB and the indication of the channel access type. So we propose to change the sub-bullet to - “FFS whether gNB is assumed to be capable of performing LBT on the scheduling unlicensed carrier in Rel-18 such that the LBT scheme to be performed by UE is directly indicated in the SL scheduling DCI.” |
| Ericsson | For this proposal, we have the following comments: - Regarding the bullets related to mode 1 RA: - We think it is reasonable to defer the study until SL aspects are clearer. - As indicated in the WID, Uu operation is limited to licensed spectrum, having the gNB perform LBT is not reasonable. We think the FFS has to be removed. - For the bullets related to mode 2 RA: - Support for the different mode 2 flavours (full/partial/no sensing) should be studied further. - Support for inter-UE coordination seems hard to justify, at least for Scheme 1, given that we do not even have a common framework for Mode 2 and LBT. We suggest focusing the work a bit by removing this part. - We think the details of the combination of channel access (LBT, COT sharing) and mode 2 (sensing, reservations) must be FFS at this point. We propose to update the proposal to: Proposal 5 (I): - The existing sidelink mode 1 RA including dynamic grant, Type 1 and Type 2 configured grants are supported for sidelink operation in unlicensed spectrum. - SL UE performs Type 1 or Type 2 LBT before SL transmission using the allocated resource(s). FFS details - Mode 2 RA schemes are supported for sidelink operation in unlicensed spectrum, subject to applicable regional regulations. - SL UE performs Type 1 or Type 2 LBT before SL transmission using the selected and reserved resources. FFS how to combine them. - FFS other channel access aspects (including COT sharing, etc.) - FFS which of the following options to support: full sensing, partial sensing, random resource selection - FFS other enhancements |
| NEC | In general, we agree with the proposal, except for the following issues: - For mode 1, gNB should not operate on unlicensed band for sidelink; - For mode 2, discuss SL-U schemes based on Rel-16 sidelink features, and postpone Rel-17 sidelink for later discussion. |
| Mediatek | We are generally fine with the proposal but with the following comments/updates: For Mode 1 RA - As mentioned by InterDigital, we also think the FSS is out of the scope. For Mode 2 RA - For the 1st FFS - We support Option 1. In this case, we think the starting point of the resource selection window should take into account the potential LBT operation time in addition to the original processing time T1. - We also think the Option 2 should be supported. We think determine the starting point of the selection window according to the LBT success occasion can avoid the invalid resource selection. - For Option 3, we think there need more clarification about “joint LBT sensing results and resource selection procedure”. What is the actual meaning of “joint”? Besides, we also have some other comments: - We agree with Futurewei, the mechanism to combat the potential LBT failure should be FFS. - For both Mode 1 and Mode 2 RA, the timeline impact due to the introduction of the LBT sensing should be FFS. |
| Huawei, HiSilicon | We are generally fine with the proposal, but with some comments as below. Simply performing LBT before granted resource in mode 1 or selected resource in mode 2, if without enhancement to address issues for operating in unlicensed band, would lead to resources unavailability caused by inter-UE LBT blocking. For example, as shown in the figure below, assuming gNB allocates resources to UE1 and UE2 in mode 1, or, UE1 and UE2 select their own resources in mode 2, in a legacy manner. After performing LBT before each allocated/selected resource, if UE1 successfully transmits on the 1st Tx resource, then such transmission will possibly impact UE2’s LBT procedure before UE2’s 1st TX resource and cause LBT failure. Thus, both mode 1 and mode 2 needs to consider how to alleviate the resources unavailability caused by LBT blocking. We suggest to add FFS point on this. For mode 2, regarding the FFS on options for the relationship between LBT and resource allocation: option 1 may have some drawbacks, e.g., the LBT blocking issue may happen. For example, as shown in the figure below, it’s possible that other high priority UE2 and UE3 reserved R4 and R5, which are close to UE1’s selected resources R1 and R2. If UE1 transmits on R1 and R2 as in Option 1, it may possibly impact UE2’s and UE3’s LBT procedure before R4 and R5, and cause LBT failure for UE2 and UE3, respectively. Such LBT blocking issue may decrease performance, especially for high priority UEs. Therefore, further study is necessary on solving such issues. One potential way is COT sharing, i.e., if UE1 can share COT with UE2 and UE3, then all of them can access the channel. Another way might be deferring channel access, e.g., UE1 can defer channel access by holding on LBT counter N or choose not to transmit immediately when counter N=0 as specified in TS 37.213, which allows UE2 and UE3 to access firstly. In general, we suggest RAN1 to further study the relationship between reservation-based resource selection and LBT, including how to address LBT blocking issue such that the reserved resources of different UEs can be efficiently used. It’s premature down-select options at this stage. Other comments: - For mode 1, as explained under Topic#3, we suggest not considering “gNB performs LBT”, so this FFS point needs to be removed. - For mode 2, we suggest to add “…the selected and/or reserved resources …” to be more accurate. Because in legacy sidelink, unreserved transmission is possible (i.e., the UE does not send SCI in the past to reserve some resource, e.g., for aperiodic traffic). - For mode 2, “inter-UE coordination” is not a RA scheme, so it should be removed. Suggested changes on the proposal are coloured in the red below: Proposal 5 (I): - The existing sidelink mode 1 RA including dynamic grant, Type 1 and Type 2 configured grants are supported for sidelink operation in unlicensed spectrum. SL UE performs Type 1 or Type 2 LBT before SL transmission using the allocated resource(s). - FFS whether gNB is assumed to be capable of performing LBT on the scheduling unlicensed carrier in Rel-18 such that the LBT scheme to be performed by UE is directly indicated in the SL scheduling DCI. - FFS how to address LBT blocking issue - The existing sidelink mode 2 RA schemes (including full sensing, partial sensing, random resource selection and inter-UE coordination) are supported for sidelink operation in unlicensed spectrum, subject to applicable regional regulations. SL UE performs Type 1 or Type 2 LBT before SL transmission using the selected and/or reserved resources. - FFS whether one or more or combination of the following options should be supported - Option 1: SL resource selection is performed based on existing R16/R17 sidelink mode 2 RA, and LBT is performed before SL transmission on the selected/reserved resources - Option 2: Type 1 LBT is performed by the UE and SL resource(s) can be (re-)selected within the COT - Option 3: UE performs joint LBT sensing results and resource selection procedure - FFS the relationship between resource selection and LBT to address LBT blocking issue - FFS whether mode 2 resource selection procedure needs to be updated / enhanced due to multi-channel access - FFS whether/how multi-consecutive slots transmission can be supported for NR sidelink operation in unlicensed spectrum, including the following aspects - channel access, resource allocation and PHY channel design |
| Apple | At this stage, the two main bullets are OK, the FFS in sub-bullets under mode 1 and mode 2 can be removed, essentially keep all details open. Suggest separating mode 1 and mode 2 discussion from multi-TTI transmission discussion. They are separate issues. For mode 2, we observe it works better when there is no WiFi, LAA/eLAA/NR-U (either same operator or different operators) working in the same carrier. Otherwise, the SL-U do not perform channel access procedures while other devices do, SL-U transmission will have reduced opportunities to transmit. This is similar for FBE which works in the case when other technology is not presented. We can consider enable mode 2 with FBE, instead of optimizing mode 2 for LBE. |
| WILUS | We are supportive of three main bullets in general. We think that the 1st FFS point should be removed as considering “Uu operation for mode 1 is limited to licensed spectrum only” in the WID. |
| Bosch | We are fine with the proposal. For first sub-bullet, we tend not to agree with the 1st FFS. For second sub-bullet, we prefer option 1, but we are fine with other options. For the last sub-bullet, better to have FFS details. |

### Proposal for round 2

Main comments:

1. Sidelink mode 1 RA,

- Main bullet

  - Add an FFS on LBT failure impact on SL Mode 1 resource allocation

  - The sentence related to when LBT is applied compared to the SL transmission could be interpreted as if the LBT is always applied at the symbol level, and should be further studied whether this is the case or whether CP extension or other methods are applied

  - Defer the study until SL aspects are clearer

- FFS (gNB performing LBT on the shared channel(s))

  - Remove / not support / unclear: InterDigital, OPPO, vivo, Futurewei, CMCC, ZTE/SC, xiaomi, Spreadtrum, CATT/GH, Samsung, Nokia/NSB, Transsion, Ericsson, NEC, MediaTek, HW/HiSi, Apple, WILUS, Bosch

  - Support: LGE (baseline, remove), Qualcomm, Sharp, Lenovo, Fraunhofer

2. Sidelink mode 2 RA,

- Main bullet

  - The sentence related to when LBT is applied compared to the SL transmission could be interpreted as if the LBT is always applied at the symbol level, and should be further studied whether this is the case or whether CP extension or other methods are applied

  - We observe it works better when there is no WiFi, LAA/eLAA/NR-U (either same operator or different operators) working in the same carrier. Otherwise, the SL-U do not perform channel access procedures while other devices do, SL-U transmission will have reduced opportunities to transmit. This is similar for FBE which works in the case when other technology is not presented. We can consider enable mode 2 with FBE, instead of optimizing mode 2 for LBE.

- 1st FFS (options of relationship between mode 2 resource selection and LBT)

  - On option 3, how UE performs joint LBT sensing results and resource selection

  - Include an FFS on how to handle LBT failure for resource re-selection for the second bullet

  - Does option 2 mean that UE should first do LBT and then perform resource selection? For option 3, we think this is a more general option in which option 1 and 2 can be covered by it, so maybe clarifications are needed for why we need such an option

  - Preference is to list the high-level aspects: “FFS whether mode 2 resource selection procedure needs to be updated / enhanced due to channel access procedure, including multi-channel access”

  - FFS bullet on whether to use the same resource allocation mechanism for those resources within or outside COT

  - Suggest to use “other options are not precluded.”

  - In option 2, even LBT is successful, the UE does not start TX at that time. This means COT is not obtained; thus selection at the virtual COT would be meaningless. Option 3 is quite unclear.

  - For Option 1, it may not be necessary to highlight “based on existing R16/R17 mode 2 RA”. This is because mode 2 RA may also need enhancements due to multi-channel access or multi-consecutive slots transmission.

  - Regarding partial sensing and inter-UE coordination, further study is needed for the applicability in SL-U.

  - We think the details of the combination of channel access (LBT, COT sharing) and mode 2 (sensing, reservations) must be FFS at this point

- 2nd FFS (mode 2 enhancement for multi-channel access)

3. FFS on multi-consecutive slots transmission

- Main bullet

  - If UE would like to transmit in consecutive slots, the UE can indicate those slots and use without any spec impact on resource allocation procedure. Further modification like TBoMS without gap/AGC symbols is not preferable from spec impact perspective.

4. Others

- We generally do not support LBT Cat 1 for coexistent access in unlicensed spectra, due to the aggressive behavior of this type of LBT towards coexistent access technologies. However, we agree with LBT Cat1 exceptions provided by NR-U TS37.213 Type 2C (e.g. #4.1.2.3, 4.2.1.2.3)

FL comments/responses:

- @CMCC, for mode 2 option 2, yes it means UE can perform LBT first and then resource selection to select resource(s) within the acquired COT. Please refer to LGE and Ericsson contributions in this meeting.

- @Futurewei, CMCC, Fujitsu, Sony, Fraunhofer, MediaTek, for mode 2 option 3, one possible scheme is during resource (re)selection the COT information from the UE itself or another UE can be taken into consideration. Another possible scheme is to take into account of multi-channel access procedure outcome during the resource (re)selection.

- @CATT, thank you for your good points on the three options under mode 2 RA. On Option 2, I am thinking even through the option described that resource selection can be done after performing LBT to obtain COT, but it is still likely that the UE needs to perform a Type 2 LBT (or even Type 1) just before the transmission when a transmission gap is 16us or longer. In this sense, the main bullet does not contradict with the mechanism in Option 2. Similarly, the same can happen with Option 3.

- @ALL,

  - The issue on whether gNB is assumed to be capable of performing LBT on the shared carrier in Rel-18 in mode 1 RA, this issue is already included in Topic #3. Hence, removed from this topic.

  - Considering many questions and unclear usage on Option 2 and Option 3 in the 1st round, and also quite a few suggestions to include other areas for updates and enhancements, I try to merge them using more generic wording in the first meeting for further study. Although the exact scenarios directions are not listed in the updated proposal, it is encouraged to everyone to study the options that has been discussed in this meeting and consider each other’s points until the next meeting.

  - Discussion of whether sidelink mode 2 operation can be supported for band n96 in the US due to local FCC regulation stating that: “In the 5.925-7.125 GHz band, client devices, except fixed client devices, must operate under the control of a standard power access point, indoor access point or subordinate devices; Subordinate devices must operate under the control of an indoor access point. … Client devices are prohibited from connecting directly to another client device.” This has been requested to be studied and discussed by two companies during the 1st round of discussion under Topic #1. I think it is important to discuss and align on a common understanding within this group. Hence, an FFS bullet is added to address this. Some background information can be found in the contribution summary section (5.1) and at the beginning of this Topic #5.

Proposal 5 (II):

- The existing sidelink mode 1 RA including dynamic grant, Type 1 and Type 2 configured grants are supported as a baseline for sidelink operation in shared spectrum. In dynamic channel access, SL UE performs Type 1 or one of the Type 2 LBTs before SL transmission using the allocated resource(s).

  - FFS whether gNB is assumed to be capable of performing LBT on the scheduling unlicensed carrier in Rel-18 such that the LBT scheme to be performed by UE is directly indicated in the SL scheduling DCI.

  - FFS whether/how to handle LBT inter-UE blocking and LBT failures issues in mode 1 RA

- The existing sidelink mode 2 RA schemes (including full sensing, partial sensing, random resource selection and inter-UE coordination) are supported as a baseline for sidelink operation in shared spectrum, subject to applicable regional regulations. In dynamic channel access, SL UE performs Type 1 or one of the Type 2 LBTs before SL transmission using the selected and reserved resources.

  - FFS whether one or more or combination of the following options should be supported

    - Option 1: SL resource selection is performed based on existing R16/R17 sidelink mode 2 RA, and LBT is performed before SL transmission on the selected/reserved resources

    - Option 2: Type 1 LBT is performed by the UE and SL resource(s) can be (re-)selected within the COT

    - Option 3: UE performs joint LBT sensing results and resource selection procedure

  - FFS whether mode 2 resource selection procedure needs to be updated / enhanced due to channel access, including LBT inter-UE blocking, LBT failures, multi-channel access, COT sharing, LBT sensing results, etc.

  - FFS whether sidelink mode 2 operation can be supported / used for band n96 (5.925-7.125 GHz) in the US due to local FCC regulation

- FFS whether/how multi-consecutive slots transmission can be supported for NR sidelink operation in unlicensed spectrum, including the following aspects

  - channel access, resource allocation and PHY channel design

- FFS whether enhancement is needed between the end of the LBT procedure and the start of the SL transmission to retain channel access

| Company | Comments |
| --- | --- |
| ZTE,Sanechips | Support in general |
| LGE | On 1st bullet, since companies have different understanding on whether gNB can perform LBT procedure or not, we need to put “Type 2 LBT” as FFS. The feasibility should be studied first for type 2 LBT for Mode 1 RA. |
| vivo | 1.Semi-static and dynamic access have been discussed in another proposal, we prefer to remove the restriction on dynamic access for mode 1 and mode 2 2.For mode 1, enhancement for CG/DG should be further studied as well, where legacy CG/DG as baseline. 3.COT sharing should be discussed for mode 1 as well, in order that gNB can schedule multiple UEs in a shared COT. 4.The intention last FFS is not clear. Is it a similar concept as “CPE”? Proposal 5 (II): - The existing sidelink mode 1 RA including dynamic grant, Type 1 and Type 2 configured grants are supported as a baseline for sidelink operation in shared spectrum. In dynamic channel access, SL UE performs Type 1 or one of the Type 2 LBTs before SL transmission using the allocated resource(s). - FFS whether gNB is assumed to be capable of performing LBT on the scheduling unlicensed carrier in Rel-18 such that the LBT scheme to be performed by UE is directly indicated in the SL scheduling DCI. - FFS whether/how to handle LBT inter-UE blocking, COT sharing and LBT failures issues in mode 1 RA - FFS potential enhancement to SL DG/ CG - The existing sidelink mode 2 RA schemes (including full sensing, partial sensing, random resource selection and inter-UE coordination) are supported as a baseline for sidelink operation in shared spectrum, subject to applicable regional regulations. In dynamic channel access, SL UE performs Type 1 or one of the Type 2 LBTs before SL transmission using the selected and reserved resources. - FFS whether one or more or combination of the following options should be supported - Option 1: SL resource selection is performed based on existing R16/R17 sidelink mode 2 RA, and LBT is performed before SL transmission on the selected/reserved resources - Option 2: Type 1 LBT is performed by the UE and SL resource(s) can be (re-)selected within the COT - Option 3: UE performs joint LBT sensing results and resource selection procedure - FFS whether mode 2 resource selection procedure needs to be updated / enhanced due to channel access, including LBT inter-UE blocking, LBT failures, multi-channel access, COT sharing, LBT sensing results, etc. - FFS whether sidelink mode 2 operation can be supported / used for band n96 (5.925-7.125 GHz) in the US due to local FCC regulation - FFS whether/how multi-consecutive slots transmission can be supported for NR sidelink operation in unlicensed spectrum, including the following aspects - channel access, resource allocation and PHY channel design - FFS whether enhancement is needed between the end of the LBT procedure and the start of the SL transmission to retain channel access |

### Proposal for Week 2 Monday GTW session

FL comments/responses:

- @LGE, Type 2A could be performed by UE for S-SSB and/or PSFCH transmission without COT sharing from gNB in mode 1.

- @vivo,

  - Point 1, If we haven’t agreed to support semi-static channel access procedure, then we should go on with the dynamic one first. Anyway, the intention is to show that the UE should perform a channel access procedure before SL transmission.

  - Point 2, added.

  - Point 3, the issue on COT sharing / gNB is capable of performing LBT is already included in Topic #3. Let’s not duplicate the discussion in two places.

  - Point 4, this was suggested by Intel. I see in some contributions also talked about the same issue. So, it is better to keep it.

- @ALL, based to CableLab’s suggestion over the RAN1 email reflector, a sentence is added (with track changes ON) in both mode 1 and mode 2 RA. I also did some clear up in the proposal for easier reading.

Proposal 5 (III):

- The existing sidelink mode 1 RA including dynamic grant, Type 1 and Type 2 configured grants are supported as a baseline for sidelink operation in shared spectrum. In dynamic channel access, SL UE performs Type 1 or one of the Type 2 LBTs before SL transmission using the allocated resource(s), in compliance with transmission gap and LBT sensing idle time requirements specified in TS37.213.

  - FFS whether/how to handle LBT inter-UE blocking and LBT failures issues in mode 1 RA

  - FFS potential enhancement to SL DG/ CG

- The existing sidelink mode 2 RA schemes are supported as a baseline for sidelink operation in shared spectrum, subject to applicable regional regulations. In dynamic channel access, SL UE performs Type 1 or one of the Type 2 LBTs before SL transmission using the selected and reserved resources, in compliance with transmission gap and LBT sensing idle time requirements specified in TS37.213.

  - FFS whether mode 2 resource selection procedure needs to be updated / enhanced due to channel access, including LBT inter-UE blocking, LBT failures, multi-channel access, COT sharing, LBT sensing results, etc.

  - FFS whether sidelink mode 2 operation can be supported / used for band n96 (5.925-7.125 GHz) in the US due to local FCC regulation

- FFS whether/how multi-consecutive slots transmission can be supported for NR sidelink operation in unlicensed spectrum, including the following aspects

  - channel access, resource allocation and PHY channel design

- FFS whether enhancement is needed between the end of the LBT procedure and the start of the SL transmission to retain channel access

Contribution summary for channel access mechanism

## Regulation aspects

- Occupied channel bandwidth (OCB) requirement

  - [1/Nokia, NSB]: For Europe, ETSI harmonized standards have explicit requirements on the occupied channel BW (OCB). It is required that the occupied channel BW shall be at least 80% of Nominal Channel BW for 5 GHz bands, respectively. However, it should be also noted that on the 5 GHz bands the transmission BW may temporarily be less than 80% of the Nominal Channel BW during a channel occupancy time, with a minimum BW being 2 MHz.

  - [17/CableLabs, Charter]: In the US, FCC prohibits direct connection of a client (e.g. STA or UE) to another client for U-NII 5, 6, 7, 8, based on FCC Title 47 CFR 15.407(a)3V and (d)5.

(5) In the 5.925-7.125 GHz band, client devices, except fixed client devices, must operate under the control of a standard power access point, indoor access point or subordinate devices; Subordinate devices must operate under the control of an indoor access point.

In all cases, an exception exists for transmitting brief messages to an access point when attempting to join its network after detecting a signal that confirms that an access point is operating on a particular channel. Access points and subordinate devices may connect to other access points or subordinate devices. Client devices are prohibited from connecting directly to another client device.

    - Accordingly, SL-U operation is not supported in US 5.925-7.125GHz (U-NII 5, 6, 7, 8) bands.

    - U-NII 6 and 8 bands do not support outdoor operation (e.g., vehicular applications).

  - [33/JHU]: in the United States, the Federal Communications Commission (FCC) requires that, “In the 5.925–7.125 GHz band, client devices, except fixed client devices, must operate under the control of a standard power access point, indoor access point or subordinate devices,” and “Client devices are prohibited from connecting directly to another client device”. These regulations are for the n96 band of 3GPP. Since the sidelink is used for direct communication between the UEs, these FCC regulations appear to prohibit sidelink operations in this band. The European Union does not require the use of access points but limits the frequency of operation to 5.945 to 6.425 GHz (band n102). Devices operating in the n46 band must comply with the Dynamic Frequency Selection (DFS) requirements established to limit interference to radar systems. These requirements are applicable to UEs operating in sidelink modes 1 and 2.

- Power spectral density (PSD) limits

  - [1/Nokia, NSB]: The regulatory limit on 5GHz band may be as low as 10 dBm/MHz meaning that transmission power is severely limited for transmissions with narrow bandwidth.

- Short control signalling transmission (SCSt)

  - According to European regulation (ETSI EN 301 893), following limitations apply

    - within an observation period of 50 ms, the number of Short Control Signalling Transmissions by the equipment shall be equal to or less than 50; and

    - the total duration of the equipment's Short Control Signalling Transmissions shall be less than 2 500 µs within said observation period.

## Evaluation methodology

- Purpose of updating SL evaluation methodology and target use cases

  - Evaluate the impact of channel access mechanism, various new features and signal/channel designs on SL performance in unlicensed spectrum

    - [1/Nokia, NSB], [16/OPPO]

  - Use cases:

    - Advanced V2X: [1/Nokia, NSB]

    - Public Safety: [1/Nokia, NSB]

    - Commercial: [2/HW, HiSi], [10/LGE], [4/ZTE], [6/vivo], [11/LGE], [16/OPPO]

- Baseline evaluation methodology

  - V2X TR 37.885: [1/Nokia, NSB],

  - NR-U TR 38.889: [2/HW, HiSi], [5/CATT, GH], [8/China Telecom], [11/LGE], [16/OPPO], [28/Ericsson], [32/ Qualcomm]

  - R17 commercial use case: [4/ZTE], [16/OPPO]

- Evaluation scenarios and layout

  - [2/HW, HiSi]

    - Indoor layout and UE dropping model with N = 6 or 12 clusters and each with M=5 UEs

    - Each cluster is a circle, with a central point and radius Rmax = 15 or 10m and Rmin = 5 or 1m

    - No overlapping among the N clusters

  - [4/ZTE]

    - Deployment scenarios agreed for commercial use case is the baseline

    - Indoor scenario is used as baseline and outdoor scenario only is optional

    - For indoor scenario, option 1 (Urban macro (500m ISD) + 1 RRH/Indoor Hotzone per cell) in TR 36.843 is used as baseline, option 4 (Urban macro (500m ISD) + 3 RRH/Indoor Hotzone per cell) is optional

    - For outdoor scenario, the dropping method of outdoor scenario in tr 38.889 is reused

  - [5/CATT, GH]

    - NR indoor hotspot for indoor scenario and NR dense urban (ISD = 200m) for outdoor scenario

    - All the UEs are randomly and uniformly dropped into the simulation area

  - [6/vivo]

    - For outdoor scenario, Manhattan model in TR 37.885 is used

    - For indoor scenario, office indoor scenario in TR 38.901 is used

      - Partial BS can be regarded as Wi-Fi node, the UE is randomly deployed and selected as Wi-Fi UE or SL-U UE

  - [7/InterDigital]

    - Urban Option A in TR 37.885 for evaluation of SL-U and Wi-Fi

    - Urban Option A and highway Option A for evaluation of SL-U and NR-U

  - [16/OPPO]

    - Reuse the indoor scenario for NR-U evaluation for TR 38.889

    - Urban macro (ISD = 500m) for outdoor scenario

    - 5 UEs/STAs associated with each gNB/AP per 20MHz

  - [28/Ericsson]

    - Reuse the indoor sub-7GHz indoor scenario from TR 38.889 without gNBs

    - 5 SL-U UE pairs (10 UEs in total) in the scenario. Each pair is separated by uniform [10, 25]m

    - 10 interfering devices, randomly distributed Uniform in x and y ranges of the scenario

  - [32/ Qualcomm]

    - Reuse layouts for indoor and outdoor from the Rel-16 NR-U study

    - There are two operators to model two RATs at a time. The one is SLU, the other one can be NR-U, Wi-Fi or SL-U

    - The topology of NR-U and Wi-Fi is the same as in TR 38.889

    - The topology of SL-U is pair topology or star topology (optional) and the SLU operators are deployed uniformly at random in the area

    - For NR-U/Wi-Fi, 5 UEs/STAs associated per each gNB/AP per 20 MHz

    - For SL-U pairs: 3, 5 or 10 pairs of UEs per 20MHz

    - For SL-U stars: 3 Hub UEs with 5 associated peripheral UEs per 20MHz

- Channel model

  - Channel model and pathloss model in TR 38.901

    - [2/HW, HiSi], [16/OPPO], [28/Ericsson], [32/ Qualcomm]

  - Channel model for R17 commercial use case with necessary modifications: [4/ZTE]

  - [5/CATT]

    - For indoor scenario, channel model defined in TR38.901 (NR InH mixed Office model) is used

    - For outdoor scenario, the outdoor-to-outdoor channel in TR 36.843 (R17 commercial use case) could be a starting point

  - [6/vivo]

    - For outdoor scenario, channel model defined in TR 37.885 is reused.

    - For indoor scenario, channel model defined in TR38.901 is used.

- Traffic model

  - Periodic and aperiodic traffic types with different traffic intensity as defined for V2X

    - [1/Nokia, NSB], [5/vivo], [16/OPPO], [28/Ericsson]

  - Periodic 3 and aperiodic 2 model: [28/Ericsson]

  - FTP model 3 (TR36.889): [2/HW, HiSi], [5/CATT, GH], [27/MTK], [16/OPPO],

[32/ Qualcomm]

  - XR cloud gaming traffic model in TR 38.838: [2/HW, HiSi], [5/CATT, GH], [27/MTK]

  - FTP model 2 (TR36.843): [4/ZTE]

  - Periodic traffic (TR37.885): [4/ZTE]

  - FTP model 1/3 for Wi-Fi: [6/vivo]

- Antenna model

  - Baseline Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ

    - [2/HW, HiSi], [6/vivo], [16/OPPO], [28/Ericsson], [32/Qualcomm]

  - Optional Tx/Rx: (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ

    - [16/OPPO], [28/Ericsson], [32/Qualcomm]

- Carrier frequency

  - 5 GHz: [2/HW, HiSi], [4/ZTE], [5/CATT, GH], [6/InterDigital], [28/Ericsson], [32/Qualcomm]

  - 6 GHz: [6/InterDigital], [28/Ericsson], [32/Qualcomm]

- Simulation bandwidth

  - 20MHz: [4/ZTE], [5/CATT, GH], [7/InterDigital], [16/OPPO], [28/Ericsson], [32/Qualcomm]

  - 40MHz: [2/HW, HiSi]

  - 80MHz: [2/HW, HiSi], [4/ZTE], [5/CATT, GH], [32/Qualcomm]

- SCS

  - 15KHz: [4/ZTE], [32/Qualcomm]

  - 30KHz : [4/ZTE], [32/Qualcomm], [16/OPPO]

- Interference model

  - Statistical model: [5/CATT, GH]

  - Wi-Fi model in TR 38.889: [4/ZTE], [6/vivo], [16/OPPO], [10/LGE], [32/ Qualcomm]

  - NR-U: [7/InterDigital], [32/ Qualcomm]

  - No need: [27/MTK]

  - Asynchronous interference model: [28/Ericsson]

    - The choice of technology is up to each company

    - The traffic for SL-U UEs is the same as or 5 times bigger than the interfering devices

    - The number of SL-U devices equals that of interfering devices

- IBE model

  - IBE model for NR V2X: [5/CATT, GH], [16/OPPO]

- Performance metric

  - PRR and PIR in TR 37.885 for periodic and aperiodic traffic

    - [1/Nokia, NSB], [2/HW, HiSi], [6/vivo], [7/InterDigital], [16/OPPO]

  - User perceived throughput (UPT) and latency used NR-U (TR36.889) for FTP model

    - [2/HW, HiSi], [5/ CATT, GH], [16/OPPO], [27/MTK]

  - User perceived throughput (UPT): [6/vivo], [7/InterDigital], [32/ Qualcomm]

  - Successful UE ratio for XR traffic: [5/CATT, GH]

## Channel access mechanisms

- LBE-based dynamic channel access

  - Support: [1/Nokia, NSB], [2/HW, HiSi], [3/Spreadtrum], [4/ZTE, SC], [5/CATT, GH], [6/vivo], [7/IDC], [8/China Telecom], [9/NEC], [10/Lenovo], [11/LGE], [12/Sony], [13/Panasonic], [14/xiaomi], [15/SS], [16/OPPO], [18/FW], [19/ASUSTeK], [20/Apple], [21/CMCC], [22/DCM], [23/Sharp], [24/WILUS], [28/E///], [29/Intel], [30/BOSCH] (DG), [32/Fujitsu]

  - Not support:

- FBE-based semi-static channel access

  - Support: [1/Nokia, NSB], [2/HW, HiSi], [3/Spreadtrum], [6/vivo], [7/IDC], [10/Lenovo], [16/OPPO], [19/ASUSTeK], [21/CMCC], [23/Sharp], [29/Intel], [30/BOSCH] (CG), [32/Fujitsu]

  - FFS: [9/NEC], [15/SS], [20/Apple], [25/Transsion], [28/E///] (benefits of energy saving and low complexity are not prominent)

- LBT bandwidth per channel

  - LBT bandwidth per unlicensed channel is 20MHz: [4/ZTE, SC], [10/Lenovo], [25/Transsion]

- Type 1 LBT from NR-U (long LBT)

  - Support: [1/Nokia, NSB], [2/HW, HiSi], [3/Spreadtrum], [4/ZTE, SC] (PSCCH/PSSCH/PSFCH), [5/CATT, GH], [6/vivo], [8/China Telecom], [9/NEC], [10/Lenovo], [6/vivo], [11/LGE], [12/Sony], [13/Panasonic], [14/xiaomi], [16/OPPO], [17/CableLabs, Charter], [18/FW], [19/ASUSTeK], [20/Apple], [21/CMCC], [23/Sharp], [24/WILUS], [28/E///]

  - [5/CATT, GH]:

    - Introduce a channel occupancy extension transmission after Type 1 LBT success and before the starting time of the selected transmission resource. The maximum duration of the channel occupancy extension needs to be further studied to avoid unfairness to other RAT access and insufficient resource usage.

- Type 2A LBT from NR-U (short LBT for Tx gap = 25µs)

  - Support: [1/Nokia, NSB], [2/HW, HiSi], [3/Spreadtrum], [4/ZTE, SC] (S-SSB), [5/CATT, GH] (also S-SSB, PSFCH), [6/vivo] (all SL CHs/signals), [7/IDC] (at least S-SSB), [8/China Telecom], [9/NEC], [10/Lenovo], [11/LGE], [12/Sony] (at least S-SSB), [13/Panasonic] (at least S-SSB), [14/xiaomi] (at least for S-SSB), [16/OPPO], [18/FW], [19/ASUSTeK], [20/Apple] (at least S-SSB), [21/CMCC], [23/Sharp] (at least S-SSB), [24/WILUS] (at least S-SSB), [28/E///] (PSFCH, S-SSB only)

- Type 2B LBT from NR-U (short LBT for 25µs > Tx gap ≥ 16µs)

  - Support: [1/Nokia, NSB], [2/HW, HiSi], [3/Spreadtrum], [4/ZTE, SC], [5/CATT, GH], [6/vivo] (PSCCH/PSSCH), [8/China Telecom], [9/NEC], [10/Lenovo], [11/LGE], [12/Sony], [14/xiaomi], [16/OPPO], [18/FW], [19/ASUSTeK], [20/Apple], [21/CMCC], [23/Sharp], [28/E///] (PSFCH only)

- Type 2C LBT from NR-U (no LBT for Tx gap < 16 µs)

  - Support: [1/Nokia, NSB], [2/HW, HiSi], [3/Spreadtrum], [4/ZTE, SC], [5/CATT, GH], [6/vivo] (PSCCH/PSSCH), [8/China Telecom], [9/NEC], [10/Lenovo], [11/LGE], [12/Sony], [14/xiaomi], [16/OPPO], [18/FW], [19/ASUSTeK], [20/Apple], [21/CMCC], [23/Sharp]

- Energy detection (ED) threshold setting

  - [4/ZTE, SC]: At least the default maximum energy detection threshold is supported.

  - [11/LGE]: NR-U UL energy detection threshold adaptation procedure is used as baseline when in-coverage; Pre-configure based on target TX power when out-of-coverage.

  - [25/Transsion]: The EDT determination method for NR-U/LAA uplink can be used as a starting point for the study of EDT determination method for sidelink unlicensed access system.

- CW adjustment

  - CW size is reset to a minimum value (follows the same DL Type 1 LBT procedure) when an ACK is received (unicast, groupcast option 2): [11/LGE], [16/OPPO]

  - CW adjustment is based on counting number of NACKs for multiple groupcast PSSCH (under NACK-only / groupcast option 1): [11/LGE]

  - CW size remains the same when SL HARQ feedback disabled: [11/LGE], [16/OPPO]

  - [3/Spreadtrum] When sidelink HARQ feedback is enabled, contention window can be adjusted according to SL HARQ feedback. When sidelink HARQ feedback is disabled, the SL CBR can be considered for contention window adjustment.

  - [4/ZTE, SC]

    - For unicast sidelink transmission with ACK/NACK enabled, it is suggested to reuse the CW adjustment mechanism in NR-U.

    - For PSCCH/PSSCH transmission with ACK/NACK disabled, the adjustment mechanism of CW window needs to be further optimized.

    - For groupcast with type 1/2 HARQ feedback, the adjustment mechanism of CW window needs to be further studied.

  - [26/Fraunhofer]: Study the impact of contention window length selection on transmission characteristics such as the SL priority, PDB and HARQ feedback.

  - [29/Intel]: The contention windows size adjustment for channel access type 1 needs to be enhanced.

- CAPC

  - NR-U CAPC table for UL is used for SL-U: [4/ZTE, SC], [9/NEC], [10/Lenovo], [14/xiaomi], [20/Apple]

  - [12/Sony]: Channel access priority class p=1 is used for at least PSFCH transmission.

  - [29/Intel]: RAN1 should discuss how to relate the ProSe Per Packet Priorities (PPPP) defined in SL and the Channel access priority classes (CAPC), since the latest must be supported for compliance to regulatory requirements. Furthermore, RAN1 should send an LS to RAN2 for the identified issue.

- Inter-UE / mutual blocking

  - [29/Intel]: RAN1 should discuss whether to support sub-channelization and in case should study mechanisms to mitigate mutual blocking across frequency multiplexed transmissions. RAN1 should investigate mechanisms to mitigate interference across UEs transmitting in a TDM’ed manner, since different UEs could have overlapping pool of resources and can potentially start transmission at the same time. RAN1 should investigate the impact of the ON/OFF and OFF/ON transient period to the LBT procedure performed by a UE when transmission of a PFSCH may require LBT and a SL slot may contain a PSFCH transmission.

    - a UE may incur into SL synchronization errors (e.g., GNSS sync error or gNB synchronization error), which may be in the order of up to 3 us.

    - a gNB SL synchronization includes propagation delays that for macro cell deployments can be in the order of several us (e.g., 2 us or 4 us for gNB-UE distance of 600 m and 1200 m respectively).

    - a UE typically has an ON/OFF and OFF/ON transient period in the order of 10 us, but these transient periods may vary based on UE’s capability.

    - UEs’ transmissions are subject to different propagation delays.

- Mode 1 operation

  - [6/vivo]: In mode 1, UE still has to perform LBT for the scheduled resources, only after it successfully accesses to the channel, can it transmit on the scheduled resources.

  - [7/IDC]: For Mode 1 scheduling, a COT can be initiated by the UE and the outcome can be reported to the gNB as discussed above. The gNB can then coordinate the COT sharing in that case by allowing a COT sharing and assigning the resources to be shared.

  - [9/NEC]: In mode 1, LBT failure may lead to invalid scheduling of the sidelink resource in unlicensed spectrum. To discuss whether and how to improve the invalid scheduling caused by LBT failure in mode 1 for SL-U.

  - [11/LGE]: If the channel sensing operation parameters is controlled by gNB, it’s beneficial to have new Mode 1 reporting related to LBT failure. If consecutive LBT failures is observed, UE reports to the gNB for changing SL resources to different RB sets or change energy detection threshold.

  - [25/Transsion]: L3 RSSI measurement and channel occupancy reporting from sidelink UE should be supported in sidelink unlicensed access system.

  - [27/MediaTek]: Study impact on the timeline for Mode 1 resource allocation due to the additional time for LBT operation at UE.

- Mode 2 operation

  - [26/Fraunhofer]: Study the impact of using flexible slot structures to incorporate LBT before transmitting in a time slot with the current sensing and resource allocation procedures.

- Others

  - [4/ZTE, SC]

    - From the perspective of the system, the gap between any two SL occasions can be (pre-)configured less than a symbol, e.g., as 16us or 25us.

    - In SL-U, it is not supported that only a part of the symbols in a slot is configured as sidelink symbols, i.e., 14 symbols in a slot as the default.

  - [5/CATT, GH] Only HARQ ACK/NACK-based feedback can be supported, i.e., NACK-only based feedback is not supported in SL-U.

  - [10/Lenovo]

    - RAN1 to study the benefit of introducing the one-shot HARQ feedback, non-numerical HARQ feedback timing indicator features for sidelink unlicensed operation.

    - RAN1 to study the benefit of delaying the generation and transmission of SL HARQ feedback using non-numerical HARQ feedback timing value for an unlicensed spectrum

    - RAN1 could further study the PSFCH enhancement to mitigate problems arising due to delayed sidelink HARQ feedback reception for an unlicensed spectrum

  - [19/Panasonic]: For PSFCH, channel access priority class is p=1.

  - [20/Apple]: For S-SSB, channel access priority class is p=1.

  - [25/Transsion]: Channel bandwidth for sidelink unlicensed access system can be {10, 20, 30, 40, 60, 80} MHz.

## COT (channel occupancy time) sharing

- Support in principle

  - [1/Nokia, NSB], [2/HW, HiSi], [3/Spreadtrum], [4/ZTE, SC], [5/CATT, GH], [6/vivo], [7/IDC], [8/China Telecom], [9/NEC], [10/Lenovo], [11/LGE], [14/xiaomi], [16/OPPO], [19/ASUSTeK], [20/Apple], [21/CMCC], [22/DCM], [23/Sharp], [24/WILUS] (FFS how), [25/Transsion], [29/Intel], [31/QC], [32/Fujitsu]

  - Not support: [17/CableLabs, Charter], [28/E///] (not for PSCCH/PSSCH only)

- Semi-static COT sharing (in FBE)

  - [11/LGE]:

    - The absence of any other technology sharing the channel can be guaranteed on a long-term basis

    - The absence of certain link(s) sharing the channel can be guaranteed on a long-term basis

    - The absence of UE with SL Mode 2 resource (re)selection procedure sharing the channel can be guaranteed on a long-term basis

    - FFS how to set FFP (fixed frame period) and what is the granularity of configuration for FFP

- Questions that should be addressed

  - Under which condition(s) or scenario(s) that allow a sidelink RX UE to take the role of a responding device and make use of a shared COT from a sidelink TX UE (which is a COT initiating UE), e.g.,

    - The RX UE is a target/intended receiver UE of the TX UE

    - The RX UE performs

      - SL unicast transmission towards the TX UE

      - SL groupcast transmission where the TX UE is a group member

      - SL broadcast transmission where the TX UE is part of the SL broadcast

      - SL unicast transmission towards another UE

      - SL groupcast transmission where the TX UE is not a group member

    - How to handle receiving multiple COT sharing information

- Mode 1 gNB to SL-U UE

  - Support at least for SL scheduling (e.g., DCI format 3_0): [11/LGE], [18/FW]

  - [11/LGE]: FFS whether COT information from gNB can be obtained in group-common DCI (e.g., DCI format 2_0) and/or UL grant DCI (e.g., DCI format 0_0/0_1/0_2/1_0/1_1/1_2)

  - [29/Intel]: RAN1 should study a unified procedure for both mode 1 and mode 2 to allow a UE to share its COT with other UE(s).

- SL-U UE to UE COT sharing

  - Supported at least in SL unicast: [1/Nokia, NSB], [8/China Telecom], [9/NEC], [10/Lenovo], [11/LGE], [14/xiaomi] (also groupcast), [16/OPPO], [18/FW], [21/CMCC], [31/QC], [32/Fujitsu]

  - [31/QC]:

    - UE that obtains a COT could signal the TDRA and FDRA (potentially different FDRA in different slots) within the COT duration/bandwidth. This way other UEs could exploit this information to select the resources for transmission accordingly and try to access the channel via COT sharing. The UEs may need also a COT structure information (COT-SI) in order to understand the composition of the set of resources that are up for sharing.

    - Study methods to enable for initiating UE to resume transmissions in its COT after a gap longer than 25 μs.

## Short Control Signalling transmission (SCSt)

- According to European regulation (ETSI EN 301 893), following limitations apply

  - within an observation period of 50 ms, the number of Short Control Signalling Transmissions by the equipment shall be equal to or less than 50; and

  - the total duration of the equipment's Short Control Signalling Transmissions shall be less than 2 500 µs within said observation period.

- Support in principle: [1/Nokia, NSB], [16/OPPO], [10/Lenovo], [29/Intel]

  - For use cases / deployment where the presence of other technologies is not expected. E.g., when FBE channel access is used.

  - Possible SL channel / signal to be considered: PSFCH, S-SSB, SL configured grants

- Further study on whether to support SCSt for PSFCH and S-SSB: [2/HW, HiSi], [4/ZTE, SC], [5/CATT, GH], [16/OPPO], [22/DCM], [29/Intel]

  - Including feasibility, reliability and benefits

## Multi-channel access

- NR-U DL Type A and Type B multi-channel access

  - Support: [2/HW, HiSi], [4/ZTE, SC], [16/OPPO], [23/Sharp], [28/E///]

## CP extension (CPE)

- Support in principle from NR-U design

  - Support: [9/NEC], [16/OPPO], [22/DCM], [25/Transsion], [31/QC]

- When to apply CPE (additional transmission)

  - [22/DCM]: Always applied in the gap symbol regardless the existing of COT sharing to avoid inter-UE blocking (when some UE applies Type 2 LBT and some applies Type 1 LBT at the same time).

## Resource allocation enhancements (mode 1 and mode 2) in SL-U

- Common aspects / enhancements

  - SL-U should be supported for in-coverage and out-of-coverage UEs, therefore both mode1 and mode2 should be supported.

  - [2/HW, HiSi] For both mode 1 and mode 2, further study the LBT blocking issue among SL TX UEs.

  - [5/CATT, GH]: UE scheduling UE is not supported in sidelink unlicensed operation, since it is out of the scope of Rel-18 SL evolution.

  - [7/IDC]

    - Prioritize slot based PSSCH/PSCCH design in SL U as in SL licensed spectrum.

    - Reservation of a periodic time window for periodic type of traffic in SL unlicensed spectrum.

  - [14/xiaomi] If a TX resource includes resources from multiple RB set, a candidate resource including resource of a single RB set would be more preferred compared with a candidate resource including resource of multiple RB sets, because UE may need to perform LBT in multiple RB sets if it transmits in candidate resource including resource from multiple RB sets.

  - Burst / back-to-back transmissions (for same TB or different TBs): [4/ZTE, SC], [5/CATT, GH], [6/vivo], [10/Lenovo], [11/LGE], [15/Samsung], [21/CMCC], [22/DCM], [28/E///], [30/BOSCH], [31/QC], [33/JHU]

    - COT retaining to avoid long gap (at slot level) between SL transmissions

    - Improve channel access efficiency by reducing number of channel accesses for transmitting a TB or multiple TBs

    - Enhancements to resource reservation to enable the overbooking of multiple consecutive slots

  - Mini-slots / multiple starting positions within a slot: [31/QC]

    - Increased channel access opportunities for SL (synchronous frame-based system)

      - Improving channel access reliability and retaining COT (at symbol level) after Type-1 LBT

    - [6/vivo]: The AGC overhead and PSCCH decoding complexity would increase, when introducing multiple starting symbols in a slot or mini-slot based transmission for SL transmission. SL transmission starting from an earlier starting symbol of a given slot would block the SL transmission starting from a later starting symbol in the same slot.

    - [20/Apple] Separate starting position can be used for resource allocation with full BW or partial BW, similar to CG PUSCH in Uu link.

    - [30/BOSCH] Due to LBT variable contention window, a fixed slot boundary may not be guaranteed. For SL-U with, at least, LBT Cat-4 study whether flexible slot starting is possible and/or configurable.

    - [34/Continental] Adapting the channel access schemes of NR-U, in particular LBT, to the existing NR-SL physical layer (sensing and resource allocation, mode 2), such that channel access time can be made at symbol-level time scales

- Mode 1 RA

  - [1/Nokia, NSB]: SL SR/BSR, DCI and RRC are not affected by unlicensed channel access, but SL control and data channels (including SL SSB, PSCCH, PSSCH, PSFCH) should follow unlicensed channel access procedures and are subject to the effect of LBT failure.

  - [7/IDC]: Support configuring Mode 1 UE with time window and set of frequency resources to initiate a COT in SL-U to reduce the impact of LBT failure.

  - [22/DCM]: Study LBT-related mechanism in mode 1 resource allocation, in consideration of PDCCH misdetection/LBT failure. For example,

    - gNB indicates LBT type in SL scheduling DCI

    - gNB indicates additional TX to make a gap with a certain duration

  - [26/Fraunhofer]: In Mode 1, the gNB should be capable of providing grants after carrying out the required channel access procedures to ensure the availability of the channel, along with the relevant information such as the slot structure to be used and the COT duration.

  - [27/MediaTek] For the transmission on the reserved resources, gNB may need to consider the sufficient time gap between the reserved resources during the resource selection for the potential LBT operation. Or gNB may have to overbook some contiguous resources addressing LBT failure. Thus, it may also require some LBT related information to be known at gNB for the proper resource selection to avoid invalid resource allocation.

  - [29/Intel]: To clarify in RAN1 as a first step the intended deployment for mode 1, since multiple interpretations may exist:

    - Mode-1-a: a gNB sends scheduling DCI on the licensed carrier, and can perform sensing on SL unlicensed carrier, but it is not allowed to transmit on the SL unlicensed carrier. This implies that a gNB can sense the SL channel but cannot occupy by its own COT.

    - Mode-1-b: a gNB sends scheduling DCI on the licensed carrier and cannot either perform sensing or transmit on SL unlicensed carrier.

- Mode 2 RA

  - [1/Nokia, NSB]: While the LBT type of channel access procedures are designed for collision avoidance and fair coexistence with other RATs which use asynchronous channel access, the SL Mode 2 sensing-based procedures are designed for coordinating the resource allocation in a distributed way among SL UEs transmitting in a frame-based structure. This means that these mechanisms may not fully replace each other, on the contrary, they should operate together.

  - [5/CATT, GH] UE should perform resource selection procedure to determine the corresponding PSCCH/PSSCH transmission resources and then perform LBT procedure, the reasons are provided as following:

    - Firstly, LBT should be carried out at identified resource(s). Otherwise, UE need to perform LBT for all LBT sub-bands, which will cause higher workload for channel access operation especially when multiple LBT sub-bands are configured, such as 100MHz bandwidth.

    - Secondly, the sensing processing time (Tproc,0) and Tx processing time (Tproc,1) should also be considered. If UE starts to perform resource selection after the successful LBT procedure, then perform resource selection, due to the duration of sensing and Tx processing time, other RAT can access and occupy the channel during the duration. The previous LBT success will be useless.

  - [5/CATT, GH] (enhancement)

    - Considering that the unavailable resources caused by Type 1 channel access and additional selected candidate resources which are not really used will not indicated in SCI, so it will not affect the sensing and resource exclusion operations for other UEs.

    - The received COT can be used to identify resource selection window or be treated as the restrictions for resource selection. UE will choose resources within the received remaining COT so that this UE only need to perform Type 2 channel access. [32/Fujitsu]

  - [11/LGE]:

    - UE first performs channel access procedure (Type 1 LBT) and initiates a COT, then selects resources for its own SL transmissions within the remaining COT (which requires a short or no LBT). SL Tproc,0 and Tproc,1 can be part of / overlap with the defer time until SL transmission is ready.

    - During resource selection (e.g., MAC layer), similar to HARQ RTT time (z=a+b slots in R’16), it is necessary to guarantee the channel sensing interval (Type 1 LBT) right before each TX resource.

    - Once UE detects reserved resources of another UE and determines to exclude these resources from the candidate resource set, the UE also needs to exclude the channel sensing interval for the reserved resources to avoid inter-UE blocking.

    - If a UE determines that the channel where other UE’s SCI is transmitted is busy, the UE can expect that the channel sensing result for the reserved resource indicated by the SCI will be busy as well.

    - When UE performs SL resource (re)selection, if there is COT duration available for the UE, the UE should first select resources inside the COT duration as much as possible. If the COT is not available for the UE to transmit SL, the UE should select resource outside the COT to avoid Type 1 LBT failure.

  - [15/Samsung]: To compensate throughput and latency performance degradation in the existing resource allocation mechanism due to frequent LBT failure,

    - One solution is multiple transmission occasions for a given (re-)transmission. UE could try to pass LBT procedure in each occasion, and after LBT procedure passed, the remaining occasions can be released or used for next (re-)transmission. Correspondingly, UE needs to determine multiple resources during sensing and selection procedure. Drawback is the overbooking issue.

    - Another solution is a new trigger of LBT failure for legacy resource allocation procedure.

    - One more solution for UEs with multiple SL transmissions using different HARQ process or HARQ disabled, is to select consecutive TX resources during resource selection procedure. UE can send signals on the guard and/or AGC symbols to eliminate transmission gap and maintain channel contiguously being occupied (see accompany contribution [2] for details), then LBT procedure only happens at the beginning of first transmission.

  - [27/MediaTek]

    - Determination of the starting point of the selection window according to the LBT success occasion can avoid the invalid resource selection. Alternatively, the UE may perform resource selection at first and then perform LBT. In this case, the starting point of the resource selection window should take into account the LBT operation time in addition to the processing time T1.

    - For the transmission on the reserved resources, the UE may have to determine the time for LBT operation according to the occasion of the reserved resource for transmission. That is, LBT operation is performed up to the reserved resource (i.e., performed some time earlier than the reserved resource for transmission) taking into account LBT counter and potential LBT failure.

  - [28/E///]: To comply with CCA regulations of unlicensed spectrum and be able to reuse most of mode-2 based resource allocation procedure for SL, we believe that LBT is to be seen as a procedure that is applied on top. For example, CCA/LBT is performed before a transmission on the resources selected based on mode-2 SL resource allocation. Enhancement in mode 2 can include:

    - SL-U Mode 2 supports opportunistic transmission (i.e., early transmission) based on LBT success.

      - Step 1: A UE performs sensing and resource selection based on the resource selection procedures specified in SL Rel-16 (or Rel-17), to select resources for an initial transmission and possibly for some retransmissions of a TB.

      - Step 2: The UE starts performing CCA/LBT as soon as the packet arrives at the buffer and in addition also selects the first available resource (from the set of available resources) when the channel is found to be available by LBT procedure. We call this as opportunistic transmission. In case the channel is not found to be available by LBT procedure before the initially selected resource, the UE waits to transmit on the initially selected resource.

    - To reduce the spread of different transmissions over time, we propose to adopt ‘frequency-first’ selection instead of random selection during resource selection procedure (step 1 above).

    - LBT failure before the selected resource triggers resource re-selection.

    - RAN1 specifies enhancements to resource selection for wideband mode such that the selected resources are confined within a single channel unless TB size demands otherwise.

  - [29/Intel]: RAN1 should discuss how to combine the LBT procedure with the SL sensing and resource selection procedure.

  - [31/QC]: Study enhancements to the exclusion step in resource selection to limit the system throughput losses due to the issue of unused reserved resources due to LBT failures.

  - [33/JHU]: In the US, the FCC requires that, “In the 5.925–7.125 GHz band, client devices, except fixed client devices, must operate under the control of a standard power access point, indoor access point or subordinate devices,” and “Client devices are prohibited from connecting directly to another client device”. These regulations are for the n96 band of 3GPP. Since the sidelink is used for direct communication between the UEs, these FCC regulations appear to prohibit sidelink mode 2 operations in this band. The European Union does not require the use of access points but limits the frequency of operation to 5.945 to 6.425 GHz (band n102).

## Others

- [5/CATT, GH] (slot structure)

  - The slot structure in NR sidelink should be reused in SL-U

- [18/FW] (congestion control)

  - Modify the definition of CBR measurement in SL unlicensed access by taking into account the transmissions from other RATs.

  - Update CR definition for SL unlicensed transmissions by taking into account the transmissions from other RATs.

- [27/MediaTek]: Study support of very low power (VLP) operation for SL-U.

- [34/Continental] Adapting the resource allocation of NR-U, in particular the interlace option, to the existing SL resource pool structure.

- References

- R1-2203122	On Channel Access Mechanism and Evaluation Methodology for SL-U	Nokia, Nokia Shanghai Bell

- R1-2203146	Channel access mechanism and resource allocation for sidelink operation over unlicensed spectrum	Huawei, HiSilicon

- R1-2203331	Discussion on channel access mechanism for sidelink on unlicensed spectrum	Spreadtrum Communications

- R1-2203365	Discussion on channel access mechanism for SL-U	ZTE, Sanechips

- R1-2203461	Discussion on channel access mechanism for sidelink on unlicensed spectrum	CATT, GOHIGH 5

- R1-2203561	Channel access mechanism for sidelink on unlicensed spectrum	vivo

- R1-2203645	SL Channel access in unlicensed spectrum	InterDigital, Inc.

- R1-2203657	Discusson on channel access mechanism for sidelink on unlicensed spectrum	China Telecom

- R1-2203693	Channel Access of Sidelink on Unlicensed Spetrum	NEC

- R1-2203703	Channel access mechanism for sidelink on FR1 unlicensed spectrum	Lenovo 8

- R1-2203713	Discussion on channel access mechanism for sidelink on unlicensed spectrum	LG Electronics

- R1-2203734	Discussion on channel access mechanism for SL-unlicensed	Sony

- R1-2203749	Sidelink channel access on unlicensed spectrum	Panasonic

- R1-2203818	Channel access mechanism for sidelink-unlicensed	xiaomi 4

- R1-2203906	On channel access mehanism for sidelink on FR1 unlicensed spectrum	Samsung

- R1-2203982	Access mechanisms of NR sidelink in unlicensed channel	OPPO

- R1-2204084	SL-U General Access Considerations	CableLabs

- R1-2204097	Discussion of channel access mechanism for sidelink transmission in unlicensed spectrum	FUTUREWEI

- R1-2204194	Discussion on sidelink on unlicensed spectrum	ASUSTeK

- R1-2204247	On channel access mechanism for sidelink on FR1 unlicensed spectrum	Apple

- R1-2204306	Discussion on channel access mechanism for sidelink on unlicensed spectrum	CMCC

- R1-2204382	Discussions on channel access mechanism in SL-U	NTT DOCOMO, INC.

- R1-2204427	Discussions on Channel access mechanism for NR sidelink evolution	Sharp

- R1-2204552	Discussion on channel access mechanism for SL on unlicensed spectrum	WILUS Inc.

- R1-2204580	Disscusion of channel access mechanism for sidelink in unlicensed spectrum	Transsion Holdings

- R1-2204607	NR Sidelink Unlicensed Channel Access Mechanisms	Fraunhofer HHI, Fraunhofer IIS

- R1-2204730	Discussion on Channel access mechanism	MediaTek Inc.

- R1-2204741	Channel access mechanism for SL-U	Ericsson

- R1-2204802	On the Channel Access Mechanisms for SL Operating in Unlicensed Spectrum	Intel Corporation

- R1-2204868	Discussion on channel access mechanism for sidelink on unlicensed spectrum	ROBERT BOSCH GmbH 5

- R1-2205033	Channel Access Mechanism for Sidelink on Unlicensed Spectrum	Qualcomm Incorporated

- R1-2205082	Considerations on unlicensed channel access for SL-U	Fujitsu Limited

- R1-2205113	Contention-Based Sidelink Burst Transmissions	Johns Hopkins University

- R1-2205118	General considerations on SL-U	Continental Automotive GmbH

- R1-2204093	Discussion on V2X use cases, scenarios, and requirements for SL-U	Toyota InfoTechnology Center

- RP-220300	WID revision: NR sidelink evolution	OPPO

Appendix (outcomes of past meetings)

## RAN1#109-e (09 – 20 May 2022)

To be updated in the next meeting (RAN1#110)